3GPP TSG-RAN WG1 Meeting # 118bis														R1-2408712

Hefei, China, 14th – 18th October 2024

Agenda item:			7

Source:	MediaTek inc.

Title:	RedCap NCD-SSB MIB parameters

WI	NR_redcap-core

Release	Rel-17

Document for:	Discussion

# Introduction

At RAN1#117 and RAN1#118 in [1] and [2], an issue was discussed in relation to the setting of some parameters in NCD-SSB MIB and corresponding UE behaviour in RRC CONNECTED state when the RedCap UE is re-configured by the network to operate on an NCD-SSB.

This document proposed a resolution to the discussion.

# Discussion

The expectation oif what the UE can expect for the setting of kSSB and pdcch-ConfigSIB1 for an NCD-SSB is clearly stated in TS38.213 clause 13 (see below). As there is no reference to a particular RRC state, we see no reason to assume that this does not apply in RRC CONNECTED state.

Observation 1: The text in TS38.213 clause 13 is clear in how the UE can expect kSSB and pdcch-ConfigSIB1 to be set for an NCD-SSB in any RRC state.

We further reviewed the document [1] originating this discussion, and actually we found nothing in thay document to suggest that there was any confusion regarding the expected setting of those parameters.

In [1] a number of points are highlighted.

- That existing specification text in TS38.211 and TS38.213 clause 13 (as referred above) is highlighted as seeming to be applicable to RedCap UE for NCD-SSB reception.

- “Obviously, for RedCap UE with NCD-SSB, the UE does not need to indicate or derive the kSSB since there is no SIB to transmit associated with this SSB”.

- “From the initial understanding, for a UE in connected mode, if the NCD-SSB is configured for this case, there is no need for the UE to read kSSB and pdcch-ConfigSIB1 since the CD-SSB and CORESET#0 configuration is known by the connected UE”.

- “Option 3: NW guarantee kSSB and pdcch-ConfigSIB1 in the NCD-SSB is correct and no confusion would not cause when the UE detect this NCD-SSB. In this case, even the NCD-SSB is configured in connected mode for one UE, still, kSSB and pdcch-ConfigSIB1 should be provided appropriately. This also limit the NW implementation and may have compatibility issues.”

We find nothing in the above points that would suggest that there is any ambiguity on what the UE can expect today. What it does seem to point out is that the UE may do some behaviour that it does not need to do, and that the network cannot set those parameters to some other value. None of those points in our view signify that something is broken in the network setting kSSB and pdcch-ConfigSIB1 parameters for NCD-SSB as described with:

- 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, or

- kSSB=31 for FR1 or kSSB=15 for FR2

Observation 2: None of the above points in [1] appear to refer to something that is functionally broken if, for an NCD-SSB, the network sets kSSB and pdcch-ConfigSIB1 according to 38.213 clause 13, i.e.:

- 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, or

- kSSB=31 for FR1 or kSSB=15 for FR2

Observation 3: It is also unclear from [1] why there would be any compatibility issue between the UE and network by both following the existing TS38.213 specification.

The following changes were then proposed in [1]:

- Proposal 1: Clarify that the RedCap UE in connected mode ignores the information related to kSSB and pdcch-ConfigSIB1 in MIB of Rel-17 introduced NCD-SSB.

- Proposal 2: Conclude that how to configure the frequency location of Rel-17 NCD-SSB, and kSSB and pdcch-ConfigSIB1 in MIB of the NCD-SSB is up to gNB.

If the UE is required to ignore kSSB then it would suggest that the network can set kSSB to a value not aligned with the above, which would cause a compatibility issue if the UE expected to receive kSSB as defined in the specification text above.

Observation 4: Mandating that the UE shall ignore kSSB from MIB in RRC CONNECTED state risks a compatibility issue for Rel-17 RedCap UEs.

# Conclusion & Proposal

Observation 1: The text in TS38.213 clause 13 is clear on how the UE can expect kSSB and pdcch-ConfigSIB1 to be set for an NCD-SSB in any RRC state.

Observation 2: None of the above points in [1] appear to refer to something that is functionally broken if, for an NCD-SSB, the network sets kSSB and pdcch-ConfigSIB1 according to 38.213 clause 13, i.e.:

- 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, or

- kSSB=31 for FR1 or kSSB=15 for FR2

Observation 3: It is also unclear from [1] why there would be any compatibility issue between the UE and network by both following the existing TS38.213 specification.

Observation 4: Mandating that the UE shall ignore kSSB from MIB in RRC CONNECTED state risks a compatibility issue for Rel-17 RedCap UEs.

Proposal: Conclude that the existing specifications are clear that a UE in any RRC state can expect that kSSB is set according to TS38.213 clause 13 for an NCD-SSB, i.e.

- 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, or

- kSSB=31 for FR1 or kSSB=15 for FR2

# References

[1]		R1-2405189, Discussion on Rel-17 RedCap remaining issues, ZTE (RAN1#117).

[2]		R1-2406795, Continuation of the RAN1#117 RedCap open issues discussion, Nokia (RAN1#118)