3GPP TSG RAN WG1 #118bis			R1-2408714

Hefei, China, October 14th – 18th, 2024

Agenda item:	9.9.1

Title:	LTM measurements related enhancements

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN#105, the revised WID for Rel-19 Mobility Enhancements was endorsed and the following objectives are included involving RAN1 work [1]:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN1-118, the following agreements were made on LTM measurements [2]:

Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study

In this contribution we continue discussion on RAN1-related aspects on measurements for the purpose of supporting LTM.

# Discussion

## L1 measurements

In RAN1-118, L1-RSRP was agreed as the measurement quantity while L1-SINR is FFS. Unlike L1-RSRP, L1-SINR can take interference into account for candidate measurements. Although there may be potential benefits of incorporating the interference impact in measurements, it may also come at the potential risk of noisy measurements. If some time-domain averaging or filtering is applied, the instantaneous changes in the L1 measurement results can be smoothened out. Otherwise, without some averaging (e.g., one-shot measurements), the measurement results may not be accurate. In addition, the benefits or validity of using L1-SINR as measurement quantity may also depend on the supported LTM event definitions for report triggering, as well as definition of other event parameters (e.g., hysteresis, time-to-trigger, etc.). In our view, L1-RSRP is sufficient and there is no need to introduce L1-SINR.

Proposal 1: Do not support L1-SINR as measurement quantity.

On the frequency of the candidate cells, both intra-frequency and inter-frequency measurements should be supported. RAN4 specification have definitions for intra-frequency and inter-frequency in case of SSB-based L1 measurements. However, such definitions are missing for L1 measurement based on CSI-RS. RAN4 should introduce the definitions for intra-frequency and inter-frequency measurements for CSI-RS based L1 measurements.

Proposal 2: Both intra-frequency and inter-frequency measurements are supported.

Proposal 3: The definitions of intra-frequency and inter-frequency measurements are up to RAN4.

Another aspect is related to the CSI-RS type for measurements. In the last RAN1-118, CSI-RS for beam management was agreed while CSI-RS for Mobility is FFS. As it is typically used for L3-based measurements, the motivation to include CSI-RS for Mobility is not clear.

Proposal 4: Do not support CSI-RS for Mobility.

On the time-domain properties of CSI-RS, periodic CSI-RS was agreed while aperiodic/semi-persistent is FFS. At least for event-triggered L1 measurement reporting, UE needs to evaluate event trigger condition(s) periodically until a trigger occurs. Hence, aperiodic/semi-persistent CSI-RS may not be sufficient as gNB cannot predict/guess when an LTM event will be triggered. So, in our view, periodic CSI-RS is necessary for event-triggered reporting, at least. In addition, for gNB-scheduled L1 measurement reporting, the reports can be configured/triggered by gNB periodically/semi-persistently/aperiodically based on the L1 measurements results on periodic CSI-RS. This is similar to L1 measurements design based on SSB in R18 LTM, wherein SSB resources are periodic. There is no strong justification to support semi-persistent or aperiodic CSI-RS.

Proposal 5: Do not support semi-persistent or aperiodic CSI-RS.

## gNB-scheduled L1 measurement reporting

The CSI-RS based report configuration framework should reuse the SSB-based L1 measurements framework in R18 LTM. CRI can be used instead of SSBRI and the L1 reports can be carried in UCI.

In addition, similarly to SSB-based L1 reporting, we also do not see a need to introduce L1 filtering. As it is left to UE implementation in both R15 Mobility and Rel-18 LTM, the similar approach can be taken for CSI-RS based L1 measurement reporting in LTM.

Proposal 6: L1 filtering for gNB-scheduled L1 reporting is up to UE implementation.

## Event triggered L1 measurement reporting

Several design aspects of event-triggered reporting are discussed in RAN2. In RAN1, the discussion should continue on the identification of serving cell beam. In the last RAN1-118, the following agreement was made [2]:

Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study

It is preferable to follow RAN2 agreement that current beam should be used rather than best beam. In addition, R18 MIMO has also agreed to support current indicated beam for serving cell RS to perform event evaluations. To be aligned with both tracks, the QCL source RS of the indicated serving cell TCI state should be used. We propose to support Option-2.

Proposal 7: Support the following for serving cell RS for event evaluation:

- Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell.

Furthermore, similar to gNB-scheduled CSI-RS-based L1 measurement reporting, L1 filtering should be left to UE implementation for event-triggered reporting case, as well.

Proposal 8: L1 filtering for event-triggered L1 reporting is up to UE implementation.

## Beam management based on CSI-RS

It was discussed that one potential enhancement for CSI-RS for beam management can be configuration it as QCL source RS in LTM TCI state. In R18 LTM, the legacy configuration LTM-TCI-Info-r18 allows TRS and/or SSB as QCL RS in the LTM TCI state(s). CSI-RS for BM can already be indicated in the aforementioned parameter; however, the RAN1 specification may need to be revised to add the QCL assumptions for CSI-RS for BM. Although this is feasible from specification perspective, the benefit of supporting CSI-RS for BM as QCL source RS in LTM TCI state(s) is not very clear, as TRS is already supported in R18 LTM for this purpose.

Observation-1: Benefits of introducing additional CSI-RS-based (i.e., non-TRS-based) TCI activation and indication is not clear.

We propose the following.

Proposal 9: Do not support CSI-RS for BM as QCL source RS in LTM TCI state(s).

## CSI Acquisition based on CSI-RS

In our view, a key understanding for supporting CSI acquisition is in the timing of the CSI reporting in relation to the LTM timeline as well as the candidate cell(s) for which the CSI is reported. We identify two different design approaches.

In one approach, the CSI reporting is performed before the cell switch command arrives and after a set of candidate cell(s) are activated, if applicable. As CSI acquisition is more complex measurement process than L1-RSRP from UE perspective, CSI for multiple candidate cells may not be feasible. It is possible that even if a relatively larger number of candidate cells are activated, UE may be able to report CSI for a much limited number of candidate cell or cells. Then, the issue relies on the fact that UE does not yet know which candidate cell will be indicated in a cell switch command in the future. In addition, UE also doesn’t know whether it will in fact receive a cell switch command at all. As it is unclear whether there will be a handover or not, the additional complexity required on the UE to perform CSI acquisition is not a good design trade-off.

In another approach, the CSI reporting is performed after the cell switch command is received by the UE. The report may be sent to the new serving cell (i.e., the indicated LTM candidate cell in the cell switch command). For example, the uplink resources can be pre-configured in LTM-CandidateConfig-r18 or it can be indicated in the cell switch MAC-CE. After UE completes the handover, the next UL transmission on the corresponding UL resources may carry the UCI with CSI information.

Based on this, we propose the following:

Observation-2: There are two possible approaches to realize CSI acquisition:

  - Approach-1) CSI report for candidate cell(s) is sent to current serving cell

  - Approach-2) CSI report for the indicated candidate cell in cell switch command is sent to target cell after the cell switch is complete.

Based on our analysis above, we support the Approach-2, i.e., CSI report for the indicated candidate cell in cell switch command is sent to target cell after the cell switch is complete.

Proposal 10:  Support the following: CSI report for the indicated candidate cell in cell switch command is sent to target cell after the cell switch is completed.

# Conclusions

We have the following observations:

Observation-1: Benefits of introducing additional CSI-RS-based (i.e., non-TRS-based) TCI activation and indication is not clear.

Observation-2: There are two possible approaches to realize CSI acquisition:

  - Approach-1) CSI report for candidate cell(s) is sent to current serving cell

  - Approach-2) CSI report for the indicated candidate cell in cell switch command is sent to target cell after the cell switch is complete.

We have the following proposals:

Proposal 1: Do not support L1-SINR as measurement quantity.

Proposal 2: Both intra-frequency and inter-frequency measurements are supported.

Proposal 3: The definitions of intra-frequency and inter-frequency measurements are up to RAN4.

Proposal 4: Do not support CSI-RS for Mobility.

Proposal 5: Do not support semi-persistent or aperiodic CSI-RS.

Proposal 6: L1 filtering for gNB-scheduled L1 reporting is up to UE implementation.

Proposal 7: Support the following for serving cell RS for event evaluation:

- Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell.

Proposal 8: L1 filtering for event-triggered L1 reporting is up to UE implementation.

Proposal 9: Do not support CSI-RS for BM as QCL source RS in LTM TCI state(s).

Proposal 10:  Support the following: CSI report for the indicated candidate cell in cell switch command is sent to target cell after the cell switch is completed.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, RAN#105, Melbourne, Australia, September 2024.

- Chairman’s notes, RAN1-118, Maastricht, Netherlands, August 2024.