3GPP TSG-RAN WG1 #118bis			R1-2408724

Hefei, China, October 14 – October 18, 2024

Agenda item:	9.7.2

Title:	Discussion on ISAC Channel Modeling

Source:	          National Institute of Standards and Technology (NIST)

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN#118, the feature lead outlined the following items in their priority orders, which could be considered in the next meeting [1].

This contribution describes our polarization measurement campaign and presents findings on signal polarization changes after interaction with a target. Furthermore, we propose two target modeling options: model a target as a single scattering point and as multiple scattering points. Both approaches consider large-scale and small-scale behaviors and the spatial-temporal consistency observed in the measurement data. Note that in this document, the terms Multipath Component (MPC) and path are interchangeable.

# Polarization Study on Direct Paths

During RAN1#118, some FFS are raised regarding the polarization modelling. In particular, the following agreements have been made:

This section will describe our polarization study, which is based on measurement campaigns with our 28 GHz channel sounder, and propose some options for modeling the target's effect on polarization.

## Polarization Measurement Campaigns

The TX antenna is a phased-array antenna with 64 microstrip antenna elements, which are synthesized into a quasi-omnidirectional pattern with 90° azimuth and 50° elevation field-of-view (FoV) to illuminate the material. The RX antenna is also a phased-array antenna, however, with 256 elements, which are activated one by one in rapid sequence to estimate the angle-of-arrival of the impinging multipath components (MPCs). After each sequence, the 256 elements are synthesized into a single, electronically steerable beam with 5.2° elevation and 3.7° azimuth beamwidth in postprocessing [3]. Next, a super-resolution MPC extraction algorithm is applied to the beamformed response to estimate the azimuth angle of arrival (AOA) and zenith angle of arrival (ZOA) of each MPC, to enhance resolution beyond the inherent beamwidth of the RX antenna. The scope is to extract and characterize the individual MPCs scattered from the material.

The heights of the TX, RX, and materials under test are 1.5 m. The size of each material specimen is 60 cm x 60 cm. The distance between the RX and the material is 1.5 m, and between TX and the material is 0.7 m, as shown in Figure 1. The bistatic angle in this measurement is fixed at 60°, while the material under test is rotated in azimuth and elevation. More specifically, the material rotates over 90° in 5° steps in azimuth and from 0° to 65° (upward) in elevation in a 5° step, respectively. Therefore, the combined number of rotations in both directions is 19(AZ) x 14(EL) = 266. With this setup, a specular reflection is expected at precisely 25° azimuth rotation and 0° tilt. Each channel acquisition takes 2.6 ms, and we assume the channel remains unchanged during each acquisition.

Figure 1: Campaign Deployment Geometry

During one channel acquisition, the transmitter and receiver toggle between V and H polarization on each end, capturing all four combinations of the transmitter and receiver polarizations (i.e., H-H, H-V, V-H, and V-V) of each path. Note that, the estimated parameters of an individual MPC i include delay(τi), AOA(θiA), ZOA(θiZ), and path gains for H-H(PGiHH), H-V (PGiHV), V-H(PGiVH), and V-V(PGiVV), respectively.

We leverage the camera and Lidar system built into our context-aware channel sounder to identify MPCs scattered from the material, and more specifically to gate any MPCs scattered from the surrounding. The camera and Lidar are synchronized with the radar system and can reconstrue the measurement scenario together. We compute the scattering points using the technique described in the Appendix based on the TX and RX locations and the estimated delay and AOA of the MPC. If the scattering points fall on the test material, the MPC is classified as a direct path from the target. For example, in Figure 2 (a), two direct MPCs (red dots) can be observed, whereas the MPCs from the surrounding (green dots) are filtered out.

As shown in Figure 3, the materials under study include drywall and brick. Their surfaces range from very smooth to rough, and some surfaces are non-uniform, such as brick. The study results can be applied to targets made of different materials.

RX

TX

RX

- (b)

Figure 2: Measurement Setup.

- (b)

Figure 3: Samples Under Test (a) Drywall (b) Brick

## Target Polarization Modeling

For a given path i, its 2x2 Cross-polarization matrix (CPM) matrix, HCPM,i=SHH,iSHV,iSVH,iSVV,i  is adopted to model how its polarization components (horizontal and vertical) change due to the interaction with the target. In HCPM,i, SHH,i and SVV,i are defined as the amplitude and phase changes of the horizontal (vertical) polarized component, and SHV,i and SVH,i are cross-polarized componets, resprenting how the incident horizontally polarized compont is converted to a vertically polarized componet and vice versa. Note that, the cross-polarization ratio (XPR) is defined as the ratio of the co-polarized power to the cross-polarized power. It can be specifically written as:  XPRVH,i=SVV,i2|SVH,i|2 and XPRHV,i=|SHH,i|2|SHV,i|2.

In this contribution, we attempt to address the following questions: 1) Should the CPM matrix HCPM,i of a direct path be modeled as angle-dependent, i.e., as a function of the incident and scatter angels? 2) How likely will the diagnoal magnitudes, i.e., |SHH,i|and |SVV,i|, have the same values? Do they follow the same distribution? Are they correlated? 3) What is the relationship between the off-diagnoal magnatudes, |SHV,i| and |SVH,i| ? 4) Is there any correlation between XPRVH,i and XPRHV,i?

As we focus on  analyzing the trends and relationships between the elements in HCPM,i, our study is based on path gain values, i.e., PGiHH, PGiHV, PGiVH, and PGiVV. Note that the difference between PGi and |Si|2is due to the free space path loss from the TX to the target and from the target to the RX, which is the same for all four poloraization combinations.

To characterize a measured direct path as either specular or diffuse, we define a metric we refer to as the diffusivity angle. It quantifies the degree of diffusivity of the path or, equivalently, the deviation of the path from perfect reflection. Precisely, the diffusivity angle is defined as the angle between the scattering direction (based on the path’s measured AOA) and the expected specular reflection direction. The expected specular reflection vector (rsp ) is determined by the angle of incidence and the surface normal. The diffusivity angle (θd) is the angular difference between the vector  rsp and the vector rs, which is used to describe the direction of the scattering path. The diffusivity angle, as shown in Figure 4, is computed as  θd=arccos(rsp⋅rs), with rsp and rs as unit vectors.

Figure 4: Diffusivity Angle

- (b)

Figure 5: (a) Distribution of the Diffusivity Angle of the Direct Paths. (b) XPR vs. Diffusivity Angle for Drywall

Figure 5(a) shows the probability density function (PDF) of the diffusivity angle for the drywall material, aggregated across a total of 343 direct paths (extracted from the 266 tilts). It demonstrates that the scattering conditions present in the measurement campaign are wide-ranging. Figure 5(b) displays the XPR vs. the diffusivity angle of the same paths, showing that the XPR can have a large spread for any given diffusivity angle. While information on the surface roughness of the materials and their exact rotation orientations were available in our measurement campaign, this information will generally not be available for a generic target, which may be more intricate than the flat materials investigated here, e.g., multifaceted, each facet with a different surface roughness and orientation, etc. In light of this, it is challenging to model the XPR’s geometrical dependency on the diffusivity angle. As such, our polarization findings are presented per material, but aggregated across all plane orientations, to study the polarization changes of a direct path after interaction with the testing samples. Based on the measurement data, we investigate how to model the co-polarization and cross-polarization elements in the CPM.

Proposal 1: When dealing with a target with an irregular surface and there is no information about the orientation of the local incident area, such as modeling a target with a single scattering point, we recommend modeling the CPM statistically, independent of the incident and scattering angles. The case for how to model large flat surfaces with known slant angles is FFS.

In the remainder of this section, we shall analyze the path gain for all four combinations of the TX and RX polarizations: VV, HH, VH, and HV.

- (b)

Figure 6 (a) CDF of the Co-polarized Power at the Receiver. (b) PDF of the Difference of the Co-polarized Receive Power.

| Table 1: Drywall RX Power Correlation Coefficients ρ; P_VV; P_VH; P_HV; P_HH. P_VV; 1; 0.8028; 0.7135; 0.9426. P_VH; 0.8028; 1; 0.6787; 0.7627. P_HV; 0.7135; 0.6787; 1; 0.7127. P_HH; 0.9426; 0.7627; 0.7127; 1. | Table 2: Brick RX Power Correlation Coefficients ρ; P_VV; P_VH; P_HV; P_HH. P_VV; 1; 0.4438; 0.2158; 0.8816. P_VH; 0.4438; 1; 0.2614; 0.4810. P_HV; 0.2158; 0.2614; 1; 0.2365. P_HH; 0.8816; 0.4810; 0.2365; 1. |
| --- | --- |

Figure 6 (a) shows the cumulative distribution function (CDFs) of the paths’ co-polarized path gain (PGiVV and PGiHH) for drywall and brick materials. Drywall has a smoother surface than brick, so the receiver is more likely to detect higher co-polarized power than brick. Furthermore, the CDFs of drywall’s VV and HH path gains exhibit similar trends, indicating that SHH and SVV can be modeled using the same distribution. Figure 6 (b) shows PDFs of the co-polarized path gain ratios PGiVV/PGiHH and PGiHH/PGiVV across all paths and tilts. It demonstrates that, for any given path i, the two co-polarized components SHH,i and SVV,i may not be equal. However, as indicated by the cross-correlation matrix in Tables 1 and 2, the co-polarized and cross-polarized path gains can exhibit very high correlation.

Figure 7: Relative Cross-polarized Receive Power for Drywall and Brick Materials.

- (b)

Figure 8: (a) CDF of the XPR (b) PDF of the Difference of the VH and HV XPRs.

| Table 3: Drywall: XPR Correlation Coefficient ρ; XPR_VH; XPR_HV. XPR_VH; 1; 0.1729. XPR_HV; 0.1729; 1. | Table 4: Brick: XPR Correlation Coefficient ρ; XPR_VH; XPR_HV. XPR_VH; 1; 0.1846. XPR_HV; 0.1846; 1. |
| --- | --- |

Figure 7 illustrates the cross-polarized path gains. As expected, the brick generally exhibits higher cross-polarized power due to its rough and non-uniform surface. Additionally, VH and HV power show statistical symmetry.

Figure 8 examines the cross-polarization terms. Figure 8(a) displays the CDF of the XPR for drywall and brick, respectively. Even though there are some differences between the VH and HV curves, Brick’s VH and HV XPRs are relatively closer to each other and both are less than drywall’s. So, it doesn't affect the relative order of materials’ XPR, which suggests that XPRVH and XPRHV of the same material may be represented using a single distribution. Figure 8(b) illustrates the ratio of XPRVH to XPRHV of each direct path, indicating XPRVH,i may not equal to XPRHV,i, for a give path i. Table 4 and 5 provide the correlation coefficients for both drywall and brick’ XPRs.

Proposal 2: For a bi-static configuration and for a given path i, the |SHH.i| and |SVV,i| of  the CPM matrix HCPM,i= SHH,iSHV,iSVH,iSVV,i  may  be modeled using the same distribution. FFS if the distribution is target dependent. FFS  if and how to model the correlation between |SHH,i| and |SVV,i|.

Proposal 3: For bistatic configurations and for a given path i, the cross-polarization ratio XPRHV,i  and XPRVH,i,  derived from CPM matrix HCPM,i= SHH,iSHV,iSVH,iSVV,i  , may be modeled using the same distribution. FFS if the distribution is target dependent. FFS if and how to model the correlation between XPRHV,i  and XPRVH,i.

# Target Modelling

In this section, we present the target RCS model, which includes both large-scale and small-scale models. Additionally, we discuss the RCS's spatial and temporal consistency and modelling methodology.

When the target is near the transmitter and the receiver, the direct signals that bounce off the target can be more easily distinguished by the receiver based on their spatial separation. As a result, the receiver will detect multiple direct signals. the number of direct paths reflected off the target may depend on the total distance of the target from the transmitter (TX) and receiver (RX) and the size and shape of the target.

These multiple scattering paths can be modelled as an equivalent scatter center/point or a stochastic cluster with multiple direct paths [5] .The single-scattering-point target model comprises only one direct path from the target, and is characterized solely by its composite radar cross-section (RCS). In contrast, the stochastic cluster target model may require additional channel model parameters such as the number of direct paths or rays in a cluster, azimuth angle-of-arrival (AOA) offset, zenith angle-of-arrival (ZOA) offset, delay offset, and power decay constants with respect to these offsets. In addition, to simulate and design ISAC systems, channel model parameters such as RCS, AOA offset, ZOA offset, and delay offset should be spatially and temporally consistent.

In this section, we will describe two models based on our measurement campaign: a single-scattering-point model, which represents the target as a single scattering point, and a single cluster model, which models the target using multiple direct paths corresponding to a cluster of scattering points.

  - Measurement and Model

For this study, we conducted radio frequency (RF) measurements using monostatic and eight bistatic configurations, as depicted in Figure 9(a). In this setup, the RX remains fixed in one location, while the TX is positioned such that the normal of TX and RX antenna arrays intersect at an angle α, where α=9∘, 22.5∘, 45∘,  67.5∘ , 90∘, 112.5∘, 135∘, 167.5∘, 180∘. The angle α=9∘ is sufficiently small to represent a monostatic setup. For each configuration, we captured a total of 950 channel acquisitions with a sampling rate of 26 milliseconds (ms), for a total measurement time of 24.2 seconds. During the measurement, an AMR starts its motion, moves along a linear trajectory, and stops. The AMR moves along a predefined 2-meter (m) trajectory either towards the TX or across the TX, i.e., parallel to the TX array, as shown in Figure 9(a). The speed of the AMR is set to 0.1m/s so that the sounder samples the channel every λ/4 of AMR motion, enabling the following spatial/temporal correlation analysis. Furthermore, the environment and the AMR target are the same as in Section 2, where the EO analysis was conducted. However, in this section, we focus on modeling the characteristics of the target and how the target channel properties change with the target motion. For this reason, we apply a background subtraction algorithm and conduct the analysis only on the signal bouncing off the target.

- Floor plan with monostatic and bistatic configurations                                (b) Snapshot of one of the bistatic configuration

Figure 9: Measurement campaign for AMR with towards and across motions. In the towards motion, the AMR moves toward the TX, whereas in the across motion, the AMR moves parallel to the TX RF array.

The single-ray target model can be derived from the measurements by identifying the scattering center of the target. The scattering center is conceptualized as the direct path or ray that most accurately represents where the majority of the signal is scattered to the receiver. Once we identify the scattering center and its motion, we can study how the electromagnetic (RCS) and geometrical properties (angle and delay) evolve over time.

In dynamic scenarios, such as when the target is in motion, the RCS becomes sensitive to several factors, including variations in bistatic angle, range, and aspect angle. These variations can be especially pronounced in short-range applications. Moreover, when a target moves, its RCS is subject to rapid fluctuations due to the constructive and destructive interference of the signals scattered from different parts of the target. The phase differences between these signals can lead to significant changes in the observed RCS. To capture the RCS variations due to target motion and multi-scatterer interference, we suggest modeling the RCS as a combination of large-scale and small-scale components. The large-scale RCS accounts for the general characteristics of the target that are relatively stable over its geometric locations. In contrast, the small-scale RCS captures the rapid fluctuations resulting from the interference patterns among multiple scatterers.

The objective of our RCS model is to realize the RCS as a time-correlated stochastic process. To accomplish this, we divide the model into two components. First, we represent the RCS as the sum of large-scale RCS, and uncorrelated small-scale RCS. Next, we correlate the stochastic process with the Auto-correlation Function (ACF) modeled from the measurements.

### Modelling Target with a Single Scattering Point

The path gain of the ray at time t is defined as PGtgt=1PLtgt,  where PLtgt represents the total path loss between the TX and the target and from the target to the RX, which can be calculated as

PLtgt=PLtx-tgt+ PLtg-rxt-10log10σtg(t)+10log104πλ2 ,                          (1)

where PLtx-tgt and PLtg-rxt are the path losses incurred for the links between the TX and the target, and from the target to the RX, respectively. The parameter σtgt represents the composite coherent RCS of the target, which is defined as:

σtgt=m=1Mtgσtg, m(t)  eiϕm(t)2, 		             (2)

where Mtg is the total number of direct paths bouncing off the target and σtg,mt and ϕm(t) are the RCS and phase associated with the mth path at time t.

The target's RCS can be modeled by considering the impact of both large-scale and small-scale behaviours. Large-scale behaviour refers to the variation of RCS over multiple wavelengths of target motion. Small-scale behaviour is related to the rapid fluctuations in the RCS caused by the contribution of multiple scattering rays.

#### Large-scale RCS Model

In this contribution, we study the large-scale RCS σtg,lsθB(t) as a function of bistatic angle θB. We aggregated 15,200 RCS estimates, obtained from measurements over 950-time instants across all 9 configurations with towards and across motions. In Figure 10, the RCS estimates, and the linear fit show that the large-scale RCS decreases as the bistatic angle θB increases. The RCS is larger at low θB, i.e., when approaching a monostatic configuration.

Figure 10: Large-scale RCS model as a function of bistatic angle θB

Observation 4: RCS decreases as the bistatic angle θB increases.

Proposal 4: The Large-scale RCS component may be modeled deterministically as a function of the bistatic angle to capture the effect of the ISAC deployment, i.e., TX, RX, and target position.

#### Small-Scale RCS Distribution

To obtain the small-scale RCS distribution model, we first obtain the small-scale RCS σtg,sst by removing the large-scale impact from the RCS σtgt value as shown below:

σtg,ss(t)= σtgt- σtg,lsθB(t),	                                      (3)

The probability density function of the estimated σtg,ss(t) aligns with a Normal distribution in dBsm scale, as shown in Figure 11. The small-scale distribution indicates that there can be significant variation in the RCS around the main trend. These variations can be attributed to the coherent combination of multiple scattered signals.

Figure 11: Gaussian distribution model for small-scale RCS.

Observation 5: After detrending the RCS, the residuals exhibit significant fluctuations due to multipath propagation and phase changes of each path caused by small-scale motion.

Proposal 5:  The small-scale component of RCS is a stochastic model that represents the fluctuations superimposed on top of the large-scale RCS value, which is caused by multipath summations.

#### RCS Small-Scale Autocorrelation

To complete the analysis of the small-scale RCS process, we study the Autocorrelation Function (ACF) to determine whether the process exhibits time-dependent patterns. Since measurements are collected in time with a target moving along a trajectory, the ACF is reported in terms of lag in distance instead of time. The distance lag Δd represents the distance travelled by the target along its trajectory in the lag Δt. This approach is useful for generalizing the ACF to targets moving at any speed, as it provides insights into the spatial correlation of the process. The ACF plot in Figure 12 reveals significant correlations across multiple distance lags Δd, suggesting that the small-scale RCS is a space (time) - correlated process.

Figure 12. Small-Scale RCS autocorrelation function as a function of distance lag.

Observation 6: The ACF of the small-scale RCS shows a significant correlation across multiple distance lags, indicating that the RCS process is spatially (temporal) correlated.

Proposal 6: The RCS may be modelled as a spatial/temporal correlated process.

### Modelling Target with Multiple Scattering Points

In contrast to the single scattering point target model, we propose a single cluster model consisting of Mtg direct paths corresponding to Mtg scattering points. This modeling approach allows to represent scenarios when the target is geometrically large when compared to its distance to the TX and RX. In this case, the target scatters multiple direct paths, each one having different geometric and electromagnetic properties. In ISAC systems, the spreading of the paths within the cluster can potentially be used to describe the size and shape of the target, which can support the evaluation of applications that require more information than just the position and speed of the target. To achieve this, in addition to the measured RCS for each individual path described in Section 3.1.1, in this section, we analyse the residual of the geometrical properties, namely AOA, ZOA, and delay.

#### Number of Direct Paths

To model target with multiple scattering points, we first need to model the number of direct paths (i.e., the number of scattering points for the target). The number of direct paths depends not only on the size of the target but also on the total distance from the TX to the target and the target to the RX.

As the total distance between the TX, target, and RX increases, the number of direct paths reflected off the target progressively decreases. This trend is illustrated in Figure 13 for both human and AMR targets, which are moving in the environment. Consequently, at larger distances, the single cluster target model reduces to a point target model.

For this analysis, the number of direct paths is illustrated at different distances from TX and RX, as shown in Figure 13.

Note that, for comparison and illustration purpose, we also include the number of direct paths results from a different measurement campaign.

(a)                   		      				 (b)

Figure 13:  Number of direct paths measured for varying total distance d, considering (a) AMR target (b) human target [4]

Observation 7: The number of direct paths reflected off the target decreases as the total distance between the TX and the target, and from the target to the RX, increases.

Proposal 7: For short-range applications, the ISAC channel model accounts for the number of direct paths reflected off the target as a function of total distance.

Based on the measurements conducted for various targets with different geometries, the number of direct paths Mtg within a cluster is determined as follows:

Mtg=maxround(Mtg(μd,Ωd)),1,                                (4)

where the round(x) function is used to round off x to the nearest integer, and the max(y,1) function ensures that the target reflects at least one direct path, especially when the target is visible to both the TX and RX but far away from them. In the above expression, Mtg is the stochastic process, described as Nakagami distribution [Figure 4.10, 5], where the shape parameter µ and the scale parameter Ω are functions of the total distance d from the TX to the target, and from the target to the RX. The shape parameter μd and the scale parameter Ωd are obtained using the exponential fitting as shown below:

μd=aμexpbμ×d,                                                          (5)

Ωd=aΩexpbΩ×d,                                                         (6)

where the parameters aμ and aΩ denote the amplitude or scaling factor, while bμ and bΩ​ represent the rate or decay constant. The parameters aμ and aΩ automatically account for the different sizes of the targets as a larger target is likely to reflect more paths compared to a smaller target. The exponential fitting of μd and Ωd for human and AMR targets is illustrated in Figure 14 and Figure 15. The fitting parameters for human and AMR targets are aμ=37.21, aΩ=273.8, bμ=-0.5946,  bΩ=-0.5767, and aμ=118, aΩ=608, bμ=-0.5142,  bΩ=-0.577, respectively. The values of aμ and aΩ​ for AMR are higher than those for human, indicating that, for a given distance, the AMR target reflects more paths than human target.

- (b)

Figure 14.  Exponential fitting for shape and scale parameters for the human target

- (b)

Figure 15.  Exponential fitting for shape and scale parameters for the AMR target

Proposal 8: For short-range applications, the ISAC channel model captures the size and shape of the target by modeling the number of direct paths.  A target-specific statistical model may be used to yield the number of rays for the target cluster.

#### Angle and Delay Offset Distribution

To obtain the angle and delay offset distribution of the direct paths, the geometrical residual is first computed. We first remove the estimated scattering center, defined as power weighted centroid of measured direct paths, from each of the measured direct path form the target. The probability density functions of the AOA residual Δϕtg,m,AOA(t), ZOA residual Δθtg,m,ZOA(t), and delay residual Δτtg,m(t)  shown in Figure 16 follow Laplacian distribution. Figure 16 shows that even if the majority of the signal is scattered from the main scattering center, other scattering centers exist and define a larger scattering region than a single scattering center would do.

- (b)                                                                           (c)

Figure 16: (a) ZOA residual (b) AOA residual (c) delay residual modeling using Laplacian distribution

#### Autocorrelation of the Angle and Delay Offset

To complete the analysis of the geometrical residual, we study the ACF to determine whether these processes exhibit time-dependent patterns. As before, the ACF is reported in terms of lag in distance. The ACF plot in Figure 17 reveals significant correlations across multiple distance lags ΔD, suggesting that the residual of ZOA, AOA and delay are space (time) - correlated process.

| (a) ZOA residual | (b) AOA residual | (c) Delay residual |
| --- | --- | --- |

Figure 17. Autocorrelation function of the residual of the geometrical parameters.

Observation 9: In our measurements, we observed that the ZOA offset (i.e., residual), AOA offset, and delay offset are following Laplacian distribution.

Proposal 9: A single cluster can be used to model a target with multiple scattering points. This cluster includes a scattering center defined by the geometry and multiple stochastically modeled paths. The angle and delay offsets of these stochastically modelled paths can be determined using PDF distributions associated with the target, which are obtained from measurements. FFS how the parameters of the PDF distributions change with incident/scattering angle and the distance of the target from the transmitter and receiver.

Observation 10: In our measurements, we observed that the continuous motion of the target resulted in scattering rays maintaining spatial (temporal) consistency in angle i.e., AOA, ZOA, and delay domains.

Proposal 10: For the direct paths from the target, their spatial (temporal) consistency in angle i.e., AOA, ZOA, and delay domains can be modelled through the ACF functions obtained from measurements.

# Conclusion

In this contribution, we analysed how the signal polarization changes after interacting with a target and provided some modeling insights. we also proposed two options for modelling the target: modeling it as a single scattering point or as multiple scattering points. Both models consist of a large-scale and a small-scale component, and they also account for spatial-temporal consistency.

This study is based on the measurement data collected using NIST channel sounder, and the proposals are summarized as follows:

Proposal 1: When dealing with a target with an irregular surface and there is no information about the orientation of the local incident area, such as modeling a target with a single scattering point, we recommend modeling the CPM statistically, independent of the incident and scattering angles. The case for how to model large flat surfaces with known slant angles is FFS.

Proposal 2: For a bi-static configuration and for a given path i, the |SHH.i| and |SVV,i| of  the CPM matrix HCPM,i= SHH,iSHV,iSVH,iSVV,i  may  be modeled using the same distribution. FFS if the distribution is target dependent. FFS  if and how to model the correlation between |SHH,i| and |SVV,i|.

Proposal 3: For bistatic configurations and for a given path i, the cross-polarization ratio XPRHV,i  and XPRVH,i,  derived from CPM matrix HCPM,i= SHH,iSHV,iSVH,iSVV,i  , may be modeled using the same distribution. FFS if the distribution is target dependent. FFS if and how to model the correlation between XPRHV,i  and XPRVH,i.

Proposal 4: The Large-scale RCS component may be modeled deterministically as a function of the bistatic angle to capture the effect of the ISAC deployment, i.e., TX, RX, and target position.

Proposal 5:  The small-scale component of RCS is a stochastic model that represents the fluctuations superimposed on top of the large-scale RCS value, which is caused by multipath summations.

Proposal 6: The RCS may be modelled as a spatial/temporal correlated process.

Proposal 7: For short-range applications, the ISAC channel model accounts for the number of direct paths reflected off the target as a function of total distance.

Proposal 8: For short-range applications, the ISAC channel model captures the size and shape of the target by modeling the number of direct paths.  A target-specific statistical model may be used to yield the number of rays for the target cluster.

Proposal 9: A single cluster can be used to model a target with multiple scattering points. This cluster includes a scattering center defined by the geometry and multiple stochastically modeled paths. The angle and delay offsets of these stochastically modelled paths can be determined using PDF distributions associated with the target, which are obtained from measurements. FFS how the parameters of the PDF distributions change with incident/scattering angle and the distance of the target from the transmitter and receiver.

Proposal 10: For the direct paths from the target, their spatial (temporal) consistency in angle i.e., AOA, ZOA, and delay domains can be modelled through the ACF functions obtained from measurements.

# References

| [1] | (Xiaomi) Moderator, R1-2406302, “Summary #4 on ISAC channel modelling,” Maastricht, Netherslands, August, 2024 (RAN1 #118). |
| --- | --- |
| [2] | 3GPP TR 38.901 , “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17),” 2023-12. |
| [3] | J. Chuang, R. Caromi, J. Senic, S. Berweger, N. Varshney, J. Wang, C. Lai, A. Bodi, W. Sloane, C. Gentile and N. Golmie, “Quasi-Deterministic Channel Propagation Model for Human Sensing: Gesture Recognition Use Case,” IEEE Open Journal of Antennas and Propagation, vol. 5, no. 3, 2024. |
| [4] | R1-2400174, “Measurement data for ISAC deployment scenarios,” NIST, RAN1 #116, March 2024. |
| [5] | ATIS’ Next G Alliance, “Channel Measurements and Modeling for Joint/Integrated Communication and Sensing, as well as 7-24 GHz Communication,” Next G Alliance Report, 2024. |

# Appendix

Figure 18: Locate the LOS target based on the path delay and AOA information.

We derive the position of the scattering center using the delay and AOA, along with the known positions and orientations of the transmit and receive antenna arrays.

The first step in this estimation process is to compute an ellipsoid based on the signal delay, which defines the surface where the scattering center could be located. The equation governing this ellipsoid is given by:

|  | dt+dr = cτ, | (7) |
| --- | --- | --- |

where dt= pt-pscis the distance between the transmitter and the scattering center, and dr= pr-psc is the distance between the receiver and the scattering center. Here, pt and pr represent the positions of the transmitter and receiver, respectively, and psc is the unknown position of the scattering center. The term c  represents the speed of light, so that cτ is the bistatic range, which corresponds to the total distance traveled by the signal. The ellipsoid, as shown in Figure 18, is characterized by its foci at the positions of the transmitter and receiver. While this ellipsoid provides the surface of potential locations for the scattering center, it does not uniquely determine the exact position.

To refine the estimate further, the AOA information is incorporated, which provides a direction vector from the receiver to the scattering center. The scattering center is constrained to lie along the line described by the AOA, and its position can be expressed parametrically as:

|  | psc= pr+druAOA, | (8) |
| --- | --- | --- |

where uAOA is the unit vector in the direction of the AOA. Substituting the parametric equation into the ellipsoid equation (2) gives:

|  | pt-(pr+druAOA)+dr=cτ. | (9) |
| --- | --- | --- |

Defining pb= pt-pr, which is the baseline vector from the transmitter to the receiver, then the ellipsoid equation becomes:

|  | pb-druAOA+dr=cτ. | (10) |
| --- | --- | --- |

Solving for dr, we obtain:

|  | dr= cτ2- pb22(pbTuAOA-cτ). | (11) |
| --- | --- | --- |

The estimation process provides two possible solutions for dr, one corresponding to the scattering center in the forward direction and the other in the backward direction, which correspond to the two possible direction of the unit vector uAOA. We retain the forward solution to ensure that the scattering center estimate is in the region in front of the receiver.

The reflection loss of each path can be calculated based on the overall measured propagation loss between TX and RX, the derived free-space path loss between TX and the target, and between the target and the RX.

To compute the cross-correlation matrix, we first create four data sequences consisting of path gains for the VV, VH, HV, and HH polarization combinations for all the direct paths. Then, we calculate the correlation coefficients between these sequences.