3GPP TSG RAN WG1 #118bis						                       R1-2408758

Hefei, China, Oct 14th – Oct 18th, 2024

Agenda Item:	9.1.4.2

Source:	AT&T

Title:	Other aspects of AI/ML framework

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. Some of the aspects of the study item include:

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

In RAN#102 as part of Rel-19 Work Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [2], it was agreed to continue to further study certain aspects of the general framework for AI/ML for air interface corresponding to data collection, model transfer/delivery, model identification and its associated LCM procedures.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

  - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

NOTE: offline training is assumed for the purpose of this project.

NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference.

NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

In RAN#105[14], it was agreed to continue study on the model identification concept and LCM procedures for two sided model, training data collection and model transfer/delivery.

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

This contribution discusses the general framework of AI/ML applied to the NR air interface, specifically the remaining open issues regarding data collection, model transfer/delivery, model identification and its associated LCM procedures.

# General AI/ML framework for NR air interface

Optimization is a fundamental challenge in deploying large-scale cellular networks as configuration and adaptation of system parameters can have significant impact on key performance indicators (KPIs) such as system capacity, user QoE, latency, reliability, coverage, and numbers of active users, etc.  This is especially critical for 5G networks as they are heterogenous in terms of frequency bands/ranges, macro and small cell deployments, diverse service offerings and traffic characteristics, and coexistence of different architectures including centralized virtual RAN functions and distributed nodes to support latency-sensitive edge computing and private networks. The same industry trends which enable network virtualization and deployment of low latency/high bandwidth services are also making application of Artificial Intelligence (AI) tools such as machine learning (ML) algorithms to 5G networks feasible and scalable.

These algorithms rely on historical data for deriving system models and training as well as real-time or near-real-time data collection to adapt to different network conditions. Furthermore, a variety of use cases can be supported by AI/ML techniques as noted in the SID including CSI feedback optimization, beam management, and positioning. Many of these use cases have common requirements in terms of data collection, KPIs for monitoring and LCM procedure. At the same time, different use cases can have vastly different requirements in terms of the impact on network nodes or functionalities. This implies that the appropriate implementation of different AI/ML techniques may involve multiple interfaces, signalling procedures, processing requirements (including requirements on data aggregation or co-location with different nodes/functions) and LCM requirements.  In RAN1#114-bis [9] the following proposal was made regarding unifying the previous functionality-based LCM and the model ID based LCM.

Agreement

- Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

The following conclusion was agreed in RAN1#115 [10] and captured in the TR regarding the general framework highlighting the need for having a common framework if needed for certain aspects.

Agreement

Capture the following into the conclusion section of the AI/ML TR

The following aspects have been studied for the general framework of AI/ML over air interface for one-sided models and two-sided models.

- Various Network-UE Collaboration Levels

- Functionality-based LCM and model-ID-based LCM

- Functionality/model selection, activation, deactivation, switching, fallback

- Functionality identification and model identification

- Data collection

- Performance monitoring

- Various model identification Types and their use cases

- Reporting of applicable functionalities/models

- Method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Model delivery/transfer and analysis of various model delivery/transfer Cases

The above studied aspects for General Framework can be considered for developing/specifying AI/ML use cases and common framework (if needed for some aspects) across AI/ML use cases.

In the previous meeting while there was a difference of understanding between the companies regarding the term “common framework” and its corresponding aspects. In this meeting it is important to clarify any ambiguity and to have a common understanding regarding this term. From our point of view, it is beneficial to have a common or unified framework especially for the model ID based LCM, data collection and model transfer for different use case will lead to an efficient design of the system while ensuring forward compatibility for new use cases or enhancements. Therefore, we have the following proposal.

Proposal 1: Study the common framework for the following aspects of the different AI/ML use case.

- Model identification

- Model delivery/transfer

- Signaling for Model ID based LCM

- Performance monitoring

- Data collection

- Reporting of additional conditions

During the previous meetings there have been several discussions regarding having a unified LCM framework which can support both functionality and model ID based LCM. While there have been some agreements on this topic, such as use of model ID to be used in functionality and having a common framework, however, there is still a difference of opinion among the companies regarding there are 2 different LCM or just one unified one. It will be beneficial to have a clear and precise agreement on topic. Therefore, we propose to have a unified LCM where functionality-based operations are supported by default and model ID based operations can be used if needed.

Proposal 2: For Rel-19, support a unified LCM providing both functionality-based and model-ID-based operations.

- Functionality-based operation is supported by default.

- Model-ID, if needed, can be used in the unified LCM for model ID based LCM operations.

In RAN2#127[15] the following agreements regarding functionality-based LCM was agreed for Beam management use case.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

These terminologies have been used in several previous agreements for functionality-based LCM in general framework. It is critical that we ensure alignment between the definition of supported functionalities, applicable functionalities and activated functionalities. amongst the different use cases. Therefore, we propose that we confirm the definitions of these terms in the general framework

Proposal 3: Confirm the following definitions for supported functionalities, applicable functionalities and activated functionalities

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

## Model Identification

Model identification is critical part of the model ID based LCM framework and agreeing on the terminology and the basic understanding of how to identify and signal an AI/ML model merited considerable debate in previous RAN1 meetings and post RAN1#112.

In RAN1#110-bis [3], the following agreements were made regarding AI/ML model identification.

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

- FFS: Detailed discussion of model ID with associated information and/or model functionality.

- FFS: usage of model ID with associated information and/or model functionality based LCM procedure

- FFS: whether support of model ID

- FFS: the detailed applicable AI/ML operations

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

- Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

In RAN1#111 [3], the following agreements were further made, related to defining model formats and structure, and model identification

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

In RAN1#112[5], further agreements have been made on model/functionality identification and their relationship, and further study is still needed to understand model identification and the relation between functionality identification and model identification. Furthermore, an agreement was also made regarding different cases for model delivery/transfer to UE for UE side model or UE part of two-sided model.

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

In RAN1#112-bis-e[6], further details regarding model ID/functionality based were agreed and further study is still needed to understand different aspects that can constitute functionality/additional aspects for functionality/model ID based LCM and their reporting, and any relationship between functionality and model ID based LCM. Furthermore, it was also agreed that the UE can report updates to the applicable functionalities/models.

Agreement

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Agreement

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

- Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

In RAN1#113[7], further agreements were made to categorize the different types of model identification and further study was needed to understand their relevant aspects, necessity, and specification impact. Furthermore, it was agreed that after model/functionalities are identified a similar procedure may be used for LCM. It was also agreed to use UE capability report as starting point for indicating supporting Model ID. It was also agreed to study how to manage UE internal conditions through Model/functionality operations and AU/ML features. Finally, it was agreed to monitor offline model/functionalities for activation/selection/switching of AI/ML model/functionalities.

Agreement

For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any).

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs

- Type B: Model is identified via over-the-air signaling,

  - Type B1:

    - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

  - Type B2:

    - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

- Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

Agreement

For functionality/model-ID based LCM,

- Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring.

Agreement

Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

- FFS: applicability to model identification, Type A, type B1 and type B2

  - FFS: Using a procedure other than UE capability report

- Note: model identification using capability report is not precluded for type B1 and type B2

Agreement

Study how to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature.

Note: it does not preclude any existing solutions.

Agreement

For the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable), study necessity, feasibility and potential specification impact for methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples:

- Assessment/Monitoring based on the additional conditions associated with the model/functionality

- Assessment/Monitoring based on input/output data distribution

- Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy

- Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs)

FFS: Requirements for the assessment/monitoring to be reliable (e.g., sufficient data coverage during evaluation)

FFS: Additional aspects specific to the case where the inactive model has never been activated before, if any.

In RAN1#114[8], further agreements were made regarding reporting of applicable functionalities/models. It was also agreed that for model identification via type A will use UE capability report as starting point and for model identification e.g., via type B2 will use the same model structure as a model which has been previously identified at the NW and UE. It was also agreed to have local model ID for LCM purposes which may not be globally unique. Finally, it was observed that the site specific models provide better performance and model transfer is beneficial for these site specific models however model transfer of unknown structure have several challenges to feasibility.

Agreement

Conclude that applicable functionalities/models can be reported by UE.

Agreement

- Once models are identified via Type A, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

  - FFS: Using a procedure other than UE capability report

- Note: The support and applicability of model identification Type A is a separate discussion.

Agreement

- When a model of a known structure at UE (e.g., Case z4) is transferred from NW, the new model being identified (e.g., via Type B2) has the same structure as an previously identified model at the Network and UE

  - Note: the need of model transfer will be discussed separately

Agreement

- Model ID in RAN1 discussion may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes.

- Note: Details can be studied in the WI phase.

Observation

- Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites).

  - At least, when UE has limitation to store all related models, model delivery/transfer, if feasible, to UE may be beneficial, at the cost of overhead/latency associated with model delivery/transfer.

  - Note: On-device Finetuning/retraining, if feasible, of a single model may be an alternative to model delivery/transfer.

  - Note: a single model may generalize well in some studied use cases.

  - Note: Model transfer/delivery to UE may also face challenges, e.g., proprietary issues /burdens in some scenarios

Observation

- Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE.

In RAN#114-bis[9], the following agreements were made regarding clarification of the additional conditions and how these additional conditions can be beneficial for different purpose of LCM of AI/ML use cases.

Agreement

- For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG.

  - It doesn’t imply that additional conditions are necessarily specified

Agreement

- Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions.

- Note: whether specification impact is needed is separate discussion

Agreement

- For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

  - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

  - Model training at NW and transfer to UE, where the model has been trained under the additional condition

  - Information and/or indication on NW-side additional conditions is provided to UE

  - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

  - Other approaches are not precluded

  - Note: it does not deny the possibility that different approaches can achieve the same function.

In RAN1#115 [10] the following agreement was made regarding example use case for type B1 and B2.

Agreement

For model identification of UE-side or UE-part of two-sided models, further clarification is made as follows.

- The following are example use cases Type B1 and B2

  - Model identification in model transfer from NW to UE

  - Model identification with data collection related configuration(s) and/or indication(s) and/or dataset transfer

- Note: Other example use cases are not precluded.

- Note: Offline model identification may be applicable for some of the above example use cases

In RAN1#116[11] the following agreements were made

Agreement

- To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases.

- To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases.

  - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

  - MI-Option 2: Model identification with dataset transfer

  - MI-Option 3: Model identification in model transfer from NW to UE

  - FFS: The boundary of the options

  - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose

  - Note: other options are not precluded

Observation

The other options are proposed for model identification type B by companies during the discussion:

- MI-Option 4. Model identification via standardization of reference models. (for CSI compression)

- MI-Option 5. Model identification via model monitoring

Agreement

Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects:

- Relationship between model ID and data collection related configuration(s) and/or indication(s)

- Information transmitted from NW to UE (if any)

- Information transmitted from UE to NW (if any)

- The associated procedure

- Usage/Applicable use case(s) of MI-Option 1

Note: whether MI-Option 1 is needed or not is a separate discussion.

Conclusion

From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

In RAN1#116bis the following agreements were made

Conclusion

From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons:

- Risk of proprietary design disclosure

- Burden of offline cross-vendor collaboration

Conclusion

From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y):

- No much benefit compared to Case y

- Risk of proprietary design disclosure

- Large burden of offline cross-vendor collaboration

- Additional burden on model storage within in 3GPP network

Conclusion

- It is clarified that MI-Option 4 refers to the Option 1 of CSI compression

  - Option 1: Fully standardized reference model (structure + parameters)

Agreement

From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity)

- A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- B: UE(s) collects the data corresponding to the associated ID(s)

- C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s).

- D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model

  - relationship between model ID(s) and the associated ID(s)

  - How model ID(s) is determined/assigned, e.g.,

    - Alt.1: NW assigns Model ID

    - Alt.2: UE assigns/reports Model ID

    - Alt.3: Associated ID(s) is assumed as model ID(s)

      - “Model ID is determined/assigned for each AI/ML model” in D is not needed

    - Alt.4: Model ID is determined by pre-defined rule(s) in the specification

  - FFS: how to report

  - Note: D is to facilitate AI/ML model inference

- Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

In RAN#117[13] the following agreements were made

Working Assumption

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

- FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Agreement

From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits):

- Alt. A

  - Step A-1: UE reports the supported known model structure(s) to network

  - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1

  - FFS: whether some additional step(s), and/or whether other information is needed

- Alt. B

  - Step B-0: UE reports to NW its support of model transfer/delivery case z4

    - Note: Step B-0 may be before or after Step B-1, or not necessary

  - Step B-1: NW indicates to UE the candidate known model structure(s)

  - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

  - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

  - FFS: whether some additional step(s), and/or whether other information is needed

- Note: Other alternative(s) is not precluded

- Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

Agreement

From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity)

- AI-Example2-1

- A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling.

  - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope.

- B: UE part of two-sided model(s) is(are) developed based on at least the above dataset.

- C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW.

- FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID)

- Note: Some step(s) may not be needed for MI-Option2

- Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training.

- Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility.

- FFS: whether/how to consider UE-side additional condition(s) for the dataset

In RAN1#118[14[ the following agreements were made

Conclusion

- From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

Conclusion

The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work.

Conclusion

The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work.

Agreement

Confirm the following Working assumption.

Working Assumption

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

- FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Agreement

From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

  - Number of layers

  - Layer types/structure (e.g., full connected, activation layer and so on)

  - Layer size (e.g., the number of parameters of a layer)

  - Connection between different layers

- model input/output related information

Conclusion

From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

For any given AI/ML use case the functionality-based LCM only allows a basic control and management of the AI/ML model. However, model identification allows model level management by NW of UE side models which is needed for several features that cannot be supported by functionality-based LCM alone. The model level management may be needed to support model transfer for UE sided model, pairing of 2 sided models, have consistency between training and interference for NW side additional conditions, provide enhanced performance for give scenario/configuration/dataset, have more fine-tuned performance monitoring, have more awareness on NW side to UE model switching. During the previous meeting some of these aspects for model identification have been discussed in some detail, however these agreements only focus on a specific aspect where the model identification is used, and it benefits. It will be better to have an agreement regarding which aspects are supported by model identification to also clarify the boundary between model ID and functionality.

For model identification type A it was agreed to use the user capability report as the starting point. However, the model identification type B1 and B2 may be used along with model transfer therefore the user capability report may not be used to support model identification type B1 and B2. Therefore, we propose to study procedures apart from user capability report to support model Identification type B1 and B2. Furthermore, for model identification type B1 and B2 the network will assign the model ID for the identified models if needed.

Proposal 4: For both model identification Type B1 and B2

- Network assigns the model ID(s) for the identified model(s) if model ID(s) assignment is needed

- FFS: How to define a model ID for assignment

It is important to understand the relationship between the model ID and associated ID in the AI-example 1 of MI-option1. We believe all the 4 options are valid and can be used based on what level of detail regarding the model needs to be identified to the network for a given use case and/or scenario. Therefore, we propose,

Proposal 5: Regarding the relationship between model ID(s) and the associated ID(s) in AI-Example1 of MI-Option1, further study the following options (including the necessity/benefit)

- ID-Rel-Option1: One model ID is linked to one associated ID by one-to-one mapping

- ID-Rel-Option2: One model ID can be linked to multiple associated IDs and each associated ID is only be linked to one model ID

- ID-Rel-Option3: One associated ID(s) can be linked to multiple model IDs and each model ID is only linked to one associated ID

- ID-Rel-Option4: Model ID(s) can be linked to associated ID(s) by many-to-many mapping

We also need to discuss the different options regarding which entity is responsible for assigning the model ID. In previous meeting 4 different options were discussed however the proposal to down select them was not agreed. We propose to discuss the down selection of different options in this meeting. Our priority is for Opt.1 where NW assigns Model ID and Opt.3 where associated ID is assumed as model ID.

Proposal 6: For AI-Example1 of MI-Option1, study and down-select the following alternatives on determining/assigning model ID (if supported).

- Opt.1: NW assigns Model ID

- Opt.2: UE assigns/reports Model ID

- Opt.3: Associated ID(s) is assumed as model ID(s)

  - “Model ID is determined/assigned for each AI/ML model” in D is not needed

- Opt.4: Model ID is determined by pre-defined rule(s) in the specification

Proposal 7: Prioritize study of Opt.1 and Opt.3 for the assessment of Model ID, in Rel-19.

# ML Model Life Cycle Management

## Model transfer

Model transfer is one of the critical aspects of LCM that is currently studied in this SI. Therefore, it is important to have an agreement that over-the-air model transfer for both UE-sided models and UE-part of two-sided models in Rel-19. The details of any down scoping regarding model transfer can be left to each sub use-case.

Proposal 8: Model transfer/delivery is supported for both UE-sided models and UE-part of two-sided models in Rel-19.

Note: Which aspects of model transfer/delivery are supported should be discussed in each sub-use-case.

Proposal 9: Regarding model transfer/delivery Case z4, Rel-19 study focuses on the option with standardized known model structure(s).

## Additional conditions

In the previous meetings we combined the model ID based LCM and functionality-based LCM into a unified LCM framework. We made further agreement regarding the additional conditions and their applicability in RAN1#114-bis[9]

Agreement

- For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

  - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

  - Model training at NW and transfer to UE, where the model has been trained under the additional condition

  - Information and/or indication on NW-side additional conditions is provided to UE

  - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

  - Other approaches are not precluded

  - Note: it does not deny the possibility that different approaches can achieve the same function.

In this meeting we need to expand on this proposal and see how it maps to the different model identification types. We should have a simple table to capture the details and requirements of the different options for the additional conditions. However, we should aim to capture the table as an observation and aim to have all aspects captured correctly. We should not have any debate on if any of the procedures can be combined or if it needs to be supported and leave those aspects to the WI. We propose the following table.

Proposal 10: The following table captures the different approaches through which the additional conditions can be indicated and how they can provide the consistency between the training and inference.

| Approach | How NW-side additional conditions are indicated | How to ensure consistency between training and inference regarding NW-side additional conditions | Analysis |
| --- | --- | --- | --- |
| Model identification Type A | Aligned offline | Indicated via an ID (model ID or ID for additional condition) for model selection | There is an offline alignment between the NW and UE regarding additional conditions and the associated model ID. The NW provides the model ID for the correct model to select for the UE based on its additional conditions. |
| Model Identification Type B2/ Model training at NW and transfer to UE | NW provides an ID in form of dataset ID or part of model ID to the UE. The UE reports the model ID for the model trained using these additional conditions. | The NW provides the UE with the ID for model selection | The NW provides an ID such as dataset ID or model ID (or part of model ID). The UE provides/confirms the model ID that was trained for the additional conditions. The NW can provide the model ID to select the appropriate model at the UE. |
| Assistance information | Provided to UE for dataset categorization in the form of an ID (determined by the NW) | Provided to UE for (transparent) model selection in the form of ID | The NW generates an ID for its additional conditions for data collection and provides it to UE to train appropriate models. The NW can later provide the additional condition during inference to assist the UE to transparently select the appropriate model. |
| Assisted Monitoring | NW provides an ID for additional condition to the UE | .. | For the models at the UE the NW provides an ID for the additional conditions. It can be provide assistance to the UE to determine if switch or turn off its model for certain additional condition (as performance requirements would not be met). |

# Conclusion

In this contribution, we discussed the general framework of AI/ML applied to the NR air interface. The following proposals were made:

Proposal 1: Study the common framework for the following aspects of the different AI/ML use case.

- Model identification

- Model delivery/transfer

- Signaling for Model ID based LCM

- Performance monitoring

- Data collection

- Reporting of additional conditions

Proposal 2: For Rel-19, support a unified LCM providing both functionality-based and model-ID-based operations.

- Functionality-based operation is supported by default.

- Model-ID, if needed, can be used in the unified LCM for model ID based LCM operations.

Proposal 3: Confirm the following definitions for supported functionalities, applicable functionalities and activated functionalities

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Proposal 4: For both model identification Type B1 and B2

- Network assigns the model ID(s) for the identified model(s) if model ID(s) assignment is needed

- FFS: How to define a model ID for assignment

Proposal 5: Regarding the relationship between model ID(s) and the associated ID(s) in AI-Example1 of MI-Option1, further study the following options (including the necessity/benefit)

- ID-Rel-Option1: One model ID is linked to one associated ID by one-to-one mapping

- ID-Rel-Option2: One model ID can be linked to multiple associated IDs and each associated ID is only be linked to one model ID

- ID-Rel-Option3: One associated ID(s) can be linked to multiple model IDs and each model ID is only linked to one associated ID

- ID-Rel-Option4: Model ID(s) can be linked to associated ID(s) by many-to-many mapping

Proposal 6: For AI-Example1 of MI-Option1, study and down-select the following alternatives on determining/assigning model ID (if supported).

- Opt.1: NW assigns Model ID

- Opt.2: UE assigns/reports Model ID

- Opt.3: Associated ID(s) is assumed as model ID(s)

  - “Model ID is determined/assigned for each AI/ML model” in D is not needed

- Opt.4: Model ID is determined by pre-defined rule(s) in the specification

Proposal 7: Prioritize study of Opt.1 and Opt.3 for the assessment of Model ID, in Rel-19.

Proposal 8: Model transfer/delivery is supported for both UE-sided models and UE-part of two-sided models in Rel-19.

Note: Which aspects of model transfer/delivery are supported should be discussed in each sub-use-case.

Proposal 9: Regarding model transfer/delivery Case z4, Rel-19 study focuses on the option with standardized known model structure(s).

Proposal 10: The following table captures the different approaches through which the additional conditions can be indicated and how they can provide the consistency between the training and inference.

| Approach | How NW-side additional conditions are indicated | How to ensure consistency between training and inference regarding NW-side additional conditions | Analysis |
| --- | --- | --- | --- |
| Model identification Type A | Aligned offline | Indicated via an ID (model ID or ID for additional condition) for model selection | There is an offline alignment between the NW and UE regarding additional conditions and the associated model ID. The NW provides the model ID for the correct model to select for the UE based on its additional conditions. |
| Model Identification Type B2/ Model training at NW and transfer to UE | NW provides an ID in form of dataset ID or part of model ID to the UE. The UE reports the model ID for the model trained using these additional conditions. | The NW provides the UE with the ID for model selection | The NW provides an ID such as dataset ID or model ID (or part of model ID). The UE provides/confirms the model ID that was trained for the additional conditions. The NW can provide the model ID to select the appropriate model at the UE. |
| Assistance information | Provided to UE for dataset categorization in the form of an ID (determined by the NW) | Provided to UE for (transparent) model selection in the form of ID | The NW generates an ID for its additional conditions for data collection and provides it to UE to train appropriate models. The NW can later provide the additional condition during inference to assist the UE to transparently select the appropriate model. |
| Assisted Monitoring | NW provides an ID for additional condition to the UE | .. | For the models at the UE the NW provides an ID for the additional conditions. It can be provide assistance to the UE to determine if switch or turn off its model for certain additional condition (as performance requirements would not be met). |

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.

[2] RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[3] Chair’s notes, RAN1#110bis, e-meeting, October 2022

[4] Chair’s notes, RAN1#111, Toulouse, France, November 2022.

[5] Chair’s notes, RAN1#112, Athens, Greece, February 2023

[6] Chair’s notes RAN1#112bis, e-meeting, April 2023.

[7] Chair’s notes RAN1#113, Incheon, Korea, May 2023.

[8] Chair’s notes RAN1#114, Toulouse, France, August 2023.

[9] Chair’s notes RAN1#114bis, Xiamen, China, October 2023.

[10] Chair’s notes RAN1#115, Chicago, USA, November 2023.

[11] Chair’s notes RAN1#116, Athens, Greece, February 2024.

[12] Chair’s notes RAN1#116bis, Changsha, China, February 2024.

[13] RAN1#117, Chair’s notes, May 2024.

[14] RAN1#118, Chair notes, August 2024

[15] RAN1#127 Chair’s Notes, Maastricht, Netherlands, 19 - 23 August 2024