3GPP TSG RAN WG1 #118bis			R1-2408779

Hefei, China, October 14th – 18th, 2024

Source:	NTT DOCOMO, INC., NTT CORPORATION

Title:	Discussion on CSI enhancements

Agenda Item:	9.2.2

Document for: 	Discussion and Decision

- Introduction

In RAN#105 meeting, the WID “NR MIMO Phase 5” was revised. The objectives for CSI enhancements are updated as follows.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we discuss CSI enhancements for up to 128 ports and non-ideal CJT deployment.

- CSI enhancements for up to 128 ports

2.1 Type I codebook refinement

2.1.1 UE behaviours on CSI-RS measurement and CQI calculation

UE behavior on CSI-RS measurement

In legacy spec., for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement is defined as follows, except for Type-II doppler CSI, which is defined separately.

- The UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise.

If K NZP CSI-RS resources are configured for 48, 64, or 128 ports, the “at least one CSI-RS transmission occasion” in legacy should be clarified as “at least one CSI-RS transmission occasion for each of the K CSI-RS resources in the corresponding CSI-RS resource set”.

Proposal 2-1

- If K NZP CSI-RS resources are configured for 48, 64, or 128 ports for Rel-19 Type-I/II codebook refinement, except for Type-II doppler CSI based refinement, for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement is updated as follows.

  - The UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the K CSI-RS resources in the corresponding CSI-RS resource set for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise.

2.1.2 Scaling factor for power control offset configuration

In last RAN1 meeting, following agreement was made for 3-bit scaling factor for power control offset configuration.

Agreement For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, study, for RI= ϑ >1, applying the 3-bit scaling factor(s) as agreed in RAN1#117, where a per-layer scaling factor applied to the ith selected SD basis vector is given by e.g. min1,ϑsi2ri, where unit scaling factor “1” is associated with the PDSCH-to-CSIRS EPRE offset “portion” contributed by the ith selected SD basis vector without the 3-bit scaling factor si configured, e.g. si∈1, 12, 13 ,14,16,18,112,116 is the scaling factor associated with the ith SD basis vector, and ri∈1,2 is the number of layers transmitted using the ith SD basis vector. - Note: This feature is a separate UE capability - Study whether per-SD-basis-vector/layer power adjustment (including boosting) needs to be supported in addition

We generally support to extend the 3-bit scaling factor for RI > 1 case for interference mitigation. But regarding the last FFS on whether power boosting is supported, we tend to think it is not needed. We see that there may be potential performance gain brought by power boosting. However, it will largely increase UE complexity for CSI calculation to achieve some marginal gain. Thus, it should not be supported.

This issue was discussed in offline email discussion [2] before this RAN1 meeting, and the views were divergent. Thus, FL proposed following proposal for compromise, in which only RI = 2 case is additionally supported. We understand current status and we could accept this Offline Proposal 1.C.1.

Proposal 1.C.1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the support for the 3-bit scaling factor(s) for RI=v >1, support only for RI=v=2 without per-SD-basis-vector/layer power adjustment/boosting - FFS: Details on per-layer scaling factor applied to each of the selected SD basis vectors, extending the agreed scaling factor for RI= v =1 si∈1, 12, 13 ,14,16,18,112,116 (in RAN1#117)

Proposal 2-2

- Support the 3-bit scaling factor configuration for RI > 1 case. We also support the Offline Proposal 1.C.1.

  - Do not support additional power boosting.

2.1.3 Active resource counting

In last RAN1 meetings, agreements were made for CSI process timeline and CPU occupation. But there is no agreement on active resource counting yet.

This issue was also discussed in offline email discussion [2] before this RAN1 meeting.

Proposal 1.B: For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, active resource counting is: - For Capability 1 timeline: 1 - For Capability 2 timeline: 1

Since CPU occupation considers the K aggregated CSI-RS resources jointly, for active CSI-RS resource counting, those K aggregated CSI-RS resources can be counted as one CSI-RS resource, except for P/SP CSI-RS resource for Type-II doppler CSI. Thus, we support Offline Proposal 1.B.

Proposal 2-3

- For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, regarding active resource counting, support Offline Proposal 1.B.

2.1.4 SRS port grouping

SRS port grouping was newly included into Rel-19 MIMO WID. It was discussed in offline email discussion [2] before this RAN1 meeting and there was a following FL proposal.

Proposal 1.A: For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource): - SRS port group 0, corresponding to CW0, comprises the even PSRS/2 out of PSRS ports; and - SRS port group 1, corresponding to CW1, comprises the odd PSRS/2 out of PSRS ports The above feature is applicable only for reportQuantity = ‘cri-RI-CQI’ No other spec enhancement is introduced on new CW-to-layer mapping, DL resource allocation, CSI feedback, and DCI format Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’. Note: different SRS ports are associated with different UE antenna ports. Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW, i.e. no enhancement is needed for the single-CW case Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration) Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI FFS (by RAN1#118bis): Whether there is impact on mapping between CWs to CSI-RS ports [For SRS antenna switching with multiple aperiodic SRS resource sets, PSRS ports indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource]

SRS port grouping is beneficial for reducing receiver complexity at UEs with 6RX or 8RX while keeping a comparable performance. The proposed fixed SRS port grouping requires only small specification impact. Thus, we support it. Regarding the FFS on mapping between CWs to CSI-RS ports, after reading companies’ input, we see that this issue exists only when ‘cri-RI-CQI’ is configured but ‘non-PMI-PortIndication’ is not provided, which should be clarified first. In this case, based on current predefined mapping between CSI-RS port and layer, one layer may be mapped to two SRS port groups. We think two simple solutions can be considered for this issue. First is to always provide ‘non-PMI-PortIndication’ so that the mapping between CSI-RS port and layers is configured by NW. Second is to predefine another mapping rule for SRS port grouping. Regarding the case when multiple aperiodic SRS resources sets are configured, we agree the port grouping should be also considered. Thus, we’re fine to support PSRS ports to be indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource.

Proposal 2-4

- For SRS port grouping for UEs with 6RX or 8RX, support Offline Proposal 1.A.

  - For the FFS on mapping between CWs to CSI-RS ports, the issue is discussed for the case only when ‘cri-RI-CQI’ is configured but ‘non-PMI-PortIndication’ is not provided. Two options can be considered.

    - Option 1: ‘non-PMI-PortIndication’ is always provided.

    - Option 2: predefine another mapping between CSI-RS ports and layers.

  - For SRS antenna switching with multiple aperiodic SRS resource sets, support PSRS ports to be indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource.

2.2 Type II codebook refinement

2.2.1 UE behavior on CSI-RS measurement

In legacy spec., for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement for Type-II doppler CSI is defined as follows,

- The UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

If KDOPP CSI-RS resource groups with K NZP CSI-RS resources per group are configured for 48, 64, or 128 ports for Rel-18 Type-II Doppler codebook refinement, the “at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource” in legacy should be clarified as “at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS Resource Set”.

Proposal 2-5

- If KDOPP CSI-RS resource groups with K NZP CSI-RS resources per group are configured for 48, 64, or 128 ports for Type-II doppler CSI refinement, for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement is updated as follows.

  - The UE reports a CSI report only after receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

2.3 Multi-CRI reporting enhancement

2.3.1 CMR configuration and CMR reporting

For multi-CRI reporting, additional indication of MR (<M) CSI-RS resources was agreed. But the signaling details are not discussed yet. Although it could be up to RAN2 as shown in RRC list for ‘mrSelectedResources’, it is good that RAN1 provides some suggestion.

Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for A-CSI only, the NW can configure MR (<M) of KS CSI-RS resources to be selected as part of reporting the M “quadruplets”: - (M–MR) CRIs, each with log2KS-MR bits are reported, along with the M sets of CQI/PMI/RI/(if applicable) LI - The value of MR is NW-configured via higher-layer (RRC) signaling - The MR selected resources are NW-configured via higher-layer (RRC) signaling - FFS: value of MR - This is an optional UE capability Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if MR is configured, the supported value(s) of MR are as follows: - Rel-15 Type-I SP: {1, 2} - Rel-16 eType-II: {1}

For multi-CRI reporting for Type-I codebook, up to KS = 8 and up to M = 4 can be configured. To indicate MR (<M) from KS CSI-RS resources, KS-bit bitmap indication would be a simple and clean configuration signaling. For multi-CRI reporting for Type-II codebook, since up to KS = 4 and up to M = 2 can be configured, there is only the case to indicate MR = 1 CSI-RS resource from the KS CSI-RS resources. For this case, bitmap indication is not needed, and  log2KS-bit indication is sufficient.

Proposal 2-6

- For multi-CRI reporting, on configuration of MR CSI-RS resource(s) (i.e., mrSelectedResources).

  - For Type-I based enhancement, support KS-bit bitmap indication.

  - For Type-II based enhancement, support log2KS-bit indication for MR = 1 CSI-RS resource (when KS > M = 2 is configured).

- CSI enhancements for non-ideal CJT deployment

It was pointed out (even during Rel-18) that Rel-18 CJT CSI may not work as expected if several offsets among multiple TRPs (which is inevitable in real fields) are considered. Therefore, as captured in WID, Rel-19 MIMO WI has been decided to work on this issue based on UE assistance, i.e., introduction of UE reporting to help gNB/NW operate CJT properly.

According to WID description, the following baselines are identified already:

- Target scenarios: FR1, both FDD and TDD

- Offsets to be measured/reported: Inter-TRP, time misalignment, frequency/phase offsets

- Report: Stand-alone aperiodic reporting on PUSCH

Based on the situation above, our view is described below.

3.1 Sub-band reporting for inter-TRP PO reports

In RAN1#118, the followings were agreed:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1): - Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - FFS: - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs} - Whether the UE performs measurement over the entire configured CSI reporting band WCSI - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P) - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1) {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}: - Supported sub-band size(s) {1, 2, 4, 8, 16} PRB - The NW configures, via higher-layer (RRC) signalling, which NSB-P sub-band(s) the UE reports.

We think RAN1 needs to discuss the overhead limit for CSI reports as highlighted. For sub-band PO reporting, the report size may exceed the maximum input sequence size of the Polar coding, Amax, depending on the transmission bandwidth, NRB, and sub-band size, NPRBSB. Therefore, the NW needs to select a sub-band size that satisfies the following equation.

log2NTRP+NTRP-1log2MΦ×NRBNPRBSB≤Amax , #x1

where Amax=1706 bits. MΦ is the PO resolution which was agreed in RAN#116bis, and the UCI parameters of the PO were agreed in RAN1#118. Therefore, if the number of TRPs, NTRP, is 4 (maximum) and the PO resolution is 32 (maximum resolution according to the latest agreements), the report size of nref is log2NTRP=2 bits, and that of the phase offset per sub-band is NTRP-1log2MΦ=15 bits. For example, in the case of NRB=273, which is the maximum transmission bandwidth for FR1 specified in table 5.3.2-1 of TS38.101-1, if NPRBSB=4, the PO report size is 2+15×2734=1037≤1706, which satisfies equation (x1). However, if NPRBSB=2, the PO report size is 2+15×2732=2057≤1706, which does not satisfy equation (x1). Therefore, it has to be ensured that the number of subbands is not excessive. We think that the other FFS listed in the agreement of the last meeting are not necessary.

Proposal 3-1

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1) {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}:

- Define the max number of subbands to ensure reasonable UCI bit overhead, e.g., 16

3.2 UE-specific delay offset pre-compensation

In RAN1#118, the following was agreed:

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) can be separately or jointly triggered [and carried on a same PUSCH (hence on a same slot)] following legacy joint triggering mechanism - (Working Assumption) When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting - FFS: whether some expiration time interval is needed - (Working Assumption) When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance FFS: NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report FFS: Whether only AP-CSI-RS, or any type of CSI-RS (P, SP, or AP) can be configured as the CMR for the Rel-18 eType-II CJT reporting FFS: Support for SP-CSI for CJTC report FFS: Whether this is also applicable for Rel-19 Type-I MP codebook The above only applies when the CMRs do not share common QCL source for average delay indication The above is UE optional feature.

When the DO and Rel-18 eType-II CJT CSI report are separately triggered, it was agreed as WA that the latest reported DO is used to calculate PMI at the UE. Meanwhile, an FFS was captured to continue the discussion on the need for expiration time about the DO referred for precompensation of PMI, however, we think that DO expiration time interval doesn’t need to be specified. We believe the DO change isn’t that fast, and thus a misdetection of the corresponding DO report is not a significant problem.

Another potential issue is how to handle the case where DO report indicates out-of-range for one or more TRPs (CMRs). For separate triggering, we believe gNB implementation can handle this well, e.g., once it detects out-of-range for a TRP, it can decide not to trigger CJT CSI report, assuming that DO precompensation will not be applied to the CJT CSI codebook to be reported. On the other hand, in case of joint triggering, this may not be the case since the assumption is that gNB will trigger both DO report and CJT CSI report by a singe PDCCH.

Proposal 3-2

- For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’) when the DO and Rel-18 eType-II CJT CSI report are separately triggered, expiration time interval should not be specified.

  - i.e., support to confirm the WA as they are

3.3 Details for SRS associated with PO report

So far, the following are agreed in terms of details for SRS associated with PO report, with FFS points highlighted in yellow:

[118] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition - Note: This doesn’t have any spec impact (no new RRC parameter is needed) - … FFS: - Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering - The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic) - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource [118] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of available port(s)) is NW-configured via higher-layer (RRC) signalling [118] Working Assumption For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic

First, for time domain behavior for the SRS resource, we believe any of periodic/semi-persistent/aperiodic should be available. Thus we propose to confirm the WA.

Regarding the identification of a port/resource of SRS for reference UE antenna port, our view is that:

- When periodic or semi-persistent associated SRS resource is configured, it seems straightforward that the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the NTRP CSI-RS occasions used for measuring ‘cjtc-P’ report.

- When aperiodic associated SRS is configured, it results in that only 1 port in the configured Q=1 SRS resource will be identified without additional information. Thus, we see no need of additional determination rule for SRS occasion in case of aperiodic SRS

Proposal 3-3

- Confirm the WA for time-domain behavior for SRS associated with PO report

- When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the NTRP CSI-RS occasions used for measuring ‘cjtc-P’ report.

- When aperiodic associated SRS is configured, no need of additional determination rule for SRS occasion

3.4 UE behavior on CSI-RS measurement

In TS 38.214, the following is specified for Rel-18 TDCP reporting:

#### 5.2.2.5 CSI reference resource definition […] For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise. […]

That is, Rel-18 TDCP report is expected only when UE can measure sufficient number of NZP CSI-RS resources such that the measurement is properly performed. Similar aspect should be considered for Rel-19 CJT calibration report as well.

For DO/FO reporting, as it reuses legacy TRS resource set, the above text for Rel-18 TDCP can be almost fully reused. The minimum specification is to add ‘cjtc-Dd’, ‘cjtc-F’, and ‘cjtc-Dd-F’ for reportQuantity. Unless additional resource configuration is agreed, we believe this change suffices.

For PO reporting, unlike DO/FO reporting, at least one ‘CSI-RS for CSI’ resource set with NTRP resources is supported. UE needs to measure each resource, not resource set here. Therefore “at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement” should be ensured, which is rather similar to the one for PMI reporting.

Irrespective of DO/FO/PO, we believe the same set of events, i.e., the CSI report (re)configuration, serving cell activation, BWP change, and whether it is located in the same DRX active time when DRX is configured, should be taken into account.

Proposal 3-4

- For the Rel-19 aperiodic standalone CJT calibration reporting, for DO report, FO report, and joint reporting of DO and FO, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

- For the Rel-19 aperiodic standalone CJT calibration reporting, for PO report, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

3.5 OCPU and ARC for phase offset report

For timeline/ARC/OCPU for CJT calibration report, the following is agreed so far:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding timeline, fully reuse those from Rel-18 TDCP reporting Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or cjtc-F’ (frequency offset), fully reuse those from Rel-18 TDCP reporting - OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO) - Fully reuse timeline and active resource counting from Rel-18 TDCP reporting - OCPU = 2X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type

It is very clear based on the first agreement that for timeline, irrespective of reported metric (i.e., delay offset, frequency offset, phase offset), the one supported for Rel-18 TDCP reporting is fully reused. Meanwhile, the remaining two agreements, for ARC and OCPU, focused on delay and/or frequency offset reporting. Therefore, RAN1 needs to conclude ARC and OCPU for phase offset reporting. One difference between phase offset reporting and the others (including TDCP reporting) is that subband reporting in addition to wideband reporting is supported for phase offset reporting.

We believe that, irrespective of wideband or subband, ARC and OCPU for PO report should fully follow what is supported for delay and frequency offset report. We’d like to point out that for ARC and OCPU for any legacy CSI report, wideband or subband makes no difference. Also, except the metric difference, we see all delay/frequency/phase offset reporting are quite similar. Given the status, we propose the following.

Proposal 3-5

- For ARC for PO reporting, irrespective of wideband or subband, fully reuse that from Rel-18 TDCP reporting

- For OCPU for PO reporting, irrespective of wideband or subband, fully reuse that from Rel-18 TDCP reporting

  - OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for PO report

- Conclusion

In this contribution, we discussed CSI enhancements for up to 128 ports and non-ideal CJT deployment. Based on the discussion, we made following proposals.

Type I codebook refinement:

Proposal 2-1

- If K NZP CSI-RS resources are configured for 48, 64, or 128 ports for Rel-19 Type-I/II codebook refinement, except for Type-II doppler CSI based refinement, for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement is updated as follows.

  - The UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the K CSI-RS resources in the corresponding CSI-RS resource set for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise.

Proposal 2-2

- Support the 3-bit scaling factor configuration for RI > 1 case. We also support the Offline Proposal 1.C.1.

  - Do not support additional power boosting.

Proposal 2-3

- For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, regarding active resource counting, support Offline Proposal 1.B.

Proposal 2-4

- For SRS port grouping for UEs with 6RX or 8RX, support Offline Proposal 1.A.

  - For the FFS on mapping between CWs to CSI-RS ports, the issue is discussed for the case only when ‘cri-RI-CQI’ is configured but ‘non-PMI-PortIndication’ is not provided. Two options can be considered.

    - Option 1: ‘non-PMI-PortIndication’ is always provided.

    - Option 2: predefine another mapping between CSI-RS ports and layers.

  - For SRS antenna switching with multiple aperiodic SRS resource sets, support PSRS ports to be indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource.

Type II codebook refinement:

Proposal 2-5

- If KDOPP CSI-RS resource groups with K NZP CSI-RS resources per group are configured for 48, 64, or 128 ports for Type-II doppler CSI refinement, for the conditions of ‘after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or DRX is configured’, UE behavior on CSI-RS measurement is updated as follows.

  - The UE reports a CSI report only after receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

Multi-CRI reporting enhancement:

Proposal 2-6

- For multi-CRI reporting, on configuration of MR CSI-RS resource(s) (i.e., mrSelectedResources).

  - For Type-I based enhancement, support KS-bit bitmap indication.

  - For Type-II based enhancement, support log2KS-bit indication for MR = 1 CSI-RS resource (when KS > M = 2 is configured).

CSI reporting enhancements for non-ideal CJT deployment:

Proposal 3-1

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1) {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}:

- Define the max number of subbands to ensure reasonable UCI bit overhead, e.g., 16

Proposal 3-2

- For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’) when the DO and Rel-18 eType-II CJT CSI report are separately triggered, expiration time interval should not be specified.

  - i.e., support to confirm the WA as they are

Proposal 3-3

- Confirm the WA for time-domain behavior for SRS associated with PO report

- When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the NTRP CSI-RS occasions used for measuring ‘cjtc-P’ report.

- When aperiodic associated SRS is configured, no need of additional determination rule for SRS occasion

Proposal 3-4

- For the Rel-19 aperiodic standalone CJT calibration reporting, for DO report, FO report, and joint reporting of DO and FO, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

- For the Rel-19 aperiodic standalone CJT calibration reporting, for PO report, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

Proposal 3-5

- For ARC for PO reporting, irrespective of wideband or subband, fully reuse that from Rel-18 TDCP reporting

- For OCPU for PO reporting, irrespective of wideband or subband, fully reuse that from Rel-18 TDCP reporting

  - OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for PO report

Reference

- 3GPP RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105, Sep. 2024.

- 3GPP R1-2408636, “Moderator summary for OFFLINE discussion on Rel-19 CSI enhancements”, Moderator (Samsung), Oct 2024