3GPP TSG RAN WG1 #118-bis	R1-2408799

Hefei, China, Oct 14th – 18th, 2024

Title 	:  Discussion on channel model validation for 7-24 GHz

Source 	:  NTT DOCOMO, INC.

Agenda item	:  9.8.1

Document for:  Discussion

- Introduction

In RAN1#118 meeting, relating to validation of channel model in TR38.901 for 7-24 GHz, an agreement and a conclusion were conducted as below [1]:

Agreement The following assumptions are modeling parameters related to suburban use case. For aspects with multiple options, FFS which option(s) to support. - BS height: - Option 1: 22.5 m - Option 2: 20 m – 25 m - Option 3: 15 m - Option 4: 35 m - Option 5: 25 m - Layout: Hexagonal grid, 19 Macro sites, 3 sectors per site, - Option 1: ISD = 1732 m - Option 2: ISD = 500 m - Option 3: ISD <= 1000 m - Option 4: 500 ~ 1000 m - Option 5: 1299 m - Option 6: ISD < 500 m - Building density - Option: [373, 440, others] buildings/km2 - UT height: 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings - Typical building heights: Up to two floors for residential buildings, up to five floors for commercial buildings - UT distribution: - Option 1: Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor - Option 2: 0.5 m for outdoor; 1.5+3(-1) m for indoor, where is the floor and is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of (number of floors).) - Indoor/Outdoor: - Option 1: 80% indoor and 20% outdoor, - FFS: ratio between residential and commercial buildings - FFS on in-car users - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor - LOS/NLOS: LOS and NLOS - Min BS - UT distance (2D): - Option 1: 25 m - Option 2: 10 m - Option 3: 35 m - FFS: Penetration model: low-loss penetration model - FFS: clutter height parameter, e.g. foliage Conclusion Study at least the following channel modelling aspects of suburban use case. The sub-bullets describing the detailed equations of the modelling aspect are examples for consideration: - Pathloss - NLOS: - LOS: Reuse pathloss model of UMa scenario in TR 38.901 - LOS: - For LOS and NLOS, reuse Winner II - LOS probability - ASD Scenarios; SMa. LOS; NLOS; O2I. AOD spread (ASD) lgASD=log10(ASD/1°); µlgASD; 0.55; 0.55; 0.55. σlgASD; 0.25; 0.25; 0.25. Cluster ASD () in [deg]; 1.5; 1.5; 1.5. - ZSD Scenarios; SMa. LOS; NLOS; O2I. ZOD spread (ZSD) lgZSD=log10(ZSD/1°); µlgZSD; 0.55; 0.55; 0.55. σlgZSD; 0.25; 0.25; 0.25.

In this contribution, we discuss and provide our view on channel model validation of TR38.901 for 7-24GHz related to suburban use cases.

- Discussion

2.1 Definition of parameters for SMa scenario

In our understanding, the suburban macro (SMa) scenario is a complementary scenario between the UMa and RMa scenarios in TR38.901 [2] as shown in Table 1.

Table 1 Evaluation parameters for UMa scenario and RMa scenarios in TR38.901

| Parameters |  | UMa | RMa |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500 m) | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1732 m or 5000 m) |
| BS antenna height |  | 25 m | 35 m |
| UT location | Outdoor/indoor | Outdoor and indoor | 50% indoor and 50% in car |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Height | Same as 3D-UMa in TR36.873 | 1.5m |
| Indoor UT ratio |  | 80% | 50% |
| UT mobility (horizontal plane only) |  | 3km/h | - |
| Min. BS - UT distance (2D) |  | 35 m | 35 m |
| UT distribution (horizontal) |  | Uniform | Uniform |

In addition, the SMa scenario has some characteristics below:

- The typical building heights are lower than those of the UMa scenario but higher than those of the RMa scenario.

- The BS are located above the rooftops of buildings to allow wide area coverage and BS height varies in wide range with the minimum value smaller than the BS height of the UMa scenario and the maximum value smaller of equal to BS height of the RMa scenario.

- The ISD is larger than those of the UMa scenario but smaller or equal to those of the RMa scenario.

- The UTs for outdoor case are vehicles (in-car users) with high speed.

- The UT height is the same as definition in 3D-UMa in TR36.873 [3].

Therefore, the parameter values of SMa scenario should be in range between those of UMa and RMa scenarios. On the other hand, so far in ITU-R M.2135-1 [4], the parameter values for the SMa scenario have already been determined as shown in Table 2.

Table 2 Evaluation parameters for SMa scenario in ITU-R M.2135-1

| Parameters |  | SMa |
| --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1299 m) |
| BS antenna height |  | 35 m |
| UT location | Outdoor/indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS |
|  | Height | - |
| UT ratio |  | 50% of users indoors and 50% users outdoor (in-car) |
| UT mobility (horizontal plane only) |  | Indoor UTs: 3 km/h, outdoor UTs: 90 km/h |
| Min. BS - UT distance (2D) |  | 35 m |
| UT distribution |  | Randomly and uniformly distributed over area |

For these reasons, we first support the parameter values for SMa scenario determining in ITU-R M.2135-1. However, changing the parameter values to align with the characteristics mentioned above is also fine for us. In addition, to align with the other scenarios in TR38.901, there is no need to determine and add the building density to the SMa scenario. Moreover, the parameter of UT mobility should be added to the SMa scenario.

Proposal 1: The parameter values for SMa scenario can be used as below:

- BS height:

  - Option 4: 35 m

  - Option 5: 25 m

- Layout:　Hexagonal grid, 19 Macro sites, 3 sectors per site,

  - Option 1: ISD = 1732 m

  - Option 4: 500 ~ 1000 m

  - Option 5: 1299 m

- UT distribution:

  - Option 2: 0.5 m for outdoor; 1.5+3(-1) m for indoor, where is the floor and is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of  (number of floors).)

- Indoor/Outdoor:

  - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor (in-car users)

- Min BS - UT distance (2D):

  - Option 1: 25 m (for the case of BS height is 25 m)

  - Option 3: 35 m (for the case of BS height is 35 m)

Proposal 2: Remove the parameter of building density from SMa scenario

- Building density

  - Option: [373, 440, others] buildings/km2

Proposal 3: Add the parameter of UT mobility to SMa scenario as below:

- UT mobility (horizontal plane only): Indoor UTs: 3 km/h, outdoor UTs: 90 km/h

2.2 Path loss model for SMa scenario

In [5], it was shown that the path loss model for the LOS case in ITU-R M.2135-1 is consistent with those (the empirical model) in ITU-R P.1411-12 [6]. Moreover, the difference between the path loss model for the LOS case in the ITU-R M.2135-1 and those of Winner II [7] is small less than 2 dB. Therefore, the path loss model for the LOS case in the ITU-R M.2135-1 should be used. For the NLOS case, because the path loss is dependent on BS height, UT height, the path loss model should be included parameters of BS height, UT height. Hence, the path loss model for the NLOS case in the ITU-R M.2135-1 can be used for wide applicability ranges.

Proposal 4: The path loss model of the SMa scenario in ITU-R M.2135-1 can be used with frequency extension up to 24 GHz

2.3 LOS probability for SMa scenario

The formula for LOS probability in the conclusion above is calculated and derived from only one region (Santa Clara, CA) with low average height of buildings. In general, the LOS probability is strongly dependent on BS height, UT height and average height of buildings. Hence, for the case of the SMa scenario with a region of higher average height of buildings such as high density of commercial buildings or apartments, the LOS probability is lower. Therefore, the LOS probability in ITU-R M.2135-1 with following equation is suitable for these cases.

Proposal 5: LOS probability formular for SMa scenario in ITU-R M.2135-1 can be used.

2.4 Other parameters of channel model for SMa scenario

In our understanding, the values of ASD and ZSD in the conclusion above are derived from one measurement. We do not think that it is better to apply the same values of ASD and ZSD for all cases of LOS, NLOS, O2I. It needs to discuss and study more detail. In addition, for SMa scenario in ITU-R M.2135-1, not only ASD but other parameters such DS, ASA, K factor, small scale parameters, etc., also exist in Table A1-7. Therefore, these parameters can be used as start point.

Proposal 6: The parameters of channel model for SMa scenario in Table A1-7 of ITU-R M.2135-1 can be used as start point.

- Conclusion

In this contribution, we provided our views and discussions on channel model validation of TR38.901 for 7-24GHz related to suburban use cases. The following proposals are made:

Proposal 1: The values of parameters for SMa scenario can be used as below:

- BS height:

  - Option 4: 35 m

  - Option 5: 25 m

- Layout:　Hexagonal grid, 19 Macro sites, 3 sectors per site,

  - Option 1: ISD = 1732 m

  - Option 4: 500 ~ 1000 m

  - Option 5: 1299 m

- UT distribution:

  - Option 2: 0.5 m for outdoor; 1.5+3(-1) m for indoor, where is the floor and is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of  (number of floors).)

- Indoor/Outdoor:

  - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor (in-car users)

- Min BS - UT distance (2D):

  - Option 1: 25 m (for the case of BS height is 25 m)

  - Option 3: 35 m (for the case of BS height is 35 m)

Proposal 2: Remove the parameter of building density from SMa scenario

- Building density

  - Option: [373, 440, others] buildings/km2

Proposal 3: Add the parameter of UT mobility to SMa scenario as below:

- UT mobility (horizontal plane only): Indoor UTs: 3 km/h, outdoor UTs: 90 km/h

Proposal 4: The path loss model of the SMa scenario in ITU-R M.2135-1 can be used with frequency extension up to 24 GHz

Proposal 5: LOS probability formular for SMa scenario in ITU-R M.2135-1 can be used

Proposal 6: The parameters of channel model for SMa scenario in Table A1-7 of ITU-R M.2135-1 can be used as start point.

References

[1] R1-2407467, Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation.

[2] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

[3] TR 36.873, Study on 3D channel model for LTE.

[4] Report ITU-R M.2135-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced.

[5] R1-2406946, Discussion on channel model validation for 7-24 GHz.

[6] Recommendation ITU-R P.1411-12, Propagation data and prediction methods for the planning of short-range outdoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 100 GHz.

[7] IST-4-027756 WINNER II, D1.1.2 V1.2, WINNER II Channel Models