3GPP TSG RAN WG1 #118bis		 	R1-2408804

Hefei, CN, Oct 14th – 18th, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.3

Document for: 	Discussion/Decision

Introduction

Following objective was included in Rel-19 NR NTN WID regarding UL capacity/throughput enhancement as updated in RAN#104 meeting [1].

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, our views on DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) for UL capacity/throughput enhancement for FR1-NTN are provided.

Discussion

For Rel-19 NR NTN study, the capacity performance on uplink is to be optimized through multiplexing techniques, where DFT-s-OFDM PUSCH enhancements via OCC is mainly considered. Whether/how to support DFT-s-OFDM PUSCH enhancements via OCC for UL capacity/throughput enhancement for FR1-NTN is discussed from following aspects.

Potential OCC techniques

Following agreement was made in RAN1#118 meeting [2]:

Agreement At least one of the OCC techniques when PUSCH repetitions are used will be specified: - Inter-slot time-domain OCC with OCC length 2 - Inter-slot time-domain OCC with OCC length 2 and 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4 - Note: combination of techniques is not precluded - PUSCH repetition Type B is not considered Conclusion Multiplexing of 8 UEs with PUSCH OCC is not discussed in RAN1 until the work for multiplexing of less than 8 UEs has been completed.

For Rel-19 NTN UL capacity enhancement study, it was agreed to support at least one of inter-slot time-domain OCC with PUSCH repetition Type A and intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4). The intra-symbol pre-DFT-s OCC has benefits of better BLER performance which is almost equal to the case without OCC. And inter-slot time domain OCC has less impact on existing specifications. In our understanding, either of them can be supported for OCC for PUSCH in Rel-19 NTN, e.g., based on considering the tradeoff between performance and specification effort. Besides, it was also concluded in previous meeting that both OCC techniques can support multiplexing of 2 or 4 UEs and achieve up to 2- or 4-times capacity gains with negligible BLER performance degradation. Which OCC techniques can be specified could be further down-selected, especially considering whether inter-slot time domain OCC length 4 can be supported.

According to simulation results provided in previous meeting, it can be observed that the performance gap of multiplexing of 4 UEs compared to the case without OCC is not so large. Besides, the performance degradation can be further reduced e.g., by applying TBoMS, multiplexing UEs with similar FO values, etc. Thus, for inter-slot OCC and intra-symbol OCC, PUSCH OCC can support multiplexing of 4 UEs when repetitions are used.

Proposal 1:

At least one of the OCC techniques is specified:

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

Mapping of OCC

Following agreement was made in RAN1#116bis meeting:

Agreement RAN1 to at least further study the potential specification aspects on OCC techniques: - TBS calculation / Rate matching - UCI multiplexing - RV cycling across repetitions - Frequency hopping, e.g. intra /inter slot - OCC indication/configuration - Power control - FFS other aspects

The mapping of OCC for the potential OCC techniques should be studied considering the abovementioned aspects.

Regarding the inter-slot time-domain OCC, how to assign each bit of an OCC sequence (e.g., +1 from [+1, +j, -1, -j]) to repetitions should be studied. For example, if OCC length equals to the repetition factor, each repetition is assigned with one bit of OCC sequence; if OCC length is smaller than repetition factor, mapping of the OCC bits and repetitions should be studied, as illustrated below.

Fig1. Example of mapping of inter-slot OCC

Moreover, the impact of RV needs to be considered, i.e., the slots which mapped to one OCC sequence shall use same RV to ensure orthogonality.

Proposal 2:

For PUSCH inter-slot time-domain OCC, the inter-slot OCC is assigned to repetitions as follows:

- If OCC length = repetition factor, each repetition is assigned with one bit of OCC sequence

- If OCC length < repetition factor, study the mapping of the OCC bits and repetitions

  - E.g., for OCC length = 4 and repetition=8, the mapping of the OCC bits can be [w0 w1 w2 w3 w0 w1 w2 w3], or [w0 w0 w1 w1 w2 w2 w3 w3]

- The slots which are associated with one OCC sequence (i.e., [w0 w1 w2 w3] ) are assigned with the same RV.

For PUSCH with inter-slot time domain OCC, legacy TBS determination can be applied without enhancement. Considering UCI multiplexing, in legacy, when PUCCH is transmitted without repetition, and the PUCCH is overlapped with PUSCH, UCI in the PUCCH is multiplexed on the PUSCH only in the overlapped slot. To ensure the orthogonality, enhancements are necessary for conditions to perform UCI multiplexing for PUSCH with OCC, e.g., UCI is multiplexed on the PUSCH in the overlapped slot and spread to PUSCH with the same set of OCC code as overlapped slot, as illustrated in Fig2.

Fig2. Example of UCI multiplexing for PUSCH with inter-slot OCC of code length 2 [w0 w1]

Proposal 3:

For UCI multiplexing of PUSCH with inter-slot time-domain OCC, UCI is multiplexed on the PUSCH in an overlapped slot and on other PUSCHs that are associated with the same set of OCC code as the overlapped slot.

Regarding intra-symbol pre-DFT OCC, the OCC sequence can be assigned across subcarriers or across RBs. As code length of 2 or 4 is supported, following mapping alternatives can be considered:

Alt1. OCC assigned within a PRB, where a code of an OCC sequence (e.g., +1 or -1) is assigned for each set of subcarriers which consist of 6 or 3 subcarriers (divisor of 12). In this case, there is no need to specify the maximum number of RBs since there is no impact on OCC design. Although scheduling with more PRBs with OCC is not meaningful, decision is up to NW scheduler.

Alt2. OCC assigned across PRBs, where a code of an OCC sequence (e.g., +1 or -1) is assigned for each RB or each set of subcarriers with non-divisor of 12 (e.g., 9). For example, when 4 PRBs are scheduled with OCC length 4, each code of an OCC sequence is applied to each PRB. For this alt, the maximum number of RBs should be restricted since it has impact on OCC design in specification. Basically, OCC is used for better capacity but OCC design such as the exampled one does not bring any benefit compared to simple FDMA. 1 PRB scheduling per UE is enough rather than 4 PRBs scheduling with OCC length 4 for 4 UEs.

Alt1 is preferable for simpler specification impact, and the motivation to support Alt2 is unclear.

Proposal 4:

For PUSCH with intra-symbol pre-DFT OCC, a code of an OCC sequence (e.g., +1 or -1) is assigned within a PRB and assigned for each set of subcarriers which consists of 6 or 3 subcarriers.

For PUSCH with intra-symbol pre-DFT OCC, frequency hopping can be applied without enhancement. Meanwhile, the impact on TBS determination and UCI multiplexing should be studied, e.g., with intra-symbol OCC the number of subcarriers for PUSCH in TBS determination and UCI multiplexing can be considered as (NSCPUSCH/M) instead of NSCPUSCH, where M is OCC length. Then after mapping data/UCI for the first set of (NSCPUSCH/M) subcarriers, block-wise spreading can be applied for the remaining set(s) as in PUCCH generation for PUCCH format 4, as illustrated in Fig3.

Fig3. Example of PUSCH with intra-symbol OCC of code length 2 [w0 w1]

Proposal 5:

For PUSCH with intra-symbol pre-DFT OCC, in TBS determination and UCI multiplexing, NSCPUSCH is replaced by NSCPUSCH/M, where M is OCC length.

- Data and (if any) UCI are mapped to the first set of (NSCPUSCH/M) subcarriers, and then block-wise spreading is performed for the remaining set(s).

OCC with TBoMS

Application of TBoMS with OCC is a discussion topic. For intra-symbol pre-DFT OCC, no additional specification impact and design complexity is required, and better performance compared to repetition case can be expected in some scenarios as shown in simulation results as provided in previous meeting.

Regarding TBoMS for PUSCH with inter-slot time domain OCC, following two options as shown in Fig4 can be considered regarding the mapping of the OCC bits and TBoMS repetitions.

Fig4. Example of mapping of inter-slot OCC in case of TBoMS

Non-optimal OCC mapping (i.e., Opt 1) results in performance degradation, but appropriate OCC mapping (i.e, Opt 2) can avoid BLER performance degradation compared to repetition without TBoMS case. Link level simulation is performed, and the simulation assumptions are shown in appendix. It can be observed from the simulation result as shown below in Fig5, that if Opt 2 is applied, the inter-slot OCC with TBoMS case has equal BLER performance compared to repetition without TBoMS case. Opt 2 experiences the same impact of doppler spread and phase variation due to frequency offset as the case without TBoMS and thereby it can avoid BLER performance degradation.

Fig5. Performance of inter-slot OCC 2 in case of TBoMS and repetition without TBoMS

Gain of TBoMS compared to normal repetition will be observed that if modulation order is different between them, according to discussion in R17 coverage enhancement WI. Although low MCS is used typically regardless of whether TBoMS is applied or not, there may be cases where lower modulation order is available only when TBoMS is applied. Further discussion on such a scenario assumption will be necessary.

Observation 1:

For inter-slot OCC 2, the OCC with TBoMS case has equal BLER performance compared to repetition without TBoMS case.

Proposal 6:

Support TBoMS at least with intra-symbol OCC for PUSCH.

Proposal 7:

Discuss whether slightly high data rate to be achieved with higher modulation order if TBoMS is not applied is considered or not.

- If considered, support TBoMS with inter-slot OCC for PUSCH. FFS: how to assign OCC sequence.

Orthogonal code generation

The OCC can be generated by various of techniques, e.g., Walsh-Hadamard matrix, DFT sequence , etc., to ensure orthogonality and minimize cross-correlation between OCC codes assigned to different users. For OCC for PUSCH, the design of OCC generation for DMRS can be considered as baseline. In current specifications, Walsh matrix is applied to generate OCC for DMRS for PDSCH with benefits of simple design. For example, with OCC length of 4, the OCC generated by Walsh matrix can be each row of following matrix.

A (4) = 111-1111-1111-1-1-1-11

Meanwhile, for PUSCH DMRS, Walsh matrix is applied to OCC length of 2 and DFT sequence is applied to OCC length of 4. The reason of adopting DFT sequence is less implementation complexity, better robustness against large delay spread, etc. For example, with OCC length of 4, the OCC generated by cyclic shift can be each row of following matrix.

B (4) = 1jj-1-j-1-j1  -1-j -j   1j1j-1

In our view, both of the sequences have benefits and either one of the legacy sequence i.e., Walsh sequences and DFT sequence as defined in TS38.211 can be adopted as baseline to generate OCC for PUSCH in Rel-19 NTN.

Proposal 8:

Legacy sequences (i.e., Walsh sequences and DFT sequence as defined in TS38.211) are used as baseline for generating OCC for PUSCH.

Determination of applied OCC

Regarding the determination of applied OCC, the length of OCC (i.e., 2 or 4) can be configured/indicated by the NW. For OCC with length N, N OCC sequences can be predefined and each of them is defined with an index. Which OCC index is allocated for each PUSCH transmission also can be configured/indicated by NW. As multiple values can be supported for the OCC related parameters (e.g., OCC length, OCC index), dynamic indication has benefits of flexibility, while leads to larger overhead. Thus, whether the indication of OCC related parameters is semi-static or dynamic can be studied.

Proposal 9:

Study whether the indication of OCC related parameters, e.g., OCC length, OCC index, is performed in a semi-static manner or in a dynamic manner.

If a dynamic manner is used, DCI format 0_0 should be assumed since the discussed scenario here is low CNR region and typically DCI with the lowest DCI size will be transmitted as assumed in R19 NTN DL coverage enhancement.

Proposal 10:

If OCC related parameters are indicated in a dynamic manner, DCI format 0_0 is assumed as the container.

Conclusion

Observation 1:

For inter-slot OCC 2, the OCC with TBoMS case has equal BLER performance compared to repetition without TBoMS case.

Proposal 1:

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

Proposal 2:

For PUSCH inter-slot time-domain OCC, the inter-slot OCC is assigned to repetitions as follows:

- If OCC length = repetition factor, each repetition is assigned with one bit OCC

- If OCC length < repetition factor, study the mapping of the OCC bits and repetitions

  - E.g., for OCC length = 4 and repetition=8, the mapping of the OCC bits can be [w0 w1 w2 w3 w0 w1 w2 w3], or [w0 w0 w1 w1 w2 w2 w3 w3]

- The slots which are associated with one OCC sequence (i.e., [w0 w1 w2 w3]) are assigned with same RV.

Proposal 3:

For UCI multiplexing of PUSCH with inter-slot time-domain OCC, UCI is multiplexed on the PUSCH in the overlapped slot and spread to PUSCH with the same set of OCC code as overlapped slot.

Proposal 4:

For PUSCH with intra-symbol pre-DFT OCC, a code of an OCC sequence (e.g., +1 or -1) is assigned within a PRB and assigned for each set of subcarriers which consists of 6 or 3 subcarriers.

Proposal 5:

For PUSCH with intra-symbol pre-DFT OCC, in TBS determination and UCI multiplexing, NSCPUSCH is replaced by NSCPUSCH/M, where M is OCC length.

- Data and (if any) UCI are mapped to the first set of (NSCPUSCH/M) subcarriers, and then block-wise spreading is performed for the remaining set(s).

Proposal 6:

Support TBoMS at least with intra-symbol OCC for PUSCH.

Proposal 7:

Discuss whether slightly high data rate to be achieved with higher modulation order if TBoMS is not applied is considered or not.

- If considered, support TBoMS with inter-slot OCC for PUSCH. FFS: how to assign OCC sequence.

Proposal 8:

Legacy sequences (i.e., Walsh sequences and DFT sequence as defined in TS38.211) are used as baseline for generating OCC for PUSCH.

Proposal 9:

Study whether the indication of OCC related parameters, e.g., OCC length, OCC index, is performed in a semi-static manner or in a dynamic manner.

Proposal 10:

If OCC related parameters are indicated in a dynamic manner, DCI format 0_0 is assumed as the container.

References

[1] RP- 241659 , “ Moderator summary on Rel-19 NR NTN”, Moderator (RAN1 Vice-Chair), Jun. 2024.

[2] Draft Report of 3GPP TSG RAN WG1 #118, R1-2407208, “Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput”, Moderator (MediaTek), May 2024.

Appendix: Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Center frequency | 2 GHz |
| SCS | 15 kHz |
| Channel model | NTN-TDL-C |
| UE speed | 3 km/h |
| PRB allocation | 2 |
| PUSCH mapping type | Type A with 14 OFDM symbols |
| DM-RS configuration type | Type 1 |
| DM-RS Length | 1 |
| DM-RS symbol | [2, 11] |
| FDM b/w data and DMRS | No |
| TBS | 208 |
| MCS | w/o TBoMS: MCS 5 in MCS Table 6.1.4.1-1 in TS38.214 w/ TBoMS: MCS 2 in MCS Table 6.1.4.1-1 in TS38.214 |
| Repetition and TBoMS | w/o TBoMS: 4, 8, 16, 20 repetitions w/ TBoMS: 2,4,8,10 repetitions of 2-slot TBoMS |
| OCC length | 2 (i.e., 2 UEs multiplexing) |
| OCC sequence | Walsh sequences |
| TO | Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts |
| FO | Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. |
| Channel estimation | Real |