3GPP TSG RAN WG1 #118bis                                                                                             R1-2408805

Hefei, China, October 14th – 18th, 2024

Agenda item:	9.12.1

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and Decision

- Introduction

At the RAN#105 meeting, Rel-19 WID for Multi-Carrier Enhancements for NR Phase 2 was approved with following objective [1].

1. Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

In this contribution, we discuss on necessary enhancements for multi-cell PUSCH/PDSCH scheduling with a single DCI to support different SCS/carrier type among co-scheduled cells and multiple PUSCHs/PDSCHs per scheduled cell.

- Discussion on necessary enhancements for different SCS/carrier type among co-scheduled cells by the single DCI

2.1	Supported cases

Rel-18 multi-cell PUSCH/PDSCH scheduling with a single DCI supports following cases in terms of SCS/carrier type of scheduling cell and co-scheduled cells. All the cases have same restriction that all co-scheduled cells have same SCS and same carrier type. Thanks to the restriction, all possible cases can be covered by followings irrespective of number of cells in a set of co-scheduled cells.

- Case 1) Scheduling cell and all co-scheduled cells have same SCS and same carrier type.

  - 1a) Scheduling cell is one of co-scheduled cells.

  - 1b) Scheduling cell is not included in co-scheduled cells.

- Case 2) Scheduling cell and all co-scheduled cells have same SCS, but scheduling cell has different carrier type from that of co-scheduled cells while all co-scheduled cells have same carrier type.

  - Scheduling cell is not included in co-scheduled cells.

- Case 3) Scheduling cell has lower SCS than that of co-scheduled cells while all co-scheduled cells have same SCS and same carrier type.

  - 3a) Scheduling cell is not included in co-scheduled cells, and scheduling cell and all co-scheduled cells have same carrier type.

  - 3b) Scheduling cell is not included in co-scheduled cells, and scheduling cell has different carrier type from that of co-scheduled cells.

- Case 4) Scheduling cell has higher SCS than that of co-scheduled cells while all co-scheduled cells have same SCS and same carrier type.

  - 4a) Scheduling cell is not included in co-scheduled cells, and scheduling cell and all co-scheduled cells have same carrier type.

  - 4b) Scheduling cell is not included in co-scheduled cells, and scheduling cell has different carrier type from that of co-scheduled cells.

Supported cases in Rel-18 multi-cell PUSCH/PDSCH scheduling

| Cases | Scheduling cell |  | Co-scheduled cells |  | Whether scheduling cell is included in co-scheduled cells or not |
| --- | --- | --- | --- | --- | --- |
|  | SCS (#1<#2) | Carrier type | SCS (#1<#2) | Carrier type |  |
| 1a | #1 | #a | #1 | #a | Yes |
| 1b | #1 | #a | #1 | #a | No |
| 2 | #1 | #a | #1 | #b | No |
| 3a | #1 | #a | #2 | #a | No |
| 3b | #1 | #a | #2 | #b | No |
| 4a | #2 | #a | #1 | #a | No |
| 4b | #2 | #a | #1 | #b | No |

According to WID objective in [1], the restriction of same SCS/carrier type among co-scheduled cells can be removed, and some new cases with different SCSs and/or carrier types among co-scheduled cells can be supported in Rel-19 multi-cell PUSCH/PDSCH scheduling with a single DCI. Once the restriction of same SCS/carrier type among co-scheduled cells is removed, there can be quite many possible cases in terms of SCS/carrier type of scheduling cell and co-scheduled cells.

Assuming the cases with two cells in a set of co-scheduled cells, following many new cases are possible.

Possible cases in Rel-19 multi-cell PUSCH/PDSCH scheduling with two cells in a set of co-scheduled cells

| Cases | Scheduling cell (= co-scheduled cell #1 if it is included in co-scheduled cells) |  | Co-scheduled cell #1 |  | Co-scheduled cell #2 |  | Whether scheduling cell is included in co-scheduled cells or not |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS (#1<#2<#3) | Carrier type | SCS (#1<#2<#3) | Carrier type | SCS (#1<#2<#3) | Carrier type |  |
| 5a | #1 | #a |  |  | #1 | #b | Yes |
| 5b | #1 | #a | #1 | #a | #1 | #b | No |
| 5c | #1 | #a | #1 | #b | #1 | #c | No |
| 6a | #1 | #a |  |  | #2 | #a | Yes |
| 6b | #1 | #a |  |  | #2 | #b | Yes |
| 6c | #1 | #a | #2 | #a | #2 | #b | No |
| 6d | #1 | #a | #2 | #b | #2 | #c | No |
| 6e | #1 | #a | #1 | #a | #2 | #a | No |
| 6f | #1 | #a | #1 | #a | #2 | #b | No |
| 6g | #1 | #a | #1 | #b | #2 | #c | No |
| 6h | #1 | #a | #2 | #a | #3 | #a | No |
| 6i | #1 | #a | #2 | #a | #3 | #b | No |
| 6j | #1 | #a | #2 | #b | #3 | #c | No |
| 7a | #2 | #a |  |  | #1 | #a | Yes |
| 7b | #2 | #a |  |  | #1 | #b | Yes |
| 7c | #2 | #a | #1 | #a | #1 | #b | No |
| 7d | #2 | #a | #1 | #b | #1 | #c | No |
| 7e | #2 | #a | #2 | #a | #1 | #a | No |
| 7f | #2 | #a | #2 | #a | #1 | #b | No |
| 7g | #2 | #a | #2 | #b | #1 | #c | No |
| 7h | #3 | #a | #2 | #a | #1 | #a | No |
| 7i | #3 | #a | #2 | #a | #1 | #b | No |
| 7j | #3 | #a | #2 | #b | #1 | #c | No |

Although it will not be shown, it is obvious that there are more and more new cases if we assume three or four co-scheduled cells where co-scheduled cells can have different SCSs and/or carrier types each other.

In addition, if we consider possible combinations of SCSs (i.e., 15/30/60/120 kHz) and carrier types (i.e., FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2) among scheduling cell and each of co-scheduled cells, there are too many possible combinations especially when three or four cells are in a set of co-scheduled cells.

Observation 1: If the restriction of same SCS/carrier type among co-scheduled cells is removed, there are quite many possible cases in terms of SCS/carrier type of scheduling cell and co-scheduled cells, i.e., there are quite many possible combinations of SCSs and carrier types among scheduling cell and each of co-scheduled cells.

If all possible cases are supported and arbitrary combination of SCSs/carrier types is allowed, one potential big concern is that UE capability design would be very complex and long discussion would be required. Therefore, we think that supported cases and combinations of SCSs/carrier types among scheduling cell and each of co-scheduled cells should be carefully discussed. Actually, this WI was approved as one of additional Rel-19 WIs with limited TU/scope based on specific commercial demand from operators. In [1], specific examples such as “3.5GHz TDD + Sub-3GHz FDD, FR1 + FR2” are described. Therefore, according to the principle of additional Rel-19 WIs, only specific cases/combinations that have commercial demand from operators should be supported in Rel-19.

Observation 2: If all possible cases are supported and arbitrary combination of SCSs/carrier types is allowed, UE capability design would be very complex and long discussion would be required.

Proposal 1: According to the principle of additional Rel-19 WIs, only specific cases/combinations of SCSs/carrier types among scheduling cell and each of co-scheduled cells should be selected/supported based on commercial demand from operators.

2.2	Specification impacts

Rel-18 restriction on same SCS/carrier type among co-scheduled cells is described in TS38.300 and TS38.306 as below.

- TS38.300 10.11 [2]

## 10.11 Multi-cell scheduling by a single DCI Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions: ~ - The scheduling PDCCH and the scheduled PDSCH(s)/PUSCH(s) can use the same or different numerologies; - The co-scheduled PDSCH(s) with a PDCCH use the same numerology; - The co-scheduled PUSCH(s) with a PDCCH use the same numerology.

- TS38.306 4.2.7.4 [3]

multiCell-PDSCH-DCI-1-3-DiffSCS-r18 Indicates whether the UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group and supports Type-2 for 'Antenna port(s)' field ~ The capability signalling comprises of the following parameters: - coScheduledCellSCS-r18 indicates scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier - combinationCarrierType-r18 indicates scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). ~

multiCell-PDSCH-DCI-1-3-SameSCS-r18 Indicates whether the UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)' field. ~ The capability signalling comprises of the following parameters: - coScheduledCellSCS-r18 indicates scheduling cell and co-scheduled cells have same SCS/carrier type. ~

multiCell-PUSCH-DCI-0-3-DiffSCS-r18 Indicates whether the UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group and supports Type-2 for 'Antenna port(s)', 'Precoding information and number of layers' and 'SRS resource indicator' fields. Scheduling cell is PCell or SCell, and a set of cells includes only SCells. ~ The capability signalling comprises of the following parameters: - coScheduledCellSCS-r18 indicates scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type. - combinationCarrierType-r18 indicates scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). ~

multiCell-PUSCH-DCI-0-3-SameSCS-r18 Indicates whether the UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)', 'Precoding information and number of layers' and 'SRS resource indicator' fields. Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. ~ The capability signalling comprises of the following parameters: - coScheduledCellSCS-r18 indicates scheduling cell and co-scheduled cells have same SCS/carrier type. ~

Therefore, specification impacts to support different SCS/carrier type among co-scheduled cells include at least 1) updating TS38.300 to remove the restriction and 2) introducing new UE capability(es) for the support of different SCS/carrier type among co-scheduled cells.

Proposal 2: Specification impacts to support different SCS/carrier type among co-scheduled cells include at least followings.

- updating TS38.300 to remove the restriction

- introducing new UE capability(es) for the support of different SCS/carrier type among co-scheduled cells

We think Rel-18 RAN1 specifications on DCI format 1_3/0_3 would be applicable to the case with different SCS/carrier type among co-scheduled cells. For example, even if scheduled PUSCHs/PDSCHs between different co-scheduled cells have different SCS/carrier type, it would be possible to reuse Rel-18 mechanisms such as type 1A/1B/1C/2 field indication, HARQ feedback and so on. In Rel-18, it is possible that scheduled PUSCHs/PDSCHs between different co-scheduled cells have different start timing/duration/etc and hence Rel-18 specifications/mechanisms could also cover the case where scheduled PUSCHs/PDSCHs between different co-scheduled cells have different SCS/carrier type.

So, RAN1 discussion on necessary enhancements for different SCS/carrier type among co-scheduled cells by the single DCI would be on supported cases/combinations in terms of SCS/carrier type of scheduling cell and each of co-scheduled cells and on design of UE capability(es).

Observation 3: Rel-18 RAN1 specifications on DCI format 1_3/0_3 would be applicable to the case with different SCS/carrier type among co-scheduled cells.

- Discussion on necessary enhancements for multiple PUSCHs/PDSCHs per scheduled cell by the single DCI

3.1	TDRA indication for multiple PUSCHs/PDSCHs per scheduled cell

In Rel-18 multi-cell PUSCH/PDSCH scheduling, one PUSCH or PDSCH per scheduled cell is scheduled in DCI format 0_3 or 1_3. To realize scheduling one or multiple PUSCHs or PDSCHs per scheduled cell in DCI format 0_3 or 1_3, a straightforward way is to enhance TDRA indication in DCI format 0_3 or 1_3.

For single-cell multi-PUSCH/PDSCH scheduling via DCI format 0_1/1_1, PUSCH-TimeDomainResourceAllocationList-r16 or MultiPDSCH-TDRA-List-r17 which consists of one or multiple row entries of TDRA combinations for multiple PUSCHs or PDSCHs is configured, and TDRA field in DCI format 0_1 or 1_1 indicates an index of the row entry so that multiple PUSCHs or PDSCHs are scheduled according to the TDRA combination.

For Rel-18 multi-cell PUSCH/PDSCH scheduling via DCI format 0_3/1_3, joint TDRA table (i.e., tdra-FieldIndexListDCI-1-3-r18 or tdra-FieldIndexListDCI-0-3-r18) is configured. Each row entry (TDRA-FieldIndexDCI-1-3-r18 or TDRA-FieldIndexDCI-0-3-r18) of the joint TDRA table has combination of TDRA indexes, and each TDRA index refers TDRA table of each BWP of each cell applicable for DCI format 1_1 or 0_1 as shown below.

- TS 38.331 6.3.2 [4]

tdra-FieldIndexListDCI-1-3-r18 SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18 OPTIONAL, -- Need R tdra-FieldIndexListDCI-0-3-r18 SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18 OPTIONAL, -- Need R TDRA-FieldIndexDCI-1-3-r18 ::= SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-Allocations-1-r18) TDRA-FieldIndexDCI-0-3-r18 ::= SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18) ~ tdra-FieldIndexListDCI-1-3, tdra-FieldIndexListDCI-0-3 Configure joint TDRA table for UL scheduling via DCI format 1_3 and DCI format 0_3, respectively. TDRA-FieldIndexDCI-1-3 Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1-1, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in 38.212, of the first cell and so on ), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 1-3, as specified in 38.212, across cells included in scheduledCellListDCI-1-3.

Fig.1: TDRA indication in DCI 1_3/0_3 and 1_1/0_1 in Rel-18

According to the WID, UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. So, when UE is configured with multi-cell multi-PUSCH/PDSCH scheduling, “TDRA table applicable for DCI format 1_1” cannot be TDRA table for multi-PUSCH/PDSCH scheduling.

Based on above, we think that Rel-19 multi-cell multi-PUSCH/PDSCH scheduling can be realized by referring TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell to the joint TDRA table entries, but since TDRA table for DCI format 1_1/0_1 for each BWP of each cell should be for single PUSCH/PDSCH scheduling, separate new TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell to be referred by the joint TDRA table is necessary.

We believe that Rel-18 mechanism such as type-1B indication of TDRA based on joint TDRA table can be reused in principle, e.g., no enhancement for TDRA field in DCI format 1_3/0_3 would be necessary.

Fig.2: TDRA indication in DCI 1_3/0_3 and 1_1/0_1 in Rel-19

Proposal 3: The principle of Rel-18 mechanism such as type-1B indication of TDRA based on joint TDRA table should be reused for multi-cell multi-PUSCH/PDSCH scheduling.

Observation 4: According to WID, when UE is configured with multi-cell multi-PUSCH/PDSCH scheduling, “TDRA table applicable for DCI format 1_1” which is referred by the joint TDRA table entries (TDRA-FieldIndexDCI-1-3-r18 or TDRA-FieldIndexDCI-0-3-r18) cannot be TDRA table for multi-PUSCH/PDSCH scheduling.

Proposal 4: Separate new TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell to be referred by the joint TDRA table needs to be introduced.

- New joint TDRA tables and entries like tdra-FieldIndexListDCI-1-3-r18, tdra-FieldIndexListDCI-0-3-r18, TDRA-FieldIndexDCI-1-3-r18 and TDRA-FieldIndexDCI-0-3-r18 are necessary to refer separate new TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell instead of “TDRA table applicable for DCI format 1_1”.

- Separate new TDRA tables for multi-PUSCH/PDSCH scheduling for each BWP of each cell from PUSCH-TimeDomainResourceAllocationList-r16 and MultiPDSCH-TDRA-List-r17 are necessary to be used for multi-cell multi-PDSCH/PUSCH scheduling via DCI format 1_3/0_3 instead of “single-cell multi-PDSCH/PUSCH scheduling via DCI format 1_1/0_1”.

3.2	HARQ enhancements for multi-cell multi-PDSCH scheduling

According to WID, following two notes should be considered for HARQ enhancements for multi-cell multi-PDSCH scheduling.

- Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling.

- Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling.

Based on the second note, we can consider up to two HARQ sub-codebooks for Type-2 HARQ-ACK codebook when multi-cell multi-PDSCH scheduling is used. Based on the principle of Type-2 HARQ-ACK codebook for multi-cell PDSCH scheduling in Rel-18, two sub-codebooks are generated as below.

- First sub-codebook: HARQ-ACK information for PDSCH(s) scheduled by DCI(s) which schedule one PDSCH and for DCI without PDSCH scheduling.

- Second sub-codebook: HARQ-ACK information for PDSCHs scheduled by DCI(s) which schedule more than one PDSCH.

  - Number of HARQ-ACK bits for second sub-codebook per DCI is a sum of max. number of PDSCHs (TBs) for a cell in the set of cells.

Proposal 5: In case of type-2 HARQ-ACK codebook for multi-cell multi-PDSCH scheduling, two sub-codebooks are generated as below.

- First sub-codebook: HARQ-ACK information for PDSCH(s) scheduled by DCI(s) which schedule one PDSCH and for DCI without PDSCH scheduling.

- Second sub-codebook: HARQ-ACK information for PDSCHs scheduled by DCI(s) which schedule more than one PDSCH.

  - Number of HARQ-ACK bits for second sub-codebook per DCI is a sum of max. number of PDSCHs (TBs) for a cell in the set of cells.

Since there can be multiple PDSCHs per scheduled cell, number of HARQ-ACK bits for second sub-codebook could be large.

For single-cell multi-PDSCH scheduling, time domain HARQ bundling is supported. Since multiple PDSCHs are scheduled on the same cell, time domain HARQ bundling may be effective to reduce number of HARQ-ACK bits for multi-PDSCH scheduling. Therefore, we think that time domain HARQ bundling should be supported for multi-cell multi-PDSCH scheduling as well.

Observation 5: Since there can be multiple PDSCHs per scheduled cell, number of HARQ-ACK bits for second sub-codebook could be very large in case of multi-cell multi-PDSCH scheduling.

Proposal 6: Time domain HARQ bundling should be supported for multi-cell multi-PDSCH scheduling.

3.3	Supported cases and maximum number of PUSCHs/PDSCHs per scheduled cell

As discussed in section 2.1, there are multiple supported cases in Rel-18 multi-cell PUSCH/PDSCH scheduling in terms of SCS/carrier type of scheduling cell and co-scheduled cells, i.e., Case 1a/1b/2/3a/3b/4a/4b in section 2.1. Then, we think whether multi-cell multi-PUSCH/PDSCH scheduling is applicable to all cases or not should be discussed. As in our proposal 1, our view is that supported cases should be justified by commercial demand from operators and without big specification impacts. Therefore, we think that multi-cell multi-PUSCH/PDSCH scheduling should be applicable to only cases with FR2 scheduled cells, as such cases are described in justification description of WID.

Proposal 7: According to the principle of additional Rel-19 WIs, multi-cell multi-PUSCH/PDSCH scheduling should be applicable to only cases with FR2 scheduled cells.

Regarding maximum number of PUSCHs/PDSCHs per scheduled cell, WID describes it is “[4 or 8]” so that RAN1 should discuss and select either 4 or 8. Since single cell multi-PUSCH/PDSCH scheduling supports 8 as maximum number, 8 may be reasonable. On the other hand, maximum 8 PUSCHs/PDSCHs per scheduled cell may cause some issue, such as large number of HARQ-ACK bits per DCI for second sub-codebook as discussed in section 3.2, potential too large size of DCI format 1_3/0_3, etc. So, the maximum number should be carefully studied with supported cases, HARQ enhancements and DCI size.

Proposal 8: The maximum number of PUSCHs/PDSCHs per scheduled cell should be carefully studied with supported cases for multi-cell multi-PUSCH/PDSCH scheduling in terms of SCS/carrier type combination between scheduling cell and co-scheduled cells, HARQ enhancements such as time-domain HARQ bundling, and DCI size.

- Conclusion

In this contribution, we made following observations and proposals.

Observation 1: If the restriction of same SCS/carrier type among co-scheduled cells is removed, there are quite many possible cases in terms of SCS/carrier type of scheduling cell and co-scheduled cells, i.e., there are quite many possible combinations of SCSs and carrier types among scheduling cell and each of co-scheduled cells.

Observation 2: If all possible cases are supported and arbitrary combination of SCSs/carrier types is allowed, UE capability design would be very complex and long discussion would be required.

Proposal 1: According to the principle of additional Rel-19 WIs, only specific cases/combinations of SCSs/carrier types among scheduling cell and each of co-scheduled cells should be selected/supported based on commercial demand from operators.

Proposal 2: Specification impacts to support different SCS/carrier type among co-scheduled cells include at least followings.

- updating TS38.300 to remove the restriction

- introducing new UE capability(es) for the support of different SCS/carrier type among co-scheduled cells

Observation 3: Rel-18 RAN1 specifications on DCI format 1_3/0_3 would be applicable to the case with different SCS/carrier type among co-scheduled cells.

Proposal 3: The principle of Rel-18 mechanism such as type-1B indication of TDRA based on joint TDRA table should be reused for multi-cell multi-PUSCH/PDSCH scheduling.

Observation 4: According to WID, when UE is configured with multi-cell multi-PUSCH/PDSCH scheduling, “TDRA table applicable for DCI format 1_1” which is referred by the joint TDRA table entries (TDRA-FieldIndexDCI-1-3-r18 or TDRA-FieldIndexDCI-0-3-r18) cannot be TDRA table for multi-PUSCH/PDSCH scheduling.

Proposal 4: Separate new TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell to be referred by the joint TDRA table needs to be introduced.

- New joint TDRA tables and entries like tdra-FieldIndexListDCI-1-3-r18, tdra-FieldIndexListDCI-0-3-r18, TDRA-FieldIndexDCI-1-3-r18 and TDRA-FieldIndexDCI-0-3-r18 are necessary to refer separate new TDRA table for multi-PUSCH/PDSCH scheduling for each BWP of each cell instead of “TDRA table applicable for DCI format 1_1”.

- Separate new TDRA tables for multi-PUSCH/PDSCH scheduling for each BWP of each cell from PUSCH-TimeDomainResourceAllocationList-r16 and MultiPDSCH-TDRA-List-r17 are necessary to be used for multi-cell multi-PDSCH/PUSCH scheduling via DCI format 1_3/0_3 instead of “single-cell multi-PDSCH/PUSCH scheduling via DCI format 1_1/0_1”.

Proposal 5: In case of type-2 HARQ-ACK codebook for multi-cell multi-PDSCH scheduling, two sub-codebooks are generated as below.

- First sub-codebook: HARQ-ACK information for PDSCH(s) scheduled by DCI(s) which schedule one PDSCH and for DCI without PDSCH scheduling.

- Second sub-codebook: HARQ-ACK information for PDSCHs scheduled by DCI(s) which schedule more than one PDSCH.

  - Number of HARQ-ACK bis for second sub-codebook per DCI is a sum of max. number of PDSCHs (TBs) for a cell in the set of cells.

Observation 5: Since there can be multiple PDSCHs per scheduled cell, number of HARQ-ACK bits for second sub-codebook could be very large in case of multi-cell multi-PDSCH scheduling.

Proposal 6: Time domain HARQ bundling should be supported for multi-cell multi-PDSCH scheduling.

Proposal 7: According to the principle of additional Rel-19 WIs, multi-cell multi-PUSCH/PDSCH scheduling should be applicable to only cases with FR2 scheduled cells.

Proposal 8: The maximum number of PUSCHs/PDSCHs per scheduled cell should be carefully studied with supported cases for multi-cell multi-PUSCH/PDSCH scheduling in terms of SCS/carrier type combination between scheduling cell and co-scheduled cells, HARQ enhancements such as time-domain HARQ bundling, and DCI size.

References

[1]	RP-242408	New WID: Multi-carrier enhancements for NR Phase 2	Lenovo

[2]	TS38.300 v18.3.0

[3]	TS38.306 v18.3.0

[4]	TS38.331 v18.3.0