3GPP TSG RAN WG1 Meeting #118bis			    	R1-2408810

Hefei, China, 14th – 18th October, 2024

Source 	: CAICT

Title 	: Considerations on ISAC channel modelling

Agenda Item	: 9.7.2

Document for	: Discussion and decision

Introductions

In the meeting for the study on ISAC channel modelling in RAN1#118, the following agreements have been reached:

Agreement If RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point. Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points Agreement For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel, - An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117 - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system - stochastically generated for a NLOS ray using section 7, 38.901 as starting point - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target - FFS The mobility of stochastic clutter - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS - FFS initial phase - FFS how to model effect of RCS at target - FFS whether/how to model polarization at target - FFS How to reduce complexity - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points Working assumption The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B - A first RCS component A (m2) is included in large scale - FFS the first RCS component is deterministic or stochastic - FFS The first component is dependent on incident and scattered directions at target - A second RCS component B (unit ratio) is included in small scale - FFS The second component is dependent on incident and scattered directions at target - FFS the second RCS component is deterministic or stochastic or combination - Note: RCS component A or B can be disabled by setting its linear value to 1 - Whether to disable a component can be discussed per target type - FFS how to determine A and B for each target - FFS whether/how to model polarization impact at target - FFS whether/how to normalize power accounting for target channel and background channel Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp - FFS whether CPMsp is angular/ray-dependent or independent. - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering. - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx. Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point. - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results. FFS whether/how to do power normalization between target channel and background channel. Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario. - FFS EO is modelled in the background channel. - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures. - FFS EO is modelled in the background channel. - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded. Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

In this paper, we will present our views on ISAC channel modelling. Observations and proposals are put forward accordingly.

Discussions

Sensing Target Modelling

It is reached an agreement that scattering of a sensing target can be modeled as single scattering point or multiple scattering points and the sensing target can be assumed in far field of sensing Tx/Rx. According to the scope of ISAC channel modelling described in the SID [1], there are five kinds of sensing targets:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

There are many factors that may influence the selection of single or multiple scattering points for the target, e.g. depending on size/shape/materials of target, the distance between target and sensing Tx/Rx, sensing resolution, etc. If the shape and material of the target are not identical to the isotropic direction and the size of the target exceeds distance resolution or angle resolution, multiple scattering points can model the target more accurately.

The theoretical formula of sensing distance resolution and angle resolution is presented in [2]. According to the typical configuration for Sub 6GHz and mmWave, values of sensing distance resolution and angle resolution are listed in the table. Considering the typical size of targets and proper frequency bands for different scenario.

Table.1 Sensing resolution analysis

| Sensing resolution | Formula | Sub 6GHz | mmWave |
| --- | --- | --- | --- |
| Distance resolution (Mono-static) |  | 1.5m (B=100MHz) | 0.375m (B=400MHz) |
| Distance resolution (Bi-static) |  | >1.5m (B=100MHz) | >0.375m (B=400MHz) |
| Angle resolution |  | 0.2215 rad (D=8*0.5λ) | 0.111 rad (D=16*0.5λ) |

Proposal1: Model sensing target as single point or multiple point as follows:

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

Environment Object Modelling

Modelling of environment object is a hot topic in last two RAN1 meetings and we have some concerns about the first three options to share. If EO channel is modeled, it may have overlap with stochastic clutters modeled by 38.901 for background, especially for EO type-2 having extremely large size. Some strategies should be proposed to reduce the impact, which can include power distribution and cluster number adjustment for target channel, EO channel and background channel. For Option 3, an additional process is brought in to get EO mapped from clusters in 38.901. If stochastic channel is generated for EO after getting EO, we think the gain for EO modelling is very limited.

Proposal2: Strategies should be studied to resolve the overlap between EO channel and background channel, such as, power distribution and cluster number adjustment.

ISAC channel modelling of slow fading

Simulation results based on ray tracing is provided to analyze large scale fading features of ISAC channel. The scenario is shown in Fig.1, where mono-static BS at 7GHz is deployed on top of the building in a campus. The upper parts are buildings and the lower parts is a street with a crosswalk. UAVs are distributed at different heights and distances. The links between BS and UAVs are LOS. Signals between UAVs are ignored.

Fig.1 Simulation scenario in campus

The trend that large scale fading varies with the height of UAV and the distance between UAV and BS is shown in Fig.2. We can observe that the distance factor has no effect on the background channel. Large scale fading of target channel increases with UAV height and distance between UAV and BS.

Fig.2 Large scale fading of background channel and target channel

2.3.1 Pathloss of target channel

The pathloss of the target channel is a combination of the pathloss from the sensing signal transmitter to the sensing target, the reflecting/scattering loss at the sensing target which usually is presented by RCS, and the pathloss from the sensing target to the sensing signal receiver. Therefore, we can formulate the pathloss of target channel as

PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS,

where PLd1 and PLd2 denote the pathloss from the sensing signal transmitter to the sensing target and the pathloss from the sensing target to the sensing signal receiver, respectively. λ is the wavelength of the sensing signal, and σRCS is the RCS of the sensing target. The pathloss PLd1 and PLd2 in different sensing scenarios like UMA-AVs, InH, Indoor-office, Indoor-factory, and highway, can use the corresponding pathloss formulations in TR 38.901, TR 37.885 and TR 36.777 as references. While when to apply current pathloss formulations in the literature, there are some constraints about the height of sensing target. Therefore, further modification and validation of the pathloss between sensing signal transmitter and sensing target, and the pathloss between sensing target and sensing signal receiver is necessary considering the sensing target height requirement.

Proposal3: It is suggested to adopt the formula PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS as a baseline to model the sensing pathloss.

Proposal4: When to apply the current pathloss formulations in the literature like TR 38.901, TR 37.885 and TR 36.777 for sensing pathloss, FFS the case when the sensing target height is not fulfilled the application constraints.

2.3.2 LOS probability of target channel

The LOS/NLOS propagation situation of ISAC channel should be modeled for target-related components and background components, respectively.

In Mono-static sensing mode, due to channel reciprocity, the LOS/NLOS states of Tx-target link and target-Rx link are the same for target-related components and there is no need to define LOS/NLOS state for background components.

In Bi-static sensing mode, there is no correlation between the LOS/NLOS states of Tx-target link and target-Rx link and they can be set separately. Therefore, states of “Tx-target link + target-Rx link” can be LOS + LOS, LOS + NLOS, NLOS + LOS, NLOS + NLOS for target-related components. Moreover, LOS/NLOS state of Tx-Rx link also needs to be defined for background components.

Therefore, one kind of LOS probability should be specified for ISAC channel of Mono-static sensing mode:

- LOS probability of Tx/Rx-target link ( LOS/NLOS states of Tx-target and target-Rx are same)

Two kinds of LOS probability should be designed for ISAC channel of Bi-static sensing mode:

- LOS probability of Tx/Rx-target link ( LOS/NLOS states of Tx-target and target-Rx are different)

- LOS probability of Tx-Rx link

Proposal5: LOS probability of Tx/Rx-target link and Tx-Rx link should be defined for each sensing scenarios like UMA-AVs, InH, Indoor-office, Indoor-factory, and highway. Existing LOS probability schemes in TR 38.901. TR 36.777, TR 37.885 can be used as references with further modification and validation.

2.3.3 Large scale fading of background channel

Conventional large scale fading model consists of pathloss, shadow fading and penetration loss. These concepts still hold for target channel and background channel in bi-static. But whether pathloss still holds for background channel in mono-static should be reconsider. Because Tx and Rx are too close to apply pathloss. Three options are agreed to study how to model the background channel for TRP and UE mono-static sensing mode in last meeting:

Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario.

Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literature.

Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

A large scale fading model based on ray-tracing simulation results is proposed in this Tdoc. The simulation scenario is shown in Fig.3. It is a campus similar to dense urban scenario. 200 mono-static TRPs are uniformed distributed in the campus with the height of 30m. Three sets of results in 7GHz, 26GHz and 100GHz are analyzed in Fig.4.

Fig.3 Ray-tracing simulation scenario for TRP mono-static

Fig.4 Large scale fading of background channel for TRP mono-static

Large scale fading of background channel for TRP mono-static can be model as a normal distribution N(μ,σ2). For 7GHz, large scale fading satisfies N(94.63dB,5.632). For 26GHz, large scale fading satisfies N(106.46dB,5.572). For 100GHz, large scale fading satisfies N(118.36dB,5.62). It can be observed that the mean value arises with frequency and variance keeps constant.

Proposal6: Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ.

ISAC channel modelling of fast fading

2.4.1 Target channel

Fast fading of target channel is modeled to describe the small scale characteristics of Tx-target link and target-Rx link for sensing scenario. There are 6 cases should be discussed.

- LOS Tx-target link and LOS target-Rx link with Mono-static sensing mode

- NLOS Tx-target link and NLOS target-Rx link with Mono-static sensing mode

- LOS Tx-target link and LOS target-Rx link with Bi-static sensing mode

- LOS Tx-target link and NLOS target-Rx link with Bi-static sensing mode

- NLOS Tx-target link and LOS target-Rx link with Bi-static sensing mode

- NLOS Tx-target link and NLOS target-Rx link with Bi-static sensing mode

In Mono-static sensing mode, there are only 2 cases with “LOS+LOS” and “NLOS+NLOS” for Tx-target link and target-Rx link, respectively. For LOS+LOS case, Type 1/2/3/4 propagation paths described in section 2.1 exist. For complexity reducing, we suggest the priority Type 1>2=3>4. For NLOS+NLOS case, only Type 4 exist. Due to channel reciprocity, we can consider that small scale characteristics of Tx-target link and target-Rx link are the same. Therefore, after generating Tx-target link’s small scale parameters, target-Rx link’s small scale parameters can also be derived.

In Bi-static sensing mode, there are 4 cases, that are “LOS+LOS”, “LOS+NLOS”, “NLOS+LOS”, and “NLOS+NLOS” for Tx-target link and target-Rx link, respectively. For LOS+LOS case, Type 1/2/3/4 exist. To reduce the complexity, we suggest the priority Type 1>2=3>4. For LOS+NLOS case, Type 2/4 exist. For NLOS+LOS case, Type 3/4 exist. For NLOS+NLOS case, only Type 4 exists. Small scale parameters of Tx-target link and target-Rx link should be generated separately.

The number of Tx-target link and target-Rx link clusters NTx-target, Ntarget-Rx can set to control the type of propagation paths of target channel as shown in Table.2. Other small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, can take steps 4-10 in TR 38.901 chapter 7.1 as reference. Specific formulas and numerical values need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Table.2 Propagation type of different link state

| Link State | NTx-target | Ntarget-Rx | Propagation Type | Sensing mode |
| --- | --- | --- | --- | --- |
| LOS+LOS | 1 | 1 | Type 1 | Mono-static/Bi-static |
|  | >1 | >1 | Type 1/2/3/4 | Mono-static/Bi-static |
|  | 1 | >1 | Type 1/2 | Bi-static |
|  | >1 | 1 | Type 1/3 | Bi-static |
| LOS+NLOS | 1 | ≥1 | Type 2 | Bi-static |
|  | >1 | ≥1 | Type 2/4 | Bi-static |
| NLOS+LOS | ≥1 | 1 | Type 3 | Bi-static |
|  | ≥1 | >1 | Type 3/4 | Bi-static |
| NLOS+NLOS | ≥1 | ≥1 | Type 4 | Mono-static/Bi-static |

Proposal7: We suggest to define the cluster number of Tx-target link and target-Rx link. The specific values require further study according to scenario and target features. One LOS ray and more than one NLOS cluster can be the starting point.

Proposal8: Small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Some simulation related to target channel is conducted. The scenario and configurations are consistent with Fig.1. We made some analyses and comparisons of the power ratio and loss of the LOS path and NLOS path in target channel, as shown in Table.3. From the results, we can see that as the height of the UAV increases, especially when the UAV is higher than the surrounding environmental objects(from No.3 to No.9), the power of the LOS path occupies almost the entire proportion of the received power signal because of empty surroundings. Therefore, K factor for target channel should be designed with consideration of the height of target and environments. And, the overall trend has certain fluctuations. It may be caused by different RCS. With the change of UAV height, different incident angle to UAV and the departure angle from UAV will change the RCS. No.1 and from No.10 to No.19 are UAVs at the same height with different distance to BS. The loss of LOS path increases monotonically as distance arises. However the loss of NLOS path has certain fluctuations that may be caused by environment.

Table.3 power ratio and loss of the LOS path and NLOS path in target channel

| No. | UAV height(m) | Horizontal distance between BS and UAV(m) | Pow ratio of LOS path(%) | Loss of LOS path(dB) | Power ratio of NLOS path(%) | Loss of NLOS path(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 40 | 45.79 | 123.78 | 54.21 | 123.04 |
| 2 | 25 | 40 | 32.35 | 124.59 | 67.65 | 121.39 |
| 3 | 30 | 40 | 97.2 | 124.69 | 2.8 | 140.08 |
| 4 | 35 | 40 | 99.96 | 112.33 | 0.04 | 145.94 |
| 5 | 40 | 40 | 97.58 | 117.97 | 2.42 | 134.03 |
| 6 | 45 | 40 | 97.36 | 125.69 | 2.64 | 141.37 |
| 7 | 50 | 40 | 96.93 | 129.46 | 3.07 | 144.44 |
| 8 | 55 | 40 | 98.71 | 126.25 | 1.29 | 145.11 |
| 9 | 60 | 40 | 98.93 | 125.09 | 1.07 | 144.73 |
| 10 | 20 | 45 | 20.88 | 126.92 | 79.12 | 121.14 |
| 11 | 20 | 50 | 41.27 | 129.30 | 58.73 | 127.77 |
| 12 | 20 | 55 | 8.65 | 131.21 | 91.35 | 120.97 |
| 13 | 20 | 60 | 12.59 | 132.78 | 87.41 | 124.36 |
| 14 | 20 | 65 | 13.58 | 134.11 | 86.42 | 126.07 |
| 15 | 20 | 70 | 15.01 | 135.06 | 84.99 | 127.75 |
| 16 | 20 | 75 | 7.65 | 136.25 | 92.35 | 125.44 |
| 17 | 20 | 80 | 8.46 | 137.06 | 91.54 | 126.71 |
| 18 | 20 | 85 | 8.96 | 137.89 | 91.04 | 127.82 |
| 19 | 20 | 90 | 30.25 | 138.46 | 69.75 | 134.83 |

Proposal9: K factor for target channel should be designed in two categories.

- Category 1 is for UAV above background environments, which define LOS power ratio larger than 95%.

- Category 2 is for UAV below background environments and other targets, which is similar to K factor in TR 38.901.

After setting small scale parameters of Tx-target link and target-Rx link, concatenation of the two links should be executed for channel coefficient generation. Methods for channel concatenation are summarized with the following options[4]:

- Option 1: Couple the LOS ray of Tx-target link and the LOS ray of target-Rx link, 1-by-1 randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link, and generate channel coefficients for Tx-target-Rx link based on the concatenated small scale parameters of Tx-target link and target-Rx link.

- Option 2: Fully couple the LOS ray and all the NLOS clusters of Tx-target link to the LOS ray and all the NLOS clusters of target-Rx link, and generate channel coefficient for Tx-target-Rx link based on the concatenated small scale parameters of Tx-target link and target-Rx link.

- Option 3: Generate channel coefficients for the Tx-target link and target-Rx link respectively, and generate channel coefficients for Tx-target-Rx link by convoluting the channel coefficients of the Tx-target link and target-Rx link.

- Option 4: Consider only LOS ray(s) in both Tx-target link and target-Rx link, and generate channel coefficient for Tx-target-Rx link based on the concatenated small scale parameters of Tx-target link and target-Rx link.

Considering the accuracy and the complexity, Option 1 is a compromised choice. While the scenario of  NTx-target≠Ntarget-Rx should be refined. An applicable solution is to couple the redundant clusters with the NLOS clusters of the other link randomly. This implies that it would be not a strict 1-by-1 randomly coupling. As a result, the number of NLOS clusters is max(NTx-target, Ntarget-Rx).

Proposal10:

- For LOS+LOS case, couple the LOS ray of Tx-target link and the LOS ray of target-Rx link, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

- For NLOS+NLOS case, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

- For LOS+NLOS or NLOS+LOS case, couple the LOS ray with one of the strongest rays in the other link, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

- Generate channel coefficient for Tx-target-Rx link based on the concatenated small scale parameters of Tx-target link and target-Rx link.

2.4.2 Background channel

Background components include interactions with the environment targets except sensing target on the sensing signal. For Bi-static sensing mode, we can reuse the stochastic model of TR 38.901 to generate its background components, with modification of the fading channel parameters for sensing purpose. For Mono-static sensing mode, two options can be considered for its background components generation:

- Option 1: Generate the environment targets first and then model the channels between sensing signal transceiver and environment targets by following the channel modelling procedure in TR 38.901 as background components.

- Option 2: reuse the stochastic model of TR 38.901 by adding channel parameters related to Mono-static sensing mode.

Proposal11: The stochastic model of TR 38.901 can be reused to generate its background components, with modification of the fading channel parameters for sensing purpose.

Conclusions

In this contribution, we have presented our considerations on the ISAC channel modelling of slow fading and fast fading, and put forward the following observations and proposals:

Proposal1: Model sensing target as single point or multiple point as follows:

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

Proposal2: Strategies should be studied to resolve the overlap between EO channel and background channel, such as, power distribution and cluster number adjustment.

Proposal3: It is suggested to adopt the formula PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS as a baseline to model the sensing pathloss.

Proposal4: When to apply the current pathloss formulations in the literature like TR 38.901, TR 37.885 and TR 36.777 for sensing pathloss, FFS the case when the sensing target height is not fulfilled the application constraints.

Proposal5: LOS probability of Tx/Rx-target link and Tx-Rx link should be defined for each sensing scenarios like UMA-AVs, InH, Indoor-office, Indoor-factory, and highway. Existing LOS probability schemes in TR 38.901. TR 36.777, TR 37.885 can be used as references with further modification and validation.

Proposal6: Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ.

Proposal7: We suggest to define the cluster number of Tx-target link and target-Rx link. The specific values require further study according to scenario and target features. One LOS ray and more than one NLOS cluster can be the starting point.

Proposal8: Small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Proposal9: K factor for target channel should be designed in two categories.

- Category 1 is for UAV above background environments, which define LOS power ratio larger than 95%.

- Category 2 is for UAV below background environments and other targets, which is similar to K factor in TR 38.901.

Proposal10:

- For LOS+LOS case, couple the LOS ray of Tx-target link and the LOS ray of target-Rx link, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

For NLOS+NLOS case, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

For LOS+NLOS or NLOS+LOS case, couple the LOS ray with one of the strongest rays in the other link, 1-by-1 or 1-by-mutiple randomly couple the NLOS clusters of Tx-target link and the NLOS clusters of target-Rx link according to relationship of NTx-target and  Ntarget-Rx.

Generate channel coefficient for Tx-target-Rx link based on the concatenated small scale parameters of Tx-target link and target-Rx link.

Proposal11: The stochastic model of TR 38.901 can be reused to generate its background components, with modification of the fading channel parameters for sensing purpose.

References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Nokia, Nokia Shanghai Bell, Dec. 2023.

- IMT-2020(5G) Promotion Group, “Study on 5G-Advanced ISAC Technologies in RAN”, April 2024.

- IMT-2020(5G) Promotion Group, “Study on Evaluation Methodology of 5G-Advanced Integrated Sensing and Communication”, June 2023.

- R1-2400573, Discussion on ISAC channel model, Xiaomi, BUPT, February 26th – March 1st, 2024.

- R1-2400899, Considerations on ISAC channel modelling, 3GPP RAN1#116, CAICT, Feb. 2024.

- R1-2403160, Considerations on ISAC channel modelling, 3GPP RAN1#116b, CAICT, April 2024.

- R1-2404724, Considerations on ISAC channel modelling, 3GPP RAN1#116b, CAICT, May 2024.

- R1- 2406897, Considerations on ISAC channel modelling, 3GPP RAN1#118, CAICT, August 2024.