3GPP TSG RAN WG1 #118-bis			R1-2408899

Hefei, China, October 14th – 18th, 2024

Source:	WILUS Inc.

Title:	Discussion on SBFD random access operation

Agenda item:	9.3.2

Document for:	Discussion/Decision

Introduction

From the RAN1#116 to RAN1#118 meeting, the followings were agreed under the agenda item of SBFD random access operation [1]-[4].

Agreement at RAN1#116 For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH) Agreement at RAN1#116 For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols. Agreement at RAN1#116 For SBFD-aware UEs in RRC CONNECTED state, further study the following two options: - Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols - a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid - Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots RAN1 to leverage the study in Rel-18 as baseline. Agreement at RAN1#116 For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1. Agreement at RAN1#116-bis Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement at RAN1#116-bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols. - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon). - FFS the details - FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s). Agreement at RAN1#116-bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option. Agreement at RAN1#116-bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211). Agreement at RAN1#116-bis For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs. Agreement at RAN1#116-bis For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used Agreement at RAN1#116-bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any). - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs) - the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB - FFS: Other condition. Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs. Agreement at RAN1#117 A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB. Agreement at RAN1#117 Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. Agreement at RAN1#117 Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Conclusion at RAN1#117 For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs Agreement at RAN1#117 For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example, - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot Agreement at RAN1#117 Update the following agreement made in RAN1#116-bis meeting: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt the following: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, consider from the following alternatives: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211) Agreement at RAN1#118 Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. Agreement at RAN1#118 For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration. Agreement at RAN1#118 Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. - The RO includes at least one DL symbol configured by tdd-UL-DL-ConfigurationCommon Agreement at RAN1#118 For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size. - Note: Other alternatives are not precluded Agreement at RAN1#118 For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation. - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Note: The Ngap here is the same as the Ngap in the current specification Agreement at RAN1#118 For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs. - FFS the association period and association pattern period. Agreement at RAN1#118 For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases. Agreement at RAN1#118 For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Agreement at RAN1#118 For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement at RAN1#118 For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement at RAN1#118 For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement at RAN1#118 For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs). Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

In this contribution, we discuss SBFD operation of UEs in RRC_IDLE/INACTIVE mode and in RRC_CONNECTED mode for random access and provide our views.

Discussion on SBFD random access operation

PRACH configuration, RO validation, and SSB-RO mapping

Configuration of RACH occasions for SBFD operation

Regarding configuration of RACH occasions for SBFD operation, regardless of whether it is a legacy UE or a SBFD aware UE, UL symbols and UL slots should have the common RO configuration and SSB to RO mapping method, based on the legacy RACH configuration. A method of setting the RO within the UL subband in the SBFD symbol and reinterpreting the SSB to RO mapping method related with ssb-perRACH-OccasionAndCB-PreamblesPerSSB parameter within the BW occupied by the RO may be considered when one single legacy RACH configuration is used. However, when two separate RACH configurations including a legacy configuration and an additional configuration are used, the additional RACH configuration for setting both the RO configuration for the SBFD aware UE and the SSB to RO mapping may be considered independently from that of the legacy UE.

On the one hand, considering a situation where Rel-19 SBFD aware UEs and legacy UEs (or non-SBFD aware UEs) would coexist on the same network, it might be desirable for legacy UEs to perform legacy operations according to the legacy RO configuration and SSB to RO mapping method in backward compatible manner. And it seems beneficial for Rel-19 SBFD aware UEs to reinterpret the legacy RO configuration (e.g. msg1-FDM, msg1-FrequencyStart and ssb-perRACH-OccasionAndCB-PreamblesPerSSB) and SSB to RO mapping method. For legacy UEs, frequency domain location and resource of PRACH occasions for PRACH preamble transmission is determined by the RRC parameter msg1-FDM and msg1-FrequencyStart while time domain location and resource for PRACH preamble is determined by the RRC parameter prach-ConfigurationIndex. Especially for the msg1-FrequencyStart, the range can be configured within initial UL BWP for legacy UEs. However, for SBFD aware UEs, starting of RO should be limited within UL subband in frequency domain for SBFD operation in SBFD symbols configured as flexible or downlink by tdd-UL-DL-ConfigurationCommon. And when one single legacy RACH configuration is used or two separate RACH configurations are used for SBFD aware UEs, the starting symbol for PRACH slot should be also reinterpreted as the starting symbol of semi-statically configured UL subband in time domain as shown in Figure 1.

Figure 1. An example configuration of PRACH occasions for SBFD operations within UL subband

For RACH configuration Option 1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), it is suggested to consider implicitly different RO resource setting from the legacy UE’s RO resource setting by reinterpreting at least msg1-FrequencyStart to configure the RO within usable PRBs in the UL subband. And it can be further considered to use additional parameters such as semi-statically configured frequency location of UL subband, UL BWP size, and UL subband size. At the same time, it is suggested that a reinterpretation of the msg1-FDM parameter should be considered to allow RO allocation within the UL subband for SBFD aware UEs, as well as an additional reinterpretation of the ssb-perRACH-OccasionAndCB-PreamblesPerSSB parameter for the mapping of valid RO and SSB index.

At the RAN1#118 meeting, the following alternatives were discussed for determination of lowest RO of additional-ROs in SBFD symbols for RACH configuration Option 1 with Alt 1-1.

Agreement at RAN1#118 For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size. - Note: Other alternatives are not precluded

We think that the re-interpretation of msg1-FrequencyStart such as Alt 2-x except Alt-1 is necessary to configure the ROs within the UL usable PRBs if the RO configured by legacy msg1-FrequencyStart is outside of UL subband. And when a large value is configured for legacy msg1-FrequencyStart by the network, methods such as Alt 2-3 and Alt 2-4 might be further considered for configuring ROs within the UL usable PRBs.

- Proposal 1: For RACH configuration Option 1, we propose to support Alt 2-x except Alt 1 for determination of lowest RO of additional-ROs in SBFD symbols by reinterpreting at least msg1-FrequencyStart to configure the RO within usable PRBs in the UL subband.

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), it was FFS at the RAN1#116-bis meeting whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration. Taking this factor into account, it may be advisable to include parameters such as msg1-FrequencyStart, msg1-FDM, and ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which are deemed to require reinterpretation at least in Option 1, in an additional RACH configuration. Additionally, the prach-ConfigurationIndex, which is used for setting RO resources in the time domain, should also be included.

- Proposal 2: For RACH configuration Option 2, it may be advisable to include parameters such as msg1-FrequencyStart, msg1-FDM, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which are deemed to require reinterpretation in Option 1, and prach-ConfigurationIndex for setting RO resources on the time domain in an additional RACH configuration.

Mapping for SS/PBCH block index to valid PRACH occasions

For the random-access procedure, UE selects a RACH occasion (RO) among validated ROs from the set of configured ROs to perform PRACH transmission. The UE selects a valid RO based on the selected or indicated SSB and based on the association of the SSB with the valid RO. By detecting PRACH preamble with (pre-)defined Rx beam on resource which the UE sends PRACH, NW can figure out which SS/PBCH block(s) the UE has received well among SS/PBCH blocks transmitted by a gNB through beam sweeping. The mapping between SSB index and PRACH occasions is defined by the following two RRC parameters such as msg1-FDM, ssb-perRACH-OccasionAndCB-PreamblesPerSSB. The msg1-FDM specifies how many ROs are allocated in frequency domain (at the same location in time domain). The ssb-perRACH-OccasionAndCB-PreamblesPerSSB specifies how many SSBs can be mapped to one ROs and how many PRACH preamble indexes can be mapped to a single SSB. To perform random access in SBFD symbols for UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode, it should be discussed how to map SS/PBCH block indexes to valid RACH occasions within UL subband for Rel-19 SBFD-aware UEs while considering different BW size between BW for ROs within UL subband for SBFD-aware UEs (BW_new) and BW for ROs for legacy UE (BW_legacy) in terms of BW for PRACH preamble transmission or different BW size between UL subband and UL BWP.  Figure 2 is an example of SSB index to RO mapping, based on legacy RACH configuration for the same configuration in UL slot or UL symbols with coexistence with legacy UEs.

Figure 2. An example mapping of SS/PBCH block indexes to valid PRACH occasions within UL subband.

- Proposal 3: For RACH configuration Option 1, we propose to support legacy SSB to RO mapping rule for validated ROs within UL subband for Rel-19 SBFD aware UEs considering BW size difference between RO’s BW size of UL subband and legacy RO’s BW size for non-SBFD aware UEs.

At the RAN1#116-bis meeting, it was discussed for SSB index to RO mapping in cases of RACH configuration Option 1 and Option 2 respectively. And it was agreed for RACH configuration Option 1 as the following.

Agreement at RAN1#116-bis For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

However, it still needs to discuss whether/how to handle SSB index to RO mapping for new ROs (i.e., non- legacy ROs) in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon even in RACH configuration Option 1 as shown in Figure 2. Considering consistent SSB index to RO mapping with SBFD aware UEs and legacy UEs, regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon for new validated ROs (i.e., non-legacy ROs) in SBFD symbols by RACH configuration Option 1, we propose to have separate SSB index to RO mapping for new validated ROs within UL subband in SBFD symbols for SBFD aware UEs.

- Proposal 4: For RACH configuration Option 1, we propose to have separate SSB index to RO mapping for new validated ROs in SBFD symbols for SBFD aware UEs, regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon.

At the previous RAN1#118 meeting, for the RACH configuration Option 2, the following was captured as working assumption for SSB-RO mapping.

Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

However, it should be still concluded whether/how to handle SSB index to RO mapping for new validated ROs (i.e., not legacy ROs) in SBFD symbols configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon and whether/how to handle the case where the legacy ROs overlap with additional ROs. Similarly to SSB index to RO mapping in RACH configuration Option 1, it might be desirable for SBFD aware UEs to have a separate and independent SSB index to RO mapping in SBFD symbols from the SSB index to RO mapping for legacy ROs regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon. This allows a gNB to avoid using different Rx beams for the reception of the PRACH preamble from SBFD-aware UEs and non SBFD-aware UEs in the same symbols. However, if the SBFD-aware UE is allowed to be configured ROs across SBFD symbol and non-SBFD symbols either in the same slot or across slots, it should be considered to map the same SSB index to validated ROs across SBFD symbols and non-SBFD symbols either in the same slot or across slots.

- Proposal 5: For RACH configuration Option 2,

  - we propose to have a separate/independent SSB index to RO mapping for new validated ROs in SBFD symbols from the SSB index to RO mapping for legacy ROs regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon.

  - for validated ROs across SBFD symbols and non-SBFD symbols either in the same slot or across slots, we propose to map the same SSB index to validated ROs across SBFD symbols and non-SBFD symbols either in the same slot or across slots.

In addition, in the case that overlapping between the legacy ROs of the legacy PRACH configuration and the additional-ROs configured by the additional RACH configuration is configured, if different SSB indexes are mapped to the overlapping ROs, it might be required for a gNB to have special handling for reception of the PRACH preamble with different Rx beams in overlapping ROs. Therefore, to avoid using different Rx beams on overlapping ROs for the reception of the PRACH preambles from the gNB's perspective, we propose to support mapping of the same SSB index to overlapping legacy RO and additional RO.

- Proposal 6: For RACH configuration Option 2, in the case that overlapping between the legacy ROs of the legacy PRACH configuration and the additional-ROs configured by the additional RACH configuration,

  - we propose to support mapping of the same SSB index to overlapping legacy RO and additional RO to avoid using different Rx beams on overlapping ROs for the reception of the PRACH preambles from the gNB's perspective.

Validation of PRACH Occasions

At the RAN1 #116-bis meeting, it was discussed on RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon for RACH configuration Option 1 and Option 2. The followings were agreed for RACH configuration Option 1, but no conclusion was reached regarding RACH configuration Option 2 with FL’s proposal.

Agreement at RAN1#116-bis For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any). - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs) - the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB - FFS: Other condition. Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs. Updated proposal 1-2-7a at RAN1#116-bis: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - for the additional-ROs in SBFD symbols configured by additional RACH configuration, they are valid if at least: - time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB FFS: Other condition.

Considering SBFD operation with UL subband in symbols configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon, it may be desirable to have the same RO validation rule when the ROs are configured in the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, regardless of which RACH configuration option is applied. Therefore, it is suggested for RACH configuration Option 2 that the configured ROs in SBFD symbols configured by additional RACH configuration are valid if at least time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB as the same manner with RACH configuration Option 1.

For RACH configuration Option 2, it might be allowed that time and frequency resource of the RO are fully within UL usable PRBs, overlapped with SSB in time domain. If so, from the perspective of a UE operating in half-duplex mode, if the cell-common and essential SSB reception overlaps at least partially with the configured RO in the time domain, the configured RO should be considered invalid. However, for the reception of SSBs that are UE-specific, it may be more appropriate to consider the configured RO as valid, thereby not reducing the opportunity for RO.

- Proposal 7: For RACH configuration Option 2, it is suggested that the configured ROs in SBFD symbols configured by additional RACH configuration are valid if at least time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB as the same manner with RACH configuration Option 1.

  - FFS whether it is permissible for the time and frequency resources of the RO to be fully within the UL usable PRBs, even if they overlap with the SSB in the time domain.

And for configured ROs in SBFD symbols, the following conditions whether or not RO validation rule specified in legacy NR system is necessary were discussed and agreed at RAN1#118 meeting.

Agreement at RAN1#118 For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation. - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Note: The Ngap here is the same as the Ngap in the current specification

On top of agreed condition #1 and condition #2 for the Ngap required for UL transmission after DL reception for half-duplex operating UE, when DL reception is required in the DL subband of the last SBFD symbol, a setting for the gap required for UL transmission after DL reception may also be necessary to determine valid RO. Therefore, we propose to further discuss condtion#4 (i.e., A valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE.) on top of agreed condition #1 and #2.

- Proposal 8: Regardless of which RACH configuration option is applied, on top of agreed Condition#1/#2 we propose to further discuss Condition#4 for RO validation.

  - Condition#4: A valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE.

PRACH resource selection during one random access procedure

Agreement at RAN1#118 For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement at RAN1#118 For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement at RAN1#118 For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

The main purpose of configuring additional ROs on the SBFD symbol according to the SBFD subband configuration through legacy PRACH configuration or additional PRACH configuration was to expand the coverage of the PRACH preamble and reduce latency. To perform PRACH transmission using only ROs of the same type in one random access procedure does not seem to correspond to the purpose of introducing these additional ROs. In addition, when using other types of ROs if certain conditions is satisfied in one random access process, there is no way for a gNB to know whether the SBFD aware UE is satisfied for certain conditions, from the viewpoint of PRACH reception at the gNB compared to the UE’s independent selection between legacy-ROs and additional-ROs, there would be little benefit and a lot of effort at the WG meeting to determine certain conditions will be required. Therefore, it may be desirable to have the SBFD aware UE perform an independent selection in the selection of legacy ROs and additional ROs for PRACH transmission, regardless of whether it is an initial PRACH transmission or PRACH transmission re-attempt in one random access procedure. Among several options at RAN1#118 meeting, we support Option 3 (i.e., It’s up to UE implementation to select one type between legacy-ROs and additional-ROs) for initial PRACH transmission attempt in one random access procedure for SBFD-aware UEs and RACH configuration Option 2. And for SBFD aware UEs and both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2, we support Option 3 (i.e., Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure) for PRACH transmission re-attempt in one random access procedure.

- Proposal 9: For SBFD-aware UEs and RACH configuration Option 2, we support Option 3 for initial PRACH transmission attempt in one random access procedure.

  - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

- Proposal 10: for SBFD aware UEs and both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2, we support Option 3 for PRACH transmission re-attempt in one random access procedure.

  - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Conclusion

In this contribution, we discussed SBFD operation of UEs in RRC_IDLE/INACTIVE mode and in RRC_CONNECTED mode for random access and summarize our views as the followings:

- Proposal 1: For RACH configuration Option 1, we propose to support Alt 2-x except Alt 1 for determination of lowest RO of additional-ROs in SBFD symbols by reinterpreting at least msg1-FrequencyStart to configure the RO within usable PRBs in the UL subband.

- Proposal 2: For RACH configuration Option 2, it may be advisable to include parameters such as msg1-FrequencyStart, msg1-FDM, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which are deemed to require reinterpretation in Option 1, and prach-ConfigurationIndex for setting RO resources on the time domain in an additional RACH configuration.

- Proposal 3: For RACH configuration Option 1, we propose to support legacy SSB to RO mapping rule for validated ROs within UL subband for Rel-19 SBFD aware UEs considering BW size difference between RO’s BW size of UL subband and legacy RO’s BW size for non-SBFD aware UEs.

- Proposal 4: For RACH configuration Option 1, we propose to have separate SSB index to RO mapping for new validated ROs in SBFD symbols for SBFD aware UEs, regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon.

- Proposal 5: For RACH configuration Option 2,

  - we propose to have a separate/independent SSB index to RO mapping for new validated ROs in SBFD symbols from the SSB index to RO mapping for legacy ROs regardless of being configured as downlink or flexible by tdd-UL-DL-ConfigurationCommon.

  - for validated ROs across SBFD symbols and non-SBFD symbols either in the same slot or across slots, we propose to map the same SSB index to validated ROs across SBFD symbols and non-SBFD symbols either in the same slot or across slots.

- Proposal 6: For RACH configuration Option 2, in the case that overlapping between the legacy ROs of the legacy PRACH configuration and the additional-ROs configured by the additional RACH configuration,

  - we propose to support mapping of the same SSB index to overlapping legacy RO and additional RO to avoid using different Rx beams on overlapping ROs for the reception of the PRACH preambles from the gNB's perspective.

- Proposal 7: For RACH configuration Option 2, it is suggested that the configured ROs in SBFD symbols configured by additional RACH configuration are valid if at least time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB as the same manner with RACH configuration Option 1.

  - FFS whether it is permissible for the time and frequency resources of the RO to be fully within the UL usable PRBs, even if they overlap with the SSB in the time domain.

- Proposal 8: Regardless of which RACH configuration option is applied, on top of agreed Condition#1/#2 we propose to further discuss Condition#4 for RO validation.

  - Condition#4: A valid RO starts at least X>=0 (FFS the value) symbols after a last SBFD symbol with DL reception for a given UE.

- Proposal 9: For SBFD-aware UEs and RACH configuration Option 2, we support Option 3 for initial PRACH transmission attempt in one random access procedure.

  - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

- Proposal 10: for SBFD aware UEs and both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2, we support Option 3 for PRACH transmission re-attempt in one random access procedure.

  - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

References

- Final Report of 3GPP TSG RAN WG1 #116 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #116-bis meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #117 meeting v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #118 meeting v0.1.0