3GPP TSG RAN WG1 #118-bis		R1-2408922

Hefei, China, October 14th – October 18th, 2024

Agenda Item:	9.1.1

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on Incoming LS on applicable functionality reporting for beam management with UE-sided model

Document for:	Discussion

# Introduction

This document includes our discussions and comments on the questions raised by the RAN2, through the LS in R2-2407848, on applicable functionality reporting for beam management with UE-sided models.

# Background

RAN2 agreed the following understandings on terminologies [1]:

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

RAN2 has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality.

# Discussion

1. Questions on granularity of functionality

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

A1: In our view, the granularity of functionality should be at the sub-use case level of beam management since the model input and output of beam management sub-use cases are different. The signaling information corresponding to each sub-use case could be specific to that sub-use case. For example, observation window and prediction window are applicable only to BM-case2 but not BM-case1.

2. Questions on NW side additional conditions and configuration

Q2: What is the content of the NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

A2: RAN1 couldn’t agree on the content of the NW-side additional conditions [2]. However, RAN1 agreed on the working assumption that the NW-side additional conditions were consistent with the associated ID between different time instances (example: training and inference). RAN2 can assume that the NW-side additional conditions are consistent with every instance of the associated ID, once the associated ID is assigned by UE or NW.

Q3: Is NW-side additional conditions functionality specific?

A3: In our view, the granularity of functionality is sub-use case level, thus, we believe that the NW-side additional conditions are functionality specific.

Q4-1: In RAN2, it is FFS whether the NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional conditions?

A4-1: For beam management, the NW-side additional conditions are mandatory. UE needs information about NW-side additional conditions to decide applicable functionalities along with other configurations from NW about the supporting conditions. However, it should be noted that for other use cases, the information about NW-side additional conditions may not be needed.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional conditions in Step 3 for UE to determine applicable functionalities?

A4-2: In our perspective, the gNB is required to provide configurations, other than NW-side additional conditions information, about supporting conditions to enable UE to select the applicable functionalities.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

A4-3: The information about NW-side additional conditions is required and essential to achieve consistency between model training and model inference. This serves as a criterion to select applicable functionalities. Additionally, there could be other criteria that help the UE in selecting applicable functionalities based on supporting conditions.

In our view, we can interpret this in 2 ways,

A) NW-side additional conditions are part of inference configuration.

B) NW-side additional conditions are the primary part of inference configuration and other configuration is the secondary part of inference configuration.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

A4-4: The content of configuration in step 3 other than NW-side additional conditions is the configuration of the supporting conditions reported by UE as capability report.

Q5: What is the content of applicable functionality reporting in Step 4?

A5: The content of applicable functionality reporting in Step 4 would be IDs of the applicable functionality(ies).

Q6: What is the content of inference configuration in Step 5?

A6: The content of inference configuration in Step 5 is the configuration of supporting conditions reported by the UE as capability report.

3. Questions on of Functionality Activation

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

A7: The activation of the functionality could be performed post the selection of applicable models based on the RRC configuration in step 3. The best model among the applicable models could be activated.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

A8: Yes, the activation of the functionality would be performed immediately after the RRC configuration in step 5. The best model among the applicable models could be activated.

Q9: If more than one functionality is configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

A9: As per the definition of functionality (granularity), multiple applicable functionalities can be activated.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

A10: A provision to signal activation/deactivation of the AI/ML model at the UE-side for beam management use cases is recommended. Note that the activation/deactivation can be implicitly signaled in the form of threshold values for model performance metrics.

# Conclusion

In this contribution, we have provided our observations and comments on a) questions in LS from RAN 2 [1] on the specification support required for AI/ML based Beam Management with UE-sided models.

Proposal 1: The granularity of functionality should be at the sub-use case level of beam management.

Proposal 2: The gNB is required to provide configurations, other than NW-side additional conditions information, about supporting conditions to enable UE to select the applicable functionalities.

Proposal 3: L1/L2 signaling for functionality activation/deactivation is needed. However, the signaling can be implicit in the form of thresholds for model performance metrics to activate/deactivate the functionality.

Observation 1: RAN2 can assume that the NW-side additional conditions are consistent with every instance of the associated ID, once the associated ID is assigned by UE or NW.

Observation 2: In our view, the granularity of functionality is sub-use case level, thus, we believe that the NW-side additional conditions are functionality specific.

Observation 3: For beam management, the NW-side additional conditions are mandatory. UE needs information about NW-side additional conditions to decide applicable functionalities along with other configurations from NW about the supporting conditions.

Observation 4: The activation of the functionality could be performed immediately after RRC configuration in Step 3 or Step 5. The best model among the applicable models could be activated.

Observation 5: NW-side additional conditions are  part of inference configuration.

Observation 6: The content of configuration in Step 3 other than NW-side additional conditions and in Step 5 is the configuration of the supporting conditions reported by UE as capability report.

Observation 7: The content of applicable functionality reporting in Step 4 would be IDs of the applicable functionality(ies).

# References

[1] R2-2407848  LS on applicable functionality reporting for beam management UE-sided model

[2] Chair Notes, RAN1 #118 Maastricht, August 2024