3GPP TSG RAN WG1 Meeting #118bis										                         R1-2408951

Hefei, China, October 14th – 18th, 2024

Agenda item:		9.5.2

Title:						On-demand SIB1 for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The revised WID for Network Energy Saving (NES) was approved in RAN#105 [1], where the agenda items for specifying on-demand SIB1 is described as follows:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

This contribution discusses aspects of the specification work needed as per the above description. We start by reviewing the on-demand SIB1 feature as per Case 2 and revisit relevant agreements and discussions from RAN1 #118 [2].

# Review of On-Demand SIB1 Case 2

As depicted in Figure 1, to enable the feature of on-demand SIB1 on an NES Cell as per Case 2, the UE needs to be covered by a (non-NES) Cell A with regular SSB/SIB1 transmissions in order to obtain UL WUS configuration of the NES cell from the Cell A. The following RAN3#125 agreements [3] are worth noting for understanding how the UL WUS configuration of the NES Cell can be obtained via the Cell A.

UL WUS configuration is decided by NES Cell DU and transfers to NES Cell CU. UL WUS configuration transmission from NES Cell DU to NES Cell CU and to CellA. Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Figure 1: NES cell with on-demand SIB1 (Case 2). UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Essentially, only in scenarios where a coverage layer formed by Cell A is available for the UEs, the on-demand SIB1 feature can be supported for the NES Cells, which act as a capacity layer in spots within the coverage of Cell A. In such a setting with the NES Cells transmitting only SSBs, the following should be addressed in specification to minimize performance impacts:

- Efficient Identification of NES Cell  to minimize impact on initial access, particularly during Initial Cell Search.

- WUS configuration updates for UEs camping on a cell  to avoid UEs having to switch to another cell.

These 2 aspects are discussed in further detail in the following sections.

# Efficient Identification of NES Cells

Consider a UE doing initial cell search under potential coverage of an NES Cell with on-demand SIB1 (only transmitting SSBs and transmitting System Information only on-demand) as well as a Cell A (non-NES cell broadcasting System Information regularly). If this UE identifies the Cell A first, UE identification of the NES cell can be performed through the UL WUS Configuration that it will receive from the Cell A. Instead, if the UE encounters the SSB transmissions from the NES Cell first, it is very beneficial to do the identification on the NES Cell itself so that it can right away start searching for other cells to eventually find the Cell A. Otherwise, the UE would unnecessarily waste time and battery in looking for SIB1 just not to find it until eventually the UE decides to continue the cell search in other GSCNs.

Observation 1: Including the identification of NES Cell in both the WUS Configuration and the NES Cell itself can minimize impact of on-demand SIB1 feature on initial access performance.

On the issue of how UE identifies an NES Cell with on-demand SIB1, the following agreement was made in RAN1 #116b [5].

In RAN1 #116b, the following is agreed: RAN1 #116b Agreement RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell

Although no further agreement was made on this issue, let us review the follow up discussion from RAN1 #118 [2], specifically the latest FL proposal given below.

### FL Proposal 6-2-3 For further work of on-demand SIB1 in idle/inactive mode, it is assumed UE identifies a NES cell with OD-SIB1 based on both K_SSB and/or the UL-WUS configuration received by the UE. - For a NES cell with OD-SIB1, K_SSB > 23 for FR1 and K_SSB >11 for FR2 - FFS: whether K_SSB is limited to 30 for FR1 and 14 for FR2 - [FFS: UE behavior after identifying NES cell]

The identification on the NES Cell can be done by the PBCH using the reserved values of K_SSB: 30 for FR1 and 14 for FR2. This approach has several advantages. First and foremost, as soon as the UE is able to detect SSB and decode MIB, the UE will already be able to evaluate that this frequency is not broadcasting SIB1 and then it can move to the next GSCN. That is accomplished without the need of any extra signalling (in addition to SSBs). Furthermore, as described in TS 38.331 [8] these values of K_SSB will lead legacy UEs to consider the cell as barred, which is not only the exact expected behaviour but also means we would need no additional cell barring solution for legacy UEs. Regarding the values 23<K_SSB<30 (for FR1) and 11<K_SSB<14, they cannot be used because legacy UEs which find such values on MIB they would search for a CD-SSB with the given offset. Because of that and the advantages of using the reserved values, we support the proposal to use K_SSB with the modification as follows.

Proposal 1: NES Cells set K_SSB = 30 for FR1 and K_SSB = 14 for FR2 to identify the NES state of OD-SIB1.

# WUS Configuration Updates for UEs Camping on a Cell

It is clear from the definition of Case 2 and the agreements made so far that cells in ‘non-NES mode’ (without the on-demand SIB1 feature enabled) can provide UL WUS configurations for ‘other cells’ that are NES Cells. What needs to be addressed then, are other possibilities to avoid unnecessary switching of UEs between NES Cells and non-NES Cells. Namely, for UEs already camping on a cell (irrespective of it being a ‘non-NES’ or a ‘NES’ cell), how can the provisioning of any kind of update of UL WUS configurations (for ‘itself’ or for ‘other cells’) be done efficiently? More specifically,

- For a UE already camping on an NES Cell, can updates to its own UL WUS configuration be provisioned without switching to a Cell A?

- For a UE already camping on an NES Cell and moving to another NES Cell’s coverage, can the WUS configuration for the other NES Cell be provisioned without switching to a Cell A?

- For a UE already camping on a non-NES cell (i.e., a Cell A) that want to switch to NES mode (with on-demand SIB1), can the WUS configuration be provisioned without switching to another Cell A?

Regarding the first question, since RAN2 has already agreed on it, we propose to agree on the FL Proposal 5-3-2 from last meeting [2].

Proposal 2: For further work of on-demand SIB1 in idle/inactive mode, the provisioning of update of UL WUS configuration is left to RAN2 discussion.

Regarding the second question, again, we support the FL Proposal 5-4 from last meeting [2] to be further discussed.

Proposal 3: RAN1 to further discuss whether the NES cell the UE camps on can provide UL WUS configuration for other NES cells.

In addition, we propose to address the third question also in RAN1 as follows.

Proposal 4: RAN1 to further discuss, whether a Cell A (non-NES mode) the UE camps on, can provide UL WUS configuration for itself, if it wants to switch to NES mode without involving another Cell A.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

Observation 1: Including the identification of NES Cell in both the WUS Configuration and the NES Cell itself can minimize impact of on-demand SIB1 feature on initial access performance.

Proposal 1: NES Cells set K_SSB = 30 for FR1 and K_SSB = 14 for FR2 to identify the NES state of OD-SIB1.

Proposal 2: For further work of on-demand SIB1 in idle/inactive mode, the provisioning of update of UL WUS configuration is left to RAN2 discussion.

Proposal 3: RAN1 to further discuss whether the NES cell the UE camps on can provide UL WUS configuration for other NES cells.

Proposal 4: RAN1 to further discuss, whether a Cell A (non-NES mode) the UE camps on, can provide UL WUS configuration for itself, if it wants to switch to NES mode without involving another Cell A.

# References

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #105 Melbourne, Australia, Sep 9th – 12th 2024.

- R1-2407217, “FL summary 5 for on-demand SIB1 in idle/inactive mode”, Moderator (MediaTek), 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024.

- RP-241866, “Status report for WI on Enhancements of network energy savings for NR”, Rapporteur (Ericsson), 3GPP TSG RAN Meeting #105.

- Chair’s Notes, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August 19th – 23rd, 2024.

- Chair’s Notes, 3GPP TSG RAN WG1 #116-bis, Changsha, China, April 15th – 19th, 2024.

- Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka, Japan, May 20th – 24th, 2024.

- TR 38.864, “Study on network energy savings for NR”, V1.0.0, December 2022.

- TS 38.331, “	NR; Radio Resource Control (RRC); Protocol specification”, v18.2.0, July 2024.