3GPP TSG RAN WG1 #118bis		 R1-2409056

Hefei, China, October 14th – 18th, 2024

Agenda item:	7

Source: 	Moderator (MediaTek Inc.)

Title: 	Summary on PDSCH reception scheduled by CORESET not following the unified TCI state (Rel-17 MIMO)

Document for:	Discussion and Decision

- Introduction

In [1], one issue in current specification for Rel-17 unified TCI framework has been raised, and a corresponding draft CR [2] is provided to address the raised issue.

In current specification for Rel-17 unified TCI framework, UE behaviors on PDSCH reception are clearly defined for the following two cases:

- For scheduling offset < timeDurationforQCL, PDSCH reception behaviors are defined in TS 38.214 Clause 5.1.5 [3], regardless of configuration of followUnifiedTCI-State and tci-PresentInDCI/tci-PresentDCI-1-2

5.1.5 Antenna ports quasi co-location Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

- For scheduling offset >= timeDurationforQCL, according to TS 38.213 Clause 10.1 [4], if a PDSCH is scheduled by a CORESET following the unified TCI state, the UE shall apply unified TCI state to the scheduled PDSCH as well.

10.1 UE procedure for determining physical downlink control channel assignment For a CORESET with index 0, - if the UE is provided TCI-State and followUnifiedTCI-State for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214] - else, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with - the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or - a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19. … If a UE is provided TCI-State in dl-OrJointTCI-StateList, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State [6, TS 38.214]. If a UE is provided followUnifiedTCI-State for a CORESET, other than a CORESET with index 0, associated at least with CSS sets other than Type3-PDCCH CSS sets, a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State.

For the case if scheduling offset >= timeDurationforQCL, and if a PDSCH is scheduled by a CORESET NOT following the unified TCI state, based on the understanding in [1], the scheduled PDSCH should follow the same QCL assumption as the one used for the CORESET corresponding to the scheduling DCI. The behavior should be applicable regardless of the presence of TCI field, However, in current specification, the behavior can be only determined from the following highlighted paragraph in TS 38.214 Clause 5.1.5 [3], which is limited only when TCI field is not present in the scheduling DCI.

5.1.5 Antenna ports quasi co-location If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling a PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

Therefore, it is proposed in [1] to clarify the following behavior for PDSCH reception scheduled by CORESET NOT following the unified TCI state in the Rel-17 specification (TS 38.214 Clause 5.1.5):

  - Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE may assume that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell.

- Discussion

Table 1 Questions for Discussion

| # | Question |
| --- | --- |
| Q1 | Whether the current specification for Rel-17 unified TCI framework is unclear for PDSCH reception scheduled by a CORESET that does not follow the unified (indicated) TCI state with the scheduling offset (the time offset between the reception of the scheduling DCI and the corresponding PDSCH) equal to or greater than timeDurationForQCL when the scheduling DCI does not have the TCI field present? |
| Q2 | If the answer to Q1 is “Yes”, whether/which one of the following potential CRs can be adopted to address the issue? Alt1: In such case for corresponding PDSCH reception, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH 5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE may assume that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell. <Unchanged parts are omitted>. Alt2: In such case for corresponding PDSCH reception, the UE applies the indicated TCI state 5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the indicated TCI state is applied to the PDSCH of the serving cell. <Unchanged parts are omitted>. Note: Based on the following agreement, Alt2 may be contradictory since the following agreement implies whether the scheduled PDSCH should follow the indicated TCI state is determined together with the corresponding PDCCH. RAN1#107-e agreement For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: - For discussion purposes, define as follows: - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [USS and/or CSS Type 3] - ‘CORESET B’: A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [CSS or CSS other than Type 3] - ‘CORESET C’: A CORESET other than CORESET#0 associated with both UE-dedicated and non-UE-dedicated reception on PDCCH in a CC - CORESET#0 - For Rel-17 TCI state indication, support per CORESET determination as follows: - For any PDCCH reception on a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC - FFS: For intra-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: For inter-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: The TCI state of CORESET 0. |

Table 2 Company input for Questions in Table 1

| Company | Input |
| --- | --- |
| CATT | We are open to discuss this issue. It is our view that from the current agreement, PDSCH follows TCI in the CORESET only when the TCI field is absent in the DCI. [Mod] To my understanding, we don’t have such agreement for Rel-17 unified TCI framework. |
| ZTE | First of all, we are open to discuss this issue and exchange views across companies. In our views, the interpretation of unified TCI indication has been decoupled from the scheduled DCI, which is a basic principle for unified TCI operation (especially for avoiding complicated rule of beam collision). Therefore, regardless of being following or not for the scheduled PDCCH/CORESET, the PDSCH should follow the indicate TCI state unless some specific clarification is provided. In our views, the following description in 38.213 seems clear. If a UE is provided TCI-State in dl-OrJointTCI-StateList, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State [6, TS 38.214].. [Mod] Above behavior seems only applicable when the scheduling DCI is from CORESET following the indicated TCI state (i.e., when it is associated with only with USS sets and/or Type3-PDCCH CSS, the indicated TCI state is always applied to the corresponding CORESET). What about the case if the PDSCH is scheduled by a CORESET not following the indicated TCI state? |
| vivo | We are fine to discuss, we have following agreement in past： RAN1#106-e agreement The following working assumption is confirmed with revision in RED. On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), Rel-17 mechanism(s) which reuse the Rel-15/16 TCI state update signaling/configuration design(s) are used to update/configure such DL RS(s) with Rel-17 TCI state(s). - Applies for both intra-cell and inter-cell beam indication. |
| Qualcomm | The CORESET not following uTCI is mainly used to schedule broadcast transmissions using fallback DCI (without TCI indication field). In this case, the legacy rule, i.e., following the QCL assumption of the scheduling CORESET would be applicable. However, for unicast transmission using non-fallback DCI with a TCI indication field, the legacy Rel-15 rule is to follow the TCI field value, which is not applicable when uTCI is configured. Therefore, in our view, a more acceptable approach is letting the PDSCH follow the current indicated TCI state, not the QCL assumption of the scheduling CORESET. Also, the TCI indication field of the non-fallback DCI in the CORESET should also be used to indicate a new uTCI. |
| vivo2 | Following agreements are related to this issue in Rel-17: RAN1#106-e agreement The following working assumption is confirmed with revision in RED. On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), Rel-17 mechanism(s) which reuse the Rel-15/16 TCI state update signaling/configuration design(s) are used to update/configure such DL RS(s) with Rel-17 TCI state(s). - Applies for both intra-cell and inter-cell beam indication RAN1#106bis-e agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI: - For DL channels/signals that do not share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH (via Rel-17 MAC-CE/DCI TCI state update), all the QCL rules defined in section 5.1.5 in 38.214 are supported - Note: For CSI-RS used to provide QCL indication for non-UE dedicated channels, the CSI-RS should only be QCLed with SSB of the same PCID as that from the serving cell - For DL channels/signals that share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH (via Rel-17 MAC-CE/DCI TCI state update), the following options on source RSs and QCL-Types are supported - Option 1: TRS is configured for QCL-TypeA source RS and CSI-RS for BM is configured for QCL-TypeD source RS - Option 2: TRS is configured for QCL-TypeA and QCL-TypeD source RS - Note: For inter-cell beam management, SSB with PCID different from that from the serving cell can be used as a QCL Type-C/D source RS for CSI-RS for BM and/or TRS - Further discuss and decide in RAN1#106bis-e whether CSI-RS for CSI can be used as a source RS or not, and if so whether some restriction(s) are needed RAN1#107-e agreement For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: - For discussion purposes, define as follows: - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [USS and/or CSS Type 3] - ‘CORESET B’: A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [CSS or CSS other than Type 3] - ‘CORESET C’: A CORESET other than CORESET#0 associated with both UE-dedicated and non-UE-dedicated reception on PDCCH in a CC - CORESET#0 - For Rel-17 TCI state indication, support per CORESET determination as follows: - For any PDCCH reception on a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC - FFS: For intra-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: For inter-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: The TCI state of CORESET 0. Then, we have following understanding for unified TCI state framework: - For the case of a PDCCH reception on a COERSET not following the indicated TCI state, QCL assumption of its respective PDSCH follows Rel-15 QCL rules (based on the RAN1#106bis-e agreement), i.e., Case a) If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. Case b) If the PDSCH is scheduled by a DCI format having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall use the TCI-State according to the value of the TCI field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. Case c) Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. - Current spec has captured Case c) with another more suitable way for intra-cell cases to avoid uncertainty of the default beam, i.e., Case c’) Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - If we follow the RAN1#106bis-e agreement for other cases, Case a) and b) should be the QCL assumption. - Alt1 is obviously contradictory to RAN1#106bis-e agreement (would NOT follow Rel-15 QCL rule when the TCI field is present) - In Alt2, it seems that “the indicated TCI” is the indicated unified TCI state, then it is contradictory to either RAN1#106bis-e agreement (would NOT follow Rel-15 QCL rules) or RAN1#107-e agreement (when configured RRC parameter indicates that a PDCCH reception on a CORESET and the respective PDSCH reception does NOT apply the indicated Rel-17 TCI state, but the PDSCH would follow the indicated Rel-17 TCI state) [Mod] To my understanding, the first agreement made in RAN1#106 is specific to DL RS instead of DL channel. The second agreement made in RAN1#106 just clarifies the QCL rules. For RAN1#107-e agreement, I tend to agree with you that Alt2 may be contradictory to it. - With our understanding in 3), in such a case that {for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL}, the TCI state indicated in the TCI field (if TCI field is present) has two functions, one is to indicate the TCI state to be used by the scheduled PDSCH (Rel-15 QCL rule), the other is to indicate a unified TCI state (Rel-17 TCI state indication). Thus, we provide Alt3 for the CR. Alt3: In such case for corresponding PDSCH reception, the UE follows the Rel-15 QCL rule (based on the Agreement in RAN1#106bis-e). For a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that is not configured with followUnifiedTCI-State and the DCI format is having the TCI field present, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE shall use the TCI-State according to the value of the TCI field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. If a PDSCH of a serving cell is scheduled by a CORESET that is not configured with followUnifiedTCI-State and the DCI format is not having the TCI field present, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. [Mod] To my understanding, the TCI field was agreed to be used for updating unified TCI state, regardless from CORESET following or not following the unified TCI state. |
| Google | Q1: Yes, it is unclear. Q2: As mentioned by vivo, if the scheduling PDCCH (i.e., DMRS of PDCCH) does not follow indicated TCI state, Rel-15/16 TCI state update signaling/configuration design(s) is reused. Nonetheless, in Rel-15/16, TCI field is for indicating TCI state for PDSCH reception; on the other hand, in Rel-17, TCI field is for indicating TCI state for all DL/UL channels/RSs, which is different functionality. With that said, there is no legacy behavior to be referred for such case. Hence, we are discussing new behavior in our views. Either Alt1 or Alt2 is actually feasible. However, let’s recall that in Rel-18, for PDSCH scheduled by a DCI format without PDSCH beam indicated, the indicated TCI state is always used, even when the scheduling PDCCH does not follow indicated TCI state. Note that Rel-18 is an extension of Rel-17, and built on top of Rel-17. If the solution for the same use case in Rel-17 and Rel-18 is different, we do not understand how it can work. With that said, if Alt 1 is to be adopted, Rel-18 behavior should also be modified accordingly, that is, PDSCH beam determination when scheduling PDCCH does not follow indicated TCI state. |
| Huawei, Hisilicon | We have agreed that for UE dedicated PDSCH, it always follow indicated TCI-state, regardless of the scheduling PDCCH following the indicated TCI-state or not. While, for non-UE-dedicated PDSCH, it may not always follow indicated TCI-state. For non-UE-dedicated PDSCH, if the scheduling PDCCH follow indicated TCI-state, the DPSCH also follows the indicated TCI-state. While, if the scheduling PDCCH does not follow indicated TCI-state, it seems unclear what TCI-state should be applied for the PDSCH. So, we are Ok to clarify it in the spec. |
| Samsung | To our understanding, the PDSCH should follow the indicated TCI state regardless of the status of “followUnifiedTCI-state” of the scheduling CORESET (Alt-2). We do not think that this is contradictory to the Rel-17 uTCI agreement provided above. |

- Conclusion

Based on feedback from companies so far, all companies agreed to discuss how to address the issue from Rel-17 unified TCI framework. However, it seems companies have different understandings on how to define the behavior for PDSCH reception scheduled by a CORESET not following the unified (indicated) TCI state with the scheduling offset equal to or greater than timeDurationForQCL. From moderator’s perspective, it would be better to clarify the corresponding behavior in the specification clearly, and three alternatives proposed by companies are listed for down-selection.

Alt1: The UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception.

- Support: Google, MediaTek

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE may assume that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell. <Unchanged parts are omitted>

Alt2: The UE applies the indicated TCI state to the corresponding PDSCH reception.

- Support: ZTE, Qualcomm, Google, MediaTek

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the indicated TCI state is applied to the PDSCH of the serving cell. <Unchanged parts are omitted>

Alt3: If the TCI field is present in the DL DCI, the UE shall use the TCI-State according to the indicated codepoint of the TCI field for determining the QCL assumption for the corresponding PDSCH reception. If the TCI field is not present in the DL DCI, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception.

- Support: vivo

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). For a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, - if the TCI field is present in the DL DCI scheduling the PDSCH of the serving cell, the UE shall use the TCI-State according to the indicated codepoint of the TCI field in the DL DCI for determining the QCL assumption for the DM-RS ports of PDSCH(s) of the serving cell. - if the TCI field is not present in the DL DCI scheduling the PDSCH of the serving cell, the UE assumes that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. <Unchanged parts are omitted>

- References

[1] R1-2408085, Discussion on Rel-17 unified TCI framework, MediaTek Inc., October 2024

[2] R1-2408198, Draft CR on PDSCH reception scheduled by CORESET not following the unified TCI state, MediaTek. Inc, NTT DOCOMO. INC, Ericsson, October 2024

[3] 3GPP TS 38.214 V17.11.0, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 17)”, September 2024

[4] 3GPP TS 38.213 V17.11.0, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for control (Release 17)”, September 2024