3GPP TSG RAN-WG1 Meeting #118bis			R1-2409072

Hefei, China, 14-18 October, 2024

Source: 	Moderator (Huawei)

Title:	Feature Lead Summary #3 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”

Document for:	Discussion and decision

Agenda item:	9.4.2.1

# Introduction

According to the chair’s agenda, this feature lead summary will cover discussions on:

- Waveform (R2D; D2R)

- Modulation (R2D; D2R)

- Coding

  - Line coding (R2D; D2R), channel coding / repetition (R2D; D2R)

  - CRC (jointly for R2D and D2R)

- Multiple access (R2D; D2R)

- Time-domain definitions (R2D; D2R)

- Bandwidth (R2D; D2R)

Proposal X.Y(z) is in Section X.Y, where (z) a Roman numeral I, II, III, IV, V, …, is the version of that proposal.

Proposals for online sessions will be added to Section 5 (link).

Decisions are authoritatively in the chair notes, and may be copied into Section 6 (link) from time to time.

Previous meetings’ decisions are in Annex A (link).

## Versions

FLS #1: R1-2409070

FLS #2: R1-2409071

FLS #3: R1-2409072

# R2D

## R2D waveform

### CP handling [ACTIVE]

Agreement RAN1#116bis For R2D CP handling for OFDM based OOK waveform: - For potential down-selection, study among the following candidate methods - Method Type 1: Removal of CP at device without specified transmit-side - … - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. - … - [Other method types are not precluded] Agreement RAN1#117 Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations

#### Round 1

From studying the papers, FL considers that for a feasibility study, the agreements to date broadly cover the options, and companies are now nicely discussing stage-3 details, among which further down-selection belongs to any potential normative phase. The proposals and evaluations from companies to date are a good feasibility study input to that.

Hence, FL has prepared the following text proposals, intending to capture what companies have designed and evaluated within the agreed framework. The two Method Types are presented in separate sections for convenience of responses, but FL expects both to be included in the TR.

The basic structure of the TPs below is to give a summary at the start of how companies have described the nature of each Method Type at high level, followed by a summary of what companies have discussed within each alternative of that Method.

##### TP for Method Type 1

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location - Some sources [vivo][Huawei][CMCC][Futurewei] report the device implementation may depend on M values For CP handling Alt M1-1, the potential impacts are discussed as follows: - Some sources [Futurewei][Huawei][Spreadtrum][vivo][InterDigital][Qualcomm] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4. - Some sources [Ericsson][ZTE] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [vivo][Apple] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [Huawei] show that the performance loss due CP handling is negligible even for large M values (e.g. M=24) under large SFO (e.g. 10^4-10^5 ppm). Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ). - Some source [CMCC][Apple] report that the device needs additional complexity to handle CP, while other sources [Huawei][InterDigital] reports that it is feasible and does not add implementation complexity based on transition edge detection. For CP handling Alt M1-2, the potential impacts are discussed as follows: - Some sources [Futurewei][Nokia][CMCC][Ericsson][Huawei][Spreadtrum][vivo][ZTE][Apple][InterDigital] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8. - One source [LGE] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [vivo] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 10^4 ppm). For Method 2, two approaches regarding subcarrier orthogonality are studied: …(unchanged parts omitted)…

| Company | Is anything missing? |
| --- | --- |
| Qualcomm | On the third bullet of CP handling Alt M1-1, we think it is helpful to add a bit more detailed information to understand “in which case” the loss is negligible or not. Following is a suggestion: - Among of them, [Huawei] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible for achieving BLER=10% even for large M values (e.g. M=24) under large SFO (e.g. 10^4-10^5 ppm). Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ). On the fourth bullet of CP handling Alt M1-1, device complexity is discussed. In our understanding, Alt M1-1 requires a certain implementation, and it is not correct to say “does not add implementation complexity”. So, we suggest to change as following. - Some source [CMCC][Apple] report that the device needs additional complexity to handle CP, while other sources [Huawei][InterDigital] reports that it is feasible in terms of and does not add implementation complexity based on transition edge detection. |
| Futurewei | The proposal is fine. |
| Lenovo | For CP handling Method Type 1, at least for Alt M1-1, we share same view as: For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE] [Lenovo] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location |
| Spreadtrum | Adding “Spreadtrum” as co-source to CP handling Method Type 1: For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE] [Spreadtrum] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location - Some sources [vivo][Huawei][CMCC][Futurewei] report the device implementation may depend on M values |
| Samsung | Adding us to the following bullet: - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE][Samsung] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location |
| ZTE, Sanechips | For the following bullet, we think it is better to capture the exact performance loss, otherwise, it is too vague for an observation. “Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ). And [ZTE] shows [1~2 dB] for M<24, and an error floor at BLER=10% for M=24” |
| Ericsson | Agree with Qualcomm’s modification suggestion |
| LGE | Okay with the proposed TP. |

##### TP for Method Type 2

In this TP, the first part is to address the “FFS: Whether/how to arrange that OOK chips have equal length after CP insertion” across both alternatives. The subsequent text is per-alternative.

FL is not sure that the text highlighted gray in Alt M2-1 is compliant with the agreed definition, since the proponents appear to assume the device skips CP, which is exactly Method Type 1, and not Method Type 2. Nevertheless, FL presents here what companies have discussed.

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. …(unchanged parts omitted)… For Method 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For CP handling Method Type 2, depends on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by, - M, and the length of OFDM symbol with CP - Results in constant chip duration at all times - M, and the length of OFDM symbol without CP - Results in chip duration which is constant, except around a CP. - Some resources [Qualcomm] report that non-constant OOK chip duration around CP may impact on decoding performance For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. For CP handling Alt M2-2, the solutions and potential impacts are discussed as follows, - [vivo][OPPO][CATT][Samsung] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC] report that CP handling of Alt M2-2 would cause interference to NR. - One source [OPPO] reports that CP handling of Alt M2-2 may result in non-constant OOK chip duration around CP. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC] report that CP handling of Alt M2-2 would increase the transmitter complexity.

| Company | Is anything missing? |
| --- | --- |
| OPPO2 | Modify the following part to reflect our view more correct For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. - One source [OPPO] reports that to ensure the chip duration (with or without CP) for all chips are same For CP handling Alt M2-2, the solutions and potential impacts are discussed as follows, - [vivo][OPPO][CATT][Samsung] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC] report that CP handling of Alt M2-2 would cause interference to NR. - One source [OPPO] reports that CP handling of Alt M2-2 may result in non-constant OOK chip duration around CP. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC] report that CP handling of Alt M2-2 would increase the transmitter complexity. |
| Futurewei | In our contribution we have two separate proposals for small and large values of M under Alt M2-1 but with two new subcategories: For small value of M, we propose Alt M2-1-3: Information bits are mapped to line codewords of n_T chips where the first OOK chip(s) and the last OOK chip(s) in the codewords are the same. For large value of M, we propose Alt M2-1-4: a modularly and linearly increased cyclic shift of the M chips’ samples with OFDM symbol index is applied at the transmitter side. Note: For Alt M2-1-3, the chip duration is determined by M and the length of OFDM symbol with CP. For Alt M2-1-4, the chip duration is determined by M and the length of OFDM symbol without CP. For Alt M2-1-3, the device can be unaware of the OFDM symbol boundaries, while Alt M2-1-1 requires the device to skip the repeated OOK chips in every OFDM symbol. For Alt M2-1-4, transitions are allowed during CP, and a constant chip duration is maintained throughout, however the device will need to be aware of a chip skipping pattern. |
| Xiaomi | We support the TP. |
| ZTE, Sanechips | (1)For the following comment, does it also apply to Method type 1? “- Some resources [Qualcomm] report that non-constant OOK chip duration around CP may impact on decoding performance” Moreover, the definition of chip duration should not illustrated herein. So the following modifications on TP are proposed: For both CP handling Method Type 1 and CP handling Method Type 2, depends on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by, - M, and the length of OFDM symbol with CP - Results in constant chip duration at all times - M, and the length of OFDM symbol without CP - Results in chip duration which is constant, except around a CP. (2)We prefer to capture the observation about the evaluated performance. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). And ZTE show that Alt M2-1-2 provides the best performance among CP handling of Method type 1 and Method type 2. - For method type 2 Alt M2-1, especially Alt M2-1-2, there are different options or detailed solutions, we prefer to capture these solutions in the TR as well for better understanding of different alternatives. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. - Alt M2-1-2-1: Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1.The first OFDM symbol carries odd value M chips, and the following OFDM symbols carry even value M chips. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - Alt M2-1-2-2: Ensure a transition edge of a Manchester codeword occurs at the start or at the end of the CP - Source [ZTE] reports that PRDCH transmission is started from the even number-th chip in the first OFDM symbol. The first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission. It has no additional overhead. For large M values, additional rising/falling edge may be introduced during the CP. [ZTE] reports that Alt M2-1-2-2 ensure no additional edge introduced by CP removal as long as the edge detection error does not exceed the length of chip. - Alt M2-1-2-3: Ensure the last OOK chip within an OFDM symbol is not shorter than the CP |
| Ericsson | Please add us to the following bullet: Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). |
| LGE | Please add LGE to the following bullets under CP handling Alt M2-2: - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC] [LGE] report that CP handling of Alt M2-2 would cause interference to NR. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC][LGE] report that CP handling of Alt M2-2 would increase the transmitter complexity. |

#### Round 2

        - TP for Method Type 1

From studying the inputs so far, FL thinks it should be OK to update some detail assumptions/results from companies’ papers corresponding to the bullets they have noted.

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE][Lenovo] [Spreadtrum][Samsung] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location - Some sources [vivo][Huawei][CMCC][Futurewei] report the device implementation may depend on M values For CP handling Alt M1-1, the potential impacts are discussed as follows: - Some sources [Futurewei][Huawei][Spreadtrum][vivo][InterDigital][Qualcomm] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4. - Some sources [Ericsson][ZTE] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [vivo][Apple] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [Huawei] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible at 10% BLER even for large M values (e.g. M=24) under large SFO (e.g. 10^4-10^5 ppm). Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ) while [ZTE] shows [1~2 dB] loss compare to no CP case for M<24, and an error floor at BLER=10% for M=24. - Some source [CMCC][Apple] report that the device needs additional complexity to handle CP, while other sources [Huawei][InterDigital] reports that it is feasible in terms of and does not add implementation complexity based on transition edge detection. For CP handling Alt M1-2, the potential impacts are discussed as follows: - Some sources [Futurewei][Nokia][CMCC][Ericsson][Huawei][Spreadtrum][vivo][ZTE][Apple][InterDigital] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8. - One source [LGE] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [vivo] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 10^4 ppm). For Method 2, two approaches regarding subcarrier orthogonality are studied: …(unchanged parts omitted)…

        - TP for Method Type 2

From studying the inputs so far, FL thinks it should be OK to update some detail assumptions/results from companies’ papers as they have noted.

However, FL think it seems not needed to capture further detail level solutions under the identified M2-1 where details are lack of discussion and could be done in later stage if a WI any, since current M2-1-1 and M2-1-2 in the TR already describe those in a high level way.

OPPO: I have trouble to understand the meaning of the sentence you added in technical/English terms. It doesn’t seem to be an analysis of a method type, rather some assertion of a claim in design? But it seems in your paper that the chip lengths are not all the same. Hence not added.

-  One source [OPPO] reports that to ensure the chip duration (with or without CP) for all chips are same

Maybe this aspect will benefit from some online discussion to find out overall views.

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. …(unchanged parts omitted)… For Method 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For CP handling Method Type 2, depends on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by, - M, and the length of OFDM symbol with CP - Results in constant chip duration at all times - M, and the length of OFDM symbol without CP - Depends on detail solutions, Results in chip duration may or may not be which is constant, except around a CP. - Some resources [Qualcomm] report that non-constant OOK chip duration around CP may impact on decoding performance For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. For CP handling Alt M2-2, the solutions and potential impacts are discussed as follows, - [vivo][OPPO][CATT][Samsung] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC][LGE] report that CP handling of Alt M2-2 would cause interference to NR. - One source [OPPO] reports that CP handling of Alt M2-2 may result in non-constant OOK chip duration around CP. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC][LGE] report that CP handling of Alt M2-2 would increase the transmitter complexity.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | For the first part of Method type 2, it seems the indentation of 3rd bullet should be adjust to in parallel to 1st and 2nd bullet and adjust 4th sub-bullet accordingly: - M, and the length of OFDM symbol with CP - Results in constant chip duration at all times - M, and the length of OFDM symbol without CP - Depends on detail solutions, Results in chip duration may or may not be which is constant, except around a CP. - Some resources [Qualcomm] report that non-constant OOK chip duration around CP may impact on decoding performance For M2-1, we share the view and would like to add the following if possible - Some sources [InterDigital][Futurewei][CMCC][Huawei] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. |
| DOCOMO | For M2-2, add DOCOMO for 2nd and 4th sub-bullet. |
| vivo | For TP for method type 1: - Some sources [Futurewei][Huawei][Spreadtrum][vivo][InterDigital][Qualcomm] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4. -》suggest to delete [vivo], and add one sentence that [vivo] reports that CP handling of Alt M1-1 can be used for small M values for OOK-4, and for large M values for OOK-4 if combined with Alt M1-2. Because in our evaluation, for M=24, Alt M1-1 alone can hardly achieve 1% BLER while combination of Alt M1-1 and M1-2 can achieve comparable performance with ideal case. - Among of them, [vivo] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 10^4 ppm). -》suggest to revise to [vivo] reports that CP handling of Alt M1-2 can be used for both small M values for OOK-4, and large M values (e.g., M=24) for OOK-4 if combined with Alt M1-1 under large SFO (e.g. 10^4 ppm). For TP for method type 2: - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). -》we suggest to add the condition e.g., adding ‘based on the assumption that device can at least roughly know the OFDM symbol duration and boundary’. Please kindly help add [vivo] for following two bullets. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] [vivo]report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. -Some sources [InterDigital][Futurewei][CMCC] [vivo] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. For CP handling Alt M2-2, some companies including vivo observe interference to NR only exists for certain scenario while no interference to NR for other scenarios. we suggest to clearly capture corresponding scenarios, by adding one bullet as below: - [vivo] reports that CP handling of Alt M2-2 would cause interference to NR, only for a scenario with indoor NR UE served by outdoor gNB but no interference for other scenarios. Even for the scenario with interference to NR, some companies including vivo observe interference to NR can be handled. we suggest to add one bullet as below: - For the scenario in which CP handling of Alt M2-2 would cause interference to NR, [vivo] reports single guard PRB would be sufficient to handle the interference. Besides, we also observe that, by Alt M2-2, it can simply device complexity. Therefore, we suggest to add one bullet as below: - [vivo] reports that CP handling of Alt M2-2 would decrease device complexity comparing with other CP handling methods/alternatives. |

### Waveform(s) [ACTIVE]

Agreement RAN1#116 A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective. - Depending on what modulation(s) are decided to be studied: - Study whether/how to handle CP at transmitter/device/design - Study other characteristics of the OFDM waveform, e.g.: - CP-OFDM - DFT-s-OFDM - Etc. - The type of OFDM waveform is transparent to A-IoT device. Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study.

Agreement RAN1#117 For R2D evaluation purposes, the R2D waveform for DFT-s-OFDM is generated as follows: - The time domain OOK signal is the M chips of one OFDM symbol. - A chip is represented (e.g. upsampled) by L samples - Companies to report L - An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - Companies to report N’, e.g. N’=128 or equal to X - Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D. - Companies report how to map and report X - An N-points IDFT is performed to obtain the time domain signal. - Companies to report N, and how value was selected Note: companies report whether/how CP samples are added.

#### Round 1

On reviewing the comments made to the RAN1#118 FL proposals and contributions to RAN1#118bis, it seems there is little or nothing more that is actually needed for a feasibility study following the above agreements. The only possible addition is a link between OOK-1/OOK-4 and CP-OFDM/DFT-s-OFDM – even that could be left to specification details of a later phase.

Proposal 2.1.2a(I): Capture in the TR:

- For OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible, and

  - For M=1, i.e. use of OOK-1 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible

  - For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible.

| Company | Views |
| --- | --- |
| TCL | In RAN 1#116, agreed OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869 and FFS value(s) of M. And also, RAN 1 #118 studied M value including M=1. In our understanding, OOK-4 with M=1 has been supported. In addition, even though companies prefer DFT-s-OFDM for M>1, CP-OFDM should not precluded in this stage. Thus, we suggest the first bullet corrects as: Proposal 2.1.2a(I): Capture in the TR: - For OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible, and - For M=1, i.e. use of OOK-1 for single-chip per OFDM symbol transmission or OOK-4 with M=1, CP-OFDM and DFT-s-OFDM are both feasible - For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible. Note: CP-OFDM is not precluded. |
| Qualcomm | The agreement captures many “companies to report”. With the agreement, it is not clear to us how the understanding “little or nothing more that is actually needed for a feasibility study” is coming. A clarification is appreciated. There is no linkage between the Proposal 2.1.2a(I) and the above RAN1#117 agreement. It is not clear how the proposal, if agreed, to be captured in the TR together with the above RAN1#117 agreement. |
| Xiaomi | We support this proposal. |
| Lenovo | OK |
| Spreadtrum | OOK-4 M=1 can also be used for single-chip per OFDM symbol transmission |
| NEC | Support with the following clarification: Proposal 2.1.2a(I): Capture in the TR: - For OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible, and - For M=1, i.e. use of OOK-1 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible - For M>1, i.e. use of OOK-M for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible. |
| Ericsson | For OFDM-based OOK waveform generation, both CP-OFDM and DFT-s-OFDM are feasible, and which one is used is transparent to the device, depending on the reader's implementation choice. We prefer Proposal 2.1.2a(I) discussed in RAN1#118, which is provided below: Proposal 2.1.2a(I): Capture in the TR that for OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible, and which is used is transparent to the device, via reader implementation choice. - CP-OFDM and DFT-S-ODFM can both be used for any value of M Or, we propose the following modification: Proposal 2.1.2a(I): Capture in the TR: - For OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible, and which one is used is transparent to the device, depending on the reader's implementation choice: - For M=1, i.e. use of OOK-1 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible - For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are feasible. |
| LGE | Okay with the proposal. |

## R2D modulation [ACTIVE]

Agreement RAN1#116 A-IoT DL study includes OOK from DL transmitter’s perspective. - For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869. - FFS value(s) of M. - FFS: Any changes needed from the definitions in TR 38.869. - FFS: Exact definition of chip - If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

Agreement RAN1#116 For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

Agreement RAN1#117 The following table is a starting point for the study of M values and the associated minimum Btx,R2D value - Reader can use any R2D transmission bandwidth >= minimum Btx,R2D - FFS: Impacts, if any, of CP handling solutions, and other impacts - Note: depending on further study, the maximum value of M may be less than 32 M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3.

### M values

From studying the papers, some companies further discuss down selection of M values from the agreed table. Some companies suggest to remove value of 32 in the table. Some companies think M values should be determined after CP handling mechanism is clear. Some companies think for large M values, the performance would be better with larger Btx,R2D than the Minimum Btx,R2D # of PRBs in the table. FL considers that these are now nicely discussing stage-3 details belonging to any potential normative phase.

Or if possible, we can try to address the followings.

#### Round 1

(1) Some company ask to remove 32 or down-select from {24, 32} in the table, thus FL would try to simply propose remove 32. Note that based on the previous calculation, the remaining values already cover the comparable peak data rate to other technologies.

(2) Some companies think the performance would be better with larger Btx,R2D than the Minimum Btx,R2D # of PRBs in the table. Thus FL would like to add a text proposal to clarify this.

Proposal 2.21(I): Agree the following text proposal for the TR:

#### 6.1.1.x R2D bandwidths …(unchanged parts omitted)… Table 6.1.1.x-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D. Table 6.1.1.x-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3. …(unchanged parts omitted)…

| Company | Views to the update on (1) and (2) |
| --- | --- |
| Qualcomm | We prefer to keep 32 in the study. Regarding down selection, we expect more discussions occur on potential down selections in future, not only for one of {24, 32}, but also for some other smaller values. So, we do not think update on (1) is necessary at this time. We are fine with (2). |
| OPPO2 | Fine, prefer to remove the first note since M=32 is removed |
| Spreadtrum | Fine with the new note and prefer to keep the first note. |
| Samsung | OK. We also consider if last row being removed, maybe it’s natural to also remove the first note. |
| LGE | We don’t support this proposal. We also think we need more discussion perhaps in WI phase on the down-selection from the starting point for M values. |

### Single / double sideband

#### Round 1

From studying the papers, several companies claim that it is up to implementation or unnecessary to discuss this for R2D. FL considers it seems no need to have proposal for R2D anymore or try to capture the following statement into TR.

Proposal 2.2.2(I): Capture the following observation in the TR:

For R2D, whether single sideband or double sideband modulation is used is up to reader implementation.

| TCL | Agree with this proposal. |
| --- | --- |
| Qualcomm | We think no need to have proposal for this. |
| Futurewei | The proposal is fine. |
| Xiaomi | We support this proposal. |
| Lenovo | OK |
| Spreadtrum | Support. |
| LGE | Okay with the proposal. |

## R2D line coding [ACTIVE]

Agreement RAN1#116 For R2D, line codes studied are: Manchester encoding and pulse-interval encoding (PIE). - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc. Agreement RAN1#117 The study assumes the following bit to chip mapping for Manchester encoding: - bit 0→chips{10}, bit 1→chips{01} - FFS: Variant of the above for CP handling

### Round 1

It seems clear that companies will not compromise to select Manchester encoding for R2D, although it is the significant majority support. FL observes that the discussions in papers is more about benefit/necessity than feasibility of the two studied R2D line codes, and thus thinks the best way forward is to capture the observations that have been made, and let any potential WI handle the usual business of down-selection.

Proposal 2.3(I): Capture the following observations in the TR for R2D line codes:

- It is reported by sources [Huawei], [CMCC], [EURECOM], [CATT], [ZTE], [Lekha], [Samsung], [ETRI], [Lenovo], [Apple], [LG] that PIE has a higher ratio of high to low voltage, which enables it to provide stable instantaneous RF energy to the device –

  - Sources [CMCC], [Samsung], [LG], [EURECOM] and [Lenovo] consider this a justification for studying PIE

  - Sources [Huawei], [CATT], [ZTE], [ETRI] and [Apple] acknowledge this aspect of PIE, but question its necessity.

- It is stated by sources [Nokia], [Ericsson], [Vivo], [CATT], [ZTE], [Apple], [DCM, [IITK, IITM] that, due to variable codeword lengths in PIE, scheduling time domain resources would be different to transmit the same amount of data due to the number of 0 and 1, as compared to the constant codeword lengths in Manchester codes.

- It is stated by sources [Huawei], [Spreadtrum], [CATT], [ZTE], [Lekha], [Apple] that, due to variable codeword lengths in PIE, the error detection of one information bit can result in error propagation for the detection of subsequent bits and therefore negatively affecting the detection of the overall data length, as compared to Manchester codes.

- According to sources [Nokia], [Ericsson], [CATT], [ZTE], [Lekha], [Samsung], [Apple], [DCM], transitions during Manchester code transmissions can be utilized for recovering the signal’s clock.

- Sources [Nokia], [Ericsson] say that the signal does not lose information or distort when fed to a DC blocker because each Manchester encoded symbol maintains a constant DC level, i.e. always the same average power, unlike if there was a long sequence of 1s or 0s.

- Evaluations are reported showing that Manchester codes perform better than PIE by 1dB to 6dB @10% BLER – [Huawei], [CMCC], [CATT], [ZTE], [IITk, IITM].

  - Source CMCC claims only a 1dB gain, stating that the gain is negligible since the downlink coverage bottleneck is the receiver sensitivity and not the decoding performance.

| Company | Views / anything missing? |
| --- | --- |
| TCL | In our understanding, R2D line coding should not precluded PIE due to it can be as RF energy for device’s energy harvesting even though its poor performance and short on self-clock compared with Manchester, which could reduce unavailable time for device. |
| Lenovo | Besides Manchester, we support PIE for R2D transmission. |
| Huawei, HiSilicon | We are fine to capture the observations for Manchester codes and PIE, although we do not support the use of PIE for R2D transmissions, due to the variable codeword lengths of PIE. |
| Ericsson | It is okay, but it would be good to also add: Source [Ericsson] says that variable codeword lengths in PIE complicates A-IoT resource management, especially in in-band deployments when a reader needs to multiplex both NR and A-IoT communication. |
| LGE | Okay with the proposal. |

### Round 2

The following proposal has an update where Ericsson’s observation has now been added.

Proposal 2.3(II): Capture the following observations in the TR for R2D line codes:

- It is reported by sources [Huawei], [CMCC], [EURECOM], [CATT], [ZTE], [Lekha], [Samsung], [ETRI], [Lenovo], [Apple], [LG] that PIE has a higher ratio of high to low voltage, which enables it to provide stable instantaneous RF energy to the device –

  - Sources [CMCC], [Samsung], [LG], [EURECOM] and [Lenovo] consider this a justification for studying PIE

  - Sources [Huawei], [CATT], [ZTE], [ETRI] and [Apple] acknowledge this aspect of PIE, but question its necessity.

- It is stated by sources [Nokia], [Ericsson], [Vivo], [CATT], [ZTE], [Apple], [DCM, [IITK, IITM] that, due to variable codeword lengths in PIE, scheduling time domain resources would be different to transmit the same amount of data due to the number of 0 and 1, as compared to the constant codeword lengths in Manchester codes.

- It is stated by sources [Huawei], [Spreadtrum], [CATT], [ZTE], [Lekha], [Apple] that, due to variable codeword lengths in PIE, the error detection of one information bit can result in error propagation for the detection of subsequent bits and therefore negatively affecting the detection of the overall data length, as compared to Manchester codes.

- Source [Ericsson] says that variable codeword lengths in PIE complicates A-IoT resource management, especially in in-band deployments when a reader needs to multiplex both NR and A-IoT communication.

- According to sources [Nokia], [Ericsson], [CATT], [ZTE], [Lekha], [Samsung], [Apple], [DCM], transitions during Manchester code transmissions can be utilized for recovering the signal’s clock.

- Sources [Nokia], [Ericsson] say that the signal does not lose information or distort when fed to a DC blocker because each Manchester encoded symbol maintains a constant DC level, i.e. always the same average power, unlike if there was a long sequence of 1s or 0s.

- Evaluations are reported showing that Manchester codes perform better than PIE by 1dB to 6dB @10% BLER – [Huawei], [CMCC], [CATT], [ZTE], [IITk, IITM].

  - Source CMCC claims only a 1dB gain, stating that the gain is negligible since the downlink coverage bottleneck is the receiver sensitivity and not the decoding performance.

| Company | Views / anything missing? |
| --- | --- |
| CMCC | Thanks for the proposal, and sorry that we haven’t got chance to response early. There are some statements saying issues of PIE with variable codeword lengths. We agree. But in our paper, it also provides some discussions and evaluations of equal length PIE codeword. Better to capture: - Source [CMCC] says that both equal and unequal codeword length in PIE can be considered from energy harvesting efficiency and link performance perspective, and equal codeword length can overcome the cons of variable codeword length in PIE. (unchanged part omitted) - Evaluations are reported showing that Manchester codes perform better than PIE by 1dB to 6dB @10% BLER – [Huawei], [CMCC], [CATT], [ZTE], [IITk, IITM]. - Source CMCC claims only a 1dB gain when comparing Manchester over equal length codeword in PIE, stating that the gain is negligible since the downlink coverage bottleneck is the receiver sensitivity and not the decoding performance. |

FL: I added the second part from CMCC. For the first part, as mentioned elsewhere, I am avoiding including every company solutions as assertions of truth, since that is rather blank cheque writing. However, I have taken in the performance counter-point of the second sentence. See section 5.3.

## R2D FEC / repetition [ACTIVE]

Agreement RAN1#116 Regarding FEC, R2D with no forward error-correction code (FEC) is studied as baseline. - Evaluations would be by comparison to this baseline Agreement RAN1#117 Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times

### Round 1

For R2D FEC, it has already been agreed that R2D FEC is not supported is the baseline. This should be enough to conclude the study from the FL’s point of view. But in case companies feel it is necessary to capture explanatory observations, FL makes the following proposals

Proposal 2.4.1a(I): For R2D FEC, the following observations are captured in TR 38.769:

- Justifications for not having R2D FEC beyond the baseline are given by sources [Huawei], [TCL], [Vivo], [ZTE], [Samsung],] and [Apple] :

  - Sources [Huawei],] and [ZTE] state that FEC decoders require complicated arithmetic or logical operations which are too complicated to be implemented in device 1.

  - Sources [ZTE],] and [Samsung] state that it would be difficult for a device to implement a FEC decoder due to its low power consumption.

  - Source [Huawei] state that FEC decoder procedures such as the de-interleaving operation or route metric caching require volatile memory of a certain size with a certain reading/writing throughput, which cannot be supported by device 1.

  - They also mention that the received signal power at the device can be relatively high (e.g., >-60 dBm), making the receiver sensitivity not the bottleneck of the link budget for target coverage, even for device 2b, thus questioning the necessity of R2D FEC.

- Justifications for using FEC in R2D for device 2b are given by sources [Ericsson], [Qualcomm] :

  - Source [Ericsson] claims that CC with small constraint lengths (e.g., 3 or less) offer a substantial performance gain over uncoded transmission, especially in a fading environment, with reasonable complexity. CC with explicit tail-biting transmission to aid decoding may be suitable for R2D.

  - Source [QC] claim that even simple block code (e.g., Golay, RM) with hard decisions can significantly reduce the required SNR for achieving a target BLER e.g., 1%.

| Company | Views / anything missing? |
| --- | --- |
| Xiaomi | We support the first sub-bullet part, for the harmonized design, the FEC is not applied to R2D transmission for all device types. |
| Huawei, HiSilicon | We do not support the use of FEC for R2D since the receiver sensitivity is not expected to be the bottleneck of the link budget for target coverage, even for device 2b. Although companies have reported performance gains, the complexity added due to including FEC is prohibitively large for device 1. |
| LGE | Agree with most of the sub-bullets under the first bullet. Okay to capture the observations in the proposal. |

For R2D repetitions, companies seem to be divided on whether to support it, and even if it is supported, views on which type of repetition to be used is also diverse. A few companies had mentioned that R2D repetitions can be used based on whether it is necessary depending on the coverage evaluations. The following proposals are presented to capture this study effort in the TR.

As in other places, the intention would be to capture both TPs – the split here is just for convenience of input.

Proposal 2.4.1b(I): For R2D repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Nokia], [Ericsson], [CMCC],] and [Vivo] that R2D repetitions should not be supported. Justifications are reported as follows:

  - Source [Nokia] mention that the transmission power of a R2D transmission is typically much greater than its corresponding D2R transmissions, and if the R2D transmission has coverage issues, then the corresponding D2R transmission would not reach the reader. Hence it should be considered for D2R transmissions alone.

  - Source [Ericsson] say that not supporting R2D repetition can be the baseline.

  - Source [CMCC] and [LG] and [Xiaomi] include that the decision to support R2D repetitions can be based on whether the activation threshold is a bottleneck according to the coverage evaluations.

  - Source [CMCC] also comment that from a device perspective, especially device 1 with low complexity and memory storage, it is not possible to combine multiple repetitions.

| Company | Views / anything missing? |
| --- | --- |
| Xiaomi | If we study the repetition of R2D, it is better to align the simulation assumption firstly, and then proves whether R2D is bottleneck channel and needs some coverage enhancements. |
| Huawei, HiSilicon | We are fine to capture these observations for why repetitions should not be supported. |
| LGE | Okay with the proposal. |

Proposal 2.4.1c(I): For R2D repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [LG], [ETRI], [QC] and [IITK, IITM] and [Lenovo] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported:

  - Bit-level repetition

Positive observations

    - Source [Ericsson] state that bit level repetition can be studied if coverage enhancement of the R2D link is required.

    - Source [OPPO] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power.

    - Source [QC] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices.

Negative observations

    - Source [CMCC] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception.

    - Source [Vivo] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination.

  - Block-level repetition

Positive observations

    - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively increases the probability that at least one of the repetitions will be received with sufficient quality to correctly decode the information gain.

Negative observations

    - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded.

  - Chip-level repetition

Positive observations

    - Source [CMCC] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps.

    - Source [IITK, IITM] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions.

Negative observations

    - Sources [Ericsson] and [CATT] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option.

| Company | Views / anything missing? |
| --- | --- |
| TCL | In our understanding, R2D repetition is unnecessary due to it will increase the reception and processing complexity. |
| Xiaomi | Similar view with the Proposal 2.4.1b. |
| Lenovo | We also support R2D repetitions: - It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [LG], [ETRI], [QC] ,[IITK, IITM] and [Lenovo] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported: |
| Huawei, HiSilicon | We do not support the use of repetitions for R2D, even for device 2b since there is no need to enhance the coverage level since the different chip lengths for R2D can support the corresponding coverage levels. We are fine to capture the observations for the proposed different types of R2D repetition, including why these repetitions should not be supported. |
| ZTE, Sanechips | Some update to make our comments clearer. - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain. |
| LGE | Okay to capture pros and cons of R2D repetitions. Final decision on the support of R2D repetitions can be made based on whether the R2D activation threshold is a bottleneck or not after the analysis of the coverage evaluation results. |

### Round 2

Proposal 2.4.1a, and Proposal 2.4.1b need no changes.

The following proposal has an update based on the comment from ZTE correcting their observation. Should hence be stable.

Proposal 2.4.1c(II): For R2D repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [LG], [ETRI], [QC] and [IITK, IITM] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported:

  - Bit-level repetition

Positive observations

    - Source [Ericsson] state that bit level repetition can be studied if coverage enhancement of the R2D link is required.

    - Source [OPPO] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power.

    - Source [QC] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices.

Negative observations

    - Source [CMCC] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception.

    - Source [Vivo] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination.

  - Block-level repetition

Positive observations

    - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain.

Negative observations

    - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded.

  - Chip-level repetition

Positive observations

    - Source [CMCC] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps.

    - Source [IITK, IITM] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions.

Negative observations

    - Sources [Ericsson] and [CATT] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option.

| Company | Views / anything missing? |
| --- | --- |
| vivo | Please add [vivo] for following bullet as blew: - Sources [Ericsson] and [CATT] and [vivo] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option. |

## R2D and D2R CRC [VOID]

See Section 4.

## R2D multiple access [ACTIVE]

Agreement From RAN1 perspective, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used. Agreement For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied.

### Round 1

Since it is clear from RAN1 and RAN2 agreements that TDMA is feasible and supported, FL now intends to focus on FDMA and CDMA, by collecting into the TR what companies have reported regarding feasibility and applicability to devices.

The following two templates are hence proposed, with an initial content based on tdocs submitted to RAN1#117. Companies are invited to update their input for finalizing the remaining statements in November.

Proposal 2.6.1(I): Capture in the TR that for R2D multiple access:

- Time-domain multiplexing is the baseline.

- Code-domain multiplexing is not considered for all devices.

- Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1.

Table 6.1.1.x-1: Observations on the feasibility and necessity of FDMA for Device 2b with IF-ED or ZIF receiver

| Aspects to be considered for feasibility/benefit | Observations |
| --- | --- |
| Inventory completion time | Sources [Ericsson, LGE, Nokia, OPPO] state that FDMA is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. |
| Device implementation | Sources [Apple, DOCOMO, Ericsson, LGE, OPPO] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Sources [ITL, MTK, Spreadtrum, TCL, Huawei] thinks that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Sources [Ericsson] state that the larger R2D responses are harder for the devices to process in the case of TDMA+FDMA/TDMA only for D2R/R2D, respectively. |
| Spectrum utilization | Sources [ITL] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. |
| Coverage (in the case of single reader) | Sources [Huawei] state the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Sources [Ericsson] think the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture). |
| Reader implementation (in the case of single reader) | Sources [Huawei] state that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions. |
| Harmonized design for all devices | Sources [ASUSTek, CMCC, ETRI] state that it is not appropriate to include FDMA only for Device 2b, while Device 1 and 2a cannot support it. |

| Company | Anything missing? |
| --- | --- |
| Qualcomm | The proposal describes “FDMA is not considered for the devices with RF-ED receiver, and studied for devices with IF/ZIF receiver considering the technical aspects in Table 6.1.1x-1”. However, the Table contains some aspects for devices with RF-ED receiver. We suggest to delete following from the 2nd row that is the aspect specifically for RF-ED receiver. Sources [ITL, MTK, Spreadtrum, TCL, Huawei] thinks that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. |
| Xiaomi | We support this proposal. |
| ETRI | Support the proposal. |
| LGE | Okay with the proposal in principle. We think most of the arguments are also valid for Device 2a with RF-ED Prefer to add device 2a with IF-ED or ZIF receiver architectures. So we prefer to change the “Device 2b with IF-ED or ZIF receiver” to either “Device 2a/2b with IF-ED or ZIF receiver” or simply “devices with IF-ED or ZIF receiver”. |

## R2D time-domain definitions

### Time unit(s) [ACTIVE]

Agreement RAN1#116bis

#### Round 1

Based on the tdocs submitted to RAN1#118 and the discussions during the meeting, the following proposal should be stable.

Proposal 2.7.2a(I): In R2D, a chip corresponds to one modulated symbol, e.g. according to agreed OOK modulation.

| Company | Views |
| --- | --- |
| Qualcomm | We are OK with the proposal. |
| OPPO2 | A chip or chip duration may depend on whether the CP can be seems as part of the chip. Furthermore, we think the definition of chip or chip duration should be from device’s point of view. In that case, this proposal can be postponed after CP handling mechanisms is clear. |
| Xiaomi | We support this proposal. |
| Spreadtrum | Support. |
| Samsung | We are OK with the principle, but consider at least for R2D, it may be misleading to modulated OFDM symbol. Maybe we can update the proposal as …corresponds to one OOK-modulated symbol |
| Huawei, HiSilicon | Agree. |
| NEC | Ok. |
| Ericsson | Ok |
| LGE | Okay with the proposal. |

There are different views on the determination of the R2D chip duration, which are summarized in the following proposal. FL considers at this stage, this question has the characteristic of a specification-level detail that is coupled to aspects such as the OFDM waveform generation corresponding to the assumption on the CP handling method. Companies have said similar in their comments, in recent meetings. Thus, FL suggests capturing the different methods for clarity in the TR, and considering subsequently if it is useful to couple the definitions with e.g. CP handling method(s), or leave that to normative decisions.

Proposal 2.7.2b(I): Capture in the TR that the following chip duration calculations have been studied, and FFS: whether/how to couple them with CP handling method(s):

- Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. [Ericsson][Huawei][IDCC][Sharp][Spreadtrum][vivo] ][Futurewei] ][LGE]

- Calculation 2: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is floating with e.g. T0-∆T0≤Tchip≤T0+∆T0, and FFS ∆T0T0. [CMCC]

- Calculation 3: Chip duration = (1/M) × {OFDM symbol duration including CP part}. [Xiaomi]

- Calculation 4: Chip duration = (1/M) × {OFDM symbol duration excluding CP part} for CP handling method type 1, while Chip duration = (1/M) × {OFDM symbol duration including CP part} with different M values in the odd and even OFDM symbols for CP handling method type 2. [DOCOMO]

- Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation [ZTE]

  - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length;

  - for other cases,

    - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part};

    - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64.

| Company | Anything missing? |
| --- | --- |
| Qualcomm | We don’t think the proposal is necessary. Overall the calculations highly depends on the decision of CP handling. Once CP handling is fixed, everything is clear. |
| OPPO2 | Same comment as Proposal 2.7.2a(I). |
| Futurewei | Adding “Futurewei” as co-source to Calculation 1: - Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. [Ericsson][Huawei][IDCC][Sharp][Spreadtrum][vivo][Futurewei] |
| Xiaomi | We support this proposal. |
| Samsung | We wonder if the calculations are a list of candidate solutions for future down-selection. If not, we consider it’s too detailed to be captured in TR, especially considering the method for CP handling is not decided yet. |
| Huawei, HiSilicon | We generally agree the proposal. The chip duration calculation is related to the details of CP handling, which is usually to be completed in the normative works. In this case, it is OK to summarize and capture the potential calculation methods from companies, which correspond to different CP handling methods. And we think Calculation 1 should be the correct way for determining the chip duration. |
| ZTE, Sanechips | Comments on the following solution: if the M value can not be divided by the IFFT size(which is typically 2048 in NR with SCS 15KHz), how to maintain the all the chip durations are constant within one OFDM symbol? Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. |
| LGE | Please add LGE under the first bullet on Calculation 1. - Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. [Ericsson][Huawei][IDCC][Sharp][Spreadtrum][vivo][LGE] |

#### Round 2

Proposal 2.7.2a(II): In R2D, a chip corresponds to one OOK modulated symbol, e.g. according to agreed OOK modulation.

- Note: Duration of a chip after OOK-1/OOK-4 modulation is the next proposal.

| Company | Any concerns? |
| --- | --- |
| vivo | OK |

Question 2.7.2b(II): According to the offline discussion, FL attempts requesting the mapping from ‘Calculation #’ in round 1 and CP handling method in this table, and FL does not intend to try down-selections among the permutations until CP handling method is decided in a later phase.

- M1-1, M1-2

- M2-1-1, M2-1-2, M2-2

| Company | Calculation 1 applicable to CP methods | Calculation 2 applicable to CP methods | Calculation 3 applicable to CP methods | Calculation 4 applicable to CP methods | Calculation 5 applicable to CP methods |
| --- | --- | --- | --- | --- | --- |
| DOCOMO | M1-1, M1-2 |  | M2-1 |  |  |
| vivo | M1-1, M1-2 |  |  |  |  |
| Huawei, HiSilicon | M1-1, M1-2, M2-2 | M2-1-2 for small M, M1-1 for large M | M2-1-1, M2-1-2, M2-2 |  | M2-1-2 |
| ZTE, Sanechips |  |  |  |  | M1-1, M1-2, M2-1-1,M2-1-2 M2-2 |

| Company | Anything missing? |
| --- | --- |
| DOCOMO | Calculation 4 has already defined for each CP handling method, therefore it can be removed from the above table, or we suggest calculation 4 as baseline. |
| Huawei, HiSilicon | “Calculation 4” proposes different calculations for different CP-handling methods, while the proposed calculations have been captured in the other columns of the table. From the perspective of mapping between the chip duration calculations and the CP handling methods, “Calculation 4” can be removed from the table. |

## R2D bandwidths [INACTIVE]

Agreement RAN1#116 At least the following bandwidths for R2D are defined for the purpose of the study: - Transmission bandwidth, Btx,R2D from a Reader perspective: The frequency resources used for transmitting R2D - Occupied bandwidth, Bocc,R2D from a Reader perspective: The frequency resources used for transmitting R2D, and potential guard band - Bocc,R2D ≥ Btx,R2D - FFS: Further constraint(s) e.g. Bocc,R2D = Btx,R2D. - Possible values of each bandwidth are FFS

Agreement RAN1#116bis For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

For Btx, R2D, see section 2.2.1.

For Bocc,R2D, or potential Bsys,R2D, the definition in the earlier agreements are sufficient for study purposes and are already captured in the TR. Hence FL sees no need for further proposals.

# D2R

## D2R waveform [ACTIVE]

### Round 1

Previous meetings had some uncertainty about the purpose of 9.4.2.4 vs this agenda item.

- The 9.4.2.4 designs are only for the RF carrier wave.

- This 9.4.2.1 agenda will, either implicitly or explicitly, define the baseband signal that is generated to modulate the RF CW.

Based on inputs in papers, the discussion here may have become unnecessary after a few meetings of agreements, and hence FL simplifies for one attempt. If this is considered not necessary or difficult, we can continue without more time here, since the inevitable outcome of the baseband + RF steps will happen regardless. The basic point that companies seem to make is that the baseband waveform should be within the ability of all the devices, i.e. that is it not OFDM-based. Some become more specific and indicate it should be single-carrier (whereas OFDM is multi-carrier), but FL recalls ‘confusion’ from some companies about the difference between single-carrier (applicable to BB waveform) and single-tone (applicable to RF CW).

Proposal 3.1a(I): The D2R baseband waveform (as distinct from the internal or external carrier wave) is non-OFDM.

| Company | Views |
| --- | --- |
| TCL | Okay with this proposal. |
| Qualcomm | We are OK with or without the proposal. Agree with the FL that this may have become unnecessary. |
| Futurewei | Support |
| Xiaomi | We support the proposal. |
| Lenovo | OK |
| Spreadtrum | Support. |
| ETRI | Support |
| Ericsson | Ok |
| LGE | Okay with the proposal. |

## D2R modulation [ACTIVE]

### Modulation scheme(s)

Agreement - OOK and Binary PSK for baseband modulation are feasible for D2R for all devices. - The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK) - FFS: whether MSK is feasible for all devices

#### Round 1

Interested companies have conducted their studies on MSK, and it covers points such as how to define it, and whether it is feasible for all, some, or none of the devices 1/2a/2b. Similar as other parts, FL suggests recording this study in the TR as it stands.

Proposal 3.2.1a(I): Capture the following observation in the TR:

- Sources [Ericsson][CATT][Qualcomm][Lekha] report that MSK is feasible in some way for at least some devices:

  - [Ericsson][CATT] say it is feasible for all devices

  - [Qualcomm] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b

- Sources [Huawei][TCL][CMCC][CTC][vivo][Xiaomi][Comba] report that MSK is either infeasible or should be deprioritized for all devices.

| Company | Views / anything missing? |
| --- | --- |
| TCL | Not support MSK due to phase consistence is hard to ensure for all devices. |
| Qualcomm | We are OK with the proposal. |
| Xiaomi | We support the second sub-bullet part, for the first sub-bullet , we think “ON-OFF” waveform for device 1 and device 2a can not maintain the phase continuous characteristics of MSK. |

### Single / double sideband

#### Round 1

From studying the papers, many companies propose to take FL’s proposal in last meetings that 2SB can be supported for all devices while 1SB can be for further study. One company think 1SB can be considered for device 2b. Thus, FL would like to keep same proposal here.

Proposal 3.2.2(I): 2SB modulation is supported for D2R transmission for all devices.

- FFS if 1SB can be supported by all, or any, devices, taking account of other issue such as how to achieve small frequency shift

| Company | Views |
| --- | --- |
| TCL | 1SB could be considered only for device 2a and 2b because of its implementation complexity. Agree consider how to achieve SFS. |
| Qualcomm | We are OK with the proposal. |
| Futurewei | 2SB is the default for D2R; support |
| Xiaomi | We support the proposal. 1SB is too complex for ambient IoT devices, there need additional block to suppress 1SB on device. |
| Spreadtrum | Support. |
| Ericsson | Ok |
| LGE | Okay with the proposal. |

## D2R line coding [ACTIVE]

Agreement For D2R, study: Manchester encoding, FM0 encoding, Miller encoding, no line coding. - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: How to achieve small frequency shift in baseband and/or FDM(A) among devices - Aspects to study include: - Spectrum shape - Complexity - Power consumption - BER, BLER - Resilience to SFO - If there is any relation to CFO Agreement RAN1#117 The study assumes the following bit to chip mapping for Manchester encoding: - bit 0→chips{10}, bit 1→chips{01} - FFS: Variant of the above for CP handling Agreement RAN1#118 For D2R line codes, the study assumes the following codewords corresponding to an information bit 0 or bit 1, before considering potential small frequency-shifting: - For FM0: - According to Figures 6-8 and 6-9 of UHF RFID standard - For Miller: - According to Figure 6-12 of UHF RFID standard.

### Line code types

#### Round 1

Similar as with R2D, FL observes that the discussions in papers is more about benefit/necessity than feasibility of the studied D2R line codes, and thus thinks the best way forward is to capture the observations that have been made, and let any potential WI handle the usual business of down-selection.

Proposal 3.3.1(I): For D2R line codes, the following observations are captured in TR 38.769:

- Sources [Huawei], [CMCC], [Vivo], [CATT], [ZTE], [Samsung] report that Manchester codes perform better than Miller codes by 0.5dB to 5.5dB @10% BLER.

- Sources [Huawei], [CMCC] [ZTE] report that Manchester codes perform better than FM0 by 0.2dB to 2.5dB @10% BLER.

  - According to sources [Vivo], [CATT],Manchester codes have a similar performance as FM0.

- Sources [Huawei], [Lekha] state that the power of Miller coding is more concentrated in the low-frequency region which is adjacent to the carrier wave frequency, as compared with FM0 code and Manchester code which leads to worse interference handling capability for Miller codes.

- Source [QC] state that performance of BPSK square wave is better than Miller/FM0 by ~3-4 dB @10%BLER with coherent demodulation.

| Company | Anything missing? |
| --- | --- |
| Qualcomm | We are OK with the proposal in principle. |
| Huawei, HiSilicon | We are fine with the observations captured for D2R line codes. |
| LGE | Okay to capture observations in the proposal. |

### Small frequency shift [ACTIVE]

#### Round 1

Agreement Small frequency shifts for D2R are studied for OOK and BPSK: - For applying with Manchester line codes - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. - Companies to report how they perform multiplying for option 2 - For applying with Miller line codes, according to Figure 6-13 of UHF RFID standard. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported - Note: small frequency shifts for D2R are studied for the same potential purposes for relevant identified BFSK variant(s)

FL observes that there are a fairly equal number of companies that have indicated support for Option 1 and Option 2 under Manchester line codes. Thus, instead of attempting to down-select, the FL focuses on the FFS and company reports for how the repetition is defined and how the multiplication operation is defined for each of the options respectively.

For the use of SFS in describing D2R bandwidths, please refer to Section 3.8.1, where the xth amount of small frequency shift is used as FSx. When writing the TR, the editor will ensure suitable arrangement of the text reflecting each agreement to produce a clear TR.

Proposal 3.3.2a(I): For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

| Company | Views |
| --- | --- |
| Qualcomm | Our proposal, each bit modulates a square wave for small frequency shift, is identical to Manchester line code Option 1. Therefore, we suggest to update as follows. For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length). - Source QC report that without line coding, each bit modulating a phase of the square wave for the small frequency shift for device 1/2a can achieve the same waveform outcome as Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1). |
| IDCC | These observations were in our contribution. Observation 1: Repetition of the Manchester codewords within the same time duration corresponding to an information bit is equivalent to multiplying the NRZ codeword with a square wave corresponding to the small frequency-shift. Observation 2: At least for OOK/BPSK, square wave modulation is equivalent to multiplying the NRZ codeword with a square wave corresponding to the small frequency-shift. Furthermore, at least for OOK/BPSK, this method is also equivalent to the square wave modulation technique. |
| Futurewei | For clarity, suggest to include “R ≥ 1” in the above proposal. |
| Xiaomi | We support the proposal. |
| Spreadtrum | Support. |
| Huawei, HiSilicon | We are supportive of the definition of the Manchester codeword repetition number and how the repetition is performed for Option 1. |
| LGE | We have similar observations with Qualcomm and IDCC. |

Proposal 3.3.2b(I): For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options:

- Sources [Futurewei], [Vivo], [CMCC] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

- Source QC report that the multiplication operation is by each bit modulating a square wave for the small frequency shift for device 1/2a and where each bit can modulate the carrier wave for device 2b.

| Company | Views |
| --- | --- |
| Qualcomm | Our proposal is rather similar to/same as Option 1. Suggest to delete the 2nd bullet. For Option 2, our understanding is same as the first bullet, i.e., the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options: - Sources [Futurewei], [Vivo], [CMCC] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. - Source QC report that the multiplication operation is by each bit modulating a square wave for the small frequency shift for device 1/2a and where each bit can modulate the carrier wave for device 2b. |
| IDCC | We think XOR or XNOR can be used and we fist have to clarify the definitions. Our understanding is that RFID Miller uses XNOR. The following was in our contribution.: Multiplication can be achieved by one of the two following options: - Assuming the low signal is denoted by “-1”, then multiplication is a scalar multiplication. - Assuming the low signal is denoted by “0”, then multiplication can be performed using the XNOR operation. One can easily see from the XNOR operation shown in Table 2 that, replacing “0” with “-1” and using scalar multiplication instead of the XNOR operation, the same result as in (i) can be achieved. Table 2 XNOR operation Manchester chip value; Subcarrier chip value; XNOR operation. 0; 0; 1. 0; 1; 0. 1; 0; 0. 1; 1; 1. |
| Futurewei | support |
| Xiaomi | We support the proposal. |
| Spreadtrum | Support. |
| Huawei, HiSilicon | We are supportive of capturing company reports on the multiplication operation for Option 2. |
| ZTE, Sanechips | - Some update on the observation: - Sources [Futurewei], [Vivo], [CMCC], [ZTE] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. Sources [ZTE] report that the multiplication operation can also be an XNOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. - More modifications on option 2 are as followings: For small frequency shifts in D2R using Manchester line codes Option2, the time duration of an information bit is Tb and the number of square wave within a codeword is M=Tb/(2*chip length), such that the amount of small frequency shift in Hz is M/Tb = 1/(2*chip length). |
| LGE | We have the following in our contribution, which is basically the same as option i) of IDCC above. “Regarding how to perform multiplication for option 2, 1s of the Manchester line codes are mapped to +1 and 0s are mapped to -1, and then the mapped {+1, -1} sequence is multiplied by a {+1, -1} square wave. The multiplied sequence is mapped to OOK or BPSK modulator.” We can also agree on the first bullet of the proposal or the option ii) of IDCC above. |

Since there were quite a lot of assorted views in terms of details regarding how the different line codes achieve the small frequency shift, the FL has captured the following observations made by the companies.

Proposal 3.3.2c(I): For small frequency shifts in D2R, the following observations are captured in TR 38.769:

- Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is similar to bit-level repetitions within the same duration prior to Manchester encoding.

- Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a flipping of bits in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration.

- Sources [Futurewei], [Ericsson] [CMCC] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is similar to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit.

| Company | Anything missing? |
| --- | --- |
| Huawei, HiSilicon | We are fine to capture these observations. |
| ZTE, Sanechips | We suggest to add the following observation: Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worse performance compared with small frequency shift (Option 1) and small frequency shift (Option 2). |
| LGE | Okay to capture the observations in the proposal. We can agree on all of the observations. |
| Qualcomm (round 2) | On Proposal 3.3.2c(II), suggest the following updates. - Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using BPSK modulation of a square-wave corresponding to the small frequency-shift without and no line codes is same as similar to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit. By the way, we put our input to v300 with “Qualcomm (round 2)”. Please use this for searching them. |

Proposal 3.3.2d(I): Regarding small frequency shift factor, capture in the TR that:

- For study purposes, the value of small-frequency-shift factor has been discussed as depending on details of design, which according to proposals considered could be e.g.

  - The repetition number of Manchester code

  - The M value of Miller in Figure 6-12 of UHF RFID standard

  - The number of square-wave periods

  - FFS FM0, if it supports small frequency shift

  - Note: the value of small frequency shift equals 1 when there is no small frequency shift

| Company | Views / Is anything missing? |
| --- | --- |
| Qualcomm | Suggest to delete 4th bullet. It was agreed not to define small frequency shift for FM0. - FFS FM0, if it supports small frequency shift |
| Futurewei | Should the first sub-bullet read as follows - The repetition number of Manchester codeword |
| Xiaomi | We support the proposal. |
| Huawei, HiSilicon | We are fine to capture the detailed design aspects that impact the value of the small frequency shift. We are also fine to remove the FFS on FM0 since we agreed that small frequency shift cannot be defined for FM0. |
| ZTE, Sanechips | We think other factors need to be considered are SFO range, device clock sample rate, some suggested update can be found as below - For study purposes, the value of small-frequency-shift factor has been discussed as depending on details of design, which according to proposals considered could be e.g. - The repetition number of Manchester code - The M value of Miller in Figure 6-12 of UHF RFID standard - The number of square-wave periods within a codeword - SFO assumption - device clock sample rate - FFS FM0, if it supports small frequency shift - Note: the value of small frequency shift equals 1 when there is no small frequency shift |
| LGE | Agree to delete the 4th sub-bullet. On the 3rd sub-bullet, is it the number of square-wave period in a bit duration? If so, it seems it is basically the same as the 2nd sub-bullet. We are not sure if the Note on the 5th sub-bullet works for the no line code case. Anyway we think the 5th sub-bullet on the Note is NOT needed. |

#### Round 2

For

- Proposal 3.3.2a:

  - Can add Qualcomm as supporting company under Proposal 3.3.2c, since the requested text is at odds with the proposal being about the line code case. 3.3.2c was added specifically for the Qualcomm proposal based on the specific discussion last meeting.

  - IDCC: Those NRZ definitions are not included in the study.

  - Futurewei: Cannot see how a repetition number can be negative or zero.

  - Hence no change, continue in Round 1

Proposal 3.3.2b

- Added QC to the XOR list and deleted the requested part.

- IDCC: Basic work on XOR and XNOR I think we don’t have time; 3GPP doesn’t need to write textbook stuff. I will add the text below and ZTE.

- I don’t understand the intent of the new text from ZTE. Seems completely different to the start proposal.

Proposal 3.3.2b(II): For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options:

- Sources [Futurewei], [Vivo], [CMCC], [QC], [LG] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

- Source QC report that the multiplication operation is by each bit modulating a square wave for the small frequency shift for device 1/2a and where each bit can modulate the carrier wave for device 2b.

- Source [InterDigital], [ZTE], [LG] report that the multiplication operation is an XNOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

| Company | Views / Is anything missing? |
| --- | --- |
| FL | No need to comment if still ok. |
| LGE | We have the following in our contribution, which is basically the same as option i) of IDCC above. “Regarding how to perform multiplication for option 2, 1s of the Manchester line codes are mapped to +1 and 0s are mapped to -1, and then the mapped {+1, -1} sequence is multiplied by a {+1, -1} square wave. The multiplied sequence is mapped to OOK or BPSK modulator.” We can also agree on the two bullets of the Proposal 3.3.2b(II). |
| ZTE,Sanechips2 | Suggest to add our views on the following as well - Sources [Futurewei], [Vivo], [CMCC], [QC], [LG],[ZTE] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. |
| Spreadtrum | Adding “Spreadtrum” as co-source to 3.3.2b(II): - Sources [Futurewei], [Vivo], [CMCC], [QC], [LG], [Spreadtrum] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. |

Proposal 3.3.2c: See round 1, added QC and IDC per their comments. Not copied here to save space, please continue in round 1 part. ZTE observation added

Proposal 3.3.2c(II): For small frequency shifts in D2R, the following observations are captured in TR 38.769:

- Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is similar to bit-level repetitions within the same duration prior to Manchester encoding.

- Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a flipping of bits in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration.

- Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is similar to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit.

- Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worse performance compared with small frequency shift (Option 1) and small frequency shift (Option 2).

Proposal 3.3.2d:

Updated per the Qualcomm and Futurewei corrections (thanks!). No need to comment if still ok.

The ZTE addition seems unexpected. Is it arguing against the use of small frequency shift? This proposal is simply saying how a SFS factor might be calculated.

Proposal 3.3.2d(II): Regarding small frequency shift factor, capture in the TR that:

- For study purposes, the value of small-frequency-shift factor has been discussed as depending on details of design, which according to proposals considered could be e.g.

  - The repetition number of Manchester codeword

  - The M value of Miller in Figure 6-12 of UHF RFID standard

  - The number of square-wave periods

  - FFS FM0, if it supports small frequency shift

  - Note: the value of small frequency shift factor equals 1 when there is no small frequency shift

| Company | Views / Is anything missing? |
| --- | --- |
| FL | No need to comment if still ok. |
| LGE | Agree to delete the 4th sub-bullet. On the 3rd sub-bullet, is it the number of square-wave period in a bit duration? If so, it seems it is basically the same as the 2nd sub-bullet. We are not sure if the Note on the 5th sub-bullet works for the no line code case. Anyway we think the 5th sub-bullet on the Note is NOT needed. |
| vivo | Just to clarify, small frequency shift factor is just a terminology used for study, not intended to say that we need this parameter finally in the spec. |
| ZTE,Sanechips2 | Suggest the following update to make the main bullet more readable. - For study purposes, the value of small-frequency-shift factor has been discussed as dependsing on details of design, which according to proposals considered could be e.g. - The repetition number of Manchester codeword - The M value of Miller in Figure 6-12 of UHF RFID standard - The number of square-wave periods - FFS FM0, if it supports small frequency shift Note: the value of small frequency shift factor equals 1 when there is no small |

To the LG comments on Proposal 3.3.2d:

- On 3rd bullet: Each bullet is for a different way of line code or SFS, the 3rd bullet is the cases for the Manchester coding option 2 and the no line code case.

- On 5th bullet note: This should have referred to “small frequency shift factor”, which I think should answer your concern.

For convenience, I have updated in-place in round 2. Please see above.

FL after round 2, I will bring these to online with updates shown in section 5:

3.3.2a: No further changes requested.

3.3.2b: Added company support names.

3.3.2c: No further changes requested.

3.3.2d: I will try the ZTE simplification.

## D2R FEC / repetition [ACTIVE]

Agreement RAN1#116bis A-IoT D2R study of FEC includes at least convolutional codes. - Comparisons are encouraged to compare to the case of no FEC - FFS details of convolutional codes, such as polynomial(s), shift-register termination, etc. - FFS if other FEC candidates/methods will be studied. Agreement RAN1#116bis Study D2R transmission in the physical layer using repetition Note: Discussions regarding higher-layer repetitions are up to RAN2

Agreement RAN1#116bis From 9.4.2.3: For PDRCH generation at the device, at least following blocks are studied as the baseline: - CRC bits are appended if there is non-zero length CRC - Note: CRC details discussed in agenda item 9.4.2.1 - Coding - Exact coding methods within the coding block, e.g. with/without line coding and/or FEC discussed under agenda 9.4.2.1 - Note: If no line coding is used, there may be an additional block (e.g. square wave generator) before/after modulation block - Modulation - Note: Other blocks could be added if agreed PDRCH generation Agreement RAN1#117 Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times Agreement RAN1#117 For D2R, study at least block-level and bit-level repetition type 1 and type 2. Agreement RAN1#118 For D2R FEC study, the LTE convolutional code polynomials are a reference. Other designs can be studied subject to: - Constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study - Mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study - FFS other details, e.g. final code rate by puncturing, shift-register initialization/termination.

### Repetition

#### Round 1

Overall, it seems that majority of the companies support the use of block level repetitions, although a few companies have indicated support for bit level repetitions, but they have not provided supporting arguments for it.

FL has split these proposals for convenience of input. The intention would be to capture both parts.

Proposal 3.4.1a(I): For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Huawei], [Nokia], [Ericsson], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] and [DCM] that block level repetitions should be supported.

General observations

  - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition

  - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block.

  - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission.

  - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum.

  - Source [Vivo] state that it may require larger buffer than bit level repetition at device side, depending on device implementation.

Performance comparisons

  - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding.

  - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1.

  - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions.

  - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping.

  - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

| Company | Is anything missing? |
| --- | --- |
| Xiaomi | We support the proposal. |
| Huawei, HiSilicon | We are fine to capture the observations on block level repetitions and its performance w.r.t other types of repetitions. |
| ZTE, Sanechips | - The following update is suggested in the following bullet. - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block if the TBS is known by the reader. - Clarification is appreciated why bit level repetition will degrade the power? Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission. - We also provide this view in our Tdoc. The following update is suggested. - Source [Vivo],[ZTE] state that it may require larger buffer than bit level repetition at device side, depending on device implementation. - The following update is suggested per our evaluation results. - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions, the performance between block level repetition and bit level repetition type 2 is the same. |
| Ericsson | We have so far only said that it should be studied, so please remove [Ericsson] from the list of companies. |
| LGE | Okay to capture the observations in the proposal. |
| DOCOMO | Same as Ericsson. |

Proposal 3.4.1b(I): For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Ericsson], [ZTE],] and [Samsung] that bit level repetitions should be supported.

  - Source [Huawei] state that for bit level type 1, the repetition data bit is decoded after FEC detection, and hence, the digital data bit cannot be combined with combining gain.

  - Source [ZTE] state that the performance of bit level type 1 repetition is degraded by ~3.5dB @ 10% BLER under the cases of AWGN and TDL-A channel when compared to bit level type 2.

  - Source [Huawei] state that since the bit level type 2 repetition is performed after FEC at the transmitter, at the receiver side, each bit would have a log-likelihood ratio (LLR) that can be combined and provided as the input for FEC decoding, thereby providing a better performance as compared to bit level type 1.

| Company | Is anything missing? |
| --- | --- |
| Huawei, HiSilicon | We are fine to capture the observations on bit level repetitions and its performance w.r.t other types of repetitions. |
| ZTE, Sanechips | In our views, the following modifications on TP are proposed: Source [ZTE] state that the performance of bit level type 1 repetition is degraded by ~3.5dB @ 10% BLER under the cases of AWGN and TDL-A channel when compared to bit level type 2. If TBCC is used, bit level type 2 repetition can be combined with the TBCC encoding process, where for each information bit input to TBCC encoding, the output is n bits, bit repetition is performed in units of n bits. Device does not need to buffer the entire TBCC coded block or to re-encode TBCC. |
| Ericsson | We have so far only said that it should be studied, so please remove [Ericsson] from the list of companies. |
| LGE | Okay to capture the observations in the proposal. |
| vivo | As analyzed in our paper, there is no performance gain of bit level repetition over block level repetition, it is duplicated with small frequency shift schemes, and it has different impact on signal spectrum. Therefore, we suggest to add following: - Source [vivo] states there is duplication of small frequency shift options and bit-level repetition, and observes different impact of signal spectrum for different line coding schemes. |

#### Round 2

For 3.4.1a, added where ZTE want their observations captured, but I don’t think we should have a process of each company trying to challenge every other, as that would rather defeat the point of capturing the study from papers.

Proposal 3.4.1a(I): For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Huawei], [Nokia], [Ericsson], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] and [DCM] that block level repetitions should be supported.

General observations

  - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition

  - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block.

  - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission.

  - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum.

  - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation.

Performance comparisons

  - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding.

  - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1.

  - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same.

  - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping.

  - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

For 3.4.1b, no change. The proposal already captures the ZTE results at the same level as other companies.

### FEC

#### Round 1

FL proposes now to collect the study effort on convolutional codes for D2R, via the following table, which summarizes what is understood from company submissions to RAN1#118 and RAN1#118bis. Comments are in the feedback box below the following table.

Proposal 3.4.2a(I): For D2R FEC, include the following table in the TR:

| Option # | CC Design | Pros | Cons |
| --- | --- | --- | --- |
| Baseline | Constraint length 7 Code rate 1/3 | - [Vivo] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. - [Vivo] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. - [CATT] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. - [Lekha] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. - [Xiaomi] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 - [Xiaomi] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 |  |
| 1 | Constraint length 4 Code rate 1/2 – 1/4 | - [Ericsson] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes. | - [CMCC] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. |
| 2 | Constraint length 6 Code rate 1/2 – 1/6 | - [QC] Reduced device complexity due to smaller number of shift registers. | - [QC] Code rate 1/2: Decoding performance is decreased by 0.6dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [QC] Code rate 1/3: Decoding performance is decreased by 0.4dB@1% BLER, when compared to baseline. - [QC] Code rate 1/4: Decoding performance is decreased by 0.2dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. - [QC] Code rate 1/6: Decoding performance is decreased by 0.15dB@1% BLER, when compared to baseline. |
| 3 | Constraint length 7 Code rate 1/2 – 1/4 | - [CATT] Code rate 1/2, TBCC: Decoding performance is increased by ~9dB@10% BLER, when compared to no CC but with Manchester line codes. - [CATT] Code rate 1/4: Decoding performance is increased by ~1.4dB@10% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is increased by 0.5dB@10% BLER, when compared to CC with code rate 1/2. - [QC] Code rate 1/4: Decoding performance is increased by 0.2dB@1% BLER, when compared to baseline. |  |
| 4 | Constraint length 8 Code rate 1/2 – 1/6 | - [CMCC] Code rate 1/2: Decoding performance is increased by ~0.22dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Mother code rate 1/4, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/4, code rate 1/4. - [ZTE] Mother code rate 1/3, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/3, code rate 1/4. - [ZTE] Mother code rate 1/2, code rate 1/6: Decoding performance is increased by ~1.7@10% BLER, when compared to constraint length 8, mother code rate 1/2, code rate 1/4. |  |

| Company | Is anything missing? |
| --- | --- |
| Qualcomm | On the 2nd row, we suggest following update. We think smaller value of K offers complexity reduction at reader side more. [QC] Reduced reader & device complexity due to smaller number of shift registers. |
| Xiaomi | We support the proposal. |
| Huawei, HiSilicon | We are fine to capture the performance related observations for the different code rates and constraint lengths as reported by companies. |
| LGE | Okay to capture the observations in the proposal. |

FL can further propose to summarize the table as follows, if companies would like to include both levels of detail:

Proposal 3.4.2b(I): For D2R FEC, include the following observations in the TR:

- Performance of other {k, m} pairs decreased 0.15 – 1.4 dB when k<7, with reported benefit of reduced device complexity due to shorter shift registers.

- Performance of other {k, m} pairs increased 0.22 – 1.8 dB when k>7, with an increase in the number of shift register resulting in increased device complexity.

- Performance of other {k, m} pairs decreased 0.3 – 2.5 dB when m<3.

- Performance of other {k, m} pairs increased 0.2 – 1.8 dB when m>3.

| Company | Views |
| --- | --- |
| Qualcomm | Same as the previous suggestion, suggest following on 1st bullet: - Performance of other {k, m} pairs decreased 0.15 – 1.4 dB when k<7, with reported benefit of reduced reader & device complexity due to shorter shift registers. |
| Xiaomi | The definition of m needs be clarified, whether m is the mother code rate? But we just agreed to study the Mother code-rate R = 1/6, 1/4, 1/3, 1/2. |
| Huawei, HiSilicon | We are fine to summarize the performance of CC for D2R for the different code rates and constraint lengths. |
| LGE | Okay to capture the observations in the proposal. |

#### Round 2

I added “reader &” per the QC statement and notation per Xiaomi. But not sure the reader complexity is any issue, since decoding of CCs is very easy for BS/UE level of hardware.

Proposal 3.4.2a(II): For D2R FEC, include the following table in the TR:

| Option # | CC Design | Pros | Cons |
| --- | --- | --- | --- |
| Baseline | Constraint length 7 Code rate 1/3 | - [Vivo] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. - [Vivo] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. - [CATT] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. - [Lekha] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. - [Xiaomi] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 - [Xiaomi] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 |  |
| 1 | Constraint length 4 Code rate 1/2 – 1/4 | - [Ericsson] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes. | - [CMCC] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. |
| 2 | Constraint length 6 Code rate 1/2 – 1/6 | - [QC] Reduced reader and device complexity due to smaller number of shift registers. | - [QC] Code rate 1/2: Decoding performance is decreased by 0.6dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [QC] Code rate 1/3: Decoding performance is decreased by 0.4dB@1% BLER, when compared to baseline. - [QC] Code rate 1/4: Decoding performance is decreased by 0.2dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. - [QC] Code rate 1/6: Decoding performance is decreased by 0.15dB@1% BLER, when compared to baseline. |
| 3 | Constraint length 7 Code rate 1/2 – 1/4 | - [CATT] Code rate 1/2, TBCC: Decoding performance is increased by ~9dB@10% BLER, when compared to no CC but with Manchester line codes. - [CATT] Code rate 1/4: Decoding performance is increased by ~1.4dB@10% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is increased by 0.5dB@10% BLER, when compared to CC with code rate 1/2. - [QC] Code rate 1/4: Decoding performance is increased by 0.2dB@1% BLER, when compared to baseline. |  |
| 4 | Constraint length 8 Code rate 1/2 – 1/6 | - [CMCC] Code rate 1/2: Decoding performance is increased by ~0.22dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Mother code rate 1/4, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/4, code rate 1/4. - [ZTE] Mother code rate 1/3, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/3, code rate 1/4. - [ZTE] Mother code rate 1/2, code rate 1/6: Decoding performance is increased by ~1.7@10% BLER, when compared to constraint length 8, mother code rate 1/2, code rate 1/4. |  |

Proposal 3.4.2b(II): For D2R FEC, include the following observations in the TR, where k = constraint length and m = 1/code-rate:

- Performance of other {k, m} pairs decreased 0.15 – 1.4 dB when k<7, with reported benefit of reduced reader and device complexity due to shorter shift registers.

- Performance of other {k, m} pairs increased 0.22 – 1.8 dB when k>7, with an increase in the number of shift register resulting in increased device complexity.

- Performance of other {k, m} pairs decreased 0.3 – 2.5 dB when m<3.

- Performance of other {k, m} pairs increased 0.2 – 1.8 dB when m>3.

## D2R CRC [VOID]

Section 4.1 will take R2D and D2R CRCs together.

## D2R multiple access [ACTIVE]

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

### Round 1

Over the course of the meeting since the original agreements on D2R MA, submitted papers have conducted a good study of the multiple access proposals, their pros/cons, feasibility, and applicability. FL proposes it is now time to collet that study into the TR via the following TPs to the “D2R multiple access” section of the TR.

Proposal 3.6a(I): Add the following TP to the TR section on “D2R multiple access” for TDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [CMCC, CATT, Ericsson, FUTUREWEI] state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [DOCOMO, LGE, MTK] state that the guard interval between consecutive D2R transmissions from different devices depends on the SFO after clock calibration (….)

| Company | Anything missing? |
| --- | --- |
| Huawei, HiSilicon | We are fine with the proposed texts. |
| Ericsson | Ok |
| LGE | Okay in general. We would like add “if supported” at the end of the red text. |

Proposal 3.6b(I): Add the following TP to the TR section on “D2R multiple access” for FDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. (FL : Would insert here the TP from Proposal 3.6a) … NOTE: For the purposes of this section, X refers to the device’s SFO expressed as 10X ppm. According to sources [Ericsson, CMCC, CATT, CEWiT, DOCOMO, FUTUREWEI, Huawei, InterDigital], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [CMCC], and potentially increased power consumption for devices according to sources [CATT, Sony, OPPO]. For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. Companies to report how they perform multiplying for option 2. - For applying with Miller line codes, according to Figure 6-13 of [6]. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: Small frequency shifts for D2R are studied for the same potential purposes for MSK. It is observed that the performance of FDMA may be impacted by the following aspects: - The large SFO of device - Sources [Qualcomm, ZTE] state that higher value of X produces higher BLER degradation due to inter-device interference from the ideal case of zero SFO - Source [Huawei] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤ 1dB) on the performance of FDMA between multiple devices. - Sources [vivo] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [Sony] think that the required guard band size increases for higher switching frequencies for passive devices. - The timing offset between devices - Sources [InterDigital] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - The maximum range of small frequency shift - Sources [MTK] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - The harmonics in the backscattered signal - Sources [vivo] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - The potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision (….)

| Company | Anything missing? |
| --- | --- |
| TCL | In our understanding, except for SFO/intermodulation/harmonic impacts, other impacts could be considered as below: - 1SB or 2SB for D2R signal will impact the use of sub-channel. For example, there are 64 sub-channel within one AIoT system BW, the available sub-channel is 64 if considering 1SB. However, the available sub-channel is only at most 32 if considering 2SB. Thus, 1SB or 2SB will impact the number of sub-channel. - For “the potential intermodulation spectral leakage in the backscattered signal”, we think it has similar impact as gap setting between CW two-tone due to the spectrum position of intermodulation signal is up to the gap of CW two-tone signal. Thus, we suggest add more details about this description. |
| Qualcomm | A minor suggestion on the following bullet. - Sources [Qualcomm, ZTE] state that large SFO (e.g. up to 105 ppm) higher value of X produces higher BLER degradation due to inter-device interference from a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO |
| Huawei, HiSilicon | We are generally OK with the proposed texts, and share the following views on D2R FDMA. - According to our analysis and simulations, the impact of SFO/harmonics can be reduced to an acceptable level by appropriate air interface design and reader receiver optimization. - The maximum range of small frequency shift depends on the assumption of the clock frequency for baseband processing at device. As discussed for the methods of small frequency shift, code/bit/square-wave repetition is used. - As repetition is expected to be supported for the improvement of D2R link budget, we don’t see the impact on device complexity and power consumption to support FDMA. |
| ZTE, Sanechips | According to our analysis and the provided simulation results, some modifications are as follows: According to sources [Ericsson, CMCC, CATT, CEWiT, DOCOMO, FUTUREWEI, Huawei, InterDigital], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [CMCC, ZTE], performance loss is observed in the presence of SFO, time offset among multiple devices, and the increased device number according to source [ZTE], and potentially increased power consumption for devices according to sources [CATT, Sony, OPPO]. For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. Companies to report how they perform multiplying for option 2. - For applying with Miller line codes, according to Figure 6-13 of [6]. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: Small frequency shifts for D2R are studied for the same potential purposes for MSK. It is observed that the performance of FDMA may be impacted by the following aspects: - The large SFO of device - Sources [Qualcomm, ZTE] state that higher value of X produces higher BLER degradation due to inter-device interference from the ideal case of zero SFO. Source [ZTE] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - ... - Frequency resource collision - The number of multiple device - Source [ZTE] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5~3dB performance loss from 6 FDMA-ed devices compared to single device. |
| Ericsson | How many frequency occasions that are possible with each method should also be discussed. |

Although CDMA is still shown as FFS for feasibility and necessity in existing agreements, FL suggests that we can have a TP as below which states what companies are reporting on those points. Whether such MA scheme is recommended for, or included in, any eventual normative phase is a later decision.

Proposal 3.6c(I): Add the following TP to the TR section on “D2R multiple access” for FDMA:

6.1.2.x D2R multiple access (…) According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Sources [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Sources [vivo] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Sources [FUTUREWEI] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Sources [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). - Sources [Qualcomm] state that quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Sources [Huawei] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - In the case of relatively smaller SFO (e.g., X = 4), - Sources [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). - Sources [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm. Besides, larger power variation severely degrades the capacity. The impact of CFO on the performance of CDMA is studied for Device 2b as follows. - Sources [Huawei] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. The impact of timing offset between CDMed D2R transmissions on the performance of CDMA is studied for Device 2b as follows. - Sources [Lenovo] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Sources [OPPO] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Sources [MTK] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Sources [Samsung] think that the different propagation delays from devices may also degrades decoding performance. - Sources [ZTE] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Sources [Qualcomm] state that the larger power variation severely degrades the capacity of CDMA for msg-1. - Sources [ZTE] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Sources [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Sources [Ericsson] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Sources [MTK] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Sources [TCL] think that CDMA leads to higher power consumption and lower data rate. - Sources [vivo] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference.

| Company | Anything missing? |
| --- | --- |
| OPPO1 | For our simulation results for SFO = 5% (which is 0.5*105 ppm), it is better to capture this under “In the case of large SFO (e.g., X = 5)” instead of “X = 4”. Furthermore, our simulation results were obtained using a sliding window correction and some observations proposed for the TR are not yet captured. So, we suggest the following modification (in blue color). - In the case of large SFO (e.g., X = 5), - Sources [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - … - Sources [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader using a sliding window correction, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. - In the case of relatively smaller SFO (e.g., X = 4), - Sources [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). - Sources [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm. Besides, larger power variation severely degrades the capacity. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Sources [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - … - Sources [OPPO] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Sources [OPPO] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. |
| Qualcomm | We suggest following: - In the case of relatively smaller SFO (e.g., X = 4), - Sources [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). - Sources [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. Besides However, larger power variation severely degrades the capacity. |
| Huawei, HiSilicon | We are generally OK to capture the proposed observations on D2R CDMA. However, we still have concern on the necessity and feasibility of R2D CDMA, especially considering the large SFO/CFO and limited memory capability of Ambient IoT devices. |
| ZTE, Sanechips | According to our analysis and the provided simulation results, some modifications are as follows: Proposal 3.6c(I): Add the following TP to the TR section on “D2R multiple access” for FCDMA: ***the Unchanged is omitted *** According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. Source [ZTE] states that CDMA can show performance gain compared with FDMA in different cases of SFO, TO and multiple device numbers. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Sources [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [ZTE] also state that the performance gain of CDMA over FDMA is 2~4dB for relatively small information block transmission and 1~2dB for larger information block transmission when CC and BPSK is used for D2R transmission, considering different SFO (i.e., X=0, 4~5), time offsets among devices (i.e., 0/1/2 chip length) and device numbers (i.e., 1/2/3/4). Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - .... - Sources [ZTE] state that CDMA with longer orthogonal codes (e.g., 16 and 64) show 1~3dB performance gain at 10% BLER compared with FDMA for different SFO, time offset and up to 4 multiple devices are considered, and about 9dB gain at 1% BLER against FDMA when 6 multiple devices and large SFO and time offset. |
| ZTE,Sanechips2 | If “TPs is not to pursue down-selection type of results”, we suggest the following update to capture our evaluation results: The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Sources [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. Sources [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the modulation scheme of D2R transmissions: Source [ZTE] state that BPSK modulation for the CDMA improve the D2R transmission performance and multiplexing capacity compared with OOK based modulation. |

### Round 2

For TDMA, I hope I understood LG correctly that the requested “if supported” has the subject of the guard interval, and hence make that update:

Proposal 3.6a(II): Add the following TP to the TR section on “D2R multiple access” for TDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [CMCC, CATT, Ericsson, FUTUREWEI] state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [DOCOMO, LGE, MTK] state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration (….)

| Company | Views? |
| --- | --- |

For FDMA

- I took the Qualcomm change.

- Added the ZTE observations in the descriptive results part, and with that it seems no need to duplicate in the opening paragraph.

- Ericsson – is that something you can bring to next meeting update?

Proposal 3.6b(II): Add the following TP to the TR section on “D2R multiple access” for FDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. (FL : Would insert here the TP from Proposal 3.6a) … NOTE: For the purposes of this section, X refers to the device’s SFO expressed as 10X ppm. According to sources [Ericsson, CMCC, CATT, CEWiT, DOCOMO, FUTUREWEI, Huawei, InterDigital], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [CMCC], and potentially increased power consumption for devices according to sources [CATT, Sony, OPPO]. For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. Companies to report how they perform multiplying for option 2. - For applying with Miller line codes, according to Figure 6-13 of [6]. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: Small frequency shifts for D2R are studied for the same potential purposes for MSK. It is observed that the performance of FDMA may be impacted by the following aspects: - The large SFO of device - Sources [Qualcomm, ZTE] state that large SFO (e.g. up to 105 ppm) produces higher BLER degradation due to inter-device interference than a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO. Source [ZTE] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - Source [Huawei] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤ 1dB) on the performance of FDMA between multiple devices. - Sources [vivo] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [Sony] think that the required guard band size increases for higher switching frequencies for passive devices. - The timing offset between devices - Sources [InterDigital] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - The maximum range of small frequency shift - Sources [MTK] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - The harmonics in the backscattered signal - Sources [vivo] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - The potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision - The number of multiplexed devices - Source [ZTE] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5~3dB performance loss from 6 FDMA-ed devices compared to single device. (….)

| Company | Anything missing? |
| --- | --- |

For CDMA

- OPPO, updated basically as you request. However, the discussion of m-ssequen

- Qualcomm: OK.

- ZTE: The intention of these TPs is not to pursue down-selection type of results, since this requires very careful cross check and comparison of results by companies, to ensure the possibility of the statements. Rather, the intention is to summarize each MA scheme on its own, at this stage.

Proposal 3.6c(II): Add the following TP to the TR section on “D2R multiple access” for CDMA:

6.1.2.x D2R multiple access (…) According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Sources [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Sources [vivo] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Sources [FUTUREWEI] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Sources [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). - Sources [Qualcomm] state that quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Sources [Huawei] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Sources [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. - In the case of relatively smaller SFO (e.g., X = 4), - Sources [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. However, larger power variation severely degrades the capacity. The impact of CFO on the performance of CDMA is studied for Device 2b as follows. - Sources [Huawei] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. The impact of timing offset between CDMed D2R transmissions on the performance of CDMA is studied for Device 2b as follows. - Sources [Lenovo] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Sources [OPPO] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Sources [MTK] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Sources [Samsung] think that the different propagation delays from devices may also degrades decoding performance. - Sources [ZTE] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Sources [Qualcomm] state that the larger power variation severely degrades the capacity of CDMA for msg-1. - Sources [ZTE] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Sources [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Sources [Ericsson] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Sources [MTK] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Sources [TCL] think that CDMA leads to higher power consumption and lower data rate. - Sources [vivo] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Sources [OPPO] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Sources [OPPO] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap.

FL after round 2 – see section 5.3:

TDMA: No changes requested after round 2

FDMA: No changes requested after round 2

CDMA: Updated for most of the ZTE request, except some partial duplication.

## D2R time-domain definitions [ACTIVE]

Agreement RAN1#116bis From 9.4.2.3: For PDRCH generation at the device, at least following blocks are studied as the baseline: - CRC bits are appended if there is non-zero length CRC - Note: CRC details discussed in agenda item 9.4.2.1 - Coding - Exact coding methods within the coding block, e.g. with/without line coding and/or FEC discussed under agenda 9.4.2.1 - Note: If no line coding is used, there may be an additional block (e.g. square wave generator) before/after modulation block - Modulation - Note: Other blocks could be added if agreed PDRCH generation

### Round 1

Similar to the R2D time-domain definition, a D2R chip is defined by the following proposal, which is presented here to contain only the part that seemed stable in the previous meeting. The next step following in proposal 3.7.1b.

Proposal 3.7.1a(I): In D2R, a chip corresponds to one modulated symbol at least for OOK and BPSK.

- FFS: the definition for MSK.

| Company | Views? |
| --- | --- |
| Qualcomm | OK with the proposal. |
| Xiaomi | For the FFS part, it can be discussed after the decision whether the MSK is feasible for all device types. |
| Huawei, HiSilicon | Agree. |
| Ericsson | Is there a reason to mention modulations? Isn’t the definition fine regardless of modulation? |
| LGE | Okay with the proposal. |

There are two different ways for the determination of the R2D chip duration, which are summarized in the following proposal. Since, similar as R2D, this has the characteristic of a normative detail which does not need to be finalized in an SI, FL suggests we can capture these both in the TR for later reference.

The RAN1#118bis agreement towards describing the small frequency-shift factor means it seems more convenient to move most of the discussion on chip durations to Section 3.3.2. If there is agreement on relevant proposals, FL will arrange the TR text in a suitable way, so don’t worry about definitional matters here.

Proposal 3.7.1b(I): Capture in the TR that the following chip duration calculations have been studied:

- Method 1: Calculated according to the transmission bandwidth, and depending on small frequency shift factor. [CMCC] [Ericsson][Huawei][Spreadtrum][vivo][Xiaomi][ZTE]

- FL NOTE: The small frequency-shift factor is in Section 3.3.2, and FL would arrange the TR text suitably after agreements.

- Method 2: One of a pre-defined set of pulse time durations. [Lenovo][vivo]

- E.g., D2R chip duration is an integer multiple (or fraction) of R2D chip duration, where a reference SCS (e.g. the R2D SCS) is used as the basis for the value of D2R chip duration. [Sharp]

| Company | Views? |
| --- | --- |
| Qualcomm | We have similar view as Method 1. However, we think it is more appropriate to use “bit length” instead of “transmission bandwidth”, since device does not know the transmission bandwidth. Method 1: Calculated according to the bit length transmission bandwidth, and depending on small frequency shift factor. |
| Xiaomi | Support the method1. For the method2, the timing for D2R and R2D is misalignment, the motivation that D2R chip duration is an integer multiple (or fraction) of R2D chip duration shall be clarified. |
| Huawei, HiSilicon | We are OK with the proposal. Actually, the two proposed methods can have a certain relationship, as the transmission bandwidth and chip duration are not independent from each other. It depends on whether to define the bandwidth size or chip duration first. For example, if to determine the size of transmission bandwidth firstly, there can be considerations on the corresponding chip duration, whose value satisfies the constraint of an integer multiple (or fraction) of R2D chip duration. |
| Ericsson | Ok |
| LGE | On Method 1, we also think that bit duration or bit length instead of the transmission is clearer. By the way, we can also think of the case where the chip duration is directly provided by the reader rather than being calculated by a device from related parameters. Can we add this case as another Method unless it is already included in Method 2. |
| Qualcomm (round 2) | We still think “a bit duration” rather than “a transmission bandwidth”. Take the Manchester option 1 as an example: Agreement Small frequency shifts for D2R are studied for OOK and BPSK: - For applying with Manchester line codes - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. A bit having a certain duration is encoded by Manchester and is repeated integer number of times within the same time duration corresponding to the bit. If the chip corresponds to the one symbol modulated by OOK/BPSK backscattering, the chip duration is obviously calculated based on a bit duration and a small frequency shift factor. In other words, for Manchester option 1, within a bit duration Tb, there are 2*M chips where each chip has a duration of Tb/(2*M). M is the small frequency shift factor. This is indeed same for Manchester option 2. For BPSK square-wave modulation, M is the number of square waves per bit. For Miller, M corresponds to the MMS value “M”. We have agreed different figures of different transmission bandwidth for different line coding schemes. Depending on the future clarification of “transmission bandwidth” for different line codes, the proposed Method 1 may cause further confusion. However, as long as “bit duration” is used, there will be no confusion – it is possible to define chip duration calculation for all the line codes / square wave in the same way. |

FL: To Qualcomm and LG’s point, if we replace ‘transmission bandwidth’ by a ‘bit duration’, we seem to have Method 2 again. Whereas in the current version, the difference is that the chip duration is somehow indirect in method 1, from the tx bandwidth, while in method 2 it is direct (and hence the tx bandwidth is indirect).

To LG’s second point: I believe what you describe is method 2, based on the above explanation.

Hence no change to the proposal, hopefully.

FL after round 2:

- Thanks QC for the further explanation. Considering LG and QC’s points together, I think we can try adding this as another explicit example under Method 2. See below and section 5.3.

## D2R bandwidths [ACTIVE]

Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS

### Bandwidth sizes

#### Round 1

For bandwidth sizes in D2R, it would be possible to face complications if we try to define their values wrt potential multi-single tone CW, due to the gap between the multiple tones. Hence, based on how FL understands the papers, the suggestion is to define them wrt to just one (or each of the) single tone(s). This should then be general across whether the tones are used by multiple CW nodes for multiple devices (somehow), or apply to one device.

Proposal 3.8.1a(I) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure in the TR, where:

- Fc is the carrier-wave frequency, internally or externally generated as appropriate

- FSx is an example of an xth amount of small frequency shift

- For 2SB transmission, +FSx and -FSx parts are included.

- For 1SB transmission, either +FSx or -FSx part is included.

- For each side-band, the bandwidths shown in the figure are to the first zero-crossing points in the spectrum of the respective D2R transmission

- NOTE (to be removed): Refer to section 3.2.2 for defining/describing small frequency-shift

| Company | Views? |
| --- | --- |
| Qualcomm | Suggest to clarify this is at least for OOK and BPSK. |
| Futurewei | For clarity, the following can be added to the above figure, the Fc+FS1 and Fc-FS1, and Fc-FS2 are the centre frequencies for FS1 and FS2, respectively. |
| Xiaomi | Support the proposal. |
| Huawei, HiSilicon | Agree. According to our analysis and simulations, three key aspects are observed as follows. - In the case of small frequency shift being applied, the unoccupied bandwidth between the +FSx and -FSx parts can be used for other devices for e.g. FDMA, which should not be counted into the size of Btx,D2R or Bocc,D2R. - The size of transmission bandwidth does not depend on chip duration, but determined by the length of one information bit. In other words, Btx,D2R does not change due to small frequency shift. - For each sideband in the spectrum of D2R transmission, the bandwidth between the corresponding zero-crossing points is reversely proportional to the length of an information bit, which should be counted as one of the two parts for transmission bandwidth for 2SB transmissions and the transmission bandwidth for 1SB transmission, respectively. |
| LGE | We think we need more discussion on the 5th bullet. |

Proposal 3.8.1b(I): State in the TR that:

- Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave would need to be set to value(s) which account for at least the following aspects:

  - The guard band Bguard,D2R would be necessary due to SFO

  - The guard band Bguard,D2R would be necessary due to CFO for Device 2b

  - The guard band Bguard,D2R is not necessarily symmetric around the transmitted D2R signal

| Company | Views? |
| --- | --- |
| Qualcomm | OK |
| Xiaomi | For the last sub-bullet, it is preferred that guard band Bguard,D2R is necessarily symmetric around the transmitted D2R signal, at least for 2SB, each side has the main lobe. |
| Huawei, HiSilicon | Agree. The detailed values of the guard band can be left to the normative works, which may depend on the details of other designs (e.g., modulation, line code, and repetitions). |
| Ericsson | Ok |
| LGE | Okay with the proposal. |

FL will update this in Section 5 for online, based on the offline comments.

#### Round 2

Proposal 3.8.1b (Bocc), FL had intended for online on Tues AM, but missed it out. Will bring it to the next online!

New proposal for Manchester option 2, and for 1SB to address those point. The final bullet here is intended to be explanatory. It could be removed, if companies wish, as the figure is a general illustration.

Proposal 3.8.1c(II) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where:

- …

- (Explanatory temporary note): When small frequency shift is applied, the power of upper half-part may or may not be higher than the lower half-part for both +FSx and -FSx parts

| Company | Views? |
| --- | --- |
| Qualcomm (round 2) | This proposal is not marked as “blue highlights” in the email. And it is not clear what FL missed out in Tue online, and which part FL wants to bring it to the next online. Our understanding is that the agreement was made already and the discussion was closed. |

FL: Following is added after the Tues PM offline, shown here for info and will bring this to online

Proposal 3.8.1d(I): For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

Proposal 3.8.1e(I) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

| Company | Views? |
| --- | --- |

# R2D and D2R

## CRC [ACTIVE]

Agreement RAN1#116 R2D study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC. - FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target

Agreement RAN1#116 D2R study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC. - FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target

Agreement RAN1#116bis Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

### Round 1

Inputs across companies have essentially arranged the discussion according to the study points noted in the above agreements. Therefore FL proposes observations to collect the feasibility study on that basis, divided up for convenience as follows. The assumption is that all the parts would be captured.

After these textual observations, there follows a table to summarize the evaluation results that some companies also use to make their statements.

Proposal 4.1a(I): For CRC, capture the following observation in the TR:

- Sources as follows recommend that the length of the CRC used for A-IoT depends on:

  - Block size or TBS-like size, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, CATT, ZTE, DCM, ITL. Furthermore:

    - Source [Huawei] states that specifically for short messages, the CRC length can directly impact the transmission efficiency due to the overhead caused by the CRC.

    - Source [ZTE] state that CRC length depends on the TBS due to its impact on the transmission overhead and FAR performance. When the TBS is too small, the overhead of CRC-6 becomes quite significant.

    - Source [ITL] states that CRC length is fundamentally proportional to the size of the message.

  - Target false alarm rate or error detection performance, according to sources Huawei, Spreadtrum, TCL, CMCC, CATT, MTK, ITL. Furthermore:

    - Source Huawei evaluated that the probability of undetected errors is higher for CRC-6 as compared to CRC-16, and transmissions with CRC-16 performs better, for a given TBS=24, and source MTK say that a longer CRC would mean a better error detection performance.

  - Type of message, such as a command/reply function, to determine when no CRC is used, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, MTK, DCM. Furthermore:

    - Sources Huawei and Spreadtrum say that similar to RFID, only important messages, decided based on their functionality, such as configuration/control/command messages, are appended with CRC to increase their reliability, while other messages are not protected by CRC to save the overhead.

    - Source TCL proposes different CRC lengths for Paging-like/select/challenge messages that may or may not contain the device ID/group ID.

    - Source CMCC states that some messages such as a QueryRep-like command do not result in serious FAR issues if mis-detected, and hence the functionality of a message/command has to be considered.

  - Overhead caused by CRC, according to sources Huawei, Spreadtrum, CMCC, ZTE, MTK

    - Sources Huawei, Spreadtrum, CMCC, MTK mention that longer CRCs have high overhead but enjoy better error detection performance, affecting the transmission efficiency. Hence, short CRC can be used for short commands to reduce the false alarm rate and make the system more robust.

    - Source CMCC states that adding CRC to QueryRep-like commands which are sent repeatedly with a small payload would waste network resources, and hence the overhead should be considered.

    - Source ZTE states that the overhead of CRC decreases as the TBS increases, and that CRC-16 has higher overhead than CRC-6.

| Company | Anything missing? |
| --- | --- |
| Qualcomm | We input our view on CRC in R1-2408851. Suggest to add it in the observation e.g., as follows. - Overhead caused by CRC, according to sources Huawei, Spreadtrum, CMCC, ZTE, MTK, Qualcomm - Sources Huawei, Spreadtrum, CMCC, MTK mention that longer CRCs have high overhead but enjoy better error detection performance, affecting the transmission efficiency. Hence, short CRC can be used for short commands to reduce the false alarm rate and make the system more robust. - Source CMCC states that adding CRC to QueryRep-like commands which are sent repeatedly with a small payload would waste network resources, and hence the overhead should be considered. - Sources ZTE, Qualcomm states that the overhead of CRC decreases as the TBS increases, and that CRC-16 has higher overhead than CRC-6. - Source Qualcomm states that additional CRC, e.g., CRC-11 can be considered to reduce the gap between CRC-6 and CR -16. One minor comment: the logic of this is not clear – better to check. |
| Xiaomi | We are fine with the proposal. |
| Huawei, HiSilicon | We are fine to capture the different factors that impact the CRC length for different TBSs. |

For convenience FL presents the further study of the relationship between TBS/block length and CRC length as a separate observation, but assumes that both this part and the part above would naturally be captured in the TR.

The text in gray highlight, FL is not sure that the method used by ZTE conforms to what’s normally expected, where overhead ratio should be CRC/(TBS + CRC), while ZTE appear to use CRC/TBS.

FL notes the proposal for 11-bit CRC from Qualcomm. However, the agreement from RAN1#116bis (see above) allows FFS only of 6- and 16-bit CRCs not from 38.212, with comparison to those from 38.212. Hence it is not included in the text below.

Proposal 4.1b(I): For CRC, capture the following observation in the TR:

- There was study regarding the length of the CRC to be used according to the number of bits Z before CRC, i.e. CRC-6 for Z<=X bits, while CRC-16 for Z > X bits. Values of X were proposed with the following justifications:

  - For sources Huawei, Spreadtrum, China Telecom, CMCC, Comba, X = 24 bits:

    - Source Huawei says that it provides a balance between the probability of error detection (Pud) and resource utilization efficiency. CRC-6 is used for TBS=24 bits to achieve a low overhead of 20% with a higher Pud of 10-6, but for TBS>24 bits, CRC-16 is used with an overhead of less than 40% being acceptable for a lower Pud of 10-9.

    - Source CMCC mentions that a higher value of X (>16 bits) is better, since the overhead when X=16 is too high.

  - For source CATT, X = 16 bits:

    - Source CMCC claims the overhead is too high, since if X=16 is used, a TBS=17 bits will have to add a 16 bits CRC, which is almost 50% overhead.

    - Source CATT states that the high overhead is justified by the low FAR, using RFID as an example.

  - For source ZTE, X = 57 bits:

    - Source ZTE states that considering the high overhead caused by CRC-16 for small TBSs, CRC-6 can be used when the TBS is less than or equal to 57, since different sizes of transport blocks have almost identical FAR @10% BLER. However, when TBS exceeds 57, the FAR gradually deteriorate and CRC-16 can be used.

| Company | Anything missing? |
| --- | --- |
| Xiaomi | Support the proposal. |
| Huawei, HiSilicon | For the entry on X=57, since the overhead calculation is incorrect, we do not think it is still justified to retain this option. For X=57, the overhead caused using CRC-6 for a 57bit TB would be only 9%, but its Pud is would be very high as compared to when using CRC-16. |
| Ericsson | The CRC length may be adaptable depending on TBS, target false alarm rate, and/or other factors for R2D and D2R. At this stage, it is better not to limit the options for the “X” value. |

For the study of CRC polynomial generators, FL notes there is little support for looking at any other polynomials than those already familiar from TS 38.212. However, following the direction of reporting the nature of the feasibility study at this stage, and leaving potential choices to a later stage, FL suggests we can have following factual summary of company reports:

Proposal 4.1c(I): For the CRC generator polynomials, capture the following observations in the TR:

- Sources China Telecom, OPPO, Xiaomi, CATT, Samsung recommend that the same polynomials from TS 38.212 are reused, giving justifications:

  - Sources CATT, OPPO state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials.

  - Source Xiaomi states that the device complexity is increased when different polynomials are introduced for the D2R transmission.

  - Source China Telecom states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used.

- On the other hand, source ZTE recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, gCRC16D=D16+D11+D6+D5+1, incorporating the CRC-6 polynomial, giving justifications:

  - The proposed new CRC-16 polynomial nests the existing CRC-6 within its structure.

  - Using separate sets of circuits to implement the polynomials for CRC-6 and CRC-16 as per TS 38.212 requires a higher number of registers and XOR gates when compared to using the new CRC-16 polynomial.

  - FAR performance is the same for the legacy and proposed new polynomial for CRC-16, but the new polynomial reduces the complexity and power consumption of the device.

- Source Qualcomm recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, and two new polynomials for CRC-11 and CRC-16 are introduced: gCRC11D=D11+D6+D5+1 and gCRC16D=D16+D11+D6+D5+1 respectively, giving justifications:

  - The proposed new CRC-11 and CRC-16 polynomials are nested polynomials.

  - Performance of undetected error probabilities is comparable with the NR CRC polynomials for each CRC length.

| Company | Anything missing? |
| --- | --- |
| Qualcomm | On the last bullet, all CRC-16, CRC-11, and CRC-6 are nested. Suggest to update as follows. - Source Qualcomm recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, and two new polynomials for CRC-11 and CRC-16 are introduced: gCRC11D=D11+D6+D5+1 and gCRC16D=D16+D11+D6+D5+1 respectively, giving justifications: - The proposed new CRC-11 and CRC-16 polynomials are nested nests with the existing CRC-6 within its structure polynomials. - Performance of undetected error probabilities is comparable with the NR CRC polynomials for each CRC length. |
| Xiaomi | Support the proposal. |
| Huawei, HiSilicon | For the entry from QC, we think this is not in accordance with the agreement we made in RAN1#116bis. We agreed to use either CRC-6, CRC-16 or no CRC. The addition of another CRC-11 was not covered in the FFSs either, and hence should not be considered here. |

Finally, there are evaluations presented to justify some of the preceding observations from companies, which FL suggests capturing directly in a table in the TR as follows. FL has unified the terms of false alarm rate and probability of false alarm into PFA.

Proposal 4.1c(I): For CRC, capture the following evaluation results in the TR:

- NOTE 1: Pud = Probability of undetected error according to [J. K. Wolf and R. D. Blakeney, “An exact evaluation of the probability of undetected error for certain shortened binary CRC codes,” Proc. Milcom ’88, vol. 1, pp. 287-292, 1988]

- NOTE 2: Overhead = CRC length / (TBS + CRC length).

| TBS | CRC-6 |  |  | CRC-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source | CRC overhead | PFA or Pud | Source | CRC overhead | FAP/FAR/Pud |
| 8 bits | HW | 43% |  | HW | 67% |  |
| 12 bits | E/// QC | 33% 33% | ~10^(-2) PFA | E/// QC | 57% 57% | ~10^(-5) PFA ~10^(-11) Pud @10-3 BLER |
| 16 bits | HW | 27% |  | HW | 50% |  |
|  | ZTE QC | 27% |  | ZTE QC | 50% 50% |  |
| 24 bits | HW E/// ZTE QC | 20% 20% 20% | ~10^(-6) Pud @ 10% BLER ~10^(-2) PFA | HW E/// QC | 40% 40% 40% | ~10^(-9) Pud @10% BLER ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 32 bits | HW ZTE QC | 16% 16% | ~10^(-5) Pud @10-3 BLER | HW ZTE QC | 33% 33% 33% | ~10^(-10) Pud @10-3 BLER |
| 40 bits | HW ZTE QC | 13% 13% | ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | HW ZTE QC | 29% 29% | ~10^(-10) Pud @10-3 BLER |
| 48 bits | E/// ZTE QC | 11% 11% | ~10^(-2) PFA ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | E/// ZTE QC | 25% 25% | ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 96 bits | HW E/// ZTE QC | 6% 5% 6% 6% | ~10^(-2) PFA ~10^(-4) Pud @10-3 BLER | HW E/// ZTE QC | 14% 14% 14% | ~10^(-5) PFA ~10^(-9) Pud @10-3 BLER |
| 128 bits | ZTE QC | 4% 4% | ~10^(-3) Pud @10-3 BLER | ZTE QC | 11% | ~10^(-9) Pud @10-3 BLER |
| 256 bits | HW QC | 2% 2% | ~10^(-3) Pud @10-3 BLER | HW QC | 6% 6% | ~10^(-9) Pud @10-3 BLER |

| Company | Anything missing? |
| --- | --- |
| Huawei, HiSilicon | We are fine to capture the performance and overhead of the of CRC-6 and CRC-16. |

### Round 2

For the proposals not changed below, companies can add to Round 1 tables (to save space!), and for those following, changes are shown.

This one is updated per the offline discussion:

Proposal 4.1a(II): For CRC, capture the following observation in the TR:

- Sources as follows recommend that the length of the CRC used for A-IoT depends on:

  - Block size or TBS-like size, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, CATT, ZTE, DCM, ITL. Furthermore:

    - Source [Huawei] states that specifically for short messages, the CRC length can directly impact the transmission efficiency due to the overhead caused by the CRC.

    - Source [ZTE] state that CRC length depends on the TBS due to its impact on the transmission overhead and FAR performance. When the TBS is too small, the overhead of CRC-6 becomes quite significant.

    - Source [ITL] states that CRC length is fundamentally proportional to the size of the message.

  - Target false alarm rate or error detection performance, according to sources Huawei, Spreadtrum, TCL, CMCC, CATT, MTK, ITL. Furthermore:

    - Source Huawei evaluated that the probability of undetected errors is higher for CRC-6 as compared to CRC-16, and transmissions with CRC-16 performs better, for a given TBS=24, and source MTK say that a longer CRC would mean a better error detection performance.

  - Type of message, such as a command/reply function, to determine when no CRC is used, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, MTK, DCM. Furthermore:

    - Sources Huawei and Spreadtrum say that similar to RFID, only important messages, decided based on their functionality, such as configuration/control/command messages, are appended with CRC to increase their reliability, while other messages are not protected by CRC to save the overhead.

    - Source TCL proposes different CRC lengths for Paging-like/select/challenge messages that may or may not contain the device ID/group ID.

    - Source CMCC states that some messages such as a QueryRep-like command do not result in serious FAR issues if mis-detected, and hence the functionality of a message/command has to be considered.

  - Overhead caused by CRC, according to sources Huawei, Spreadtrum, CMCC, ZTE, MTK, Qualcomm

    - Sources Huawei, Spreadtrum, CMCC, MTK mention that longer CRCs have high overhead but enjoy better error detection performance, affecting the transmission efficiency. In order to balance the overhead and performance, Hence, short CRC can be used for short commands to reduce the false alarm rate and make the system more robust as compared to no CRC.

    - Source CMCC states that adding CRC to QueryRep-like commands which are sent repeatedly with a small payload would waste network resources, and hence the overhead should be considered.

    - Sources ZTE, Qualcomm states that the overhead of CRC decreases as the TBS increases, and that CRC-16 has higher overhead than CRC-6.

| Company | Anything missing? |
| --- | --- |
| FL | No need to comment if still fine. |

And this one to remove the CRC-11 for same reasons as not included elsewhere, and re-capture the part of that proposal that deals with CRC-16 in the preceding bullet, since the CRC-16 polynomial is the same and it seems the observations are also equivalent including nesting, etc.

Proposal 4.1c(II): For the CRC generator polynomials, capture the following observations in the TR:

- Sources China Telecom, OPPO, Xiaomi, CATT, Samsung recommend that the same polynomials from TS 38.212 are reused, giving justifications:

  - Sources CATT, OPPO state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials.

  - Source Xiaomi states that the device complexity is increased when different polynomials are introduced for the D2R transmission.

  - Source China Telecom states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used.

- On the other hand, sources [ZTE],[Qualcomm] recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, gCRC16D=D16+D11+D6+D5+1, incorporating the CRC-6 polynomial, giving justifications:

  - The proposed new CRC-16 polynomial nests the existing CRC-6 within its structure.

  - Using separate sets of circuits to implement the polynomials for CRC-6 and CRC-16 as per TS 38.212 requires a higher number of registers and XOR gates when compared to using the new CRC-16 polynomial.

  - FAR performance is the same for the legacy and proposed new polynomial for CRC-16, but the new polynomial reduces the complexity and power consumption of the device.

- Source Qualcomm recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, and two new polynomials for CRC-11 and CRC-16 are introduced: gCRC11D=D11+D6+D5+1 and gCRC16D=D16+D11+D6+D5+1 respectively, giving justifications:

  - The proposed new CRC-11 and CRC-16 polynomials are nested polynomials.

  - Performance of undetected error probabilities is comparable with the NR CRC polynomials for each CRC length.

| Company | Anything missing? |
| --- | --- |
| FL | No need to comment if still fine. |

## Scrambling

### Round 1

FL sees only two companies, Qualcomm and OPPO, proposing to include scrambling in the tx/rx chain for R2D, where the reason given is interference randomization between readers in the case they for some reason transmit simultaneously in an overlapping area. In D2R just one company proposes scrambling, and no reason is given.

Proposal 4.2.1a(I): Scrambling is not studied for at least the D2R link. Regarding the R2D link, the TR captures:

- Although scrambling is not recommended for the R2D link, the reason given by sources [Qualcomm], [OPPO] to study it was to randomize interference in the case where more than one reader transmits simultaneously with overlapping coverage areas.

| Company | Views |
| --- | --- |
| Qualcomm | We did not mention the reason of scrambling for D2R because the reason is same as for R2D. |
| OPPO2 | It is possible that there is coverage overlapping in real deployment, in which case, interference randomization should be considered. We think it is too early to exclude this feature at SI stage. We prefer to keep it open for further study. |
| Huawei, HiSilicon | We are fine to not support scrambling for R2D and D2R, due to the entailing complexity and memory requirements it would impose on devices. |
| Ericsson | Ok |
| LGE | Okay with the proposal in principle if it is agreeable to the proponents. |
| Qualcomm (round 2) | We suggest to capture the following: - Study of scrambling was not agreed for R2D link and D2R link during the study item. However, sources [Qualcomm], [OPPO] suggested to study it for interference randomization e.g., in the case where more than one reader operates simultaneously with overlapping coverage areas. |

# FL: I will try the QC suggestion for online discussion.

# Proposals for online sessions

## Tues AM

After some corridor technical discussions, FL will bring an additional figure & proposal to cover Manchester with option 2 (hopefully during Tuesday for round 2 discussion), and asks that we agree to this part now, for the options indicated by the added text.

Proposal 3.8.1a(II) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure in the TR, where:

- Fc is the carrier-wave frequency, internally or externally generated as appropriate

- FSx depict examples of amount of small frequency shift

- For 2SB transmission, +FSx and -FSx parts are included.

- For 1SB transmission, either +FSx or -FSx part is included.

- For small frequency-shift by: Manchester with option 1, and by Miller line-code, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift):

  - For each side-band, the bandwidths shown in the figure are to the first zero-crossing points in the spectrum of the respective D2R transmission

Proposal 3.7.1a(I): In D2R, a chip corresponds to one modulated symbol at least for OOK and BPSK.

- FFS: the definition for MSK.

FL: I tidied up this offline version as follows based on another reading of papers:

Proposal 3.2.2(II): 2SB modulation is supported for D2R transmission for all devices.

- Support of 1SB would depend on the impacts due to issues including: device-side filtering requirements i.e. image suppression, RF resource usage / spectral efficiency, etc.

FL: Updated per the offline discussion on mention phase non-continuity, and other aspects from papers to capture:

Proposal 3.2.1a(II): Capture the following observation in the TR:

- Sources [Ericsson][CATT][Qualcomm][Lekha] report that MSK is feasible in some way for at least some devices:

  - [Ericsson][CATT] say it is feasible for all devices

  - [Qualcomm] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b

  - For device 1 and 2a this type of MSK does not have continuous phase

- Sources [Huawei][TCL][CMCC][CTC][vivo][Xiaomi][Comba] report that MSK is either infeasible or should be deprioritized for all devices.

  - [Huawei][CMCC][CTC][vivo][TCL][Xiaomi][Comba] say that MSK is less spectrally efficient than OOK and BPSK

  - [Huawei][CTC][TCL][vivo][Xiaomi] say that MSK would increase reader and device complexity

  - [vivo] say that MSK performance for device 2b would materially degrade due to CFO, and [Huawei] say that for device 2b, there are issues due to poor phase accuracy in the device,

FL: This is reduced to just adding the new Note, and (for now) keeping M=32.

Proposal 2.21(II): Agree the following text proposal for the TR:

#### 6.1.1.x R2D bandwidths …(unchanged parts omitted)… Table 6.1.1.x-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D. Table 6.1.1.x-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3. …(unchanged parts omitted)…

FL: Although this one was not discussed offline, it seems many companies are fine to have it. Noting Qualcomm’s statement that it’s not needed, I wonder nevertheless if the general nature of the statement might be acceptable since we are writing an informative TR, to show that RAN1 did have discussions and writing about this?

Proposal 2.2.2(I): Capture the following observation in the TR:

For R2D, whether single sideband or double sideband modulation is used is up to reader implementation.

FL: There is some offline checking on this proposal, so it’s here, unchanged, as just-in-case

Proposal 3.7.1a(I): In D2R, a chip corresponds to one modulated symbol at least for OOK and BPSK.

- FFS: the definition for MSK.

## Weds AM

FL: I skipped this one in copy-paste for Tues AM, so it is here instead! Offline seemed no changes requested.

Proposal 3.8.1b(I): State in the TR that:

- Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave would need to be set to value(s) which account for at least the following aspects:

  - The guard band Bguard,D2R would be necessary due to SFO

  - The guard band Bguard,D2R would be necessary due to CFO for Device 2b

  - The guard band Bguard,D2R is not necessarily symmetric around the transmitted D2R signal

For D2R bandwidths, after offline and checking, the following is believed to be complete for 1SB. Black text is same as agreement, red text is to indicate to companies the difference here:

Proposal 3.8.1c(II) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where:

- …

- (Explanatory temporary note): When small frequency shift is applied, the power of upper half-part may or may not be higher than the lower half-part for both +FSx and -FSx parts

Proposal 3.8.1d(I): For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

Proposal 3.8.1e(I) For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

Proposal 2.6.1(I): Capture in the TR that for R2D multiple access:

- Time-domain multiplexing is the baseline.

- Code-domain multiplexing is not considered for all devices.

- Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1.

Table 6.1.1.x-1: Observations on the feasibility and necessity of FDMA for Device 2b with IF-ED or ZIF receiver

| Aspects to be considered for feasibility/benefit | Observations |
| --- | --- |
| Inventory completion time | Sources [Ericsson, LGE, Nokia, OPPO] state that FDMA is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. |
| Device implementation | Sources [Apple, DOCOMO, Ericsson, LGE, OPPO] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Sources [ITL, MTK, Spreadtrum, TCL, Huawei] thinks that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Sources [Ericsson] state that the larger R2D responses are harder for the devices to process in the case of TDMA+FDMA/TDMA only for D2R/R2D, respectively. |
| Spectrum utilization | Sources [ITL] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. |
| Coverage (in the case of single reader) | Sources [Huawei] state the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Sources [Ericsson] think the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture). |
| Reader implementation (in the case of single reader) | Sources [Huawei] state that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions. |
| Harmonized design for all devices | Sources [ASUSTek, CMCC, ETRI] state that it is not appropriate to include FDMA only for Device 2b, while Device 1 and 2a cannot support it. |

Conclusion 2.1.2a(II): For OFDM-based OOK waveform generation, no further study is needed.

Regarding the CP handling method TPs, FL refers to the sections further up to avoid lengthy copy-paste here:

Proposal: Adopt the TPs in Section 2.1.1.2.1, Section 2.1.1.2.2.

Similarly, propose to adopt the TPs for R2D FEC

Proposal 2.4.1c(II): For R2D repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [LG], [ETRI], [QC] and [IITK, IITM] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported:

  - Bit-level repetition

Positive observations

    - Source [Ericsson] state that bit level repetition can be studied if coverage enhancement of the R2D link is required.

    - Source [OPPO] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power.

    - Source [QC] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices.

Negative observations

    - Source [CMCC] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception.

    - Source [Vivo] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination.

  - Block-level repetition

Positive observations

    - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain.

Negative observations

    - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded.

  - Chip-level repetition

Positive observations

    - Source [CMCC] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps.

    - Source [IITK, IITM] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions.

Negative observations

    - Sources [Ericsson] and [CATT] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option.

D2R repetitions

Proposal 3.4.1a(I): For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Huawei], [Nokia], [Ericsson], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] and [DCM] that block level repetitions should be supported.

General observations

  - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition

  - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block.

  - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission.

  - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum.

  - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation.

Performance comparisons

  - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding.

  - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1.

  - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same.

  - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping.

  - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

Proposal 3.4.1b(I): For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Ericsson], [ZTE],] and [Samsung] that bit level repetitions should be supported.

  - Source [Huawei] state that for bit level type 1, the repetition data bit is decoded after FEC detection, and hence, the digital data bit cannot be combined with combining gain.

  - Source [ZTE] state that the performance of bit level type 1 repetition is degraded by ~3.5dB @ 10% BLER under the cases of AWGN and TDL-A channel when compared to bit level type 2.

  - Source [Huawei] state that since the bit level type 2 repetition is performed after FEC at the transmitter, at the receiver side, each bit would have a log-likelihood ratio (LLR) that can be combined and provided as the input for FEC decoding, thereby providing a better performance as compared to bit level type 1.

D2R FEC

Proposal 3.4.2a(II): For D2R FEC, include the following table in the TR:

| Option # | CC Design | Pros | Cons |
| --- | --- | --- | --- |
| Baseline | Constraint length 7 Code rate 1/3 | - [Vivo] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. - [Vivo] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. - [CATT] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. - [Lekha] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. - [Xiaomi] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 - [Xiaomi] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 |  |
| 1 | Constraint length 4 Code rate 1/2 – 1/4 | - [Ericsson] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes. | - [CMCC] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. |
| 2 | Constraint length 6 Code rate 1/2 – 1/6 | - [QC] Reduced reader and device complexity due to smaller number of shift registers. | - [QC] Code rate 1/2: Decoding performance is decreased by 0.6dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [QC] Code rate 1/3: Decoding performance is decreased by 0.4dB@1% BLER, when compared to baseline. - [QC] Code rate 1/4: Decoding performance is decreased by 0.2dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. - [QC] Code rate 1/6: Decoding performance is decreased by 0.15dB@1% BLER, when compared to baseline. |
| 3 | Constraint length 7 Code rate 1/2 – 1/4 | - [CATT] Code rate 1/2, TBCC: Decoding performance is increased by ~9dB@10% BLER, when compared to no CC but with Manchester line codes. - [CATT] Code rate 1/4: Decoding performance is increased by ~1.4dB@10% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is increased by 0.5dB@10% BLER, when compared to CC with code rate 1/2. - [QC] Code rate 1/4: Decoding performance is increased by 0.2dB@1% BLER, when compared to baseline. |  |
| 4 | Constraint length 8 Code rate 1/2 – 1/6 | - [CMCC] Code rate 1/2: Decoding performance is increased by ~0.22dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Mother code rate 1/4, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/4, code rate 1/4. - [ZTE] Mother code rate 1/3, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/3, code rate 1/4. - [ZTE] Mother code rate 1/2, code rate 1/6: Decoding performance is increased by ~1.7@10% BLER, when compared to constraint length 8, mother code rate 1/2, code rate 1/4. |  |

Proposal 3.4.2b(II): For D2R FEC, include the following observations in the TR, where k = constraint length and m = 1/code-rate:

- Performance of other {k, m} pairs decreased 0.15 – 1.4 dB when k<7, with reported benefit of reduced reader and device complexity due to shorter shift registers.

- Performance of other {k, m} pairs increased 0.22 – 1.8 dB when k>7, with an increase in the number of shift register resulting in increased device complexity.

- Performance of other {k, m} pairs decreased 0.3 – 2.5 dB when m<3.

- Performance of other {k, m} pairs increased 0.2 – 1.8 dB when m>3.

CRC

Proposal 4.1a(II): For CRC, capture the following observation in the TR:

- Sources as follows recommend that the length of the CRC used for A-IoT depends on:

  - Block size or TBS-like size, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, CATT, ZTE, DCM, ITL. Furthermore:

    - Source [Huawei] states that specifically for short messages, the CRC length can directly impact the transmission efficiency due to the overhead caused by the CRC.

    - Source [ZTE] state that CRC length depends on the TBS due to its impact on the transmission overhead and FAR performance. When the TBS is too small, the overhead of CRC-6 becomes quite significant.

    - Source [ITL] states that CRC length is fundamentally proportional to the size of the message.

  - Target false alarm rate or error detection performance, according to sources Huawei, Spreadtrum, TCL, CMCC, CATT, MTK, ITL. Furthermore:

    - Source Huawei evaluated that the probability of undetected errors is higher for CRC-6 as compared to CRC-16, and transmissions with CRC-16 performs better, for a given TBS=24, and source MTK say that a longer CRC would mean a better error detection performance.

  - Type of message, such as a command/reply function, to determine when no CRC is used, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, MTK, DCM. Furthermore:

    - Sources Huawei and Spreadtrum say that similar to RFID, only important messages, decided based on their functionality, such as configuration/control/command messages, are appended with CRC to increase their reliability, while other messages are not protected by CRC to save the overhead.

    - Source TCL proposes different CRC lengths for Paging-like/select/challenge messages that may or may not contain the device ID/group ID.

    - Source CMCC states that some messages such as a QueryRep-like command do not result in serious FAR issues if mis-detected, and hence the functionality of a message/command has to be considered.

  - Overhead caused by CRC, according to sources Huawei, Spreadtrum, CMCC, ZTE, MTK, Qualcomm

    - Sources Huawei, Spreadtrum, CMCC, MTK mention that longer CRCs have high overhead but enjoy better error detection performance, affecting the transmission efficiency. In order to balance the overhead and performance, Hence, short CRC can be used for short commands to reduce the false alarm rate and make the system more robust as compared to no CRC.

    - Source CMCC states that adding CRC to QueryRep-like commands which are sent repeatedly with a small payload would waste network resources, and hence the overhead should be considered.

    - Sources ZTE, Qualcomm states that the overhead of CRC decreases as the TBS increases, and that CRC-16 has higher overhead than CRC-6.

Proposal 4.1c(II): For the CRC generator polynomials, capture the following observations in the TR:

- Sources China Telecom, OPPO, Xiaomi, CATT, Samsung recommend that the same polynomials from TS 38.212 are reused, giving justifications:

  - Sources CATT, OPPO state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials.

  - Source Xiaomi states that the device complexity is increased when different polynomials are introduced for the D2R transmission.

  - Source China Telecom states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used.

- On the other hand, sources [ZTE],[Qualcomm] recommends that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, gCRC16D=D16+D11+D6+D5+1, incorporating the CRC-6 polynomial, giving justifications:

  - The proposed new CRC-16 polynomial nests the existing CRC-6 within its structure.

  - Using separate sets of circuits to implement the polynomials for CRC-6 and CRC-16 as per TS 38.212 requires a higher number of registers and XOR gates when compared to using the new CRC-16 polynomial.

  - FAR performance is the same for the legacy and proposed new polynomial for CRC-16, but the new polynomial reduces the complexity and power consumption of the device.

FL notes in case of plentiful online time:

- Line codes

- Small FS

## Thu PM

FL: I missed on TP on CRCs yesterday, which is stable. Hence:

Proposal 4.1c(I): For CRC, capture the following evaluation results in the TR:

- NOTE 1: Pud = Probability of undetected error according to [J. K. Wolf and R. D. Blakeney, “An exact evaluation of the probability of undetected error for certain shortened binary CRC codes,” Proc. Milcom ’88, vol. 1, pp. 287-292, 1988]

- NOTE 2: Overhead = CRC length / (TBS + CRC length).

| TBS | CRC-6 |  |  | CRC-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source | CRC overhead | PFA or Pud | Source | CRC overhead | FAP/FAR/Pud |
| 8 bits | HW | 43% |  | HW | 67% |  |
| 12 bits | E/// QC | 33% 33% | ~10^(-2) PFA | E/// QC | 57% 57% | ~10^(-5) PFA ~10^(-11) Pud @10-3 BLER |
| 16 bits | HW | 27% |  | HW | 50% |  |
|  | ZTE QC | 27% |  | ZTE QC | 50% 50% |  |
| 24 bits | HW E/// ZTE QC | 20% 20% 20% | ~10^(-6) Pud @ 10% BLER ~10^(-2) PFA | HW E/// QC | 40% 40% 40% | ~10^(-9) Pud @10% BLER ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 32 bits | HW ZTE QC | 16% 16% | ~10^(-5) Pud @10-3 BLER | HW ZTE QC | 33% 33% 33% | ~10^(-10) Pud @10-3 BLER |
| 40 bits | HW ZTE QC | 13% 13% | ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | HW ZTE QC | 29% 29% | ~10^(-10) Pud @10-3 BLER |
| 48 bits | E/// ZTE QC | 11% 11% | ~10^(-2) PFA ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | E/// ZTE QC | 25% 25% | ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 96 bits | HW E/// ZTE QC | 6% 5% 6% 6% | ~10^(-2) PFA ~10^(-4) Pud @10-3 BLER | HW E/// ZTE QC | 14% 14% 14% | ~10^(-5) PFA ~10^(-9) Pud @10-3 BLER |
| 128 bits | ZTE QC | 4% 4% | ~10^(-3) Pud @10-3 BLER | ZTE QC | 11% | ~10^(-9) Pud @10-3 BLER |
| 256 bits | HW QC | 2% 2% | ~10^(-3) Pud @10-3 BLER | HW QC | 6% 6% | ~10^(-9) Pud @10-3 BLER |

FL: For R2D line codes, some updates as explained in section 2.3.2.

Proposal 2.3(II): Capture the following observations in the TR for R2D line codes:

- It is reported by sources [Huawei], [CMCC], [EURECOM], [CATT], [ZTE], [Lekha], [Samsung], [ETRI], [Lenovo], [Apple], [LG] that PIE has a higher ratio of high to low voltage, which enables it to provide stable instantaneous RF energy to the device –

  - Sources [CMCC], [Samsung], [LG], [EURECOM] and [Lenovo] consider this a justification for studying PIE

  - Sources [Huawei], [CATT], [ZTE], [ETRI] and [Apple] acknowledge this aspect of PIE, but question its necessity.

- It is stated by sources [Nokia], [Ericsson], [Vivo], [CATT], [ZTE], [Apple], [DCM, [IITK, IITM] that, due to variable codeword lengths in PIE, scheduling time domain resources would be different to transmit the same amount of data due to the number of 0 and 1, as compared to the constant codeword lengths in Manchester codes.

- It is stated by sources [Huawei], [Spreadtrum], [CATT], [ZTE], [Lekha], [Apple] that, due to variable codeword lengths in PIE, the error detection of one information bit can result in error propagation for the detection of subsequent bits and therefore negatively affecting the detection of the overall data length, as compared to Manchester codes.

- Source [Ericsson] says that variable codeword lengths in PIE complicates A-IoT resource management, especially in in-band deployments when a reader needs to multiplex both NR and A-IoT communication.

- Source [CMCC] says that equal codeword length can overcome the cons of variable codeword length in PIE.

- According to sources [Nokia], [Ericsson], [CATT], [ZTE], [Lekha], [Samsung], [Apple], [DCM], transitions during Manchester code transmissions can be utilized for recovering the signal’s clock.

- Sources [Nokia], [Ericsson] say that the signal does not lose information or distort when fed to a DC blocker because each Manchester encoded symbol maintains a constant DC level, i.e. always the same average power, unlike if there was a long sequence of 1s or 0s.

- Evaluations are reported showing that Manchester codes perform better than PIE by 1dB to 6dB @10% BLER – [Huawei], [CMCC], [CATT], [ZTE], [IITk, IITM].

  - Source CMCC claims only a 1dB gain when comparing Manchester over equal length codeword in PIE, stating that the gain is negligible since the downlink coverage bottleneck is the receiver sensitivity and not the decoding performance.

For D2R line codes, this TP appears stable:

Proposal 3.3.1(I): For D2R line codes, the following observations are captured in TR 38.769:

- Sources [Huawei], [CMCC], [Vivo], [CATT], [ZTE], [Samsung] report that Manchester codes perform better than Miller codes by 0.5dB to 5.5dB @10% BLER.

- Sources [Huawei], [CMCC] [ZTE] report that Manchester codes perform better than FM0 by 0.2dB to 2.5dB @10% BLER.

  - According to sources [Vivo], [CATT],Manchester codes have a similar performance as FM0.

- Sources [Huawei], [Lekha] state that the power of Miller coding is more concentrated in the low-frequency region which is adjacent to the carrier wave frequency, as compared with FM0 code and Manchester code which leads to worse interference handling capability for Miller codes.

- Source [QC] state that performance of BPSK square wave is better than Miller/FM0 by ~3-4 dB @10%BLER with coherent demodulation.

For small frequency shifts, here the Round 2 proposals as updated:

3.3.2a: No further changes requested.

3.3.2b: Added company support names.

3.3.2c: No further changes requested.

3.3.2d: I will try the ZTE simplification.

Proposal 3.3.2a(I): For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

Proposal 3.3.2b(II): For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options:

- Sources [Futurewei], [Vivo], [CMCC], [QC], [LG], [ZTE], [Spreadtrum] and [Xiaomi] report that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

- Source QC report that the multiplication operation is by each bit modulating a square wave for the small frequency shift for device 1/2a and where each bit can modulate the carrier wave for device 2b.

- Source [InterDigital], [ZTE], [LG] report that the multiplication operation is an XNOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

Proposal 3.3.2c(II): For small frequency shifts in D2R, the following observations are captured in TR 38.769:

- Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is similar to bit-level repetitions within the same duration prior to Manchester encoding.

- Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a flipping of bits in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration.

- Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is similar to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit.

- Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worse performance compared with small frequency shift (Option 1) and small frequency shift (Option 2).

Proposal 3.3.2d(II): Regarding small frequency shift factor, capture in the TR that:

- For study purposes, the value of small-frequency-shift factor has been discussed as depending on details of design, which according to proposals considered could be e.g.

  - The repetition number of Manchester codeword

  - The M value of Miller in Figure 6-12 of UHF RFID standard

  - The number of square-wave periods

  - FFS FM0, if it supports small frequency shift

  - Note: the value of small frequency shift factor equals 1 when there is no small frequency shift

D2R multiple access TPs:

TDMA: No changes requested after round 2

FDMA: No changes requested after round 2

CDMA: Updated for most of the ZTE request, except some partial duplication.

(Also various grammatical tidy-up)

Proposal 3.6a(II): Add the following TP to the TR section on “D2R multiple access” for TDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [CMCC, CATT, Ericsson, FUTUREWEI] state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [DOCOMO, LGE, MTK] state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration (….)

Proposal 3.6b(II): Add the following TP to the TR section on “D2R multiple access” for FDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. (FL : Would insert here the TP from Proposal 3.6a) … NOTE: For the purposes of this section, X refers to the device’s SFO expressed as 10X ppm. According to sources [Ericsson, CMCC, CATT, CEWiT, DOCOMO, FUTUREWEI, Huawei, InterDigital], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [CMCC], and potentially increased power consumption for devices according to sources [CATT, Sony, OPPO]. For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. Companies to report how they perform multiplying for option 2. - For applying with Miller line codes, according to Figure 6-13 of [6]. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: Small frequency shifts for D2R are studied for the same potential purposes for MSK. It is observed that the performance of FDMA may be impacted by the following aspects: - The large SFO of device - Sources [Qualcomm, ZTE] state that large SFO (e.g. up to 105 ppm) produces higher BLER degradation due to inter-device interference than a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO. Source [ZTE] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - Source [Huawei] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤ 1dB) on the performance of FDMA between multiple devices. - Sources [vivo] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [Sony] think that the required guard band size increases for higher switching frequencies for passive devices. - The timing offset between devices - Sources [InterDigital] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - The maximum range of small frequency shift - Sources [MTK] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - The harmonics in the backscattered signal - Sources [vivo] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - The potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision - The number of multiplexed devices - Source [ZTE] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5~3dB performance loss from 6 FDMA-ed devices compared to single device. (….)

Proposal 3.6c(II): Add the following TP to the TR section on “D2R multiple access” for CDMA:

6.1.2.x D2R multiple access (…) According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Source [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Source [vivo] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Source [FUTUREWEI] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Source [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. - Source [Qualcomm] state that quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Source [Huawei] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Source [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. - In the case of relatively smaller SFO (e.g., X = 4), - Source [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. However, larger power variation severely degrades the capacity. The impact of CFO on the performance of CDMA is studied for Device 2b as follows. - Source [Huawei] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. The impact of timing offset between CDMed D2R transmissions on the performance of CDMA is studied for Device 2b as follows. - Source [Lenovo] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Source [OPPO] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Source [MTK] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Source [Samsung] think that the different propagation delays from devices may also degrades decoding performance. - Source [ZTE] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Source [Qualcomm] state that the larger power variation severely degrades the capacity of CDMA for msg-1. - Source [ZTE] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Source [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Source [Ericsson] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Source [MTK] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Source [TCL] think that CDMA leads to higher power consumption and lower data rate. - Source [vivo] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Source [OPPO] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Source [OPPO] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. - Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. - Source [ZTE] state that BPSK modulation for the CDMA improve the D2R transmission performance and multiplexing capacity compared with OOK based modulation.

D2R time domain definitions

- Thanks QC for the further explanation. Considering LG and QC’s points together, I think we can try adding this as another explicit example under Method 2. See below and section 5.3.

Proposal 3.7.1b(I): Capture in the TR that the following chip duration calculations have been studied:

- Method 1: Calculated according to the transmission bandwidth, and depending on small frequency shift factor. [CMCC] [Ericsson][Huawei][Spreadtrum][vivo][Xiaomi][ZTE]

- FL NOTE: The small frequency-shift factor is in Section 3.3.2, and FL would arrange the TR text suitably after agreements.

- Method 2: One of a pre-defined set of pulse time durations. [Lenovo][vivo]

- E.g., D2R chip duration is an integer multiple (or fraction) of R2D chip duration, where a reference SCS (e.g. the R2D SCS) is used as the basis for the value of D2R chip duration. [Sharp]

- E.g., D2R chip length is calculated from bit length, which can be one from a set of pre-defined values [LGE] [Qualcomm]

FL: Scrambling. We can try the proposal as refined by Qualcomm:

- Study of scrambling was not agreed for R2D link and D2R link during the study item. However, sources [Qualcomm], [OPPO] suggested to study it for interference randomization e.g., in the case where more than one reader operates simultaneously with overlapping coverage areas.

Proposal 3.1a(I): The D2R baseband waveform (as distinct from the internal or external carrier wave) is non-OFDM.

# Summary

# References

- R1-2407610	Discussion on physical layer design for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2407628	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2407638	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2407645	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2407669	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2407707	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum Communications

- R1-2407734	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2407862	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2407906	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2407970	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2408048	Discussion on general aspects of physical layer design	CATT

- R1-2408067	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408147	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2408206	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2408250	Discussion on general aspects of physical layer design	Sharp

- R1-2408272	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2408308	On General Physical Layer Design Considerations for Ambient IoT Applications	Lekha Wireless Solutions

- R1-2408410	General aspects of Ambient IoT physical layer design	Sony

- R1-2408466	On remaining general physical layer design aspects for AIoT	Apple

- R1-2408568	Discussion on general aspects of physical layer design	ETRI

- R1-2408599	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2408647	Considerations for general aspects of Ambient IoT	Samsung

- R1-2408672	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2408685	Discussion on physical layer design for Ambient IoT	Comba

- R1-2408701	General aspects of physical layer design	MediaTek Inc.

- R1-2408730	On the general aspects of physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2408765	Discussion on A-IoT physical layer design	ASUSTeK

- R1-2408787	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2408851	General aspects of physical layer design	Qualcomm Incorporated

- R1-2408875	Discussion on multiple access for D2R	LG Uplus

- R1-2408941	Discussion on General aspects of physical layer design of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408967	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2409005	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2406315	Consideration on general aspects of physical layer	Fujitsu

- R1-2407119	General aspects of physical layer design for Ambient IoT	ITL