3GPP TSG-RAN WG1 Meeting #118bis	Tdoc R1-2409286

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.1.2

Source:	Moderator (Ericsson)

Title:	Final summary of specification support for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#118bis for the agenda item 9.1.2, Specification support for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-242399 (revised WID) [1] below.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In the following, the delegates from participating companies are listed. Please check and add the contact information. We can use this list for future offline discussion, if such a need arises.

| Company/organization name | Contact person | Email of contact person |
| --- | --- | --- |
| Ericsson | Yufei Blankenship | yufei.blankenship@ericsson.com |
| Ericsson | Florent Munier | florent.munier@ericsson.com |
| Fujitsu | Xin WANG Yujia SHAN | wangxin@fujitsu.com shanyujia@fujitsu.com |
| MTK | harrison | harrison.chuang@mediatek.com |
| ZTE | Cong WANG | wang.cong8@zte.com.cn |
| CMCC | Yi Zheng Yongchang Liu | zhengyi@chinamobile.com liuyongchang@chinamobile.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| vivo | Huaming Wu | huaming.wu@vivo.com |
| vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| NEC | Chen Wei Miao Zhaobang | chen_wei@nec.cn miao_zhaobang@nec.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| OPPO | Zhihua Shi | szh@oppo.com |
| Pengcheng laboratory | Pingye XIANG | xiangpy@pcl.ac.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| TCL | Tianqi Wu | tianqi1.wu@tcl.com |
| LG Electronics | Jaehoon Chung | jhoon.chung@lge.com |
| Qualcomm | Mohammed Hirzallah (Ali) | mhirzall@qti.qualcomm.com |
| ETRI | Seungjae Bahng | sjbahng@etri.re.kr |
| Baicells | Xiang YUN Xiaonan WANG | yunxiang@baicells.com wangxiaonan@baicells.com |
| Apple | Kome Oteri | ooteri@apple.com |
| Fraunhofer | Mohammad Alawieh Georgios Kontes | mohammad.alawieh@iis.fraunhofer.de georgios.kontes@iis.fraunhofer.de |
| Sony | Basuki Priyanto Yujie Zhang | Basuki.priyanto@sony.com Yujie.zhang@sony.com |
| Samsung | Qi XIONG | Q1005.xiong@samsung.com |
| NTT DOCOMO | Luhua You Kosuke Shima | youlh@docomolabs-beijing.com.cn kousuke.shima.nr@nttdocomo.com |
| Intel | Chatterjee, Debdeep | debdeep.chatterjee@INTEL.COM; |
| Nokia | Dick Carrillo Melgarejo | dick.carrillo_melgarejo@nokia.com; |
| CEWiT | Shiv Shankar Dhivagar Baskaran | shivshankar@cewit.org.in dhivagar.b@cewit.org.in |
| Lenovo | Robin Thomas | rthomas7@lenovo.com |
| HW/HiSi | Thorsten Schier | Thorsten.schier@huawei.com |
| Honor | Guozeng Zheng | zhengguozeng@honor.com |
| IIT Madras | K Sai Prasanth | saiprasanth@5gtbiitm.in |
| PHY Wireless | Steve Thompson | steve.thompson@phywireless.com |
| Sharp | Makoto Kitahara Liqing Liu | kitahara.makoto@sharp.co.jp liu.liqing@sharp.co.jp |

# Model input

## Sample-based vs path-based measurements for model input

In the following, selected inputs from companies’ contributions are provided.

### Definition sample-based measurement (e.g., starting time of Nt window, parameter value range, selection rule)

#### Companies’ view from contribution

- vivo (R1-2407849) Observation 11: For sample-based reporting, determining their separate starting time of Nt consecutive samples for each TRP is more robust and can provide better positioning performance than a shared starting time for all TRPs. Proposal 11: The starting time of the list of Nt consecutive samples should be related to the timing of the first detected path for each TRP. Proposal 12: Allowing an offset or margin between the first detected path and the starting time of Nt consecutive samples can not only provide more flexibility, but also combat potential first-path estimation error caused by some non-ideal factors (e.g., UE timing error, imperfections in algorithm implementation). Observation 12: When taking the first path with better timing resolution than samples as the start of Nt consecutive samples, moving the off-grid first detected path (the start of Nt consecutive samples) to a grid point can achieve better positioning performance than positioning with off-grid samples. Proposal 13: Support to round down the off-grid first detected path (the start of Nt consecutive samples) to a grid point when taking the first path with better timing resolution than samples as the start of Nt consecutive samples. Proposal 14: The candidate Nt could be {32, 64, 128}. UE/gNB can report samples within the list of Nt consecutive samples or within the duration time of Nt consecutive samples. Proposal 15: The candidate Nt’ could be {8, 16, 32}. LMF can indicate one of the candidate Nt’ values to UE/gNB, and UE/gNB should report Nt’ samples within the list of Nt consecutive samples or within the duration time of Nt consecutive samples. Proposal 16: The candidate k could be {3, 4, 5}. The k’ value associated with sample-based reporting should be no larger than the k value indicated by LMF.

- Huawei, HiSilicon (R1-2407654) Proposal 1: For the study of sample-based measurements for Case 3b, we do not need to pursue tightly specified rule (e.g., determination of the selection window) or tightly aligned parameters (e.g., k, Nt, and Nt'). - As long as the rule and parameters could be specified to any extent, that would be helpful for alleviating the ambiguity across vendors (if exists). Proposal 2: For the study of sample-based measurements for Case 3b, the starting time of the selection window is based on the timing of the first detected path. Proposal 3: For the study of sample-based measurements for Case 3b, gNB can flexibly determine the value of k, Nt, and Nt' for the reported samples after receiving the signaled parameters from LMF. - The parameters signalled from LMF could serve as a reference for determining the parameters of reported samples by gNB implementation. Proposal 4: The maximum number of Nt' shall not be larger than 16 to protect proprietary channel estimation performance.

• CATT, CICTCI (R1-2408028) Proposal 11: For case 3b, a unified starting time of Nt samples is supported for multiple TRPs. Proposal 12: For case 3b, a unified starting time of Nt samples for multiple TRPs is determined by UL RTOA reference time T0+tSRS and an offset relative to the UL RTOA reference time. Proposal 13: For case 3b, the following one or two options of timing information reporting of sample-based measurements are considered: - Option 1: Sample-based reporting - Time offset: the time offset is the difference between the timing of first sample and UL RTOA reference time; - Bitmap: the bitmap is used to represent the timing information of Nt’ samples and the first bit is corresponding to the first sample of Nt’ samples; - Option 2: Path-based reporting - Resolution step of the timing information of path should be an integer multiple of sampling periods; - Path-based reporting may be enhanced to support reporting more samples. Proposal 15: For channel measurement reporting in case 2b/3b, the handling of time offset need to be consistent between training and inference phase.

• Fujitsu (R1-2408104) Proposal-1: For sample-based measurement, study LMF configuration-based method to indicate the starting point of the list of Nt samples and to aid UE’s selection of Nt'.

- Ruijie Networks Co. Ltd (R1-2408381) Proposal 1: For the definition of sample-based measurement, support to use the timing of the first detected path as the starting time, for one estimated channel impulse response. Proposal 2: For the definition of sample-based measurement, NW can configure/signal the value range of Nt subject to UE/device capability.

- NEC (R1-2408214) Proposal 1: For the definition of sample-based measurement, the value range of Nt' is advised to: - Less than or equal to 9, or - Large than or equal to 9 but less than or equal to 256. Proposal 2: For the definition of sample-based measurement, the value range of granularity factor k is advised to: - Large than or equal to 0 but less than or equal to 5. Proposal 3: The starting time of list of Nt samples should have the feature that at least can be align among different vendors rather than determined by UE’s/gNB’s methodology to avoid introducing new ambiguity again. Proposal 4: For case 3b, LMF determines consistent sampling parameters, e.g., value of k and Nt' to select the Nt' samples and informs them to data collection entities, e.g., TRP/gNB, for generating sample-based measurement. Proposal 7: Support to apply the same alternative, e.g., sample-based measurement, at least for case 3b, unless that new evaluation demonstrates additional benefits from applying the different alternatives for different cases. Proposal 36: For the definition of sample-based measurement, the rule for selecting the Nt' samples should be, selecting the strongest power from the estimated channel response in time domain.

- Sony (R1-2408402) Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

- Samsung (R1-2408631) Proposal 9. RAN1 supports the the value range of Nt is determined from UE reported value (i.e., UE capability) Proposal 10. RAN1 supports using a reference time and the offset to this reference time regarding the starting time of the Nt samples.

- Apple (R1-2408453) Proposal 2: Support modification of path-based feedback to support sample type feedback for case 2b and 3b (for inference) and all types (for data collection in model training). The existing path-based feedback specification can be modified to support sample-based model input by: Proposal 2-1: Timing is eqi-spaced on a grid with signalling that indicates the equal spacing size - The relative time difference of each reported path (nr-RelativeTimeDifference) is an integer multiple of a configured timing and represents a sample timing - The starting time of the list of Nt samples may be the 1st detected path - The value of k can range from [-6,5] as in the existing specification - k can be positive or negative Proposal 2-2: Increase the number of additional paths supported (the value range of Nt) to [16, 32, 64]. - Option 1: The samples reported may be Nt consecutive samples - Option 2: The samples reported are the the Nt strongest samples

- NTT DOCOMO, INC. (R1-2408774) Proposal 7: For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective， - For the report granularity k of sample-based measurement, legacy parameters Timing Reporting Granularity Factor and Timing Reporting Granularity Factor Extended in TS 38.455 can be considered as baseline, the value range can be reused, i.e., (0…5) and (-6…-1). - For the number of samples Nt’ which compose the measurement for sample-based measurement, legacy parameters Additional Path List and Extended Additional Path List in TS 38.455 can be considered as baseline, while large value can be introduced for sample-based measurement, e.g., the value range of Nt’ can be (32, 64, 128, …). Proposal 8: For the definition of sample-based measurement, the start of the list of Nt samples is determined by the first selected set of Nt’ samples. - Terms related to path-based measurement in current spec is rephased by the term of sample-based measurement.

- vivo (R1-2407849) Proposal 8: It is necessary to further specify the rules of selecting samples from channel measurement if sample-based measurement is specified. Proposal 9: As for the rules of selecting samples from channel measurement, at least selecting earliest Nt’ samples and selecting strongest Nt’ samples are feasible, and can be considered as a start point.

- Xiaomi (R1-2407951) Proposal 4：Define the starting time of the timing window as the reference time for measurement for sample-based input

- CMCC (R1-2407893) Proposal 2: For selecting Nt' samples by UE, assuming that these Nt' samples are the strongest power samples in the sampling window or introducing a threshold could be considered. Proposal 3: For selecting Nt' samples, entity derive AI model could have opportunity to report its recommended number of samples.

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 2: The starting time of the list of Nt samples is determined by the first detected path in time in the Nt window

- ETRI (R1-2408559) Proposal 2: For UE-sided models (Case 1/2a), if sample-based measurement is adopted as the model input, the UE-side determines the parameters (Nt, Nt’, k). Proposal 3: For gNB-sided model (Case 3a), when both training and inference are performed on the gNB-side, the model input format is determined by the gNB manufacturers. Proposal 4: For gNB-sided model (Case 3a), if sample-based measurement is adopted as the model input, the gNB-side determines the parameters (Nt, Nt’, k). For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective • LMF can signal parameter values of (Nt, Nt') to gNB via NRPPa. • The timing granularity parameter k is implicitly determined through SRS configuration.

- Sharp (R1-2408609) Proposal 2: For sample-based measurement of the channel measurement representation, both Option 1 and Option 2 are supported. - Option 1: the start of the list of Nt samples is based on a reference timing - Option 2: the start of the list of Nt samples is determined by the first detected path in time Proposal 3: An LMF configures the start of the list of Nt samples is whether determined by the first detected path in time or based on reference time. Proposal 4: For AI/ML based positioning, Legacy-like format and Bitmap format should be supported for channel measurement for sample-based measurement.

- CEWiT (R1-2408923) Proposal 2: Support unified sample based approach where input measurement is truncated channel measurement samples rather than selecting/subsampling paths based on strongest power.

- TCL (R1-2408267) Proposal 8: For measurement by UE, down-select the rule for selecting the Nt' samples: a. The Nt' samples with the highest power are selected. b. The first Nt' samples are selected.

- MediaTek Korea Inc. (R1-2408903) Proposal 2c-1: There is no need to additionally define the start of Nt consecutive samples in order to select Nt’ samples. In legacy, the UL-RTOA is within the range between -0.5ms and 0.5ms from the reference time. Then the Nt’ samples can be simply chosen within this range

- OPPO (R1-2408159) Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for UE-assisted/LMF based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - The existing reporting (from UE to LMF or gNB to LMF), which are path-based measurement, can be used as the input of LMF-side model - It can be done by LMF’s implementation. Thus, there is no RAN1 specification impact - If the detailed scheme for sample-based measurement and the performance gain is justified, it can be supported as an optional UE feature for Case 2b / Case 3b.

- Tejas Network Limited (R1-2407747) Proposal-2: Based on different deployment scenarios, the Nt∈{32, 64, 128, 256} can be supported. Proposal-3: The existing values of k supported for path-based measurements can be reused for sample-based measurements as well. Proposal-4: The value of Nt' should be a fraction of the window size (Nt) given by Nt'=αNt where α should be either selected by the TRP and signalled to the LMF or vice-versa. Proposal-5: The parameter i should be signalled by the LMF to TRP for LMF side model as the variation in the value of α between the training and inference phase can results in inconsistencies. Proposal-6: In outdoor scenarios, the value of n0​ may need to accommodate a significantly wide range, as 5G networks can connect with UEs located up to 100 km away from the TRPs. Proposal-7: In indoor scenarios, the values of n0 can lie in the interval 0<n0<512 for dISD=50m.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 14: At least for case 3b, the procedure for determining the number of samples and timing granularity k for sample-based measurement report follows the current framework, while the selection of timing granularity should follow LMF’s configuration. Proposal 15: Support LMF configuring the value of Nt for sample-based measurement, wherein the starting point of Nt can be determined based on one of the following options: • Option 1: the measurement report of Nt samples is determined based on the first detected path in time. • Option 2: the measurement report of Nt samples is determined based on the first path whose power is stronger than a configured threshold. • Option 3: the measurement report of Nt samples is configured by LMF. • Option 4: the measurement report of Nt samples is determined by TRP. Note: the candidate values of Nt can be {64, 128, 256}.

- Nokia (R1-2408545) Proposal 31: When sample-based representation is used, LMF configures the UE/gNB to determine N’t and choose sample selection strategy/rule-based for a fair trade-off between positioning requirements and signaling overhead.

- Lenovo (R1-2408429) Proposal 2: Nt samples may be captured with a sampling window with a configurable start time and sample window length, where all Nt samples fall within the window. FFS whether start time of window is defined with respect to a reference time. Proposal 3: The value range of Nt’ samples need to take into account total number of continuous samples Nt, i.e. the condition Nt’ ≤ Nt for sample-based measurements should be maintained. Proposal 4: Legacy values for the reporting range factor (k) may be used as a starting point, while the value range (v) of [-v×Tc, v× Tc], e.g., v = [-985024: 985024] or other min/max values, may be further discussed. Proposal 7: RAN1 to at least support the following selection rules for sample-based measurement: • Sample selection based on configured highest path power threshold or • Sample selection between min and max path power FFS whether selection rules also apply to path-based measurements for selecting up to N additional paths.

- Ericsson (R1-2407649) Proposal 6 For the definition of sample-based measurement, the timing grid for the Nt consecutive samples is determined by the reference time Tref (seconds), together with the timing granularity T. Proposal 7 The starting time of the Nt consecutive samples is the timing of the first detected path, quantized to the timing grid. Proposal 8 For Case 3b (and 2a if needed), support {Nt, Nt'} parameter combinations: {9, 9}; {16, 16}, {16, 9}; {32, 16}, {32, 9}; {64, 32}, {64, 16}, {64, 9}. Proposal 9 For Case 3b (and 2a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

#### 1st round discussion

Regarding the definition of sample-based measurements for model input, the following agreements were made in RAN1#117 (May 2024) and RAN1#118 (August 2024).

Agreement (RAN1#118) For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement (RAN1#118) For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. ===================================================================== Agreement (RAN1#117) Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement (RAN1#117) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

The remaining issues are mainly: (a) how to define the grid to generate the samples; (b) how to define the starting point of Nt consecutive samples; (c) the candidate values for parameters Nt, Nt’, k.

Regarding the issue of defining the starting point of Nt consecutive samples, companies had intensive discussion in RAN1#118. In contributions to RAN1#118bis, companies performed further evaluation and shared further analysis. Extensive evaluation results and careful analysis can be found in several contributions including: vivo (R1-2407849), Ericsson (R1-2407649). Please check companies’ contributions for the detailed evaluations. The investigation provided by companies are very valuable and highly appreciated!

Based on companies’ input, companies’ view are captured in the table below (Please update/correct if your view is not captured correctly).

How to define the starting point of the Nt consecutive samples?

- starting point of the Nt consecutive samples = First detected path

      - Supported by (10 sources): vivo, Ericsson, Huawei/HiSilicon, Apple, Fraunhofer IIS/Fraunhofer HHI, Sharp, Ruije, Docomo

- starting point of the Nt consecutive samples = Reference time

      - Supported by (3 source): Sharp, Xiaomi, TCL

- starting point of the Nt consecutive samples = Reference time – 0.5 ms (R1-2408903: the Nt’ samples can be simply chosen within the range between -0.5ms and 0.5ms from the reference time.)

      - Supported by (1 source): MediaTek

- A reference time and an offset to the reference time

      - Supported by (3 source): CATT/CICTCI, Samsung

- Configurable

      - Supported by (2 source): Lenovo, Fujitsu

Other: 		Select one of 4 options (ZTE)

Extensive evaluation results were provided to demonstrate that Option A is effective and robust, see vivo (R1-2407849), Ericsson (R1-2407649). Several companies also discussed that the timing of the detected path is likely not on the grid, hence it should be rounded down to the grid. Based on companies’ input, it is reasonable to adopt Option A.

Proposal 2.1.1.2-1

For the definition of sample-based measurement, for the measurement of one channel impulse response between a pair of UE and TRP, the starting time of the list of Nt samples is:

- the timing of the first detected path, which is rounded down to the timing grid with the granularity of T.

|  | Company |
| --- | --- |
| Support | DOCOMO |
| Not support | CATT/CICTCI,HwHiSi, Nokia |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | We also provide evaluation in our tdoc (R1-2408028) and show that Option A will lead to 7% performance degradation when Nt’=5. For better discussion we prefer to make the following options on the table for further discussion: - Option A: starting point of the Nt consecutive samples = First detected path; - Option B: starging point of the Nt consecutive samples is pre-defined/configured based on the reference time. |
| ZTE | Support CATT’s suggestion. Option A has been discussed extensively in the last meeting, indeed, there are several drawbacks, i.e., lose some useful information before the first detected path, depends on the receivers’ implementation, etc. Option E is our first preference, and option D specify the detailed configuration of option D. My suggestion is merge option E and option D. |
| New H3C | Share the similar view with CATT |
| HW/HiSi | Before discussing further the definition of sample based reporting, we would like to raise the issue whether there is any ambiguity for Case 3b. As described in our paper, we do not see any ambiguity as long at the method used by each gNB does not change between training and inference. From R1-2407654: As a comment to proposal made by the FL here, it should first be agreed whether the starting time of the list of Nt samples is based on the first detected path or not, before discussing any further details. As such, the mentioning of the timing grid with the granularity of T and of the rounding of the timing of the first detected path are unnecessary. The related RAN1#118 agreement already states that the Nt samples have timing granularity T. Thus, we propose the following simplified version of the proposal: Proposal 2.1.1.2-1 For the definition of sample-based measurement, for the measurement of one channel impulse response between a pair of UE and TRP, the starting time of the list of Nt samples is: based on the timing of the first detected path, which is rounded down to the timing grid with the granularity of T. |
| Nokia | Including the term “rounded down“ is not useful if the intention was to reduce potential ambiguities with detailed definition of Sample based. |
| Apple | Okay with Huawei’s update |
| NEC | If we use ‘the first detected path’ as starting time, the inconsistency will be caused if training data collected from PRU, and inference data from the UE, where the UE and PRU use the different methods(up to its implementation) to identify ‘the first detected path’. |
| Qualcomm | This proposal lays down some unnecessary details. The first sample can be initiated with respect to a detected path. We also have never introduced a timing grid in earlier definitions. We also need not to mention UE or TRP at this stage. Updated proposal to align wording with the original definition: Proposal 2.1.1.2-1 (updated) For the definition of sample-based measurement, for the measurement of estimated channel response in time domain one channel impulse response between a pair of UE and TRP, the starting time of the list of Nt samples is: - the timing of the first detected path, which is rounded down to the timing grid with the granularity of T. |
| OPPO | Fine with HW/HiSi’s update. |
| Sharp | We have similar view with CATT/CICTCI. Option C and D (and may also Option E) can be sub-option of Option B, and we prefere both of Option A and Option D. Option B commented by CATT is supported by at least 6 companies. Thus, we should have further discussion for this topic. |
| Fujitsu | We think it is not a good choice to couple the sample-based measurement with the first detected path. If so, its advantage of reducing the implemenation ambiguites over path-based method become unclear. We tend to share the similar view as that of ZTE: Merge Option D and Option E can be considerred as the way for further study. |

Regarding the issue of defining the grid for generating the samples, Ericsson proposed that “the timing grid for the Nt consecutive samples is determined by the reference time Tref (seconds), together with the timing granularity T.” Several other companies (e.g., Sharp, MediaTek, Samsung, CATT/CICTCI, Xiaomi, TCL) also mention reference time as related to determining the start of Nt consecutive samples. Thus it is reasonable to consider that reference time is used in defining the grid for generating samples.

Proposal 2.1.1.2-2

For the definition of sample-based measurement, the timing grid for the Nt consecutive samples is determined by the reference time together with the timing granularity T.

- The reference time defines a grid point.

- The timing granularity T defines the duration between two adjacent grid points.

Note: for Case 3b, the reference time has agreed to be the existing UL RTOA reference time T0+tSRS.

|  | Company |
| --- | --- |
| Support | NEC, DOCOMO, TCL |
| Not support | Hw/hiSi |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | OK. |
| ZTE | Will we discuss the case if the starting time FFT window is not on the timing grid? Or it depends on UE/TRP’s implementation to fit the grid? |
| New H3C | OK |
| HW/HiSi | We wonder whether there is a need for this agreement given the RAN1#116b agreement, that states that the timing information for Case 3b is represented relative to the existing UL RTOA reference time. It is our understanding that the UL RTOA reference time would then correspond to a grid point, similar as how in legacy path reporting the UL RTOA reference time is used as reference for the possible quanitized timing values that can be reported. Irrespective of the timingReportingGranularityFactor k, which determines the timing granularity T, a reported timing of UL RTOA=0 in legacy would correspond to a timing information equal to the UL RTOA reference time. The definition of sample-based measurements timing grid can follow exactly the same understanding as for the path reporting in legacy. |
| Qualcomm | We think this proposal is unnecessary and there are precedence details not yet discussed. These are some details: - First, we have not agreed to have a timing grid in the original or subsequent definitions. Let’s first discuss necessity of such a timing grid and potential options. - In addition, it should also be noted that the reference time discussed in the original sample-based definition (RAN1#117) (see below) was meant for timing representation when reporting. It should not be confused with any potential grid alignment and initialization. - For the Note on Case3b, this can be discussed in a separate proposal. Agreement (RAN1#117) Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. |

Regarding the value range of Nt, Nt', k, the following proposals are based on the values that have more support based on companies’ contributions. FL’s understanding is, they also have evaluation results from companies to demonstrate the achievable performance.

Proposal 2.1.1.2-3

For the definition of sample-based measurement,

- The candidate values for Nt are: 16, 32, 64, 128;

- The candidate values for Nt' are: 9, 16, 32, 64; Note: Nt'<= Nt.

- The candidate values for k are: 3, 4, 5.

|  | Company |
| --- | --- |
| Support |  |
| Not support | HW/hiSi, Nokia |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Can be discussed latter. |
| ZTE | For Nt, 256 could be a possible candidate value, as most companies provide evaluation result on Nt=256. And for k, all Rel-16 candidate values can be considered, i.e., k=0~5. |
| HW/HiSi | We have a strong concern that a too large chosen value of Nt’ discloses proprietary implementation information, e.g. frequency domain channel estimator (interpolation method, frequency domain filter). Therefore, the upper limit of Nt’ should be nt more than 16. Regarding Nt, evaluations have shown that 64 is sufficient. Additionally, the gNB may have better understanding about the radio conditions than the LMF. For example a too large Nt’ may in some conditions result in the reporting of noise values leading to a performance degradation, and a too low number may in some cases result in too little information. In such sense the parameters provided by the LMF should be seen as reference that can be modified by the gNB. Proposal 2.1.1.2-3 For the definition of sample-based measurement, signaled from LMF - The candidate values for Nt are: 16, 32, 64, 128; - The candidate values for Nt' are: 9, 16, 32, 64; Note: Nt'<= Nt. - The candidate values for k are: 3, 4, 5. Note: The gNB can flexibly determine reported the value of k, Nt, and Nt' for the reported samples after receiving the signaled parameters from LMF. |
| Nokia | This topic may be discussed only in case Sample based is agreed. |
| NEC | For value range of k, we also think legacy value can be reuse, i.e., {0, 1, 2, 3, 4, 5} |
| Qualcomm | This proposal has signaling impact and not directly related to clarifying the definition. Postpone discussion until sample-based measurement gets supported. |
| DOCOMO | For timing reporting granularity k, legacy values can be considered as a starting point. |
| TCL | For the candidate values for k, the determination of k for UE measurement can be discussed first. |
| OPPO | This can be discussed after the definition of sample-based measurement is clear. |

#### 3rd round discussion

Regarding Proposal 2.1.1.2-2, Huawei and QC comments indicate that this proposal is unnecessary, since it is implied by the agreement already made. With this understanding, there is no need to further discuss it.

Proposal 2.1.1.3-1

For the definition of sample-based measurement, for the measurement of an estimated channel response, RAN1 study the following options for determining the starting time of the list of Nt samples:

- Option A. the starting time of the list of Nt samples is determined by the timing of the first detected path.

- Option B. the starting point of the Nt consecutive samples is determined by a fixed offset relative to the reference time.

  - Proponent companies provide details on the value of the fixed offset.

- Option C. the starting point of the Nt consecutive samples is determined by a configured offset relative to the reference time.

  - Proponent companies provide details on the value range and signaling details of the configured offset.

Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time.

|  | Company |
| --- | --- |
| Support | TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Fine to leave the door open. Is option B a new option? In previous we didn’t see such kind of option where a fixed offset works for all scenarios/TRPs. |
| CATT, CICTCI | OK |
| Qualcomm | We are ok to study these options (with assumption of down selection). - The note seems unnecessary because it can preclude potential options that proponants can provide for details. Let’s delete it. - The main senstence needs to be only related to definition. Details on measurement type (e.g., CIR) or mapping to channels (e.g., TRP and UE channel) can be discussed after being supported: For the definition of sample-based measurement, for the measurement of one channel impulse response between a pair of UE and TRP, RAN1 study and downselect the following options for determining the starting time of the list of Nt samples: |
| HW/HiSi | For clarification: E.g. for Option A, do you mean with „starting time is determined“, that the first detected path is the anchor (fixed reference point) for the starting time, or do you mean that it is the earliest sample that can be reported? In out view, samples prior to the first deteted path could also be reported, and how many of them could be up the gNB to decide. This is also supported already in legacy path reporting, where the additional paths can be earlier or later than the first path which is rpeorted in absolute time. Agree with QC on to focus the discussion on the definition, and support their modification of the main sentence. Other Suggested modification: Option A. the starting time of the list of Nt samples is determined by gNB by the timing of the first detected path and can be earlier than the first detected path. |
| vivo | We have a comment on the option A. In the first round of discussion, there’s a sub-bullet „the timing of the first detected path, which is rounded down to the timing grid with the granularity of T.“ Why is that removed? The round-down operation is to assure the first detected path will be for sure included inside the Nt samples. This is very important as the first detected path is most likely the LOS path. Current wording „determined by„ is to vague and not clear how the start time is decided. If leave to implemenation, then we may lose the benefits of an aligned sample selection compared to existing path based measurement. We propose to following revision to option A. - Option A. the starting time of the list of Nt samples is determined by the timing of the first detected path rounded down to the nearest timing granularity of T relative to a reference time. Regarding the note, we think it’s important to keep the note as Nt and Nt‘ samples need to be on the time grid with granularity T and it applies to all options. |
| OPPO | Fine with the FL proposal. For option A, since the time for the detection of first path can be different between different pairs of TRP and UE, I think it is better to keep the wording ‘for the measurement of one channel impulse response between a pair of UE and TRP‘. |

Regarding the value range of the parameters, we can try the candidate values that are generally acceptable, and other values can be further discussed and added later (if needed).

Proposal 2.1.1.3-2

For the definition of sample-based measurement,

- The candidate values for Nt include at least: 16, 32, 64;

- The candidate values for Nt' include at least: 9, 16; Note: Nt'<= Nt.

- The candidate values for k include at least: 3, 4, 5.

Proposal 2.1.1.3-2A

For the definition of sample-based measurement,

- The set of candidate values for Nt include at least: 16, 32, 64, 128;

- The set of candidate values for Nt' include at least: 9, 16, [32]; Note: Nt'<= Nt.

- The set of candidate values for k include at least: 0…5.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Honest speaking, a maximum of Nt‘=16 and Nt=64 seems too small that we see no much difference than path based measurement (1+8). Since it is not so urgent, we can discuss later whether Nt=128 and Nt‘=25 is acceptable. |
| Qualcomm | Ranges has potential signaling impact. Let’s postpone discussion until approach is supported. |
| HW/HiSi | The gNB may have better understanding about the radio conditions than the LMF. For example a too large Nt’ may in some conditions result in the reporting of noise values leading to a performance degradation, and a too low number may in some cases result in too little information. In such sense the parameters provided by the LMF should be seen as reference that can be modified by the gNB. Updated Proposal 2.1.1.3-2 For the definition of sample-based measurement, signaled from LMF - The candidate values for Nt include at least: 16, 32, 64; - The candidate values for Nt' include at least: 9, 16; Note: Nt'<= Nt. - The candidate values for k include at least: 3, 4, 5. Note: The gNB can flexibly determine reported the value of k, Nt, and Nt' for the reported samples after receiving the signaled parameters from LMF. |
| Nokia | In case Sample based is agreed, we can come back to this topic. It is not related to the definition, but it is defined to details that may be specified. By now our suggestion is to delay the discussion. |

#### 4th round discussion

Thursday offline:

Proposal 2.1.1.4-1

For the definition of sample-based measurement, for an estimated channel response, RAN1 study the following options for down-selection for determining the starting time of the list of Nt samples:

- Option A. the starting time of the list of Nt samples is the timing of the first detected path (quantized with timing granularity T).

- Option B. the starting point of the Nt consecutive samples is determined by a fixed offset relative (quantized with timing granularity T) to the reference time.

  - Proponent companies provide details on the value of the fixed offset.

- Option C. the starting point of the Nt consecutive samples is determined by a configured offset (quantized with timing granularity T) relative to the reference time.

  - Proponent companies provide details on the value range and signaling details of the configured offset.

Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Qualcomm | The quantization mentioned in proposal would not be needed in some scenarios. For example, if reference time is first path ToA of the reference TRP, then Nt starting point for the reference TRP measurements will be same as ToA and thus quantization will not be needed. The second part in the Note is also repetitive to quantization meantioned earlier. Let’s delete it. Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time. Let’s consider careful wording: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (quantized with timing granularity T). If reference time is timing of first detected path of a reference TRP, no quanitization is needed for the starting time of the list of Nt samples corresponding to the reference TRP. - Option B. the starting point of the Nt consecutive samples is determined by a fixed offset relative (quantized with timing granularity T) to the reference time. If reference time is timing of first detected path of a reference TRP, no quanitization is needed for the starting time of the list of Nt samples corresponding to the reference TRP. - Proponent companies provide details on the value of the fixed offset. - Option C. the starting point of the Nt consecutive samples is determined by a configured offset (quantized with timing granularity T) relative to the reference time. If reference time is timing of first detected path of a reference TRP, no quanitization is needed for the starting time of the list of Nt samples corresponding to the reference TRP. - Proponent companies provide details on the value range and signaling details of the configured offset. |
| OPPO | Since the time for the detection of first path or the offset value can be different between different pairs of TRP and UE, it is not unified among TRPs. I think it is better to add the wording ‘between a pair of UE and TRP‘ in the main bullet. |

Proposal 2.1.1.4-2

For the definition of sample-based measurement,

- The set of candidate values for Nt include at least: 16, 32, 64, 128;

- The set of candidate values for Nt' include at least: 9, 16, [32]; Note: Nt'<= Nt.

- The set of candidate values for k include at least: 0…5.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Qualcomm | We do not support the current wording of the proposal. Not a priority for discussion. These values can be different for different cases and depend on the deployment scenario. They can also subject to device capability. Let’s wait, they can be discussed if there is support for sample based measurements. |

### Enhancements for path-based measurement

#### Companies’ view from contribution

- Huawei, HiSilicon (R1-2407654) Proposal 7: For Case 3b, support enhancing the legacy reporting by increasing the number of reported delay and power values from the gNB to LMF.

- NEC (R1-2408214) Proposal 5: Enhancement on increasing number of path based on legacy path-based measurement for AI/ML positioning is necessary, if path-based measurement will be supportted.

- Tejas Network Limited (R1-2407747) Proposal-8: For reporting path-based measurements, the existing framework from release-17 standards should be utilized. If required, the number of additional measurements should be increased from 8 to a higher value.

- Lenovo (R1-2408429) Proposal 5: Further consideration may be given to extend the maximum number of reported additional paths for path-based measurements. FFS the value of the maximum number of reported paths.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 16: For path-based measurement, enhancement on the number of reported paths is not needed.

- vivo (R1-2407849) • From our simulation observation, less than 8 paths are detected for most of channel measurements (e.g., 3 detected paths for 20MHz bandwidth on average), especially when beamforming (16 horizontal beams and 2 vertical beams) is considered. In other words, there’s no enough path physically that could be detected to begin with. • The performance gain provided by reporting more paths in addition to the existing 8 additional paths is questionable, if the path selection rule is not aligned and the accuracy of estimated paths is not guaranteed.

#### 1st round discussion

On the path-based measurement, the following agreement was made in RAN1#117:

Agreement (RAN1#117) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Regarding the potential enhancement on the number of reported paths, only vivo conducted simulations to check the potential enhancement, both in RAN1#118 and RAN1#118bis. According to vivo (R1-2407849), “less than 8 paths are detected for most of channel measurements (e.g., 3 detected paths for 20MHz bandwidth on average)”. “In other words, there’s no enough path physically that could be detected to begin with.” ZTE also argued that “enhancement on the number of reported paths is not needed”.

So far, no proponents of path-based measurements have any evaluation results to prove the need or benefit of more reported paths.

Thus, it is reasonable to conclude that no enhancements on the number of reported paths are to be introduced in Rel-19, if path-based measurements is supported for determining model input.

Proposal 2.1.2.2-1

Rel-19 does not introduce enhancements on the number of reported paths, if path-based measurement is supported for determining model input.

|  | Company |
| --- | --- |
| Support | ZTE, DOCOMO |
| Not support | InterDigital, Hw/HiSi, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | At this point, this proposal is not necessary. We should study whether more paths are needed for existing path-based measurement. |
| CATT, CICTCI | OK. |
| New H3C | OK |
| HW/HiSi | Path based and sample based reporting are very similar in their characteristics. During previous meetings it was said that sample based may be one version of path based reporting. If sample based reporting can support more than 8 samples, then path based reporting should support this. This can be a general enhancement and not only needed to determine the model input. Proposal 2.1.2.2-1 Rel-19 does not introduce enhancements on the number of reported paths, if path-based measurement is supported for determining model input. |
| Nokia | In our view, there is not concensus to do this agreement/conclusion. Our suggestion is to study if more paths are needed to complement the legacy, in this way, more companies will have the chance to complement the current analysis. |
| Apple | ## From our simulations (R1-2408453), we found the following: From the results, we see that the overhead of path-based measurement input (6.41 24 samples on average for 100MHz) is less than that of sample-based measurements (16.5393 samples on average for 100 MHz) although some of the sample-based measurements should be able to be dropped with minimal performance impact. Given that this is an average, increasing > 9 may not be a bad thing to do just in case. |
| Qualcomm | We can still further discuss enhancements on number of paths (if necessary). We do not support the proposal. |
| OPPO | Enhancement of the number of paths can be further studied. |

### Whether to support sample and/or path based measurements

#### Companies’ view from contribution

- vivo (R1-2407849) Observation 8: Sample-based measurement performs better than path-based measurement for AI/ML based positioning, regardless of the path-extraction methods. Specifically, • when there are sufficient number of TRPs and the bandwidth is large enough (e.g., 18 TRPs, 100MHz), their performance is similar. • when there are limited number of TRPs and the bandwidth is limited (e.g., 6TRPs, 20MHz), the performance of sample-based measurement is significantly better than its counterpart. Proposal 10: Specify sample-based channel measurement reporting for Case 2b and 3b of AI/ML based positioning. Observation 10: Sample-based measurement can always provide better positioning performance than path-based measurement for AI/ML based positioning under various timing error assumptions.

- Google (R1-2407989) Proposal 3: Support the NW to configure whether the UE should report the sample-based or path-based measurement results.

- CATT, CICTCI (R1-2408028) Proposal 10: At least for case 3b and 2b, support sample-based channel measurements as the AI/ML model input. Proposal 14: For case 3b, if both sample-based reporting and path-based reporting are supported, the choice of sample-based reporting and path-based reporting is up to gNB/TRP selection, or up to LMF indication.

- Baicells (R1-2408309) Proposal 1. Support sample-based measurements as model input at least for Case 3b. Preclude path-based measurements in R19.

- Sony (R1-2408402) Proposal 6: Support sample-based time domain channel measurements as the AI/ML model input.

- Intel Corporation (R1-2408290) Proposal 9: • Support path-based measurement for representation of time domain channel measurements in AI/ML based positioning with potential enhancement of the number of reported paths. Note, see Observation 2: • Sample-based approach is a specific implementation of the path-based approach which is already supported in the existing specifications with potential enhancements on the number of reported paths (if needed)

- Qualcomm Incorporated (R1-2408838) Proposal 4: In AI/ML positioning Case3b, for model input measurements, support Alt-B path-based measurements as the default and baseline measurement reporting for LMF-side model. - FFS: necessity of Alt-A sample-based measurements.

- InterDigital, Inc. (R1-2408522) Proposal 21: Conclude whether to introduce sample-based measurements for Case 1 Proposal 22: For case 3b and case 1, support Alternative (b) (path based measurements) for representation of time domain channel measurements and reuse path-based reporting with the granularity up to k=-6 as specified in Release 18. Proposal 22: For case 3b and case 1, support Alternative (b) (path based measurements) for representation of time domain channel measurements and reuse path-based reporting with the granularity up to k=-6 as specified in Release 18. Proposal 23: For case 3a, support Alternative (b) (path based measurements) since an AIML model may be able to estimate ToA that is not aligned with sampling periods.

- Xiaomi (R1-2407951) Proposal 1: - For Case 2b and Case 3b, support sample-based input. - For Case 1, Case 2a and Case 3a, it is up to implementation to determine the model input Proposal 3: If there is need for the model input specification, specify multiple values of Nt or N't, for configuration.

- Samsung (R1-2408631) Proposal 8. Path-based measurement could be considered with potential overhead reduction.

- Apple (R1-2408453) Proposal 1: Support both path-based measurement input and sample-based measurement input. • Sample based model input can be viewed as a special case of path-based input with equi-spaced timing samples Proposal 4: On the applicability to the different sub-use cases in the LCM stages of AI/ML positioning: Path/Sample Input for different Sub-use cases Training Measurement Input; Inference; Monitoring. Case 1; Specify both; N/A; Depends on location and monitoring input required. Case 3a; Specify both; Specify both. Case 3b; Specify both; N/A.

- NVIDIA (R1-2408391) Proposal 2: Support both of the following alternatives for time domain channel measurements for AI/ML based positioning: • Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. • Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

- NTT DOCOMO, INC. (R1-2408774) Proposal 6: Support sample-based measurement for AI/ML based positioning in Rel-19. - If both sample and path-based measurements are supported, LMF indicates whether sample-based or path-based measurement is applied subject to UE capability

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 1: Support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report.

- Lenovo (R1-2408429) Proposal 6: On the time domain channel information representation, consider support for a hybrid approach in relation to the cases: - For Cases 1, 2b, and 3b: Support sample-based time domain representation - For Cases 2a and 3a: Support legacy-based path timing time domain representation - Note1: For each of the above use cases, use of one type of time domain representation (e.g. sample-based approach) in training and another type of time domain representation (e.g. path-based approach) during inference is not supported. - Note2: Discussion on Cases 2a and 2b may be revisited in RAN1#120.

- CEWiT (R1-2408923) Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, Time domain sample-based measurement reporting is preferred.

- CMCC (R1-2407893) Proposal 1: Sample-based measurements for AI/ML positioning is slightly preferred.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 17: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, support sample-based measurements.

- Nokia (R1-2408545) Proposal 30: RAN1 to consider both: path-based and sample-based representation for AI/ML positioning cases.

- NEC (R1-2408214) Proposal 6: Prioritized sample-based measurements over path-based measurement for model input collection, if down-selection between sample-based input and path-based input is necessary. - The compromised scheme that supports both sample-based measurement and path-based measurement is also acceptable in order to not block the process.

- Ericsson (R1-2407649) Proposal 4 For Rel-19 AI/ML based positioning Case 3b/2b, the measurement report of (PDP, DP) support sample-based measurement. Proposal 5 For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample measurement, or (b) legacy path-based measurement.

#### 1st round discussion

Regarding sample-based and/or path-based measurements for model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024).

Agreement (RAN1#116) In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

On the question of whether to support sample-based and/or path-based measurements for model input, companies’ views are captured below, based on the submitted contributions to RAN1#118. (Please update/correct if your view is not captured correctly)

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement), 21 sources | vivo, CEWiT, CATT/CICTCI, NEC, CMCC, Baicell, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi, TCL, Ruijie Networks, NTT Docomo, OPPO (optional), Lenovo (Case 1,2b,3b), Tejas |
| Alternative B (path-based measurement), 9 sources | Huawei/HiSilicon, Qualcomm, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a), Tejas |
| Specify both, 9 sources | Apple, Nokia, Google, NVIDIA, Lenovo, NEC (compromise), Ericsson, Fraunhofer IIS/Fraunhofer HHI, |

As a reference, the RAN1#118 status is copied below (see R1-2407272):

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement), 22 sources | vivo, CEWiT, Fujitsu, CATT/CICTCI, NEC, CMCC, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi, TCL, IITM/IIT Kanpur, Ruijie Networks, DOCOMO, OPPO (optional), Lenovo (Case 1,2b,3b) |
| Alternative B (path-based measurement), 8 sources | Huawei/HiSilicon, Qualcomm, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a) |
| Specify both, 5 sources | Lenovo, Apple, Nokia, NVIDIA, Google (configure) |
| Unclear 1 source | Fraunhofer |

Companies’ view on sample-based vs path-based measurement stays almost the same as in last meeting, except that more companies are open to support both. Since RAN1 is unlikely to converge to one Alternative only, it is suggested that both are supported. The LMF can signal which measurement is to be reported for Case 3b.

Proposal 2.1.3.2-1

For Rel-19 AI/ML based positioning, for determining model input, support both Alternative (a) sample-based measurement report and Alternative (b) path-based measurement report.

- For Case 3b, LMF signals to gNB whether (a) or (b) is expected to be reported.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| InterDigital | The reasons are stated in our contribution. In addition, the discussion about whether sample-based measurement is supported or not is applicable to Case 1. |
| CATT, CICTCI | We can decide this once definition of sample-based measurement and enhancement of path-based measurement are both clear. |
| ZTE | OK for compromise. This discussion lasts for several meetings, and companies diverges at the very beginning. I guess this is the only compromised solution. |
| New H3C | OK |
| Hw/HiSi | The number of paths should be extended compared to legacy |
| Apple | Okay… good to ensure that there is progress. |
| NEC | Support to as start point. |
| Qualcomm | Let’s finalize aspects related to sample-based definition before discussing support. |
| DOCOMO | If both alt (a) and (b) are supported, indication from LMF to gNB is necessary. |
| OPPO | Ok to support for progress. |
| Sharp | OK |
| Fujitsu | Supporting both would be a better way for progress. |

## Timing measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 6: Regarding the type of channel measurements for Case 3b, support the use of legacy mechanism subject to path-based measurements. Note: the contribution refers to UL RTOA and RSRPP

• Google (R1-2407989) Proposal 1: Support the following options for the channel measurement for training data collection - Option 1: Time information and power information - Option 2: Time information, power information and phase information

- ITL (R1-2408908) Proposal 1: For AI/ML based positioning Case 3b, the existing UL RTOA reference time T0+tSRS is applicable for only UL part, and TRP transmit timing of DL subframe should be used to support multi-RTT.

- Intel Corporation (R1-2408290) Proposal 11: • Support relative differential time instance representation for representation of timing information of the channel response for model input in AI/ML based positioning using CIR/PDP/DP. Proposal 12: • If path-based measurement is supported, the existing representation/quantization framework for timing-based metric reporting (e.g., RSTD, RTOA), with the smallest value of ReportingGranularityfactor (‘k’) of -6 as introduced in Rel-18, can be reused for representation of timing information of the channel response for model input in AI/ML based positioning.

- InterDigital, Inc. (R1-2408522) Proposal 25: For direct AI/ML based positioning, for case 1, adopt RSRP, RSRPP and DL-RSTD measurement for the UE to determine input for an AIML model for direct AIML positioning Proposal 35: Support multi-RTT positioning method under Case 3b without any specification enhancement

#### FL view

For timing and power measurement for model input, some of the proposals are covered by pre-existing agreements. Timing measurements have been agreed to be supported together or separately from power measurement, and re-using the RTOA definition agreed in 38.215. Please see the following agreements:

Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT

Regarding the support of multi-RTT and Case 1 model input, considering the need for making progress in other more pressing issue, there is no plan to discuss multi-RTT support and model input for Case 1 at this meeting.

Also, regarding multi-RTT support, RAN1 has discussed in RAN1#117 and had this understanding on how to proceed, see R1-2405389:

The commonly acknowledged way forward is: RAN1 proceed with the investigation assuming each case is deployed individually, unless RAN plenary expand the scope of the WID to explicitly support certain combination of cases.

Since the WID scope is not revised to explicitly support certain combination of cases, RAN1 continue to proceed with the investigation assuming each case is deployed individually. It is understood that LMF may combine certain cases by implementation and without any specification impact, and this is beyond the scope of RAN1 discussion.

## Power measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• Google (R1-2407989) Proposal 1: Support the following options for the channel measurement for training data collection - Option 1: Time information and power information - Option 2: Time information, power information and phase information

- CATT, CICTCI (R1-2408028) Proposal 17: For case 3b, if sample-based measurement is supported, reuse the existing definitions of UL SRS-RSRPP to represent the power information by replacing ‘path’ to ‘sample’, and reuse the existing report mapping table of UL SRS-RSRPP to report the power information.

- Intel Corporation (R1-2408290) No proposal but comments that current proposal on starting point is enough.

- InterDigital, Inc. (R1-2408522) Proposal 25: For direct AI/ML based positioning, for case 1, adopt RSRP, RSRPP and DL-RSTD measurement for the UE to determine input for an AIML model for direct AIML positioning

- Apple (R1-2408453) Proposal 2-3: Support both absolute and differential RSRPP mapping for RSRPP values - The magnitude depends on the input type - Delay profile: no magnitude - Power Delay Profile: RSPP - CIR: RSPP and phase

- MediaTek Korea Inc. (R1-2408903) Proposal 2b-1: The RSRP with respect to the PDP for reporting is not lower than the RSRP of the PDP obtained in any of the individual receiver branches Proposal 2b-2: The receiver branch(es) used for combination are under the same RX TEG

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 13: The mapping table of RSRPP in TS 38.133 can be reused for AI/ML positioning.

## Phase measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- vivo (R1-2407849) Observation 13: First-path phase, when combined with multi-model/view processing, can additionally offer at least 20% gain of positioning accuracy over PDP with negligible extra reporting overhead. Observation 14: There are at least three methods which can be utilized to eliminate the impact of random initial phases of transceiver: - PRU-assisted phase calibration - Relative phase with reference to a reference path/sample - Initial phase measurement and reporting Observation 15: Apart from improving positioning accuracy, phase information can also be taken as assistance information to assist model monitoring, e.g., by cross-checking the positioning results from different views. Proposal 19: In addition to delay and power, at least RSCP and RSCPD should be supported for AI/ML based positioning. Proposal 20: Raw CIR reporting can be supported, and LMF can eliminate the impact of random initial phases of transceiver by implementation.

- Huawei, HiSilicon (R1-2407654) Proposal 8: For Case 3b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. - The use of phase information for the measurement reporting would need further justification.

• Google (R1-2407989) Proposal 1: Support the following options for the channel measurement for training data collection - Option 1: Time information and power information - Option 2: Time information, power information and phase information

- CATT, CICTCI (R1-2408028) Proposal 16: Phase measurement and reporting is supported for case 2b and case 3b, up to UE/TRP capability. Subject to UE/TRP capability indication, LMF makes the decision on whether to report the phase measurement and send the corresponding indication to UE/gNB/TRP.

• Fujitsu (R1-2408104) Proposal-2: Study on phase measurement and reporting for the model input of Case2b/3b is deprioritized in Rel-19 normative work.

- InterDigital, Inc. (R1-2408522) Proposal 28: Do not introduce CIR where all paths or samples (if supported) have phase information Proposal 29: Support first-path phase measurement, namely RSCP and RSCPD, for AIML based positioning

- NEC (R1-2408214) Proposal 12: For case 1, whether support to use phase information as model input is up to UE implementation. Proposal 13: For case 3a, whether support to use phase information as model input is up to gNB implementation. Proposal 14: For case 3b, support phase information reported from gNB to LMF as model input. - The phase information introduced in Rel-18 can be a start point

• Baicells (R1-2408309) Proposal 3. Support phase information reporting for Case 3b. Proposal 8. For Case 3b data collection for model training, new measurements (if necessary) such as CIR/PDP may need to be specified same as Case 3b model inferencing.

• Sony (R1-2408402) Proposal 1: For data collection, support radio channel characteristics reporting in a form of channel impulse response (CIR) for AI/ML positioning.

- Qualcomm Incorporated (R1-2408838) Proposal 5: In AI/ML positioning Case3b, for model input measurements, no support for reporting phase information (e.g., CIR) for model input running at LMF side.

- Intel Corporation (R1-2408290) Proposal 13: If phase information as model input is supported, consider extending the existing definitions of DL RSCP and UL RSCP to represent phase information for CIR as AI/ML model input to include phase information for the additional detected paths for path-based measurement or additional samples for sample-based measurement of the CIR as model input. - FFS: Further adaptations to the existing definitions.

- Xiaomi (R1-2407951) Proposal 5: The support of CIR should be deprioritized

- NTT DOCOMO, INC. (R1-2408774) Proposal 9: For AI/ML based positioning, in addition to timing information and power information, phase information report is considered for determining model input. - Rel-18 measurements (e.g., DL RSCPD, DL RSCP, UL RSCP) are considered as a baseline for phase information report.

- Ericsson (R1-2407649) Proposal 12 Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

- Apple (R1-2408453) Proposal 6: On the use of CIR model input for AI/ML positioning: - The relative performance of CIR and PDP depends on the complexity of the AI/ML model. - As complexity increases, CIR shows better performance than PDP. - With phase mismatch between the transmitter and receiver, the accuracy of AI/ML based positioning degrades. - Option 1: This can be mitigated by training the model with data that suffers a similar mismatch. - Option 2: This can be mitigated by training the model with data that is compensated to remove the effect of the mismatch. Proposal 7: On other aspects, limit the overhead, a model may be able to support mixed input with CIR input for the TRPs closest and PDP/DP for TRPs further away. Proposal 8: RAN1 to support using phase information (in addition to timing information and power information) for determining model input (i.e. support CIR based model input).

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 4: Support a complex valued sample-based reporting offering: - Lossless reporting of the channel impulse response - Supporting future enhancements of the AI/ML model

- NVIDIA (R1-2408391) Proposal 1: Support reporting phase information in time domain channel measurements for AI/ML based positioning.

- Lenovo (R1-2408429) Proposal 9: RAN1 to support the following additional model input types for DL-based Direct AI/ML positioning measurements based on DL-PRS: • Support channel observation measurements in the form of DL CIR measurements Proposal 10: RAN1 to support the following additional model input types for UL-based Direct AI/ML positioning measurements for case 3b based on SRS for positioning: • Support channel observation measurements in the form of UL CIR measurements • Support the inclusion of additional channel profiles such as UL-based angle-delay domain (paired angle/phase and timing information).

- Sharp (R1-2408609) Proposal 5: For AI/ML based positioning, continuous phase information is supported for model input.

- CMCC (R1-2407893) Proposal 6: For AI/ML based positioning, more discussion is needed for the comparison between CIR and PDP as model inputs.

- OPPO (R1-2408159) Proposal 2: For R19 AI-based positioning, NOT support the reporting based on phase information (in additional to timing information and power information).

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 12: In AI/ML based positioning, support using phase information for determining model input.

- Nokia (R1-2408545) Proposal 32: CIR is not supported for inference input when the model running at the LMF-side, gNB-side or UE-side cases. In addition, if first legacy RSCP/RSCPD are considered as inference input, the LoS determination and integer ambiguity needs to be studied.

### 1st round discussion

Regarding the issue of using phase information (in addition to time and power) for determining model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024).

Agreement (RAN1#116) For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

For this topic, no decision could be made in RAN1#117 or RAN1#118. In contributions to RAN1#118bis, companies provided further analysis and expressed their views as follows (please update/correct if your view is not captured correctly).

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (16) | vivo, Sharp, NTT DOCOMO, Apple, Sony, Fraunhofer IIS/Fraunhofer HHI, ZTE/Pengcheng Laboratory, NVIDIA, Lenovo, NEC, CATT/CICTCI, Google, Baicell, New H3C |
| No (11) | Qualcomm, Nokia, Fujitsu, MediaTek, OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson, Intel |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (4) | vivo, InterDigital, ZTE/Pengcheng Laboratory |
| No (7) | Qualcomm, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson, Intel |

For reference, RAN1#118 status (see R1-2407272) was shown below. Note that RAN1#117 status was similar to RAN1#118.

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (14) | vivo, Sharp, DOCOMO, Apple, Sony, Fraunhofer IIS/Fraunhofer HHI, ZTE/Pengcheng laboratory, NVIDIA, Lenovo, NEC, CATT/CICTCI |
| No (10) | QC, Nokia, Fujitsu, MediaTek(?), OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson |
| Undecided (1) | CMCC |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (3) | vivo, InterDigital, ZTE |
| No (7) | QC, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson, Intel |

As shown above, companies’ view is almost the same in RAN1#117, RAN1#118, RAN1#118bis. No consensus is expected to be achieved at this meeting. Based on the discussion status since the start of the work item, it is unlikely that RAN1 can achieve convergence even if more time is given.

Therefore, it is suggested to conclude this discussion as below.

Conclusion 2.4.2-1

For Rel-19 AI/ML based positioning for all use cases, there is no consensus in RAN1 to support CIR for determining model input.

|  | Company |
| --- | --- |
| Support | Hw/HiSi |
| Not support | CATT/CICTCI, ZTE |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Considering the potential benefit that shown in some contributions, we think this can be optionally supported by UE/TRP, and LMF may or may not require UE/TRP to report phase information, subject to their capability. |
| ZTE | Share similar view with CATT. |
| New H3C | Share similar view with CATT. |
| NEC | ‘all use cases’ can be replaced by ‘case 3b’. The reasons are: (1) for case 1 and case 3a (and case 2a), there is no need to discuss whether support phase information as model input because it may be unnecessary to specify model input for the case that deploy model at UE/gNB, and it depend on UE/gNB implementation to decide whether input phase information to its deployed model. (2) we will not discuss case 2a and case 2b here according to the guidance of reviewed WID |
| DOCOMO | Share similar view with CATT. |
| Fujitsu | OK |

Conclusion 2.4.2-2

For Rel-19 AI/ML based positioning for all use cases, in terms of using phase information (in addition to timing information and power information) for determining model input, there is no consensus in RAN1 to support using one phase value (e.g., Rel-18 measurements DL RSCPD, DL RSCP, UL RSCP) for the first path or first sample only.

|  | Company |
| --- | --- |
| Support | Hw/HiSi |
| Not support | InterDigital, ZTE |

| Company | Comments |
| --- | --- |
| InterDigital | Phase information for the first path brings performance benefits. |
| CATT, CICTCI | Same answer as above. |
| ZTE | Share similar view with InterDigital, CATT. |
| NEC | Same as the last comment. |
| DOCOMO | Share similar view with the above companies. |

## Other

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 9: For Case 3b, there is no need to specify the purpose of the measurement report from the gNB to the LMF as the usage of the measurements is LMF implementation and is transparent to the gNB.

• CATT, CICTCI (R1-2408028) Proposal 8: For case 1 and case 2a, at least when UE-side model is trained by UE-side, UE generates the channel measurement for AI/ML model inference based on the PRS transmission. The input type or format is up to UE implementation. Proposal 9: For case 3a, at least when gNB-side model is trained by gNB/NW-side, gNB/TRP generates the channel measurement for AI/ML model inference based on the SRS-pos transmission. The input type or format is up to gNB/TRP implementation.

- NEC (R1-2408214) Proposal 15: Support to setting the validity period of measurements to correspond with an instance of model input to overcome the measurement overhead.

- Qualcomm Incorporated (R1-2408838) Proposal 1: For AI/ML positioning cases (Case1 to Case3b), from RAN1 perspective, AI/ML positioning can be part of existing positioning methods and modes as follows: - Case1 as part of UE-based DL-TdoA and DL-AoD positioning - Case3a and Case3b as part of NG-RAN-assisted UL-TdoA, NG-RAN-assisted UL-AoA, and multi-RTT positioning - FFS: Case2a and Case2b can be discussed after Q4 2024.

- InterDigital, Inc. (R1-2408522) Proposal 30: For AI/ML based positioning case 1, at least the following types of measurements should be requested by the LMF to be made by the UE: • timing information; • paired timing information and power information. • phase measurement Proposal 37: For Case 3a and Case 3b, use existing UL or DL & UL positioning methods as the starting point for AIML-based positioning method Proposal 38: For Case 1, enhance the existing UE-based DL-TDOA positioning method to support AIML-based positioning

- Lenovo (R1-2408429) Proposal 11: Support the re-use of legacy positioning measurements as model input types for the following Direct AI/ML positioning cases to derive a fingerprint: • Case 1, 2b (DL-based): Support DL-RSTD, UE Rx-Tx time difference, DL-PRS RSRP/RSRPP measurements • Case 3b (UL-based): Support UL-RTOA, gNB Rx-Tx time difference, UL-AOA and SRS RSRP/RSRPP measurements • Note: The above legacy measurements may be considered in conjunction with channel observation measurements.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 22: For AI/ML-based positioning, the model output of AI/ML assisted positioning can be angle information. - Other intermediate features in addition to the output listed in TR 38.843 are not precluded.

# Model output for assisted AI/ML positioning

This section discusses model output for assisted AI/ML positioning and potential measurement enhancements.

## LOS/NLOS indicator for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 10: For Case 3a, LOS/NLOS indicator can reuse the same format as the existing IE “LoS/NLoS Information” in 38.455.

- CATT, CICTCI (R1-2408028) Proposal 18: For AI/ML-assisted positioning, no need to report LOS/NLOS indicator when timing information is reported based on the output of AI/ML model.

• Fujitsu (R1-2408104) Proposal-3 For AI/ML assisted positioning (Case2a/Case3a) when the LOS/NLOS indicator is reported, from RAN1 perspective, - The LOS/NLOS indicator provides the information on confidence level of the timing information corresponding to LOS path from the model output, with a value of 1 corresponding to 100% confidence, and a value of 0 corresponding to 0% confidence. - The LOS/NLOS indicator can reuse the same format as the existing IE "LoS/NLoS Information" in 38.455 for Case 3a, and the existing IE LOS-NLOS-Indicator-r17 in 37.355 for Case 2a.

- NEC (R1-2408214) Proposal 17: At least for case 3a, there is no need to report LOS/NLOS indicator to LMF if the model output is timing information.

• Baicells (R1-2408309) Proposal 4. Reuse existing “LoS/NLoS information” in TS 38.455 for Case 3a.

• Sony (R1-2408402) Proposal 7: For AI/ML assisted positioning case 2a and case 3a, the positioning measurement reporting includes LOS/NLOS indicator using the legacy reporting format (i.e., as in TS 37.355 for case 2a , and as in 38.455 for case 3a) and the indication whether the measurement is based on AI/ML computation or not.

- Intel Corporation (R1-2408290) Proposal 16: - For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, - consider the below alternatives: - Alt A: the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS, where the propagation path corresponds to the associated timing metric that is determined as AI/ML model output and reported in the same report; - Alt B: the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS, where the propagation path corresponds to the actual radio propagation path between the transmitter and the receiver and may not correspond to the timing metrics included in the same report; - the LOS/NLOS indicator reuses the same format as the existing IE "LoS/NLoS Information" in TS 38.455; - one LOS/NLOS indicator is optionally reported per UE as in the existing specification.

- InterDigital, Inc. (R1-2408522) Proposal 32: For Case 3a, keep the current definition of the LOS indicator shown in TS 37.355, “the likelihood of a Line-of-Sight propagation path from the source to the receiver”

- Apple (R1-2408453) Proposal 10: For AI/ML assisted positioning (i.e., 3a), when LOS/NLOS indicator is reported, - the existing IE in 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective. This may indicate a “physical LOS” path. - “Made with AI” indicator. This may indicate a “virtual LOS” path for a timing measurement - Time stamp

- NTT DOCOMO, INC. (R1-2408774) Proposal 14: - For case 3a, LOS/NLOS indicator can reuse the existing IE “LOS/NLOS Information” in 38.455. LOS/NLOS indicator is reported associated with timing measurement.

- Sharp (R1-2408609) Proposal 12: In case that the associated timing information is not predicted by AI/ML, the meaning of the LOS/NLOS indicator is same as the existing specifications, regardless the LOS/NLOS indicator is predicted by AI/ML. Proposal 13: For Rel-19 AI/ML assisted positioning, meaning of the LOS/NLOS indicator is changed to meaning of confidence when the associated timing information is predicted by AI/ML.

- CMCC (R1-2407893) Proposal 10: For AI/ML assisted positioning, legacy LOS/NLOS information reporting could be used.

- MediaTek Korea Inc. (R1-2408903) Proposal 2a-1: Change the LOS/NLOS indicator definition slightly when AI/ML applies, and there is no need to additionally induce signalling to describe how AI/ML model could estimate the LOS delay Proposal 2a-2: The reporting which is through the AI/ML model output may need to be indicated to NW

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 23: For AI/ML-assisted positioning Case 3a, when the LOS/NLOS indicator is reported together with the legacy timing measurement result, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported; - One LOS/NLOS indicator value is optionally reported from the source to the receiver as in the existing specification. - For Case 3a, the LOS/NLOS indicator can reuse the existing IE LoS/NLoS Information in 38.455. Proposal 24: For AI/ML-assisted positioning Case 3a, when the LOS/NLOS indicator is reported with the AI/ML model output, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight virtual path for the model output over the resource for with the measurement is reported. - For Case 3a, the LOS/NLOS indicator can reuse the same format as the existing IE LoS/NLoS Information in 38.455.

- Nokia (R1-2408545) Proposal 33: In Case 3a, when timing information is estimated with AI/ML, no LOS/NLOS indicator associated with this estimation is reported.

- Ericsson (R1-2407649) Proposal 22 For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LOS and a value of 0 corresponding to NLOS; the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455; one LOS/NLOS indicator is optionally reported per TRP as in the existing specification. Proposal 23 For case 3a and 2a, the timing measurement for the first path is associated with a virtual line of sight path. • If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS) • Note: when the TRP-UE link is LOS, the virtual LOS coincides with the physical LOS. • Details of the association with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request. • Send an LS to RAN2/RAN3 with the final agreement.

### 1st round discussion

On the LOS/NLOS indicator when it is generated as model output for AI/ML assisted positioning, companies have shared their views on the definition of the indicator, the values, and whether/how to reuse the existing IE.

The first issue to resolve is the definition of LOS/NLOS indicator, when it is reported for AI/ML assisted positioning. Based on the analysis and proposals submitted in contributions to RAN1#118bis, companies’ views are summarized below (Please update/correct if your view is not captured correctly).

For the meaning of LOS/NLOS indicator, several different views are provided by companies:

- (Same meaning as legacy LOS/NLOS indicator) LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported

  - Supported by (12 sources): Ericsson, CMCC, Apple, InterDigital, Huawei/HiSilicon (?), Baicells, Sony, NTT DOCOMO, Sharp (when associated timing info is not by AI), ZTE/Pengcheng Laboratory (when reported with legacy timing measurement)

- Meaning of the LOS/NLOS indicator is changed when the associated timing information is predicted by AI/ML; when associated timing info is not by AI, use the same meaning as legacy LOS/NLOS indicator

  - Supported by (4 sources): Sharp (change to confidence), ZTE/Pengcheng Laboratory (change to likelihood of LoS virtual path), Fujitsu (?)

- Change the LOS/NLOS indicator definition slightly when AI/ML applies (3 versions of likelihood provided in MediaTek (R1-2408903)

  - Supported by (1 sources): MediaTek

- Multiple LOS/NLOS indicators for one measurement report of a pair of transmit and receive points (e.g., multi-hypothesis measurements)

  - Supported by (1 sources): Qualcomm

Also, NEC, CATT/CICTCI, Nokia proposed that: “when timing information is estimated with AI/ML, no LOS/NLOS indicator associated with this estimation is reported.”

Based on the summary above, the majority support that the LOS/NLOS indicator generated by AI/ML reuse the same meaning as legacy, at least when the associated timing information is generated by legacy method.

When the associated timing information is generated by AI, there are different views on how to define the meaning of LOS/NLOS indicator. Also, 4 companies (NEC, CATT/CICTCI, Nokia) think that no LOS/NLOS indicator is reported if the associated timing information is generated by AI. Thus this aspect can be further considered by companies.

Proposal 3.1.2-1

For AI/ML assisted positioning Case 3a/2a, when the LOS/NLOS indicator is reported for a measurement result, and the associated timing information is provided by legacy method, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported;

- One LOS/NLOS indicator value is optionally reported as in the existing specification.

- For Case 3a, the LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455;

- For Case 2a, the LOS/NLOS indicator can reuse the existing IE LOS-NLOS-Indicator-r17 in 37.355;

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

|  | Company |
| --- | --- |
| Support | CATT/CICTCI, New H3C, HW/HiSi, DOCOMO, Sharp |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | In RAN#105, it was agreed not to discuss 2nd priority topics so Case 2a should be dropped from this proposal. Motivation for associating the LOS indicator with timing information is not clear. From our perspective, the AIML is inference can be the LOS indicator whose definition follows the existing definition. |
| CATT, CICTCI | With the understanding that timing information is provided by non-AI method in this case, we support this proposal. |
| ZTE | This version is fine for us. |
| HW/HiSi | @FL: please update in the summary above, that we support reusing the legacy definition of LOS/NLOS indicator |
| Nokia | Lets focus only on 1st priority cases, in this case only on Case 3a. |
| Apple | Support in general. However, current definition is per path. If sample based measurements are supported, definition needs to be updated. |
| NEC | Support. If the timing information is provided by legacy method, there is no reason to change the legacy meaning of LOS/NLOS indicator(associated with this timing information). |
| Qualcomm | Let’s postpone 2a as guided by the RAN plenary. First, Option A in FL’s discussion (and 1st bullet in proposal) does not seem to be definition of LOS in existing specifications. There is no “physical” mentioned in association with path. ### [TS 38.455] ### 9.2.77 LoS/NLoS Information This IE contains the LoS/NLoS information for UL measurement. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE LoS/NLoS Indicator; M. >Soft Indicator. >>LoS/NLoS Indicator Soft; M; INTEGER (0..10); Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS.. >Hard Indicator. >>LoS/NLoS Indicator Hard; M; ENUMERATED (NLoS, LoS). ### [TS 37.355] #### – LOS-NLOS-Indicator The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver. -- ASN1START LOS-NLOS-Indicator-r17 ::= SEQUENCE { indicator-r17 CHOICE { soft-r17 INTEGER (0..10), hard-r17 BOOLEAN }, ... } -- ASN1STOP LOS-NLOS-Indicator field descriptions. indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. - soft: Integer value '0' indicates likelihood 0, integer value '10' indicates likelihood 1. Scale factor 0.1; range 0 to 1. - hard: FALSE indicates likelihood '0', TRUE indicates likelihood '1'.. Second, existing specifications allow considering multiple LOS-NLOS indicators per TRP/UE (multiple LOS indications on antenna ARP level and/or resource level – see TS 38.455, TS 37.355, and TS 38.355). No need to put restriction on number of indicators. Multiple indicators can still be provided. Third, we also the discussion need not to be decoupled between legacy or AIML timing. We can include both and skip the last FFS. We propose the following updates: - The LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resource resources for which the measurement is reported; - One LOS/NLOS indicator value is Multiple LOS/NLOS indicator values are optionally reported as in the existing specification as a starting point. - FFS: Meaning of multiple LOS/NLOS indicator values |
| Fujitsu | We share the similar view as that of QC. |

### 2nd round discussion

For Proposal 3.1.2-1, it’s fine to remove Case 2a in response to companies’ comments.

Regarding QC comments:

- It’s fine to remove ‘physical’ to align with existing specification text. But it should be understood that this refers to physical path, as there was no ‘virtual path’ in legacy methods.

- Regarding “multiple LOS-NLOS indicators per TRP/UE”: the excerpt from 38.455 indicates that there is one LOS/NLOS indicator in a measurement result. If multiple measurement results are reported (up to maxnoPosMeas=16384) in a positioning message, then there can be multiple. In the proposal, it’s stated that the proposal is “for a measurement result”. Thus the proposal is consistent with legacy behavior.

38.455

9.2.37 	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |

Proposal 3.1.3-1

For AI/ML assisted positioning Case 3a/2a, when the LOS/NLOS indicator is reported for a measurement result, and the associated timing information is provided by legacy method, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported;

- One LOS/NLOS indicator value is optionally reported for a measurement result, as in the existing specification.

- For Case 3a, The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455.

- For Case 2a, the LOS/NLOS indicator can reuse the existing IE LOS-NLOS-Indicator-r17 in 37.355;

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

|  | Company |
| --- | --- |
| Support | NEC, DOCOMO |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | Motivation for associating the LOS indicator with timing information is not clear. From our perspective, the AIML is inference can be the LOS indicator whose definition follows the existing definition. |
| Nokia | The proposal looks that we are trying to specify behaviours such as combination of AIML LOS/NLOS indication and Legacy timing information. In our view, RAN1 should be limited to specify the indication of individual intermediate features, independent of such combination. So, we require a clarification on the necessity of this proposal. The same comment applies for the FFS sentence. |
| Qualcomm | -The first bullet only covers the soft-representation of LOS indicator definition. The third bullet already covers the first one, we can probably skip it. -The reporting in TRP Measurement Result already supports more than one LOS indicatios per TRP. The intention of the second bullet seems to preclude enhancements on meaning when multiple LOS indicators are provided per a resource. The second bullet can be reworded to reflect existing specification more carefully: - One or more LOS/NLOS indicator value is optionally reported for a TRP Measurement Result, as in the existing specification. -The reporting in TRP Measurement Result already supports more than one LOS indicatios per TRP. We can reword the FFS: FFS: whether one or more LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator. |

### 3rd round discussion

As explained, “One LOS/NLOS indicator value is optionally reported for a measurement result” is repeating legacy behavior. No multiple LOS/NLOS indicators are listed for one measurement result in 38.455. The clean version of the proposal is provided below for further checking.

Proposal 3.1.4-1

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported for a measurement result, and the associated timing information is provided by legacy method, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- One LOS/NLOS indicator value is optionally reported for a measurement result, as in the existing specification.

- The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455.

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

|  | Company |
| --- | --- |
| Support | CATT/CICTCI, NEC |
| Not support | InterDigital (Our proposal is shown below) |

| Company | Comments |
| --- | --- |
| ZTE | The general idea is fine. But the current wording seems confusing. In current 38.455, LOS/NLOS indicator is not only limited to timing measurement. Does AI model inference LOS/NLOS indicator for power/phase/angle measurement? (If my understanding is wrong, pls correct me) IE/Group Name. Measured Result Item. >CHOICE Measured Results Value. >>UL Angle of Arrival. >>UL SRS-RSRP. >>UL RTOA Measurement. >>gNB Rx-Tx Time Difference. >>Z-AoA. >>Multiple UL-AoA. >>UL SRS-RSRPP. >>UL RSCP. >Time Stamp. >Measurement Quality. >Measurement Beam Information. >SRS Resource type. >ARP ID. >LoS/NLoS Information. |
| InterDigital | From our understanding, regarding inference for Case 3a, the following agreement made in RAN1#116b is the only relevant agreement. We never followed up to discuss whether the gNB reports a LOS/NLOS indicator only, or both LOS/NLOS indicator and timing information. Firstly, we should discuss the gNB’s reporting behavior for the LOS/NLOS indicator, namely, whether we support Option 1 or Option 2 shown below. - Option 1 : gNB reports inferred LOS/NLOS indicator - Option 2 : gNB reports both inferred LOS/NLOS indicator and associate timing information (FFS : whether the associated timing information is generated by an AI/ML) Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. |
| Qualcomm | @FL: Please check TS 38.455. There is no IE called “measurement result“. It is called “TRP Measurement Result“. - Reporting more than one LOS indicator per TRP is already supported in TS 38.455. Please check. - The first bullet precludes the hard info representation of LOS, which contradicts with the description of the third bullet. |
| HW/HiSi | The basic direction is fine. The terminology should be aligned with existing spec, though, as QC pointed out. |
| Nokia | In general, if the intention is to remain on the legacy, ok. In addition, it is important to mention that we have an agreement directly related to the reporting of intermediate features for Case 3a. In our view, there are details that requires clarification, especially in the FFS. So, to provide a chance to companies to further explain their inputs, we suggest to completely reword the proposal. Proposal 3.1.4-1 (Updated) For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported for a measurement result, and the associated timing information is provided by legacy method, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported; - One LOS/NLOS indicator value is optionally reported for a measurement result, as in the existing specification. - The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455. FFS: RAN1 to study whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator. |

### 4th round discussion

Thursday offline:

Proposal 3.1.5-1

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the other associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning.

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| Nokia | Agree with the note, is RAN3 the WG on getting the final decision on using timing information estimated by legacy along or not with LOS/NLOS indicator done by AIML. So, does not make sense for RAN1 the mixing of reporting between legacy and AIML as indicated in the main sentence. |

Proposal 3.1.5-2

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, RAN1 study whether the following reporting are supported:

(1) LOS/NLOS indicator is reported together with other associated measurement (e.g., timing information), which is also reported for Rel-19 Case 3a;

  - If supported, study the meaning of the LOS/NLOS indicator;

(2) LOS/NLOS indicator is reported without any other associated measurement (e.g., timing information);

  - If supported, the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |

## Timing information for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Fujitsu (R1-2408104) Proposal-5: For Case 2a/3a, study enhancement of timing information reporting for (virtual) LOS path: - Multiple timing information reporting for (virtual) LOS path is supported when the value of LOS/NLOS indicator is low. - One or more than one timing information reporting can be configured by LMF referring to the LOS/NLOS indicator value.

- Intel Corporation (R1-2408290) Proposal 14: • For AI/ML-assisted positioning (Cases 2a and 3a), consider support of reporting of the following information from UE and gNB to LMF respectively: o UE to LMF (Case 2a): RSRP/RSRPP for first and additional paths along with associated path timings, e.g., DL RSTD, UE Rx-Tx time difference. O gNB to LMF (Case 3a): AoA and/or RSRP/RSRPP for first and additional paths along with associated path timings, e.g., UL RTOA, gNB Rx-Tx time difference.

- Ericsson (R1-2407649) Proposal 27 (for conclusion) For AI/ML assisted positioning at gNB (Case 3a), the gNB is not expected to report additional paths for AI/ML based measurement report enhancements. Note: the network can simply omit the optional field for additional paths. It is not necessary to make specification changes to the measurement report IE. Proposal 28 For AI/ML assisted positioning at gNB (Case 3a), power is optionally reported using UL SRS RSRP (Case 3a). The gNB is not expected to report per-path power.

- Qualcomm Incorporated (R1-2408838) Proposal 25: In AI/ML positioning Case3a, for model output, consider enhancements in reporting of earliest path LOS and timing info measurements in which gNB reports multiple-hypotheses soft info of LOS indicator and timing information. Existing additional path reporting (up to 9 paths) could be repurposed to support the multiple-hypothesis LOS/timing indications of first path.

#### FL view

Two companies (Fujitsu, Qualcomm) mention the use of multiple hypothesis / multiple reports corresponding to the same path. Ericsson proposes to focus on a single path reporting when AIML is used. Intel proposes to add AOA measurement reporting support for case 3a, additionally to the already agreed time measurements.

Considering the need for making progress in other more pressing issue, there is no plan to discuss these issues at this meeting. But they can be discussed later if time allows.

## Indication of AI/ML based reports

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 11: For Case 3a, to distinguish the reported timing information from the legacy measurement-based timing information, consider the following alternatives - Alt.1: Reuse the timing quality indicator, where reported timing information of Case 3a can be represented with high timing quality and NLOS. - Alt.2: Introduce an indicator to tell that the report is subject to measured timing information or non-measured timing information.

• Fujitsu (R1-2408104) Proposal-4: For AI/ML assisted positioning (Case2a/Case3a), no need to introduce additional indication for the indication of an AI/ML-based positioning report.

- Qualcomm Incorporated (R1-2408838) Proposal 16: In AI/ML positioning Case1, for inference related request, support at least the following information from LMF to UE: - Signaling on whether location information needs to be obtained using AI/ML, legacy, or both AI/ML and legacy method Proposal 19: In AI/ML positioning Case1, for inference related reporting of location information, consider information to indicate location information obtained using AI/ML. Proposal 20: In AI/ML positioning Case3a, for inference related measurement request (LMF to gNB), consider information to indicate measurements that need to be obtained using AI/ML. Proposal 21: In AI/ML positioning Case3a, for inference related measurement response (gNB to LMF), consider information to indicate measurements obtained using AI/ML.

- InterDigital, Inc. (R1-2408522) Proposal 31: For AIML assisted positioning, support an indication in the measurement report to indicate the reported timing measurement is inferred by an AIML model(s). Proposal 36: Support multi-RTT positioning method under Case 3a with a request from the LMF to the gNB to use an AIML model for generating gNB Rx-Tx time

- NTT DOCOMO, INC. (R1-2408774) Proposal 13: - For AI/ML assisted positioning (i.e., Case 3a), the necessity to indicate whether the intermediate measurement is generated by an AI/ML model or not needs further justification.

- Apple (R1-2408453) Proposal 10: For AI/ML assisted positioning (i.e., 3a), when LOS/NLOS indicator is reported, • the existing IE in 38.455 (NRPPa for Case 3a) can be re-used from RAN1 perspective. This may indicate a “physical LOS” path. • “Made with AI” indicator. This may indicate a “virtual LOS” path for a timing measurement • Time stamp

- Ericsson ( Proposal 29 For AI/ML assisted positioning at gNB (Case 3a) and UE (Case 2a), the measurement report contains an indicator that AI/ML is used to produce the measurements. The signaling details are up to RAN3/RAN2.

### 1st round discussion

Notification of report generated by AI/ML model (vs legacy) is proposed in most contributions, with the following variations:

- Use of “made with AI indication” IE: Apple, InterDigital, Qualcomm, Huawei, Ericsson (how to signal is up to RAN2)

- Need to be justified first: Docomo, Fujitsu

From the FL perspective, RAN1 can discuss the need of AI/ML indication, and leave the IE details to RAN2/RAN3.

Proposal 3.3.2-1

For Rel-19 AI/ML based positioning, the report contains an indication of the use of an AI/ML model at least for:

- intermediate positioning measurement reports based on model output (Case 3a, 2a), or

- location estimate based on model output (Case 1).

Note: It is up to RAN2/RAN3 to decide whether a new IE is needed to provide the indication.

|  | Company |
| --- | --- |
| Support | Apple |
| Not support | HW/HiSi, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | We should remove Case 2a from the discussion as discussed in RAN#105. |
| CATT, CICTCI | This is related to whether functionality activation and reporting AI/ML-based output is coupled or not. In agenda 9.1.1 AI/ML based beam management they are discussing this issue. We think there is no need for UE/TRP to indicate. On the contrary, whether reported information is based on AI/ML should be configured/indicate by LMF. |
| ZTE | This depends on whether AI/ML is a separate positioning method. If it is, then the measurement result could be embedded in this positioning method. If not, whether report an indicator could be further discussed. |
| HW/HiSi | We should first align on what type of output info this is would be applicable (LOS or timing) and for the latter what actually the output timing information is. In our understanding it describes the propagation delay between of a direct (virtual) path between UE/PRU and the TRP. Thus, as opposed to the legacy timing information which is measured, this timing info is non-measured. Further there is no need to discuss the timing info for Case 1 since it is implementation for that, and neither for case 2a (as per plenary decision). Proposal 3.3.2-1 For Rel-19 AI/ML based positioning, when timing information is model output for Case 3a the report contains an indication of the use of an AI/ML model at least for: - un-measured UL RTOA is reported. Intermediate positioning measurement reports based on model output (Case 3a, 2a), or - location estimate based on model output (Case 1). Note: It is up to RAN2/RAN3 to decide whether a new IE is needed to provide the indication. |
| Nokia | Fully agree with InterDigital. |
| NEC | This issue can be postponed until more discussion on LCM. If LMF is the entity to control LCM operation, LMF may be aware of whether the reported result is obtained from AI/ML model or not as the LMF can control whether to activate the AI/ML model. |
| DOCOMO | We agree it’s important for LMF to know whether the report is generated by AI/ML or not, but in our view, LMF can implicitly know this information from LCM procedures or other signalings which facilitate AI/ML based positioning. |
| TCL | Generally fine. |
| Fujitsu | Considerring the functionality-based LCM, whether the report is AI/ML-based or not is known by LMF when the AI/ML model/functionality is activated. If so, there is no strong need to have this indicator. |

# Training data collection

This section discusses issues for the LCM stage of model training.

## Content of training data

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

#### Measurement side (part A)

- Huawei, HiSilicon (R1-2407654) Proposal 12: For indicating the quality of measurement for training (if needed), consider reusing the legacy quality indication reported by the measurement entity, or indicating the quality criterion to the measurement entity.

- Google (R1-2407989) Proposal 2: Support the L1-SINR as the quality indicator of channel measurement.

- CATT, CICTCI (R1-2408028) Proposal 6: For training data collection of AI/ML based positioning, the time stamp can be defined as follows: - For part A, the time stamp of channel measurement could be the time instance at which the channel measurement is performed.

- NEC (R1-2408214) Proposal 22: Use „UTC time + SFN + slot index“ as a baseline, with details FFS Proposal 23: Consider following two options as baseline to design the time stamp for channel measurement in case 3b: - Option 1: use „UTC time + SFN + slot index“ as baseline - Option 2: use „Measurement time“ only as baseline

• Baicells (R1-2408309) Proposal 9. For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message. Proposal 10. For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

• Sony (R1-2408402) Proposal 8: Define a set of parameters (e.g., part/all of DL-PRS configuration, received signal quality, etc) representing reference signal characteristics to be used for AI/ML positioning.

- Qualcomm Incorporated (R1-2408838) Proposal 11: In AI/ML positioning data collection, for Part A time stamp of channel measurement, as a starting point, consider IE UTC-time-r15 (i.e., UTCTime and utc-time-ms-r15) and IE NR-TimeStamp (TS 37.355).

- Samsung (R1-2408631) Observation 1. Full-size (or raw) channel measurement is not suitable for collected data sample. Proposal 7. Both truncated channel measurement and feature extracted channel measurement can be considered as candidate for the channel measurement in the collected data sample. Proposal 16. RAN1 supports to specify data type and size, the data type includes the measurement results and processed measurement results.

- NTT DOCOMO, INC. (R1-2408774) Proposal 1: For data collection, for the time stamp of the channel measurement and label, existing Ies are reused. - For case 1, UTC-time is optionally reported, if necessary. - For case 3b, Measurement time is optionally reported. Proposal 2: For data collection, for the quality indicator of the channel measurement and label, existing Ies are reused. - For case 1, reuse NR-TimingQuality in TS 37.355, if necessary. - For case 3b, reuse Timing Measurement Quality or Location Uncertainty in TS 38.455.

- Apple (R1-2408453) Proposal 13: For part A, the quality indicator of the channel may be mapped to a timing quality, a power quality and a phase quality depending on the type of input Proposal 15: The “Measurement time” (NRPPa) and UTC-time (LPP) are mandatorily included together with SFN time to ensure uniqueness of the time stamp.

- Sharp (R1-2408609) Proposal 8: For training data collection of AI/ML based positioning, “Measurement Time” can still be optional presence. Proposal 9: For training data collection of AI/ML based positioning, UTC level time stamp should be introduced.

- Ericsson (R1-2407649) Proposal 31 For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity. Proposal 32 For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B. • For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp. • For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp. • Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3. Proposal 33 From RAN1 perspective, the existing Ies and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps. Proposal 35 For training data collection in all cases, the quality of timing measurements in Part A (PDP, DP) can reuse existing IE NR-TimingQuality, which has resolution down to 0.1m. Proposal 36 For training data collection in all cases, the quality of power measurements is provided for PDP in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB.

#### Label side (Part B)

- Note: model output can be location or LOS NLOS state in the proposals

- Huawei, HiSilicon (R1-2407654) Proposal 13: For direct positioning, if needed, the UE location quality used in legacy reporting, i.e., LocationUncertainty, can be used for the quality indication of the label for training. There is no need to discuss other options. Proposal 14: For Case 3a, the label of the LOS/NLOS information generated from LMF can be in forms of the expected likelihood of a LOS propagation path from a TRP to the target device. Proposal 15: For Case 3a, the label of the timing information generated from LMF can be in forms of the ToF/propagation delay between the TRP and the PRU. - The provision of this label may not cause additional user privacy issue on top of legacy positioning.

- Xiaomi (R1-2407951) Proposal 12: Reuse existing IE as the quality indicator of label in Case 2b and Case 3b

- CATT, CICTCI (R1-2408028) Proposal 6: For training data collection of AI/ML based positioning, the time stamp can be defined as follows: - For part B, the time stamp of ground truth label can be: - The time for which the location estimate is valid; - The time when the associated SRS-pos is sent; - The time when the PRS is measured.

- NEC (R1-2408214) Proposal 22: Use „UTC time + SFN + slot index“ as a baseline, with details FFS

- Qualcomm Incorporated (R1-2408838) Proposal 12: In AI/ML positioning data collection, for Part B time stamp of label, as a starting point, consider IE UTC-time-r15 (i.e., UTCTime and utc-time-ms-r15) and IE NR-TimeStamp (TS 37.355). Proposal 13: In AI/ML positioning data collection, for Part B label and quality indicator of direct AI/ML positioning, as a starting point, consider location coordinate types and uncertainty of IE LocationCoordinateTypes (TS 37.355). Proposal 14: In AI/ML positioning data collection, for Part B label and quality indicator of AI/ML assisted positioning with timing information, as a starting point, consider IE timingQuality (TS 37.355). Proposal 15: In AI/ML positioning data collection, for Part B label and quality indicator of AI/ML assisted positioning with LOS/NLOS indication, as a starting point, consider IE LOS/NLOS Indicator Soft (TS 38.455).

- InterDigital, Inc. (R1-2408522) Proposal 9: A ground truth label quality indicator is associated with a UE or a PRU location Proposal 10: In Case 1, the LMF is the only entity that can generate the ground truth label quality indicator associated with location information Proposal 11: In Case 1, the LMF can provide a generated ground truth label quality indicator associated with location information to the UE Proposal 13: For Case 1, support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for the ground truth label quality indicator which is generated by the LMF and sent to the training entity Proposal 14: For Case 1, a soft or hard ground truth label quality indicator x=1 is associated with PRU location Proposal 15: For Case 1, a soft ground truth label quality indicator 0<x<1 is associated with UE location estimate Proposal 16: For Case 1, a soft or hard ground truth label quality indicator x=0 is associated with UE generated measurements/ground truth using interpolated or extrapolated “virtual” measurements

- Apple (R1-2408453) Proposal 14: For part B, the quality of the ground truth label may be set as a value from 0 to 1 similar to the LOS/NLOS soft value.

- Lenovo (R1-2408429) Proposal 14: The content of ground truth label and related data may at least comprise of: - For Cases 1, 2b: - PRU UE location information, PRU UE location uncertainty, PRU UE location timestamp. - For Case 2a: - PRU UE LOS/NLOS indicator associated to the positioning measurement, timestamp associated to LOS/NLOS measurement. - For Case 3a: - PRU UE LOS/NLOS indicator associated to the positioning measurement, PRU UE location of SRS transmission, timestamp associated to LOS/NLOS measurement and PRU UE SRS Tx location, PRU UE location uncertainty - For Case 3b: - TRP Location and PRU UE location of SRS transmission, timestamp associated PRU UE SRS Tx location, PRU UE location uncertainty. - NOTE1: Determination or transfer of the ground truth label and related data may be up to other WGs. - NOTE2: Discussion on Cases 2a and 2b can be revisited in RAN1#120.

- Ericsson (R1-2407649) Proposal 38 From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity. Proposal 39 For training data collection, the label quality for Part B reuse the existing uncertainty Ies for indicating UE location error, e.g., horizontalUncertainty-r16 and verticalUncertainty-r16.

#### Other (e.g., Association of Part A and Part B, measurement window configuration)

• CATT, CICTCI (R1-2408028) Proposal 5: For case 3b, the measuring time difference between Part A and Part B should be restricted to ensure the validity of a training data sample.

- NEC (R1-2408214) Proposal 10: Study necessary signaling exchange between UE/gNB and LMF to help UE/gNB to pair Part A and Part B as valid training data. Proposal 21: Further study how to match a measurement and a label as a valid instance of training data or monitoring data, if the measurements and the labels are generated by different entities.

• Sony (R1-2408402) Proposal 3: The association of part A and part B in the collected data sample (for training data collection of AI/ML based positioning) can be based on timing information or location information.

- Intel Corporation (R1-2408290) Proposal 3: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, as part of assistance data regarding DL PRS configuration, the LMF could provide a UE with one or more configurations of measurement time windows during which the UE is expected to perform measurements towards data collection and/or report ground-truth labels. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues is considered as a starting point. Proposal 4: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows to configure and transmit DL PRS with certain requested configuration(s) for data collection for model training. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues is considered as a starting point. Proposal 5: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide a UE with configuration of a measurement validity area within which the UE may perform the measurements and/or report ground-truth labels for data collection for model training. - The concept of AreaID-CellList, as described in TS 37.335, to list the NR Cell-Ids of the TRPs belonging to a particular network area within which the UE is to perform measurements and/or report ground-truth labels, can be reused. Proposal 6: For NG-RAN node-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows during which the NR-RAN node is expected to schedule SRS transmissions from one or more of served Ues and perform associated measurements on the SRS resources for data collection for model training. - For the measurements on UL SRS from Ues in other cells, LMF could provide NG-RAN nodes with information for SRS scheduling in neighboring cells similar to the current UL positioning framework. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous UL positioning measurements across Ues is considered as a starting point.

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 5: Training data collection can be configured to collect any proportion of unlabeled (data samples with only part A) and labeled (data samples with both parts A and B) data.

- ETRI (R1-2408559) Proposal 8: For training data collection, an additional tag is required for both Part A and Part B to ensure efficient dataset classification. • FFS: What specific information should be included in the tag. Figure 2 Data tagging example

- CEWiT (R1-2408923) Proposal 7: To associate the measurement and its related data to collected labels and its related data, the data generating entities can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and Ids, Area ID along with the collected measurement, labels and its corresponding time stamps.

- TCL (R1-2408267) Proposal 10: Down-select the method for training data association: a. UE validates the data association by implementation and sends an availability indicator or causes of unavailability to LMF. b. LMF validates the data association with the information of immobility duration.

- Tejas Network Limited (R1-2407747) Proposal-9: We propose that Option-2 should be used to defines the quality of Part-A of the training data and Option-1 should be used to defines the quality of Part-B of the training data.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 3: For training data collection of AI/ML-based positioning, from RAN1 perspective, the quality indicator of label reuses the existing IE NR-TimingQuality in TS 37.355. Proposal 4: The Measurement time in time stamp can be an optional field for training data collection, if Measurement time is not present, the LMF may assume that the value is the same as TRP’s SFN initialization time: IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Measurement time; O; Relative Time 1900 9.2.36; If this IE is not present, the LMF may assume that the value is same as TRP’s SFN nitialization time.. Proposal 6: The association between part A and part B can be determined by the model training entity based on the collected measurement information and label information. - Part A can be provided by: - TRP measurement result in TS 38.455 (case 3a, case 3b). - SignalMeasurementInformation in TS 37.355 (case 1, case 2a and case 2b). - Part B can be provided by: - ProvideLocationInformation in TS 37.355 (case 2b, case 3b). - ProvideAssistanceData in TS 37.355 (case 1, case 2a). - New assistance information sent from LMF to gNB (case 3a). - Part A and Part B can be associated using: - the timestamp provided in measurement and/or location information. - UE ID associated with the measurement and/or location information.

- vivo (R1-2407849) Proposal 2: For pairing Part A and Part B as one training data sample, the following methods can be further considered: • Method 1: Paired Part A and Part B from different entities at least according to the attached time stamps of Part A and Part B. • Method 2: Reporting paired Part A and Part B if both information comes from the same entity Proposal 3: Specify the data format that includes paired Part A and Part B Proposal 4: For the time stamp, the existing mechanism can be reused for data collection. Proposal 5: Reuse the existing IE, i.e., the combination of confidence and uncertainty, as the quality indicator of label for AI/ML based positioning.

- Nokia (R1-2408545) Proposal 21: The quality of content of the dataset used for training, RAN1 considers the sample’s density (#samples/m2) and the distribution similarity between the target dataset and the uniform distribution. Proposal 22: For ground truth data collection, to assess the quality of ground truth, LMF may assist UE for obtaining label consistency score/quality of the positioning estimates from one or multiple positioning sources. Proposal 23: For data collection of high-quality ground truth, LMF assists Ues on linking the PRU location (absolute information) with the relative information generated by sensors (e.g. accelerometers, gyros, magnetometers) deployed in the UE. Proposal 24: For training data collection of AI/ML based positioning, quality indicator of label is represented by a value between a predefined range (e.g. between 0 and 1 with 1 indicating the highest quality). FFS the predefined range. Proposal 25: For training data collection of AI/ML based positioning, an additional data (e.g., TRP ID, beam ID) may be included in Part A along with the data type to indicate if it is mandatory or optional. Proposal 26: For reporting of UTC time with SFN time info as time stamp of channel measurements and label is considered in RAN1 as configurable. Proposal 27: In data collection for ground truth generation by the target UE for UE-side models, LMF may indicate UE positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) for an estimation to be used as ground truth.

- CMCC (R1-2407893) Proposal 5: For AI/ML based positioning, it needs more discussion on the feasibility of obtaining the ground-truth label via PRUs, in which case the training dataset size is large. - Proposal 8: For UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the reliability or the positioning accuracy should also be reported.

• CATT, CICTCI (R1-2408028) Proposal 7: For training data collection of case 1 and case 2b, further study how to match the measurements from multiple TRPs with one label by related time stamps. Proposal 19: For case 1 and case 2a, when UE sends a data collection request to LMF, the data collection request contains some assistance information related to the PRS/TRP set expected by UE.

### 1st round discussion

Regarding the issue of time stamp for channel measurement and label generated by PRU or non-PRU UE, majority companies support including UTC time as a part of the time stamp (Please update/correct if your view is not captured correctly).

Whether to include UTC Time in the time stamp for channel measurement and label generated by UE (PRU or non-PRU UE)?

| Option | Supported by: |
| --- | --- |
| Include UTC Time (7 sources) | Qualcomm, Apple, NEC, Ericsson, Sharp, NTT DOCOMO (optional), Nokia (configurable) |
| No need of UTC Time | N/A |

Thus, it is proposed that both Ies are included in the time stamp, instead of choosing one.

Proposal 4.1.2-1

For training data collection of AI/ML based positioning, when channel measurement and/or label is generated by UE (PRU or non-PRU UE), both Ies are included in the time stamp: NR-TimeStamp and UTCTime in TS 37.355;

- Note: NR-TimeStamp and UTCTime are intended to refer to the same/similar clock time.

Proposal 4.1.2-1A

For training data collection of AI/ML based positioning Case 1, when channel measurement and/or label is generated by UE (PRU or non-PRU UE), both Ies are included in the time stamp: NR-TimeStamp and UTCTime in TS 37.355;

- Note: NR-TimeStamp and UTCTime are intended to refer to the same/similar clock time.

| NR DL-TDOA Positioning | NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, … } OPTIONAL, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |
| --- | --- |
| NR DL-AoD Positioning | NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { sfn-time-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

…

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

|  | Company |
| --- | --- |
| Support | InterDigital, Nokia, Apple, NEC, TCL |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| HW/HiSi | The proposal contains the case when channel measurements (and label) can be generated by a PRU, which would correspond to Case 2b, which is not be discussed in 2024Q4. Regarding the reports of position label by a PRU, e.g., to support Case 3b, legacy already supports the indication of the UTCTime in the locationTimestamp-r13 within the common Ies for providing location information. Thus, no agreement is needed. |
| Nokia | We should agree on the proposed sentence at least as a conclusion. |
| Qualcomm | First, this needs to be applied to LMF’s generated label and measurements (if any). Second, time stamps can be optionally included. It also need to include UTCTime-msec field in the list. [TS 37.355] UTC-Time-r15 ::= SEQUENCE { utcTime-r15 UTCTime, utcTime-ms-r15 INTEGER (0..999), … } utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.. Third, it is not clear to us the intention of the last Note. Could you please clarify? Let’s also decouple discussion of measurements and/or label. Proposal 4.1.2-1 (updated) For training data collection of AI/ML based positioning, when channel measurement and/or label is generated by UE (PRU or non-PRU UE), for label, both Ies can be included in the time stamp: NR-TimeStamp or UTC-Time-r15 in TS 37.355; - Note: NR-TimeStamp and UTCTime are intended to refer to the same/similar clock time. |
| DOCOMO | Both NR-TimeStamp and UTCTime can be included, but in terms of overhead, both are not need to be reported at the same time. UTCTime can be optionally reported. |
| Fujitsu | OK |

For the quality indicator of timing measurements in Part A (channel measurements), NTT DOCOMO and Ericsson proposed to reuse the existing IE NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455.

Proposal 4.1.2-2

For training data collection of AI/ML based positioning, the quality indicator of timing measurements in Part A (PDP, DP) is provided by:

- Reusing the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

Proposal 4.1.2-2A

For training data collection of AI/ML based positioning, the quality indicator of timing measurements in Part A (e.g., PDP, DP) is provided by:

- Reusing the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

|  | Company |
| --- | --- |
| Support | Hw/HiSi, DOCOMO, TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | The general principle is fine (CIR could be added in the bracket if support phase information). |
| Apple | What about the power dimension for PDP? Do we have a „power quality“ for this ? |

Regarding the quality indicator of label for Case 1/2b/3b, the following proposal was discussed in RAN1#118, i.e., study and decide between Option 1 and 2.

Proposal 4.1.3-2 (RAN1#118, R1-2407272) For training data collection of AI/ML based positioning Cases 1, 2b, 3b, from RAN1 perspective, quality indicator of label is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE that provides UE location uncertainty. For example, LocationUncertainty-r16 in 37.355; - Option 2: define a new IE on UE location quality. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - FFS: how to determine the value.

In contributions to RAN1#118bis, companies followed up on this topic and expressed their view. Based on the analysis and proposals submitted, companies’ view are captured in the table below (Please update/correct if your view is not captured correctly).

Whether to support Option 1 or Option 2 for quality indicator of label in Case 1/2b/3b?

| Option | Supported by: |
| --- | --- |
| Option 1 (existing IE) 10 Sources | Qualcomm, Huawei/HiSilicon, Tejas, Ericsson, ZTE/Pengcheng Laboratory, vivo, Xiaomi, Lenovo |
| Option 2 (new IE, 0…1) 3 Sources | InterDigital, Apple, Nokia |

Thus, it is recommended that the majority view is to be adopted. Note that, companies have detailed proposals on what existing Ies should be used, and whether different Ies are used for different model output. Such details can be further discussed.

Proposal 4.1.2-3

For training data collection of AI/ML based positioning Cases 1/3a/3b, from RAN1 perspective, quality indicator of label is represented by

- reusing an existing IE that provides UE location uncertainty. For example, LocationUncertainty in 37.355;

Proposal 4.1.2-3A

For training data collection of AI/ML based positioning Cases 1/3a/3b, from RAN1 perspective, quality indicator of label (when the label is reported) is represented by

- For Case 1/3b: reusing an existing IE that provides UE location uncertainty. For example, LocationUncertainty in 37.355;

- For Case 3a: reusing an existing IE that provides uncertainty of the intermediate positioning measurement.

|  | Company |
| --- | --- |
| Support | DOCOMO |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | We should at least identify options we have on the table and study further for each option. We suggest to start from the proposal discussed in RAN1#118. We made a small modification to the proposal. Modified Proposal 4.1.2-3 For training data collection of AI/ML based positioning Cases 1, 3b, from RAN1 perspective, quality indicator of label is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE that provides UE location uncertainty. For example, LocationUncertainty-r16 in 37.355; - Option 2: define a new IE on UE location quality, issued by LMF. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - FFS: how to determine the value. |
| ZTE | For direct AI/ML positioning, i.e., case 1/3b, a quality indicator can be associated with the location. For case 3a, the label could be intermediate features. This could be another discussion for case 3a. |
| HW/HiSi | If the label is signaled within 3GPP specification, then it is fine to reuse LocationUncertainty. For Case 1 it is not for sure that training data collection needs to be specified. For Case 3b, the label may already be available at the LMF, then there is no point to signal it. For Case 3a we do not support this proposal since the label is not a UE location. |
| Nokia | 3 companies agree on supporting Option 2 (new IE indicator). However, the proposal is not reflecting it. In the worst case we can say that there is no concensus. In our view, there are many aspects that define the Label quality indicator in addition of error in meters. |
| Qualcomm | For Case3a, the label can be timing or LOS, it is not clear how it is related to the field For Case1/3b, existing specifications include a listing of potential uncertainty and confidence fields for different location shape types (not only “LocationUncertainty”). Proposal 4.1.2-3 (updated) For training data collection of AI/ML based positioning Cases 1/3a/3b, from RAN1 perspective, quality indicator of label is represented by - reusing an existing IE that provides UE location and uncertainty. For example, LocationUncertainty LocationCoordinateTypes in 37.355; - Note: The label and quality indicator can be coupled in one IE as in existing Ies |
| TCL | OK. Reusing the exisiting IE is preferred. |
| OPPO | Similar view with companies. For case 3a, since the label is not UE location, should be discuss seperately. |

### 2nd round discussion

Regarding Proposal 4.1.2-1, the intention is that both NR-TimeStamp and UTCTime are used, which is different from “CHOICE” in existing measurementReferenceTime.

Regarding the Note: it was from draft proposals in RAN1#118. It was to address the concern that the two timing IE should not refer to two different measurement time.

| NR DL-TDOA Positioning | NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, … } OPTIONAL, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |
| --- | --- |
| NR DL-AoD Positioning | NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { sfn-time-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

…

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

The proposal is updated below to address the comments.

Proposal 4.1.3-1

For training data collection of AI/ML based positioning Case 1, if channel measurement and/or label is generated by UE (PRU or non-PRU UE) and reported, both Ies are included in the time stamp: NR-TimeStamp and UTCTime in TS 37.355;

- Note: NR-TimeStamp and UTCTime are intended to refer to the same/similar clock time when channel measurement and/or label is generated.

|  | Company |
| --- | --- |
| Support | NEC |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| ZTE | UTC time could an optional field, as in some cases, the measurement is not always reported in a bundle. If the time span of the collected data is less than 1024 SFN, NR-TimeStamp should be sufficient. |
| OPPO | Same view with ZTE. Considering the overhead, UTC time can be optional. |
| HW/HiSi | The UTC time should be optional, thre is no eed to change the legacy. Also, the LMF can handle to set a time tag based on its implementation. |
| Nokia | Agree with ZTE comment, based on this, we propose the following rewording: Proposal 4.1.3-1 (Updated) For training data collection of AI/ML based positioning Case 1, if channel measurement and/or label is generated by UE (PRU or non-PRU UE) and reported, both Ies are included in the time stamp: NR-TimeStamp and UTCTime in TS 37.355; - Note: NR-TimeStamp and UTCTime are intended to refer to the same/similar clock time when channel measurement and/or label is generated. - Note: . If the time span of the collected data is larger than 1024 SFN, UTCTime complements NR-TimeStamp. |
| Qualcomm | We don’t support. Let’s first discuss necessity of letting UE report measurments and/or label for data collection in Case1. |
| DOCOMO | The UTC time is needed only when the measurement periodicity exceeds 10.24s. We don’t support mandotorily report the UTC time as the overhead will be large. |

Proposal 4.1.3-2

For training data collection of AI/ML based positioning Cases 1/3a/3b, from RAN1 perspective, quality indicator of label (when the label is reported) is represented by

- For Case 1/3b: reusing an existing IE that provides UE location uncertainty. For example, LocationUncertainty in 37.355;

- For Case 3a: reusing an existing IE that provides uncertainty of the intermediate positioning measurement.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| ZTE | Case 3a should be further discussed. For example, the intermediate measurement could be power information, but the current spec doesn’t define power measurement quality indicator. |
| InterDigital | As we commented offline, regarding the first bullet we should start from the study proposal. We copied and pasted the proposal we suggested in the previous round below. Modified Proposal 4.1.2-3 For training data collection of AI/ML based positioning Cases 1, 3b, from RAN1 perspective, quality indicator of label is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE that provides UE location uncertainty. For example, LocationUncertainty-r16 in 37.355; - Option 2: define a new IE on UE location quality, issued by LMF. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - FFS: how to determine the value. |
| HW/HiSi | We suggest to exclude Case 3a from the proposal. Here companies need to discuss further on the type of label firstly. |
| Nokia | Agree with InterDigital. In addition, our view is that all 2nd priority cases should not be mentioned/discussed anymore, at least on this meeting and RAN1#119. |
| Qualcomm | This needs to clarify the direction of the reporting, otherwise, let’s be aligned with potential directions discussed in working assumptions. Updated version: For training data collection of AI/ML based positioning Cases 1/3a/3b, from RAN1 perspective, quality indicator of label (when if the label is reported to/provided from LMF) is represented by - For Case 1/3b: reusing an existing IE that provides UE location uncertainty. - For Case 3a: reusing an existing IE that provides uncertainty of the intermediate positioning measurement. |

### 3rd round discussion

Since a few companies ask to study the two options, then we can go back to the previous proposal.

Proposal 4.1.4-1

For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, quality indicator of label (when the label is reported) is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2.

- Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use.

- Option 2: define a new IE on UE location quality. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

  - FFS: how to determine the value.

|  | Company |
| --- | --- |
| Support | TCL |
| Not support | InterDigital (our proposal is shown below) |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | OK |
| InterDigital | Our proposal was to clarify the details for Option 2 which is not reflected in the FL’s proposal. Our proposed modification is shown below (as we have indicated in the last two rounds). Modified Proposal 4.1.4-1 For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, quality indicator of label (when the label is reported) is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality provided by LMF to the UE. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - FFS: how to determine the value. |
| Qualcomm | We are ok to study Can you please clarify potential direction of reporting? Please recheck our response from previous round. |
| HW/HiSi | Support clearly option 1. For option 2, it is hard to judge for the entity receiving the rpeort how to judge the quality. |
| OPPO | Open to study. |
| Nokia | Ok with the intention for study. Only lets take care in the wording. RAN1 should indicate if the new proposed quality indicator is beneficial or not. In our view it is extremely useful considering the dependency of AIML positioning on the Label quality. If non-PRU UE is used for data collection, the quality indicator in meters (Option 1) is not enough. We suggest the following rewording. Proposal 4.1.4-1 (Updated) For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, quality indicator of label (when the label is reported) is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - FFS: how to determine the value. RAN1 study if Option 2 is beneficial in addition to Option 1. |

### 4th round discussion

Thursday offline:

Proposal 4.1.5-1A

For training data collection of AI/ML based positioning Cases 1, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2.

- Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use.

- Option 2: define a new IE on UE location quality provided by LMF to the UE. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

  - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia | Our preference is to join Proposal 4.1.5-1A and Proposal 4.1.5-1B. For this, we suggest to back to the previous wording before the offline today’s session. |
| Qualcomm | Discussing representation is not urgent. We first discuss source and destination relevant to this proposal and necessity of specifying content related to Case1. |

Proposal 4.1.5-1B

For training data collection of AI/ML based positioning Cases 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented by reusing an existing IE.

- FFS: the existing IE to use for each case.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| Nokia | Our preference is to join Proposal 4.1.5-1A and Proposal 4.1.5-1B. For this, we suggest to back to the previous wording before the offline today’s session. |

## Case 1: Assistance data and other signaling for training data collection

### Companies’ view from contribution

Note: on demand PRS support is mentioned but spec impact proposed is not clear.

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Observation 11: The on-demand PRS mechanism existing in legacy positioning may be optionally used for training data collection in Case 1: • For training data collection of Case 1, the UE-initiated request of on-demand PRS may be optionally used. o The applicability of LMF initiated training data collection is not clear. • Note that the on-demand PRS may not be necessarily needed for initiating training data collection. Proposal 17: For training data collection of Case 1, no enhancement is needed for the provision of assistance information (regarding area, PRS, network synchronization error, antenna/beam, etc.) to the UE.

• CATT, CICTCI (R1-2408028) Proposal 1: In terms of DL PRS configuration for training data collection of Case 1 and 2a, both option A and option B are feasible from RAN1 perspective. - Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. - Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Proposal 2: The existing on-demand PRS procedures can be reused for data collection in Case 1 and 2a.

- Ruijie Networks Co. Ltd (R1-2408381) Proposal 3: For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, both Option A and Option B should be supported.

- Intel Corporation (R1-2408290) Proposal 8: For Cases 1, 2a, 2b, assistance information to generate ground-truth labels based on estimated location coordinates for a target UE are provisioned to a UE. - Use of NR-PositionCalculationAssistance defined in TS 37.355 is considered as a starting point.

- Qualcomm Incorporated (R1-2408838) Proposal 6: In AI/ML positioning Case1, for model training and development, deprioritize NW-side training. Proposal 8: In AI/ML positioning Case1, for data collection, support two labelling assistance options from LMF to UE: - Option 1: Ground truth label (e.g., Part B including approximate ground truth label) is generated by LMF and provided to UE - Option 2: Position calculation assistance data (e.g., IE NR-PositionCalculationAssistance TS 37.355 [7]) is provided from LMF to UE, and UE calculates ground truth label (e.g., Part B) Proposal 9: In AI/ML positioning Case1, for data collection, support Option A (UE-initiated) in which UE/PRU requests from LMF PRS configuration and activation for data collection. As a starting point, consider the following options for data collection PRS configuration and activation: - Option A1: Data collection PRS configuration/activation as part of existing procedure (e.g., on-demand PRS and existing positioning sessions) - Option A2: Data collection PRS configuration/activation as part of new data collection dedicated procedure.

- Samsung (R1-2408631) Proposal 6. Both option A and B can be supported in terms of DL PRS configuration for collecting training data

- NTT DOCOMO, INC. (R1-2408774) Proposal 4: For data collection of case 1, at least following assistance information is indicated from LMF to UE for consistency between training and inference. Further study the necessity of other information. - PRS configuration - Training data validity area (e.g., AreaID-CellList), where PRS can be assumed to be consistent within a validity area

- InterDigital, Inc. (R1-2408522) Proposal 26: For Case 1, include a timestamp in the measurement forwarded by the LMF in absolute time to indicate when the measurement was made Proposal 27: For Case 1, the timestamp with absolute time should be included per measurement in the measurement forwarded by the LMF

- vivo (R1-2407849) Proposal 6: The assistance information contained in the existing IE NR-PositionCalculationAssistance, can be reused for training data collection. Proposal 7: Suggest to firstly identify the assistance information necessary for model training and management, which should be mandatorily provided by LMF. Among the necessary assistance information, the privacy-sensitive information can be implicitly provided by an associated ID.

- Apple (R1-2408453) Proposal 17: For model training, the following elements shall be specified as part of the data collection procedure: … - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) Proposal 26: For Rel-19 AI/ML positioning, the assistance data be signaled as assistance information or configured as part of the procedure. These conditions can include one or more of the following: - Validity Area Conditions: - Validity/ Geographical area information /Cell-list /scenario/ site information, TRP set - Configuration Conditions: - Reference signal configuration(s) used to generate training data - Data Quality Conditions - Measurement data quality range (e.g. SNR/SINR range), - label data quality range (e.g. mean label positioning error) - Time range when data generated - Hardware Conditions - Network Synchronization Error - Rx-Tx timing error - Phase offset error - Antenna/beam pattern information

- NVIDIA (R1-2408391) Proposal 3: Support both of the following options for DL PRS configuration for training data collection: • Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. • Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

- Lenovo (R1-2408429) Proposal 16: AI/ML assistance data/information may include validity conditions for which a model be valid including area validity, e.g., AreaID, zone ID, area list and/or temporal validity, e.g., validity time indicators. Proposal 17: AI/ML assistance data/information may also include NW and UE additional conditions, which can be used to ensure consistency of models during training and inference. Proposal 18: For Case 1, RAN1 to support the UE-initiated DL-PRS request and on-demand PRS framework (Option A) for deriving AI/ML model input according to the requirements of the UE-sided model. Note: Case 2a can be revisited in RAN1#120.

- Sharp (R1-2408609) Proposal 10: For training data collection for AI/ML based positioning, both Option A and Option B are supported. Proposal 11: For training data collection for AI/ML based positioning, at least validity area information is supported to ensure consistency between training and inference for Option A.

- CEWiT (R1-2408923) Proposal 8: For all the use cases (Case 1, Case2a, Case 2b, Case 3a, Case 3b), RS configuration, related UE and gNB associations and Area validity from which training data is collected should be provided as assistance information to the data collecting entity. Proposal 9: Depending on the deployment scenario, network should provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), entity’s available storage/ entity’s capability (data collecting entity), and the type of input given to the model for any required preprocessing of the dataset as assistance. Observation 2: Use of IE AreaID-CellList for defining the validity of an area for data collection can be supported.

- TCL (R1-2408267) Proposal 12: The legacy assistance data provided by LMF, such as TRP information, PRS assistance data and on-demand PRS configurations, can serve as the NW-side additional conditions for UE-side model. Proposal 13: UE can provide cell information and on-demand PRS configurations to LMF to ensure consistency between training and inference for UE-side model.

- OPPO (R1-2408159) Proposal 6: In order to facilitate the training data collection at UE side for Case 1 and/or Case 2a, introduce a mechanism to enable that UE can send request for preferred or supported configuration(s) to LMF, i.e., Option A (UE initiated) • FFS: Whether this request is sent via UE capability signaling or other signaling/procedure (e.g., functionality identification). Proposal 7: Regarding the training data collection at UE side for Case 1 and/or Case 2a • Reuse the legacy LPP signaling to configures UE with the corresponding positioning RS for UE-side data collection • The associated ID is signaled along in order to facilitate the insurance of the consistency between training and inference. • No additional signaling/triggering is needed from network to start the data collection procedure at UE side. • The format/content of collected data are up to implementation of UE and no specification is needed in 3GPP Proposal 8: For training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), support LPP signaling from LMF to indicate at least the following information - Configuration of Positioning RS - The type of measurement results - How to report the collected data (e.g., periodic reporting, event-triggered reporting)

- Tejas Network Limited (R1-2407747) Proposal-10: The PRS/SRS configurations, TRP-ID, frequency layer ID, and spatial filter information should be captured as part of the assistance information.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 8: For training data collection of case 1 and case 2a, new assistance data associated with training data validity area is not needed. - The current configuration information can be used for training data collection. Proposal 9: For training data collection of case 1 and case 2a, in terms of DL PRS configuration for collection training data, both Option A and Option B are feasible: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Proposal 10: For option A (UE initialed DL PRS configuration), the following enhancement can be supported: - On-demand PRS request includes the suggested number of activated TRPs that transmit DL PRS.

- Nokia (R1-2408545) Proposal 29: For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 to consider as start point Option B on assistance data.

- Ericsson (R1-2407649) Proposal 41 For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 concludes that the following options on assistance data, using legacy mechanisms, can be supported: • Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. • Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

### 1st round discussion

Regarding DL PRS configuration for collecting training data in Case 1/2a, RAN1 agreed to study the following two options:

Agreement (RAN1#118) For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Based on the analysis and proposals submitted in contributions to RAN1#118bis, companies’ views are captured in the table below (Please update/correct if your view is not captured correctly).

Whether to support Option A and/or B for DL PRS configuration for collecting training data in Case 1?

| Option | Supported by: |
| --- | --- |
| Option A (UE initiated) only (4 sources) | Qualcomm, Lenovo, OPPO, Intel |
| Option B (LMF initiated) only (1 source) | Nokia |
| Both Options A and B (10 sources) | NVIDIA, Ericsson, Sharp, CATT/CICTCI, ZTE/Pengcheng laboratory, Ruijie Networks, Huawei/HiSilicon (UE-initiated is Optional),New H3C, NEC |

The summary above shows that majority companies believe both Option A and B can be supported.

Proposal 4.2.2-1

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, both options are supported, and legacy mechanisms are reused:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Proposal 4.2.2-1A

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, both options are supported, and using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital (last note should be removed, case 2a should be excluded) |

| Company | Comments |
| --- | --- |
| InterDigital | We are not sure about the purpose of the last note |
| CATT, CICTCI | Agree. |
| ZTE | Does this imply that no more enhancement is required for AI/ML towards option A and option B? The previous agreement said ‘using legacy mechanisms as a starting point’. We prefer using the previous wording. |
| New H3C | OK |
| HW/HiSi | Seems ok |
| Nokia | If Option A and Option B are agreed. In our view there is no motivation to discuss on this topic. |
| Apple | Okay apart from last note |
| NEC | We also support both Option A and Option B(Have added our company in above table.). |
| DOCOMO | OK |
| Fujitsu | OK |

Proposal 4.2.2-2

For AI ML positioning Case 1, the assistance data from the LMF to the UE include at least:

- Area information

  - FFS: details for area information assistance data, including granularity of the area information, e.g., use the existing AreaID-CellList as a starting point.

- DL PRS configurations

  - Note: Existing IEs on DL PRS configurations, if necessary, can be reused from RAN1 perspective.

- FFS: other assistance data, e.g., TRP location information, time synchronization information of the TRPs, spatial direction information of the DL-PRS resources. Existing Ies, if applicable, can be reused as a starting point.

|  | Company |
| --- | --- |
| Support | DOCOMO |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | Is PRS configuration considered as assistance data? Our understanding is that assistance data and PRS configurations are separate and complementary to each other. |
| ZTE | For case 1, all current assistance data are required as case 1 is UE-based positioning, the change of TRP location, RTD, etc., could affect the model. A generalized wording could be “For AI ML positioning Case 1, the existing assistance data from the LMF to the UE can be reused as a starting point.” |
| HW/HiSi | The need of the other assistance data in the FFS should be justified before including it in the proposal. As such, we propose a modified version of the proposal by removing the FFS. We think further that for both DL PRS and Area information, it is clear existing IEs can be re-sued. Proposal 4.2.2-2 For AI ML positioning Case 1, re-use the existing assistance data from the LMF to the UE as a starting point, include at least: - Area information - FFS: details for area information assistance data, including granularity of the area information, e.g., use the existing AreaID-CellList as a starting point. - DL PRS configurations - Note: Existing IEs on DL PRS configurations, if necessary, can be reused from RAN1 perspective. FFS: other assistance data, e.g., TRP location information, time synchronization information of the TRPs, spatial direction information of the DL-PRS resources. Existing IEs, if applicable, can be reused as a starting point. |

## Case 3a/3b: Assistance data and other signaling for training data collection

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• CATT, CICTCI (R1-2408028) Proposal 3: For case 3a, the following methods for ground truth label generated by LMF are considered: - Method 1: UE/PRU provides the location related information to LMF for generating the ground truth label. - Method 2: Multiple gNBs/TRPs provide the SRS-pos measurements (e.g. UL RTOA) to LMF for estimating UE’s location coordinate and the UE’s location coordinate is used to generate the ground truth label.

- NEC (R1-2408214) Proposal 31: Specific the gNB-initiate RS configuration request to facilitate model training/retraining/fine-tuning or ground truth based model monitoring at gNB side in Rel-19, at least for the case where data transfer is not supported.

- Baicells (R1-2408309) Proposal 7. For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

- InterDigital, Inc. (R1-2408522) Proposal 33: Study feasibility of obtaining the ground truth for timing information for Case 3a

- Xiaomi (R1-2407951) Proposal 14: For Case 3a, leave RAN3 to assess whether there is specification impact on the collected data format definition, information exchange between TRP and gNB and information exchange between different gNBs considering different training location Proposal 15: For Case 3a, RAN1 is responsible to identify whether there is any need for assistance information from UE and what kind of assistance information is needed

- NTT DOCOMO, INC. (R1-2408774) Proposal 3: For data collection of 3b, the measurement report and related assistance information for AI/ML based positioning is determined by LMF, - For case 3b, for data collection of training, inference and performance monitoring, LMF initiates corresponding measurement reporting at gNB.

In previous meeting, the following was agreed for training data collection of case 3a/3b:

Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

#### FL view

Based on the received proposal, FL has not identified further proposals for case 3a/3b assistance data to be discussed.

- it is proposed to provide more details on how to produce the label used for case 3a (CATT). From the FL perspective, the methods 1 and 2 proposed are already supported, since the LMF can already request measurements or request locations.

- It is also proposed to support a gNB initiated request for RS configuration in case 3a (NEC). however, this is a RAN3 procedure issue. NRPPa procedures are currently always initiated at the LMF.

- Feasibility of transforming the ground truth label into a model output (RTOA, gNB RxTx) is proposed by Interdigital. The assistance data in the form of SRS configuration is already agreed, thus how to proceed with the study is not clear.

## Other (e.g., model training entity, deliver collected data to model training entity)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• CATT, CICTCI (R1-2408028) Proposal 4: For case 2b and case 3b, when LMF side collects training data, LMF side can use a quality indicator condition or criteria to indicate the required quality of the collected data.

- InterDigital, Inc. (R1-2408522) Proposal 12: In Case 1, support the target UE to request to the LMF a specific level (e.g., above a threshold) of a ground truth label quality for LMF measurement forwarding

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 6: Support opportunistic/event-based training data collection for follow-up model improvement/adaptation. Data collection can be triggered for example when: - A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected. - The UE or a known PRU is in a sub-area that has become challenging for the model. - High quality GTLs can be generated. - A monitoring event indicating reduced model performance (and possibly switching to a different model or to fallback) has been reported.

- NEC (R1-2408214) Proposal 8: For case 1, confirm the following working assumptions for generating the measurement and its related data and for generating the label and its related date for training data: - For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. - For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF - Note: transfer of the label and its related data is out of RAN1 scope. Proposal 9: Confirm the following working assumptions reached in RAN1 #117 meeting for the generating the label for training data case 3b: - For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF - Note: transfer of label and its related data is out of RAN1 scope. - Note: It is assumed that user data privacy of non-PRU UE is preserved. - Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Proposal 11: Endorse the integration of mixed datasets from diverse drop/clutter parameters, network synchronization errors, or deployment scenarios to train a model with robust generalization capabilities, or for fine-tuning the model to achieve higher accuracy in the target scenario in Rel-19. Proposal 18: A quality indicator should be defined for a data sample, and it’s determined based on the quality indicator(s), if available, of associated measurement and ground truth label. Proposal 19: Data generation entity can initially report the data fulfilling the quality indicator threshold and then reports supplemental data in case that previously reported quality data is not adequate. Proposal 20: Support to collect UE coordination information in a zone-based format where the zone size is predefined. Proposal 32: Specific distinct RS configuration patterns for the different stage of the LCM, e.g., employing different time domain period for data collection between model training and model monitoring.

- Sony (R1-2408402) Proposal 4: Support model transfer operation from server node (e.g., LMF) to UE/gNB to support AI/ML-assisted positioning. Proposal 5: Define the AI/ML model structure and parameters for model transfer to support AI/ML-assisted positioning.

- ITL (R1-2408908) Proposal 3: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF

- Intel Corporation (R1-2408290) Proposal 1: • For AI/ML-based positioning, consider the entities for ground-truth label generation as presented in Table 2. Table 2 Summary of entities responsible for ground-truth labels for different positioning cases Entity; Case 1; Case 2a; Case 2b; Case 3a; Case 3b. UE with estimated/known location. Positioning Reference Unit (PRU). Network entity (other than LMF); *. LMF with known PRU location. Note *: RAN1 #116bis agreement states “whether other network entities can generate label for Case 3a is out of RAN1 scope”.. Proposal 2: The necessity/applicability of information transfer (including one or more of: measurement data, ground-truth labels, inference results) and, if supported, the source-destination entities of information transfer for model input and model output depends on particular AI/ML based positioning sub-use-case as captured in Table 3. Table 3. Summary of information transfer support necessary for different positioning cases Case 1; Case 2a; Case 2b; Case 3a; Case 3b. Model input; Opt. A: No data transfer Opt. B: Data transfer including ground-truth labels: LMF UE/UE-side server; No data transfer; Channel measurement transfer: UE LMF; No data transfer; Channel measurement transfer: NG-RAN node LMF. Model output; NA; Model inference output transfer: UE LMF; NA; Model inference output transfer: NG-RAN node LMF; NA.

- Qualcomm Incorporated (R1-2408838) Proposal 7: In AI/ML positioning Case1, for data collection, agree on working assumptions (from RAN1#116bis) for measurement and data generation entities in Case1.

- Samsung (R1-2408631) Proposal 13. RAN1 supports UE to notify network whether such UE is a data provider or not. Proposal 14. RAN1 supports UE to notify network that it is inapplicable to provide data sample when certain condition is not satisfied. FFS certain condition, e.g., data sample requirement related condition. Proposal 15. RAN1 supports the collected data signalled from PRU (other UE) to UE deployed with AI model.

- OPPO (R1-2408159) Proposal 9: Regarding the training data collection for NG-RAN node assisted positioning with gNB-side model (Case 3a) - gNB do measurement based on SRS - The existing procedure/mechanism can be reused for this purpose - gNB can decide itself when/how to start the data collection procedure. - The contents of collected data are up to implementation and no specification is needed in 3GPP - LMF delivers ground-truth label(s) (or the information based on that the label can be derived) to gNB via NRPPa siganling - gNB needs to send request to LMF on what the groud-truth label (or the information based on that the label can be derived) is Proposal 10: For training data collection at NW side for NG-RAN node assisted positioning with LMF-side model (Case 3b), support the following mechanisms - gNB reports measurement results (corresponding to model input) and the associated timestamps via NRPPa protocol - The corresponding label(s) can be reported optionally or LMF generates the associated labels based on the know location of the corresponding PRU - NRPPa signaling from LMF to indicate gNB - The type of measurement results - How to report the collected data (e.g., periodic reporting, event-triggered reporting)

### 1st round discussion

Multiple companies propose to convert the working assumption to agreement. Considering that these WA are already normative and that the editors can already start working on the specs based on working assumptions, we propose not to dedicate online time to this.

On the issue of quality threshold for the training data, multiple companies (e.g., NEC, InterDigital, CATT) propose to support a quality threshold for a measurement in the LMF request to the UE /gNB.

Proposal 4.4.2-1

For training data collection of AI/ML based positioning at least for Case 3b, the LMF may include a measurement quality threshold in the measurement request to the gNB. Measurements reported by the gNB are expected to have a quality better than the measurement quality threshold. Here the measurement refers to a full channel measurement.

FFS: details of the measurement quality threshold

|  | Company |
| --- | --- |
| Support | CATT/CICTCI,New H3C, NEC |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Support. This can avoid reporting unnecessary low-quality data. |
| ZTE | The general idea is fine. It is feasible to let LMF configure data collection requirements for AI/ML positioning model. |
| Apple | Need to define the quality metric before making this decision. |
| Fujitsu | OK |
| Nokia | Getting a generic agreement may not be helpful. We disagree on the current proposal version; further details are required to discuss it. |

Proposal 4.4.2-2

For training data collection of AI/ML based positioning at least for Case 1 and 3a, the LMF may be provided with a label quality threshold in the label request from the UE (Case 1) or gNB (Case 3a). The provided labels by LMF are expected to have a quality better than the label quality threshold.

Proposal 4.4.2-2A

For training data collection of AI/ML based positioning at least for Case 1 and 3a, the LMF may be provided with a label quality threshold in the label request from the UE (Case 1) or gNB (Case 3a). The labels provided by LMF to UE (Case 1) or gNB (Case 3a) are expected to have a quality better than the label quality threshold.

|  | Company |
| --- | --- |
| Support | InterDigital, CATT/CICTCI,New H3C |
| Not support | Apple |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Support. This can avoid reporting unnecessary low-quality data. |
| Apple | Define quality metric before making this decison |
| NEC | For case 3a, LMF may not need to provide quality threshold of label to gNB as gNB may not be the entity to generate label. |

# Model inference

This section discusses issues for the LCM stage of model training.

## Ensure consistency between training and inference

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 18: For Case 1 and 2a, no need to introduce an associated ID to provide NW-side additional condition for ensuring the consistency between training and inference. • Legacy assistance parameters in legacy positioning method can be reused if needed.

- CATT, CICTCI (R1-2408028) Proposal 26: For AI/ML-based positioning, both the following concepts of validity area for training data collection can be considered: - Concept 1: Assistance data or measurement validity area configured by LMF, i.e. AreaID-CellList introduced in Rel-17. - Concept 2: Validity area composed of TRP(s) corresponding to channel measurement, keeping the AI/ML model valid. Proposal 27: For case 1 and case 2a, if NW-side additional condition is identified as necessary for positioning, the following options can be considered to ensure the consistency between training and inference regarding NW-side additional conditions for inference at UE-side model. - Option 1: Based on associated ID. - FFS on what can be assumed by UE with the same associated ID across training and inference. - Option 2: Based on performance/model monitoring.

- Fujitsu (R1-2408104) Proposal-6: Taking training-inference consistency into account, the UE is provided with the following assistance data for its training data collection, which can be used for checking model applicability in model inference phase: - Assistance data on validity area - Existing AreaID-CellList can be as the starting point. - Assistance data on DL PRS configurations - Existing IEs on DL PRS configurations can be the starting point. Proposal-7: The study on associated ID to indicate NW-side additional condition/configuration involving multiple TRPs is suggested to be deprioritized in AI/ML-based positioning.

- NEC (R1-2408214) Proposal 28: Support to introduce validity area to ensure the consistency between training and inference for UE side model. - The legacy AreaID-CellList specified in TS37.355 can be reused as the start point to specify the validity area. Proposal 29: Support to introduce associated ID as an implicit way to ensure consistency between training and inference for AI/ML based positioning. Further study on: - Associated ID related to the additional conditions for one TRP/gNB separately or for multiple TRPs/gNBs jointly. - Whether disclose gNB’s additional conditions to LMF or not. Proposal 30: For case 1, following three alternatives can be used as starting point to ensure the consistency between training and inference: - Alt.1: UE requests the expected metadata from LMF. UE can provide the expected metadata to LMF for UE side inference data collection based on the metadata for training data collection such that consistency between training and inference will be ensured. - Alt.2: LMF maintains the association between metadata and UE/model. LMF can ensure consistency between training and inference based on the maintained association. - Alt.3: UE maintains the association between metadata and model. UE firstly determine whether consistency is ensured and perform model inference accordingly.

• Baicells (R1-2408309) Proposal 5. To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning. Proposal 6. For Case 1, support the transfer of UE-side model information to LMF via LPP.

- Ruijie Networks Co. Ltd (R1-2408381) Proposal 4: Associated ID should be adopted at least as a part of assistance data to DL PRS configuration for collecting training data, in Case 1 and 2a.

• Sony (R1-2408402) Proposal 9: For ensuring the AI/ML model consistency between the training and inference operation, support the association of the reference signal (e.g., DL-PRS) characteristics to the trained AI/ML model and AI/ML model inference operation. Proposal 10: Further study the signalling procedure to ensure the consistency between AI/ML training and AI/ML inference.

- Intel Corporation (R1-2408290) Proposal 21: • To ensure consistency between model training and inference, the measurement validity area configuration that may be provided for model training is assumed to apply also for model inference. Proposal 22: Consider the consistency between RS configurations as a NW-side additional condition to ensure consistency between training and inference. - FFS details of conditions of consistency between RS configurations Proposal 23: Consider the consistency between channel/link conditions/characteristics as a NW-side additional condition to ensure consistency between training and inference. - FFS details of conditions of consistency between channel/link conditions/characteristics Proposal 24: Consider the consistency between model input formats as a NW-side additional condition to ensure consistency between training and inference.

- Qualcomm Incorporated (R1-2408838) Proposal 2: In AI/ML positioning Case1, for training and inference consistency, UE is responsible to ensure consistency and LMF provides UE with necessary information. Proposal 3: In AI/ML positioning Case1, for training and inference consistency, as a starting point, support at least information in existing assistance data from LMF to UE during both training data collection and inference times - FFS: Other assistance data not defined in existing IEs Proposal 10: In AI/ML positioning Case1, for data collection, as a starting point, support at least information in existing assistance data (up to Rel-18) from LMF to UE for enabling model development and ensuring training and inference consistency

- InterDigital, Inc. (R1-2408522) Proposal 1: Prioritize discussion on determination of consistency in NW side additional conditions and LCM details for Case 1 and deprioritize discussion related to potentially new measurements for Case 3b Proposal 2: At least frequency layer IDs, cell IDs and TRP IDs should be the same between training and inference phase to maintain consistency Proposal 3: To ensure consistency between data collection phase and inference phase, the UE compares existing assistance information without an associated ID Proposal 4: Existing assistance information, namely NR-DL-PRS-BeamInfo, NR-RTD-Info, NR-TRP-BeamAntennaInfo and NR-TRP-LocationInfo, shall be used to check consistency between training data and inference Proposal 5: Tolerance for difference in assistance information (e.g., network synchronization error, difference in boresight angles) should be specified Proposal 6: During performance monitoring for LCM, consistency in network-side additional conditions should be checked Proposal 8: Time validity for an AIML model(s) should be specified

- Xiaomi (R1-2407951) Proposal 26: Support additional condition alignment is beneficial for both model-based LCM and functionality-based LCM in AI-based positioning use case Proposal 27: RAN1 further investigates the necessity and feasibility for the following potential network additional conditions - Scenario/site information - Measurement configuration - Network synchronization error - Antenna information Proposal 28: Consider solutions to keep consistency regarding UE additional condition for network side model

- Samsung (R1-2408631) Proposal 26. Consistence check should be done when/before the entity deciding which model to use. Proposal 27. RAN1 to study the features or characteristics of the training data to be used for consistence check, e.g., the aspects related to generate the data input etc.

- NTT DOCOMO, INC. (R1-2408774) Proposal 5: Regarding RS configurations of data collection for each case, - For case 1, UE may send requests to NW for corresponding RS configurations, or NW may send configurations to UE. - For case3a/3b, NW sends configurations to UE for SRS transmissions. - RS configurations may include/associate with an associated ID to implicitly indicate the NW side additional conditions. Proposal 10 For consistency checking between training and inference, following information is considered: - Validity area, PRS configuration - Hardware conditions (e.g., network synchronization error, UE/gNB RX and TX timing error - PRS beam information, TRP beam antenna information Proposal 11 For AI/ML based positioning, to ensure consistency between training and inference, the existing IEs are reused as a starting point. - E.g., AreaID-CellList Proposal 12 For AI/ML based positioning, to ensure consistency between training and inference, in addition to the existing IEs, following impacts can be considered: - An associated ID can be configured within/ together with PRS related configuration - Validity area is configured/indicated, where the NW side additional conditions can be assumed to be consistent within a validity area

- ETRI (R1-2408559) Proposal 9: For the AI/ML positioning model, ensure consistency between training and inference by adopting the associated-ID. • FFS: How to configure the associated-ID

- TCL (R1-2408267) Proposal 14: LMF can indicate the PRS-RSRP threshold or SRS-RSRP threshold to ensure consistency between training and inference for LMF-side model. Proposal 15: UE can provide PRS-RSRP to LMF or gNB can provide SRS-RSRP to LMF along with the measurements for model input to ensure consistency between training and inference for LMF-side model. Proposal 16: The associated ID can be adopted for UE-side model in the use case of AI/ML based positioning for consistency between training and inference. FFS: the format of associated ID(s) for AI/ML based positioning.

- MediaTek Korea Inc. (R1-2408903) Proposal 3a-1: The associated ID could be provided to UE at training data generation and at inference to discriminate whether there is significant change in NW side implementation Proposal 3a-2: The associated ID for positioning use case could be considered at area level. If it is at cell level, there would be a certain number of IDs sent to the UE which is not so efficient

- OPPO (R1-2408159) Proposal 3: Regarding the consistency of training and inference for AI/ML based positioning of Case 1 and Case 2a, the method(s) based on “validity area of model training” and “validity area of model inference” is not workable since - It can only differentiate the AI models applicable for different areas - It can only differentiate the AI models applicable for different times in the same areas (e.g., before and after some NW optimization operations) Proposal 4: For AI/ML based positioning of Case 1 and Case 2a, some indication (e.g., in form of associated ID) is signaled from network to ensure the consistency of AI model training and AI model inference (e.g., consistency on NW-sided conditions / additional conditions) - e.g., the ID can be a special ID for positioning configurations, and can be indicated to differentiate the associated training data - The proprietary information of network should not be disclosed. Proposal 5: For AI/ML based positioning of Case 1 and Case 2a, the combination of the same associated ID, the same cell(s) and the same PRS configuration implicitly the consistency between training and inference (e.g., consistency on NW-sided conditions / additional conditions) • FFS: additional information is needed for the above combination (e.g., the same cell(s), or the same area).

- Tejas Network Limited (R1-2407747) Proposal-11: The following parameters should be collected as a part of assistance information to ensure the consistency between training and inference phase: - TRP-ID or Area-Cell ID, - Frequency layer ID, - PRS/SRS Configurations - COMB Factor (KTC), - Number of RBs (NRB), - PRS Resource ID, - PRS Resource Set ID. Proposal-11: The associated ID should be computed as a function of TRP-ID or Area-Cell ID, Frequency layer ID, PRS/SRS Configurations COMB Factor, Number of RBs, PRS Resource ID, PRS Resource Set ID etc.

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 26: There is no need to specify any NW-side additional conditions for AI/ML positioning.

- vivo (R1-2407849) Observation 17: The inconsistency of beam pattern between training and inference can result in severe performance degradation for AI/ML based positioning. Proposal 29: Further study the impact of beamforming related aspects on positioning accuracy, to support the application of AI positioning in FR2 scenarios. Proposal 30: Consider adopting the following approaches to ensure the consistency between training and inference. - For beam pattern information, NW side provides indication to assist UE in maintaining the consistency between training and inference, such as “associated ID” information or explicit beam pattern information as specified in legacy UE-based AoD positioning. o UE can indicate applicable “associated ID” or beam pattern in other procedures. Proposal 31: Further study how to connect multiple TRPs or cells with associated ID for AI/ML based positioning, and connecting an associated ID with a cell/TRP list could be considered. Proposal 32: Consider adopting the following approaches to ensure the consistency between training and inference. - UE can report supported PRS bandwidth.

- Nokia (R1-2408545) Proposal 16: RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on Assistance data, (ii) based on Associated ID, and (iii) consistency assisted by monitoring. Proposal 17: For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, an assistance data (e.g. PRS configuration) used for data collection (training) or inference shall be associated with a global cell identity (GCI) and a network-identifier (associated ID). FFS: details of network identifier (associated ID). Proposal 18: For Case 1, RAN1 to study the possibility defining validity area and granularity requirements of this area for consistency purposes. Proposal 19: To ensure consistency between training and inference, UE can indicate the timeframe of the dataset it used for training, upon which the NW compares the PRS configuration and Associated ID between the indicated timeframe and the upcoming inference time, and informs UE about any changes in the PRS configuration and Associated ID.

- Ericsson (R1-2407649) Proposal 60 When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment. Proposal 61 For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information. Proposal 62 For Case 1, the explicit assistance data provided by LMF to UE include: validity area info and NR-DL-PRS-AssistanceData-r16. Proposal 63 For Case 1, to ensure the consistency of NW-side additional condition (e.g., beam configuration of DL PRS) across training and inference, support associated ID for indicating NW conditions/configurations. Proposal 64 For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately. Proposal 65 For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

### 1st round discussion

Regarding NW-side additional conditions for ensuring consistency between training and inference regarding, companies have considered whether associated ID should be supported for the positioning use case. Note that associated ID has been agreed for the beam management use case. Based on the analysis and proposals submitted in contributions to RAN1#118bis, companies’ view are captured in the table below (Please update/correct if your view is not captured correctly).

Whether to support associated ID to provide NW-side additional condition for Case 1?

| Support (11 sources) | MediaTek, ETRI, Ericsson, TCL, OPPO, Tejas, vivo, NEC, NTT DOCOMO, ETRI, TCL |
| --- | --- |
| Not support (7 sources) | Qualcomm, ZTE/Pengcheng Laboratory, Huawei/HiSilicon, Fujitsu, InterDigital |
| Unclear/Neutral (3 source) | Nokia (3 approaches to ensure consistency), CATT/CICTCI (associated ID or monitoring) |

The above indicates that companies cannot converge on introducing associated ID or not. Among the three approaches:

    - consistency based on Assistance data,

    - based on Associated ID, and

    - consistency assisted by monitoring.

companies either support (1) or (2). No company indicated strong support for (3). Thus (3) can be excluded for now, with (1) and (2) remaining for further discussion.

Proposal 5.1.2-1

For ensuring consistency between training and inference for AI/ML positioning Case 1, RAN1 does not support:

- consistency assisted by monitoring

FFS: whether associated ID is to be introduced for providing NW-side additional condition for Case 1.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, CATT/CICTCI, New H3C, HW/HiSi, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | We are not sure about what we are not supporting. Could the FL clarify the details for “consistency assisted by monitoring”? An important discussion topic is to identify what an associated ID represents in the context of AIML positioning. Perhaps we can list different meanings an associated ID can have, or how it can be used. In addition, granularities of a value of an associate ID should also be discussed and clarified during this meeting. |
| CATT, CICTCI | We think it is safer to keep this option on the table. We agree to focus on ‘associated ID’ or assistance data first to see whether we can converge, but no need to remove a backup option at this point. BTW we think associated ID is also one kind of assistance data. |
| ZTE | The necessity of this kind of proposal is unclear saying RAN1 DOES NOT SUPPORT something. |
| HW/HiSi | Consistency assisted by monitoring does not need to be precluded per se. |
| NEC | Agree with CATT. |
| DOCOMO | Agree with CATT. |
| Fujitsu | We think introdcuing associated ID is impractical to solve the consistency issue, we are open to put all the options on the talbe and have Pros/Cons comparison. |

### 2nd round discussion

Considering companies’ input to proposal 5.1.2-1, the proposal is updated to the following. Companies can study the options and provide details for each option. Then we can make progress based on companies’ input.

Proposal 5.1.3-1

For AI/ML positioning Case 1, to provide NW-side additional condition for ensuring consistency between training and inference, RAN1 study the following options:

- Option A. using existing assistance data, which is sent at model training and model inference.

  - Proponent companies provide details on the existing assistance data to use, e.g., IEs in NR-DL-PRS-AssistanceData.

- Option B. introduce an associated ID to provide NW-side additional condition. The associated ID is sent from LMF to UE at model training and model inference.

  - Proponent companies provide details on the associated ID.

  - Note: If introduced, the associated ID can be included as a part of assistance data.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Open to study. |
| InterDigital | “Details” related to associated ID require more clarification. We have a suggestion for modification of the proposal. Modified Proposal 5.1.3-1 For AI/ML positioning Case 1, to provide NW-side additional condition for ensuring consistency between training and inference, RAN1 study the following options: - Option A. using existing assistance data, which is sent at model training and model inference. - Proponent companies provide details on the existing assistance data to use, e.g., IEs in NR-DL-PRS-AssistanceData. - Option B. introduce an associated ID to provide NW-side additional condition. The associated ID is sent from LMF to UE at model training and model inference. - Proponent companies provide details (e.g. definition, granularity of an associated ID value) on the associated ID. - Note: If introduced, the associated ID can be included as a part of assistance data. |
| New H3C | OK |
| OPPO | Fine with the proposal. |
| NEC | Support the motivation of this proposal. |
| Nokia | We support the proposal. In case any company does not agree on the current version, as a compromise we can agree to study, and we may decide on the next meeting. |
| Qualcomm | A few suggestions below to enhance the context of the proposal: Updated proposal: For AI/ML positioning Case 1, to provide NW-side additional condition for ensuring consistency between training and inference, with existing assistance data as a starting point, RAN1 study the following options: - Option A. using Providing existing assistance data, which is sent form LMF to UE, during data collection and inference at model training and model inference. - Proponent companies can provide details on enhancements to the existing assistance data to use, e.g., IEs in NR-DL-PRS-AssistanceData. - Option B. introduce an associated ID to provide NW-side additional condition. The associated ID is sent from LMF to UE during data collection and inference at model training and model inference. - Proponent companies provide details on the associated ID. - Note: If introduced, the associated ID can be included as a part of assistance data. |
| Fujitsu | Fine with the direction of the revised proposal. |
| PCL | Ok |
| DOCOMO | Generally fine with this proposal. But if we are not intend to block “consistency assisted by monitoring” in this discussion, at least an FFS is needed. |

### 3rd round discussion

During the online session, the need for associated ID was discussed. From the FL perspective, there seem to be a lack of alignment between companies regarding what is provided by the UE in Case 1 as assistance data. This was actually never agreed in detail, except for DL PRS configuration.

The discussion on assistance data depends on what company see as a baseline solution for Case 1. Based on simulation results in Rel-18 study item and Rel-19 work item, the UE in Case 1 is expected to perform “timing based finger printing”. This means DL-TDOA can be used as a baseline for Case 1 design. While no simulation results have been extensively discussed in RAN1 for AI/ML generated AoD, some companies have also mentioned using DL-AoD as a baseline for Case 1 design. The baseline assumption affects what IEs (contents in the container) can be viewed as a baseline carried in the assistance data (container). After that, companies can provide views on whether/which IEs can be removed or added for supporting Case 1.

To facilitate discussion, we can group the following component IEs in the assistance data delivered to the UE for supporting Case 1:

- (AD-IE-Group1) Assistance data IEs necessary for the generation of channel measurements for model input, i.e., assistance data needed to support reception of the DL PRS.

  - AD-IE-Group1 can be obtained from the existing AD IEs of the baseline method. If not all are needed for Case 1 DL PRS reception, some of the existing AD IEs of the baseline method do not need to be included in AD-IE-Group1.

- (AD-IE-Group2) If any, further assistance data IEs may be necessary to provide NW-side additional condition and ensure consistency. Assuming these assistance data IE(s) can take the form of explicit IEs, then a list of them can be made using IEs in legacy specification, i.e., build AD-IE-Group2 using legacy IE.

After AD-IE-Group2 is clarified, then companies can discuss and decide whether they should be provided by LMF to UE using the explicit form (i.e., sending values for the NW-side additional condition needed), or they can take the implicit form by introducing associated ID (i.e., indication of consistency of the NW-side additional condition between training and inference). Both could work since such assistance data IE are not directly needed for generating channel measurements for model input. Companies can further discuss whether the explicit form or implicit form for providing AD-IE-Group2 should be adopted.

#### Assistance data IEs necessary for Case 1 DL PRS reception (AD-IE-Group1)

As a first step, we start with the discussion of AD-IE-Group1.

Available assistance data IE in stage 2 positioning procedures for UE based positioning are applicable for UE-based DL-AoD or UE-based DL-TDOA (UE-assisted DL-TDOA and UE-assisted DL-AoD also exist, but are not analogous to Case 1). Below is a summary of the provided information for UE based positioning for each method, according to 38.305.

Table 1. Assistance data that may be transferred from LMF to the UE

| # | Information | UE-based DL-TDOA | UE-based DL-AoD | AI/ML Case 1 |
| --- | --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes | Yes |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | Yes |  |
| 3 | DL-PRS configuration of candidate NR TRPs | Yes | Yes |  |
| 4 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes | Yes |  |
| 5 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | Yes | Yes |  |
| 6 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | Yes | Yes |  |
| 7 | PRS-only TP indication | Yes | Yes |  |
| 8 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | No | Yes |  |
| 9 | LOS/NLOS indicators | Yes | Yes |  |
| 10 | On-Demand DL-PRS-Configurations | Yes | Yes |  |
| 11 | Expected Angle Assistance information | No | Yes |  |
| 12 | PRS priority list | No | Yes |  |
| 13 | Validity Area of the Assistance Data | Yes | Yes |  |
| 14 | Data facilitating the integrity results determination of the calculated location | Yes | Yes |  |
| 15 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | Yes | No |  |
| 16 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | No |  |
| 17 | The association information of DL-PRS resources with TRP Tx TEG ID | Yes | No |  |
| 18 | PRU measurements together with the location information of the PRU | Yes | No |  |

Companies are encouraged to provide their views on which elements should be included in AD-IE-Group1. For brevity, please mention which method should be the baseline, and what information elements can be removed, considering only the reception of DL PRS for generating channel measurement. Then:

AD-IE-Group1 = (full list of AD IE of baseline legacy method) – (AD IEs to be removed).

Note: Additional information for consistency is discussed separately.

Table. Companies’ view on the approach

| Company | Comments |
| --- | --- |
| Qualcomm | Sorry, I added this table because there no comment Section. I hope FL won’t mind it. First, I would like to thank FL for effort of bringing this lenghty and important discussion. Indeed, it is important to build understanding on assistance and information needed from LMF to UE in Case1. We have different perspective on the approach. I have following comments and suggestions: - We find the AD-IE-Group1 is inevtiable as neither DL-TdoA, DL-AoD, DL part of mRTT, and AIML Case1 can work without it. So let’s not waste time on it. Simply take common information provided from LMF to UE in existing methods. Let’s not start from scratch and begin with DL-PRS reception, it is a work that has been done in previous releases. - For AD-IE-Group2 on NW-side additional conditions, the RAN1 understanding is that NW additional conditions correspond to information not provided in existing specifications (i.e., before AIML is introduced). The discussion in AD-IE-Group2 is not supposed to be additional conditions. In BM and CSI, there is no spatial filter/precoder signaling in previous specifications, and that is why there were considered additional conditions. In positioning, it is a different discussion, we are not likely seeking information beyond what is already there in existing specifications (before Rel-19). |
| Nokia | As the main motivation is consistency, we believe that AD-IE-Group1 discussion is not relevant. As commented by FL, DL-TdoA and additional IEs may be the logical reference based on the SI outcome. What is not clear to us is that we should show our preferences on the indication of the baseline and potential removing some IEs. Is FL intention to have a Table of AD for legacy and another for AIML?. Our understanding is that we are losing the direction of the discussion, which is Consistency. |

Example input for AI/ML Case 1:

| # | Information | UE-based DL-TDOA | UE-based DL-AoD | AI/ML Case 1 |
| --- | --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes | Yes | Yes |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | Yes | No |
| 3 | DL-PRS configuration of candidate NR TRPs | Yes | Yes | Yes |
| 4 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes | Yes | Yes |
| 5 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | Yes | Yes | Yes |
| 6 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | Yes | Yes | No |
| 7 | PRS-only TP indication | Yes | Yes | Yes |
| 8 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | No | Yes | No |
| 9 | LOS/NLOS indicators | Yes | Yes | No |
| 10 | On-Demand DL-PRS-Configurations | Yes | Yes | Yes |
| 11 | Expected Angle Assistance information | No | Yes | No |
| 12 | PRS priority list | No | Yes | No |
| 13 | Validity Area of the Assistance Data | Yes | Yes | Yes |
| 14 | Data facilitating the integrity results determination of the calculated location | Yes | Yes | No |
| 15 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | Yes | No | Yes |
| 16 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | No | No |
| 17 | The association information of DL-PRS resources with TRP Tx TEG ID | Yes | No | No |
| 18 | PRU measurements together with the location information of the PRU | Yes | No | No |

Question: what should be included in AD-IE-Group1 for receiving DL PRS in Case 1?

Table 2. Companies’ view on AD-IE-Group1

| Company | Baseline legacy method (UE-based DL-TDOA, or UE-based DL-AoD) | IEs that can be removed from those of baseline legacy method (Use numbering in Table 1) |
| --- | --- | --- |
| Company1 | Example: UE-based DL-TDOA | Example: Remove #14 (integrity AD) |
| ZTE | I’m confusing about the classification. For AI/ML, it’s a blackbox inside. Whether AOD or TDOA is used is not visible. Why we define the baseline method? UE may use a combination of TDOA and AOD or use none of these. |  |
| CATT, CICTCI | need more time to check |  |
| InterDigital | Either UE-based DL-TDOA or UE-based DL-AoD | We don’t understand why the FL is asking us which IEs to remove. Existing assistance information, namely NR-DL-PRS-BeamInfo, NR-RTD-Info, NR-TRP-BeamAntennaInfo and NR-TRP-LocationInfo, shall be used to check consistency between training data and inference. From our perspective which ever the positioning method we use as the starting point, we should include assistance information that was not included in the original positioning method (e.g., NR-TRP-BeamAntennaInfo for UE-based DL-TDOA). |
| Qualcomm | There is no comment section. So, I added one above (if FL won’t mind it). Support info enabled for both DL-TdoA and DL-AoD. |  |
| NEC | UE-based DL-TDOA | Need more time to check |
| TCL | either | Since AD-IE-Group1 is used for the generation of channel measurements for model input, at least #3 DL-PRS configuration of candidate NR TRPs is mandatory. |

Proposal 5.1.4-1 TBD (AD-IE-Group1)

For AI/ML positioning Case 1, to enable the UE to receive the DL PRS and perform the needed channel measurements, the LMF provide the following assistance data to the UE:

- XXX

- YYY

| Company | Comments |
| --- | --- |
| Qualcomm | I added this comment table. I hope FL won’t mind it Discussion on reception of DL-PRS has been already extensively investigated in previous releases. Let’s not waste time. |

#### Additional assistance data IE for NW-side additional condition (AD-IE-Group2)

For AI/ML positioning Case 1, companies have discussed potential additional information on NW-side additional condition to maintain consistency between training and inference. Apart from AD-IE-Group1 (i.e., DL-PRS reception assistance data) in 5.1.4.1, companies can provide views on AD-IE-Group2, which is needed for maintaining consistency between training and inference. Please enter “NONE” if no IE beyond AD-IE-Group1 needs to be sent from LMF to UE for ensuring consistency.

Question: what should be included in AD-IE-Group2 for providing NW-side additional condition in Case 1?

Table 3. Companies’ view on AD-IE-Group2

| Company | AD-IE-Group2 (Use numbering in Table 1) |
| --- | --- |
| ZTE | Why we specify the purposes of assistance data? DL PRS configuration could also be used for ensuring consistency between training and inference. Our preference to select possible assistance information from current LPP and provide to UE, wherein the selected assistance information can from DL-TDOA and/or DL-AOD and/or common assistance data, e.g., DL PRS configuration. |
| Qualcomm | Proposal below seems to be conflicting with RAN1 understanding on additional conditions. Additional conditions where meant for information that are not already provided in existing specifications. If legacy specs provide info, no need to discuss additional conditions. Let’s support existing info for AIML Case1 |
| NEC | Should the content of AD-IE-Group2 be selected only from the item from above Tabel 1? In our understanding, the AD-IE-Group2 may contain the information that beyond the current Assistance data listed in above Tabel 1(this informations may be the NW’s proprietary which should not be disclosed to UE). |
| Nokia | We encourage companies to take care to mix BM and Positioning discussion. At some point the meaning of NW-side additional condition is generic and it may be different between use cases. to clarify, we copied the following section of the TR: ### 4.2.3 Additional conditions For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion.. Additional conditions may or may not specified. So, NW-side additional conditions is not only related to NW assistance data that is not specified. The point is that if this information is disclosed or not to UE for consistency purposes. Today, there are existent IEs that are not disclosed to UE however, they may be delivered by Associated ID. To answer FL question, AD IEs that are mandatory disclosed in legacy, may be complemented by the Container named “Associated ID” to disclose NW details that may or not be specified. |

Proposal 5.1.4-2 TBD (AD-IE-Group2)

For AI/ML positioning Case 1, information in the following list of legacy assistance data IEs are identified for providing NW-side additional condition:

- XXX

- YYY

After converging on AD-IE-Group2, and if AD-IE-Group2 is not empty, then companies can discuss and decide how to signal, i.e., explicit or implicit format. Some companies expressed that a combination of additional info and associated ID should not be precluded, so it is included in the study.

Proposal 5.1.4-2 TBD (how to signal AD-IE-Group2)

For AI/ML positioning Case 1, to provide NW-side additional condition for ensuring consistency between training and inference, RAN1 study following option for providing additional assistance data (AD-IE-Group2), which can be provided in addition to (AD-IE-Group1) the assistance data needed to receive the DL PRS and perform the needed channel measurements.

- Option A: Additional assistance data is provided in the format of explicit information, consisting of the following information: (enter AD-IE-Group2 here)

- Option B: Additional assistance data is provided in the format of implicit information, accounting for the following: (enter AD-IE-Group2 here)

- Option C: A list of IE (enter a subset of AD-IE-Group2 here) are provided in the format of explicit information, and the remining list of IE (enter the remaining IEs of AD-IE-Group2 here) are provided in the format of implicit information.

Table. Companies’ view on the approach

| Company | Comments |
| --- | --- |
| Nokia | To enable the progress on this topic, we suggest coming back to the study for AIML positioning if NW-side additional condition that is not available today in the UE (by using legacy) is beneficial or not. With this study, we believe that companies will understand the importance of considering Associated ID or not. With this in line, we may facilitate the next steps. Proposal For AI/ML positioning Case 1, RAN1 study the benefit and feasibility of NW-side additional condition that is not available today in the UE (by using legacy). |

### 4th round discussion

Proposal 5.1.6-1

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE during training and inference, for ensuring consistency between training and inference,

- Companies are encouraged to provide input using the excel template in their contribution to RAN1#119.

- Note: this does not mean that training and inference phase are mentioned in assistance data

|  | Company Name | Company 1 |
| --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |
| 4 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |
| 5 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |
| 6 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |
| 7 | PRS-only TP indication |  |
| 8 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |
| 9 | LOS/NLOS indicators |  |
| 10 | On-Demand DL-PRS-Configurations |  |
| 11 | Expected Angle Assistance information |  |
| 12 | PRS priority list |  |
| 13 | Validity Area of the Assistance Data |  |
| 14 | Data facilitating the integrity results determination of the calculated location |  |
| 15 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |
| 16 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |
| 17 | The association information of DL-PRS resources with TRP Tx TEG ID |  |
| 18 | PRU measurements together with the location information of the PRU |  |

| Legend |  |
| --- | --- |
| IE content is essential for Case 1 for ensuring consistency between training and inference. IE is provided to UE via explicit signalling in the assistance data. | E |
| IE content provides NW-side additional condition for ensuring consistency between training and inference. Implicit signalling is acceptable if explicit signaling is not provided. IE is provided to UE via implicit signalling in the assistance data. | I |
| IE is irrelevant for Case 1 inference, and/or for ensuring consistency. No need to signal. | N |

|  | Company |
| --- | --- |
| Support | Nokia |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Qualcomm | It is very essential to discuss training and inference consistency. Unfortunately, above proposal with Table will not be leading to an efficient outcome. Existing IEs have been developed for more than three releases and shall be the baseline for support in Case1. The discussion shall be on enhancements to existing IEs or addition of new IEs (if needed). If a few companies have concerns on existing IEs representation, we can probably discuss enhancements to address their concern. We have the following updated proposal: Updated Proposal: For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE during training and inference, for ensuring consistency between training and inference, as a starting point, support existing assistance data of NR DL-TdoA and DL-AoD - Companies can further provide inputs for further enhancement on representation of existing IEs (e.g., implicit indication) or new IEs. - Note: this does not mean that training and inference phase are mentioned in assistance data |

## Model inference operation (input, output, configuration)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Qualcomm Incorporated (R1-2408838) Proposal 18: In AI/ML positioning Case1, for inference related assistance data (AD), support at least the following information from LMF to UE: - AD on PRS resource configurations: At least AD in existing methods (e.g., UE-based DL-TdoA and UE-based DL-AoD), as a starting point - AD on training and inference consistency: At least AD for positioning calculation (e.g., NR-PositionCalculationAssistance) and beam information (e.g., NR-DL-PRS-Beam-Info), as a starting point

- InterDigital, Inc. (R1-2408522) Proposal 7: Specify a fallback positioning method in case network additional condition is not consistent

- Xiaomi (R1-2407951) Proposal 24: For inference in Case 3a, leave RAN3 to assess whether is any specification impact on deliver the input from TRP to the associated gNB and deliver the input from one gNB to another gNB in different AI model deployment options. Proposal 25: For inference in Case 3b and Case 2b - Input data format for inference need to be specified and common input data format can be used for both data collection and inference - If multiple input data formats are defined, LMF configure the input data format for inference - The signaling exchange between gNB/UE and LMF is left to RAN3/RAN2

- Samsung (R1-2408631) Proposal 17. In attached to the model output, the time stamp and quality information is supported. Proposal 18. RAN1 supports TRP to signal the model output to LMF in case 3a.

- Apple (R1-2408453) Proposal 18: For model inference, the following elements shall be specified as part of the model inference procedure: - Part A - Measurement (corresponding to model input), - Time stamp, - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.)

- OPPO (R1-2408159) Proposal 12: For the model inference for UE-based positioning with UE-side model (Case 1), - The associated ID is signaled from network to ensure the consistency of AI model training and AI model inference - No need to specify the format/contents of AI model input since they are up to UE implementation and transparent from the perspective of air interface - UE can report the estimated location information via existing LPP signaling - Introduce new information to indicate the reported results are generated by legacy method or AI-based method - FFS: whether the current field about the uncertainty can be reused or some new field/IE should be introduced to report the associated quality/probability/confidence of AI model estimated location Proposal 15: For the model inference for NG-RAN node assisted positioning with gNB-side model (Case 3a), - No need to specify the format/contents of AI model inputs since they are up to gNB/TRP implementation and transparent from the perspective of air interface - gNB can report the measurement results that are based on AI model output via existing NRPPa signaling - Introduce new information to indicate the reported results are generated by legacy method or AI-based method - FFS: whether to reuse the existing quality (e.g., Measurement Quality) or introduce a new field/IE to reflect the quality/confidence/probability of reported measurement results that are based on AI model output Proposal 16: For the model inference for NG-RAN node assisted positioning with LMF-side model (Case 3b) - gNB/TRP can do the measurement and report the results via existing NRPPa signaling - The existing measurement types are reused - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8).

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 18: For AI/ML positioning, additional assistance information of DL PRS configuration is not necessary to be defined for UE side model. Proposal 19: At least for data collection with LMF-side model, support UE/TRP to report more detailed channel measurements, e.g., CIR. Proposal 20: For AI/ML positioning, additional assistance information of UL SRS configuration is not necessary to be defined for gNB side model.

- Nokia (R1-2408545) Proposal 35: Inference of UE-side models may be requested by LMF via LPP Request Location Information. In response, UE reports its inference output to LMF via LPP Provide Location Information. Proposal 36: For Case 3b (LMF side model), for inference LMF may request reporting of UL measurements from gNB via NRPPa.

### 1st round discussion

Based on the contributions submitted, there is a need to clarify that Part A and Part B agreed for training data collection are applicable to model inference as well.

Proposal 5.2.2-1

For model inference of AI/ML positioning Cases 3b, the gNB sends at least the following in a measurement report to LMF:

- Channel measurement;

- Quality of the channel measurement;

  - FFS: details on quality

- Time stamp of the channel measurement.

[check spec about optional/mandatory]

  - As an example, if the channel measurement is a list of paired timing information and power information, quality refers to the whole list, not individual elements within the list;

|  | Company |
| --- | --- |
| Support | InterDigital, DOCOMO |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | If this is for inference, is there still a need to report quality indicator? |
| HW/HiSi | If the LMF sets a threshold for the quality, then each and every measurement does not need to include the quality indication. For the time stamp, is there an agreement needed? The LMF can simply request a time stamp for a measurement, which already is supported in legacy. |
| Qualcomm | Content of legacy measurement reporting (e.g., measurements of UL-TdoA, UL part of mRTT measurements) shall be the starting point. |
| Nokia | The impact on including a quality indicator on the channel measurement for inference purposes may include spec. Impact. So, it is expected to relay on the legacy and potential additions may be studied before do any agreement. |

Proposal 5.2.2-2

For model inference of AI/ML positioning Cases 3a, the gNB sends at least the following in a measurement report to LMF:

- Intermediate positioning measurement as generated by the model at gNB;

- Quality of the intermediate positioning measurement;

- Time stamp of the intermediate positioning measurement.

|  | Company |
| --- | --- |
| Support | InterDigital, DOCOMO |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | If this is for inference, is there still a need to report quality indicator? |
| HW/HiSi | For LOS the quality can be indicated as in legacy already with soft indicator, no need for an agreement. For timing info, the legacy timing quality can be requested and reported, if needed. No need for an agreement. |
| Qualcomm | Similar to previous proposal. Existing measurement report from gNB to LMF shall be the starting point. |
| Nokia | Same comment than Proposal 5.2.2-1. |

### 2nd round discussion

Regarding the two proposals above, we ran into the issue whether the quality and time stamp are mandatory or optional. According to 38.455, quality is optional, time stamp is mandatory. We can follow this precedent. The proposals are updated accordingly.

38.455:

9.2.37		TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |

Proposal 5.2.3-1

For model inference of AI/ML positioning Cases 3b, at least the following are supported in a measurement report from gNB to LMF:

- (Mandatory field) Channel measurement;

- (Optional field) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory field) Time stamp of the channel measurement.

|  | Company |
| --- | --- |
| Support | DOCOMO |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| ZTE | This version is generally fine for us. For the FFS, I guess there are two aspects to be determined: - The number of quality indicator, i.e., whether both timing and power quality indicator should be included. - The association between quality and measurement, i.e., per measurement and per measurement element. |
| New H3C | OK |
| Nokia | As it is mostly a RAN3 and/or SA2 scope, we prefer to do a conditional agreement. So, we propose the following rewording: Proposal 5.2.3-1 (Update) From RAN1 perspective, for model inference of AI/ML positioning Cases 3b, at least the following are supported in a measurement report from gNB to LMF: - (Mandatory field) Channel measurement; - (Optional field) Quality of the channel measurement; - FFS: details of the quality - (Mandatory field) Time stamp of the channel measurement. Note: The final decision is up to RAN3 |
| Qualcomm | The controversial point was that existing reporting supports quality for timing measurements. The proposal needs to be clear that the quality is meant for timing measurements: - (Optional field) Quality of the channel timing measurement; - FFS: details of the quality |
| Fujitsu | OK in general. |
| PCL | Share the same view with ZTE. Sending a quality indicator along with all measurements is overhead consuming, therefore, the quality indicator in forms of a criterion could be signaled to the gNB and can then be applied in the generation process of the measurements, this can effectively reduce the overhead. |

Proposal 5.2.3-2

For model inference of AI/ML positioning Cases 3a, at least the following are supported in a measurement report from gNB to LMF:

- (Mandatory field) Intermediate positioning measurement as generated by the model at gNB;

- (Optional field) Quality of the intermediate positioning measurement;

  - FFS: details of the quality

- (Mandatory field) Time stamp of the intermediate positioning measurement.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| Nokia | Request clarification: what could be the difference of Quality of the intermediate feature and Monitoring (label-free and label-based) in gNB side for Case 3a? . Our view is that in this specific reporting, we may reuse the legacy and nothing else. |
| Qualcomm | Same to previous proposal. Second bullet needs to be specific for timing measurements. There is no quality indicator for LOS info in existing specifications. |

### 3rd round discussion

Now that Proposal 5.2.3-1 is agreed, Proposal 5.2.3-2 is updated to match the agreed Proposal 5.2.3-1. Regarding QC comments, yes, only “Timing Measurement Quality” in existing “Measurement Quality” in 38.455 is relevant. That is, there is no quality indicator for LOS/NLOS indicator. Thus this part is updated.

9.2.43	 Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  | Corresponds to information provided in NR-TimingQuality IE as defined in TS 37.355 [14] |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) |  |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |

Proposal 5.2.3-2

From RAN1 perspective, for model inference of AI/ML positioning Cases 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Intermediate positioning measurement as generated by the model at gNB;

- (Optional) Quality of the timing information when the intermediate positioning measurement includes the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp of the intermediate positioning measurement.

Proposal 5.2.3-2A

From RAN1 perspective, for model inference of AI/ML positioning Cases 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) LOS/NLOS indicator and/or timing information, which corresponds to the model output of gNB-side model;

- (Optional) Quality of the timing information when the timing information is reported;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

|  | Company |
| --- | --- |
| Support | NEC, TCL |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | If the intermediate features is angle information, quality indicator could also be reused as legacy spec already support this feature. |
| CATT, CICTCI | fine assuming this is time information prediction. |
| Qualcomm | Fine with the proposal. Let’s align the subbullet on quality with agreement of Case3b. |
| HW/HiSi | For model output in Case 3a, we have agreed to have timing information or LOS status as model output. And how to report the model output has been agreed in RAN1#116: Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. The proposed terminology „Intermediate positioning measurement as generated by the model at gNB“ is new and unclear. It is not really a measurement, at least not when model output is timing information. In fact, it is an un-measured UL RTOA, that is based on propagation delay needed between the locations of PRU/UE and TRP. We should stick to the agreed model output terminology. To avoid confusion, we suggest to replace „measurement“ with „result“. Regarding the optional quality field: Can it be clarified from where the quality is coming? Is it generated by the model itself? Proposal 5.2.3-2 From RAN1 perspective, for model inference of AI/ML positioning Cases 3a, at least the following are mandatorily or optionally supported when LOS/NLOS indicator and/or timing information is in a measurement reported from gNB to LMF: - (Mandatory) Intermediate positioning result measurement as generated by the model at gNB; - FFS: (Optional) Quality of the timing information when the intermediate positioning measurement includes the timing information; - Existing IE “Timing Measurement Quality” can be reused. (Mandatory) Time stamp of the intermediate positioning measurement.. |
| OPPO | Generally fine with the proposal. For case 3a, intermediate positioning measurement can be LOS/NLOS indicator and/or timing information according to the agreement in RAN1#116 meeting, better to make it clear in the proposal. |
| Nokia | As commented yesterday to FL, introducing Quality indicator to model output of AIML model deployed in the gNB for Case 3a is introducing a parallel analysis of the performance monitoring of the model. With this indication, probably monitoring may not be useful. In addition, further clarification is required in the quality of the timing information, specially considering that it will be generated by AIML model inference. Is the intention to generate the quality by the same AIML model doing timing inference?. Is the quality indicator related only to timing information?. there are many open questions that requires to be clarified before to go ahead with this proposal. |

### 4th round discussion

Thursday offline:

Proposal 5.2.5-1

From RAN1 perspective, for model inference of AI/ML positioning, when timing information is reported for Rel-19 Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| ZTE | In previous agreement (in RAN1#116), at least LOS/NLOS and/or timing information are supported. But others are undetermined yet. If other reports are supported, for example RSRP/angle, the corresponding measurement and quality could be consider. Therefore, the FFS should be updated: FFS: LOS/NLOS indicator and other report types (if supported) |
| Nokia | As suggested in the offline session, as a compromise, we agree on this proposal if (mandatory) and (optional) are removed from all bullets. So, we propose the following rewording: Proposal 5.2.5-1 (Updated) From RAN1 perspective, for model inference of AI/ML positioning, when only timing information is reported for Rel-19 Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator Note: The final decision of mandatorily or optionally of each report is up to RAN3. |

# Model monitoring

This section discusses issues for the LCM stage of model performance monitoring.

## General principles and methods

This section discusses generic methods and principles for model monitoring, which are applicable to all cases.

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Intel Corporation (R1-2408290) Proposal 17: Support both input-based and output-based functionality/model monitoring for AI/ML based positioning.

- Xiaomi (R1-2407951) Proposal 34: Confirm the necessity of assessment/monitoring of inactive models / functionalities, with the following assumptions as the starting point: - One way to monitor inactive models/functionalities is by activating them and reusing mechanisms defined for monitoring of active models/functionalities. - The following aspects may be considered for further study or in WI to assess the applicability and expected performance of an inactive model/functionality: - Configuring an AI/ML model for monitoring without activation (e.g., monitoring-only mode without reporting predicted beams in BM Case 1 and 2) - Dataset delivery from the network to the UE for assessment/monitoring of the applicability and expected performance of the model/functionality. - NW may provide performance criteria/preference for UE’s model selection. - Other aspects are not precluded for further study or specification.

- Samsung (R1-2408631) Proposal 19. RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics. Proposal 21. RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model. Proposal 22. RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content. Proposal 23. RAN1 supports the signalling for the monitoring refence signal configuration, monitoring measurement and its report. Proposal 24. Monitoring outcome determination could be based on one time metric or multiple time metrics. Proposal 25. Post monitoring behaviour should be specified.

- Apple (R1-2408453) Proposal 19: For model monitoring, the following elements shall be specified as part of the monitoring procedure: - Part A: - Measurement (corresponding to model input), - Time stamp, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model monitoring), - Part B - Ground truth label, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model training), - Time Stamp - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.). Proposal 21: To specify transfer of measurement data for both PRS and UL SRS for positioning, the following parameters may need to be configured: - The measurement type: delay Profile (timing only), Power Delay Profile (timing, power), Channel Impulse Response (timing, power and phase) - The number of measurements (taps): 8, 9, 16, 32, 64, [128], [256] (+1) - Feedback Format - First path parameters: first path magnitude, first path phase first path timing, timing granularity factor. - Additional path parameters - Relative time difference for additional paths - Time reporting granularity factor - Time stamp - Part A quality - Part B quality - Additional path magnitude and phase - Option 1: additional path magnitude, additional path phase - Option 2: additional path relative magnitude, additional path phase Proposal 28: the default option is for the monitoring to occur at the entity with the AI/ML model.

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 14: The possible actions/decisions of the functionality/model management entity can be: - More enhanced long-term monitoring for a specific model/functionality: - Configuration of event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports. - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels - Functionality or model (de-)activation/switching or fallback (temporary) - Functionality or model permanent marked as non-applicable in an area, with possible triggering for new data collection - AI/ML feature permanently deactivated in an area Proposal 15: Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas. Proposal 16: In both functionality-based and model-based LCM for Case 1 and Case 3a, UE/gNB provides an expected performance vs cost/overhead information to the LMF that is associated with a functionality or model. The cost/overhead information includes: - information on (logical or physical) model properties, such as size, computational complexity or power consumption - availability of the physical model on the device - latency for activating functionality or model. Proposal 17: Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window. Proposal 18: Support a monitoring report including the outcome (performance, robustness, overhead/cost, etc.) of functionality or model management decisions when required.

- NVIDIA (R1-2408391) Proposal 6: For AI/ML based positioning, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report.

- ETRI (R1-2408559) Proposal 14: For label-free performance monitoring in direct AI/ML models, consider using legacy measurements and assisted positioning model outputs, in addition to leveraging model input/output statistics.

- CMCC (R1-2407893) Proposal 12: For AI/ML based positioning, the relationship between model monitoring and positioning integrity can be considered.

- TCL (R1-2408267) Proposal 2: It is recommended to perform the monitoring metric calculation at the model inference entity for label-free model monitoring. Proposal 3: The monitoring metrics for UE-side model include at least error of location, error of speed or acceleration, error of measurement value, F1-score of LOS/NLOS indicator, standard deviation or variance of location and standard deviation or variance of measurement value. Proposal 4: The monitoring metrics for gNB-side model include at least error of measurement value, F1-score of LOS/NLOS indicator and standard deviation or variance of measurement value.

- OPPO (R1-2408159) Proposal 17: For functionality/model performance monitoring, Rel-19 is NOT to specify dedicated specification enhancement for the mechanism without ground-truth labels - Functionality/model performance monitoring without ground-truth labels can be done by implementation without any specification enhancement - Note: we also categorize some mechanisms based on “the approximate ground-truth label” in to that without ground-truth labels.

- Tejas Network Limited (R1-2407747) Proposal-12: For supporting UE side MPM metric calculation, the assistance data should be provided by the LMF to the UE or TRP for Case-1 and Case-3a respectively. Proposal-13: The range error or the positioning error should be considered as model performance metric for decision-making. Proposal-14: Both the MPM metric calculation and the MPM decision should be taken at one entity which can be either LMF or UE/TRP for Case-1 and Case-3a respectively. Proposal-15: The performance of the AI-ML models should be monitored periodically or aperiodically.

- vivo (R1-2407849) Proposal 28: Support both label-based and label-free monitoring for AI/ML based positioning.

- Nokia (R1-2408545) Proposal 2: RAN1 to standardize both monitoring approaches, label-free based monitoring and label-based monitoring for first priority cases. Proposal 3: For Case 1 and Case 3a, label-free monitoring may consist of: i) confidence level associated with model inference output, e.g., UE location estimate; ii) statistics of measurement(s) compared to the statistics of measurements in the training data set; iii) standard deviation as monitoring metric based on the historical/buffered/successive inference output. Proposal 4: For Case 1, necessary data for label-free monitoring consists of i) statistics of the measurements corresponding to inference input, i.e., RSRPP measurements; ii) historical/buffered inference output (UE location); and iii) characteristics (e.g., statistics) of training data. Proposal 5: Performance metric based on ground truth label (or its approximation) is defined as the statistical difference (e.g., MSE) between ground truth label and inference output, where measurements associated with ground truth information can be used as inference input. Proposal 6: Necessary data for label-based performance monitoring (or its approximation) consists of at least: i) ground truth (or its approximation), e.g., collected from PRU; ii) measurements corresponding to inference input, e.g., measurements collected from PRU; and iii) inference output, e.g., UE location estimation.

- Ericsson (R1-2407649) Proposal 50 For AI/ML based positioning, support self-monitoring by model inference entity as a baseline. Furthermore, self-contained monitoring is preferred, where no estimated label needs to be signaled from external entities. Only when necessary, introduce signaling to provide information on ground truth label to the model inference entity. Proposal 51 The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation. Proposal 52 For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. Proposal 53 For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

### 1st round discussion

Based on companies’ input (e.g., vivo, Nokia, Intel, Samsung, ETRI, OPPO, Ericsson), both label-free and label-based monitoring methods are to be supported. On the other hand, companies have different views on whether label-free monitoring has specification impact, or it can be up to the implementation of the model inference entity. At least one specification impact is identified, i.e., the model inference entity should send the monitoring decision to LMF, to support functionality-level management by LMF.

Proposal 6.1.2-1

For AI/ML positioning Cases, label-free monitoring methods are supported, where the model inference entity performs self-monitoring without external information on the ground truth label.

- For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF.

- FFS: other specification impact (e.g., signalling of statistics of model input, statistics of model output).

|  | Company |
| --- | --- |
| Support | DOCOMO, Nokia |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | We are not sure whether label-free monitoring methods is completely implementation-based. Instead of agreeing to support label-free monitoring methods, our proposal is to study the spec impact of label-free monitoring methods. |
| CATT, CICTCI | OK. But for FFS, we think it is hard to converge, since how to determine metric is up to UE/TRP. |
| HW/hiSi | Label-free monitoring should be up to implementation. As stated in TR 38.843, the AI/ML functionality-based or model-based LCM is only applicable to UE-side models and/or UE-part of two-sided models. As such, functionality/model-based LCM is not applicable to gNB-side model of Case 3a. In addition, for Case 3a, the gNB can indicate a fallback as monitoring decision by reporting a measured value instead of a non-measured value, e.g., of the timing information based on the model output. The gNB indicates the non-measured value as a continuing activation of the model. |
| Apple | UE should have flexibility to perfrom label free monitoring and switch models i.e. May send the monitoring information tot he LMF. |
| NEC | Support this proposal in general. However, if there is no guidance from LMF, different UE/gNB will do different decision for the same situation. A reasonable way is UE/gNB send the recommended monitoring decision to LMF, and LMF makes the final decision. |
| Fujitsu | We think it would be the better to keep the option of LMF making the decison on the table by now since LCM issues are not well discussed yet. |

Proposal 6.1.2-2

For AI/ML based positioning all cases, LMF is responsible for functionality-level performance monitoring and management decision making (e.g., functionality selection, activation, deactivation, fallback to a legacy method).

|  | Company |
| --- | --- |
| Support | InterDigital, NEC, DOCOMO, Nokia |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | We agree on the last part ‘management decision making (e.g., functionality selection, activation, deactivation, fallback to a legacy method)’. It is unclear what is functionality-level performance monitoring. |
| ZTE | This could be discussed case by case. This seems contradict with proposal 6.1.2-1, previous proposal let UE/gNB send decision to LMF, but this proposal let LMF make decision. |
| HW/HiSi | Not for all cases, we should discuss this case-by-case. |
| Apple | Agree on the need for case-by-case support |
| Qualcomm | We need first to establish understanding on functionality. |

## Case 1: Model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 19: Label-based model monitoring for Case 1 should be based on PRU channel measurements and their known locations.

- Google (R1-2407989) Proposal 4: The model monitoring should not require the UE to report the model inference result to the NW. - Option A is preferred compared to Option B - The model performance monitoring should be transparent

- CATT, CICTCI (R1-2408028) Proposal 21: For case 1 and case 2a, for model performance monitoring metric calculation in label-based model monitoring, at least the following options for generating ground truth label can be considered. - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option B. The LMF performs monitoring metric calculation. - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

- Fujitsu (R1-2408104) Proposal-8: Regarding UE-side monitoring metrics calculation of Case 1: - Option A-1/A-2 is supportive, if the availably of the high-quality ground truth label for performance monitoring at UE-side can be confirmed. - Option A-3 is supportive, if the format of PRU measurement input can be the same as the format of UE’s model input. Proposal-9: Regarding LMF-side monitoring metrics calculation of Case 1: - Option B-1 is supportive, if the availably of the high-quality ground truth label for performance monitoring at LMF-side can be confirmed. - Option B-2 is supportive, if the format of PRU measurement input can be the same as the format of UE’s model input.

- NEC (R1-2408214) Proposal 25: Support the main bullet of both Option A and Option B for case 1, i.e., both LMF and UE can perform monitoring metric calculation. Proposal 26: Prioritize Option A-3 if performs monitoring metric calculation at the target UE side. Proposal 27: Prioritize Option B-2 if performs monitoring metric calculation at the target LMF side.

• Baicells (R1-2408309) Proposal 11. Support Option B-1 for Case 1, which can be achieved with minimum specification effort.

• Sony (R1-2408402) Proposal 11: For case 1, support Option A and the LMF sends criteria (e.g., parameters for performance evaluation, thresholds) to the UE performing performance metrics (PM) computation. Proposal 13: For case 1 and case 3a with option A, UE or gNB to provide an indication of AI/ML model validity to the AI/ML server/management (e.g., LMF).

- Intel Corporation (R1-2408290) Proposal 18: For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Options A-1 and A-2 should be considered further. - For Option A-1, ground-truth labels (or their approximations) are determined by the LMF based on legacy measurement metric(s) for RAT-dependent and/or RAT-independent positioning methods to LMF by target UE/NG-RAN node. - For Option A-2, ground-truth labels (or their approximations) are determined by the target UE based on legacy assistance information received from the LMF. - Option A-3 may be considered as a supplementary method in addition to one or both of Options A-1 and A-2. Proposal 19: For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Option B-1 should be considered further. - Estimated coordinates from AI/ML model at UE side are reported to the LMF by the target UE. - Ground-truth labels (or their approximations) are determined by the LMF based on known coordinates of PRUs or estimated coordinates for non-PRU UEs.

- Qualcomm Incorporated (R1-2408838) Proposal 22: In AI/ML positioning Case1, for monitoring, support the following assistance options (in RAN1#116bis agreement) for label-based monitoring metric calculation: - Option A-1 - Option A-2 - Option B-1 - FFS Option A-3 and A-4 (while considering specifications-based PRU data transfer, e.g., using SL)

- Xiaomi (R1-2407951) Proposal 29: For performance monitoring in Case 1 - The performance metric calculation can be performed on UE side - The LCM decision making can be performed by UE side Proposal 30: Further study the following two options for the case that UE make the LCM decision in Case 1 - Option 1: UE make the LCM decision based on UE’s implementation and then report the LCM decision to LMF - Option 2: LMF provide a performance monitoring metric threshold, UE could make the LCM decision by comparing the calculated performance metric and the configured threshold. - Standardization effort is needed for the performance metric definition

- NTT DOCOMO, INC. (R1-2408774) Proposal 17: For performance monitoring metric calculation of AI/ML based positioning of case 1, both option A and B are considered. Option B is a baseline if option 2 for decision making of case 1 is a baseline according to RAN2 agreement. Proposal 18: For case 1, when option A is used, UE performs the monitoring metric calculation following NW indication. - The indication includes at least model ID/functionality information, performance metrics/threshold.

- Huawei, HiSilicon (R1-2407654) Proposal 20: For Case 1, there is no need to involve the LMF for metric calculation, as the LMF may not have the knowledge to monitor/manage the UE-side models. Proposal 21: For model monitoring in Case 1, conclude that Option A-4 is sufficient where the monitoring decision is performed in the same UE-side entity that derives the monitoring metric. - The monitoring is done in implementation manner and a monitoring metric does not need to be specified. - There is no need to restrict the type of measurements used as model input.

- CATT, CICTCI (R1-2408028) Proposal 23: For case 1 and case 2a, study the feasibility of using a higher-quality model deployed at LMF side to assist performance monitoring for the UE side model. Proposal 24: For model performance monitoring of AI/ML positioning Case 1, the following options with regard to how to generate information on ground truth label can be considered. - Option A. The target UE side performs monitoring metric calculation. - LMF sends the ground truth label (UE’s location) derived by AI/ML based method or non-AI/ML based method and UE performs monitoring metric calculation. - Option B. The LMF performs monitoring metric calculation. - UE sends the inference result (UE’s location) to LMF, and LMF performs monitoring metric calculation based on both UE’s inference results and its own information of UE’s location derived by LMF-side AI/ML model inference outcome.

• Baicells (R1-2408309) Proposal 12. Support gNB-side / LMF-side monitoring.

- Qualcomm Incorporated (R1-2408838) Proposal 23: In AI/ML positioning Case1, for monitoring, support UE-initiated monitoring with the following assistance from LMF to UE: - RS resources configurations/activations related to monitoring - Monitoring assistance information (as discussed in Option A-1/-2 monitoring metric calculation in RAN1#116bis agreement) - ground truth label - position calculation assistance data.

- Samsung (R1-2408631) Proposal 20. RAN1 considers to define the measurement conditions (e.g., SNR level, LoS indication etc) to facilitate the performance (model) monitoring.

- Apple (R1-2408453) Proposal 30: RAN1 should support at least Option A-2, A-3 and B-1. Proposal 31: To ensure the accuracy of the GT - Step 1: use a LOS/NLOS identifier to identify >= 3 LOS links. Note that LOS probability may be used to derive the measurement quality. - Step 2: use legacy positioning method to estimate GT position - Step 3 (for case 2a): estimate GT timing for each of the LOS links based on estimated GT position

- NTT DOCOMO, INC. (R1-2408774) Proposal 15: For performance monitoring of AI/ML based positioning of case 1, at least LMF is supported for decision making of performance monitoring.

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 7: At least for Case 1 and Case 3a, functionality management is provided by the network (LMF). Proposal 8: At least for Case 1 and Case 3a, functionality monitoring is triggered by the NW (LMF). Proposal 9: For Case 1, UE-sided model monitoring can be triggered by the UE or the NW (LMF). Proposal 11: For model performance monitoring of AI/ML positioning, ensure that Rel-19 supports the necessary signaling mechanisms for Options A-1, A-2, A-3, B-1, and B-2, including signaling for ground truth labels, monitoring events, time intervals, and assistance data. Proposal 13: For both Case 1 and Case 3a consider a two-stage monitoring process: upon identifying of a potential model performance issue through early metrics, activate a broader set of monitoring metrics to confirm the actual presence and impact of the issue.

- Lenovo (R1-2408429) Proposal 23: For Option A – target UE model monitoring for AI/ML positioning Case 1, support Options A-1, A-2 and A-3 in terms of receiving ground truth label information. Proposal 24: For Option B – LMF model monitoring for AI/ML positioning Case 1, further study whether Option B meets the near-real time latency requirements for performance monitoring.

- ETRI (R1-2408559) Proposal 10: For label-based model performance monitoring of AI/ML positioning Case 1, Option A is preferable to Option B. Proposal 11: In Option A for label-based model performance monitoring in AI/ML positioning Case 1: • Option A-1 faces issues with the reliability of ground truth labels, especially in NLoS environments where traditional methods fail. • Option A-2 raises concerns about the reliability of assistance data, which is not useful in environments where traditional positioning methods are ineffective. • Option A-3 is the most feasible and reliable solution for monitoring. • Option A-4 has no impact on the specifications.

- Sharp (R1-2408609) Proposal 14: For performance monitoring in AI/ML based positioning Case 1, support Option A-3 in case that PRU is involving in the performance monitoring. Proposal 15: For performance monitoring in AI/ML based positioning Case 1, support Option A-2 in case that PRU is not involving in the performance monitoring.

- CMCC (R1-2407893) Observation 7: For AI/ML based positioning case 1, the target UE side can perform monitoring metric calculation. Observation 8: For option A-3, PRU measurement for model monitoring seems workable, while detail solution needs further discussion. Observation 9: If inference result is currently supported measurement report, then LMF can perform monitoring metric calculation.

- TCL (R1-2408267) Proposal 1: For model performance monitoring metric calculation in label-based model monitoring for AI/ML positioning Case 1, Option A-2 is preferred.

- MediaTek Korea Inc. (R1-2408903) Proposal 4-1: For case 1 performance monitoring, the metric calculation is conducted by UE Proposal 4-2: For case 1 performance monitoring, Option B is not considered. Items within Option A can all be considered

- OPPO (R1-2408159) Proposal 19: For UE-based positioning with UE-side model (Case 1), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring - Functionality/model performance monitoring can be done by UE implementation without impact on air interface - The feasibility of Option A-1, A-3, A-4, B-2 has not been justified - For Option A-2, it is not clear what the solution it is. - For Option B-1, there is no additional spec impact and it is not clear how LMF use this information for monitoring metric calculation

- ZTE Corporation, Pengcheng Laboratory (R1-2407797) Proposal 28: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support option B: - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

- vivo (R1-2407849) Proposal 23: For model performance monitoring of AI/ML positioning Case 1, the target UE side performs monitoring metric calculation should be supported. Proposal 24: For model performance monitoring of AI/ML positioning Case 1, option A-1 and option A-3 can be supported for the target UE side performs monitoring metric calculation. Proposal 25: More PRU information can be provided to UE as assistance information to ensure the validity of PRU information for the target UE side performs monitoring metric calculation. For example: - The cell ID where PRU belongs to - Channel condition (e.g LoS condition) of PRU measurement

- Nokia (R1-2408545) Proposal 8: For label-based monitoring of Case 1, for UE to derive monitoring metric, support Option A-3: LMF provides the UE necessary data for monitoring over LPP which contains measurements (e.g., measurements (e.g., RSRPP) collected from PRU(s)) and associated ground truth (e.g., PRU location) including quality indicators. In addition, target UE computes the monitoring metric internally and provides it to the LMF if requested to do so. Proposal 9: For label-based monitoring of Case 1, for LMF to derive monitoring metric, support Option B-2: PRU(s) provides measurements (e.g., RSRPP) to LMF. LMF provides measurements collected from PRU(s) to the UE. UE reports inference output, based on measurements collected from PRU(s), to LMF. LMF compares the UE inference output with PRU ground truth labels associated with these measurements to calculate monitoring metric. Proposal 10: For label-free monitoring of UE-side models, for UE to derive monitoring metric, LMF may provide UE necessary data and configuration for monitoring. Proposal 11: For label-free monitoring of UE-side models, for LMF to derive monitoring metric, UE may provide LMF necessary data for monitoring, which may contain: (i) statistics of UE measurements (e.g., RSRPP), (ii) standard deviation of UE inference output (e.g., UE location estimation in Case 1).

- Ericsson (R1-2407649) Proposal 54 For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1). Proposal 55 For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics. Proposal 56 For Case 1/3a, the target UE (Case 1) and gNB (Case 3a) makes the model management decision and recommendation as a result of model performance monitoring. The target UE and gNB sends its model management decision and recommendation to LMF, e.g., failure cause values as in the positioning framework. Proposal 57 For Case 1/3a, LMF make functionality-level decisions, including whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

- InterDigital, Inc. (R1-2408522) Proposal 17: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support Option A-1, A-2 and A-3. Proposal 18: Prioritize specification of variants of Option A over specification of Option B.

### 1st round discussion

At RAN1 #116bis and RAN1#118 meeting, the following was agreed or concluded for label-based monitoring for Case 1:

Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement (RAN1#116bis) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the submitted contributions to RAN1#118bis, Companies’ preference for the Options is as follow (please update/correct if your view is not captured correctly):

|  | Supported by: | Not supported by: |
| --- | --- | --- |
| Option A in general | Sony, Xiaomi, Google, Docomo, ETRI, MediaTek 6 sources | OPPO 1 source |
| Option A-1 | Qualcomm, Ericsson, vivo, Fraunhofer IIS/Fraunhofer HHI; Lenovo; CATT/CICTCI; 8 sources | OPPO 1 source |
| Option A-2 | Qualcomm, Ericsson, Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, CATT/CICTCI, TCL 10 sources | OPPO (not clear what A-2 is) 1 source |
| Option A-3 | Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, vivo, Nokia, NEC 8 sources | OPPO, Ericsson 2 sources |
| Option B in general | ZTE/Pengcheng Laboratory 2 sources | MediaTek, Ericsson, OPPO 3 sources |
| Option B-1 | Qualcomm, Apple, Fraunhofer IIS/Fraunhofer HHI, CATT/CICTCI, Docomo 7 sources | MediaTek, Ericsson, OPPO 3 sources |
| Option B-2 | Fraunhofer IIS/Fraunhofer HHI, Nokia, NEC 4 sources | MediaTek, Ericsson, OPPO 3 sources |

For reference, RAN1#118 status (see R1-2407272) was shown below.

|  | Supported by: | Not supported by: |
| --- | --- | --- |
| Option A in general | Sony, Xiaomi, Google, Lenovo, DOCOMO(?) 5 sources |  |
| Option A-1 | QC, InterDigital, CATT/CICTCI, Intel, vivo, Ericsson, Nokia 8 sources | OPPO 1 source |
| Option A-2 | QC, InterDigital, CATT/CICTCI, Intel, Ericsson, Apple, Nokia 8 sources | OPPO (not clear) 1 source |
| Option A-3 | InterDigital, Sharp, vivo, NEC, Apple, Nokia 6 sources | OPPO, Ericsson 2 sources |
| Option A-4 | Huawei/HiSilicon, Sharp 3 sources | InterDigital, OPPO, Fujitsu (deprioritize), Nokia (defer), Ericsson 5 sources |
| Option B in general |  |  |
| Option B-1 | QC, CATT/CICTCI, Intel, Apple, ZTE, Nokia 7 sources | OPPO 1 source |
| Option B-2 | Sharp, ZTE, Nokia, Sony, NEC, Fujitsu 6 sources | OPPO, Ericsson 2 sources |

Based on the summary above, Option A gained more support than Option B, and Option A has less objections than Option B. It’s reasonable that Option A is supported in Rel-19. Option A-1, A-2, A-3 can be further discussed, e.g., to clarify and address OPPO’s concern.

Proposal 6.2.2-1

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A is supported in Rel-19:

- Option A. The target UE side performs monitoring metric calculation.

- FFS: Option A-1, A-2, A-3.

Proposal 6.2.2-1A

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A is supported in Rel-19:

- Option A. The target UE side performs monitoring metric calculation.

- FFS: Option A-1, A-2, A-3.

Note: The definition of the monitoring metric is up to implementation.

Proposal 6.2.2-1B

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-1, A-2 are supported in Rel-19:

Note: The definition of the monitoring metric is up to implementation.

Proposal 6.2.2-1C

Deprioritize B-2

|  | Company |
| --- | --- |
| Support | InterDigital |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | As A-1 requires the UE to report measurements to the LMF, whether sample-based and/or path-based measurement is supported should be discussed for Case 1 as well. |
| CATT, CICTCI | Add back our support on A-1, A-2 and B-1 in the table. We think Option B-1 is needed since NW usually has better understanding on the positioning performance. |
| ZTE | We support option B. |
| HW/HiSi | Fine in principle. We think one important aspect is that this proposal implies that the metric does not need to be specified, which would be good to capture in a note. Proposal 6.2.2-1 For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A is supported in Rel-19: - Option A. The target UE side performs monitoring metric calculation. - FFS: Option A-1, A-2, A-3. Note: The definition of the metric is up to implementation |
| NEC | Add our preferences in above table. Option A and Option B all are acceptable. |
| DOCOMO | Not only option A but also option B can be considered. |
| TCL | Agree with the proposal and we support Option A-2. |
| Fujitsu | We think it is too early to make the conclusion. |
| Nokia | As indicated in our contribution, discussion of this topic is very important to further discuss details of sample-based for Case 1 as indicated by InterDigital. Because Option B has less support is not a strong argument to avoid to discuss it. So, we suggest the following rewording: Proposal 6.2.2-1(Updated) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A and Option B is supported in Rel-19: - Option A. The target UE side performs monitoring metric calculation. - FFS: Option A-1, A-2, A-3, B-1, B-2. Note: The definition of the metric is up to implementation |

### 2nd round discussion

Based on TCL comment, I added TCL support to Option A-2. CATT and NEC have updated their views in the table.

@All: Please check and update the summary table in section 6.2.1, if your view is missing or not captured correctly!

Regarding the options, the two possible ways to make progress are to support the options with most support & least objections, or to deprioritize the option with least support & most objections. The two proposals discussed in Monday offline are presented below for discussion.

If none of the options are agreeable, then we can spend more time refining the proposals on self-monitoring and label-free methods.

Proposal 6.2.3-1

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19.

Note: The definition of the monitoring metric is up to implementation.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| ZTE | This can be discussed together with model monitoring decision. Who will calculate monitoring metric and who will make monitoring decision. For case 1, if monitoring metric is calculated by UE, but the monitoring decision is made by LMF, then UE have to report the metric or monitoring suggestion to LMF. If we support option A-1,A-2, extra signalling is required for monitoring considering proposal 7.2.3-1. |
| InterDigital | We do not understand why A-3 was removed. If the method is based on existing specification, there should not be an issue to support A-3. We have a suggestion for wording below. Modified Proposal 6.2.3-1 For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 and Option A-3 are supported in Rel-19. In addition, in the FL’s table, companies’ preference for Option A-4 are still listed. If we don’t further discuss Option A-4, why is our view captured in the table? Please also remove our name from the Option A-4 row as we stopped discussion related to Option A-4 in our contribution because of the following conclusion we made in RAN1#118. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. [Moderator] Option A-4 is removed from the table for this meeting. The table for previous meeting is copied for reference, which had Option A-4. |
| OPPO | We have some questions for these two options: For option A-1, if NW can get reliable labels, why does NW to trigger the AI-based positioning procedure. If NW cannot get reliable labels, then UE can also get an estimation of the location by legacy method without any spec impact (it can be done by UE implementation). In this case, why do we need to waste the signaling from UE/gNB to LMF and from LMF to UE? For option A-2, The normative work should focus on the spec impact and what enhancement is needed. For this option, existing information for UE-based positioning method has been supported. What’s the additional spec impact? |
| ETRI | - Option A-1 has an issue with the reliability of ground truth labels, especially in NLoS environments where non-AI/ML positioning methods fail. So does Option A-2. Are there any evalutation results showing that unreliable ground truth labels are helpful for label-based monitoring? - Option A-3 should be included. |
| HW/HiSi | Can it be clarified what specification impact there is expected for OptionA-1 and A-2? In our understanding both are re-using legacy, so in that case this proposal would not be needed and we can simply conclude: Proposal 6.2.3-1 Conclude that For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 can be are supported in Rel-19 by re-using the legacy signalling. Note: The definition of the monitoring metric is up to implementation. |
| NEC | Option A-3 should also be here. Option A-1 and Option A-2 preform the model monitoring based on the ground truth label calculated from LMF(based on UE’s measurement) or UE, which may be unrealiable sometimes(especially after the model is acitivated). Option A-3 can be a candidated way when Option A-1/A-2 is not feasiable. |
| Fujitsu | We think all the options cannot always work well. For Option A-1/A-2/B-1, the availably of the high-quality ground truth label for performance monitoring at UE-side need to be confirmed. For Option A-3/B-2, how to ensure the format of PRU measurement input can be the same as the format of UE’s model input needs to be clarified. Thus, the potential solution for sovling the issues for each option should be clariied first before drawing the conclusion and down-selection. |
| PCL | Share the same view with OPPO, We also have a question about the option A-1, whether we can obtain the reliable labels to ensure the reliability and accuracy of the model. Also agree with the proposal of ZTE, to discuss model performance monitoring metric calculation together with model monitoring decision |
| DOCOMO | Add our preference on B-1 in above table. Seems no reason to drop Option A-3 and B-1. |

Proposal 6.2.3-2

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option B-2 is deprioritized in Rel-19.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| NEC | Acceptable if majority companies also think it should be deprioritized. |
| Nokia | We believe that there is not ground to deprioritize Option B-2 when there are 6 companies supporting it. We expect a compromise between companies to go ahed on this topic. currently , the following companies are supporting this option: Sharp, ZTE, Nokia, Sony, NEC, Fujitsu 6 sources |

## Case 3a: Model monitoring

In RAN1#118, the following agreement was made regarding model monitoring for Case 3a.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 22: For performance monitoring of AI/ML positioning Case 3a, the request/report of a measured or non-measured result can be used to indicate the activation/fallback between LMF and gNB - For Option A. NG-RAN node performs monitoring metric calculation, gNB can report LMF with measured result (implying fallback) or non-measured result (implying activation). - For Option B. LMF performs monitoring metric calculation, LMF can request gNB with measured result (implying fallback) or non-measured result (implying activation). Proposal 23: For performance monitoring of AI/ML positioning Case 3a, no need to specify the type of metric, i.e., the monitoring entity can calculate the metric by implementation.

- Fujitsu (R1-2408104) Proposal-10: Regarding performance monitoring metric calculation in label-based monitoring of Case 3a, model LCM control/decision is suggested to be done by gNB/TRP for both Option A and Option B.

• Sony (R1-2408402) Proposal 12: For case 3a and with option B, support LMF to provide an indication of AI/ML model validity to NG-RAN node. Proposal 13: For case 1 and case 3a with option A, UE or gNB to provide an indication of AI/ML model validity to the AI/ML server/management (e.g., LMF).

- Xiaomi (R1-2407951) Proposal 32: - For input-based performance monitoring - gNB calculates the performance metric and makes the LCM decision for the assisted positioning with multi-TRP construction - gNB calculates the performance metric and LMF makes the LCM decision for assisted positioning with single TRP construction - Standardization effort is needed for the performance metric definition and report - For ground truth-based performance monitoring - LMF calculates the performance metric and makes the LCM decision

- Apple (R1-2408453) Proposal 35: For Case 3a Option A - NG-RAN node performs monitoring metric calculation for its own model in the LOS/NLOS use case. - The NG-RAN node is not allowed to know the location of the non-PRU UEs connected to it. This is to maximize privacy for the non-PRU UEs connected to the NG-RAN node - The NG-RAN node is not allowed to perform the monitoring metric calculation for the timing estimation use case.

- NTT DOCOMO, INC. (R1-2408774) Proposal 16: For performance monitoring of AI/ML based positioning of case 3a, the entity which performs performance metric calculation (i.e., LMF or gNB, corresponds to option 1/2) is applied for decision making. - If gNB makes decision, gNB shall notify the LMF of its model management decision/recommendation.

- Fraunhofer IIS, Fraunhofer HHI (R1-2408385) Proposal 10: For Case 3a, gNB-sided model monitoring can be triggered by the gNB or the NW (LMF).

- Ericsson (R1-2407649) Proposal 55 For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics. Proposal 56 For Case 1/3a, the target UE (Case 1) and gNB (Case 3a) makes the model management decision and recommendation as a result of model performance monitoring. The target UE and gNB sends its model management decision and recommendation to LMF, e.g., failure cause values as in the positioning framework. Proposal 57 For Case 1/3a, LMF make functionality-level decisions, including whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

- OPPO (R1-2408159) Proposal 22: For NG-RAN node assisted positioning with gNB-side model (Case 3a), in order to facilitate ground-truth-label-based functionality/model performance monitoring at gNB, support the enhancement on NRPPa signaling to enable the delivery of the ground-truth label or the information that can derive the ground-truth label from LMF to gNB • Functionality/model performance monitoring can be done by gNB implementation Proposal 23: For NG-RAN node assisted positioning with LMF-side model (Case 3b), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring at LMF • Functionality/model performance monitoring can be done by LMF implementation without impact on NRPPa signaling

## Other proposals for model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 24: For model monitoring in Case 3b, no further assistance information or measurement report in addition to inference is required to be sent to the LMF.

- CATT, CICTCI (R1-2408028) Proposal 22: For case 2b and case 3b, for model performance monitoring metric calculation in label-based model monitoring, LMF generates UE’s location as ground truth label and performs monitoring metric calculation.

- Fujitsu (R1-2408104) Proposal-11: Performance monitoring for case 3b can be realized by implementation manner and no need for further study.

• Baicells (R1-2408309) Proposal 13. No specification impact is expected for Case 3b model monitoring since monitoring can be deployed within the NW and based on implementation.

- InterDigital, Inc. (R1-2408522) Proposal 34: For Case 3b, specification enhancement is not needed for assistance information and/or measurement report sent from UE/PRU and/or gNB to LMF to assist LMF for the performance monitoring.

# Model identification and model functionality

## Model identification, model ID

### Companies’ view from contribution

- Xiaomi (R1-2407951) Proposal 17: Support model identification for Case 1 and Case 2a Proposal 18: Whether there is functionality identification / model identification-like procedure between gNB and LMF is up to RAN3

- NVIDIA (R1-2408391) Proposal 9: For AI/ML based positioning, introduce specification support for additional conditions to be included in model description information during model identification.

## Model functionality, UE capability reporting

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2407654) Proposal 25: Support functionality-based LCM for UE-side model of Case 1/2a.

- NEC (R1-2408214) Proposal 24: Support semi-supervised learning for AI/ML based positioning.

- Intel Corporation (R1-2408290) Proposal 20: • At least for UE-side models, model/functionality selection or switching could be realized via explicit indication from a network entity (e.g., LMF) or implicitly based on configurations related to data collection for training and/or inference, e.g., configurations related to data collection in time dimension or over a certain geographical area.

- Qualcomm Incorporated (R1-2408838) Proposal 24: In AI/ML positioning Case1, for LCM, RAN1 proceeds with understanding that LMF can request UE to activate or deactivate positioning session that involves AI/ML operation, and UE is responsible for AI/ML LCM decision and actions, LMF provides assistance data to help in UE decision (e.g., training and inference consistency information).

- InterDigital, Inc. (R1-2408522) Proposal 19: An AIML positioning functionality is defined by input type and output type of an AIML model(s) Proposal 20: Support dynamic UE capability signaling for Case 1 for AIML based positioning to indicate UE capability that can change dynamically (e.g., remaining battery power) to the LMF

- Xiaomi (R1-2407951) Proposal 17: Support model identification for Case 1 and Case 2a Proposal 19: Categorize the AI/ML feature from the following 3 aspects for AI-based positioning - Output type of the AI models - The location where AI models are deployed - The positioning RS type i.e., SRS-based or PRS-based Proposal 20: At least measurement related aspects e.g., PRS bandwidth, TRP configuration can be considered as conditions to categorize the AI/ML functionality Proposal 21: Support UE directly report the applicable functionality/model via e.g., an identifier. Proposal 22: Applicable functionality/model report can be supported in both the case that associated AI/ML feature is enabled and the case that associated AI/ML feature is not enabled Proposal 35: The fallback mode including the non-AI positioning approach and related configuration are preconfigured to UE Proposal 36: UE group-based LCM operation (e.g., model activation/deactivation/switch) can be considered

- NVIDIA (R1-2408391) Proposal 7: For AI/ML based positioning, introduce specification support for UE capability signalling including model training, model inference and model monitoring. Proposal 8: For AI/ML based positioning, introduce specification support for conditions of a Feature/FG available for functionality.

- OPPO (R1-2408159) Proposal 24: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a) - The functionality/model can be activated or disactivated by LPP signaling, which is the same as the legacy positioning procedure - UE can autonomously deactivate the AI operations and fall back to the legacy operations - UE reporting includes some field/IE to indicate whether the results are based on legacy operation or AI model output. Proposal 25: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), specify the UE capability signaling to report the supported configuration(s) associated with given functionality(ies) - E.g., DL-PRS resources capability, DL-PRS Processing capability, measurement capability Proposal 26: For UE-based positioning with UE-side model (Case 1) and UE-assisted/LMF based positioning with UE-side model (Case 2a), UE can report the applicable functionalities by sending a ProvideCapabilities message to the LMF (namely, triggering the capability indication procedure of TS 37.355) - FFS: whether some enhancement is needed to reduce the signaling overhead

- Nokia (R1-2408545) Proposal 38: For cases 1, inference configuration for AI/ML positioning functionality is indicated to UE via the PRS configuration provided in LPP Assistance Data and the measurement configuration provided in LPP Request Location Information. Proposal 39: For UE-sided models, no reconfiguration is expected on any functionality based on the preferred dynamic/online indications coming from the UE. Proposal 40: For all use cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

- Ericsson (R1-2407649) Proposal 53 For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

### 1st round discussion

RAN2#127 made the following agreement on functionalities for beam management.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

The above agreements are generic and applicable to at least positioning Case 1. Considering the above and several RAN1 companies’ input (e.g., Huawei/HiSilicon, OPPO, Nokia), the following proposal is recommended.

Proposal 7.2.2-1

Rel-19 support functionality-based LCM for AI/ML positioning Case 1.

- To prepare for model inference, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF. Similarly, LMF can send a RequestCapabilities message to inquire on the capability of the target device.

- The functionality can be activated or disactivated via LPP signaling.

  - The decision to activate or deactivate is made by LMF.

  - The UE may provide information to support LMF’s decision, e.g., information from model performance monitoring by UE.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| InterDigital | This is a RAN2 topic as we discuss applicability signaling. The second bullet is already being discussed in Proposal 6.1.2-2 (e.g., LMF manages functionalities). The second-sub-bullet under the second bullet should be discussed in model monitoring. |
| CATT, CICTCI | Can be discussed latter. |
| ZTE | The main bullet is fine. Functionality-based LCM can be supported. For the second bullet, disactivated => deactivated. Subbullet requires further discussion. This can be discussed after decide the performance monitoring entity. |
| Qualcomm | Let’s first discuss and establish understanding on functionalities in AI/ML positioning use case. |
| DOCOMO | Generally fine. Understanding on functionalities can be alingned among companies. |
| Nokia | Agree in general, we suggest the following rewording: Proposal 7.2.2-1(Updated) Rel-19 support functionality-based LCM for AI/ML positioning Case 1. - To prepare for model inference operation, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF based on the initial set of functionalities set by LMF in previous steps. Similarly, LMF can send a RequestCapabilities message to inquire on the capability of the target device. - The functionality can be activated or disactivated via LPP signaling. - The decision to activate or deactivate is made by LMF. - The UE may provide information to support LMF’s decision, e.g., information from model performance monitoring by UE. |

### 2nd round discussion

The proposal 7.2.2-1 is updated below to address companies’ comments.

Proposal 7.2.3-1

Rel-19 support functionality-based LCM for AI/ML positioning Case 1.

- To prepare for model inference, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF. Similarly, LMF can send a RequestCapabilities message to inquire on the capability of the target device.

- The functionality can be activated or deactivated via LPP signaling.

  - The decision to activate or deactivate is made by LMF.

  - The UE may provide information to support LMF’s decision, e.g., information from model performance monitoring by UE.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| ZTE | See comment in proposal 6.2.3-1. |
| InterDigital | We should wait for progress related to applicability determination procedure in RAN2 for this topic. |
| New H3C | Share the similar view with IDC |
| HW/HiSi | We think this is a RAN2 issue. At least we should wait for the progress in RAN2 as IDC commented above. |
| Nokia | RAN2 is also providing similar comments than IDC, but they are expecting further inputs from RAN1. So, we agree with the intention of FL. However, we suggest to avoid to mention the term Model in our agreements, especially if we are talking about the functionality LCM, it could be mixed with the model LCM framework. In addition, RAN2 is expecting a RAN1 discussion on the necessity of including new methods or update existing ones for 1st priority cases. |
| Qualcomm | It is too early to discuss these aspects unless functionalities are clarified in AI/ML positioning. In addition, the messaging seems a RAN2 scope? |
| DOCOMO | We can wait for RAN2 progress. |

### 3rd round discussion

Functionality-based LCM was studied extensively in RAN1 since Rel-18 SI phase. Thus it should be fine for RAN1 to discuss it for the positioning use case. Also, RAN1 has a large responsibility in UE capability discussion. Thus RAN1 should be allowed to proceed. The earlier proposal is shortened below to focus on the very basic.

Proposal 7.2.3-1

Rel-19 support functionality-based LCM for AI/ML positioning Case 1.

- To prepare for inference operation, the target device reports the applicable functionalities by sending a ProvideCapabilities message to the LMF. LMF can send a RequestCapabilities message to inquire on the capability of the target device.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| ZTE | The direction is fine for us. But the UE capability could be discussed after all the basic functions are stable. |
| CATT, CICTCI | In general, we think this proposal is straightforward. However, seeing the discussion in AI/ML beam management regarding the reply LS to RAN2 on applicable functionality & associated ID, we think it is safer to wait for a while. RAN1 AI/ML BM seems NOT in favor of using the term of functionality in Rel-19 WI. The main reason is that many companies in RAN1 think functionality is not defined in spec languige but just for discussion purpose. They prefer to use FG, component, configurations instead. It is strange if one use case do that but another use case do not. |
| InterDigital | We should wait for RAN2 discussion to progress. Even we start discussing the FL’s discussion, we first need to define what a functionality is in the context of AIML positioning. |
| Qualcomm | @FL: We are ok to further discuss related functionality capability aspects. But first would you please list functionalities of AIML positioning? Let’s start by developing understanding on functionality listing of AI/ML positioning. This is something that we have not discussed and shall be the starting point in the discussion. |
| Nokia | In our view, there are previous step before the step that the proposal is focusing. For example, the applicable functionalities is not the same to reporting UE capabilities. In our view, Applicable functionalities (List of Functionalities) should be a subset of Functionalities that were already defined/indicated by LMF in a previous step. In summary, the initial steps should be: Step 1: UE provides UE capabilities Step 2: LMF set a list of Functionalities to UE (LIST 1) Step 3: UE indicates applicable functionalities (LIST 2) Step 4: …. LIST 2 is a subset of LIST 1. |

# Proposals for online sessions

## Proposals for Monday online session

Proposal 4.1.2-2A

For training data collection of AI/ML based positioning, the quality indicator of timing measurements in Part A (e.g., PDP, DP) is:

- the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

Proposal 4.2.2-1A

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are supported, and using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Proposal 5.2.2-1

For model inference of AI/ML positioning Cases 3b, the gNB sends at least the following in a measurement report to LMF:

- Channel measurement;

- Quality of the channel measurement;

  - FFS: details on quality

- Time stamp of the channel measurement.

Proposal 5.2.2-2

For model inference of AI/ML positioning Cases 3a, the gNB sends at least the following in a measurement report to LMF:

- Intermediate positioning measurement as generated by the model at gNB;

- Quality of the intermediate positioning measurement;

  - FFS: details on quality

- Time stamp of the intermediate positioning measurement.

Proposal 6.2.2-1A

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option B-2 is deprioritized.

## Proposals for Tuesday online session

Proposal 5.1.3-1A

For AI/ML positioning Case 1, to provide NW-side additional condition for ensuring consistency between training and inference, RAN1 study the following options:

- Option A. using assistance data available in specification, which is sent from LMF to UE during training data collection and model inference.

  - Proponent companies provide details on the assistance data to use (e.g., IEs in NR-DL-PRS-AssistanceData) and necessary enhancements, if any.

- Option B. introduce an associated ID to provide NW-side additional condition. The associated ID is sent from LMF to UE during training data collection and model inference.

  - Proponent companies provide details (e.g. definition, granularity of an associated ID value) on the associated ID.

  - Note: If introduced, the associated ID can be included as a part of assistance data.

Proposal 5.2.3-1

From RAN1 perspective, for model inference of AI/ML positioning Cases 3b, at least the following are supported in a measurement report from gNB to LMF:

- (Mandatory field) Channel measurement;

- (Optional field) Quality of the channel measurement;

  - FFS: details of the quality, e.g., whether both timing and power quality are provided if the channel measurement includes both timing information and power information, the association between quality and measurement

- (Mandatory field) Time stamp of the channel measurement.

Proposal 5.2.3-2

For model inference of AI/ML positioning Cases 3a, at least the following are supported in a measurement report from gNB to LMF:

- (Mandatory field) Intermediate positioning measurement as generated by the model at gNB;

- (Optional field) Quality of the intermediate positioning measurement;

  - FFS: details of the quality

- (Mandatory field) Time stamp of the intermediate positioning measurement.

Proposal 3.1.3-1

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported for a measurement result, and the associated timing information is provided by legacy method, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- One LOS/NLOS indicator value is optionally reported for a measurement result, as in the existing specification.

- The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455.

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

## Proposals for Wednesday online session

Proposal 5.2.3-2A

From RAN1 perspective, for model inference of AI/ML positioning Cases 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) LOS/NLOS indicator and/or timing information, which corresponds to the model output of gNB-side model;

- (Optional) Quality of the timing information when the timing information is reported;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

Proposal 4.1.4-1A

For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when necessary, quality indicator of label is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2.

- Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use.

- Option 2: define a new IE on UE location quality provided by LMF to the UE. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality.

  - Proponent companies provide information on how to determine the value.

Proposal 3.1.4-1A

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is generated by the AI/ML model at NG-RAN and reported for a measurement result, and the associated timing information is not generated by AI/ML, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455, which can be soft indicator or hard indicator.

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

Proposal 3.1.4-1B

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is generated by the AI/ML model at NG-RAN and reported, RAN1 study whether the following options are supported:

(1) LOS/NLOS indicator can be reported with associated timing information which is generated by AI/ML;

  - If supported, the meaning of the LOS/NLOS indicator;

(2) LOS/NLOS indicator can be reported without any associated timing information;

  - If supported, the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

Proposal 2.1.1.3-1A

For the definition of sample-based measurement, for the measurement of an estimated channel response, RAN1 study the following options for down-selection for determining the starting time of the list of Nt samples:

- Option A. the starting time of the list of Nt samples is the timing of the first detected path rounded down to the nearest timing granularity of T relative to a reference time.

- Option B. the starting point of the Nt consecutive samples is determined by a fixed offset relative to the reference time.

  - Proponent companies provide details on the value of the fixed offset.

- Option C. the starting point of the Nt consecutive samples is determined by a configured offset relative to the reference time.

  - Proponent companies provide details on the value range and signaling details of the configured offset.

Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time.

Proposal 2.1.1.3-2A

For the definition of sample-based measurement,

- The set of candidate values for Nt include at least: 16, 32, 64, 128;

- The set of candidate values for Nt' include at least: 9, 16, [32]; Note: Nt'<= Nt.

- The set of candidate values for k include at least: 0…5.

## Proposals for Thursday online session

Proposal 5.2.5-1

From RAN1 perspective, for model inference of AI/ML positioning, when timing information is reported for Rel-19 Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator and other report types (if supported for Rel-19 Case 3a).

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Proposal 5.1.6-1

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE during training and inference, for ensuring consistency between training and inference,

- Companies are encouraged to provide input using the excel template in their contribution to RAN1#119.

- Note: this does not mean that training and inference phases are mentioned in assistance data.

|  | Company Name | Company 1 |
| --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |
| 4 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |
| 5 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |
| 6 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |
| 7 | PRS-only TP indication |  |
| 8 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |
| 9 | LOS/NLOS indicators |  |
| 10 | On-Demand DL-PRS-Configurations |  |
| 11 | Expected Angle Assistance information |  |
| 12 | PRS priority list |  |
| 13 | Validity Area of the Assistance Data |  |
| 14 | Data facilitating the integrity results determination of the calculated location |  |
| 15 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |
| 16 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |
| 17 | The association information of DL-PRS resources with TRP Tx TEG ID |  |
| 18 | PRU measurements together with the location information of the PRU |  |

| Legend |  |
| --- | --- |
| IE content is essential for Case 1 for ensuring consistency between training and inference. IE is provided to UE via explicit signaling in the assistance data. | E |
| IE content provides NW-side additional condition for ensuring consistency between training and inference. Implicit signaling is acceptable if explicit signaling is not provided. IE is provided to UE via implicit signaling in the assistance data. | I |
| IE is irrelevant for Case 1 inference, and/or for ensuring consistency. No need to signal. | N |

Proposal 2.1.1.4-1

For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for down-selection for determining the starting time of the list of Nt consecutive samples:

- Option A. the starting time of the list of Nt consecutive samples is the timing of the first sample of detected channel response.

- Option B. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time.

  - Proponent companies provide details on the value of the fixed offset.

- Option C. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time.

  - Proponent companies provide details on the value range and signaling details of the configured offset.

Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time.

## Proposals for Friday online session

Proposal 4.1.5-1A

For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2.

- Option 1: reuse an existing IE. For each case, proponent companies provide input on the existing IE to use.

- Option 2: define a new IE on UE location quality. Use hard (0 or 1) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicating the highest quality.

  - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

Proposal 3.1.5-1

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the other associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning.

FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

Proposal 3.1.5-2

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, RAN1 study whether the following reporting are supported:

(1) LOS/NLOS indicator is reported together with other associated measurement (e.g., timing information), which is also reported for Rel-19 Case 3a;

  - If supported, study the meaning of the LOS/NLOS indicator;

(2) LOS/NLOS indicator is reported without any other associated measurement (e.g., timing information);

  - If supported, the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported.

Proposal 2.1.1.4-2

For the definition of sample-based measurement,

- The set of candidate values for Nt include at least: 16, 32, 64, 128;

- The set of candidate values for Nt' include at least: 9, 16, [32]; Note: Nt'<= Nt.

- The set of candidate values for k include at least: 0…5.

Proposal 2.1.1.4-1

For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for down-selection for determining the starting time of the list of Nt consecutive samples:

- Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T).

- Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable.

- Option C. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time.

  - Proponent companies provide details on the value of the fixed offset.

- Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time.

  - Proponent companies provide details on the value range and signaling details of the configured offset.

Proposal 5.1.6-1A

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- RAN1 consider the following options:

  - Option 1: for each of the existing assistance data IE of UE-based DL-TDOA and UE-based DL-AoD, study whether it should be: (a) explicitly indicated; or (b) implicitly indicated; or (c) not applicable.

  - Option 2: Support explicit indication of each of the existing assistance data IE of UE-based DL-TDOA and UE-based DL-AoD

    - Study which of the existing assistance data IE listed could also be provided through an implicit indication

  - Option 3: for each of the existing assistance data IE of UE-based DL-TDOA, study whether it should be: (a) explicitly indicated; or (b) implicitly indicated; or (c) not applicable.

  - Option 4: Re-use IEs from UE-based DL-TDOA as is.

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | E | E |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | E | E |
| 3 | DL-PRS configuration of candidate NR TRPs | E | E |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | E | N |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | E | E |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | E | E |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | E | E |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | E | N |
| 9 | PRS-only TP indication | E | E |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID | E | N |
| 11 | LOS/NLOS indicators | E | E |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | E | E |
| 13 | Validity Area of the Assistance Data | E | E |
| 14 | PRU measurements together with the location information of the PRU | E | N |
| 15 | Data facilitating the integrity results determination of the calculated location | E | E |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | N | E |
| 17 | Expected Angle Assistance information | N | E |
| 18 | PRS priority list | N | E |

* E=Explicitly indicated; N=Not applicable

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

# Agreements made at RAN1#118bis

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

# Future work

During RAN1#118bis, there are several issues that companies discussed, but no agreement was made. Companies are invited to continue the investigation and share the views in contribution to RAN1#119, based on the last version of the proposals (most of them are copied in section 8.5).

In the following, the open issues are listed for discussion at RAN1#119.

- Model input

  - (Section 2.1.1) Sample-based measurement.

    - The starting time of the list of Nt consecutive samples. There are four options listed in Proposal 2.1.1.4-1 in section 8.5. Please analyze the options and give recommendation in RAN1#119.

    - The set of candidate values for parameters Nt, Nt’, k. Proposal 2.1.1.4-2 in section 8.5 was the outcome of discussion among companies. Please provide your input whether RAN1 can start with these values, and what other values (if any) should be considered.

  - (Section 2.1.2) Path-based measurement

    - Regarding the potential enhancement on the number of reported paths, vivo conducted simulations and concluded that “enhancement on the number of reported paths is not needed”. Proponent companies are invited to provide simulation results on the need to enhance, and if so, how many more paths to be reported. Please be concrete with your analysis and recommendation.

- Model output

  - (Section 3.1) Regarding LOS/NLOS indicator and the associated measurement (e.g., timing information), companies converged on two proposal: Proposal 3.1.5-1 and 3.1.5-2 (see section 8.5). For the two issues to be investigated in proposal 3.1.5-2, please share your input in RAN1#119.

- Training data collection

  - (Section 4.1) For quality indicator of label, two options are described in Proposal 4.1.5-1A (see section 8.5). Please provide your view on the issues to RAN1#119.

- Ensure consistency between training and inference (Section 5.1)

  - An agreement was made on the assistance data IEs, with the 18 existing information of UE-based DL-TDOA and DL-AoD listed in the table. Please share your views on this topic.

  - To ease the burden of summarizing input from all companies, it is much appreciated if you can enter your input for Case 1 using one of three choices: E=explicit, I=implicit, O=other. Please provide your input using the table in the agreement, so that all companies use the same ordering of the AD information. Aligning the input format is essential so that all companies’ input can be summarized into one big table in Excel by copy-and-paste.

  - For further enhancements beyond the AD information in the table, please describe the enhancements separately.

- Model monitoring

  - (Section 6.1) Regarding label-free methods and self-monitoring, there was an effort to identify the specification impact, if any. Here most monitoring procedures are up to implementation and out of 3GPP scope. It would be good if we can conclude on the specification impact, if any.

  - (Section 6.2) For Case 1, there was an attempt to decide what to support, or not to support, regarding the options A-1, A-2, A-3, B-1, B-2. Please check the views from all companies, and give your suggestion. We can try again to make progress in RAN1#119.

- Functionality and UE capability reporting

  - (section 7.2) Companies have submitted views on this topic since the start of the work item, but no decision has been made in RAN1. Since agreements are being made in RAN2 progressively, RAN1 can ramp up the discussion here as well.

# References

RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

R1-2407649	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2407654	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2407747	Specification support for positioning accuracy enhancement	Tejas Network Limited

R1-2407797	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2407849	Specification support for positioning accuracy enhancement	vivo

R1-2407893	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2407951	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2407989	AI/ML based Positioning	Google

R1-2408028	Discussion on AI/ML-based positioning	CATT, CICTCI

R1-2408104	Discussion on specification support for AI/ML-based positioning accuracy enhancement	Fujitsu

R1-2408159	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2408214	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2408267	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2408290	Specification support for AI/ML for positioning accuracy enhancement	Intel Corporation

R1-2408309	Specification support for positioning accuracy enhancement	Baicells

R1-2408381	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2408385	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2408391	Specification support for AI-enabled positioning	NVIDIA

R1-2408402	Support for AI/ML for positioning accuracy enhancement	Sony

R1-2408429	Specification impacts for AI/ML positioning	Lenovo

R1-2408453	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2408522	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2408545	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2408559	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2408609	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2408631	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2408774	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2408838	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2408903	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2408908	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

R1-2408923	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

# Appendix A. Agreements from RAN1#118

Model input

Agreement

For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples

- The Nt samples have timing granularity T.

- FFS: the starting time of the list of Nt samples

- FFS: the value range of Nt

For the sample-based measurement (if accepted in Rel-19),

- For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power.

- Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Agreement

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Model training

Agreement

For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement,

- For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective

- Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Agreement

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Model monitoring

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Conclusion

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

All LCM stages

Conclusion

From RAN1 perspective, for Case 3a measurements,

- The existing procedures can be reused in terms of SRS configuration.

  - Note: parameter values for SRS configuration can be further discussed

- These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures.

- Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications