3GPP TSG RAN WG1 #119	R1-2409366

Orlando, US, November 18th - 22nd, 2024

Source:	Nokia

Title:	R2D and D2R channel/signal aspects for Ambient IoT

Agenda item:	9.4.2.3

Document for:	Discussion and Decision

# Introduction

In this document, we will present our views on the list of physical channels that can be considered for study and evaluation in the context of AIoT use case. We will present the topics that are highlighted in the previous RAN1#118 meeting by FL.

Please consider at least following aspects for RAN1#119. R2D - Remaining design details for R2D signals including clock-acquisition part and other signals (if any, potentially for frequency synchronization) - Remaining R2D control information for D2R scheduling and/or R2D reception D2R - Remaining design details for D2R signals including preamble, postamble (if any), midamble (if any) - D2R control information, if any In general, companies can provide further observations for different options that we have agreed to capture in the TR.

# Previous Agreements

### RAN1#118 considerations

In RAN1#118bis, the following agreements were made.

Agreement For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769: - Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration Agreement For the PRDCH design details, following is captured in the TR 38.769: Following design options have been studied for PRDCH: - Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling - Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively Agreement For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769: - Following options have been studied for the start-indicator part of the R2D time acquisition signal: - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions. - Miss-detection ratio (MDR), false-alarm ratio (FAR) and detection complexity have been considered for the design of the R2D start indicator part by following companies - It is observed by 1 source [Huawei] that for an FAR of ~0%, the MDR of less than 1% can be achieved with Alt 2 of option 1 (considering 2 ON-OFF transmissions with different durations) and it is also observed that low-complexity and reduced power consumption can be achieved - 1 source [ZTE] evaluated Alt 1 of option 1 (considering same duration for ON and OFF) and Alt 2 of option 1 (considering multiple ON-OFF transmissions with same duration) and observed that for an FAR of ~0%, the MDR of less than 1% can be achieved and Alt 1 of option 1 performs better than Alt 2 of option 1. - 1 source [CATT] observed with ON-OFF pattern, that for an FAR of ~0%, the MDR of less than 1% can be achieved with a duration of at least 1 OFDM symbol - 1 source [Qualcomm] compares the performance between option 1 and option 2. It shows almost similar coverage range (SNR requirement) for target MDR of 1%. For MDR of 10%, it shows that sequence-based design provides better performance, and it is observed that during the available time, it is feasible for all devices to detect the start-indicator sequence within the power budget. It is further observed that the FAR with sequence-based design can be improved in case of interference scenarios when compared with pattern-based design. - For both the options, it may be beneficial that the start-indicator part is distinguishable at least from other parts of the R2D transmissions Agreement For single PDRCH generation at the device, following options are captured in TR 38.769: - Option 1: With line coding - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1 - Note: Presence of the block “small frequency shift with square wave generation” depends on the agreed options for small frequency shift under agenda 9.4.2.1 - Note: the figure below is for the case where FEC and repetition are present - The figure will be updated by updating the text in the dotted box as follows: “Small frequency shift” - Option 2: Without line coding - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1 - Note: the figure below is for the case where FEC and repetition are present Note: Further updates to PDRCH generation can be considered, depending on the progress Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion Agreement For the D2R preamble, binary signal is considered. Agreement For the topology 2, following is captured in the TR 38.769: For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied: - Option 1: Higher-layer signaling is used - Option 2: L1 and higher-layer signaling are used From RAN1 perspective, detailed aspects on the two options are left up to work item phase. Following observations related to the two options have been made by following companies: - Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2. - One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE - Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1 For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

In RAN1#118, the FL proposed the following consideration [1]:

R1-2407555 FL summary#1 on downlink and uplink channel/signal aspects Moderator (Apple) Agreement For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH. Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Conclusion Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item. Solution 1 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission - then the device is determined as near to the reader based on measurements at the reader side Solution 2 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader Agreement For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied - FFS: length/pattern of ON/OFF. - FFS: when TD2R_min is applicable, whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2 Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher layer signaling and/or L1 R2D control signaling is used Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) FFS: other information FFS: For each information, whether higher layer signaling and/or L1 R2D control signaling is used

# Discussion

In the following, the terminologies ‘R2D’, ‘D2R’, ‘CW’, ‘CW2D’ and ‘RF-EH’ are adopted as captured in [2]. For clarity, Figure 1 depicts the following terminologies:

- Reader to device (R2D) refers to the signal between the reader and the device (tag). In the FL summary in section 1, the R2D channel has been referred to as downlink signal.

- PRDCH is the physical channel between the activator and the AIoT device.

- PDRCH refers to the physical channel between the AIoT device and the reader.

- Device to reader (D2R) signal is the signal sent/backscattered by the device and which the reader is observing and attempting to detect. In the FL summary from section 1, this channel has been referred to as uplink signal.

- Contention-based access is used instead of random access.

Figure 1: AIoT Terminology.

## Topic 1: R2D (reader-to-device transmission channels)

### Synchronization Signals

In this section, we will discuss the design and need of various synchronization signals intended to provide timing, frequency, or sampling corrections. Fig. 2 highlights PRDCH transmission with different segments/parts of synchronization signal, which are discussed below.

Fig. 2 PRDCH format with optional synchronization signals

In RAN1#118bis, FL provided the way forward for next meeting discussion topics.

R2D - Remaining design details for R2D signals including clock-acquisition part and other signals (if any, potentially for frequency synchronization) Remaining R2D control information for D2R scheduling and/or R2D reception

#### Start delimiter

To avoid any unwanted AIoT tag triggering to perform preamble correlation, a start delimiter is used only to initiate the AIoT tag to get into active state to initiate preamble detection followed by PRDCH reception. Thus, the start delimiter is used only to initiate the AIoT tag, during which the tag has no timing accuracy. Thus, to ensure reliable start delimiter detection, the silence length should be longer with two ON segments to avoid any unwanted glitch getting detected as a start delimiter. Furthermore, the width of two ON pulses should be wide enough to provide sufficient energy to trigger the AIoT tag.

Agreement For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied - FFS: length/pattern of ON/OFF. - FFS: when TD2R_min is applicable, whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2 Agreement For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769: - Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration

Furthermore, the start delimiter signal must be robust enough to be detected even in the presence of interference. As the device is assumed to perform preamble detection only after the start delimiter detection, any failure in detecting start delimiter may cause the complete failure of AIoT transmission destined to the tag. Thus, the start delimiter should be designed by considering both interference and interference-free scenarios.

Thus, to ensure proper detection of start delimiter, having a ON/OFF pattern with high/low transition enables the device to identify the presence of start delimiter even in the presence or absence of interference as long as the interference stays constant for a duration of start delimiter. However, having OFF only pattern for start delimiter may not solve the purpose of reliable detection as the device cannot ensure the constant voltage with variations is due to interference or silence. Therefore, having a constant low voltage may not guarantee proper detection when there is interference. During silence period, having a low voltage as start delimiter will force the device to start preamble detection unnecessarily.

Start delimiter should be long enough to ensure reliable detection due to large sampling offset observed in the AIoT tag before estimating/correcting from preamble.

RAN1 should ensure start de-limiter to have at least two ON pulses with sufficient width and separation to avoid false activation and to provide enough energy to validate the threshold.

#### Preamble

Given the nature of the devices to be read and the agreed topologies, there is no need for periodic synchronization signal transmission. Instead, a preamble-based synchronization is the preferred option, where the preamble is attached to the R2D signal. The preamble serves at least two purposes i.e., that of synchronization and to provide some knowledge about the channel condition between the reader and the AIoT device. The following agreement was agreed in previous RAN1 meeting 118, where the start delimiter is not a part of preamble but will be used before the transmission.

Agreement For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH. Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion

The preamble is a known sequence that is used by the AIoT tag upon activation to obtain the synchronization by determining the start of a transmission, i.e., identifying the frame boundary. The AIoT tag must perform a sequence correlation with a known preamble sequence and determine the peak to obtain the timing of PRDCH receptions. The use Manchester coding is subjected to the synchronization accuracy and the quality of autocorrelation from the resulting sequence correlation.

Fig. 3 Preamble design with or without Manchester encoding

The need for Manchester encoding on preamble sequence is not imperative unlike the payload, which is detected bit by bit. As the preamble detection is typically performed as a sequence correlation, it is essential to design the sequence robust against all receiver impairments, such as, frequency and sampling frequency offsets. The timing estimation is the primary target of preamble sequence, and thus the preamble sequence must demonstrate better autocorrelation properties. Thus, to satisfy the above properties, such as, balanced number of ones and zeros to aid AGC tuning and threshold determination together with better autocorrelation properties, “m-sequence” family of sequences can be considered.

Preamble sequences must have better autocorrelation properties to obtain better timing accuracy even with large sampling offset.

OOK based sequences are inherently robust against frequency offset, thus, frequency offset may not pose a bigger threat if there is sufficient guard band around the transmission BW.

Increasing the value of M limits the coverage of transmission but improves the timing accuracy.

Increasing the value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length.

Consider fixed transmission duration irrespective of the value of M, i.e., option 2 is preferrable.

To evaluate the impact of preamble length on the timing estimation accuracy, we considered preamble sequence design with L={8,16,24,32,48,64} using “m-sequence” without Manchester encoding. The channel used is “TDLA30ns” with 1.44MHz transmission BW and 1.92MHz occupied BW and sampling rate of the AIoT device. The “m-sequence” is generated with the respective polynomial of same degree to ensure equal number of 1’s and 0’s to aid AGC tuning if used. The comparison is drawn with sampling offsets {0,100} ppm and at SNR of {3,9}dB.

Fig. 4 illustrates 90 percentile timing estimation accuracy against the preamble length, OOK chip rate M∈{1,2,4,8}, and sampling offsets. In the ideal case, i.e., when SFO is 0ppm, the estimation accuracy improves with the increase in the length of preamble sequence. This is mainly attributed to the processing gain of larger sequence. It is worth noting that the impact of preamble sequence length does not affect the estimation accuracy when the SNR is 9dB as the noise is negligible.

On the contrary, when the SFO is high, i.e., ≈100ppm, the accuracy of estimating timing offset degrades with the increase in the preamble sequence length. This is due to the drift experienced over time, which is quite evident with lower M={1,2} than with higher M values. Thus, the preamble sequence length should be designed by considering both OOK M value and the minimum SFO that must be supported by the sequence design.

| (a) 90% timing estimate at SNR of 3dB | (b) 90% timing estimate SNR of 9dB |
| --- | --- |

Fig. 4 Impact SFO on OOK chip rate M and preamble length on timing estimation accuracy.

Use of M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

The length of preamble sequence must be decided considering coverage, timing estimation accuracy, and the robustness against SFO.

RAN1 should consider preamble sequence of length in the range of [16,32] together with M={4,8}.

RAN1 should design preamble based on “m-sequence” as it provides better autocorrelation properties and has balanced number of ones and zeros.

The use of preamble to estimate sampling frequency requires multiple preamble correlations and calculating the timing drift at each correlation attempts. However, using a single preamble to determine the sampling offset may not be efficient as the drift observed over a smaller duration cannot provide robust estimate of the sampling offset.

If sampling offset can be estimated to a certain extent at AIoT device, it facilitates the support of larger payload sizes without the need of midamble and postamble for resynchronization.

RAN1 should study how preamble can be used to determine sampling frequency offset to support larger payload in PRDCH.

Figure 5 Overlaying chirp sequence in ON duration to facilitate clock acquisition for device type 2b.

The clock acquisition signal containing the sequence that provides frequence synchronization for device type 2b shall be considered as an overlay sequence on the ON duration of binary sequence. Unlike the device type 1/2a, device type 2b uses I/Q branch for receiver side processing, thus ensuring frequency synchronization is paramount for efficient detection as such devices are driven by a batter source without external carrier signal.

Figure 5 demonstrates an approach to overlay a chirp signal, which sweeps the frequency from the lower part of transmission BW to the upper part of transmission BW of AIoT signal. The overlay sequence also provides better performance for I/Q based device type 2b receivers as it uses coherent combining of chirp sequence to boost the post processing SNR. Thus, the advanced device type 2b devices can benefit from the chirp sequence to tune the clock as it uses specific frequency sweeping pattern. Thus, no additional clock acquisition signal is needed other than the preamble carrying AIoT reader specific ID, which follows the start delimiter section.

Overlaying a chirp like constant envelope sequence in the ON duration of preamble/clock acquisition signal facilitates frequency synchronization needed for device type 2b.

#### Midamble and postamble

RAN1 also discussed the need of a midamble in R2D signal for ensuring that the device remains synchronized with the reader (activator). It is worth noting that the AIoT transmission uses low data rate and may last only for a shorter period. Therefore, impact of sampling frequency offset, which is in the order of 10Xppm, may not severely degrade the detectability of AIoT device.

The midamble may additionally help the reader to estimate the tag channel and CW interference (only if the target signal is in each SINR range) and ease the reader in performing the CW interference prior to detection.

As the Manchester encoding is assumed for PRDCH payload, midamble may not be needed as Manchester coding itself is a self-synchronizing sequence.

Lastly, a postamble to end the R2D signal reception, was also discussed by many companies in the previous meeting. The use of postamble may not be needed if the length of each R2D transmission uses a fixed length or the size of the payload is indicated by other means, namely, a control signal or by using a unique preamble sequence. This has been captured by the FL summary in one of the proposals presented [1], which is as follows.

For ambient IoT devices, consider following options for postamble for downlink transmissions. - Alt. 1: Postamble is not considered. - Alt. 2: Postamble is considered for adding at the end of the downlink transmission to signal the termination of downlink. FFS: Whether it is always added irrespective of the transport block size

The use of midamble is needed to reacquire the timing synchronization for the device having large sampling offset, if the duration of the transmission lasts longer following the preamble.

Even with large sampling offset, midamble may not be needed if the payload length is short, since the initial timing alignment is carried out by the preamble sequence.

Terminating the PRDCH transmission with a postamble may provide two benefits, namely, the variable payload length and to provide timing acquisition before the subsequent transmission of either PDRCH or PRDCH, thus improving the detectability at both reader and the device, respectively.

As Manchester encoding is considered for R2D transmission, the need for postamble is not imperative.

### Broadcast channel

The following FL proposal was discussed in RAN1#116 [1]:

For ambient IoT devices, periodic broadcast channel, i.e. PBCH is not considered. - FFS: Whether/how dynamic/aperiodic broadcast information is transmitted (if needed), e.g. via downlink data channel

And the following agreements were reached in RAN1#116:

Agreement For ambient IoT devices, a dedicated physical broadcast channel for R2D, e.g. PBCH-like, is not considered for study. Agreement For ambient IoT devices, at least for R2D data transmission, a physical channel (PRDCH) is studied, - System information (if defined) is transmitted on the PRDCH - FFS Whether/how control information is transmitted on the PRDCH - Note: the naming of PRDCH is used for the sake of the stud

As per the agreements, no PBCH-like dedicated physical broadcast channel is considered. Instead, any system information (SI), if defined, is transmitted on the PRDCH.

In AIoT scenarios, it may be beneficial to provide the configuration on at least access parameters, e.g., if defined, backoff time, resources or time-frequency resource offset, etc., via SI. However, the AIoT devices have different memory capability and not all devices may be able to hold the SI in the memory for long. Therefore, at first, the essential system information needs to be identified. In addition, how the SI-contained PRDCH can effectively be transmitted to avoid SI duplication at the device needs to be studied.

The broadcast information containing configuration for channel access, e.g., initial contention-based access, may be beneficial, and may precede any device-specific configuration information.

RAN1 to study what broadcast information is required for the AIoT device and to identify how it should be included in the PRDCH channel.

### Control and data channels

The following FL proposal is relevant to control channel study:

R2D - Remaining design details for R2D signals including clock-acquisition part and other signals (if any, potentially for frequency synchronization) Remaining R2D control information for D2R scheduling and/or R2D reception

The need for a separate control channel or not depends on number of factors that can be considered for an AIoT transmission, which are listed as follows.

- If there are fewer modulation schemes determined during the SI as the probable schemes, then the choice of modulation scheme used for the payload must be informed. Furthermore, a tag must know the possible modulation scheme used for the control information apriori, i.e., only a subset of modulation available for the data channel can be used for control channel to limit the blind detection complexity. Thus, defining a separate control channel may be efficient, since the configurable parameters used to indicate the payload construction can vary quite large due to the capability of device types used in AIoT SI.

- If the trade-off between the power and complexity is large enough to limit the number of configurable parameters used for different device types, then the control information can be appended to the data channel itself with a single CRC for both control and payload, thus reducing the complexity of the tag. In such a case, having a separate control channel may not be efficient as the CRC overhead for few configurable control bits may be redundant.

PRDCH may thus contain wakeup/query commands, other data traffic or configuration information for the subsequent PDRCH transmission. The presence of such information may depend on the AIoT device type being served. The content of PRDCH payload may carry the following information.

- (wake-up info) used to indicate the ID of one or more AIoT devices which should reply to the activation signal. The IDs may be communicated in the native format, i.e., the signal may encode the ID in its phase and/or amplitude, or it may be encoded and modulated by the complex symbols e.g., modulated OOK waveform carrying a sequence in the ON duration for device types 2a/2b.

- The operation mode selected for the AIoT device – this is relevant for hybrid AIoT devices that can switch between type 1, 2a and 2b. If the operation mode is pre-configured by a session control unit, this field may be absent from the payload fields.

- (config info) carries configuration information for the AIOT reply including:

  - Query command e.g., if the AIoT reply is for e.g., reading or localization. This information may be used by the AIoT device to trigger a given reply mode including the type of signal it transmits.

  - The transmit power in case the device is type 2.

  - The reply offset with respect to the reception time of the PRDCH.

  - The frequency offset with respect to the reception frequency for the PRDCH in case of device 2a/2b.

  - Modulation and any FEC usage on OOK symbols.

- An illustration of various formats describing the control channel is provided in Figure 6. The use of control only with CRC should be considered as a baseline as it ensures reliable detection of control to decode the following payload (data) correctly. However, if the payload size is smaller, then the CRC can be combined with the data and added at the end.

Figure 6 PRDCH/PDRCH structures with embedded control information.

Some device type 1 AIoT devices could support both OOK or BPSK for the PRDCH, depending on the configured frequency of the CW signal and a potential dedicated RF harvesting signal on a different frequency, leading to the following two scenarios:

Scenario 1: Only a CW signal à The device type 1/2a AIoT devices will use OOK for the PRDCH.

Scenario 2: A CW and a dedicated RF harvesting signal on different frequencies à The device type 1 AIoT devices will use BPSK for the PRDCH to increase coverage.

The device type 1 AIoT might not be capable of detecting the presence of both a CW signal and a dedicated RF harvesting signal on different frequencies for scenario 2, so the session control unit could keep track of this and communicate this information to the AIoT device directly or indirectly by modulation type.

The used frequency of a RF harvesting signal could be known by the session control unit or even specified by the session control unit.

The PRDCH may carry a wake-up info to select a subset of devices for the inventory process in addition to the modulation and resource allocation fields targeted towards a specific device type.

RAN1 to study the necessary control information to be contained in PRDCH depending on the AIoT device type, including activation and query commands.

RAN1 should consider a flexible design, i.e., by having control only, data only, or control and data fields all carried by PRDCH itself.

To determine the maximum length of payload that can be supported by PRDCH transmission, we compared the performance of AIoT PRDCH message of sizes {20,96,400} bits in the presence of residual sampling frequency offset of 100ppm under TDLA30ns channel. Fig. 7 demonstrates the SNR requirement to achieve the target BLER of {1,10}% by varying the OOK chips per equivalent NR symbol, i.e., M, together with the length of payload size.

| (a) SNR required for 10% BLER | (b) SNR required for 1% BLER |
| --- | --- |

Fig. 7 Comparison of SNR requirement to achieve {1,10}% BLER varying payload size {20,96,400} bits.

The simulation is performed by constructing a PRDCH transmission with preamble of length 32 followed by payload of sizes 20, 96, 400 bits with Manchester encoding. The preamble sequence is transmitted without Manchester encoding and the timing is determined by the detection accuracy of preamble estimation. Thus, the simulation includes the impact of estimation error, which is like the realistic scenario. Note that the residual sampling offset is assumed to be of 100ppm, which is plotted in the lower half of Fig. 7a, and Fig. 7b. As can be seen, by varying the M={1,2,4,8}, the performance degrades significantly, when the length of the payload is smaller. However, when the payload size is 400 bits, the detection is not possible to achieve even 10% BLER when the sampling offset is greater than 100ppm.

Under the assumption that the residual sampling error is ≥100ppm, detection is not possible to achieve even 10% BLER when the payload size is ≥400bits.

RAN1 should consider preamble design to assist SFO correction/calibration to support larger payload size.

Unlike NR, where there is definite frame counter associated with each transmission, AIoT transmission slot counter is aligned with the symbol boundary of NR transmission. However, the period elapsed between two successive slots is not getting reflected in the slot counter as it is up to the reader, who is initiating the transmission. Thus, even if the AIoT device aims at calibrating the clock to reduce the SFO, the elapsed time between two successive timing estimates, which is usually considered for SFO calibration, cannot be performed.

Furthermore, if there is any scheduled sleep triggered by the reader to a AIoT device, without having a clock reference, the device cannot fully utilize the number of sleep slots before the impending wake-up as the current AIoT slot counter has arbitrary period. Thus, to facilitate both SFO correction and for better sleep management, the control information must carry certain field providing the time stamp with respect to a reference, which can be a NR frame number for simplicity. Thus, the need for SFO correction at the AIoT device is necessary to enable sleep modes to retain the stored energy or to perform energy harvesting before impending R2D/D2R transmissions, the use of preamble alone cannot ensure reliable clock calibration.

Time stamp should be considered in the control information of R2D to assist AIoT devices to perform scheduled sleeps and SFO calibration.

## Topic 2: D2R (device-to-reader transmission channels)

### Synchronization signals (including preamble/midamble/postamble)

#### Preamble

Agreement For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH.

The D2R signal transmission between the AIoT device to the reader contain preamble without the start delimiter as the reader can detect the preamble sequence once the R2D transmission ends. This has been agreed in previous RAN1 meeting. Thus, following our earlier discussions on the preamble length and the sequence type, it can reuse the same content provide for R2D, i.e., preamble sequence can be based on “m-sequence” of length [16,32] without Manchester encoding to obtain better autocorrelation peak, which helps in determining the timing estimation.

As the BW of transmission is narrower due to low data rate in D2R link, frequency hopping is proposed for D2R transmission to utilize the frequency diversity, thereby improving the robustness of the link. Even though it is considered for data transmission, it must be used also for preamble as it provides robustness against frequency selective nature of the channel.

D2R preamble serves multiple purposes, among which are:

  - uniquely identifying the AIOT device,

  - synchronizing the reader to the AIOT device transmission (i.e. determining time and frequency offsets).

  - estimating the D2R channel response, the latter operation being necessary in order to decode the AIOT data.

  - supporting multiple access, i.e. to enable concurrent D2R transmissions which are ultimately distinguishable at the reader side.

To address points 1-4 above, D2R preamble may be designed relative to at least the CW signal and/or R2D signal.

Study if the CW and/pr R2D signal preambles may be used to determine the D2R preamble to address points 1-4.

To determine the start of the uplink frame and to detect and correct the sampling offset introduced by the device transmission, the need for preamble is inevitable in the PDRCH transmission.

Depending on the quality of preamble transmitted from two discreet tones, the following PDRCH payload can be transmitted only from the selected link.

RAN1 should consider D2R preamble sequence of length of 16 or 32 without Manchester encoding.

Preamble sequence shall be transmitted from two discreet tones, if frequency hopping is used for PDRCH payload that follows.

RAN1 should design preamble based on “m-sequence” as it provides better autocorrelation properties and has balanced number of ones and zeros.

RAN1 should consider generating preamble sequence for D2R transmission using m-sequence as it is simpler to implement at AIoT device and has better auto/cross correlation properties.

#### Midamble and postamble

For ambient IoT devices, consider following options for midamble for uplink transmissions - Alt. 1: Midamble is not considered - Alt. 2: Midamble is considered for adding between uplink transmissions/segments - FFS: Design details

The proposal for using a D2R midamble inserted between every D2R transmission segments has two motivations:

- to help the reader with SFO compensation and

- to enable the reader to perform channel and SINR estimation, which is required for equalization and subsequent data detection.

Depending on the payload sizes, i.e., the size of PDRCH transmission, midamble may be needed to compensate the sampling offset when the payload size is large. However, it should be configurable per device as it depends on the observed sampling offset created by the device transmission. Furthermore, it is unclear whether a D2R midamble is different from a D2R reference signal.

RAN1 to assess in AI 9.4.2.2 whether both D2R midamble and D2R postamble are needed depending on the AIoT device type.

For ambient IoT devices, consider following options for postamble for downlink transmissions - Alt. 1: Postamble is not considered - Alt. 2: Postamble is considered for adding at the end of the uplink transmission to signal the termination of uplink - FFS: Design details

The D2R transmission may end with a set of samples indicating the termination of the frame.

Study in AI 9.4.2.2 the design of the D2R postamble which may contain a known stop sequence to indicate the end of the frame.

### Contention-based access channel

Contention-based access is captured in the following FL excerpts [1]:

D2R - High-Priority: Remaining design details for preamble part including (at least based on agreed functionalities for study) - sequence/pattern - Length - additional functionalities if any - High-Priority: PDRCH generation considering additional agreements under agenda 9.4.2.1 - High-Priority: Consideration on Msg1 design and whether/how it is carried by PDRCH or not

- The channel for Msg1 should facilitate multiple access for contention based random access. As the device type 1/2a utilizes the external carrier wave generated by a CW node to transmit a random-access preamble, with or without a small frequency shift, the length of the preamble and the list of supported small frequency shifts should be provided by the R2D transmission as a broadcast information.

Depending on the type of AIoT device, contention based random access can include additional frequency translations if the device supports frequency translation.

R2D control information intended for Msg1 should indicate list of small frequency shifts supported for any subsequent Msg1 preamble transmissions from AIoT device.

Msg1 preamble sequence should be based on m-sequence with a random initializer chosen by the AIoT device for CBRA with the length ranging from [32,64] to provide better cross correlation and randomization.

### Control and data channels

For D2R control channel, the following agreement was made in RAN1#116-bis.

Agreement For PDRCH generation at the device, at least following blocks are studied as the baseline: - CRC bits are appended if there is non-zero length CRC - Note: CRC details discussed in agenda item 9.4.2.1 - Coding - Exact coding methods within the coding block, e.g. with/without line coding and/or FEC discussed under agenda 9.4.2.1 - Note: If no line coding is used, there may be an additional block (e.g. square wave generator) before/after modulation block - Modulation - Note: Other blocks could be added if agreed PDRCH generation

The control information may include the following.

- (control information) A set of symbols used to indicate:

  - The modulation of the data symbols carrying the AIOT payload.

  - The coding (if applicable) of the payload.

  - Whether reference symbols are interleaved with data symbols. If that is the case, the pilot positions in time-frequency and their values need to be communicated.

  - A set of symbols used to indicate the index of the message block; in case the message has been split into multiple blocks. This may happen if the resources for the AIOT reply are too small to accommodate the full message, or if the AIOT device does not have sufficient charge to accommodate the full message transmission at the nominal power level.

Due to three different device types considered in this AIoT SI, RAN1 should consider either a separate control channel or reuse the existing PDRCH channel to carry control only, data only, or control and data fields in a flexible way.

RAN1 should study the flexible channel structure for PDRCH to carry control only, data only or both control and data using the same channel.

PDRCH may thus contain AIoT specific data, and other control information necessary to decode said data, and the presence of such control information may depend on the AIoT device type. PDRCH may contain one or more of the following:

- (data information) A set of symbols encoding the payload of the AIOT device e.g.:

- Identifier for the tag.

- Data collected by the AIOT device and stored in the internal memory if applicable.

- Reference signals.

RAN1 to study the necessary information to be contained in PDRCH depending on the AIoT device type, including control and data information.

## Topic 3: Proximity determination for topology 2

The proximity determination aspects were discussed in RAN1#118, and the following agreement was made.

Conclusion Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item. Solution 1 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission - then the device is determined as near to the reader based on measurements at the reader side Solution 2 For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

To configure the activator and reader for a set of AIoT devices, the network needs to discover which intermediate nodes are closer to the Ambient IoT device and which of these nodes are available to AIoT operation. The discovery of these nodes and their proximity to the AIoT device should be supported by the NR system with minimal spectral overhead and considering the target latency of the Ambient IoT session.

To enable the Ambient IoT device discovery, but also determining which nodes are in the proximity of the device, AIoT specific mechanisms for proximity determination need to be defined.

RAN1 to study the reader measurement types for proximity determination, where such measurements may be power-based or timing measurements of the D2R signal.

An inherent problem of proximity determination as defined so far is that the current proposal does not account for the CW interference, nor for the CW node proximity to the AIOT device. Specifically:

- If the CW node is far from the device, the AIOT device never gets to reply (as it does not hear the CW signal), thus, even though the AIOT device is close to the reader, the reader does not receive the AIOT signal, and may wrongly conclude the AIOT device is far.

- If the CW signal is received with much higher power than the AIOT reply at the reader, then the reader may fail to detect the AIOT reply and may wrongly conclude that the AIOT device is far.

- The RX power of the AIOT reply is a direct consequence of:

  - Not only the distance reader-AIOT device but also

  - The distance CW node- AIOT device,

  - thus, if the CW node is far from the AIOT device, the subsequent AIOT reply will be received with low power at the reader, and it may cause the reader to wrongly conclude that the AIOT device is far.

RAN1 to study and define ways to identify the conditions that affect the proximity determination outcome at least for D2T2 due to the involvement of intermediate node. Some of these conditions may be:

- Lack of AIOT response due to AIOT device being out of CW node’s range i.e., very large distance CW node – AIOT device or due to the lack of activation i.e., activation R2D signal is of insufficient strength.

- Failure to detect AIOT reply due to high levels of CW interference.

- Receiving a low power AIOT response due insufficient strength of the CW signal as observed by the AIOT device (due to large distance CW node – AIOT device).

## Resource allocation for topology 2

Proposal 2.4-1C RAN1 study at least following aspects (including their necessity) related to DL/UL signaling between the network and intermediate UE for topology 2 related to ambient IoT device(s): - Resource allocation for the intermediate UE - DCI enhancements, if any - UCI enhancements, if any Agreement For the topology 2, following is captured in the TR 38.769: For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied: - Option 1: Higher-layer signaling is used - Option 2: L1 and higher-layer signaling are used From RAN1 perspective, detailed aspects on the two options are left up to work item phase. Following observations related to the two options have been made by following companies: Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2. One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1 For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

The choice of identifying an intermediate UE to act as a CW node must be controlled by the NW and any subsequent control related to tone selection and resource allocation can be handled via DCI/UCI message.

# Conclusion

In this contribution, we have made the following observations and proposals related to Ambient IoT.

Observation 1:	Start delimiter should be long enough to ensure reliable detection due to large sampling offset observed in the AIoT tag before estimating/correcting from preamble.

Observation 2:	Preamble sequences must have better autocorrelation properties to obtain better timing accuracy even with large sampling offset.

Observation 3:	OOK based sequences are inherently robust against frequency offset, thus, frequency offset may not pose a bigger threat if there is sufficient guard band around the transmission BW.

Observation 4:	Increasing the value of M limits the coverage of transmission but improves the timing accuracy.

Observation 5:	Increasing the value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length.

Observation 6:	Use of M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

Observation 7:	The length of preamble sequence must be decided considering coverage, timing estimation accuracy, and the robustness against SFO.

Observation 8:	If sampling offset can be estimated to a certain extent at AIoT device, it facilitates the support of larger payload sizes without the need of midamble and postamble for resynchronization.

Observation 9:	As the Manchester encoding is assumed for PRDCH payload, midamble may not be needed as Manchester coding itself is a self-synchronizing sequence.

Observation 10:	The use of midamble is needed to reacquire the timing synchronization for the device having large sampling offset, if the duration of the transmission lasts longer following the preamble.

Observation 11:	Even with large sampling offset, midamble may not be needed if the payload length is short, since the initial timing alignment is carried out by the preamble sequence.

Observation 12:	Terminating the PRDCH transmission with a postamble may provide two benefits, namely, the variable payload length and to provide timing acquisition before the subsequent transmission of either PDRCH or PRDCH, thus improving the detectability at both reader and the device, respectively.

Observation 13:	The broadcast information containing configuration for channel access, e.g., initial contention-based access, may be beneficial, and may precede any device-specific configuration information.

Observation 14:	The PRDCH may carry a wake-up info to select a subset of devices for the inventory process in addition to the modulation and resource allocation fields targeted towards a specific device type.

Observation 15:	Under the assumption that the residual sampling error is ≥100ppm, detection is not possible to achieve even 10% BLER when the payload size is ≥400bits.

Observation 16:	To determine the start of the uplink frame and to detect and correct the sampling offset introduced by the device transmission, the need for preamble is inevitable in the PDRCH transmission.

Observation 17:	Depending on the quality of preamble transmitted from two discreet tones, the following PDRCH payload can be transmitted only from the selected link.

Observation 18:	RAN1 should design preamble based on “m-sequence” as it provides better autocorrelation properties and has balanced number of ones and zeros.

Observation 19:	Depending on the payload sizes, i.e., the size of PDRCH transmission, midamble may be needed to compensate the sampling offset when the payload size is large. However, it should be configurable per device as it depends on the observed sampling offset created by the device transmission. Furthermore, it is unclear whether a D2R midamble is different from a D2R reference signal.

Observation 20:	The D2R transmission may end with a set of samples indicating the termination of the frame.

The channel for Msg1 should facilitate multiple access for contention based random access. As the device type 1/2a utilizes the external carrier wave generated by a CW node to transmit a random-access preamble, with or without a small frequency shift, the length of the preamble and the list of supported small frequency shifts should be provided by the R2D transmission as a broadcast information.

Observation 21:	Depending on the type of AIoT device, contention based random access can include additional frequency translations if the device supports frequency translation.

Observation 22:	Due to three different device types considered in this AIoT SI, RAN1 should consider either a separate control channel or reuse the existing PDRCH channel to carry control only, data only, or control and data fields in a flexible way.

Observation 23:	To enable the Ambient IoT device discovery, but also determining which nodes are in the proximity of the device, AIoT specific mechanisms for proximity determination need to be defined.

Proposal 1:	RAN1 should ensure start de-limiter to have at least two ON pulses with sufficient width and separation to avoid false activation and to provide enough energy to validate the threshold.

Proposal 2:	Consider fixed transmission duration irrespective of the value of M, i.e., option 2 is preferrable.

Proposal 3:	RAN1 should consider preamble sequence of length in the range of [16,32] together with M={4,8}.

Proposal 4:	RAN1 should design preamble based on “m-sequence” as it provides better autocorrelation properties and has balanced number of ones and zeros.

Proposal 5:	RAN1 should study how preamble can be used to determine sampling frequency offset to support larger payload in PRDCH.

Proposal 6:	Overlaying a chirp like constant envelope sequence in the ON duration of preamble/clock acquisition signal facilitates frequency synchronization needed for device type 2b.

Proposal 7:	As Manchester encoding is considered for R2D transmission, the need for postamble is not imperative.

Proposal 8:	RAN1 to study what broadcast information is required for the AIoT device and to identify how it should be included in the PRDCH channel.

Proposal 9:	RAN1 to study the necessary control information to be contained in PRDCH depending on the AIoT device type, including activation and query commands.

Proposal 10:	RAN1 should consider a flexible design, i.e., by having control only, data only, or control and data fields all carried by PRDCH itself.

Proposal 11:	RAN1 should consider preamble design to assist SFO correction/calibration to support larger payload size.

Proposal 12:	Time stamp should be considered in the control information of R2D to assist AIoT devices to perform scheduled sleeps and SFO calibration.

Proposal 13:	Study if the CW and/pr R2D signal preambles may be used to determine the D2R preamble to address points 1-4.

Proposal 14:	RAN1 should consider D2R preamble sequence of length of 16 or 32 without Manchester encoding.

Proposal 15:	Preamble sequence shall be transmitted from two discreet tones, if frequency hopping is used for PDRCH payload that follows.

Proposal 16:	RAN1 should consider generating preamble sequence for D2R transmission using m-sequence as it is simpler to implement at AIoT device and has better auto/cross correlation properties.

Proposal 17:	RAN1 to assess in AI 9.4.2.2 whether both D2R midamble and D2R postamble are needed depending on the AIoT device type.

Proposal 18:	Study in AI 9.4.2.2 the design of the D2R postamble which may contain a known stop sequence to indicate the end of the frame.

Proposal 19:	R2D control information intended for Msg1 should indicate list of small frequency shifts supported for any subsequent Msg1 preamble transmissions from AIoT device.

Proposal 20:	Msg1 preamble sequence should be based on m-sequence with a random initializer chosen by the AIoT device for CBRA with the length ranging from [32,64] to provide better cross correlation and randomization.

Proposal 21:	RAN1 should study the flexible channel structure for PDRCH to carry control only, data only or both control and data using the same channel.

Proposal 22:	RAN1 to study the necessary information to be contained in PDRCH depending on the AIoT device type, including control and data information.

Proposal 23:	RAN1 to study the reader measurement types for proximity determination, where such measurements may be power-based or timing measurements of the D2R signal.

Proposal 24:	RAN1 to study and define ways to identify the conditions that affect the proximity determination outcome at least for D2T2 due to the involvement of intermediate node. Some of these conditions may be:

	Lack of AIOT response due to AIOT device being out of CW node’s range i.e., very large distance CW node – AIOT device or due to the lack of activation i.e., activation R2D signal is of insufficient strength.

	Failure to detect AIOT reply due to high levels of CW interference.

	Receiving a low power AIOT response due insufficient strength of the CW signal as observed by the AIOT device (due to large distance CW node – AIOT device).

# References

- R1-2407555, Final FL summary on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2401874, “FL summary (final) for Ambient IoT evaluation”