3GPP TSG RAN WG1 #119		R1-2410113

Orlando, US, November 18th – 22nd, 2024

Source:	OPPO

Title:	Discussion on LS on UE RF issues for Rel-19 MIMO enhancement

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In RAN4#112bis meeting, the potential UE RF issues for Rel-19 MIMO enhancement from RAN4 aspect have been discussed, and a LS including the identified questions was sent to RAN1 [1]:

1. Overall Description: RAN4 is discussing the ‘asymmetric DL sTRP/UL mTRP deployment scenario’ objective of the WI for FR2 deployments. In one proposed scenario, the UL TRP(s) have full UL reception functionality but downlink functionality limited to SSB transmission. Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID. RAN4 noticed that UL full power transmission mode 0 and legacy Rel-15 power scaling is supported in the WID for 3Tx. RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks as below for example but not with 1Layer 3-port codebooks. 13100100, 13100001, 13 001001. Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks. 2. Actions: To RAN1 ACTION: RAN4 respectfully asks RAN1 to provide feedback to the above questions. 3. Dates of next TSG RAN WG 4 meetings 3GPP RAN4#113 18 – 22 November 2024 Orlando, US 3GPP RAN4#114 17 – 21 February 2025 Athens, Greece

In this contribution, we provide our views on above questions identified by RAN4. A draft reply LS for some responses is provided in our companion contribution [2].

Discussion

Based on RAN1’s discussion on ‘asymmetric DL sTRP/UL mTRP deployment scenario’ and ‘3Tx’, some answers to RAN4’s questions can be provided.

On Question 1

In the justification part of the Rel-19 MIMO WID, it says that “As an option to further reduce energy consumption, the micro nodes can, for instance, reduce or even turn off DL transmissions.” From our understanding, it is up to gNB to reduce its DL transmissions, e.g. only SSB left, or turn off all the DL transmissions, i.e. behaving like an “UL-only” TRP. UE behavior can be dependent on NW’s configuration on PL offset(s). Given the current progress for asymmetric MTRP scenarios, there is no additional specification impact, if we present the following answer to RAN4.

Proposal 1: If the PL offset associated with one of the indicated joint/UL TCI state(s) is configured to a UE, the UE does not expect to receive SSB from UL TRP. Otherwise, it is possible for UE to receive SSB from UL TRP.

On Question 2

For UL full power transmission mode 0, full rated PAs on each Tx chain is supported and there is no power scaling for both 1 layer and 2 layers uplink transmission for 3Tx UE. Therefore, a 3Tx UE supporting UL full power transmission mode 0 should achieve the same maximum output power with both 1 Layer and 2 Layers.

Proposal 2: A 3Tx UE supporting UL full power transmission mode 0 should achieve the same maximum output power with both 1 Layer and 2 Layers.

On Question 3

For a 3Tx UE, non-coherent uplink codebooks for single-layer transmission and two-layer transmission as follows:

Single-layer transmission: 13100,13010,13001

Two-layer transmission: 13100100, 13100001, 13 001001

If UE report the capability to support full power transmission mode 0 and PUSCH-Config is set to fullpower, the power scaling factor is ‘1’ for uplink power control. For a 3Tx UE that supports UL full power transmission mode 0, UE should support full power transmission for both 2-Layer 3-port codebooks and 1-Layer 3-port codebooks.

Proposal 3: For a 3Tx UE that supports UL full power transmission mode 0, full power transmission should be supported for both 2-Layer 3-port codebooks and 1-Layer 3-port codebooks.

Conclusion

In this contribution, we discussed the RAN4 LS on UE RF issues for Rel-19 MIMO enhancement. The following proposals are made:

Proposal 1: If the PL offset associated with one of the indicated joint/UL TCI state(s) is configured to a UE, the UE does not expect to receive SSB from UL TRP. Otherwise, it is possible for UE to receive SSB from UL TRP.

Proposal 2: A 3Tx UE supporting UL full power transmission mode 0 should achieve the same maximum output power with both 1 Layer and 2 Layers.

Proposal 3: For a 3Tx UE that supports UL full power transmission mode 0, full power transmission should be supported for both 2-Layer 3-port codebooks and 1-Layer 3-port codebooks.

Reference

[1]       R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, RAN WG4.

[2]       R1-2410114, Draft reply LS on UE RF issues for Rel-19 MIMO enhancement, RAN1 #119 meeting.