3GPP TSG-RAN WG1 #64 	R1-110978

Taipei, Taiwan, 21st-25th, Jan, 2011

Agenda Item:	6.2.1

Source: 	ITRI

Title: 	Further simulation result for inter-cell interference of PUCCH Format 3

Document for:	Discussion and Decision

1	Introduction

Most of the link-level inter-cell interference mitigation simulations focus on single strong interferer case. Therefore, we think that it is necessary to simulate multiple inter-cell interferers in the link-level simulation to reflect more realistic scenarios. In this contribution, we evaluate the performance of PUCCH format 3 inter-cell interference randomization schemes proposed in [1] in the cases of multiple interferences.

2	Discussion

In this contribution, we evaluate several PUCCH format 3 inter-cell interference randomization schemes, i.e. different scrambling symbol and scrambling sequences in the settings of multiple inter-cell UEs. In [1], it was proposed that the cell specific scrambling in slot ns is performed according to

where Q = 2 or 4 corresponds to BPSK or QPSK scrambling symbol and the candidate scrambling sequences is generated as follow: [2], [3], [4]

Rule A [3]:  is determined based on the pseudo-random sequence and the initial seed is

(1)

where NIDcell denotes the cell ID.

Rule B [2, 4]:

(2)

where x =12 or 64.

Table 1 and Table 2 list the link-level simulation parameters and inter-cell interference assumptions. Figure 1 illustrates the detailed simulation scenario and the simulation results are shown in Figure 2. From the simulation results, the performance utilize QPSK scrambling symbols outperform the case of BPSK, and the scrambling sequence proposed in [2] outperform the others sequences.

Table 1: Link-level simulation assumptions

| Parameters | Value |
| --- | --- |
| Carrier Frequency | 2.0GHz |
| Bandwidth | 10MHz |
| Tx/Rx Antenna Configuration | 1 Tx / 2 Rx |
| Rx Antenna correlation | Uncorrelated |
| Channel Model | ETU, Doppler Frequency = 70Hz |
| Receiver Type | MMSE |
| CP Type | Normal CP |
| # of interference UEs | 5 inter-cell *Note: refer to Table 2 |
| Channel Estimation | Practical |
| A/N payload size | 8 bits |
| DTX detection/PFA | Disable |

Table 2: Inter-cell interference assumptions

|  | OCC index | Serving cell-ID | Relative power [dB] |
| --- | --- | --- | --- |
| Target signal | 0 | 0 | - |
| Interferer 1 | 0 | 1 | 0 |
| Interferer 2 | 1 | 1 | -5 |
| Interferer 3 | 2 | 1 | -10 |
| Interferer 4 | 3 | 1 | -15 |
| Interferer 5 | 4 | 1 | -20 |

Figure 1: Simulation scenario of multiple inter-cell interferences.

Figure 2: Simulation result

3	Conclusions

In this contribution, we compared the performance of the inter-cell interference mitigation scheme for PUCCH Format 3 proposed in [1] in the case of multiple inter-cell interferences. Based on the simulation results, the inter-cell randomization scheme proposed in [2] with QPSK scrambling symbol and provide the best performance -.

Reference:

[1]	R1-110327, Inter-Cell Randomization for PUCCH format 3, Qualcomm

[2]	R1-110534, Way forward on PUCCH format 3 randomization, InterDigital, Samsung, ETRI, Pantech, HTC, Qualcomm

[3]	R1-110538, Way forward on remaining details of PUCCH format 3 in 36.211, Ericsson, ST-Ericsson, CATT, Texas Instruments, LG Electronics, LG-Ericsson, CMCC

[4]	E-mail discussion on 63bis-04-LTE-A, PUCCH Format 3 randomisation