# 3GPP TSG RAN WG1 Meeting #74	 	R1-132982

# Barcelona, Spain, 19th – 23rd August 2013

# Title: 	Uplink Power Control Enhancement for Small Cell Enhancements

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	7.2.6.1.3

# Document for: 	Discussion/Decision

# Introduction

In the previous contribution [1] for RAN1#73, we have discussed the main challenges of the current fractional power control (FPC) algorithm, especially in the dense deployment of small cells and the scenarios with correlated shadowing.  It was suggested considering the PL to neighboring cells and/or the interference caused to neighboring cells into the UL power control mechanism for small cell enhancements, in order to overcome the inter-cell interference (ICI) issues.

In this contribution we investigate an interference-suppressed UL power control algorithm for small cell enhancements, taking the abovementioned two interference elements into account.

# An Interference-Suppressed Power Control Algorithm

This interference-suppressed power control (ISPC) algorithm considers both the useful signals’ power spectral density (PSD) at the serving cell and the level of ICI generated by a UE to one or more neighbouring cells.  Firstly, a common interference density target, Γ0, is set that all UEs within the serving cell need to comply to.  Secondly, the UE-specific interference density is compensated according to the aggregated pathlosses (PLs) to the selected neighbouring cell(s), with the goal of having each UE not exceeding the target amount of interference.  To trade off the useful signals’ PSD and ICI PSD for a certain UE, these two components are partially compensated by different scales, α1 and α2, respectively.

This UL power control algorithm could be expressed by,

PSD=minPSDmax,Γ0+α2∙PLI+α1∙PLS+∆MCS+δ

where PSD is the UE’s transmitted PSD, Γ0 is the target of received interference PSD generated by the UE, which is common for all UEs within the serving cell.  The interference PSD target is beneficial for the system IoT control.  PLI is the aggregated interference PL to the selected neighbouring cell(s), which is used to partially control the generated ICI to the system.  PLS is the PL from the desired UE to the serving cell, which indicates how the UL transmit power is related to the useful signal.  α1 and α2 are the compensation factors to the useful signal PL and the interference PL, respectively, which could trade off the effects of signal PL and interference PL on the UL transmit power.

The interference PL, PLI, could be calculated by aggregating the reciprocal of the PLs to individual interfered neighbouring cells, as given by,

PLI=-10∙log10i=1N10-110∙PLi

where N≤7, considering that in current system, the UE measures and reports the RSRP values from 7 neighboring cells in addition to the serving cell.  This means that, for a given average path loss to the interfered cells, the more cells the UE interferes with, the lower its transmit power will be.

The PL of respective neighboring cell, PLi, could be derived from the RSRP measurements as follows:

PLi=RSRPi-TxPoweri

For simplification purpose, we just consider the interference to 3 neighboring cells, i.e., setting N=3, under the assumption that the interference of the desired UE to these 3 closest neighboring cells dominates the most interference to the entire system.

In the following, we discuss two implementation solutions according to the location of interference PL calculation.

# Implementation Solution I – Calculating interference PL at serving cell

To simplify the complexity of UE behaviour, the aggregated interference PL, PLI, could be calculated at the serving cell while the serving cell PL estimation remains at UE as in the current system.  Since the interference PL is calculated at the serving cell, it is unnecessary to inform the DL transmit power of neighbouring cells to the UE.

The detailed procedures and operations of this solution are illustrated in Fig. 1.

Fig. 1  Procedures of ISPC Solution I

To minimize the impact on the specifications, we consider an implementation solution of this algorithm which preserves the conventional open-loop power control formula.  The interference PSD target, Γ0, could be configured by setting cell-specific P0 component, i.e, P0_Cell=Γ0.  The effect from the interference PL could be considered by setting UE-specific P0 component, i.e, P0_UE=α2∙PLI.  Since the interference PL spans from several tens to more than one hundred dB, the range of UE-specific P0 component, P0_UE, should be extended to the values up to several tens dB from the current range of 15dB.  Furthermore, in the FPC algorithm, the cell-specific P0 component, P0_Cell, is set as the desired received signal PSD; while in this new algorithm, P0_Cell  is set as the wanted ICI PSD, Γ0, so that they have significantly lower dynamic ranges.  Thus P0_Cell, may be extended due to its different utilization in the new UL PC mechanism.

# Implementation Solution II – Calculating interference PL at UE

In Solution I, the interference PL is updated every time there is a RSRP report, which may lead to frequent updates of P0_UE, and may also require RSRP reports to be sent more frequently than would otherwise be necessary.  In Solution II, the UE estimates the PL from the neighbouring cells directly.  The DL transmit power of the selected multiple neighbouring cells needs be signalled to the UE by introducing additional higher-layer signalling.  But these values are pretty static, and therefore seldom updated.

The detailed procedures and operations of Solution II are illustrated in Fig. 2.

Fig. 2  Procedures of ISPC Solution II

Note that in Solution II, the formula of open-loop power control component would be changed to involve the effect of the UE’s interference PL into the UL power control.

# Simulation Results

In this section, we provide some simulation results to demonstrate the performance advantages of the described UL interference-suppressed PC mechanism.  The simulation scenario is SCE Scenario 2a as described in [2].  Some additional assumptions and parameters are summarized in Table 2.

We investigate the performance of ISPC in SCE Scenario 2a with 1 cluster, in terms of IoT distribution and throughput performances, as shown in Fig. 3, Fig. 4 and Fig. 5, respectively.  In Fig. 3 and Fig. 4, the IoT distribution curves of ISPC has steeper slope than those of FPC, which demonstrates that the new mechanism ISPC has a better IoT control than the conventional FPC algorithm in SCE Scenario 2a.

Fig. 3  IoT distribution of Macro-Cell in SCE #2a (1cluster)

Fig. 4  IoT distribution of Small Cells in SCE #2a (1cluster)

Fig. 5  Normalized User Throughput Performance for SCE Scenario 2a (1 cluster)

Table 1  Gains of ISPC over FPC for SCE Scenario 2a (1 cluster)

|  | IoT level (dB) |  | Normalized User Throughput (bps/Hz/User) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Macro cell | Small cell | 5% | Gains | Mean | Gains |
| FPC, α=0.6, P0=-57dBm (Macro) P0=-63dBm (Small) | 7.0355 | 11.5488 | 0.0018 | – | 0.0452 | – |
| ISPC, α1=0.8 , α2=0.2 Γ0=-108dBm | 6.5751 | 11.2291 | 0.0028 | 55.6% | 0.0581 | 28.5% |

Table 1 compares the throughput performance of ISPC and FPC under the assumption of similar IoT levels.  With the given configuration, ISPC could achieve 55.6% cell-edge performance gain while 28.5% average throughput performance gain over FPC.

It is observed that the cell-edge performances of both ISPC and FPC are very low, which implicitly illustrates the coverage problem of SCE Scenario 2a with outdoor small-cell eNBs and 80% indoor UEs.

# Conclusions

In this contribution, we provide an Interference-Suppressed PC algorithm considering both the PL to neighbouring cells and the wanted interference PSD caused to neighbouring cells, in order to address the complicated interference situation in SCE.  One way in which it can be implemented is by just extending the dynamic range of P0 in current UL PC formula.  Our simulation results show that ISPC has significant performance gain over FPC in terms of both cell-edge and average throughput.  Therefore, we suggest,

- Proposal: The UL power control mechanism to take into account interference generated to non-serving cells.

# References

- R1-132059, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, “Small cell interference mitigation”, RAN1#73, May 2013.

- 3GPP TR36.872, “Small Cell Enhancements for E-UTRA and E-UTRAN – Physical Layer Aspects (Release 12)”.

# Annex

Table 2  Additional Simulation Assumptions and Parameters

| System Parameters | Network topology | - SCE Scenario 2a with 1 cluster/sector - SCE Scenario 2a with 2 clusters/sector |
| --- | --- | --- |
| Scheduler Parameters | Traffic model | Full buffer |
|  | UL Scheduler | PF |
|  | Scheduling granularity | 5 PRBs |
|  | UL link adaptation | Non-idea link adaption with 7ms delay |
|  | BLER target | 10% |
|  | UL HARQ | Synchronous HARQ with max. 4 transmissions for CC |
|  | UL receiver | MMSE |
| Channel Parameters | Propagation scenario | SCE Scenario 2a |
|  | ICI modeling | Explicitly |
|  | Channel estimation | ideal |
| Macro Cell | Macro cell layout | Hexagonal grid, 3 sectors/site, 7 Macro sites, wrap-around |
|  | Antenna configuration | 2 Rx antennas |
| Small Cell | Small cell layout | 4 small cells/cluster |
|  | Bias | 6dB |
|  | Antenna configuration | 2 Rx antennas |
| UE | UL transmission scheme | 1x2 SIMO |
|  | Antenna configuration | 1 Tx antenna (0 dBi antenna gain, omni-directional) |
| UL power control parameters | FPC | α=0.6 P0=-57dBm (Macro cell) P0=-63dBm (Small cell) |
|  | ISPC | α1=0.8, α2=1-α1 Γ0=-110dBm |
|  | CLPC | Not used |