3GPP TSG-RAN WG1 #75 	R1-135314

November 11 – 15, 2013

San Francisco, United States

Agenda item:	6.2.7.3

Source: 	Qualcomm Incorporated

Title: 	Initial calibration of channel models for elevation beamforming and FD-MIMO

Document for:	Discussion/Decision

# Introduction

In RAN1#74, working assumptions have been agreed for 3D channel modeling calibration.  As agreed in RAN1#73, the calibration of 3D channel models will be conducted in two phases.  In this contribution, we provide some Phase-1 calibration results of geometry, coupling loss, and elevation related parameters.

# Initial Calibration Results

To facilitate the evaluation of elevation beamforming and/or FD-MIMO, simulators need to be calibrated to ensure that they produce comparable results.  We follow the agreed working assumptions in [1][2] and focus on the first phase calibration.  Detailed simulation assumptions are listed in the Appendix.  We evaluate several cases with different antenna configurations as shown in Table 1.  For both 3D UMa and 3D UMi scenarios, we provide the coupling loss distributions, geometry distributions and the LOS elevation distributions.

Table 1  Antenna configuration for initial calibration.

| Configuration | Case A-DT96 | Case A-DT99 | Case A-DT102 | Case B | Case C |
| --- | --- | --- | --- | --- | --- |
| Antenna Model | AAS w/ K = M | AAS w/ K = M | AAS w/ K = M | AAS w/ K = 1 | Passive antenna |
| Downtilt θetilt | 96° | 99° | 102° | N/A | 102° |
| Feeder loss [dB] | 0 | 0 | 0 | 0 | 2 |

In Figure 1, we show the distribution of coupling loss.    It shows that UE suffers from signal power loss as the downtilt angle increasing from 96° to 102°.  As shown in Figure 2, while larger downtilt leads to more severe signal power loss, each cell may spray less interference power to its neighbor cells.  In turn, the geometry under large downtilt is superior.  In Figure 3, we provide the distribution of elevation angles of LOS to the serving cell.

# Conclusion

In summary, this contribution provides some initial calibration results of the 3D UMa and 3D UMi channel models.

# References

- R1-133803, Final report of 3GPP TSG RAN WG1 #73 v1.0.0.

- Draft report of 3GPP TSG RAN WG1 #74 v0.1.0.

- 3GPP TR 36.814, “Further advancements for E-UTRA physical layer aspects.”

| - 3D UMa. | - 3D UMi. |
| --- | --- |

Figure 1 Distribution of coupling loss.

| - 3D UMa. | - 3D UMi. |
| --- | --- |

Figure 2 Distribution of geometry.

| - 3D UMa. | - 3D UMi. |
| --- | --- |

Figure 3 Distribution of LOS EoD.

# Appendix

Table 2  Parameters for initial calibration.

| Deployment Scenario | 3D UMa | 3D UMi |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 micro sites,3 sectors per site |  |
| BS antenna height | 25 m | 10m |
| Min. UE-eNB distance | 35 m | 10m |
| Total BS Tx power | 46 dBm for 10 MHz | 41 dBm for 10 MHz |
| Carrier frequency | 2 GHz |  |
| Indoor UE fraction | 80% |  |
| UE dropping and height model | UEs are uniformly distributed in the cell. The UE antenna height is modelled as hUE = 3(nfl – 1) + 1.5 m. - For outdoor UEs, nfl = 1; - For indoor UEs, nfl is uniformly distributed in {1, 2, …, x}, where x is the number of floors. - The number of floors is uniformly distributed with an average and variation range; - Average number of floors is 6; - Variation range is {-2, -1, 0, 1, 2}. |  |
| UE attachment | Based on LOS direction only |  |
| LOS probability | Pr3D-UMa-LOS(d2D,hUT)=(1+C(d2D,hUT))PrITU-UMa-LOS(d2D), C(d2D,hUT) is defined in R1-133880. | Reuse ITU LOS probability with 2D distance. |
| Environment height hUE | 1m w/ prob. Pr(d, hUT) = (1+C(d2D, hUT))– 1; hUE~U{12, 15, …, hUE – 1.5} w/ prob. 1–Pr(d2D, hUT) | 1m |
| Path loss model for outdoor UEs | Reuse ITU path loss models with 3D distance. The LOS path loss formula after breakpoint distance is modified according to R1-133899. |  |
| Path loss model for indoor UEs | PL(d) = PLb(d) + PLtw + PLin, PLb (d) = PLITU-LOS (d), for LOS, the formula after breakpoint distance is modified according to R1-133899. PLb (d) = max{PLITU-NLOS (d, 1.5) – α (hUE – 1.5), PLITU- LOS (d, hUE)), for NLOS, where d is 3D distance in meters between the eNB and the UE; PLtw = 20 dB is the penetration loss, PLin = 0.5 din, din = Uniform(0, min(d2D, 25)) is the indoor path loss; PLITU- LOS (d, hUE) is the LOS path loss model for ITU-UMa or ITU-UMi with hUT = hUE; PLITU- NLOS (d, hUE) is the NLOS path loss model for ITU-UMa or ITU-UMi with hUT = hUE; α is the scaling factor for UE height dependent loss, α = 0.6 for 3D-UMa and α = 0.3 for 3D-UMi. For LOS, the breakpoint distance is a 2D distance. |  |
| eNB antenna pattern | - For active antenna system (AAS), the element pattern is defined as follows. - Combined pattern: AE(φ, θ) = GE, max – min{– [AE, H (φ) + AE, V (θ)], Am}, GE, max = 8 dBi; - Horizontal pattern: AE, H (φ) = – min{12(φ / φ3dB)2, Am} dB, φ3dB = 65°, Am = 30 dB; - Vertical pattern: AE, V (θ) = – min{12[(θ – 90°)2 / θ3dB)2, SLAv} dB, θ3dB = 65°, SLAv = 30 dB. - The antenna port weighting is wm = K–1/2 exp{j2π (m – 1)dv cosθ etilt}, m = 1, 2, …, K, where K in {1, M}, M = 10, θetilt is the downtilt, dv = 0.5 λ, 0.8 λ is the vertical element spacing. - For passive antennas, the antenna pattern is defined as follows. - Combined pattern: A (φ, θ) = G0 – min{– [AH (φ) + AV (θ)], Am}, G0 = 17 dBi; - Horizontal pattern: AH (φ) = – min{12(φ / φ3dB)2, Am} dB, φ3dB = 70°, Am = 20 dB; - Vertical pattern: AV (θ) = – min{12[(θ – θetilt) / θ3dB]2, SLAv} dB, θ3dB = 10°, SLAv = 20 dB. |  |