3GPP TSG-RAN WG1 #75 	      R1-135426

San Francisco, USA, 11th -15th November 2013

Source:     MediaTek Inc.

Title:      Functionality and coverage analysis of PUCCH

Agenda Item:    6.2.2.2.3

Document for:   Discussion and Decision

# Introduction

Up to now, there is no conclusion about the need of coverage improvement of PUCCH. Based on the functionality analysis of PUCCH, this contribution presents link level simulation of PUCCH coverage improvement, and gives our view whether PUCCH should be supported for coverage enhanced mode MTC devices.

# Functionality Analysis

The functionality of PUCCH includes:

1. Scheduling request (SR)

In the connected mode, UE should send SR in PUCCH for UL data transmission if SR resources are configured. Otherwise, UE should perform random access to request UL grant. However, the 4-step contention based random access may introduce larger signaling overhead.  For example, the signaling overhead of Msg 2 and Msg 4 is 7 bytes and 20 bytes, respectively. To improve the coverage for MTC UEs within coverage hole, much more resources will be needed, compared to SR transmission. Further, the collision probability of random access will increase significantly if there is a large number of MTC UEs or frequent UL traffic, resulting in a larger delay to UL resource request, if SR is replaced by random access.

Observation #1: SR should be supported for MTC devices in a coverage-enhanced operation.

2. ACK/NACK feedback

There were many discussions on possible removal of HARQ and PUCCH, and one alternative solution is RLC layer ARQ [1]. However, RLC ARQ is triggered only after reaching the maximal number of HARQ transmission for normal UEs, according to current specification.  Then, specification effort will be needed to apply RLC ARQ for MTC UEs in bad coverage, if HARQ is removed. Furthermore, RLC ARQ cannot contribute to coverage improvement since no redundancy gain can be obtained under RLC ARQ. Then, it’s expected that more repetition number is needed for PDSCH if RLC ARQ is adopted.

If HARQ is transmitted in another channel such as PUSCH (only ACK/NACK bit plus CRS), it needs more resource overhead to achieve comparable performance of PUCCH format 1a/1b, since more repetition number is needed for PUSCH with larger payload. Table 1 compares the resource overhead and ACK/NACK delay under the case that ACK/NACK is transmitted in PUCCH format 1a and PUSCH. For PUSCH, 1 bit payload with 24-bit CRC is assumed. The analysis is based on references to the evaluation in TR 36.888[1] and simulation results in next subsection.

Observation #2: DL HARQ ACK/NACK feedback in PUCCH should be supported for MTC devices in a coverage-enhanced operation.

Table 1   Resource overhead and delay comparison between PUCCH 1a and PUSCH

| Physical channel name | PUCCH | PUSCH |
| --- | --- | --- |
| Maximum Coupling Loss for FDD (MCL) (dB) | 147.2 | 140.7 |
| Required improvement (dB) | 8.5 | 15 |
| Repetition number | ~25 | >100 |
| Resource overhead (RE) | ~2400 | >14400 |
| ACK/NACK delay (ms) | ~25 | >100 |

3. CSI feedback

The main purpose for CSI feedback, including PMI, RI and CQI, is for link adaptation to allow eNB to schedule appropriate MCS/RI to UEs based on the fast fading conditions. However, the benefits may be limited for MTC UEs in a bad coverage as long-term pathloss dominates the performance. Diversity transmission will be appropriate for MTC UEs to guarantee the performance. Hence, RI feedback is not required. Similarly, PMI is also not necessary since closed loop precoding is not applicable under numerous repetitions of feedback. Further, eNB can select different precoders for each repetition. Since MTC devices in coverage hole is in low mobility or stationary, the channel state is expected to vary very slowly. Therefore, CQI in UCI is not necessary. From this perspective, for MTC operation in coverage enhanced mode, enhancement of PUCCH format 1a or 1b can be enough.

Observation #3:  CSI feedback in PUCCH is not required for MTC UEs.

From the above observations, the following proposal is made

Proposal #1: The functionality of PUCCH SR and DL HARQ ACK/NACK feedback should be supported by MTC UEs in coverage enhancement mode.

Proposal #2: PUCCH format 1a or 1b could be enhanced for MTC UE in coverage enhancement mode.

# Simulation Results

Figure 1 shows PUCCH format 1a performance with different numbers of repetitions. It can be observed that 8.5dB coverage improvement for PUCCH 1a can be achieved by applying ~25 times repetitions.

Observation #4: Coverage improvement of 8.5dB for PUCCH 1a can be achieved by around 25 repetitions in time domain, to meet the target of 15dB network coverage improvement.

Figure 1  PUCCH format 1a performance vs SNR with different repetition numbers

Table 2. Link-level simulation parameters for PUCCH

| Parameter | Value |
| --- | --- |
| System | 10 MHz, FDD, 2.0 GHz |
| Physical Channel | PUCCH format1a |
| Antenna Configuration | 1x2 low correlation |
| Channel Model | EPA |
| Doppler Shift | 1Hz |
| Frequency Error | 100 Hz |
| Channel Estimation | Realistic channel estimator |
| Performance Target | 1% BER |

# Conclusion

This paper discussed the functionality of PUCCH, and presents some evaluation results based on repeated transmission for coverage improvement. Based on the simulation results and theoretical analysis, we have the following observations and proposals.

Observation #1: SR should be supported for MTC devices in a coverage-enhanced operation.

Observation #2: DL HARQ ACK/NACK feedback in PUCCH should be supported for MTC devices in a coverage-enhanced operation.

Observation #3:  CSI feedback in PUCCH is not required for MTC UEs.

Observation #4: Coverage improvement of 8.5dB for PUCCH 1a can be achieved by around 25 repetitions in time domain, to meet the target of 15dB network coverage improvement.

Proposal #1: The functionality of PUCCH SR and DL HARQ ACK/NACK feedback should be supported by MTC UEs in coverage enhancement mode.

Proposal #2: PUCCH format 1a or 1b could be enhanced for MTC UE in coverage enhancement mode.

# References

- R1-114203, “Discussion on the necessity of HARQ for low cost MTC LTE UE”, III, Coiler Corporation, November, 2011

- 3GPPTR 36. 888, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on provision of low-cost MTC UEs based on LTE, V2.1.0, 2013-05.