3GPP TSG RAN WG1 #82	R1- 153844

August 24th  – 28th, 2015

Beijing, China

Agenda item:	7.2.4.1

Source: 	Cisco Systems

Title: 	Uplink Channel Access for LAA

Document for:	Discussion and Decision

# Introduction

Following RP-68, a work item for LAA [1] was agreed with the intent of specifying support for only DL transmissions with the understanding that UL channel access principles would be agreed but not specified (as given below).

The work item should only specify support for LAA SCells operating with only DL transmissions. When specifying support for LAA SCells with only DL transmission, the following for the UL should be agreed (but not specified): the principles of UL channel access and the necessary forward compatibility mechanism so that the UL for LAA SCells  can be added in future release without modifications to the DL design.

This contribution outlines Cisco Systems’ views on the channel access procedures for uplink LAA operation in unlicensed spectrum. A companion contribution [2] covers the downlink channel access procedures for LAA.

# Self-Scheduling and Cross-Carrier Scheduling

Four possible scheduling combinations for an LAA SCell have been identified as given below.

- Combination 1: Self-scheduling on both DL and UL

- Combination 2: Self-scheduling on DL and cross-carrier scheduling on UL

- Combination 3: Cross-carrier scheduling on DL and self-scheduling on UL

- Combination 4: Cross-carrier scheduling from the same scheduling carrier for both DL and UL

Combinations 1 and 4 are already supported in Rel-12. With dynamic self-scheduling, uplink channel access potentially requires LBT at both eNB and UE.  The reason is two-fold: First, the eNB has to perform LBT prior to sending the UL grant over an LAA Scell. Second, if the scheduling command has been successfully received at the UE, the UE has to perform LBT prior to transmitting in UL. Thus, the average uplink latency is expected to be larger than the average downlink latency. To reduce the scheduling latency on UL in case of dynamic SCell UL scheduling, it would be useful to support Combination 2 as well.

Proposal 1: Support Combination 2 for minimizing latency for UL LAA transmissions.

# Uplink LBT

In RAN1 #81, the following was agreed:

- LAA supports UL LBT at the UE.

    - The UL LBT scheme can be different from the DL LBT scheme (e.g. by using different LBT mechanisms or parameters) e.g., since the LAA UL is based on scheduled access which affects a UE’s channel contention opportunities

      - Other considerations including multiplexing of multiple UEs in a single subframe

        - Possibly other considerations

In view of above, options such as the eNB performing LBT on behalf of the UE are ruled out. Following options can be considered:

Option A: LBT over uplink employs Load Based Equipment (LBE) similar to 3GPP Category 4 Channel Access for DL.

Option B: LBT operation employs Frame Based Equipment (FBE).

Our preference is adopt Option A, the rationale being that it is preferable for principles of uplink channel access for LAA to be as close to Wi-Fi’s EDCA Channel Access which applies for channel access at both AP and non-AP STAs.

Proposal 2: Adopt an LBE uplink channel access, identical to the 3GPP Category 4 channel access on DL. Seek IEEE input in case a different channel access principle is considered on UL, relative to the 3GPP LBT Category 4.

To minimize hidden stations, and to ensure that UE’s uplink power control does not negatively impact transmissions at neighbour Wi-Fi BSS, it is desirable that the UE’s listening range is large enough to minimize the UE’s interference impact at Wi-Fi devices.

Proposal 3: Adopt a configurable energy detection threshold (EdThresh) such that an UE’s energy detection thresholds is chosen according to the following rules:

- EdThresh is no larger than -77 dBm if pure energy detection is employed.

- EdThresh is no larger than -62 dBm if the UE has the capability to perform Wi-Fi preamble detection at <= -82 dBm

# PUSCH Starting Offset

Figure 2: PUSCH Segments or sub-subframes to allow for PUSCH transmissions to start within a subframe

Assuming Load Based Equipment (LBT) channel access on UL, the completion of the LBT may not be aligned with an uplink subframe boundary. The UE transmits a reservation signal (following contention backoff during ECCA procedure), which is used to reserve the channel, prior to an impending PUSCH transmission.

To ensure maximum usage of a transmission opportunity and to minimize reservation signal transmissions solely for reserving the medium, PUSCH transmissions should be allowed to start within a subframe. For example, as shown in Figure 2, the Rel-8 subframe may be divided into 4 “sub subframes” or subframe segments.

Proposal 4: Consider reservation signal transmission on uplink, following successful completion of LBT at the UE, to indicate to the eNB the start of its PUSCH transmission. The reservation signal is transmitted on n >= 1 OFDM symbols until the start of the next OFDM segment.

Proposal 5: Divide UL subframe into K segments each of which is a valid offset for PUSCH transmission.

One issue we foresee is with partial uplink sub frames is that when multiple UEs employ an LBT based CCA procedure, the UE that completes its backoff will transmit on the earliest PUSCH segments. This transmission will block a second UE’s transmission at the same time due to CCA indicating medium busy at that second UE. This results in poor scheduling efficiency and increases detection complexity at the eNB.

Some contributions [6] propose that in order to minimize inter-user blocking, all UEs begin CCA at the same time, and if successful, they begin to transmit at the same time. In our view, such behaviour is hard to realize in practice since per Rel-12, different UEs transmit at different times on UL due to timing advance.

We propose a simple probabilistic channel access mechanism to minimize inter-user blocking due to LBT. When a UE gains channel access, it transmits probabilistically based generating a binomial random variable with parameter p, the access probability. By choosing the value of the access probability p appropriately, a UE experiencing smaller interference does not persistently block other UEs from transmitting on uplink. Since the eNB does not have access to the interference levels at the UE, the simplest way would be to specify the UE behavior in terms of the mechanism for adapting p. The details of how the parameter p is chosen can be left for further study.

Proposal 6: Consider a probabilistic channel access for uplink to minimize inter-user blocking due to CCA.

# Scheduling for LAA Uplink

As discussed in earlier section, cross-carrier scheduling on uplink is one way to address the unpredictable latency between the time of scheduling grant and time of channel access at the UE. Because cross-carrier scheduling can result in significant control overhead on licensed carrier, there are two alternatives for 3GPP to consider in terms of self-scheduled SCell channel access:

- Rel-8 Dynamic Scheduling with Asynchronous UL H-ARQ

- UE autonomous uplink transmissions (similar to Wi-Fi).

# Dynamic Scheduling

In unlicensed spectrum, considering Wi-Fi can access the medium in either direction at any given time, having a fixed frame structure would put LAA at a competitive disadvantage. With dynamic scheduling, it is preferable therefore, not to constrain a priori a given subframe within a radio frame to be either a DL subframe or UL subframe, but rather make it demand driven based on the amount of traffic over DL/UL. Similar views have been presented in [7].

Proposal 7: Do not tie LAA uplink channel access to an existing LTE frame structure. With centralized LAA scheduling, the eNB should be able to dynamically modify the direction of channel access based on relative DL/UL traffic loads.

With dynamic scheduling, if the eNB’s scheduling grant cannot be respected by the UE due to the channel being busy, it is preferable that the UE simply drops the UL grant and waits until the next scheduling assignment. This simplifies the scheduler complexity at the eNB. If the UE is able to transmit yet the uplink transmissions are not decoded at the eNB, the eNB can send a subsequent UL grant indicating the HARQ-ID and redundancy version for the retransmission.

Proposal 8: To minimize increase in eNB scheduler complexity due to LBT at the UE, we propose the following UE behaviour:  a UE should drop the UL grant if it detects that the medium is busy during an indicated UL subframe.

# Distributed Scheduling (UE Autonomous Uplink Transmissions)

With dynamically scheduled transmissions, the eNB does not have a clear visibility in terms of the interference conditions at the UE. As a result, with LBE based channel access, the latency between grant reception time and uplink transmission can be variable.

As an alternative to centralized scheduling, we argue that a distributed scheduling approach, similar to Wi-Fi, makes a more compelling argument for LAA uplink. With distributed (UE autonomous UL transmissions), the UE accesses the channel without requiring a dynamic uplink grant from the eNB. Detection at the eNB is assisted using a combination of the uplink reservation signal and restriction of the UE’s transmission to within a set of PUSCH starting offsets. The frequency domain resources and MCS for a given UE’s transmissions may be assigned to the UE on a slow time-scale basis (e.g. via RRC signalling).

Proposal 9: Consider distributed (UE driven) uplink transmissions for LAA, identical to Wi-Fi’s channel access, for minimizing latency of channel access in uplink.

Figure 3 shows one such example of how UE autonomous access would occur. Each UE would be configured for transmission on eligible UL subframes on a set of frequency resources. Depending on the aggregate traffic load (obtained via BSR), the set of configured UL resources at different UEs may be non-overlapping (to allow for FDM-ed UL transmissions) in case of high load, or overlapping (to achieve statistical multiplexing gain) in low load scenarios. The UE performs CCA, upon completion of defer time plus contention back off, transmits a reservation signal and starts its UL transmission.

Figure 3: UE Autonomous Uplink Transmissions

# Conclusion

This contribution has highlighted Cisco Systems’ preferences in terms of uplink channel access for LAA. Our proposals are summarized below.

Proposal 1: Support Combination 2 for minimizing latency for UL LAA transmissions.

Proposal 2: Adopt a LBE uplink channel access, identical to the 3GPP LBT Category 4 channel access on DL. Seek IEEE input in case a different channel access principle is considered on UL, relative to the 3GPP LBT Category 4.

Proposal 3: Adopt a configurable energy detection threshold (EdThresh) such that an UE’s energy detection thresholds is chosen according to the following rules:

- EdThresh is no larger than -77 dBm if pure energy detection is employed.

- EdThresh is no larger than -62 dBm if the UE has the capability to perform Wi-Fi preamble detection at <= -82 dBm

Proposal 4: Consider reservation signal transmission on uplink, following successful completion of LBT at the UE, to indicate to the eNB the start of its PUSCH transmission. The reservation signal is transmitted on n >= 1 OFDM symbols until the start of the next OFDM segment.

Proposal 5: Divide UL subframe into K segments each of which is a valid offset for PUSCH transmission.

Proposal 6: Consider a probabilistic channel access for uplink to minimize inter-user blocking due to CCA.

Proposal 7: Do not tie LAA uplink channel access to an existing LTE frame structure. With centralized LAA scheduling, the eNB should be able to dynamically modify the direction of channel access based on relative DL/UL traffic loads.

Proposal 8: To minimize increase in eNB scheduler complexity due to LBT at the UE, we propose the following UE behaviour:  a UE should drop the UL grant if it detects that the medium is busy during an indicated UL subframe.

Proposal 9: Consider distributed (UE driven) uplink transmissions for LAA, identical to Wi-Fi’s channel access, for minimizing latency of channel access in uplink.

# References

- RP-151045, Work Item on Licensed-Assisted Access to Unlicensed Spectrum

- R1-153844, Downlink Channel Access for LAA, Cisco Systems

- ETSI EN 301893 V1.8.0, Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE directive

- R1-152735, LBT Operation for LAA UL, LG Electronics

- R1-152790, Uplink Waveform for LAA, Qualcomm

- R1-153143, UL Transmission for LAA, InterDigital Communications

- R1-153132, On Dynamic DL/UL Scheduling, Ericsson