3GPP TSG RAN1 Meeting #83								R1-157391

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 	Ericsson

Title:	NB-IoT – Exception report latency comparison for standalone operation

Agenda Item:	6.2.6.1.1

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives for the IoT device to meet is the latency target of 10 ms for an MAR exception report. Even if a possible down selection of DL and UL numerology could be seen as independent evaluations, in order to estimate the latency, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we will compare the latency of a NB-IoT system as described in [2] with the procedures and latency evaluation in [3], in the following called NB-LTE, to the NB-CIoT system as described in [1]. For NB-LTE, the difference to the analysis in [3] is an update using the latest results for cell search found in [4].

# Comparison

## Total time to send exception report 90 % confidence

The time required to successfully deliver an exception report with 90% confidence is given in Table 1 as the initial message BLER for UL report is less than 10% where the results for NB-LTE are taken from [3] and for NB-CIOT from [1].

Table 1 Exception report delivery time with 90% confidence

| Coupling loss (dB) | 144 |  |  | 154 |  |  | 164 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System | NB-LTE | NB-CIoT | Delta | NB-LTE | NB-CIoT | Delta | NB-LTE | NB-CIoT | Delta |
| Tsync(ms) | 250 | 480 | 230 | 270 | 480 | 210 | 370 | 960 | 590 |
| TMIB(ms) | 151 | 170 | 19 | 151 | 170 | 19 | 631 | 810 | 179 |
| TPRACH(ms) | 324 | 84 | -240 | 688 | 84 | -604 | 1440 | 644 | -796 |
| TFREE PRB(ms) | 580 |  | -580 | 580 |  | -580 | 580 |  | -580 |
| TRAmsg2-4 (ms) | 42 |  | -42 | 128 |  | -128 | 480 |  | -480 |
| TULgrant(ms) | 48 | 345 | 297 | 45 | 995 | 950 | 49 | 1090 | 1041 |
| TULdata(ms) | 39 | 80 | 41 | 553 | 500 | -53 | 1923 | 2780 | 857 |
| Total time (ms) | 1434 | 1159 | -275 | 2415 | 2229 | -186 | 5473 | 6284 | 811 |

## Total time to send exception report 99 % confidence

In case the exception report needs to be re-transmitted, i.e., a negative acknowledgment is received and normal operation is that the network sends an UL grant for the terminal to re-transmit the exception report. This achieves 99% confidence of the successful delivery of the exception report, as less than 10% BLER is assumed for each transmission. Table 2 summarizes the transmission times, taken from [3] for NB-LTE and for NB-CIoT from [1].

For the 164 dB MCL case, the results for NB-CIoT in [1] are based on having 99% BLER already in the first transmission. For direct comparison, the same has been assumed for NB-LTE.

Table 2 Exception report delivery time with 99% confidence

| Coupling loss (dB) | 144 |  |  | 154 |  |  | 164 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System | NB-LTE | NB-CIoT | Delta | NB-LTE | NB-CIoT | Delta | NB-LTE | NB-CIoT | Delta |
| Tsync(ms) | 250 | 480 | 230 | 270 | 480 | 210 | 370 | 960 | 590 |
| TMIB(ms) | 151 | 170 | 19 | 151 | 170 | 19 | 631 | 810 | 179 |
| TPRACH(ms) | 324 | 84 | -240 | 688 | 84 | -604 | 1440 | 644 | -796 |
| TFREE PRB(ms) | 580 |  | -580 | 580 |  | -580 | 580 |  | -580 |
| TRAmsg2-4 (ms) | 42 |  | -42 | 128 |  | -128 | 480 |  | -480 |
| TULgrant(ms) | 48 | 345 | 297 | 45 | 995 | 950 | 49 | 1090 | 1041 |
| TULdata(ms) | 39 | 80 | 41 | 553 | 500 | -53 | 2300 | 2780 | 857 |
| TAck(ms) | 41 | 345 | 304 | 47 | 995 | 948 |  |  |  |
| TULdata(ms) | 39 | 80 | 41 | 553 | 500 | -53 |  |  |  |
| Total time (ms) | 1514 | 1584 | 70 | 3015 | 3724 | 709 | 6000 | 6284 | 284 |

As mentioned above, for 164 dB MCL, 99% confidence is achieved using an initial BLER of 1%. In Table 3, we show the latency for NB-LTE with initial target BLER of 10%. The 10 s latency requirement is still met.

Table 3 Exception report with 99% confidence for NB-LTE with initial BLER 10% for 164 dB MCL

| Delay | 164 dB MCL |
| --- | --- |
| Tsync(ms) | 370 |
| TMIB(ms) | 631 |
| TPRACH(ms) | 1440 |
| TFREE PRB(ms) | 580 |
| TRAmsg2-4 (ms) | 480 |
| TULgrant(ms) | 49 |
| TULdata(ms) | 1923 |
| TAck(ms) | 77 |
| TULdata(ms) | 1923 |
| Total time (ms) | 7473 |

Based on these results we make the following observation:

Observation 1:

- NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes

Observation 2:

- For 99% confidence level, NB-LTE can have lower latency for an exception report than NB-CIoT for all coverage classes

Observation 3:

- NB-LTE can meet the 10 s latency target for MCL 164 dB even with retransmissions

# Conclusions

This contribution presents the latency evaluation of delivering the exception report in NB-LTE. The following observations were made:

Observation 1:

- NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes

Observation 2:

- For 99% confidence level, NB-LTE can have lower latency for an exception report than NB-CIoT for all coverage classes

Observation 3:

- NB-LTE can meet the 10 s latency target for MCL 164 dB even with retransmissions

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156027, “NB-IoT – Exception report latency evaluation”, Ericsson, ZTE, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, Samsung, LGE

R1-157418, “NB-IoT – Improved Synchronization Channel Design”, Ericsson