3GPP TSG RAN WG1 Meeting #86bis                             R1-1608685

# Lisbon, Portugal 10th - 14th October 2016

# Source:        ZTE Corporation, ZTE Microelectronics

Title:          Port layout and Class A codebook design for 20-32 ports

Agenda Item:   7.2.2.1.4

Document for:  Discussion and Decision

- Introduction

In RAN#71, the WID [1] about enhancements on Full-Dimension (FD) MIMO for LTE has been approved. The work item aims to specify the enhancements identified for utilizing both elevation and azimuth domains with 1D and 2D port layouts with cross-poles at eNBs. In RAN #86, no agreement achieved in terms of the supported antenna layout in Rel.14.

In this contribution, we discuss possible antenna layouts and respective oversampling factors for the newly supported number of antenna ports.

- Antenna layout and oversampling restrictions

As the number of antenna ports is increased to {20, 24, 28, 32}, the number of possible (N1, N2) combinations is up to 19. Moreover, different combinations of (O1, O2) further make the spec workload extremely high. Thus it is necessary to perform down-selection on (N1, N2) and/or (O1, O2).

- Restriction on (N1, N2)

N1 and N2 are the number of antenna ports for horizontal and vertical per polarization respectively. We start from the analysis and evaluation for 1D layout. From the aspect of codebook design, (N1,N2) =(N,1) and (N1,N2)=(1,N) is the same. Further we conduct a simulation to compare the performance of 1D layout and 2D layout for 32 ports with 2x1 virtualization. The simulation results are collected in Table 1.

Table 1 Performance comparison between 1D and 2D layouts

| Umi scenario |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | RU | Mean | 5% | 50% |
| (4,4,4,4) config 2 | 0.61 | 25.56 | 7.49(0%) | 24.53 |
| (8,2,8,4) config 2 | 0.57 | 24.38 | 6.43 | 23.26 |
| (16,1,8, -) | 0.68 | 24.24 | 5.37(-28.3%) | 22.03 |
| Uma scenario |  |  |  |  |
|  | RU | Mean | 5% | 50% |
| (4,4,4,4) config 2 | 0.60 | 24.52 | 8.07 | 23.76 |
| (8,2,8,4) config 2 | 0.54 | 24.52 | 8.29(0%) | 23.80 |
| (16,1,8, -) | 0.58 | 25.96 | 8.43(+1.7%) | 25.44 |

It can be observed that the 1D layout provide marginal performance gain compared with (N1, N2) =(4,4) only in UMa scenario. Wheras severe performance loss can be seen in UMi scenario, especially the cell edge performance. This performance loss comes from the fact that the 1D layout may not be the optimal solution to match the traffic distribution on the horizontal and vertical dimensions. Moreover, for the 1D layout, the formed beam is very narrow in the horizontal beam. Hence it may cause robustness issues for the cases with time/frequency varying channels, e.g., UE mobility. On the other hand, not all the possible 1D antenna layouts are supported in Rel.13. Hence considering the simplicity of specification and implementation complexity, there is no need to support all the possible 1D antenna layouts in Rel.14.

Some companies have the concern that the support of both N1>=N2 and N1<N2 antenna layout will lead to excessive codebook definition in specification. In fact, what makes the specification spends up to 13 pages to define the codebook is that we have to define the codebook for different codebook configurations and different layers, as well as addressing some 1D layout special cases. The relationship between N1 and N2 just impacts the master beam group definition which will not increase specification work.

Observation 1: The 1D antenna layout does not provide significant performance gain over 2D antenna layout.

Observation 2: Not all the possible 1D layouts are supported in Rel.13.

Observation 3: It is various codebook configurations and various layers causing excessive codebook definition in specification. The relationship between N1 and N2 only impacts the master beam group definition.

- Restriction on (O1, O2)

In Rel-13, the possible oversampling factors are (4, 4), (8, 4) and (8, 8). We simulate these three types of oversampling factors for different layouts and scenarios. The simulation results are collected in Table 2-4.

Table 2 Performance comparison of different oversample factors with (N1, N2) = (4, 4)

| MU scenario, FTP service,, lambda = 4.4, K=2, config 2 (N1, N2) = (4, 4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (O1, O2) | RU | Mean | 5% | 50% |
| 3D-UMi | (8, 4) | 0.62 | 25.11 | 7.42 | 24.66 |
|  | (4, 4) | 0.63 | -1.4% | -3.8% | -1.8% |
|  | (8, 8) | 0.62 | +0.6% | +2.4% | -0.8% |
| 3D-UMa | (8, 4) | 0.61 | 24.12 | 8.39 | 23.72 |
|  | (4, 4) | 0.63 | -2.2% | +0.8% | -4.2% |
|  | (8, 8) | 0.61 | -0.3% | -3.7% | -2.7% |

Table 3 Performance comparison of different oversample factors with (N1, N2) = (8, 2)

| MU scenario, FTP service,, lambda = 4.4, K=4, config 1 (N1, N2) = (8, 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (O1, O2) | RU | Mean | 5% | 50% |
| 3D-Umi | (8, 4) | 0.57 | 27.73 | 9.18 | 27.22 |
|  | (4, 4) | 0.58 | -0.4% | -0% | +0.4% |
|  | (8, 8) | 0.57 | +0.3% | +1.6% | -0.4% |
| 3D-UMa | (8, 4) | 0.54 | 27.97 | 10.87 | 28.35 |
|  | (4, 4) | 0.54 | -0.5% | -4.0% | -0% |
|  | (8, 8) | 0.54 | +0.2% | -4.3% | -0.2% |

Table 4 Performance comparison of different oversample factors with (N1, N2) = (2, 8)

| MU scenario, FTP service,, lambda = 4.4, K=1, config 1 (N1, N2) = (2, 8) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (O1, O2) | RU | Mean | 5% | 50% |
| 3D-Umi | (8, 4) | 0.51 | 26.44 | 9.03 | 26.61 |
|  | (4, 4) | 0.51 | -1.1% | -4.1% | -2.9% |
|  | (8, 8) | 0.51 | +0.2% | +1.6% | -0.9% |
| 3D-UMa | (8, 4) | 0.57 | 22.85 | 8.33 | 21.04 |
|  | (4, 4) | 0.58 | -1.3% | -7.7% | -0.6% |
|  | (8, 8) | 0.56 | +0.4% | +0.2% | +2.1% |

It can be observed from these evaluation results that:

- In Umi scenario, the oversampling factor combination (8, 8) performs the best, but at most 2.4% cell-edge performance gain compared with (8, 4), and even more marginal gains in mean performance.

- In Uma scenario, O2 = 4 is enough to have good performance, O2 = 8 shows performance loss in some cases.

From above observations, we can conclude that (O1, O2) = (8, 8) can be excluded since no performance gain obtained. In addition, larger oversampling factor will cause larger feedback overheads and increase UE complexity. Moreover, (4, 4) and (8, 4) should be applied to all the 2D layouts, such that the port number does not need to be re-indexed when the network changes the oversampling factors.

Based on the above considerations, we have the following proposal:

Proposal: The following antenna layouts and oversampling factors should be supported.

| Number of CSI-RS antenna ports | (N1, N2) | (O1, O2) |
| --- | --- | --- |
| 20 ports | (5, 2), (2, 5) | (4, 4), (8, 4) |
| 24 ports | (6, 2), (2, 6), (4, 3), (3, 4) |  |
| 28 ports | (7, 2), (2, 7) |  |
| 32 ports | (4, 4), (8, 2), (2, 8) |  |

- Conclusions

In this contribution, we analyze the complexity and performance problem of extending the Rel.13 codebook parameters to Rel.14. Based on above discussion and simulation results, we have the observations and proposal as follows.

Observation 1: The 1D antenna layout does not provide significant performance gain over 2D antenna layout.

Observation 2: Not all the possible 1D layouts are supported in Rel.13.

Observation 3: It is various codebook configurations and various layers causing excessive codebook definition in specification. The relationship between N1 and N2 only impacts the master beam group definition.

Proposal: The following antenna layouts and oversampling factors should be supported.

| Number of CSI-RS antenna ports | (N1, N2) | (O1, O2) |
| --- | --- | --- |
| 20 ports | (5, 2), (2, 5) | (4, 4), (8, 4) |
| 24 ports | (6, 2), (2, 6), (4, 3), (3, 4) |  |
| 28 ports | (7, 2), (2, 7) |  |
| 32 ports | (4, 4), (8, 2), (2, 8) |  |

- Reference

[1] R1-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung.

- Appendix

| System level simulation parameters |  |
| --- | --- |
| Scenarios | 3D-UMi 200m ISD and 3D-Uma 200m ISD |
| Antenna Configurations | Default is 2x1 virtualization, otherwise depends on the value of K. |
| Antenna Spacing | (dV,dH)=( 0.8λ, 0.5λ) |
| Number of UE antenna | 2Rx cross-polarized antenna |
| Traffic model | FTP 1 with packet size 0.5M byte |
| OLLA | Target at 10% BLER |
| CSI-RS | Period is 5 ms and overhead is accounted. |
| Codebook | Extension of Rel-13 Class A codebook |
| HARQ | Max 4 retransmissions |
| Transmission rank | 1, 2 |
| SU/MU pre-coding | ZF |
| Scheduling | Proportional fair, up to 2 UEs, up to 2 layers |
| CQI/PMI reporting interval and frequency granularity | 5ms for CSI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2, Ideal channel covariance /PMI feedback |
| Delay for scheduling and AMC | 6ms |
| Receiver | MMSE-IRC. With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | FTP1 model with 0.5Mbyte |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |