3GPP TSG RAN WG1 Meeting #86bis	R1-1609424

Lisbon, Portugal, October 10-14, 2016

Agenda Item:	8.1.6.2

Source:	Huawei, HiSilicon

Title:	Evaluation of flexible duplex in urban macro scenario

Document for:	Discussion

Introduction

In RAN1 #86 meeting, it was agreed that [1]

- NR should support at least following design targets: - It should allow FDD operation on a paired spectrum - It should allow different transmission directions in either part of a paired spectrum - It should allow TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changing - It should allow TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing - FFS: It should allow support of full duplex in a forward compatible way - Note: transmission directions include all of downlink, uplink, sidelink, and backhaul link - Note that additional discussion is needed about the timing to support above targets, particulally the second sub-bullet - Note that some design targets may or may not be transparent to UE

To study the feasibility as well as to identify the challenges of flexible duplex on paired and un-paired spectrum in various deployment scenarios, comprehensive evaluations are needed. In RAN1 #86 meeting, some evaluation assumptions for flexible duplex were agreed [2][3][4]. Dense urban, urban macro and indoor hotspot would be considered. In this contribution, some preliminary evaluation results for urban macro are provided and discussed.

Discussion on key simulation assumptions

In this section, some simulation scenarios and assumptions are discussed. These key assumptions are important for identifying the technical feasibility and challenges of flexible duplex. For detailed system-level simulation parameters, the parameters in Appendix A are used, which are aligned with RAN1#86 agreements [2][3][4].

Network layout

In this contribution, the focus is on one operator scenario and the single-layer layout for urban macro is used. The detailed layout set is as follows:

- Macro layer: Hex. Grid

Further, for urban macro scenario where inter-site distance (ISD) is usually 500m or even larger, lower frequency band will be used. For this consideration, 2GHz and 4GHz are employed in the evaluation,

In addition, paired spectrum needs to be considered as well as unpaired spectrum. 2GHz would be a good candidate to evaluate paired spectrum case. Therefore, two simulation cases are considered for urban macro single-layer deployment:

- 2GHz: Macro layer with paired spectrum

- 4GHz: Macro layer with unpaired spectrum

Downlink and uplink resource allocation

For flexible duplex, the downlink-uplink resource allocation on both paired spectrum and unpaired spectrum is summarized in Table 1.

In RAN1 #86 meetings [2], it was agreed that FTP traffic model 3 with packet size 0.5M bytes can be used and the ratio of DL/UL traffic load is {4:1}.

For unpaired spectrum, the baseline resource allocation is a static DL/UL subframe ratio for all TRPs. Specifically, when the traffic ratio of DL/UL is 4:1, the baseline downlink-uplink subframe ratio is 4:1[6]. For flexible duplex on unpaired spectrum, dynamic DL/UL subframe ratio allocation is used and adapted to the instantaneous DL/UL traffic need.

For paired spectrum, the baseline resource allocation is based on FDD, where a symmetric DL and UL bandwidth is allocated. For flexible duplex on paired spectrum, we consider a flexible downlink and uplink transmission on UL carrier of the traditional FDD, and employ dynamic DL/UL subframe ratio allocation on that carrier.

Table 1 provides the summary of the employed subframe allocation for baseline and flexible duplex

Table 1 simulation setup parameters and resource allocations

(a) Paired spectrum

|  | # DL/UL | # Baseline (Traditional FDD) | # Flexible duplex |
| --- | --- | --- | --- |
| DL/UL subframe ratio | # DL carrier | # DL only transmission | # DL data transmission and UL SRS transmission (see below) |
|  | # UL carrier | # UL only transmission | # Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | # DL carrier | # 2GHz: 10MHz |  |
|  | UL carrier | 2GHz: 10MHz |  |

(b) Unpaired spectrum

|  | Baseline (Traditional TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | semi-static subframe allocation - For DL:UL traffic ratio = 4:1, DL/UL subframe = 4:1 | Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | 4GHz: 20MHz (DL+UL) |  |

SRS transmission on DL carrier

Accurate measurement of channel state can improve the system performance. If sounding reference signal (SRS) could be allowed in the part of the paired spectrum used dominantly for DL data transmission, channel reciprocity can be exploited to improve the DL MIMO performance. In the preliminary simulation, we select 1 subframe out of 10 subframes to transmit SRS, and 2 OFDM symbols are used for SRS transmission, which is depicted as in Figure 1. The overhead induced by SRS is taken into account in the results.

Figure 1: Example of SRS @ DL dominant spectrum for flexible duplex evaluation on paired spectrum

Modelling of cross-link interference mitigation

In flexible duplex, cross-link interference, e.g. TRP-to-TRP and UE-to-UE interference, exists in case that neighboring cells use different transmission directions on the same time-frequency resource regardless paired or unpaired spectrum. An example is shown in Figure 2.

Figure 2: Example of cross-link interference

In the simulation, the cross-link interference is modeled with appropriate propagation characteristics as given in [3] and [4]. It is noted that for urban macro scenario, 80% UEs are indoor UEs and 20% UEs are outdoor UEs, and the UE-to-UE cross-link refers to the link between UEs connected to different TRPs. In this case, it is very likely such cross-link is between UEs in different bulidings. In such cases, the  peneration loss would be large. The UE-to-UE cross link interference in  urban macro scenario is therefore not very severe. On ther other hand, all TRPs are outdoor, with large transmission powers. And the height of TRPs are higher than UE side, which results in higher probability of LOS channel betewen TRPs. All these factors lead to severe interference between  TRP-to-TRP cross-link and therefore such interference needs to be mitigated [7]. In this contribution, for the TRP-to-TRP interference mitigation, the semi-static beam coordination is used.

Simulation results

This section shows the preliminary evaluation results for different scenarios based on the above simulation assumptions.

Flexible duplex on paired spectrum @ 2GHz

SRS @ DL part carrier

In this section, the evaluation results of flexible duplex for 2GHz paired spectrum case with SRS on DL carrier are presented in Table 2, as discussed in section 2.3.

Table 2 DL Performance evaluation with SRS@ DL dominant spectrum

| Feature | DL RU (%) | DL Average UPT(Mbps) | DL Average UPT Gain | DL 5%-tile UPT(Mbps) | DL 5%-tile UPT Gain |
| --- | --- | --- | --- | --- | --- |
| Baseline (DL only) | 50.76 | 23.08 | - | 2.65 | - |
| SRS@ DL part carrier |  | 25.87 | +12.09% | 3.58 | +35.09% |

It is observed from the simulation that DL performance can be improved with the channel state measurement by SRS on DL dominant spectrum due to the more accurate measurement of channel state.

Based on the above simulation results, we have the following observation.

Observation 1:

Flexible duplex is feasible in urban macro scenario for 2GHz paired spectrum. Preliminary evaluation results show that with the measurement based on sounding reference signal on DL dominant spectrum, it achieves considerable significant throughput gain in downlink (e.g., average UPT up to 10%, 5%-tile UPT up to 35%) in urban macro scenario.

Flexible DL and UL transmission @ UL part carrier

In this section, the evaluation results of flexible duplex for 2GHz paired spectrum with flexible UL/DL subframe ratio allocation in UL part carrier are presented in Table 3, as discussed in section 2.2. For flexible duplex, in UL carrier of the paired spectrum, the DL-UL subframe ratio is dynamically changed according to the DL and UL traffic load ratio. For some frames, if there is no UL traffic, the DL-UL subframe allocation ratio is set to be 9:1 at the UL carrier.

Table 3 Performance evaluation of flexible duplex at paired spectrum @ 2GHz

| Ratio of DL/UL traffic | Feature | DL RU(%) | DL Average UPT(Mbps) | DL Average UPT Gain | UL RU(%) | UL Average UPT(Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4:1 | Baseline | 22.97 | 30.01 | - | 7.55 | 14.33 | - |
|  | Flexible duplex |  | 38.57 | +28.52% |  | 13.59 | -5.2% |

It is observed from the simulation that DL performance can be improved due to the increased resource for the downlink transmission. For the UL performance, some performance degradation is observed due to the fact that less resource can be used for UL compared to the baseline. However, it is observed that the loss is not significant.

It shall be noted that a simplified interference mitigation scheme is used in the evaluations. It is expected that with better interference mitigation scheme, higher gains can be achieved for flexible duplex on paired spectrum.

Based on the above simulation results, we have the following observation.

Observation 2:

Flexible duplex is feasible in urban macro scenario for 2GHz paired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission on one part carrier, it achieves considerable gain over traditional FDD with symmetric DL and UL bandwidth in downlink (e.g., up to 28%) and has a small loss in uplink (e.g., approximately 5%) in urban macro scenario.

Flexible duplex on unpaired spectrum @ 4GHz

In this section, the evaluation results of flexible duplex for 4GHz unpaired spectrum case are presented in Table 4. The DL-UL subframe ratio of 6:4 and 4:1 are used in baseline, as discussed in section 2.2. The flexible duplex uses dynamic subframe ratio allocation, which is adapted according to the downlink and uplink traffic load ratio.

Table 4 Performance evaluation of flexible duplex at unpaired spectrum @ 4GHz

| Ratio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU(%) | DL Average UPT(Mbps) | DL Average UPT Gain | UL RU(%) | UL Average UPT(Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:1 | Baseline | 4:1 | 26.17 | 40.72 | - | 29.00 | 3.79 | - |
|  | Flexible duplex | Dynamic |  | 41.11 | +0.96% |  | 6.19 | +63.32% |

It is observed from the simulation that there are occasions when the network needs to serve DL packet only, or UL packet only. In this case, flexible duplex enjoys the flexibility of allocating DL/UL subframe ratio appropriately to match the instantaneous need of DL and UL traffic, e.g., to allocate a high DL ratio when there is dominant DL traffic. In this case, the DL radio resource is boosted compared to static DL/UL subframe ratio allocation, where the UL resource ratio is always reserved irrespective whether there is UL traffic or not. Such resource increase benefits the user perceived throughput. The interference mitigation schemes are the same with section 3.1.2.

Based on the above simulation results, we have the following observation.

Observation 3:

Flexible duplex is feasible in urban macro scenario for 4GHz unpaired spectrum. Preliminary evaluation results show that it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 63%) in this case.

Conclusions:

In this contribution, we provided some preliminary evaluation results of flexible duplex in urban macro scenarios. The observations are summarized as follows.

Observation 1:

Flexible duplex is feasible in urban macro scenario for 2GHz paired spectrum. Preliminary evaluation results show that with the measurement based on sounding reference signal on DL dominant spectrum, it achieves considerable significant throughput gain in downlink (e.g., average UPT up to 10%, 5%-tile UPT up to 35%) in urban macro scenario.

Observation 2:

Flexible duplex is feasible in urban macro scenario for 2GHz paired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission on one part carrier, it achieves considerable gain over traditional FDD with symmetric DL and UL bandwidth in downlink (e.g., up to 28%) and has a small loss in uplink (e.g., approximately 5%) in urban macro scenario.

Observation 3:

Flexible duplex is feasible in urban macro scenario for 4GHz unpaired spectrum. Preliminary evaluation results show that it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 63%) in this case.

- References

[1]	3GPP, Chairman's Notes, RAN1 #86, Aug 2016.

[2] R1-168053, “WF on scenarios and evaluation assumptions for flexible duplex,” RAN1#86, LG Electronics, Huawei, HiSilicon, Gothenburg, Sweden, Aug 22 - 26, 2016.

[3] R1-168372, “WF on channel model issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, Intel, Gothenburg, Sweden, Aug 22 - 26, 2016.

[4] R1-168373, “WF on penetration issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, LG Electronics, Gothenburg, Sweden, Aug 22 - 26, 2016.

[5] R1-1609421, “General discussion on flexible duplex,” RAN1#86bis, Huawei, HiSilicon, Lisbon, Portugal, Oct 10 - 14, 2016.

[6] 3GPP TR 36.828 V2.0.0, “Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation,” June 2012.

[7] R1-1608830, “Discussion on TRP-to-TRP interference mitigation schemes,” RAN1#86bis, Huawei, HiSilicon, Lisbon, Portugal, Oct 10 - 14, 2016.

- Appendix A

| Parameters | Urban Macro |
| --- | --- |
| Layout | Single layer: -Macro layer: Hex. Grid 19 site, 3sector per site |
| Inter-BS distance | 500m |
| Minimum BS-UE (2D) distance | 35m |
| Minimum UE-UE (2D) distance | 3m |
| Carrier frequency | 2GHz，4GHz |
| Aggregated system bandwidth | 2GHz: Up to 40 MHz (DL+UL) 4GHz: Up to 200 MHz (DL+UL) |
| Simulation bandwidth | 2GHz : 10MHz (DL)+ 10MHz(UL) (FDD) 4GHZ: 20MHz (TDD) |
| Channel model | Below 6GHz: Macro-to-UE: 3D UMa Micro-to-UE: 3D UMi Macro-to-Macro: 3D UMa (h_UE=25m) Macro-to-Micro: 3D UMa (h_UE=10m) Micro-to-Micro: 3D UMi (h_UE=10m) UE-to-UE: A.2.1.2 in TR36.843 |
| Penetration loss | Follow [4] |
| BS Tx power | Below 6GHz: 49 dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49 dBm |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | Below 6GHz: Baseline: (M,N,P,Mg,Ng)=(8,8,2,1,1) (dH,dV)=(0.5,0.8)λ |
| BS antenna configuration |  |
| BS antenna height | # 25m |
| BS antenna element gain + connector loss | # 8dBi |
| BS antenna tilt | 99deg |
| BS receiver noise figure | Below 6GHz: 5 dB |
| UE antenna configuration |  |
| UE antenna elements | Below 6GHz: 2Tx and 2Rx |
| UE antenna | for outdoor UEs: 1 for indoor UEs: ~uniform(1, ) where ~uniform(4,8) |
| UE antenna gain | For below 6GHz: Follow the modeling of TR36.873 |
| UE receiver noise figure | Below 6GHz: 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes Ratio of DL/UL traffic = {4:1} , the DL user arrival rate λ is 0.1 and 0.15 for different RU. |
| UE distribution | For FTP traffic model 3: 10 users per macro TRP 80% indoor (3km/h) and 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| BS receiver | MMSE-IRC |
| UE association | based on RSRP measurement |
| Transmission mode | SU-MIMO with rank adaptation |