3GPP TSG-RAN WG1 #86bis												                                 R1-1609516

# Lisbon, Portugal, 10th – 14th October, 2016

Source: 	Intel Corporation

Title:	Beam management for initial and candidate beam discovery (BM P-1)

Agenda item:	    8.1.4.1

Document for:	Discussion and Decision

# Introduction

As it is widely known that as the carrier frequency increases, the severe pathloss becomes a crucial limitation to guarantee the sufficient coverage area. Since the path loss is inversely proportional to logarithm of the carrier frequency, the link quality decreases severely for high frequency band system using the same antenna configuration at the TRP/UE and at the same separation distance. Utilizing dozens or even hundreds of antenna elements to generate the beam formed signal is an effective way to compensate the severe path loss by providing significant beam forming gain, e.g. approximate 18dB gain for 8×8 antenna array at TRP and about 9dB gain for 2×4 antenna array at UE.  In RAN1 #86 it has been agreed that three beam management procedures are supported. This contribution focuses on procedure P-1 which is used to identify initial and candidate TRP/UE beams. Our companion contributions [1], [2] discuss beam management procedures P-2 and P-3.

# Beam acquisition mechanisms

In order to benefit from beamforming, TRP and UE needs to properly manage the beamforming weight at both ends of the link. For downlink, both TRP transmission beam and UE reception beam needs to be managed. The uplink may or may not reuse the downlink beam management procedure [3]. Several options exist for TRP/UE beam acquisition as discussed in [4] [5]. In this section we discussed these options briefly.

Separate determination: For UE beam acquisition, the TRP transmits repeated reference signals with a wide beam during multiple consecutive OFDM symbols, which enables UE to perform beam sweeping and choose a good beam weight according to received signal quality. In the reverse link, PRACH can be transmitted with repeated waveform by the UE to enable TRP side beam acquisition. This option has short beam acquisition latency but reliability is limited by beamforming at one end only.  It is also possible that the acquired TRP beam and UE beam are mismatched for data communication.

Joint determination: TRP transmits the reference signal with different narrow beams during multiple consecutive OFDM symbols, which enables the receiver to perform the TRP beam acquisition, as well as UE beam acquisition at the same time. For instance, TRP transmits specifically designed beam training RS periodically. Then UE may determine the TRP beam using omnidirectional beam first, and determine UE   beam further or determine TRP/UE side beam pair using narrow beam in one sweep cycle. Here, the preferred TRP beam is reported by UE. This option may have large beam acquisition latency but high reliability as beamforming gain at both TRP and UE are incorporated into the beam acquisition procedure. Also the acquired TRP and UE beam are well aligned for data communication.

Multi-stage determination: Joint determination is performed in multi-stages with different beam width. More details can be found in section 3.1.

Since the beam forming gain can be utilized to compensate the severe path loss, it is preferred to leverage the TRP side and UE side beam forming gain as much and early as possible, for example during initial access phase. If reciprocity is assumed, the acquired beam of one link direction can be leveraged to improve the quality of the other link direction.

Observation 1: It is preferred to incorporate beamforming gain at both TRP and UE sides as much and as early as possible for system operation.

# Discussion on Beam acquisition mechanisms

In this section we discuss the joint determination and multi-stage determination beam acquisition mechanisms in section 2 in more details. More schemes can be also developed by combining those schemes.

# Joint determination with single narrow beam width

Figure 1 illustrates an example of utilizing multiple beam with single beam width to cover an area with horizontal angular span of 120 degrees, and vertical angular span from 45 degree to 135 degree, where each beam with narrow beam width orients toward different azimuth/zenith angle. The TRP will apply these beams on a specific reference signal, so that UE can acquire one or multiple preferred beam candidates.

Figure 1: An example of single step and narrow beam width based beam acquisition

Suppose the total number of narrow beams at the TRP and UE sides are Nbeam,TRP and Nbeam,UE (Nbeam,TRP, Nbeam,UE >> 1) respectively, then Nbeam,TRP*Nbeam,UE iterations need to be performed for exhaustive beam searching, which will involve high computation complexity for beam quality calculation and comparison. Besides the complexity, the required time period for exhaustive beam searching is very long, which causes large delay for beam acquisition and tracking. Suppose TRP can generate Npanel,TRP beams at a given time slot,  where Npanel,TRP is limited by the hardware equipment, and might be small for low cost consideration. Assuming one TRP panel can transmit one beam in an OFDM symbol, Nbeam,TRP*Nbeam,UE/Npanel,TRP OFDM symbols are required to perform an exhaustive beam search. Additional time may be needed as the processing time for beam strength computation and comparison.

In [6], we have analyzed how many narrow beams are needed at both TRP and UE sides to ensure cell coverage. And we have shown that in order to ensure sufficient coverage at both TRP and UE side, an oversampled narrow beams are required. With an oversampled beam pattern to cover different angles, the beams covering adjacent azimuth/zenith angles can have high spatial correlation. As a result, spatially correlated beam pairs may have correlated beam pair strength than spatially uncorrelated beam pairs. By leveraging this characteristic, hierarchical beam search method can be defined to reduce the beam search latency of exhaustive search. UE can perform a full spatial beam sweeping with a coarse angular granularity at the first step to determine the UE beam without much loss of accuracy, and in the second step UE can use the acquired UE beam to search all TRP narrow beams to further improve the accuracy of TRP beam than in the first step. Thus the overhead of single narrow beam based acquisition can be greatly reduced compared with exhaustive narrow beam based search without much loss of beam searching accuracy. In the following we give more details on the hierarchical beam search based method.

Observation 2: Single narrow beam based exhaustive search has the best beam acquisition accuracy and large delay. However, hierarchical beam search method can be used to reduce the latency without much loss of accuracy.

In order to enable hierarchical beam search, the beams at the TRP side can be divided into several subsets according to their spatial correlation. Each subset can cover the whole space with sparse angular granularity, and the beams within one subset orients toward different angles. Figure 2(a)-(c) illustrate an example of three beam subsets. All the three subsets are spatially equivalent in terms of determining whether one UE beam is good or not.  This allows UE to determine whether one UE beam is good or not by only using it to sweep 1/3 of all TRP beams.

| - Illustration of beam subset1 | - Illustration of beam subset2 |
| --- | --- |
| - Illustration of beam subset3 | - The layout of beam subset |

Figure 2: Multiple narrow beam pattern at the TRP side

To reduce the searching period, the beams within one subset will be transmitted on several OFDM symbols of one subframe, and an example of three subsets is illustrated in the Figure 3(d), where Npanel,TRP beams within one subset are transmitted at the same OFDM symbol.  In this way, the time period for one subset is also reduced to 1/Npanel,TRP. If multiple beam procedure is performed at the UE side, that is Nbeam,UE>1, UE can utilize one UE beam to sweep one subset #beamNW,subset1, switch to another UE beam to sweep subset #beamNW,subset2, and so forth. Thus the total beam sweeping latency is reduced to 1/3 of exhaustive search by having three TRP beam subsets.

Proposal 1: All beams at the TRP side can be divided into multiple subsets, where each subset can cover the full horizontal/vertical space with sparse angular granularity. Multiple subsets are equivalent from determining UE beam’s view point.

To further reduce the complexity and latency for beam search, the hierarchical beam search can be applied at the UE side too. Divide the UE side beams into multi-tier subsets, where one beam on the higher-tier is associated with a subset on the lower-tier. An example is illustrated in the Figure 4, where beam {#beamUE,2 #beam UE,5 #beam UE,8 #beamUE,10 #beamUE,13 #beam UE,17 #beam UE,20 #beamUE,24 #beamUE,27} is the first tier subset, and each beam is associated with a second tier subset, e.g. {#beamUE,1 #beamUE,3 #beamUE,12} for #beamUE,2. At the first sweep stage, the beams in the first-tier subset are swept, where each beam will be utilized to sweep one TRP side beam subset, and then in the second step, the second tier beam subset which is associated with the beam with the strongest strength in the first stage will be chosen for the next round beam sweep, e.g. {#beamUE,15 #beamUE,23 #beamUE,25}if #beamUE,24 is the acquired best UE beam in the first round.

Figure 3: Hierarchical beam search at the UE side

Proposal 2: The beams at the UE side can be divided into multi-tier subsets, and the hierarchical beam search can be realized by beam search tier by tier.

# Multi-stage determination with variable wide/narrow beam width

The multi-step and variable wide/narrow beam width based scheme is illustrated in the Figure 1, where UE searches TRP/UE beams in two steps. In the first step, TRP creates wide beams with each wide beam covering the same area of a number of narrow beams. UE performs TRP side beam searching among the wide beams with UE side narrow beams. And UE acquires the best TRP wide beam and UE narrow beam in the first step. The number of wide beam can be designed to make a trade-off between beam search latency and beam reliability. In the second step, UE will search all the narrow beams corresponding to the best wide beam it has acquired in the first step.  Depending on the design, UE may or may not feedback the acquired wide beam during the first step. When UE does not feedback the acquired wide beam in the first step, TRP has to transmit all the candidate narrow beams in the second step. The beam acquisition reliability is limited by the first step as the TRP side beamforming gain is limited by the wide beam. When UE feeds back the acquired wide beam in the first step, TRP may only need to transmit the narrow beams corresponding to the recommended wide beam. Compared with no UE feedback in the first step, the downlink overhead in the second step is reduced. However, additional overhead and procedure is needed for the uplink feedback. So the overall system overhead maybe similar but additional uplink feedback procedure in the first step may reduce the beam acquisition reliability and error may occur during the feedback. Also additional waiting latency needs to be considered when evaluating the overall beam acquisition latency because UE may wait extra time in order to start the first step acquisition procedure considering wide/narrow beams are transmitted in different occasions.

Observation 3: Multi-step variable beam-width based beam acquisition may reduce beam acquisition latency but the beam acquisition reliability is limited by the wide beam-width in the first step.

Figure 4: An example of multi-step variable beam width based beam acquisition

# Comparison of beam acquisition schemes

In order to compare different beam acquisition schemes, the system level simulation is performed to evaluate the beam acquisition reliability and analysis is given for the beam acquisition overhead, where antenna configuration at the TRP side is (4, 8, 2, 2, 2, 0.5λH/V) with XPOL, and at the UE side is (2, 4, 2, 1, 2, 0.5λH/V) with +POL. The beam energy of different beam search algorithms based on different beam schemes are compared: 1) omni directional TRP and UE beam; 2) omni directional TRP beam and directional UE beam; 3) directional TRP beam and omni directional UE beam; 4) wider directional TRP beam and directional UE beam; 5) narrow TRP and UE beam based on hierarchical beam search, where two-tier are utilized by UE to sweep a subset of TRP beam; 6) narrow TRP and UE beam based on exhaustive beam search. Among the tested options, option 1) is mainly for benchmarking purposes when no beamforming gain is available at both ends of the link. Although different beam width options are used in different options, the final data communication can all be based on narrow beams at both TRP and UE. But different options can have significant beam acquisition reliability and overhead.

The beam reliability is adopted as the evaluation metric and is calculated according to the following equation:

(1)

where Hf,H(f) represent the equivalent channel matrix in frequency domain with beamforming gain; Wtx and Wrx are the beamforming weight vector at the TRP and UE side; fH and fL represent the upper and lower bound of subcarrier index. The result is shown in Figure 5, where the beam reliability is calculated based on the best TRP beam at a given subframe and the acquired best UE beam during the previous beam acquisition procedure. It can be seen that performing narrow beam at both TRP and UE side can provide the best beam reliability.

Figure 5: system level simulation comparison of beam acquisition reliability for different beam search algorithms

Table 1 compares the maximum beam search times, delay and beam reliability of different beam acquisition algorithms, where we set the parameters for different algorithms as (Nbeam,TRP,Nbeam,UE,Nsubset,TRP,Nsubset,UE,Npanel,TRP,Nwidebeam,TRP)=(48,27,3,9,4,4). Compared with exhaustive beam search, the hierarchical beam search algorithm has 0.5dB reliability loss as the exhaustive beam search with reduced overhead. The reliability of beam scheme with wide/narrow NW beams is about 6dB worse than hierarchical beam search, while the involved delay and complexity is reduced with appropriate beam pattern. In the wide/narrow beam scheme, two subframes are required to transmit the wide beam RSs for UE to traverse the available UE beams, and one or two subframes for narrow Tx beam sweeping based on narrow UE beams. Depends on the starting subframe, the beam search overhead/latency may vary.

Table 1: Comparison of different beam search algorithms

|  | Reliability of 20%-tile (dBm) | Reliability of 90%-tile (dBm) | Beam search overhead or latency |
| --- | --- | --- | --- |
| Exhaustive search (‘Full Tx’) | -122.99 | -90.53 | Nbeam,TRP*Nbeam,UENpanel,TRP (24 subframes) |
| Hierarchical search | -124.17 | -91.13 | Nbeam,TRPNpanel,TRPNsubset,TRP*(Nsubset,UE+Nbeam,UENsubset,UE) (5 subframes) |
| Wide/Narrow Tx (‘Tx wider beamed, Rx beamed’) | -130.39 | -97.47 | 1Npanel,TRP(Nwidebeam,TRPNbeam,UE +2Nbeam,TRP/Nwidebeam,TRP) (3~6 subframes) |
| Tx Omni/Rx Narrow and then Tx/Rx Narrow | -137.40 | -104.20 | Nbeam,UE+Nbeam,TRP/Npanel,TRP (2-3 subframes) |

Observation 4: Hierarchical narrow beam based search has similar beam search latency compared to multi-step variable beam width based beam search and similar reliability as exhaustive narrow beam based search.

When channel is changing, e.g. due to UE movement or rotation, it is important to quickly acquire the preferred TRP and UE side beam. The reliability of beam acquisition with UE rotation is evaluated, where the rotation speed is 360 degrees per second. As illustrated in the Figure 8, the performance of exhaustive search is significantly decreased, e.g. >5dB, while no noticeable performance loss of hierarchical beam search is observed owning to its reduced beam search delay.

Figure 6: system level simulation of beam acquisition with UE rotation

Observation 5: Under the time-varying environment, the hierarchical beam search can outperform the exhaustive beam search owning to the reduced beam search delay.

# Reference Signal design for P-1

In this section we discuss several aspects related to the reference signal of P-1. For the convenience of discussion, we name it as Beam Reference Signal (BRS) although all the reference signals in high band maybe beamformed.

# Number of beams for BRS

It is well known that the beam width becomes narrower with more antenna elements. So the number of BRS beams needed for the same coverage area should also be increased with number of antenna elements. In order to accommodate possible product differentiation with different number of antenna elements, it is desirable to have some flexibility in number of supported BRS beams. For example, support a set of BRS beams with number {50, 100, 150, 200}. Although it may be nice to support a much wider range of BRS beams, the exact number of BRS beams is also limited by the frame structure and allowed antenna radiation power, there also exists tradeoff between the number of BRS beams and beam sweeping latency. So increasing number of BRS beams to a very large number may result in excessive sweeping latency which offsets most of its benefit.

In our companion contribution [6], the impact of number of BRS beams is analyzed and simulation results are provided. It is observed that oversampling is needed to pack the narrow beams in order to fully reap the beamforming gain. Basically for the agreed TRP antenna array (M, N, P) = (4, 8, 2) configuration at 30GHz, the recommended number of BRS beams should be close to 50. Details can be found in [6].

Proposal 3: BRS should have certain flexibility to accommodate at least different number of beams. For the agreed TRP antenna @ 30GHz, an estimation on the number of BRS beams is close to 50.

# BRS mapping in frequency domain

In frequency domain, the BRS mapping could be either localized or distributed. With localized transmission, one BRS occupies consecutive REs while non-consecutive REs are used for distributed transmission. With distributed mapping, the BRS can be transmitted over wider band and it may represent the channel characteristics more accurately. However, due to beamforming effect, the difference between localized and distributed BRS mapping may be limited as the post beamforming channel becomes flatter. But distributed BRS maybe better suitable as demodulation reference signal for the critical system information in the same subframe if the system information occupies the REs which are FDMed with the BRS REs. If BRS is used as demodulation reference signal for system information, it is better to have two ports transmission which can leverage the polarization diversity.

Figure 7:  Example of BRS mapping in frequency domain

Figure 7 shows an example for the localized and distributed BRS mapping in the frequency domain. BRS for one beam occupies consecutive REs or non-consecutive REs respectively.

There could be two ways for the localized BRS mapping in the frequency domain. One option is that the BRS for the same beam occupies the same REs in all the sub-frames. And the other option is that the BRS for the same beam occupies different REs in different sub-frames, i.e. there is frequency hopping in different sub-frames.

Proposal 4: If BRS is also used as demodulation RS for other system information, frequency distributed mapping with two ports BRS is more suitable.

# BRS mapping in time domain

The BRS mapping in the time domain could also be localized or distributed. With localized mapping in the time domain, the BRS is mapped periodically in certain sub-frames and the whole sub-frame is used for BRS and other signals/channels requiring beam sweeping. For example, the BRS is sent in sub-frame 0, sub-frame 25, and so on. The other way is to send the BRS with distributed mapping, i.e. the BRS is sent in each sub-frame but only a few OFDM symbols, e.g. the first one, is used for BRS. Figure 8 shows an example for the localized and distributed BRS mapping in time.

Figure 8  BRS mapping in the time domain

Both mapping methods may have similar beam sweeping latency if the total overhead for BRS is similar. But time domain localized mapping may be more compatible with, e.g. dynamic TDD kind of frame structure. On the other hand, time domain distributed mapping maybe more compatible with single beam operation in which BRS maybe more like CRS which can serve the purposes of both RRM and demodulation.

For distributed BRS transmission, there are two options which can lead to different BRS position in the sub-frame, one option is that the BRS is used only for measurement, the other option is that the BRS is used not only for measurement but also for PDCCH decoding. If the BRS is used for measurement only, the BRS can be mapped to the symbol after PDCCH. If the BRS is also used for PDCCH decoding, the BRS can be mapped to the first symbol before PDCCH, in which the BRS is used to estimate the channel of PDCCH.

Proposal 5: Time domain localized transmission is more preferred for better beam sweeping performance.

# BRS based beam state report

The TRP and the UE should maintain the pair of NW beam and UE beam for communication. So it is important for both sides to have the up-to-date information about the beam state, i.e. whether the signal quality from certain beam is good or not. The UE should keep monitoring the beam state, for example, by measuring the received power of BRS (BRS-RP) and report the beam state to the TRP. In this way the UE and the TRP can know whether the signal strength for one beam is strong enough and can do beam switching timely if necessary.

There are two ways to do the beam state report: periodic reporting or aperiodic reporting. For periodic reporting, the UE reports the beam states every certain period while the aperiodic reporting is triggered by some event, for example, the BRS-RP becomes lower than a threshold. The periodic reporting can provide more information but with relatively high overhead. The periodic and aperiodic reporting can be utilized to complement each other.

Further, there can be two options for aperiodic reporting:

Option 1: TRP ordered. If the TRP finds the signal strength from one beam becomes lower than certain threshold or has discovered a better beam than the previously recommended NW beams, it sends the UE an order to report the beam state.

Option 2: UE triggered. The UE will trigger the beam state reporting if it finds the BRS-RP for one beam becomes lower than certain threshold or find a new beam which is better than previously recommended NW beams.

Proposal 6: RAN1 supports both NW ordered and UE triggered beam state reporting.

# Conclusions

In this contribution, we have discussed different beam acquisition algorithms. Based on the analysis and simulations, the hierarchical beam search seems to be an attractive option in terms of reliability and latency/overhead. The observations/proposals are listed:

Proposal 1: All beams at the TRP side can be divided into multiple subsets, where each subset can cover the full horizontal/vertical space with sparse angular granularity. Multiple subsets are equivalent from determining UE beam’s view point.

Proposal 2: The beams at the UE side can be divided into multi-tier subsets, and the hierarchical beam search can be realized by beam search tier by tier.

Proposal 3: BRS should have certain flexibility to accommodate at least different number of beams. For the agreed TRP antenna @ 30GHz, an estimation on the number of BRS beams is close to 50.

Proposal 4: If BRS is also used as demodulation RS for other system information, frequency distributed mapping with two ports BRS is more suitable.

Proposal 5: Time domain localized transmission is more preferred for better beam sweeping performance.

Proposal 6: RAN1 supports both NW ordered and UE triggered beam state reporting.

Observation 1: It is preferred to incorporate beamforming gain at both TRP and UE sides as much and as early as possible for system operation.

Observation 2: Single narrow beam based exhaustive search has the best beam acquisition accuracy and large delay. However, hierarchical beam search method can be used to reduce the latency without much loss of accuracy.

Observation 3: Multi-step variable beam-width based beam acquisition may reduce beam acquisition latency but the beam acquisition reliability is limited by the wide beam-width in the first step.

Observation 4: Hierarchical narrow beam based search has similar beam search latency compared to multi-step variable beam width based beam search and similar reliability as exhaustive narrow beam based search.

Observation 5: Under the time-varying environment, the hierarchical beam search can outperform the exhaustive beam search owning to the reduced beam search delay.

# References

- R1-1609515, On network beam switching and refinement (BM P-2), Intel Corporation

- R1-1609514, Reference Signal and Procedure for UE Beam Refinement (BM P-3), Intel Corporation

- R1-167128, Discussion on uplink beam management in multi-beam operation, Intel Corporation

- R1-166565, Beam management without prior beam information, Intel Corporation

- R1-168388, WF on beam management, CATT et al

- R1-167130, evaluation of NR systems above 6GHz, Intel Corporation

# Appendix

Table 2, Simulation Parameter

| Parameters | Values |
| --- | --- |
| Layout | 19 sites, 3 sectors/site, ISD = 200 |
| Carrier frequency | 30GHz |
| System bandwidth | 80MHz |
| Channel model | 3D,UMi, TR38.900 with spatial consistency |
| TRP tx power | 35dBm |
| Antenna configuration | (4, 8, 2, 2, 2, 0.5λH/V) with XPOL for TRP (2, 4, 2, 1, 2, 0.5λH/V) with +POL for UE |
| TRP port mapping | All 32 elements for each polarization on each panel maps to one port, where the horizontal/vertical angles are swept. |

Table 3, Antenna Radiation Pattern of TRP

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

Table 4, Antenna Radiation Pattern of UE

| Parameter | Values |
| --- | --- |
| Vertical antenna element radiation pattern |  |
| Horizontal antenna element radiation pattern |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |