3GPP TSG RAN WG1 Meeting #86bis	R1-1609869

Lisbon, Portugal, 10th – 14th October 2016

Source:	Sharp

Title:	Shortened processing time operation for 1ms TTI

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

# Background

In RAN1 #86, the following agreements were made for processing time reduction of 1ms TTI [1].

Agreement:

- Reduced processing time(s) are RRC configured for the UE

- Working assumption: A mechanism for dynamic fallback to legacy processing timings (n+4) is supported

  - Details FFS

  - Working assumption can be revisited if it is not found to be feasible

Agreement:

- For FS1,2&3, a minimum timing n+3 is supported for UL grant to UL data and for DL data to DL HARQ for UEs capable of operating with reduced processing time with only the following conditions:

  - A maximum TA is reduced to x ms, where x <= 0.33ms (exact value FFS);

  - At least when scheduled by PDCCH

- For FS2, new DL HARQ and UL scheduling timing relations will be defined

  - Details FFS

- FFS:

  - Possible minimum timing of n+2 TTI

    - FFS max TA in this case

    - FFS what other restrictions (if any) on when reduced processing times of n+2 could be applied

  - Possibility of scheduling by EPDCCH.

In this contribution, we present our views on fallback mode operation and support of n+2 processing time reduction.

# Fallback mode support with processing time reduction

In RAN1 #86, a working assumption was made to support a mechanism for dynamic fallback to legacy processing timings (n+4) [1]. Fallback mode operation is needed at least during transition between legacy and reduced processing time configuration. Common search space (CSS) scheduling is one important case of using fallback mode operation [2]. Since CSS is shared among all UEs, legacy DCI formats should be used. To ensure consistent behavior of CSS scheduled transmissions, if a PDSCH or PUSCH transmission is scheduled in CSS, legacy timing should be used.

Furthermore, dynamic fallback operation may be beneficial to avoid HARQ collision in some cases. If dynamic fallback operation is supported, extra bits may be added to a new DCI format to indicate the actual processing time required for a DL or UL transmission. The dynamic indicated processing time could provide a unified solution for processing time reduction, LAA and 5G new RAT (NR).

Proposal 1: Fallback operation to legacy timing should be supported at least for DL and UL transmissions scheduled in CSS.

# Support of n+2 processing time reduction

In RAN1 #86, it was agreed that a minimum timing of n+3 for UL grant to UL data and for DL data to DL HARQ is supported for UEs capable of operating with reduced processing time with no restricted maximum transport block size (TBS) if the maximum TA value is reduced to x ms (e.g. x=0.33, exact value FFS). The reduced processing time of n+3 is applicable at least for PDCCH [3].

A reduced n+3 processing time is 25% less compared with legacy n+4 timing. A reduced n+2 processing time can further achieve a 50% reduction over legacy timing. Thus, it is highly desirable to support processing time reduction of n+2 for PDSCH and/or PUSCH under certain conditions, e.g. with reduced maximum TA and restricted maximum TBS.

Since reduced processing time(s) are RRC configured for the UE, the UE should not be configured with reduced processing time if the maximum TA is greater than the restricted value. For a UE that is configured with n+2, the UE may not expect to process UL and DL data that exceeds the TBS threshold. Or dynamic fallback to n+3 or legacy timing can be supported based on CSS scheduling, TBS restriction and/or dynamic DCI indication, etc.

Proposal 2: Reduced processing time of n+2 is supported with restricted TBS and maximum TA value. Details FFS.

# Conclusion

In this contribution, we consider the reduced processing time on legacy 1ms TTI, and analyze potential issues with coexistence of legacy and reduced processing timings. And we propose

Proposal 1: Fallback operation to legacy timing should be supported at least for DL and UL transmissions scheduled in CSS.

Proposal 2: Reduced processing time of n+2 is supported with restricted TBS and maximum TA value. Details FFS.

# References

- Chairman’s note RAN1 #86, Aug 2016

- R1-168231 	WF on fallback transmission for shortened processing time in 1ms TTI

- R1-168192	WF on support of reduced processing time for 1ms TTI