3GPP TSG RAN WG1 Meeting #86bis                                                               	R1-1610072

Lisbon, Portugal 10th - 14th October 2016

Source:	NTT DOCOMO

Title:	Phase-1 MIMO Calibration Results

Agenda Item:	8.1.4.5

Document for: 	Discussion

# Introduction

At the RAN1 #86 meeting [1], it was agreed to do MIMO calibration.

Agreements:

- For the purpose of calibration, companies are encouraged to provide baseline results for NR MIMO in the following meetings, including link-level and system level simulations

  - Discuss further on baseline simulation assumptions and metrics for calibration in email discussion after RAN1#86

  - Thereafter, email reflector is used to collect results (no online time for calibration discussions, similar to channel model calibration)

Agreements:

- For the purpose of calibration, companies are encouraged to provide baseline results for NR MIMO in the following meetings, including link-level and system level simulations

  - Discuss further on baseline simulation assumptions and metrics for calibration in email discussion after RAN1#86

  - Thereafter, email reflector is used to collect results (no online time for calibration discussions, similar to channel model calibration)

In the email discussion of [86-20], detailed evaluation methodologies, performance metrics and simulation assumptions were discussed and agreed. Related agreements were summarized in [2].

According to the agreement, the calibration shall be conducted based on the following timeline.

Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

In this contribution, we provide initial results for phase-1 MIMO calibration.

# Phase-1 System Level Performance

According to the work plan, phase-1 system level calibration shall provide the CDF of wideband SINR with and without beamforming. In our understanding, the motivation here is to check the wideband SINR with single beam transmission and with multi-beam transmission. And the SINR calculation is obtained by assuming signal transmitted from a single TXRU. No digital domain beamforming is considered. TXRU to antenna element mapping is based on a 2D DFT weighting. And the following angles are considered for single-beam and multi-beam cases, respectively.

- For single beam case (fixed beam): in the horizontal dimension, the beam is pointed to 0 degree; and in vertical dimension, the beam is pointed to 102 degrees.

- For multi beam case (beam sweeping): in the horizontal dimension, multiple beams uniformly span [-60, 60] degree; om the vertical dimension, multiple beams uniformly span [90, 160] degree.

In the multi-beam case, each TRP may choose one of the several beams for transmission. According to the agreed evaluation assumption, each UE has two panels. We assume that the UE will perform a panel selection. For each panel, the UE RX beam is fixed. For each UE, its own beam selection is based on RSRP measurement. The TX and RX beam pairs with the highest RSRP is selected. To modelling the interference, we simply consider a random beam from the neighboring TRP. Other evaluation assumptions are provided in Table A in the Appendix.

Based on the above mentioned evaluation assumptions, the achieved SINR CDF curves for the urban macro scenario are shown in Figure 1.

Figure 1. CDF of wideband SINR with and without beam sweeping.

From the wideband SINR results, it can be seen that beam sweeping can improve the SINR performance.

# Summary

In this contribution, we present the SINR performance in the UMa scenario for MIMO calibration.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #86 (v0.2.0),”  August 2016.

- 3GPP, R1-1608661, ZTE, “Summary of [86-20] email discussion on MIMO calibration for NR”, October, 2016.

# Appendix

Table A: Evaluation Assumptions for MIMO Calibration in UMa Scenario

| Parameters | Values |
| --- | --- |
| Layout | Single Layer Macro layer: Hex. Grid |
| Inter-BS distance | 500 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 60 KHz |
| Channel model | 3D UMa |
| BS Tx power | 43 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH, dV, dHg,dVg) = (0.5, 0.5, 4.0, 2.0) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 25 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (2,4,2,1,2) (dH, dV, dHg, dVg) = (0.5, 0.5, 0, 0) |
| UE antenna height | 1.5 m |
| UE receiver noise figure | 10 dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic (10 users per TRP is the baseline with full buffer traffic. 20 users per TRP with full buffer traffic is not precluded.) |
| Beam selection method | Select a beam among the limited set of DFT beams with oversampling factor=1. Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. Companies provide the details of beam selection. Beam sweeping range: Azimuth [-60,60]; Zenith [90,160] |
| Evaluation Metrics | CDF of wideband SINR with and without beamforming |