3GPP TSG RAN WG1 Meeting #87	R1-1612420

# Reno, USA 14th - 18th November 2016

Agenda item:	6.2.2.2

Source: 	Samsung

Title: 	Simulation results for precoder cycling based on semi-open-loop transmission

Document for:	Discussion and Decision

# Introduction

In RAN1#86b, the following agreement about semi-open loop transmission scheme was made [1].

Agreements:

- Denote the modulation symbol index as i, RE-level PDSCH processing for semi-open-loop MIMO is based on DMRS ports 7/8

  - Rank-1: Tx diversity

  - Rank-2:  co-phasing cycling

- For the purpose of CSI feedback, UE assumes the following beamforming on port 7/8 (to N CSI-RS ports)

  - Dual-stage codebook:  W(j)= W1W2(j)

    - W1: 	 wideband i1 (e.g. grid of beams) of rank-2

    - Rank-1 (options to be downselected at RAN1#87):

      - Option 0: SFBC without defined cycling pattern (e.g. codebook subset restriction applicable to i2, and/or hybrid A+B CSI (with second eMIMO type associated with single-stage codebook), and/or Class B)

      - Option 1: SFBC only + fixed beam selection

      - Option 2: SFBC + per-N-PRB-pair cycling with defined cycling pattern

        - W2(j):  pre-determined set of beam selection matrices of rank-2

          - W2(j) cycling per PRG, where PRG comprises N consecutive PRB pairs

          - value of N FFS

          - size/order of beam selection matrices for cycling FFS

    - Rank-2 (options to be downselected at RAN1#87)

      - Option 0:  without defined cycling pattern  (e.g. codebook subset restriction applicable to i2, and/or hybrid A+B CSI (with second eMIMO type associated with single-stage codebook), and/or Class B)

      - Option 1: fixed beam selection

      - Option 2:  per-N-PRB-pair cycling with defined cycling pattern

        - W2(j):  pre-determined set of beam selection matrices of rank-2

          - W2(j) cycling per PRG, where PRG comprises N consecutive PRB pairs

          - value of N FFS

          - size/order of beam selection matrices for cycling FFS

  - …

Based on the agreement, the companion contributions [2-4] proposes Option 2 ((highlighted text) for both rank 1 and rank 2 semi-open loop transmission, i.e. per-N-PRB-pair cycling with defined cycling patterns. This contribution provides simulation results in support of this proposal.

# Semi-open-loop transmission schemes for evaluation

Figure 1: An illustration of RE level co-phase cycling of co-phase values (for 2nd polarization) in an RB

Table 1: RE level cycling of 2 co-phase values (for 2nd polarization)

| Scheme | W1 beam cycling granularity | Co-phase for 2nd polarization |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cycling | Co-phase values | Number of co-phase values for cycling | Number of RBs for co-phase cycling |
| Scheme 0 | 1 RB | RE level cycling | {1,j,-1,-j} | 4 co-phase values | 1 |
| Scheme 1 | 1 RB |  | {1,-1} | 2 co-phase values | 1 |
| Scheme 2 | 2 consecutive RBs |  | {1,-1}, {j,-j} | 2 pairs of 2 co-phase values | 2 |
| Scheme 3 | 4 consecutive RBs |  | {1,j}, {j,-1}, {-1,-j}, {-j,1} | 4 pairs of 2 co-phase values | 4 |
| Scheme 4 | 6 consecutive RBs |  | {1,-1}, {j,-j}, {1,j}, {j,-1}, {-1,-j}, {-j,1} | 6 pairs of 2 co-phase values | 6 |

In this evaluation, we assume that a UE feeds back the first PMI (i1,1,i1,2) indicating L = 4 beams using Rel. 13 Class A codebook with Codebook-Config = 2, 3, and 4 for precoder cycling based semi-open-loop (semi-OL) transmission by the eNB. Using the reported first PMI, the eNB performs RB level cycling of 4 reported beams, and RE level cycling of co-phase values (for the 2nd antenna polarization; note: co-phase for 1st antenna polarization is 1) from the QPSK alphabet {1, j, -1, -j}. Depending on whether all or a subset of 4 co-phase values (for 2nd polarization) are cycled in an RB, we have the following rank 1 precoder cycling schemes, which are illustrated in Figure 1 and summarized in Table 1.

- Scheme 0: In this scheme, four beams, beam 0 – 3, are cycled at RB level and four co-phase values {1, j, -1, -j} are cycled at RE level in each RB.

- Scheme 1: In this scheme, four beams, beam 0 – 3, are cycled at RB level and two co-phase values {1, -1} are cycled at RE level in each RB.

- Scheme 2: In this scheme, four beams, beam 0 – 3, are cycled at RB level and two pairs of two co-phase values {1, -1} and {j, -j} are cycled at RE level in two consecutive RBs, k and k + 1. The co-phase values {1, -1} are cycled in RB k and the co-phase values {j, -j} are cycled in RB k + 1. The set of co-phase values changes from {1, -1} to {j, -j} in RBs k and k + 1, respectively. Note that the same beam is considered in RBs k and k + 1, i.e., beam cycling is performed in a group of 2 consecutive RBs.

- Scheme 3: In this scheme, four beams, beam 0 – 3, are cycled at RB level and four pairs of two co-phase values {1, j}, {j, -1}, {-1, -j}, and {-j, 1} are cycled at RE level in four consecutive RBs, k to k + 3. The co-phase values {1, j} are cycled in RB k, the co-phase values {j, -1} are cycled in RB k +1, the co-phase values {-1, -j} are cycled in RB k + 2, and the co-phase values {-j, 1} are cycled in RB k + 3. The set of co-phase values changes from RBs k to k + 3, respectively. Note that the same beam is considered in RBs k to k + 3, i.e., beam cycling is performed in a group of 4 consecutive RBs.

- Scheme 4: This example is similar to Scheme 2 and 3 except that six pairs of two co-phase values {1, j}, {j, -1}, {-1, -j}, {-j, 1}, {1, -1}, and {j, -j} are cycled at RE level in six consecutive RBs, k to k + 3. Note that the same beam is considered in RBs k to k + 5, i.e., beam cycling is performed in a group of 6 consecutive RBs.

Observation:

- Schemes 1 and 2 can be used for rank-2 pre-coder cycling in which two co-phase values {1, -1} and {j, -j} are used for the two layers.

- The required number of DMRS sequences per RB is 4 for Scheme 0 and is 2 for Scheme 1 – 4.

# Simulation results

To compare the performance of candidate semi-OL schemes in Table 1, non-full-buffer system-level evaluation is carried out for UMa-500m channel model (36.814) in heavy (70% target RU) traffic loading scenario, where UE speed is assumed to be 120 kmph. The SU-MIMO rank 1 transmission with SB scheduling (where SB is 4 PRBs) is considered in the simulation. The detailed results are provided in Table 3 and Table 4 in the Appendix. The results are provided for 16 antenna ports with (N1,N2) = (2,4) and 32 antenna ports with (N1,N2) = (2,8), where we assume that the first dimension is horizontal and the second dimension is vertical. The downtilt angles in the elevation domain are chosen according to [5]. In these simulations, full-port non-precoded CSI-RS is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling have been used. The relevant simulation parameters are enlisted in Table 2. The rest of the simulation assumption is according to [5].

The performance gains with close-loop (CL) transmission as reference are shown in Figure 2 - Figure 7 for Codebook-Config 2, 3, and 4, and 16 and 32 ports. From these results, we can make the following observation.

Observation:

- Precoder cycling based semi-open-loop transmission (Scheme 0 – 4) shows significant performance gain over closed loop transmission: ~10-18% avg. UPT gain, ~27-65% 5% UPT gain.

- RE level cycling of two pairs of two co-phase {1,-1},{j,-j} (Scheme 2) achieves the best performance among the four schemes (Scheme 1 – 4) for co-phase cycling of two co-phase; and its performance is close to co-phase cycling of four co-phase (Scheme 0).

- RE level cycling of two co-phase {1,-1} (Scheme 1) shows performance loss when compared with four co-phase {1,j,-1,-j} (Scheme 0): ~1-2% loss in avg. UPT, ~5-10% loss in 5% UPT.

Based on these observations, we make the following proposal.

Proposal: For semi-open-loop transmission, RB level beam cycling and RE level co-phase cycling is supported. For co-phase cycling, at least one of the following two is supported:

- Four co-phase values {1,j,-1,-j} are cycled in an RB;

- Two pairs of two co-phase values {1,-1},{j,-j} are cycled in two consecutive RBs.

Figure 2: (N1,N2) = (2,4), Config = 2

Figure 3: (N1,N2) = (2,4), Config = 3

Figure 4: (N1,N2) = (2,4), Config = 4

Figure 5: (N1,N2) = (2,8), Config = 2

Figure 6: (N1,N2) = (2,8), Config = 3

Figure 7: (N1,N2) = (2,8), Config = 4

# Conclusions

In this contribution, simulation results are provided for DMRS based semi-open-loop transmission scheme relying on partial PMI feedback, proposed in [2-4]. The observations and proposals made are summarized as follows.

Observation:

- Precoder cycling based semi-open-loop transmission (Scheme 0 – 4) shows significant performance gain over closed loop transmission: ~10-18% avg. UPT gain, ~27-65% 5% UPT gain.

- RE level cycling of two pairs of two co-phase {1,-1},{j,-j} (Scheme 2) achieves the best performance among the four schemes (Scheme 1 – 4) for co-phase cycling of two co-phase; and its performance is close to co-phase cycling of four co-phase (Scheme 0).

- RE level cycling of two co-phase {1,-1} (Scheme 1) shows performance loss when compared with four co-phase {1,j,-1,-j} (Scheme 0): ~1-2% loss in avg. UPT, ~5-10% loss in 5% UPT.

Proposal: For semi-open-loop transmission, RB level beam cycling and RE level co-phase cycling is supported. For co-phase cycling, at least one of the following two is supported:

- Four co-phase values {1,j,-1,-j} are cycled in an RB;

- Two pairs of two co-phase values {1,-1},{j,-j} are cycled in two consecutive RBs.

# References

- RAN1#86b, Chairman’s notes

- R1-1612421, “Transmission scheme for semi-open-loop,” Samsung

- R1-1612422, “CSI reporting for semi-open-loop,” Samsung

- R1-1612523, “DMRS enhancements for semi-open-loop,” Samsung

- 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

Appendix: Simulation Assumptions and Results

Table 2: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Heavy load 70% Target RU, Lambda = 3) |
| Channel model | UMa-500m (36.814) |
| Number of BS (H,V) antenna elements | (8,8) for 16 ports and (8,16) for 32 ports, x-polarized, subarray partition |
| (N1,N2, P) | 16 ports: (2,4,2) and 32 ports: (2,8,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| UE speed | 120 kmph |
| SU/MU pre-coding | CB |
| Scheduling | SU, Proportional fair, SB scheduling (SB = 4 PRBs) |
| Channel estimation | Non-ideal |
| Transmission rank | 1 |
| Receiver | MMSE-IRC |
| Codebook | Rel. 13 Class A: (O1,O2) = (8,8), Codebook-Config = 2,3,4 |

Table 3: Non-full buffer simulation results: 16 ports

| Codebook-Config | Scheme | Avg. UPT | 50% UPT | 5% UPT | RU | Avg. UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | CL | 10.56 | 8.60 | 1.67 | 66.6% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 12.51 | 10.83 | 2.69 | 59.9% | 118.5% | 126.0% | 161.2% |
|  | Scheme 1 | 12.36 | 10.75 | 2.52 | 60.5% | 117.1% | 125.0% | 151.5% |
|  | Scheme 2 | 12.49 | 10.90 | 2.69 | 60.0% | 118.3% | 126.7% | 161.6% |
|  | Scheme 3 | 12.44 | 10.75 | 2.64 | 60.1% | 117.9% | 125.1% | 158.2% |
|  | Scheme 4 | 12.49 | 10.84 | 2.66 | 59.9% | 118.3% | 126.1% | 159.9% |
| 3 | CL | 10.46 | 8.47 | 1.62 | 66.9% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 12.43 | 10.85 | 2.65 | 60.1% | 118.8% | 128.2% | 163.6% |
|  | Scheme 1 | 12.23 | 10.43 | 2.50 | 60.8% | 117.0% | 123.2% | 154.3% |
|  | Scheme 2 | 12.36 | 10.74 | 2.56 | 60.3% | 118.2% | 126.8% | 157.6% |
|  | Scheme 3 | 12.37 | 10.65 | 2.61 | 60.4% | 118.3% | 125.8% | 161.0% |
|  | Scheme 4 | 12.30 | 10.67 | 2.54 | 60.5% | 117.7% | 126.0% | 156.4% |
| 4 | CL | 10.40 | 8.48 | 1.59 | 67.1% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 12.34 | 10.75 | 2.64 | 60.4% | 118.7% | 126.8% | 166.6% |
|  | Scheme 1 | 12.23 | 10.486 | 2.50 | 60.9% | 117.6% | 123.7% | 157.3% |
|  | Scheme 2 | 12.35 | 10.576 | 2.58 | 60.3% | 118.8% | 124.8% | 162.9% |
|  | Scheme 3 | 12.34 | 10.664 | 2.59 | 60.4% | 118.7% | 125.8% | 163.0% |
|  | Scheme 4 | 12.30 | 10.648 | 2.49 | 60.5% | 118.3% | 125.6% | 156.9% |

Table 4: Non-full buffer simulation results: 32 ports

| Codebook-Config | Scheme | Avg. UPT | 50% UPT | 5% UPT | RU | Avg. UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | CL | 13.32 | 11.89 | 3.09 | 56.7% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.90 | 13.71 | 4.23 | 52.2% | 111.9% | 115.4% | 136.9% |
|  | Scheme 1 | 14.73 | 13.42 | 4.09 | 52.9% | 110.6% | 112.9% | 132.4% |
|  | Scheme 2 | 14.90 | 13.75 | 4.29 | 52.2% | 111.9% | 115.7% | 138.6% |
|  | Scheme 3 | 14.86 | 13.64 | 4.23 | 52.4% | 111.6% | 114.7% | 136.9% |
|  | Scheme 4 | 14.89 | 13.75 | 4.26 | 52.3% | 111.8% | 115.7% | 137.6% |
| 3 | CL | 13.28 | 11.86 | 3.13 | 56.7% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.81 | 13.57 | 4.20 | 52.4% | 111.6% | 114.5% | 134.2% |
|  | Scheme 1 | 14.63 | 13.27 | 3.98 | 53.1% | 110.2% | 111.9% | 127.2% |
|  | Scheme 2 | 14.79 | 13.54 | 4.17 | 52.6% | 111.4% | 114.2% | 133.2% |
|  | Scheme 3 | 14.79 | 13.60 | 4.16 | 52.6% | 111.3% | 114.7% | 133.1% |
|  | Scheme 4 | 14.78 | 13.52 | 4.15 | 52.6% | 111.3% | 114.0% | 132.7% |
| 4 | CL | 13.19 | 11.68 | 3.00 | 57.0% | 100.0% | 100.0% | 100.0% |
|  | Scheme 0 | 14.73 | 13.45 | 4.18 | 52.6% | 111.6% | 115.1% | 139.4% |
|  | Scheme 1 | 14.54 | 13.28 | 3.98 | 53.3% | 110.2% | 113.7% | 132.7% |
|  | Scheme 2 | 14.74 | 13.47 | 4.14 | 52.6% | 111.7% | 115.3% | 138.1% |
|  | Scheme 3 | 14.70 | 13.43 | 4.08 | 52.8% | 111.4% | 115.0% | 135.9% |
|  | Scheme 4 | 14.69 | 13.51 | 4.11 | 52.8% | 111.3% | 115.7% | 137.1% |