3GPP TSG RAN WG1 Meeting #87	R1-1612892

Reno, Nevada, USA 14th - 18th November 2016

Agenda Item:	7.1.2.6

Source: 	Sony

Title:	UL mobility scenarios for NR

Document for:	Discussion / Decision

# Introduction

RAN2 have agreed that DL-based mobility will be supported and are studying the benefits of UL based mobility compared to DL based mobility, based on performance analysis. The following agreement has been made by RAN2:

Concerning RRC driven UL-based connected mode mobility:

- For connected active state mobility, DL-based handover is supported, and UL based mobility can continue to be studied.

- For connected inactive state, DL-based reselection is supported, and UL-based mobility can also be studied

- Benefits of UL based mobility, compared to DL based mobility, should be studied with performance analysis

The outcome of the RAN2 study will impact the work that is being carried out in RAN1 on mobility. Initial investigations in RAN1 have shown that UL-based mobility can provide significant benefits to the UE [1], [2].

RAN2 has already agreed that DL-based mobility will be supported in both CONNECTED_Active and CONNECTED_Inactive modes. Given this, it is assumed that UL mobility will be applied in scenarios where there are benefits to UL mobility: in other cases, the baseline downlink mobility will be supported.

Given the limited time left in the NR study item, it is important that both RAN1 and RAN2 focus their studies on UL mobility on those scenarios where UL mobility is most likely to be beneficial and used. This document hence considers those scenarios where UL mobility is expected to provide a gain.

Again, given the limited time that is left in the NR study item, it is important for RAN1 to discuss the Layer 1 aspects of UL mobility, such that the RAN1 physical layer design is compatible with the decisions that RAN2 will take based on their study of UL mobility. The RAN1 design for UL mobility will also necessarily have an impact on the comparative performance analysis that RAN2 have committed to. It is thus important for RAN1 to have an initial design of UL mobility to allow RAN2 to fully progress their comparative study. A companion document [3] gives our opinions on some of the design areas that need to be considered by RAN1.

# Scenarios

## 2.1	High speed

The most significant advantage claimed for UL mobility is that the UE power consumption can be improved considerably when the UE is moving at high speed. In [1] it has been shown that when particular schemes are used there is a potential saving of up to 50% at high speed, while there may be some disadvantage for stationary UEs. From a UE vendor point of view, this is an extremely attractive improvement and we expect that RAN2 will study this aspect with a high priority.

Proposal 1: RAN1 considers high speed scenarios in its UL mobility design.

## 2.2	Dense Urban

It has been observed [2] that in dense urban environments, UL-based mobility can reduce the amount of downlink common signaling required and reduce the UE power consumption. In the CONNECTED_Inactive state, UL mobility can reduce the amount of paging traffic (since the network can page the UE from a known good TRP rather than within a wider Tracking Area). In the CONNECTED_Active state, UL mobility can reduce the measurement and signaling burden from the UE.

Paging is a fundamental aspect of the Phase 1 NR study. In a dense urban scenario, there are more eNodeBs and more UEs in a geographic area. Depending on the division of paging resources in a Tracking Area between UEs, UEs may be woken up more frequently to unnecessarily read paging channels, leading to reduced battery life. In contrast, UL Mobility can page UEs in the TRP that received its UL beacon transmission with the highest quality. Hence RAN1 needs to study the design of the paging procedure, including dimensioning the paging channels and the resources required for paging. Since UL mobility has the potential to significantly reduce the paging load on the network and UE, and given that the paging load is a function of the size of the Tracking Area and density of cells, it is important for RAN1 to consider the dense urban scenario when considering UL mobility.

Proposal 2: RAN1 considers the dense urban scenario in its UL mobility design.

## 2.3	High carrier frequency

At high carrier frequencies, the SINR can change very quickly due to the directivity of transmissions and the propagation losses through objects. If DL mobility were used in these scenarios, the UE would have to perform measurements very quickly in order to be able to handover at a rate that is higher than the rate at which the channel can deteriorate. UL mobility could reduce the rate of measurement and quantity of measurement report signaling, thus leading to improved battery lifetime, lower handover failure and greater signaling efficiency.

Beamforming is likely to be employed at high carrier frequencies. The number of beams visible to the UE at the cell edge may be considerable and beams may arrive at the UE primarily via non-LOS paths. Thus L1 beam tracking to the UE may become difficult. The lower data rate requirements of the UL mobility signals (compared to the data rate requirements for the user service or handover measurement commands) may ease the requirements on UL mobility signals, making UL mobility signals more appropriate for high carrier frequency scenarios.

The benefits of UL mobility in high speed / low frequency scenarios have already been shown [2].

Proposal 3: RAN1 considers both high carrier frequency and low frequency scenarios in its UL mobility design.

## 2.4	RRC Connection Modes

RAN2 has already agreed that it will compare the performance of UL mobility and DL mobility in both CONNECTED_Active and CONNECTED_Inactive modes. This has an impact in RAN1 from the perspective of the identities used by the UEs, the reference signals assigned to the UEs, the power control and timing / frequency synchronisation accuracy of the UEs. Hence RAN1 should consider the scenarios where the UE is in either CONNECTED_Active or CONNECTED_Inactive mode.

It has been agreed in RAN1#86bis/Lisbon that DL cell-mobility is supported in IDLE mode. We do not currently see the need to support UL mobility in IDLE mode, so IDLE mode operation does not have to be a scenario for UL mobility at the moment.

Proposal 4: RAN1 considers UL mobility design in RRC CONNECTED_Active and RRC CONNECTED_Inactive modes.

## 2.5	Relays

An NR relay is not necessarily in a fixed piece of equipment with a power supply: instead an NR device could operate a relay functionality (e.g. a smartphone relaying to a wearable device). In such cases, it may be undesirable to transmit synchronisation and system information from the relay, since this has negative power consumption implications. In such cases, power consumption of the relay device (e.g. smartphone) may be reduced when UL mobility is applied. It is assumed that the remote device can synchronise to the host eNodeB of the relay.

In a system operating with many relays (e.g. many smartphones relaying to remote devices), the protocol for determining which relays should page the remote devices may become complicated with DL mobility (given that there will be many devices operating as relays in the system). For the UL mobility case, it is pretty obvious to the system which relays are suitable for paging remote devices, hence the paging procedure with UL mobility is probably simplified, compared to the DL mobility case.

UL mobility hence has the potential to reduce power consumption in the relay device and to simplify the paging procedure and is hence a valid scenario for UL mobility.

Proposal 5: RAN1 considers UL mobility design in the relay scenario.

.

## 2.6	URLLC

In previous sections, it has been observed that, for the current handover architecture, handover reliability is impaired in some environments, such as dense urban, high speed and high carrier frequency deployments. Unreliable handover has a clear impact on the ability to support URLLC communications. In our opinion, URLLC can be used over a broad range of applications that is not limited to small packet mission critical MTC applications. Virtual reality, gaming and other new applications also fall into the scope of URLLC and need to provide reliable communications in handover situations. UL mobility has been proposed in RAN2 as a method to improved handover reliability and since reliability is a critical aspect of URLLC communication, URLLC is a scenario that is applicable to the UL mobility study.

Proposal 6: RAN1 considers the URLLC scenario in the UL mobility design.

## 2.7	mMTC

mMTC devices are constrained in many ways. Two significant constraints are battery limitations and UE complexity.

mMTC devices can operate in a variety of scenarios. In the reliable tracking scenario (e.g. the tracking of people or cars), the devices can be highly mobile (tracked people on high speed trains). These devices are typically highly battery constrained. As has been previously shown [1], savings of up to 50% can be achieved with UL mobility. These power savings are particularly useful for battery constrained mMTC devices.

When operating in high speed, dense urban or high carrier frequency scenarios, the UE measurement load increases for the case of DL mobility, both in terms of the number of measurements that have to be made and the frequency with which they have to be made. The processing loads associated with performing UE measurements may increase UE complexity. Hence the UE complexity of mMTC devices should be taken into account when designing the mobility solution for mMTC devices.

Proposal 7: RAN1 considers the mMTC scenario in the UL mobility design.

# Conclusion

This document has considered some of the scenarios where UL mobility can provide significant gain and that RAN1 needs to take into account when considering mobility for NR. The following proposals on UL mobility scenarios are made:

Proposal 1: RAN1 considers high speed scenarios in its UL mobility design.

Proposal 2: RAN1 considers the dense urban scenario in its UL mobility design.

Proposal 3: RAN1 considers both high carrier frequency and low frequency scenarios in its UL mobility design.

Proposal 4: RAN1 considers UL mobility design in RRC CONNECTED_Active and RRC CONNECTED_Inactive modes.

Proposal 5: RAN1 considers UL mobility design in the relay scenario.

Proposal 6: RAN1 considers the URLLC scenario in the UL mobility design.

Proposal 7: RAN1 considers the mMTC scenario in the UL mobility design.

# References

[1]	R1-1610167. “Comparative Power Analysis between UL and DL-based Mobility”. Qualcomm. RAN1#86bis. Lisbon, Portugal. 10-14 October 2016.

[2]	R1-1609437. “Scenarios and initial evaluation for UL-based RRM measurement and mobility”. Huawei, HiSilicon. RAN1#86bis. Lisbon, Portugal. 10-14 October 2016.

[3]	R1-1613011. “L1 Design Issues for UL mobility”. Sony. RAN1#87. Reno, Nevada, USA. 14-18 November 2016.