3GPP TSG RAN WG1 Meeting #87		R1-1613686

Reno, USA, 14th - 18th November 2016

Title:	draft LS on minimum system information

Response to:

Release:	Rel-14

Work Item:	FS_NR_newRAT

Source:	LG Electronics

To:	RAN2

Copy:

Contact Person:

Name:	Eunsun Kim

- E-mail Address:	esun.kim@lge.com

Attachments:	None

Overall description

Followings are RAN1 agreements on minimum system information transmission in RAN1#87.

- Consider followings for minimum system information transmission: - NR-PBCH is a non-scheduled broadcast channel carrying minimum system information with fixed payload size and periodicity predefined in the specification depending on carrier frequency range - Alt. 1: NR-PBCH carries a part of minimum system information - Alt 1-1 : remaining minimum system information is transmitted via other channel at least partially indicated by NR-PBCH - Alt 1-2: Remaining minimum system information is transmitted via other channel not indicated in NR-PBCH - Alt. 2: NR-PBCH carries all of minimum system information - Study further NR-PBCH design examples with the following clarification of the agreements - Example 1 for Alt. 1-1: NR-PBCH carries a part of minimum system information including information necessary for the UE to receive channel carrying remaining minimum system information - Example 2 for Alt. 1-1: NR-PBCH carries information necessary for the UE to perform initial UL transmission (not limited to NR-PRACH, e.g. PRACH msg. 1) and possibly information necessary to receive the response to initial UL transmission (e.g., PRACH msg. 2) in addition to information in Example 1 - Example 3 for Alt. 2: NR-PBCH carries all minimum system information - Example 4 for Alt. 1-2: NR-PBCH carries information necessary for the UE to perform initial UL transmission (not limited to NR-PRACH, e.g. PRACH msg. 1) and information necessary to receive the response to initial UL transmission (e.g. PRACH msg. 2) - Information necessary to receive remaining minimum system information is provided after initial UL transmission - Other examples are not precluded - For study of the cases where NR-PBCH carries a part of minimum system information, consider the following alternatives (or combinations) for the minimum system information other than those included in NR-PBCH : - Alt. 1: NR defines the additional channel as secondary broadcast channel - Secondary broadcast channel may be different design from NR-PBCH, e.g. payload size, resource mapping, periodicity and etc. - FFS on transmission: beam-specific, cell-specific, and/or TRP-specific, etc. - Alt. 2: The remaining information is transmitted in shared downlink channel similar to ,e.g. NR-PDSCH - FFS on transmission: UE-specific, UE group-specific, beam-specific, cell-specific, and/or TRP-specific, etc. - Note: This does not preclude defining of other mechanisms transmitting Other SI

However, the contents or size of minimum system information is not clear and RAN1 wonders whether it can be delivered by on demand, which affects NR-PBCH design aspect in RAN1.

Actions

RAN1 respectfully asks RAN2 to clarify

- the contents and RAN2 preferred size of minimum system information

- whether it can be delivered by on demand.

Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting NR AdHoc	Jan 16-20, 2016			US

TSG RAN WG1 Meeting #88	Feb 13-17, 2016		      Athens