3GPP TSG-RAN WG1 Meeting #95	Tdoc R1-1812131

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	6.2.2.5

Source:	Ericsson

Title:	Support of quality report in Msg3 for non-anchor access in NB-IoT

Document for:	Discussion and Decision

# Introduction

In the Rel-16 work item description (WID) on “Additional enhancements for NB-IoT”, one of the objectives is to improve the multi-carrier operation as follows [1].

- Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2]

In RAN1#94, the following agreements and working assumptions were made that

“

Agreement

For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%

- FFS: Whether the details on the hypothetical NPDCCH are specified or not

Working Assumption

For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR)

- FFS: Whether the UE performs measurement on other carriers

Agreement

For non-anchor access, RAN1 further studies how UEs report the measured channel quality

”.

In RAN1#94bis, the following agreements were made that

“

Agreement

RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access.

Agreement

From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed

Agreement

RAN1 does not define measurement reference resource for non-anchor access.

For further study:

The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access.

- For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE

- PUR

”.

In Rel14 NB-IoT, multi-carrier operation in idle mode for paging and random access were introduced. During the maintenance of Rel14 NB-IoT, the support of msg3 quality reporting was brought up and discussed [1]. It was agreed that channel quality reporting for anchor carrier would be introduced, and RAN2 has introduced the corresponding signalling based on the input from RAN4. In an LS from RAN4 to RAN1 [3], RAN4 informs RAN1 and RAN2 the number of candidate values should be reported, and RAN4 would continue to discuss the actual values reported in MSG3. In this contribution, we continue to discuss the support of Msg3 quality reporting for non-anchor access.

# 2	Discussion

During the maintenance discussion in Rel14 NB-IoT, both anchor and non-anchor DL quality reporting in msg3 were discussed [4]. However, due to time limitations, it was only agreed that DL quality report for the anchor carrier was introduced. In RAN4, for anchor carrier, it has introduced the NPDCCH repetition level for CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB is chosen from the supported NPDCCH repetition levels [6][7]. To be more specific, in TS36.133, it defines that

“

#### 9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1

Table 9.1.22.15-1: Downlink channel quality measurement report mapping of CQI-NPDCCH-NB when the DL channel quality reporting is supported [6]

| Reported value | NPDCCH repetition level |
| --- | --- |
| noMeasurement | No measurement reporting |
| candidateRep-A | 1 |
| candidateRep-B | 2 |
| candidateRep-C | 4 |
| candidateRep-D | 8 |
| candidateRep-E | 16 |
| candidateRep-F | 32 |
| candidateRep-G | 64 |
| candidateRep-H | 128 |
| candidateRep-I | 256 |
| candidateRep-J | 512 |
| candidateRep-K | 1024 |
| candidateRep-L | 2048 |

Table 9.1.22.15-2: Downlink channel quality measurement report mapping of CQI-NPDCCH-Short-NB when the DL channel quality reporting is supported [6][7]

| Reported value | NPDCCH repetition level |
| --- | --- |
| noMeasurements | No measurement reporting |
| candidateRep-1 | Rmax/8 (Note 1) |
| candidateRep-2 | Rmax (Note 2) |
| candidateRep-3 | 4×Rmax (Note 3) |
| Note 1: When Rmax is less than 8, set candidateRep-1 to 1. Note 2: When Rmax is 1, set candidateRep-2 to 2. Note 3: When Rmax is more than 512, set candidateRep-3 to 2048. |  |

”.

The same principle for the channel quality reporting should be applied on the non-anchor measurement also. Therefore, RAN2 should study the corresponding signalling support, i.e., where and how many bits are available for the reporting, and RAN4 should define the proper channel quality metric and new requirements/test cases (if needed).

Send LS to RAN2 and request their studies on the available number of bits can be used for DL channel quality reporting in msg3 for non-anchor carrier for both EDT and non-EDT case.

In the current system, if a DL carrier is configured for RAR, regardless whether there are UEs attempting to access the system, there are always NRS in the RAR window and in subframes before the RAR window. Therefore, there are enough period NRS on the DL carrier for the UE to measure the channel quality, and those NRSs are associated with the type 2 search space. However, the UE may need to average over several occasions, e.g., in bad coverage, to obtain more reliable DL channel quality.

On the non-anchor carriers where RAR is sent, there are enough NRS subframes for the UE to perform the measurements.

Figure 1 Random access procedure in NB-IoT

Figure 2 UE procedure for reporting channel quality in MSG3

The random access procedure in NB-IoT is depicted in Figure 1 and Figure 2. In Rel-14 NB-IoT enhancement, non-anchor carrier can be used for paging and random access. In the case of receiving RAR on a non-anchor carrier, after sending msg1, the UE monitors msg2 on a carrier that is configured in the SIB. The carrier that carries msg2 is associated with the carrier where the UE sends msg1. However, the DL carrier that the UE receives msg2 may not necessary be the carrier it is assigned in connected mode. The current NB-IoT system offers the flexibility that during the RRC setup, the network can assign a different DL carrier to the UE than the one it monitors the RAR for load balance or interference management purposes. Currently, in idle mode, the UE is only required to measure the quality on the anchor carrier. Hence, if the UE reports the DL quality measured on the carrier different than the one it is assigned later for connected mode, the reporting becomes useless. Notice that that NRSRP with carrier specific NRS-offset is not sufficient, since it does not capture interference and the quality as indicated in [5]. Therefore, it can be beneficial the UE can report the channel quality of a DL carrier that is indicated by the network. However, the challenge here is that the UE cannot make any assumptions in idle mode regarding the NRS presence if this carrier is not used for paging or RAR. It would be difficult for the UE to monitor RAR on one carrier but to report the channel quality of another carrier.

Therefore, it is proposed that

When an NB-IoT UE is capable of the msg3-based channel quality reporting and non-anchor for RAR is configured, the UE should report the downlink quality of the carrier indicated by the network. In the absence of such indication, the UE should report the downlink quality of the carrier where the UE monitors msg2.

In the carrier indicated by the network other than the carrier where the UE monitors msg2, specify the NRS patterns that the UE can use for the channel quality measurements in idle mode.

In RAN1#94bis, it is proposed that

“

The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access.

- For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE

- PUR

”

For the PUR, whether to include the DL channel quality report depends on the PUR scheme. For contention based PUR, it is better to keep the message small, and it is likely it will only be used for small data transmission in the UL. Therefore, including the DL channel quality report may not be beneficial. However, for dedicated PUR, there can be use cases that the eNB would like to setup connection to send UE DL data after the UE finishes its UL transmission. In such cases, inclusion of the DL channel quality reporting is useful.

Depending on the use cases, further study whether it is beneficial to include DL channel quality reporting when PUR is used.

In the current spec, TS36.300 Section 10.1.5.1, it only requires msg3 to convey the C-RNTI of the UE for PDCCH ordered NPRACH. Therefore, the inclusion of other information is not mandatory in the msg3 associated with PDCCH ordered NPRACH, and no RRC messages should be included. In Rel-14, the channel quality reporting is included in RRC message, and therefore a Rel-14 NB-IoT UE would not report the DL channel quality in a msg3 associated with PDCCH ordered NPRACH. However, for Rel-15 non-anchor case, this is not clear as we can add in support for such reporting. The benefits of include such report in connected mode is obvious, as it gives information to the eNB to adjust the scheduling parameters if necessary. However, there are extra overhead associated, i.e., the UE needs to perform the random-access procedures first, and then report the channel quality. This is at a cost of the UE energy, and it consumes both UL NPRACH resources and DL resources for RAR. Furthermore, the UE may also need to listen to the RAR on a different DL carrier, depending on the instruction from the DCI used for PDCCH ordered NPRACH. This requires additional frequency switching at the UE, which results in significant delays in the reporting. Therefore, it is preferred that if the DL channel quality can be directly reported in connected mode.

In connected mode, support direct DL channel quality report instead of channel quality reporting in msg3 associated with PDCCH ordered NPRACH in connected mode.

# Conclusion

In the previous sections we made the following observation:

Observation 1	On the non-anchor carriers where RAR is sent, there are enough NRS subframes for the UE to perform the measurements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send LS to RAN2 and request their studies on the available number of bits can be used for DL channel quality reporting in msg3 for non-anchor carrier for both EDT and non-EDT case.

Proposal 2	When an NB-IoT UE is capable of the msg3-based channel quality reporting and non-anchor for RAR is configured, the UE should report the downlink quality of the carrier indicated by the network. In the absence of such indication, the UE should report the downlink quality of the carrier where the UE monitors msg2.

Proposal 3	In the carrier indicated by the network other than the carrier where the UE monitors msg2, specify the NRS patterns that the UE can use for the channel quality measurements in idle mode.

Proposal 4	Depending on the use cases, further study whether it is beneficial to include DL channel quality reporting when PUR is used.

Proposal 5	In connected mode, support direct DL channel quality report instead of channel quality reporting in msg3 associated with PDCCH ordered NPRACH in connected mode.

# References

RP-181674, “WID revision: Additional enhancements for NB-IoT”, Huawei, 3GPP TSG RAN Meeting #81, Gold Coast, Australia, September 2018.

R1-1804087 Discussion on remaining issues for DL CE Level determination in NB-IoT, Source CMCC, 3GPP TSG-RAN WG1 Meeting #92 bis, Sanya, China, Apr. 16th-20th, 2018.

R1-1808005, LS response on NB-IoT downlink channel quality determination and report, Source RAN4, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, August 20th – 24th, 2018.

R1-1804168, Remaining issues on DL measurement reporting in NB-IoT, source Ericsson, 3GPP TSG-RAN WG1 Meeting #92 bis, Sanya, China, Apr. 16th-20th, 2018.

R1-1802031, Discussion on DL CE Level determination issue in NB-IoT, source CMCC, RAN1#92.

R4-1813669, Finalize MSG3-based channel quality report for NB-IoT, 3GPP TSG-RAN WG4 Meeting #88bis, Chengdu, China, 8 – 13 October 2018.

R4-1813504, Correction to downlink channel quality reporting, 3GPP TSG-RAN WG4 Meeting #88bis, Chengdu, China, 8 – 13 October 2018.