3GPP TSG RAN WG1 Meeting #95	R1-1812136

Spokane, USA, November 12-16, 2018

Agenda Item:	6.2.2.4

Source:	Huawei, HiSilicon

Title:	Performance enhancements for NB-IoT coexistence with NR

Document for:	Discussion and decision

# Introduction

Coexistence with NR

- Study NR and LTE specifications to identify possible issues related to coexistence of NB-IoT with NR [RAN4, RAN1, RAN2]

At RAN#80, the Rel-16 work item on additional enhancements for NB-IoT was approved. One of the objectives [1] in this work item is to study NR and LTE specifications to identify possible issues related to coexistence of NB-IoT with NR.

Observation

From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT

Agreement

RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

Agreement

The following aspects are studied to improve the performance of coexistence of NB-IoT with NR

- Resource reservation in NB-IoT

- Overlap of NR SSB with NB-IoT

In RAN1#94 and RAN1#94b, the observation and agreements below for NB-IoT coexistence with NR were reached. This contribution discusses enhancements for improving the performance of coexistence of NB-IoT with NR.

# Mechanism supporting coexistence

In NR, an SS/PBCH block consists of 240 consecutive subcarriers where PSS, SSS and PBCH are mapped within the SS/PBCH block. In the frequency domain, the SS/PBCH block should be placed in the synchronization raster SSREF which is determined by GSCN (Global Synchronization Channel Number). The range of GSCN is 2-26639 shown in Table 1 [2].

Table 1: GSCN parameters for the global frequency raster in NR

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note) | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N= 0:14756 | 7499 + N | 7499 – 22255 |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz N= 0:4383 | 22256 + N | 22256 – 26639 |
| NOTE: The default value for operating bands with SCS spaced channel raster is M=3. |  |  |  |

In NB-IoT, a NB-IoT PRB including anchor or non-anchor should be placed in the channel raster which is determined by EARFCN which may include an offset around channel raster [3] in the frequency domain. Due to the large range of GSCN and EARFCN, it is possible to place SS/PBCH block and a NB-IoT PRB without overlap by choosing the appropriate value of GSCN and EARFCN in the frequency domain. An illustration is shown in Figure 1.

Fig. 1 Coexistence of NB-IoT with NR SSB without overlap.

Observation 1: It is possible to avoid the overlap by properly placing a NR SSB and NB-IoT carrier(s).

# Potential enhancements

## 3.1	Overlap of NR SSB with NB-IoT

If a NB-IoT carrier can be placed within NR SS/PBCH block, then there is no limitation for the NB-IoT deployment to coexist with NR. But if overlapping of NR SSB with NB-IoT occurs, then collision issues need to be further studied. In the time domain, each NR frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9. For 15kHz SCS, the first symbol of SS/PBCH block(s) within a half-frame can be symbol 2 or 8 of the first or second subframe which is indicated by a bitmap ssb-PositionsInBurst. The periodicity can be {ms5, ms10, ms20, ms40, ms80, ms160}. If a NB-IoT anchor carrier is placed within a NR SS/PBCH block with 5ms periodicity, then some symbols of NPBCH and NPSS would collide with some symbols of NR SS/PBCH block. An example is shown in Figure 2. The NRS may also collide with the NR SSB.

Fig. 2: Collision of NB-IoT channels/signals with NR SSB

Proposal 1: If a NB-IoT carrier can be placed within a NR SSB, the following aspects needs to be further studied to improve the performance of coexistence of NB-IoT with NR:

- Collision of NR SSB with NPBCH

- Collision of NR SSB with NPSS

- Collision of NR SSB with NRS

## 3.2	Resource reservation in NB-IoT

In the last meeting, it was agreed to further study resource reservation in NB-IoT to improve the performance of coexistence of NB-IoT with NR. One benefit for resource reservation in NB-IoT is to avoid the collision between NB-IoT transmission and NR transmissions.

In current NB-IoT system, there are multiple non-anchor carriers (i.e. multiple 180 kHz PRBs) for unicast, paging and RACH that can be placed within the whole NR CC (Component Carrier) bandwidth. When the multiple NB-IoT UEs are scheduled in multiple NB-IoT carriers, the NR transmissions should avoid the collision with NB-IoT transmissions. Generally, the NB-IoT transmission would last for a long time especially for deep coverage NB-IoT UEs. So in this case, some NB-IoT resources can be reserved to avoid overlapping with NR transmissions repeatedly, especially for URLLC services in NR which use a large number of contiguous RBs for a single transmission due to the 10-5 BLER target and tight latency requirements. In NB-IoT, when the resource is reserved, it means that NB-IoT UEs cannot use these reserved resources. Thus postponed or dropped operation can be considered.

From NB-IoT point of view, the reserved resource mechanism can also introduce forward compatibility for NB-IoT. A Rel-16 eNB can configure the resources as the reserved resources for Rel-16 NB-IoT UEs and beyond. Rel-16 and beyond NB-IoT transmission is postponed or dropped around these resources. Therefore, eNB does not need to handle the collision issue between Rel-16 transmission and beyond Rel-16 transmission, which complicates the scheduling and may impact the resource efficiency of NB-IoT.

Proposal 2: In Rel-16, configuration of reserved resources in NB-IoT which are postponed or dropped by NB-IoT UEs is considered to improve the performance of coexistence of NB-IoT with NR. FFS whether and how the reserved resources are used for NR UEs.

# Conclusion

In this paper, we discuss the coexistence of NB-IoT and NR. Some additional NB-IoT enhancements for improving the performance of coexistence of NB-IoT with NR are proposed to be further considered.

Observation 1: it is possible to avoid the overlap by properly placing a NR SSB and NB-IoT carrier(s).

Proposal 1: If a NB-IoT carrier can be placed within a NR SSB, the following aspects needs to be further studied to improve the performance of coexistence of NB-IoT with NR:

- Collision of NR SSB with NPBCH

- Collision of NR SSB with NPSS

- Collision of NR SSB with NRS

Proposal 2: In Rel-16, configuration of reserved resources in NB-IoT which are postponed or dropped by NB-IoT UEs is considered to improve the performance of coexistence of NB-IoT with NR. FFS whether and how the reserved resources are used for NR UEs.

# References

RP-181674, “WID Revision: Additional enhancements for NB-IoT”, Huawei, Gold Coast, Australia, September 2018.

TS 38.104

TS 36.104