3GPP TSG-RAN WG1 Meeting #95	 R1-1812167

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Transport Industry Scenario

Document for:	Discussion, Decision

# Introduction

After clarification of eURLLC study item a transport industry use case including remote driving was explicitly added [1]. In this contribution we evaluate reliability and latency performance of URLLC remote driving use case by system level simulations.

Assumptions on system level simulations are based on the conclusion in [2] with some modifications disclosed in appendix. The results show some statistic and percentage of users satisfying requirements.

# Discussion

According to [2], requirements for transport industry use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for transport industry use case

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Transport Industry (22.186: 5.5) Remote driving | 99.999 | 5 (end to end latency) Note: 3ms air interface latency | For UL: 2.5 Mpbs; Packet size 5220 bytes For DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model |
| Transport Industry (23.501, 22.261) Intelligent transport system (ITS) | 99.999 | 10(end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps, Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model |

In this contribution, we focus on the remote driving use case as more challenging one.

## System Level Simulation

Based on system level simulation assumption in Table A-1 in the appendix, we obtain DL and UL SINR distribution with periodic traffic arrival assumption as shown in Fig. 1 below. We note that more aggressive power control parameters in UL are applied to ensure URLLC probability. Another modification of agreed assumption is related to layout, where we show result for the hexagonal deployment instead.

Figure 1: DL and UL SINR distribution.

Figure 1 shows that UL SINR distribution is inferior to DL. Hence UL performance is the bottleneck in URLLC design.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- 3GPP RAN1 #94bis Chairman’s note

# Appendix

Table A-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid 7 sites with 3 sectors |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 8Tx/8Rx ports; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (3m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| Traffic | Periodic |
| UE power control | alpha = 0.8, target SINR = 50 dB |
| HARQ/repetition | Allowed |
| Channel estimation | Ideal |
| UE speed | 60 km/h |