3GPP TSG RAN WG1 Meeting #95	R1-18xxxxx

# Spokane, USA, November 12-16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on DCI format ambiguity within overlapped CSS and USS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1 meeting #94bis, the following agreement is achieved, - If DCI format 0-1 (format 1-1) have the same size as 0-0/1-0, add one bit of padding to DCI format 0-1 (format 1-1) to ensure different DCI sizes. The corresponding TP to 38.212 is as below: - Note: DCI size budget is counted after adding the padding bit. However, unnecessary overhead is introduced in DCI format 0-1 and/or DCI format 1-1, and it further complicates the calculation on DCI size budgets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If Fallback DCI and non-Fallback DCI have the same DCI payload size and with CRC scrambled by C-RNTI/CS-RNTI, UE expects that only one type of DCI format is transmitted among the overlapped PDCCH candidates: - When CSS with Fallback DCI and USS with non-Fallback DCI are overlapped, UE only expect that the PDCCH candidates for Fallback DCI is transmitted. - When USS with Fallback DCI and USS with non-Fallback DCI are overlapped, UE only expect that the PDCCH candidates for non-Fallback DCI is transmitted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can not calculate the payload size of DCI format 0-0/1-0 and DCI format 0-1/1-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Start of Text Proposal -------------------------------------------

10.1 UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_1 and a DCI format 1_1  from a USS set in a CORESET with the same index on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set and a second PDCCH candidate for a DCI format 0_1 and a DCI format 1_1  from a USS set in a CORESET with the same index on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	There is no PDCCH candidate from CSS has the same set of CCEs with the first PDCCH candidate and the second PDCCH candidate, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_1/1_1 associated with the second PDCCH candidate.

< Unchanged parts are omitted >

---------------------------------------------- End of Text Proposal -----------------------------------------------