3GPP TSG RAN WG1 Meeting #95	R1-18xxxxx

# Spokane, USA, November 12-16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the limit of BDs/CCEs for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1.The numerology of the monitored BDs/CCEs is not clear in current spec. 2.The numerology of the configured cell to determine the BDs/CCEs limit is not clear. 3.Per CC limit in current agreements are not captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The numerology of the monitored BDs/CCEs and the configured cell to determine the BDs/CCEs limit are clarified. Per CC limit in current agreements are captured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The numerology of the monitored BDs/CCEs and the configured cell to determine the BDs/CCEs limit are not clear. Per CC limit in current agreements not captured in current spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ----------------------------------------------- Start of Text Proposal -----------------------------------------

< Unchanged parts are omitted >

## 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , for an active DL BWP with SCS configuration  for a UE per slot for operation with a single serving cell.

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration  that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number  of monitored non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

If a UE

-	is capable for operation with carrier aggregation with a maximum of 4 downlink cells or indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  downlink cells with DL BWPs having SCS configuration  where  or , respectively, and

-	for cross-carrier scheduling over  downlink cells, including a scheduling cell, from the  downlink cells where the scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration

Where the SCS configuration of the active cell is the SCS configuration of the active DL BWP and the SCS configuration of the inactive cell is the SCS configuration of the DL BWP with the firstActiveDownlinkBWP-Id.

the UE is expected to be able to monitor  PDCCH candidates and  non-overlapped CCEs per slot on the active DL BWP of the scheduling cell. The case of  corresponds only to self-scheduling for the scheduling cell.

If a UE

-	indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells, and

-	is configured with  downlink cells with DL BWPs having SCS configuration  where , and

the UE is expected to be able to monitor a total of  PDCCH candidates and a total of  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

A UE does not expect to monitor more than  PDCCH candidates and non-overlapped CCEs per slot for the active DL BWP(s) of each scheduled cell.

A UE does not expect to be configured CSS sets that result to corresponding total numbers of monitored PDCCH candidates and non-overlapped CCEs per slot that exceed the corresponding maximum numbers per slot.

For same cell scheduling or for cross-carrier scheduling where a scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration, a UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per slot on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot.

For cross-carrier scheduling, the number of monitored PDCCH candidates and the number of non-overlapped CCEs per slot are separately counted for each serving cell.

< Unchanged parts are omitted >

## ----------------------------------------------- End of Text Proposal -----------------------------------------