3GPP TSG RAN WG1 Meeting #95	R1-18xxxxx

# Spokane, USA, November 12-16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CCE indexes for cross-carrier scheduling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current spec, same search space ID linkage between scheduling cell and scheduled cell in the function of CCE indexes calculaltion results in more restrictions on the search space set configuration of a scheduled cell and in contrary to the BWP configuration flexibility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The functions according to different search space set of scheduling cell and scheduled cell are used to calculate the CCE indexes of the PDCCH candidates for scheduling cell and scheduled cell respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | More restrictions on the search space set configuration of a scheduled cell. And the restrictions are in contrary to the BWP configuration flexibility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ----------------------------------------------- Start of Text Proposal -----------------------------------------

< Unchanged parts are omitted >

## 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

For a search space set  sm associated with CORESET ,

- the CCE indexes for aggregation level  corresponding to PDCCH candidate  msm,nCI of the search space set in slot  nsm,fμ for an active DL BWP of a scheduling serving cell corresponding to carrier indicator field value  are given by

L∙Yp,nsm,fμ+msm,nCI∙NCCE,pL∙Msm,maxL+nCImodNCCE,pL+i

- the CCE indexes for aggregation level  corresponding to PDCCH candidate   msn,nCI of the search space set in slot nsm,fμ for an active DL BWP of a scheduled serving cell corresponding to carrier indicator field value  are given by

L∙Yp,nsm,fμ+msn,nCI∙NCCE,pL∙Msm,maxL+nCImodNCCE,pL+i

where

for any CSS, ;

for a USS, , ,  for ,  for ,  for , and ;

;

is the number of CCEs, numbered from 0 to , in CORESET

is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS,

msm,nCI=0,…,Msm,nCIL-1, where  Msm,nCILis the number of PDCCH candidates the UE is configured to monitor for aggregation level  of a search space set  sm for a scheduling serving cell corresponding to

msn,nCI=0,…,Msn,nCIL-1, where Msn,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level  of a search space set sn for a scheduled serving cell corresponding to

for any CSS, Msm,maxL=Msm,0L

for a USS, Msm,maxL is the maximum of  Msj,nCIL over all configured  values for a CCE aggregation level  of search space set sj which is corresponding to sm in the active BWP of the cell indicated by

the RNTI value used for  is the C-RNTI.

< Unchanged parts are omitted >

## ----------------------------------------------- End of Text Proposal -----------------------------------------