3GPP TSG-RAN WG1 Meeting #95 	R1-18xxxxx

Spokane, November 12th –16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE beam switching restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The maximum number of Tx/Rx beam switching in one slot is reported by UE feature 2-27, but there is no description on how the Rx/Tx beam switching will be counted at UE side. In other words, gNB does not know how not to exceed UE capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding descriptions on how the Tx/Rx beam switching will be counted at UE side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to count Tx/Rx beam switching is not aligned between gNB and UE, and UE feature 2-27 does not provide useful information to gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.1.5	Antenna ports quasi co-location

< Unchanged parts are omitted >

Within a given slot, a UE is not expected to be configured with a number of spatial domain filter changes that is larger than the reported UE capability maxNumberRxTxBeamSwitchDL. When counting the number of spatial domain filter changes within a slot, the following are assumed:

- If a UE is configured with SS-PBCH block(s) for L1-RSRP measurement, the UE should apply the same spatial domain filter to receive all OFDM symbols carrying one SS-PBCH block.

- If a UE is configured with a CSI-RS resource set with high layer parameter repetition set to be “ON”, the UE is assumed to apply different spatial domain filters to receive different CSI-RS resources in the CRI-RS resource set.

- If a UE is configured with CSI-RS resource that has no TCI state configured or SRS resource that has no spatial relation configured, the UE is assumed to apply a spatial domain reception/transmission filter that is different from those already used in the current slot.

< Unchanged parts are omitted >

References

R1-1812064, RAN1 NR UE feature list, RAN1#94bis, Chengdu, China, 8th – 12th October 2018.