3GPP TSG RAN WG1 Meeting #95                                                                           R1-1812255

Spokane, USA, November 12th – 16th, 2018

Source:              ZTE

Title:                   CSI Enhancement for MU-MIMO Support

Agenda Item:     7.2.8.1

Document for:   Discussion and Decision

Introduction

In RAN#80 meeting, the WID of NR MIMO enhancement was approved in [1]. In the detailed scope of NR MIMO enhancement, the following points propose MU-MIMO enhancements relevant to Type II CSI feedback.

- Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2

In RAN1#94bis, the following agreements are achieved on Type II CSI enhancements for MU-MIMO.

Agreement

On the issue of Type II overhead reduction (rank 1, 2), to further progress, interested companies are to submit evaluation results (especially performance-overhead tradeoff) in RAN1#95 once the evaluation methodology is finalized in RAN1#94B.

- Focus on proposals based on linear combination codebook as in Rel-15

- Also investigate potential common ground between frequency domain and time domain approaches, e.g. merging these two into one category

Agreement

The study and, if needed, work on Type II higher rank extension is performed as follows:

- Only for rank 3 and 4 by taking into account the outcome of Type II overhead reduction for rank 1-2

- Simple extension of Rel.15 Type II without any additional optimization (which results in ~3-4x overhead over rank-1) is ruled out

In this contribution, we give our views on the Type II CSI enhancements in the above MU-MIMO scope.

Type II overhead reduction

2.1 Type II overhead reduction in Rel-15

In NR Rel-15, CSI feedback framework is flexible to support various deployment and implementation requirements. Based on the CSI framework, Type II CSI overhead can be controlled flexibly.

One approach to achieve semi-static Type II overhead reduction is through CSI reporting band configuration. In NR Rel-15, a BWP is divided into numerous sub-bands based on the sub-band size determined by a configuration parameter and BWP size. CSI reporting band is defined as a selection of the sub-bands in the BWP. The selection of sub-bands is realized by a bitmap, which can be contiguous or non-contiguous. Hence the number and location of sub-bands, as well as the sub-band CSI overhead, can be controlled by CSI report band configuration. To be more specific, beam information in Type II is wide-band reported, and phase/amplitude is sub-band reported. The major overhead of Type II report is the sub-band phase and amplitude overhead. With a proper CSI reporting band configuration, the overhead of Type II sub-band report can be reduced along with the number of selected sub-bands. For example, if CSI reporting band is configured as in Fig. 1, where the even sub-bands are selected based on a bitmap {1010101010}, half of the Type II sub-band overhead can be reduced compared with including the entire BWP in the CSI reporting band. Upon receiving the Type II report, gNB can derive the amplitude/phase of the non-reported sub-bands via interpolation.

Fig.1 CSI reporting band configuration

Observation 1: NR Rel-15 CSI reporting band configuration can achieve semi-static overhead reduction for Type II feedback.

Another approach to achieve dynamic Type II overhead reduction is through partial CSI omission. UE can omit the sub-band payload based on a comb pattern as in Fig.1 if the allocated PUSCH resource is not sufficient to transmit the entire Type II CSI with a specific code rate. If the allocated PUSCH cannot carry the full CSI, UE would omit the sub-band Part 2 CSI, which include the sub-band phase and amplitude for Type II, in odd sub-bands. The total sub-band overhead of Type II depends on RI a lot, which changes dynamically with the channel variation. gNB does not know the exact payload before decoding RI. With this mechanism, gNB can perform PUSCH resource allocation more flexibly, and CSI overhead can be reduced dynamically. Upon receiving the Type II report, gNB can derive the amplitude/phase of the non-reported sub-bands via interpolation.

Observation 2: NR Rel-15 partial CSI omission can achieve dynamic overhead reduction for Type II feedback.

Based on the above analysis, Type II CSI overhead can be controlled well in Rel-15. Thus when studying Type II enhancements for overhead reduction, we should consider the overhead reduction mechanism supported in Rel-15 as a baseline.

Proposal 1: Enhancements for Type II overhead reduction shall consider the overhead reduction mechanism in Rel-15 as a baseline

- Rel-15 overhead reduction mechanism includes flexible CSI reporting band configuration and partial CSI omission.

2.2 Enhancements for Type II overhead reduction

As mentioned above, the major Type II overhead comes from sub-band phase and amplitude. Based on previous study, for each beam’s phase in one layer is correlated in time domain, as the phase mainly depends on the delay of that path. Further, the frequency selectivity of amplitude is not strong, which means sub-band amplitudes also have frequency domain correlation. Therefore, to exploit frequency-domain correlation of the combination coefficients potentially provides solutions to further Type II overhead reduction. Except utilizing frequency-domain correlation of combination coefficients, other solid solutions haven’t been identified in previous study. Considering limited TU allocated for Rel-16 MIMO, we propose to focus on using frequency-domain correlation of phase/amplitude for Type II overhead reduction.

In fact, the Rel-15 overhead reduction mechanisms described in Section 2.1 have already taken advantage of the frequency-domain correlation for the combination coefficients. In the above mechanisms, UE reports sub-band coefficient information for only partial sub-bands in the BWP or configured CSI reporting band. gNB can derive the amplitude/phase of the non-reported sub-bands via interpolation. The performance of the interpolation can be guaranteed as the phases and amplitudes have frequency-domain correlation. The performance of these mechanisms is given in [2] with 1/2 or 3/4 overhead reduced.

The enhancements for Type II overhead reduction can be classified into two types of directions.

- The first type is to define a new codebook structure or CSI configuration. The new structure or configuration is usually selected semi-statically. This type can be seen as semi-static overhead reduction.

- The other type is to extend the partial CSI omission rule. As the exact CSI overhead changes dynamically along with some CSI values, e.g., RI, this type of overhead reduction is to reduce the dynamic overhead so that the PUSCH resource allocation can be more flexible and simplified. This one can be regarded as dynamic overhead reduction.

Semi-static overhead reduction

As discussed above, the potential new codebook structure or CSI configuration for overhead reduction should focus on using frequency-domain correlation of the combination coefficients.

Differential coding of sub-band phases

The phases of the coefficients in the linear combination vary from 0 to 2π across sub-bands with a range of 2π. In NR Rel-15, the phase is reported with quantized values uniformly distributed in [0,2π). For example, 8 PSK is quantized with the set of {0,π/4,π/2,3π/4,π,5π/4,3π/2,7π/4}. Due to the frequency-domain correlation of the combination coefficients, the range of the differential phases between sub-bands can be smaller, usually less than π. So instead of the direct phase quantization, the differential phases can be reported with a more compact quantization set, i.e., the value range of the elements can be smaller than 2π. For example, the set of {-π/4, 0, π/4, π/2} can be used for the differential phases. Thus the overhead can be reduced with differential phase reporting, while the set for differential phases has the same quantization granularity as the set for direct phase reporting, i.e., the same reporting performance can be achieved. Further, if we restrict the value range of the differential phases to less than π, the set for differential phases can also be {-π/2,-π/4, 0, π/4}, we can use 1 wideband bit to indicate the used quantization set. To further exploit the frequency-domain correlation of the combination coefficients and reduce overhead, the differential phase between sub-band m and sub-band m+2 is reported, and the phase of sub-band m+1 can be calculated with interpolation. To recover the coefficients of all sub-bands, one direct phase of a sub-band should be reported. This approach can be seen as using linear function to perform interpolation among sub-bands.

To be more specific, the proposed differential phase reporting is performed as shown in Fig. 2.

Fig. 2 Proposed differential coding scheme

As show in Fig. 2, the sub-band phases for the M sub-bands of W1 beam l are {p0l,p2l,…, pM-1l}. The differential phase of sub-band m and sub-band m+2 is calculated and reported. Then the reported differential phase is

δ0l=p2l-p0l, δ1l=p4l-p2l,…, δM-12l=pM-1l-pM-3l

Each entry of δ0l, …,δM-12l is quantized with {-π/4, 0, π/4, π/2} or {-π/2,-π/4, 0, π/4}, and 1 wideband bit is used to select one of the two sets for each W1 beam l. If M is an even number, δ0l, …,δM-22l is reported, and the phase of the last sub-band pM-1l is additionally reported. To recover the phase of all the sub-bands in gNB side, one of p0l, p2l,…,pM-1l needs to be reported, and the phases of the odd sub-bands are recovered through interpolation.

SVD based operation on the sub-band coefficients

The above example and some related approaches, e.g., the flexible CSI reporting band configuration in Rel-15, exploit the frequency domain correlation by using a pre-defined frequency-domain pattern to reduce sub-band overhead. However, as the frequency-domain pattern can be different for different channel realizations, a pre-defined pattern would reduce performance. One potential approach taking advantage of the channel characterization for overhead reduction is based on SVD on the combination coefficients. It’s equivalent to use the dominant eigenvector(s) of “W2” across subbands as “interpolation” function to achieve frequency-domain compression

Fig.3 Matrix composed by coefficients of all sub-bands

Consider a Type II CSI report for N ports and M sub-bands with L beams. A rank R Type II precoder in sub-band m can be expressed as follows

Fm=W100W1W2m=v1, ⋯,vL00v1, ⋯,vLc1,11⋮ cL,1mcL+1,1m⋮c2L,1(m)⋯c1,R1⋮ cL,RmcL+1,Rm⋮c2L,R(m)

where v1, ⋯,vL are the L combined beams and cl,rm denotes the coefficient for beam l in sub-band m and layer r. Then for each layer r, each sub-band has 2L coefficients. The coefficients for all the beams and sub-bands in layer r can be expressed as the following 2L*M matrix.

Cr=c1,r1⋮ cL,r1cL+1,r1⋮c2L,r(1)⋯c1,rM⋮ cL,rMcL+1,rM⋮c2L,r(M)

The composition of the matrix Cr is also depicted in Fig. 3. Based on SVD, the correlation of the different sub-bands can be derived for Cr, i.e.,

Cr=UrDrVrH

The correlation of the coefficients in beam domain and frequency domain is realized in Ur and Vr, respectively. Then if we extract the first d vectors in Ur and Vr, and multiply the first d singular values of Dr into Ur or Vr, we have

Cr=Ur(d)Vr(d)H

where Ur(d) is a 2L*d matrix, Vr(d) is a M*d matrix and the vectors in Ur(d) or Vr(d) are orthogonal mutually. Then UE can quantize and feed back the amplitude and phase of the entries in Ur(d) and Vr(d). gNB can recover the precoders for different sub-bands based on Cr. Further, sub-band CQI is derived and reported based on Cr.

In the above SVD based feedback, only (M+2L)*d*R coefficients are quantized and fed back compared with 2L*M*R coefficients in original Type II CSI. If d is set to 2, using Type II quantization approach in Rel-15, up to half of the Type II sub-band overhead can be reduced for L=4 and M>=9.

Dynamic overhead reduction

In Rel-15, dynamic overhead reduction for Type II is realized by partial CSI omission. Specifically, if the allocated PUSCH cannot carry the CSI report based on a given code rate, UE would omit the sub-band phase and amplitude of the Type II CSI in odd sub-bands as shown in Fig. 1. Based on this mechanism, half of the CSI overhead can be reduced dynamically.

The current granularity of the omission is half of the sub-bands. If reducing half of the sub-band CSI cannot fulfill the PUSCH capacity, the entire sub-band CSI is omitted. It can be enhanced by introducing more levels of omission. For example, if omitting half of the sub-band cannot meet the PUSCH capacity, UE can try to omit 3/4 of the subbands by reporting the sub-band CSI in sub-band 0, 4, 8, …, i.e., by a comb-4 pattern. If the CSI overhead based on comb-4 can meet the PUSCH capacity but CSI overhead of comb-2 cannot, UE will omit the sub-band CSI in sub-bands except sub-band 0, 4, 8…. The performance of comb-4 is simulated in [2], which shows the performance loss is not large by using gNB-side interpolation.

Extending the partial CSI omission rule to a finer granularity will give gNB more flexibility to do PUSCH resource allocation. Especially, if the rank of Type II is extended to RI<=4, the dynamic range of the sub-band overhead is much larger than the current RI<=2. Even if half of the sub-band CSI is omitted, the total CSI overhead difference for different RI values can still be as larger as a one-layer sub-band CSI payload, i.e., more than 100 bits. Thus the current CSI omission granularity is not sufficient. The introduction of a comb 4 would reduce the rank-4 overhead to a similar overhead as rank 1.

Proposal 2: Both semi-static overhead reduction and dynamic overhead reduction should be enhanced in Rel-16.

- Overhead reduction should focus on using frequency-domain correlation of the combination coefficients.

2.3 Simulation results

We simulate the above schemes and some other candidate schemes summarized in [5]. They are compared with the conventional Type I/II and the baseline, which is Type II with comb-like CSI reporting band configuration. The detailed schemes are

- Type I

- Type II: 8PSK phase and wideband amplitude.

- Type II-Comb: Type II with comb-like CSI reporting band configuration (a comb-2 pattern is used)

- Type II-Diff: Type II with differential coding of the sub-band phases (as proposed in 2.2, where the original phase pM-12l is reported with 8PSK quantization, each entry in δ0l, …,δM-12l is quantized with 2 bits, and one wideband bit per W1 beam is used to select the quantization set.)

- Type II-SVD: Type II with SVD based operation on the sub-band coefficients (as proposed in 2.2 with d=2). Each amplitude in the dominate eigenvectors is quantized with 2 bits, and each phase is quantized with 3 bits.

- Type II-DFT (sub-band level): The co-efficient matrix across subbands is linearly combined by 4 DFT vectors. Each amplitude or phase of the coefficient matrix after DFT is quantized with 3 bits. The DFT vectors are chosen from x4 oversampled grid.

The simulation assumptions are listed in Appendix. In this simulation, the total number of gNB layers is 4 , the number of UE Rx antennas is 2, and the maximum rank for each UE is 2.

The throughput performance of the above schemes are given in Table 2.1.

Table 2.1 Throughput performance and average overhead

|  | Mean UE throughput （Mbps） | 5% tail UE throughput（Mbps） | RU |
| --- | --- | --- | --- |
| Low RU (around 20%) |  |  |  |
| Type I | 46.0305 (-8.33%) | 18.5185 (-22.22%) | 0.2366 |
| Type II | 50.2136 (+0%) | 23.8095 (+0%) | 0.2105 |
| Type II-Comb | 49.4364 (-1.55%) | 23.1107 (-2.93%) | 0.2142 |
| Type II-Diff | 49.3301 (-1.76%) | 23.0916 (-3.01%) | 0.2115 |
| Type II-SVD | 50.1357 (-0.16%) | 23.4335 (-1.58%) | 0.2106 |
| Type II-DFT (Sub-band level) | 49.0357 (-2.35%) | 22.8571 (-4.00%) | 0.2160 |
| Middle RU (around 50%) |  |  |  |
| Type I | 30.0238 (-16.84%) | 7.3973 (-31.21%) | 0.5635 |
| Type II | 36.1050 (+0%) | 10.7527 (+0%) | 0.4764 |
| Type II-Comb | 34.6907 (-3.92%) | 10.1523 (-5.58%) | 0.4929 |
| Type II-Diff | 33.3254 (-7.70%) | 9.7547 (-9.28%) | 0.4984 |
| Type II-SVD | 35.5783 (-1.46%) | 10.4782 (-2.55%) | 0.4855 |
| Type II-DFT (Sub-band level) | 34.6902 (-3.92%) | 10.0464 (-6.57%) | 0.4965 |
| High RU (around 70%) |  |  |  |
| Type I | 23.0550 (-13.65%) | 3.7003 (-43.11%) | 0.8260 |
| Type II | 28.6997 (+0%) | 6.5041 (+0%) | 0.7108 |
| Type II-Comb | 27.3719 (-4.63%) | 5.5031 (-15.39%) | 0.7410 |
| Type II-Diff | 26.4345 (-7.89%) | 5.6190 (-13.60%) | 0.7409 |
| Type II-SVD | 27.8376 (-3.00%) | 6.6370 (+2.04%) | 0.7263 |
| Type II-DFT (Sub-band level) | 26.8661 (-6.39%) | 5.8224 (-10.48%) | 0.7403 |

Table 2.2 Per-rank overhead of the Type II based schemes

|  | Per-rank overhead |  |
| --- | --- | --- |
|  | Rank 1 | Rank 2 |
| Type II | 312 (+0%) | 609 (+0%) |
| Type II-Comb | 176 (-43.6%) | 336 (-44.8%) |
| Type II-Diff | 151 (-51.6%) | 287 (-52.9%) |
| Type II-SVD | 227 (-27.3%) | 439 (-27.9%) |
| Type II-DFT (Sub-band level) | 219 (-29.8%) | 423 (-30.5%) |

It is observed from the above simulation results that

- From throughput performance perspective, Type II SVD has the best performance among overhead reduction schemes, with better performance than the Type II Comb. The performance loss compared to normal Type II is quite small, which is negligible in average. In high RU case, which is the typical use case for MU-MIMO, cell-edge performance of Type II SVD is even a bit higher than legacy Type II. The overhead reduction for the proposed Type II SVD is quite attractive, which is more than 25%.

- The number of reduced bits of Type II Diff is very large. The reduced overhead is more than 50% of the legacy Type II report, which is the best one among the enhanced schemes. The performance loss is tolerable. It still achieves more than 10% gain over the Type I codebook for middle and high RU cases. For high RU case, the performance gain it provides over Type I is even more than 15%. Compared with Type II Comb, it achieves about 14%-15% overhead reduction, but the performance loss in middle and high RU cases is only about 3%-4%. It means the Throughput/Overhead performance of Type II Diff is even better than the baseline Type II Comb. Hence the Type II Diff achieves better trade-off between overhead and throughput than Rel-15.

- The performance of DFT based approach is not as good as the SVD based approach. Especially in middle and high RU cases, where MU-MIMO is usually performed, the performance loss of DFT based approach is quite significant. In high RU case, the average performance of DFT based approach is even worse than the comb-based approach supported in Rel-15. The cell-edge performance loss is quite large compared with legacy Type II. The overhead of DFT based approach is similar as SVD based approach.  Moreover, the overhead of DFT based approach is not reduced much compared to Type II Diff which means it is not a good trade-off scheme between overhead and throughput.

Observation 3:

- Type II with differential coding on sub-band phase achieves quite large overhead reduction, and it still can achieve more than 10% performance gain over Type I.

- The proposed SVD based Type II report achieves negligible performance loss compared to the legacy Type II, with more than 25% overhead reduction.

As the SVD based approach achieves the best performance and provides significant overhead reduction, and differential coding provides largest overhead reduction with tolerable performance loss, we propose to down-select the candidate schemes to one of these two.

Proposal 3: Adopt at least one from the following schemes as the Type II overhead reduction scheme

- SVD based scheme: Using dominant eigenvector(s) of “W2” across subbands as “interpolation” function to achieve frequency-domain compression.

- Differential coding of phase among subbands: Using linear function as “interpolation” function to achieve frequency-domain compression.

Higher rank support for Type II

3.1 Enhancement on higher rank support

A study point of Type II enhancement in Rel-16 is to evaluate the higher rank support of Type II. Type II CSI is major targeted to the MU-MIMO use cases. If rank 4 is supported for Type II, gNB would have more flexibility to do MU scheduling, e.g., 4+4 MU paring can be possible. However, the performance of supporting higher rank Type II should be justified.

In fact, usually, a similar performance of 4+4 MU paring can also be achieved by a 2+2+2+2 MU pairing. Then to compare the rank-2 performance and rank-4 performance in MU, we should limit the total number of streams to the same maximum number of streams, e.g. 8, in gNB to enable both of the above two MU scheduling strategies. Further, in practical network, the gain of rank-4 MU may come from the case that gNB cannot find 4 UEs to be scheduled in the cell. Hence, in order to simulate this case, FTP traffic model should be assumed instead of full buffer.

Additionally, overhead reduction is also a significant issue in supporting higher rank Type II. The total number of rank 4 CSI can be twice as the rank 2 CSI without overhead reduction. All the overhead reduction schemes, including both semi-static and dynamic overhead reduction, should consider rank 4 if the performance gain of rank 4 is justified. Especially for the dynamic overhead reduction, the current partial CSI omission granularity is not sufficient for rank 4 based on the analysis above. Hence the CSI omission rule should be extended to finer granularity, e.g., including comb-4 pattern, if Type II CSI is extended to higher rank.

Proposal 4: If Type II is extended to higher rank, the CSI omission rule for partial sub-bands should also be extended.

3.2 Simulation results

To compare the performance of higher rank Type II CSI, we conduct simulations on rank-2 Type II codebook and rank-4 Type II codebook in 3D UMi scenario. The total number of gNB streams is 8. The number of Rx antennas for both rank-2 and rank 2 is 4. The other simulation assumptions are given in Appendix. The throughput performance Type II-rank2 and Type II-rank4 is given in Table 3.1.

Table 3.1 Throughput performance

|  | Mean UE throughput（Mbps） | 5% tail UE throughput（Mbps） | 50% tail UE throughput（Mbps） | 95% tail UE throughput（Mbps） | RU |
| --- | --- | --- | --- | --- | --- |
| Type II-rank2 | 46.98 | 22.13 | 48.78 | 63.49 | 0.47 |
| Type II-rank4 | 55.25 (17.60%) | 18.97 (-16.66%) | 52.63 (7.89%) | 100.62 (58.48%) | 0.46 |

It is seen from the above simulation results that for average throughput, allowing rank 4 can achieve attractive gain over rank 2. However, for cell-edge UEs, rank 4 suffers large performance loss. The possible reason is that for cell-edge UEs, the channel estimation and CQI calculation may not be accurate, which causes too optimistic estimation on the rank value. The issue can be solved by UE specific configuration of the allowed RI values. For cell-edge UEs, gNB can forbid them to select higher ranks by RI restriction.

Observation 4: Comparing Type II rank 4 and Type II rank 2, rank 4 achieves average throughput gain but suffers cell-edge throughput loss.

Proposal 5: Extend Type II to rank 4 with RI restriction.

Conclusion

In this contribution, we discuss remaining issues in Type II enhancement for MU-MIMO. Based on the discussion, we have the following observations and proposals.

Observation 1: NR Rel-15 CSI reporting band configuration can achieve semi-static overhead reduction for Type II feedback.

Observation 2: NR Rel-15 partial CSI omission can achieve dynamic overhead reduction for Type II feedback.

Proposal 1: Enhancements for Type II overhead reduction shall consider the overhead reduction mechanism in Rel-15 as a baseline

- Rel-15 overhead reduction mechanism includes flexible CSI reporting band configuration and partial CSI omission.

Proposal 2: Both semi-static overhead reduction and dynamic overhead reduction should be considered in Rel-16.

- Overhead reduction should focus on using frequency-domain correlation of the combination coefficients.

Observation 3:

- Type II with differential coding on sub-band phase achieves quite large overhead reduction, and it still can achieve more than 10% performance gain over Type I.

- The proposed SVD based Type II report achieves negligible performance loss compared to the legacy Type II, with more than 25% overhead reduction.

Proposal 3: Adopt at least one from the following schemes as the Type II overhead reduction scheme

- SVD based scheme: Using dominant eigenvector(s) of “W2” across subbands as “interpolation” function to achieve frequency-domain compression.

- Differential coding of phase among subbands: Using linear function as “interpolation” function to achieve frequency-domain compression.

Proposal 4: If Type II is extended to higher rank, the CSI omission rule for partial sub-bands should also be extended.

Observation 4: Comparing Type II rank 4 and Type II rank 2, rank 4 achieves average throughput gain but suffers cell-edge throughput loss.

Proposal 5: Extend Type II to rank 4 with RI restriction.

References

[1] RP-181453, WI Proposal on NR MIMO Enhancements, Samsung

[2] R1-1715456, Overhead reduction for Type II CSI, ZTE

[3] R1-1717423, Remaining details on CSI reporting, ZTE

[4] Chairman’s notes, RAN1#94bis

[5] R1-1811654, Summary of CSI enhancement for MU-MIMO support, Samsung

Appendix

Table 6.1 Simulation assumptions

| System level simulation parameters |  |
| --- | --- |
| Scenarios | TR38.901: 3D-Uma (200m) for overhead reduction; 3D-Umi for higher rank support |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | DL 10 MHz |
| SCS | 15KHz |
| Antenna Spacing | (dV,dH)=( 0.8λ, 0.5λ) |
| NB antenna configurations | For 32 ports: (MTXRU, NTXRU, P) = (2, 8, 2) (M,N,P,Mg,Ng)= (8,8,2,1,1) |
| UE antenna configurations | Isotropic antenna gain pattern: (M, N, P) = (1, 1, 2) or (1, 2, 2) |
| Transmission scheme | SU/MU-MIMO adaption with max rank 2/4, total 4/8 layers |
| Traffic model | FTP 3 with packet size 0.5M byte |
| CSI-RS | Period is 5 ms and overhead is accounted. |
| Delay for scheduling and AMC | 4ms |
| Scheduler | PF |
| Receiver | MMSE-IRC |
| HARQ Scheme | Chase Combining |
| Feedback Assumption | Non-ideal modeling of channel estimation, with error modeling is used. |
| Handover margin | 3dB |
| DL Overhead calculation | 2 OFDM symbols for PDCCH, 24 RE/PRB for DMRS |
| Metric | Average and 5% tail UE throughput; Per-rank PMI overhead; |