3GPP TSG RAN WG1 Meeting #95                                        R1-1812261

Spokane, USA, November 12th - 16th, 2018

Source:         ZTE

Title:            Evaluation assumptions for URLLC multi-TRP/panel

Agenda item:    7.2.8.6

Document for:	Discussion and Decision

Introduction

Based on the approved WID [1], multi-TRP techniques for URLLC requirements should be discussed in MIMO session.

- Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul:

  - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission

  - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission

  - Multi-TRP techniques for URLLC requirements are included in this WI

To justify the efficient transmission schemes based on simulation results, evaluation assumptions should be consistent among companies. In RAN1#94 meeting, many simulation assumptions for multi-TRP for eMBB were agreed. And a guidance for URLLC requirement is also agreed as follows [2]

Agreement:

- For URLLC multi-TRP/panel performance evaluation, choose a subset of evaluation scenarios/assumptions agreed in the URLLC agenda item

However, there are many details still to be specified. In this contribution, we provide our views for the simulation assumptions of multi-TRP techniques for URLLC requirements.

Discussion

In URLLC session, multiple use cases have been agreed including Power distribution, Factory automation, Rel-15 enabled use case, Transport Industry(22.186: 5.5), Transport Industry(23.501, 22.261), etc., wherein the assumptions of power distribution with some modification can be used for Rel-15 enabled use case with urban macro (UMa) scenario, and the assumptions for Factory automation with some modification can be used for Rel-15 enabled use case with indoor hotspot scenario.

Since it has been agreed to choose a subset of evaluation scenarios agreed in the URLLC agenda item for URLLC multi-TRP/panel performance evaluation, we propose Rel-15 enabled use case with UMa and indoor hotspot scenarios as the baseline for MIMO evaluation in order to align the agreed assumptions for eMBB,especially for UE distribution.

Proposal 1: Support Rel-15 enabled use case with UMa and indoor hotspot scenarios as the baseline for URLLC multi-TRP/panel performance evaluation.

In the following two sub-sections, we collect the proposed SLS and LLS assumptions, and suggest making them as the baseline.

Proposal 2: Make Table 1 and Table 2 as the baseline for SLS and LLS respectively.

SLS

In this section, we collect the proposed SLS assumptions in one table for Rel-15 enabled use case with UMa and indoor hotspot scenarios respectively. Basically, most parameters marked by red in the table have been approved in URLLC agenda.

Table 1 SLS assumptions for URLLC multi-TRP/panel performance evaluation

| Parameters | Dense urban (Macro Only) | Indoor hotspot |
| --- | --- | --- |
| Center frequency | 2, 4 GHz | 4GHz 30GHz |
| Data packet size and traffic model | 32 and 200 bytes, FTP model 3 or periodic with different arrival rates(Random offset between UEs) Companies can report the arrival rate. |  |
| Latency requirement | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes |  |
| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports dH = 0.5λ, dV = 0.8λ; Companies report the antenna tilt For 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) For 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4) | For 4GHz: 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports For 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2) |
|  |  | For 30GHz: 2 TX/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np)=(4,4,2,1,1; 1,1) dH = dV = 0.5 λ |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 For 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) | For 4GHz 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 For 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
|  |  | For 30GHz: 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1,1), (dH,dV)=(0.5,0.5)λ Static panel selection |
| Number of UEs per cell | Up to 20 Note: Example of the number of users can be 5, 10, 15, 20 *The number of UEs in the above table is the number of pure URLLC UEs | Up to 20 Note: Example of the number of users can be 5, 10, 15, 20 *The number of UEs in the above table is the number of pure URLLC UEs |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modeled according to low loss model | 100% of users are indoor: 3 km/h UE-speed |
| Simulation bandwidth and SCS | 40MHz BW and 30 kHz SCS. 20MHz BW and 15 kHz SCS. | For 4GHz: 40MHz BW and 30 kHz SCS. |
|  |  | For 30GHz: 160MHz BW and 120KHz SCS |
| Backhaul | Ideal backhaul: 0ms Non-ideal backhaul: 2ms, 5ms(FFS, optional) |  |
| Coordination assumptions | Each company to provide details on cluster size, coordination scheme, etc |  |
| Baseline | Single TRP transmission |  |
| Metrics | Throughput Percentage of users satisfying reliability and latency requirements |  |

LLS

In this section, we collect the proposed LLS assumptions in one table for UMa and indoor hotspot scenarios respectively. Basically, most parameters marked in red in the table have been approved in URLLC agenda, except the channel models which should use CDL-C/D instead of TDL and modulation order.

Table 2 LLS assumptions for URLLC multi-TRP/panel performance evaluation

| Parameters | UMa | Indoor hotspot |  |
| --- | --- | --- | --- |
| Carrier frequency for evaluation | 2, 4GHz | 4GHz | 30 GHz |
| Channel model | CDL-C (delay spread: 300ns) as in 38.901 | CDL-D (delay spread: 30ns) as in 38.901 CDL-C (delay spread: 100ns) as in 38.901 | CDL-A (delay spread: 20 ns) as in 38.901 |
| UE speed | 3 km/h for power distribution and Rel-15 enabled use case; | 3 km/h, 30 km/h | 3 km/h, 30 km/h |
| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports | 2 Tx/2 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports | 2 Tx/4 Rx antenna ports | 2 Tx/2 Rx antenna ports |
| System bandwidth | 40 MHz | 40 MHz | 160 MHz |
| Sub-carrier spacing | 30 kHz | 30 kHz | 120 kHz |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| Coordination assumptions | Each company to provide details on cluster size, coordination scheme, etc |  |  |
| Modulation | QPSK, 16 QAM with code rate = 1/2 |  |  |

Conclusions

In this contribution, we propose the simulation assumptions for URLLC multi-TRP/panel evaluation for both SLS and LLS.

Proposal 1: Support Rel-15 enabled use case with UMa and indoor hotspot scenarios as the baseline for URLLC multi-TRP/panel performance evaluation.

Proposal 2: Adopt Table 1 and Table 2 as the baseline for SLS and LLS respectively.

References

- RP-182067, Revised WID: Enhancements on MIMO for NR , Samsung

- Chairman notes of RAN1#94bis meeting