3GPP TSG RAN WG1 Meeting #95		R1-1812337

Spokane, USA, Nov 12th – 16th, 2018

Source:	vivo

Title:	Considerations on Techniques for NR positioning

Agenda Item:	7.2.10.3

Document for:	Discussion and Decision

- Introduction

In RAN1#94bis, the requirements and evaluation methodology have been agreed at least for NR RAT-dependent positioning.  Based on the output [1] of last meeting, NR positioning technologies are discussed in this contribution.

- OTDOA  related Techniques

Considering the existing studies on positioning in LTE/LTE-A, positioning techniques are based on OTDOA, UTDOA and E-CID had been mainly focused and there were well-defined simulation methodologies. NR could reuse the methodology in LTE/LTE-A and update the existing technologies according to new technologies  which have been specified in Rel-15. OTDOA was considered as a baseline technique for previous positioning evaluation. OTDOA related techniques could provide more accurate UE location and have greater impact on RAN1 specification than other positioning technologies. From our point of view, OTDOA should be also considered as baseline positioning method for NR positioning evaluation methodology. Therefore RAN1 needs to study potential enhancement of NR OTDOA for more stringent regulation and commercial requirements. In addition, some considerations were discussed in our contribution of last meeting [2], where the following considerations were made for the NR positioning reference signal design.

NR positioning reference signal design should consider at least the points proposed below

-NR beam mechanism.

-Multi-TRP or multi-cell transmission.

-The operation in low and high frequency band of NR radio-technology.

-The relationship with other signal/reference signals of NR radio-technology.

-Other time/frequency/spatial configurations of NR system.

-Smaller influence on NR specification, lower overhead and power consumption.

-Positioning requirements such as accuracy and latency.

-The frequency offset caused by local crystal oscillator, phase noise, high speed UE or other unknown reasons.

-Increasing hearablity from other positioning node and reducing the interference from other cell or TRP.

On the one hand, positioning RS design in NR should provide good performance to meet the requirements of regulatory and commercial use case. On the other hand, positioning RS should be flexible enough to match NR framework. In this sector, additional considerations are discussed.

Proposal 1:

- OTDOA can be considered as a baseline for positioning evaluation.

  - Support beam-related OTDOA technique.

2.1	Considerations of positioning RS for NR OTDOA positioning

Some companies proposed that use the CSI-RS for tracking signal (TRS) as NR downlink baseline RS. TRS is used to track time/frequency offset and calculate Doppler/delay spread. If using TRS as a baseline RS for NR positioning, the less overhead will be introduced.  In LTE, PRS with diagonal patterns has been specified. Therefore, it is also reasonable to reuse LTE PRS as baseline RS for NR positioning. Figure 1 shows two different patterns with same RE overhead. The pattern on the left is TRS pattern and the REs are aligned in frequency domain. The other pattern is a kind of staggered pattern.

(a) TRS                                                             (b)  a staggered RS

Figure 1 Possible pattern of NR positioning RS

Based on the two patterns above, autocorrelation performance simulation results are depicted in Figure 2.

(a) TRS                                                                (b) a staggered RS

Figure 2 Autocorrelation performance of possible NR positioning RS

It can be seen in Figure 2 that the staggered RS can provide better correlation performance than TRS with the same RE overhead. Using staggered RS design could suppress secondary cross-correlation peaks caused by RS from different symbols. Furthermore, the staggered RS can be extended to a diagonal RS occupying more symbols like LTE PRS. Besides the 2 patterns above, we also simulate 2 patterns depicted in Figure 3.

(c) a diagonal RS                                                         (d) LTE PRS

Figure 3 Possible pattern of NR positioning RS

We compared the positioning performance of 4 patterns above in UMa scenario. The simulation assumptions can be found in the appendix in our contribution [3].

Figure 4 Positioning performance of NR possible positioning RSs

The performance order is LTE PRS, a diagonal RS, a staggered RS, and TRS. The diagonal RS and LTE PRS (which can be also regarded as a diagonal RS) in Figure 3 seem to be a better choice for positioning. Therefore, based on above simulation results, we propose to use LTE PRS as the baseline pattern for NR positioning evaluation.

Proposal 2:

- Support LTE PRS as baseline positioning RS for NR downlink positioning evaluation.

Considering the complicated and flexible configurations of NR framework, we proposed that NR PRS could be configured like a kind of NZP CSI-RS resource set including one or several configured CSI-RS, which simplifies RRC signaling overhead and provides enough flexibility. Moreover, some configurations of NR CSI-RS need to be modified for positioning purpose. By using a kind of CSI-RS, not only PRS pattern can be flexibly configured and satisfied, we may also reuse part of CSI-RS design aspects, e.g. flexible periodicity, colliding rules and multiple configurations.

Proposal 3:

- Considering forming NR OTDOA positioning RS pattern with basic NZP CSI-RS patterns specified in Release-15.

Beam configuration is also an important part that should be supported. Beam management information can provide spatial and angular domains information such as accurate arrival/departure angle, which can provide additional benefit for UE position measurement. In addition, analog beamforming brings scheduling restriction that each symbol is transmitted using one beam only per panel, which effects on positioning RS design especially in FR2. Beam configuration for downlink positioning RS could refer to beam configuration of CSI-RS for mobility.

Proposal 4:

- Beam configuration for NR OTDOA positioning RS could refer to beam configuration of CSI-RS for mobility.

Periodic PRS has been already agreed for downlink positioning in LTE. Unfortunately, there are many events in NR which may need more than periodic PRS. In multiple use cases such as beam switching, BWP switching, aperiodic positioning request from UE/network etc., periodic positioning RS is not available. Therefore, it is essential to introduce aperiodic or semi-persistent positioning RS in some events.

Proposal 5:

- Support periodic, aperiodic and semi-persistent transmission for NR OTDOA positioning RS.

CP is used to eliminate intersymbol interference and the CP length is generally greater than the delay spread of the channel. CP configuration has been agreed in NR. Table1 demonstrates the CP length in different numerologies. For 120kHz subcarrier bandwidth, the CP length is 0.57us corresponding to a distance of 171m. In some scenarios which network configures positioning reference signal on the cells far from UE, we cannot guarantee that RSTD is within the CP length. Thus it is better to allow configuration of extended CP for NR OTDOA positioning RS.

Table1 Supported transmission numerologies.

|  |  | Cyclic prefix |
| --- | --- | --- |
| 0 | 15 | Normal |
| 1 | 30 | Normal |
| 2 | 60 | Normal, Extended |
| 3 | 120 | Normal |
| 4 | 240 | Normal |

Proposal 6:

- Support configuration of extended CP for NR OTDOA positioning RS.

2.2	Other considerations for NR OTDOA positioning

NLOS is a big challenge in positioning study which can bring severe positive ToA measurement errors. This could lead to inaccurate location estimation especially in TDOA-based techniques. It is known that UMa scenario has large probability caused by large distance between gNB and UE. The following simulations in Figure1 are based on UMa scenario.

It is observed that positioning performance in the cases with normal NLOS probability is much worse than those cases with 100% LOS probability.  The positioning performance can be serious impacted by NLOS even though conducting with large bandwidth, so it is necessary to address the problems brought by NLOS during the period of NR positioning study. PRS design should also be considered to reduce NLOS effect.

Figure 5 NLOS effect on positioning performance

Observation 1:

- NR PRS design should consider how to address the problems brought by NLOS.

In Figure2, we present the positioning performance of Indoor office with a PRS bandwidth of 100MHz and 100% LOS probability. It can be observed that the indoor positioning accuracy of 100MHz doesn’t meet the accuracy requirement of 1m. In addition, the performance is also worse than the performance with 100% LOS probability presented above. One reason seems to be that the ToA resolution is not finer enough, e.g. the ToA measurement error can be at most 0.5Ts causing an error of about 1.5 meters. This could lead to a bad positioning performance especially in scenarios which has short BS-UT distance. For indoor scenarios, higher ToA or RSTD resolution is needed and technique enhancement should be studied to satisfy the stringent requirement. Another reason could be that the deployment of indoor office TRP leads to a large GDOP and some UEs cannot be surrounded by TRPs, which brings bad positioning performance. Therefore, PRS design should consider enhancing indoor positioning performance to meet indoor positioning requirement.

Figure 6 Positioning performance of indoor scenario with 100% LOS probability

Observation 2:

- NR PRS design should consider enhancing indoor positioning performance to meet indoor positioning requirement.

Observation 3:

- The GDOP problems and RSTD resolution problems for indoor scenarios should be studied.

  - UTDOA related techniques

Considering UTDOA technology in LTE/LTE-A, NR SRS should be also used for NR UTDOA-based positioning. UTDOA is a network-based technology. The network can use the uplink positioning measurement provided by SRS to obtain RSTD between positioning related BSs and UEs. UTDOA-based positioning should also consider new framework like beam mechanism, multi-TRP or multi-cell transmission, low and high frequency band and other time/frequency/spatial configurations, etc.

A SRS can be transmitted on every 2 (comb-2) or 4 (comb-4) subcarrier and spans 1,2or 4 consecutive OFDM symbols within the last 6 symbols of a slot. For better correlation performance, SRS pattern in Figure3 should be a candidate pattern.

Figure 7 a candidate SRS pattern

Proposal 7:

- UTDOA techniques should be updated according to NR enhancement.

  - Support comb-4 SRS over 4 symbols with repeat and comb shifting for wideband operation.

- Hybrid positioning

According to the 3GPP TS 22.261, the 5G system shall support the use of 3GPP and non-3GPP technologies to achieve higher-accuracy positioning. Hybrid positioning methods include both the combination of 3GPP positioning technologies (RAT-denpendent) and the combination of 3GPP positioning technologies with non-3GPP positioning technologies (RAT-indenpendent) such as GNSS (e.g. Beidou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), sensors (e.g. barometer, IMU), WLAN/Bluetooth-based positioning. The purpose of hybrid positioning is to achieve better performance than those using only one positioning technology and the hybrid technology could be the combiantion of two technologies or more above.

The most important consideration for hybrid positioning is how to combine different positioning technologies. For example, in some indoor scenarios with dense obstacle,RAT-dependent positioning is seriously effected by NLOS and can’t provide accurate UE location. In this case, UE could use additional information provided by WLAN or Bluetooth to reduce the impact of NLOS. It seems that UE-based data processing/approach could obtain additional information easily and make a better results in the combination of RAT-denpendent and RAT-indenpendent positioning technologies.

RAT-independent techniques have small impact on RAN1 specifications, while it mainly affects high layer specifications and signaling. Thus RAN1 should focus on RAT-dependent techniques.

Proposal 8:

- Support UE-based positioning data processing/approach for RAT-denpendent, RAT-indenpendent and  hybrid of these positioning technologies.

Proposal 9:

- RAN1 should focus on RAT-dependent techniques.

- Positioning in idle/inactive mode

In some situations, there is no data transmission between the gNB and the UE. Therefore ,the UE may be in the RRC-idle or RRC-inactive mode to reduce power consumption. Based on the positioning techiques now, once some APPs need to make some location updates in idle or inactive mode, UE has to switch to RRC-connective mode to provide positioning service. Considering power saving, it is necessary to study positioning processing in idle and inactive mode.

Proposal 10:

- Positioning processing in idle and inactive mode should be studied.

- Conclusion

In this contribution, we discuss NR positioning techniques with the following proposals:

Proposal 1:

- OTDOA can be considered as a baseline for positioning evaluation.

  - Support beam-related OTDOA technique.

Proposal 2:

- Support LTE PRS as baseline positioning RS for NR downlink positioning evaluation.

Proposal 3:

- Considering forming NR OTDOA positioning RS pattern with basic NZP CSI-RS patterns specified in Release-15.

Proposal 4:

- Beam configuration for NR OTDOA positioning RS could refer to beam configuration of CSI-RS for mobility.

Proposal 5:

- Support periodic, aperiodic and semi-persistent transmission for NR OTDOA positioning RS.

Proposal 6:

- Support configuration of extended CP for NR OTDOA positioning RS.

Proposal 7:

- UTDOA techniques should be updated according to NR enhancement.

  - Support comb-4 SRS over 4 symbols with repeat and comb shifting for wideband operation.

Proposal 8:

- Support UE-based positioning data processing/approach for RAT-denpendent, RAT-indenpendent and  hybrid of these positioning technologies.

Proposal 9:

- RAN1 should focus on RAT-dependent techniques.

Proposal 10:

- Positioning processing in idle and inactive mode should be studied.

Observation 1:

- NR PRS design should consider how to address the problems brought by NLOS.

Observation 2:

- NR PRS design should consider enhancing indoor positioning performance to meet indoor positioning requirement.

Observation 3:

- The GDOP problems and RSTD resolution problems for indoor scenarios should be studied.

References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 94bis, Chengdu, China, 8 –12th October, 2018

- R1-1810419 ,Considerations on techniques for NR positioning,3GPP TSG-RAN WG1 #94bis, Chengdu, China, 8 –12th  October, 2018

- R1-1812336, Remaining issues on evaluation methodology for NR positioning, 3GPP TSG-RAN WG1 #95, Spokane, USA, 12-16th ,November, 2018