3GPP TSG RAN WG1 meeting #95  						 	R1-1812372

Spokane, USA, November 12-16, 2018

### Agenda Item:	7.2.10.2

### Source:	MediaTek

### Title:	Remaining details on evaluation methodology

Document for:	Discussion and decision

# Introduction

In RAN1#94b, based on TR 38.802/TR 38.901, three scenarios have been agreed for NR positioning performance evaluation, including indoor office, UMi Street Canyon, and UMa. Also, channel model in TR 38.901 is reused. In this contribution, we address the following three points:

- Cluster delay generation in channel model TR 38.901

- A baseline algorithm transforming RSTD measurements to the location of an UE

- System level analysis on SNR/SINR for comb-4,6 PRS structure

For the first point, we note that in TR 38.901, for each UE-TP link, the delay of the first cluster is set to zero for both LOS/NLOS cases. This has to be corrected as UEs need to measure TDOA for downlink positioning. The second point is an FFS left in RAN1#94b for which we think a baseline algorithm to be agreed upon is necessary and we propose a solution in this contribution. Finally, we show our initial system level analysis on SINR for PRS with comb-k structure, where k = 4,6.

# Cluster delay generation

The procedure for generating channel coefficients is shown in the following figure [1]:

Figure 1: procedure for generating channel coefficients

In step 5, the delays are generated as follows:

- Generate cluster delay τn'=-rτ∙DS∙ln⁡(Xn)

  - n = 1,…,N: cluster index

  - rτ: delay distribution proportionality factor, depends on scenario and LOS/NLOS condition (refer to Table 7.5-6 in [1])

  - DS: delay spread, refer to Table 7.5-6 in [1]

  - Xn = uniform(0,1)

- Get τn by sorting τn'-min⁡(τ0', …,τN') in ascending order

- In the case of LOS:  τn=τn/Cτ, where Cτ depends on Ricean K-factor

The cluster delays generated by this model can be interpreted as the following figure:

Figure 2: illustration of cluster delay generation

Note that for every UE-TP link, we have τ1=0, which is not applicable for downlink positioning as UEs need to measure TDOA.

Observation 1: For every UE-TP link, the delay of the first cluster is set to zero for both LOS/NLOS cases in TR 38.901, which is not applicable for downlink positioning as UEs need to measure TDOA.

In the following we discuss the solution to the problem.

## Reference cell link is LOS

In this case, for a link between UE and a neighbor cell, an offset δ=(α∙d2-d1)/c should be added to the cluster delays of the neighbor cell, where d1 is the LOS distance between the UE and the reference cell, d2 is the LOS distance between the UE and the neighbor cell, c is the speed of light, and α depends on LOS/NLOS.

For the case that the link is LOS, we simply set α=1. See the following figure for an illustration:

Figure 3: Correction of cell cluster delays for a neighbor cell with LOS link

For the case that the link is NLOS, the parameter α can be modelled as a variable randomly drawn from an interval (1,β) for some β>1.

For the value of β, first consider the inter-cell case as shown in Figure 4, in which the LOS path (the red line) from gNB i to the UE is blocked by an obstacle. For simplicity, we assume that first signal path from gNB i to the UE experienced one reflection in the cell where the UE is located. We assume θ≤300 and the signal direction is changed at least 900. It follows that

dNLOS<1.37∙dLOS

where

dNLOS: distance travelled by the first NLOS signal path from gNB i to the UE.

dLOS:    LOS distance from gNB i to the UE.

As such, we may set β=1.37 in this case.

Figure 4: NLOS path model for inter-cell case

Now consider the intra-cell case as shown in Figure 5, in which the LOS path (the red line) from the gNB to the UE is blocked by an obstacle. Again we assume that first signal path from the gNB to the UE experienced one reflection within the cell. We assume θ≤600 and the signal direction is changed at least 900. It follows that

dNLOS<1.37∙dLOS.

As such, we may also set β=1.37 in this case.

Figure 5: NLOS path model for intra-cell case

## Reference cell link is NLOS

Similar to the discussion above, for a link between UE and a neighbor cell, an offset δ=(α2∙d2-α1∙d1)/c should be added to the cluster delays of the neighbor cell, where d1 is the LOS distance between the UE and the reference cell, d2 is the LOS distance between the UE to the neighbor cell, c is the speed of light, α1=unform(1,1.37), and α2 depends on LOS/NLOS. If the link between the UE and the neighbor is LOS, then α2=1; otherwise α2=unform(1,1.37).

Based on above discussion, we have the following proposals:

Proposal 1: For an UE, the cluster delays of the reference cell remain unchanged after they are generated in step 5.

Proposal 2: For an UE, after all small scale parameters are generated (right after step 9), the cluster delays of a neighbor cell should be added by an offset

δ=(α2∙d2-α1∙d1)/c,

where

d1: LOS distance between the UE and the reference cell,

d2: LOS distance between the UE to the neighbor cell,

c:  speed of light,

αi=1,  &if the corresponding link is LOSunform(1,β),  &otherwise

for some β>1.

Note that αi is determined once for each link i.

Proposal 3: Set the value β to be 1.37.

# A baseline algorithm transforming RSTD measurements to the location of an UE

The algorithm proposed in the following for transforming RSTD to the 2D location of UE is based on [2]. Let (x,y) be the location of the UE and (xi,yi) be the location of the gNBi. Let ri=(xi-x)2+(yi-y)2  . Suppose gNB1 is the reference cell for the UE.

Figure 6: Coordinates of UE and gNBs

Let ri1=ri-r1. It follows that

ri=ri1+r1  ri2=ri12+r12+2ri1r1  ri12+2ri1r1=ri2-r12.                            (1)

By straight forward derivation, we also have

ri2-r12=Ki-K1-2x∙xi1-2y∙yi1,                                                                     (2)

where Ki=xi2+yi2, xi1=xi-x1, and yi1=yi-y1.

It follows from (1) and (2) that

ri12-Ki+K1=-2x∙xi1+y∙yi1+ri1r1=-2[xi1  yi1  ri1]xyr1

Suppose there are N+1 gNBs for OTDOA measurement, we then have

x21y21r21x31y31r31⋮⋮⋮xN1yN1rN1xyr1=-12r212-K2+K1r312-K3+K1⋮rN12-KN+K1.                                                            (3)

For ease of representation, we write (3) as

G∙z=h.                                                                                                                       (4)

Note that in OTDOA, (Ki, xi1, yi1) are known and (x,y,r1, ri1) are unknown. We would like to estimate ri1 and then get the estimated value of (x,y,r1). Let ( x, y, r1, ri1) be the estimated value of (x,y,r1, ri1). Let h be the estimated h (note that h is given by RSTD measurement). Using least square approximation to estimate z, we have

z=xyr1 =(GTG)-1GT∙h                                                                                          (5)

Since x, y, r1 may have the problem that (x-x1)2+(y-y1)2≠ r12. We need to do one more least square approximation for finding (x*,y*) such that the following estimation error is minimized:

100111x*-x12y*-y12-x-x12y-y12 r12.

It follows that (x*,y*) is given by solving the following equation:

x*-x12y*-y12=101011100111-1∙101011∙x-x12y-y12 r12 .                         (6)

Proposal 3: For a baseline algorithm transforming RSTD measurements to the location of an UE, adopt the least square approach in equation (5) and (6) described above.

# Initial System level SINR analysis

We consider UMa scenario as shown in Figure 7. Suppose that PRS structure is comb-6, then PRS IDs are modulo by 6 as shown in the Figure 7. We consider UEs in the center hexagon with positions (x,y) = (0,120), (10, 120), …, (250, 120), where ISD = 500m. Suppose these UEs are receiving signal from a neighbour cell (marked blue line) and suffering interference from other cells transmitting PRS on the same REs at the same time (marked red line), which is the colliding condition. For the first initial analysis, we assume all links are LOS and consider only path loss and gNB antenna patterns, which can be treated as deterministic parameters. The shadowing effect and the Doppler effect are not added since a random variable denotes the variation around the mean value. The other detail of environment settings are given in the appendix.

Reuse the same settings as above, but consider that comb-6 PRS structure is replaced by comb-4 PRS structure, then the PRS IDs are modulo by 4 as shown in the Figure 8. As can be seen from Figure 8, the interference sources are more that in the comb-6 PRS structure case (signal is marked blue, red and gray lines are interferences).

In Figure 9 and 10 we show the SNR for each TP-UE link. Note that ISD = 1m in Figure 7 is only for ease of representation. The SNR analysis is given by using ISD = 500m. From Figure 9 and 10, we see that for comb-4 PRS transmission the larger number of interfering cells are encountered for all x-axis positions, while for comb-6 the interference drops sharply as UE moves along +x direction.

The SINR result is shown in Figure 11. We have the following observation:

Observation 2: Structure of larger comb may have more non-colliding cells and thus lower SINR.  Reducing SINR for small comb structure requires muting some interference TPs, which leads to larger PRS overhead.

Figure 7: [Left] UMa scenario. PRS IDs are modulo by 6

[Right] UE positions (the orange line)

Figure 8: [Left] UMa scenario. PRS IDs are modulo by 4

Figure 9: Single link SNR for comb-6 PRS

Figure 10: Single link SNR for comb-4 PRS

Figure 11: SINR for comb-4,6 PRS

# Conclusion

We have the following observations and proposals:

Observation 1: For every UE-TP link, the delay of the first cluster is set to zero for both LOS/NLOS cases in TR 38.901, which is not applicable for downlink positioning as UEs need to measure TDOA.

Proposal 1: For an UE, the cluster delays of the reference cell remain unchanged after they are generated in step 5.

Proposal 2: For an UE, after all small scale parameters are generated (right after step 9), the cluster delays of a neighbor cell should be added by an offset

δ=(α2∙d2-α1∙d1)/c,

where

d1: LOS distance between the UE and the reference cell,

d2: LOS distance between the UE to the neighbor cell,

c:  speed of light,

αi=1,  &if the corresponding link is LOSunform(1,β),  &otherwise

for some β>1.

Note that αi is determined once for each link i.

Proposal 3: Set the value β to be 1.37.

Observation 2: Structure of larger comb may have more non-colliding cells and thus lower SINR.  Reducing SINR for small comb structure requires muting some interference TPs, which leads to larger PRS overhead.

# References

- “Study on channel model for frequencies from 0.5 to 100 GHz”, TR 38.901, V15.0.0, 2018-07.

- “A Simple and Efficient Estimator for Hyperbolic Location”, Y. T. Chan and K. C. Ho, IEEE transactions on signal processing, vol. 42, no. 8, August 1994.

# Appendix: Environment Setting

| gNB carrier frequency | 4 GHz |
| --- | --- |
| Bandwidth | 20 MHz |
| Thermal noise | -174 dB per Hz |
| UE Rx noise figure | 9 dB |
| UE height | 1.5m |
| BS height | 35m |
| BS power | 44 dBm |
| BS downtilt angle | 200 |
| BS antenna element polarization | 450 |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (4, 4, 1, 1, 1), (dH, dV) = (0.5, 0.8)λ |
| gNB Antenna Radiation Pattern | Directional, 8dBi same as Table A.2.1-6: in TR 38.802 |
| Channel model | Assume all links are LOS. Path loss: as in Table 7.4.1-1 in TR 38.901, UMa LOS case For simplicity, no other factors are considered |