3GPP TSG RAN WG1 Meeting #95 R1-1812380 Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.212; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; CR to 38.212 for DCI size determination. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-10-28. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; The current determining the DCI sizes are umbigous. Not definition of “configured DCI sizes”. Summary of change:; Introducing the definition of configured DCI sizes in 7.3.1 and add missing padding/truncation for format 1-0 in 7.3.1.2.1. Consequences if not approved:; Incomplete specifications and mismatch between gNB and UE.. Clauses affected:; 7.3.1, 7.3.1.2.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of PUSCH in one cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of PDSCH in one cell |
| 2_0 | Notifying a group of UEs of the slot format |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |

For a BWP, several configured DCI format sizes are defined as:

Configured size A: DCI format 0-0/1-0 in CSS, 0-0/1-0 in USS and 2-2/2-3 in CSS

Configured size B: DCI format 0-1 in USS

Configured size C: DCI format 1-1 in USS

Configured size D: DCI format 2-0 in CSS

Configured size E: DCI format 2-1 in CSS.

……

7.3.1.2.1	Format 1_0

……

If DCI format 1_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

and if the number of information bits in the DCI format 1_0 monitored in the UE specific search space prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 monitored in the UE specific search space until the payload size equals that of the DCI format 1_0 monitored in the common search space.

If DCI format 1_0 is monitored in UE specific search space but does not satisfy at least one of the following

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

and if the number of information bits in the DCI format 1_0 monitored in the UE specific search space prior to padding is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bit-width of the frequency domain resource allocation field in the DCI format 1_0 monitored in the common search space is reduced by truncating the first few most significant bits such that the size of DCI format 1_0 monitored in the UE specific search space equals to the size of the DCI format 1_0 monitored in the common search space.

If DCI format 1_0 is monitored in UE specific search space and satisfies both of the following

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell, and

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

and if the number of information bits in the DCI format 1_0 prior to padding is less than the payload size of the DCI format 0_0 monitored in UE specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 until the payload size equals that of the DCI format 0_0.