3GPP TSG RAN WG1 Meeting #95 R1-181xxxx Spokane, USA, November 12th – 16th, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; CR to 38.213 for PDCCH monioring in the Type 3 CSS. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2018-10-28. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In the current specification, it is unclear when a UE monitor USS 1-0/0-0 in the Type 3 CSS. Summary of change:; Specifying the conditions for monitoring USS DCI format 1-0/0-0 in the Type 3 CSS.. Consequences if not approved:; Incomplete specifications and mismatch between gNB and UE.. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

## 10.1	UE procedure for determining physical downlink control channel assignment

......

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MasterInformationBlock or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s).

......

If a UE is provided one or more search space sets by corresponding one or more searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and the UE is provided a C-RNTI, an MCS-C-RNTI, or a CS-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

If a UE is provided a search space set by SearchSpace in a PDCCH-Config with searchSpaceType = common, and the UE is provided a C-RNTI, an MCS-C-RNTI, or a CS-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one the search space set in a slot only when DCI format 0_0 and DCI format 1_0 is configured in the search space set.