3GPP TSG RAN WG1 Meeting #95	R1-1812414

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.6.1.2

Source: 	Fujitsu

Title: 	Discussion on HARQ-ACK feedback over PUCCH and PUSCH

Document for:	Discussion/Decision

Introduction

To satisfy the higher reliability and lower latency requirement of URLLC in Rel-16, the SI in RAN #80 pointed out to increase the number of HARQ transmission possibility within a slot [1]. In RAN1 #94, it was agreed to study how to enable more than one HARQ-ACK in a slot, and finer indication of the HARQ-ACK feedback timing [2].

SI in RAN #80 URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified, - UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements Agreements in RAN1 #94: Study further how to enable more than one PUCCH for HARQ-ACK transmission within a slot. Study further whether/how to enable enhanced reporting procedure/feedback for HARQ-ACK. - Enhanced HARQ-ACK multiplexing on PUSCH and PUCCH piggyback - Finer indication for HARQ feedback timing, e.g. symbol-level, half-slot, etc. - Note: this may be related to more than one PUCCH for HARQ-ACK tx within a slot - Other enablers are not precluded

To enable more than one HARQ-ACK in a slot, the first problem need to be resolved is how to provide the UL resource in a slot for more than one HARQ-ACK transmission. As is known, HARQ-ACK can be transmitted on PUCCH resource or piggybacked on PUSCH resource. Hence, in this contribution, we will discuss on URLLC enhanced HARQ-ACK feedback over PUCCH and PUSCH, separately.

HARQ-ACK carried on PUCCH

Based on whether there is available separate PUCCH resource for URLLC HARQ-ACK, we have the following discussions.

Separate PUCCH resource for URLLC HARQ-ACK

Fig.1 Illustration of dedicated PUCCH resource for enhanced HARQ-ACK.

When there is available separate PUCCH resource for URLLC HARQ-ACK, symbol-level indication of the HARQ-ACK feedback timing is required in consideration of the URLLC latency requirement. Besides the length of PUCCH resource (1 or 2 symbols), three possible methods of indicating the starting symbol of the PUCCH resource are discussed.

- UE autonomously selects one from the predefined symbol-level resource set.

This method is suitable for the case where the latency requirement is not very high, such as 1ms or more than 1ms-latency.

For example, symbol-level resource could be defined via predefining the starting symbols. Each resource is predefined, e.g. 2 symbols. Suppose the subcarrier spacing is 30kHz, the URLLC L1 latency bound is 1ms, and assume the predefined starting symbols for HARQ-ACK in a slot is the 3rd, 7th, and 10th symbol. As shown in Fig.1 (a), when the URLLC PDSCH ends at the 2nd symbol, the UE sends the enhanced HARQ-ACK from the 7th symbol.

Furthermore, to improve the resource utilization, other UCI and PUSCH can be transmitted on the above predefined resource as well, but the URLLC HARQ-ACK has the highest priority.

- Autonomous and synchronous HARQ-ACK feedback, i.e. the enhanced HARQ-ACK starts after a fixed period of time where the URLLC PDSCH ends

This method is suitable for the scenario where the latency requirement is high, such as less than 1ms latency.

Assume the URLLC PDSCH ends at symbol s, then the enhanced HARQ-ACK starts at symbol s+k. For example, as shown in Fig.1 (b), where s=2, k=2, and the subcarrier spacing is 30kHz, the URLLC L1 latency bound is 0.5ms.

- DCI dynamically indicate the starting symbol of HARQ-ACK from any of the available UL symbols.

This method can provide more flexibility, where any of the UL symbol that satisfy the latency requirement can be indicated to be the starting symbol of the URLLC enhanced HARQ-ACK, as illustrated by Fig.1 (c).

Proposal 1: Support symbol-level indication for enhanced HARQ-ACK feedback. Following methods could be taken into account

- UE automatically selects PUCCH resource from predefined resource set;

- c;

- The starting symbol of PUCCH resource is dynamically indicated by DCI from any of the available UL symbols in a slot.

No available separate PUCCH resource for URLLC HARQ-ACK

When there is no available separate PUCCH resource that can satisfy the URLLC latency requirement, URLLC HARQ-ACK could be transmitted by preempting or being multiplexed with the PUCCH resource of eMBB HARQ-ACK.

Preemption of the PUCCH resource

Fig.2 shows the case where URLLC enhanced HARQ-ACK preempts the PUCCH resource for eMBB HARQ-ACK transmission. Specifically, UE can be configured to drop all or part of the eMBB HARQ-ACK information bits.

Fig.2 URLLC enhanced HARQ-ACK preempts the PUCCH resource of eMBB HARQ-ACK

Multiplexing the PUCCH resource

Consider to multiplex the UL resource of eMBB HARQ-ACK. Fig.3 shows the case of URLLC enhanced HARQ-ACK multiplexing the UL resource for eMBB HARQ-ACK, where the HARQ-ACK codebook is determined by jointly considering the URLLC enhanced HARQ-ACK and eMBB HARQ-ACK. However, it is worthwhile to mention that, how to satisfy the reliability requirement of URLLC HARQ-ACK when multiplexing with eMBB HARQ-ACK needs further study.

Fig.3 URLLC enhanced HARQ-ACK multiplexes the PUCCH resource with eMBB HARQ-ACK

Proposal 2: When there is no available separate PUCCH resource for URLLC HARQ-ACK, consider the following two kinds of methods:

- URLLC HARQ-ACK pre-empts the whole or part of the overlapped PUCCH resource;

- URLLC HARQ-ACK multiplexes the overlapped PUCCH resource with eMBB HARQ-ACK.

HARQ-ACK piggybacked on PUSCH

The case when PUCCH with only URLLC HARQ-ACK overlaps with PUSCH

For eMBB PUSCH, when the latency requirement can be met, DCI format 1_1 with  βoffset indicator can be used to dynamically adjust the number of occupied REs by HARQ-ACK. For URLLC PUSCH, the corresponding PDCCH itself requires very high reliability and thus may not contain βoffset indicator by default. In such case, a straightforward solution is to configure separate sets of βoffset for eMBB and URLLC.

Proposal 3: Consider to allow configuration of separate sets of βoffset for eMBB and URLLC.

The case when PUCCHs with URLLC HARQ-ACK and other UCIs overlap with PUSCH

In this case, we assume the PUCCH with URLLC HARQ-ACK has the highest priority. Specifically, gNB can configure to drop all or part of the information bits of other UCIs, which include both eMBB HARQ-ACK and other UCIs (e.g., CSI, SR) of URLLC and eMBB.

Proposal 4: Consider to allow to drop all or part of the information bits of eMBB HARQ-ACK to prioritize the URLLC HARQ-ACK on PUSCH.

Dynamic switching between HARQ-ACK and HARQ-less retransmission

When there is temporally no available UL resource after the first transmission, gNB can notify UE not to feedback the A/N and retransmit the PDSCH to wait for the available UL resource for HARQ-ACK. Fig.4 shows the case where the first Tx has no available UL resource for HARQ-ACK, while the first retransmission has the available resource for HARQ-ACK feedback.

Fig.4 Dynamic switching between HARQ-less and HARQ-ACK.

Proposal 5: Consider the dynamic switching between HARQ-ACK Tx and HARQ-less Tx, based on the availability of UL resource for HARQ-ACK.

Conclusions

In this contribution, the following observations and proposals are summarized:

Proposal 1: Support symbol-level indication for enhanced HARQ-ACK feedback. Following methods could be taken into account

- UE automatically selects PUCCH resource from predefined resource set;

- Autonomous and synchronous HARQ-ACK feedback;

- The starting symbol of PUCCH resource is dynamically indicated by DCI from any of the available UL symbols in a slot.

Proposal 2: When there is no available separate PUCCH resource for URLLC HARQ-ACK, consider the following two kinds of methods:

- URLLC HARQ-ACK pre-empts the whole or part of the overlapped PUCCH resource;

- URLLC HARQ-ACK multiplexes the overlapped PUCCH resource with eMBB HARQ-ACK.

Proposal 3: Consider to allow configuration of separate sets of βoffset for eMBB and URLLC.

Proposal 4: Consider to allow to drop all or part of the information bits of eMBB HARQ-ACK to prioritize the URLLC HARQ-ACK on PUSCH.

Proposal 5: Consider the dynamic switching between HARQ-ACK Tx and HARQ-less Tx, based on the availability of UL resource for HARQ-ACK.

- References

[1] RP-181477, SID on Physical Layer Enhancements for NR URLLC.

[2] 3GPP RAN1Chairman's Notes #94, Gothenburg, Sweden, August 20th - 24th, 2018.