| 3GPP TSG RAN WG1 Meeting #95 | R1-1812428 |
| --- | --- |
| Spokane, USA, 12th – 16th November 2018 |  |

Agenda item:	6.2.1.1

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title: 	Group Wake-Up Signal for MTC

Document for:	Discussion and Decisions

# Introduction

At the RAN plenary #80, a new Work Item (WI) on “Additional MTC enhancements” was approved and revised in RAN#81 [1]. The WID states that one objective is to specify the following improvement for machine-type communications for BL/CE UEs:

Improved DL transmission efficiency and/or UE power consumption:

•	Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

In RAN1#94bis meeting [2] the following was agreed:

Agreement UE grouping is based on at least UE ID or some function of UE ID Agreement The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled - Performance impact on legacy WUS should be carefully considered Agreement Group WUS is based on at least legacy WUS and UE-group ID. Agreement Configuration of group WUS is at least signaled in SI Agreement A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS

In this contribution we focus on the topic of UE-group WUS for MTC.

# Discussion

In RAN1#94bis it was agreed that TDM, FDM and CDM multiplexing are the options for the WUS in MTC. This opens up at least the following combinations for multiplexing UE-groups: TDM, CDM, TDM+CDM.

Depending on the number of planned groups, TDM might not be enough to multiplex all the groups. For CDM the added complexity on the UE needs to be considered. Nevertheless a fine granularity for grouping is beneficial for the UE power consumption in case of high number of devices. Multiple WUS in parallel reduce the coverage and shall be avoided (multi-CDM). Instead the concept of super-groups shall be considered to address more than one group with a single-CDM WUS.

- Rel-16 WUS uses single CDM for addressing UE-groups.

The agreements from RAN1#94bis state, that Rel-16 capable UEs shall also be capable of Rel-15 WUS. Whether Rel-15 WUS is a fallback for Rel-16 WUS in case Rel-16 WUS is not enabled in the cell or whether Rel-16 UEs shall also receive Rel-15 WUS even is Rel-16 WUS is active is not defined yet.

This opens the possibility to use Rel-15 WUS as a wake-up beacon for Rel-16 grouping WUS feature. The impact of Rel-15 WUS as a beacon for Rel-16 UEs should be considered carefully.

- The Rel-15 behavior of Rel-16 UEs should be further studied.

A higher number of subgroups reduces the probability of unnecessary wake-ups. The number of groups is still FFS. Considering the complexity of the WUS and the advantages of a reasonable number of groups, a 2-step approach with sub-groups is a well-suited solution.

The Rel-16 WUS can therefore introduce a 2-step addressing scheme for sub-grouping that consists of

- A: Rel-15 WUS + Rel-16 CDM WUS for sub-groups

- B: Rel-16 CDM WUS + 2nd Rel-16 CDM WUS for sub-sub-groups

Each WUS needs to support only a small number of (sub-)groups and therefore reduces the necessary complexity of the WUS receiver.

- Rel-16 WUS uses a 2-step addressing for sub-groups.

The Rel-15 WUS can either be ignored (Option B) or can be used as an initial beacon (Option A). In both cases, subgrouping for Rel-16 UEs is possible. Option B does not use the Rel-15 WUS but uses a dedicated Rel-16 CDM WUS sequence as a first beacon and a second one for sub-grouping. This option allows for a fine granular addressing of UE-groups and can even be used to differentiate between “groupcast” and “unicast” if found beneficial.

Figure 1 – Rel-15 WUS with REL-16 WUS subgrouping – Option A

Figure 1 shows the concept of subgrouping for Option A. The use of Rel-15 WUS as a “super-group” allows for backward compatibility. The Rel-15 devices and Rel-16 devices may be considered already as “subgroup x”. The effect on legacy devices should be considered and is FFS. Figure 2 illustrates the Rel-16 UE behavior of Rel-15 WUS which is NOT used as a supergroup-beacon. For activation of Rel-16 UEs, no Rel-15 WUS is needed and Rel-15 UEs do not wake up.

Figure 2 – Rel-16 WUS subgrouping w/o Rel-15 WUS – Option B

In addition to a Rel-15 beacon, a 2-step Rel-16 WUS with CDM WUS sequence for each step allows sub-subgrouping and fine granular wake-up functionality. The grouping of 1st and 2nd step CDM WUS is done via UE_ID, but may also include another differentiation criteria.

- The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

In [3] there are 5 possible options to base UE-grouping upon, i.e. UE ID and Services. Supplementary to the proposed 2-step WUS design, using two basis for the grouping is possible. A common denominator is the UE ID, which was proposed by multiple companies [4]. A second option to address different classes of reporting-intervals – and therefore lifetime – is possible.

- The UE-grouping is based on the UE ID. A service ID might be used for a second level of differentiation.

# Conclusions

In this contribution the following observations and proposals have been made:

Proposal 1:	Rel-16 WUS uses single CDM for addressing UE-groups.

Proposal 2:	The Rel-15 behavior of Rel-16 UEs should be further studied.

Proposal 3:	Rel-16 WUS uses a 2-step addressing for sub-groups.

Proposal 4:	The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

Proposal 5:	The UE-grouping is based on the UE ID. A service ID might be used for a second level of differentiation.

# References

| [1] | "RP-181878: WID: Additional MTC enhancements for LTE". |
| --- | --- |
| [2] | "R1-1811658 - Summary of 6.2.1.1 UE group MWUS". |
| [3] | "RP-181873: Status Report to TSG - Additional MTC enhancements for LTE". |
| [4] | "R1-1809532: Feature summary of 6.2.2.1 UE-group wake-up signal". |