3GPP TSG RAN WG1 Meeting #95										  R1-1812656

Spokane, USA, November 12th – 16th, 2018

Agenda item:	7.2.8.2

Source:	NEC

Title:               Discussion on multi-TRP transmission

Document for:	Discussion and Decision

- Introduction

In RAN#81 meeting, enhancements on MIMO for NR was updated [1],

- Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul:

  - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission

  - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission

  - Multi-TRP techniques for URLLC requirements are included in this WI

While multi-TRP/panel transmission has already been discussed in Rel-15, and some agreements were achieved:

Agreements:

- Adopt the following for NR reception:

    - Single NR-PDCCH schedules single NR-PDSCH where separate layers are transmitted from separate TRPs

    - Multiple NR-PDCCHs each scheduling a respective NR-PDSCH where each NR-PDSCH is transmitted from a separate TRP

    - Note: the case of single NR-PDCCH schedules single NR-PDSCH where each layer is transmitted from all TRPs jointly can be done in a spec-transparent manner

      - Note: CSI feedback details for the above case can be discussed separately

And the DMRS ports within a CDM group should be QCLed at least for single TRP case.

Agreement

A UE in MU-MIMO should be scheduled ports first within a specific CDM group, and then across CDM groups from UE perspective

- The ports within the same CDM group should be QCL-ed

- The above applies for the case of single TRP

And in RAN1#94bis meeting [2], agreement on multi-TRP/panel transmission was achieved as

Agreement

For eMBB multi-TRP/panel transmission down-select among the following in RAN1#95:

- Alt0: Support only single PDCCH design

  - FFS: Whether multiple PDCCH design is also needed

- Alt1: Support only multiple PDCCH design

  - FFS: Whether single PDCCH design is also needed

- Alt2: Support both multiple PDCCH and single PDCCH design

- FFS: PDCCH design for URLLC

Aspects to be considered in the down-selection: backhaul latency, downlink control overhead, specification impact (including RAN2 specs), UE complexity (related to power control, timing adjustment, and blind dection), DCI/UCI design, scheduler flexibility, intra-UE PUCCH/PUSCH transmission, Rel-15 PDCCH blockage probability, CSI feedback, etc

In this contribution, we provided our views on the multi-TRP/panel transmission.

- Discussion

- Configuration of DMRS and QCL

Considering multi-TRP transmission, the TRPs are usually geographically separated, leading to different transmission paths. Naturally, the signals from different TRPs are non-QCLed. On the other hand, to guarantee the orthogonality between DMRS ports multiplexed with CDM, these DMRS ports should be QCLed. So the DMRS ports within the same CDM group should also be QCLed for multi-TRP transmission.

Proposal 1: DMRS ports within the same CDM group should also be QCLed for multi-TRP/panel transmission.

Based on the non-QCLed DMRS ports from different TRPs, DMRS ports from different CDM groups should be indicated for the case of single NR-PDSCH with separate layers from separate TRPs, as shown in Figure 1. In addition, separate QCL parameters should be configured for the different DMRS port groups. This can be achieved by introducing one more TCI state in DCI or configuring two RS sets for one TCI state.

Figure 1. Single PDSCH with separate layers from separate TRPs

For the case of single PDSCH from separate TRPs, there may be a case of the link from one TRP getting worse, and UE falls back to be served by single TRP. As the link changes dynamically, PDCCH should be designed to support the dynamic switching between multi-TRP and single-TRP transmission. On the other hand, to retain less complexity for UE detection, it’s better to reuse the current DCI format, and same payload size of PDCCH for multi-TRP and single-TRP transmission.

In addition, multiple PDCCHs each scheduling a respective PDSCH are more suitable to cater for non-ideal backhaul.

Proposal 2: Support dynamic switching between single-TRP and multi-TRP scheduling.

Proposal 3: Support both single and multiple PDCCH design.

The links from different TRPs are usually different, while based on current spec, MCS is separately configured per codeword, so it’s better to allocate separate codewords for different TRPs. And also, if a UE is served by two TRPs, the UE is usually located at the boundary of the two TRPs, the total number of layers may not be too large. On this hand, supporting of two codewords with less than 4 layers is required.

In addition, based on the restriction of different CDM groups for DMRS ports from different TRPs, the number of port indications for multi-TRP is much less than that for single-TRP in current spec. Further overhead reduction may be achieved based on this. The reserved values for two codewords may be reused for indicating multi-TRP scheduling.

Furthermore, in the WID of MIMO enhancement, improved reliability and robustness is also proposed. One simple way is to reuse the SFN scheme for multi-TRP transmission, where single PDSCH with each layer is transmitted from all TRPs. While considering the high frequency band, impact of phase noise should be considered. In this case, spec-transparent manner is not suitable as the independent phase noises from different TRPs can not be estimated accurately based on SFN scheme. To achieve improved reliability and robustness, separate PTRS should be transmitted from different TRPs. Accordingly, separate DMRS ports with same transport block can be transmitted from different TRPs, and UE combines the data to achieve diversity gain.

Proposal 4: Enhancement on CW to layer mapping should be considered for multi-TRP transmission.

- CSI measurement and feedback enhancement

Considering the dynamic switching between single-TRP and multi-TRP transmission, the CSI measurement and feedback should be enhanced to support both cases. For the case of single PDSCH from different TRPs, joint CSI calculation based on two TRPs should be supported. And the CSI should include number of layers per TRP, i.e. RI1 and RI2. If the transmission is single-TRP based, either RI1 or RI2 is 0. While among the possible values of RI1 and RI2, some values are impossible under the restriction of maximum number of layers and codewords supported for one UE. In addition, considering the typical case for multi-TRP transmission, the two links should be matched, for example, there is impossible for the case of RI=1 and RI2=5. Based on this, the overhead reduction for joint RI report can be considered.

Proposal 5: Enhancement on CSI measurement and feedback for dynamic switching between single-TRP and multi-TRP transmission should be supported. And overhead reduction can be studied for the typical cases.

- Uplink transmission

For uplink transmission based on multi-TRP, at least dynamic switching between different TRPs should be supported. Non-codebook based transmission in Rel-15 can support this already, since higher layer parameter spatialRelationInfo is configured per SRS resource. While considering the different transmission path, the timing differences to different TRPs may be different. So independent TA for each TRP should be supported.

Proposal 6: Multiple TA should be supported for uplink transmission based on multi-TRP.

- Conclusion

In this contribution, we provided our views on the multi-TRP transmission, and we proposed that:

Proposal 1: DMRS ports within the same CDM group should also be QCLed for multi-TRP/panel transmission.

Proposal 2: Support dynamic switching between single-TRP and multi-TRP scheduling.

Proposal 3: Support both single and multiple PDCCH design.

Proposal 4: Enhancement on CW to layer mapping should be considered for multi-TRP transmission.

Proposal 5: Enhancement on CSI measurement and feedback for dynamic switching between single-TRP and multi-TRP transmission should be supported. And overhead reduction can be studied for the typical cases.

Proposal 6: Multiple TA should be supported for uplink transmission based on multi-TRP.

- References

- RP-182067, 3GPP TSG RAN Meeting #81, Gold Coast, Australia, Sept 10 - 13, 2018

- Chairman’s note, 3GPP TSG RAN1 Meeting #94bis, Chengdu, China, Oct 8 – 12, 2018