3GPP TSG RAN WG1 Meeting #95	R1-1812669

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.1.5

Source:	Huawei, HiSilicon

Title:	SLS evaluations and observations in uRLLC

Document for:	Discussion and Decision

Introduction

In RAN1#92 and RAN1#92bis, the system-level evaluation parameters for NOMA have been agreed as in the Appendix. In this contribution, we present system-level evaluation results for URLLC scenario.

System-level Evaluation

Evaluation method

Evaluation Parameters

For the SLS evaluation of URLLC scenario, inter-site distance (ISD) of 200m and carrier frequency of 4GHz are assumed. The allocated bandwidth is 12 PRBs, and the subcarrier spacing is 60 KHz.  There are 10 UEs in each cell, and the UEs are preconfigured with periodic time/frequency resources. The packet arrival per UE is based on FTP model 3 with Poisson arrival. The packet size is 60 bytes, and the TBS is fixed to be 60 bytes. The length of each transmission opportunity (TO) and the periodicity of TOs are both 7 OFDM symbol (OS). One-symbol DMRS is assumed, and the DMRS pool size is 12, i.e., there is no DMRS collision. In the simulation, there is no HARQ retransmission, and the number of repetitions can be 1 or 2. The evaluated NOMA schemes and assumptions on the receiver are similar to that of mMTC scenario [1]. The evaluation parameters are summarized in Table 1.

Table 1. Assumptions for NOMA evaluation in URLLC scenario

| Parameters | URLLC |
| --- | --- |
| ISD + Carrier Frequency | 200m + 4GHz |
| Simulation bandwidth & SCS | 12 PRBs, 60KHz |
| Resource allocation | 12PRBs per UE |
| Packet size | 60 bytes, no segmentation |
| Time duration and periodicity of each TO | 7OS |
| HARQ/repetition | 1 or 2 repetition, no retransmission |
| Number of UEs per cell | 10 |
| Packet arrival | FTP Model 3, Poisson arrival |
| UE power control | P0 = -85 dBm, alpha = 0.93 |
| Number of BS antennas | 4Rx |
| BS antenna downtilt | 102 |
| Channel estimation | Realistic |

Performance Metrics

As agreed in RAN1#92bis, the percentage of users satisfying reliability and latency requirements vs. packet arrival rate (PAR) is the performance metric for URLLC scenario [2]. The target reliability is 99.999% and the target delay requirement is 1ms.

The physical layer abstraction method has been presented in [3]. A simplified method system-level evaluation has been agreed in RAN1#93 [4], by which the mean BLER of a UE can be used to represent the reliability of the UE.

Evaluation Results and Analysis

Table 2 presents the configurations of evaluated NOMA schemes for SLS. For each configuration in terms of TBS, SF, and code rate (CR) for a NOMA scheme, the setting is the same for all the UEs and will keep unchanged during the simulation.

Table 2. Configurations of NOMA schemes

|  | SCMA | MUSA | SL-RSMA | ML-RSMA | LCRS |
| --- | --- | --- | --- | --- | --- |
| TBS = 60 bytes | CR=0.287 | CR=0.574 | CR=0.574 | CR=0.287 | CR=0.287 |

TO = 7OS, 1 repetition

Figure 1 shows the example of packet arrival and transmission when the length of TO is 7 OS and the number of repetition is 1. If one packet arrives during one TO, the packet will be transmitted at the next TO. If more than one packet arrive during one TO, then the packets will be transmitted according to the order of arrival.

Figure 1. Example of packet arrival and transmission (1 repetition)

PAR vs Satisfied UE ratio

In Figure 2, the satisfied UE ratios versus the PARs for different NOMA schemes are presented. As the PAR increases, the satisfied UE ratio decreases. It can be observed that the performance of SCMA, ML-RSMA, and LCRS are similar, and better than that of MUSA and SL-RSMA. At the agreed target 95% of UEs satisfying the reliability and latency requirements, the PAR of different NOMA schemes are summarized in Table 3. The results show that SCMA has better performance than other NOMA schemes.

Observation 1: In URLLC scenario of TO=7OS and 1 repetition, the NOMA schemes with same code rate have similar performance (with SCMA slightly better than LCRS and ML-RSMA), and the NOMA schemes with lower code rate perform better than higher code rate.

Figure 2. Satisfied UE ratio vs. packet arrival rate

Table 3. PAR at Satisfied UE ratio = 95%

| Evaluated Schemes | PAR at Satisfied UE ratio = 95% (pkts/s/cell) | Relative gain of SCMA |
| --- | --- | --- |
| SCMA | 2350 | / |
| MUSA | 1690 | 39% |
| SL-RSMA | 1690 | 39% |
| ML-RSMA | 2290 | 2.5% |
| LCRS | 2320 | 1.3% |

UE distribution

The UE distribution of different NOMA schemes when the PAR is 2000pkt/s/cell is shown in Figure 3. As there is no retransmission, the UE distribution is the same for all the NOMA schemes. It can also be observed that the number of UEs is less than 5 for almost all the cases.

Figure 3. UE distribution (PAR = 2000pkts/s/cell)

Table 4. Statistics of UE distribution

| # of UEs | SCMA | MUSA | SL-RSMA | ML-RSMA | LCRS |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.77635 | 0.77635 | 0.77635 | 0.77635 | 0.77635 |
| 1 | 0.19898 | 0.19898 | 0.19898 | 0.19898 | 0.19898 |
| 2 | 0.02302 | 0.02302 | 0.02302 | 0.02302 | 0.02302 |
| 3 | 0.00157 | 0.00157 | 0.00157 | 0.00157 | 0.00157 |
| 4 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 |
| 5 | 0 | 0 | 0 | 0 | 0 |

Corresponding LLS Performance

Given the UE distribution, we can further verify the SLS results by LLS. When TBS is 60 bytes and the number of receive antennas is 4, the LLS performance in terms of required SNR at BLER=10-3 versus SNR is given in Figure 4. The LLS results show that SCMA, LCRS, and ML-RSMA have similar performance, and better than MUSA and SL-RSMA, which is consistent with the above SLS results.

Figure 4. Required SNR @10E-3

TO = 7OS, 2 repetitions

In order to improve the reliability, repetitions can be applied. Due to the low latency requirement of URLLC scenario, the number of repetitions cannot be too large. Figure 5 shows the example of packet arrival and transmission when the length of TO is 7 OS and the number of repetition is 2.

Figure 5. Example of packet arrival and transmission (2 repetitions)

In Rel-15 NR, the transmission of a TB with repetitions can start at a TO associated with RV0 in a period but must end at the last TO in the same period, which can be referred to as ‘flexible start’. Flexible start of the repetitions can help to reduce the queuing time of a TB while ending at the last TO can remove the ambiguity on HARQ ID calculation even if some of the repetitions are miss-detected. However, when the repetitions of a TB start at the TOs other than the first TO within a period, the actual repetition number of the TB will be less than the configured number K, which may have reliability issue especially for reliability-critical services. In this sense, to guarantee a low-latency and yet reliable transmission, repetitions should be continued in time as long as the repetition number hasn’t reached K and no early ACK or UL grant is received.

PAR vs Satisfied UE ratio

In Figure 6, the satisfied UE ratios versus the PARs for different NOMA schemes are presented. As the PAR increases, the satisfied UE ratio decreases. It can be observed that the performance of SCMA, ML-RSMA, and LCRS are similar, and better than that of MUSA and SL-RSMA. At the agreed target 95% of UEs satisfying the reliability and latency requirements, the PAR of different NOMA schemes are summarized in Table 5. The results show that SCMA can have better performance than other NOMA schemes. Compare the results in Figure 2 and Figure 6, we can also observe that more repetitions can help to improve the overall performance of URLLC scenario.

Observation 2: In URLLC scenario of TO=7OS and 2 repetitions, the NOMA schemes with same code rate have similar performance (with SCMA performing better that LCRS and ML-RSMA), and the NOMA schemes with lower code rate perform better than higher code rate.

Figure 6. Satisfied UE ratio vs. PAR

Table 5. PAR at Satisfied UE ratio = 95%

| Evaluated Schemes | PAR at Satisfied UE ratio = 95% (pkts/s/cell) | Relative gains of SCMA |
| --- | --- | --- |
| SCMA | 2930 | / |
| MUSA | 2170 | 35.1% |
| SL-RSMA | 2170 | 35.1% |
| ML-RSMA | 2810 | 4.3% |
| LCRS | 2810 | 4.3% |

UE distribution

The UE distribution of different NOMA schemes when the PAR is 3000pkts/s/cell is shown in Figure 7. As there is no retransmission, the UE distribution is the same for all the NOMA schemes. It can also be observed that the number of UEs is less than 7 for almost all the cases.  Similar to the case of TO=7OS and 1 repetition, the SLS results are also consistent with the LLS results.

Figure 7. UE distribution (PAR = 3000pkts/s/cell)

Table 6. Statistics of UE distribution

| # of UEs | SCMA | MUSA | SL-RSMA | ML-RSMA | LCRS |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.45844 | 0.45844 | 0.45844 | 0.45844 | 0.45844 |
| 1 | 0.37209 | 0.37209 | 0.37209 | 0.37209 | 0.37209 |
| 2 | 0.13553 | 0.13553 | 0.13553 | 0.13553 | 0.13553 |
| 3 | 0.02936 | 0.02936 | 0.02936 | 0.02936 | 0.02936 |
| 4 | 0.00414 | 0.00414 | 0.00414 | 0.00414 | 0.00414 |
| 5 | 0.00041 | 0.00041 | 0.00041 | 0.00041 | 0.00041 |
| 6 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 |

Conclusions

In this contribution, we present the system-level evaluation results for URLLC scenario. Based on the discussion, we have following observations:

Observation 1: In URLLC scenario of TO=7OS and 1 repetition, the NOMA schemes with same code rate have similar performance (with SCMA slightly better than LCRS and ML-RSMA), and the NOMA schemes with lower code rate perform better than higher code rate.

Observation 2: In URLLC scenario of TO=7OS and 2 repetitions, the NOMA schemes with same code rate have similar performance (with SCMA performing better that LCRS and ML-RSMA), and the NOMA schemes with lower code rate perform better than higher code rate.

- References

R1-1812668, “SLS evaluations and observations in mMTC”, Huawei, HiSilicon, RAN1#95, Spokane, USA, November 12th – 16th, 2018.

“RAN1 Chairman’s Notes,” RAN1#92bis, Sanya, China, April 16th-20th, 2018.

R1-1812706, “Email discussion to collect PHY abstraction methods”, ZTE, RAN1#94bis, Chengdu, China, Oct. 8th-12th, 2018.

“RAN1 Chairman’s Notes,” RAN1#93, Busan, Korea, May 21st-25th, 2018.

R1-1812096, “Email discussion to collect PHY abstraction methods”, ZTE, RAN1#94bis, Chengdu, China, October 8th – 12th, 2018.

- Appendix

- A.1 System-level evaluation parameters

Table 2 System-level assumptions for NOMA evaluation

| Parameters | URLLC |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 200m for 4GHz 500m for 700MHz |
| Carrier frequency | 4GHz or 700MHz |
| Simulation bandwidth | 12 PRBs |
| Number of UEs per cell | Companies report |
| Channel model | UMa in TR 38.901; The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz. |
| UE Tx power | Max 23 dBm |
| BS antenna configurations | 2 Rx or 4 Rx for 700MHz; 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), 2 TXRU; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; dH = dV = 0.5λ; BS antenna downtilt: companies to report, FFS a single value 4 Rx or 16 Rx for 4GHz; 4 ports: (M, N, P, Mg, Ng) = (10, 2, 2, 1, 1), 4 TXRU; 16 ports: (M, N, P, Mg, Ng) = (10, 8, 2, 1, 1), 16 TXRU; dH = 0.5λ, dV = 0.8λ; BS antenna downtilt: companies to report, FFS a single value |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 1Tx as starting point |
| UE antenna height | Follow the modelling of TR 38.901 |
| UE antenna gain | 0dBi as starting point |
| UE distribution | For URLLC: 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell; Note: Other option(s) not precluded, e.g., 500m ISD, 80% of users are outdoors (3km/h), 20% of users are indoor (3km/h). |
| UE power control | Open loop PC for mMTC. Companies report the PC mechanisms used for eMBB and URLLC. |
| HARQ/repetition | Companies report (including HARQ mechanisms). |
| Channel estimation | Realistic |
| BS receiver | Advanced receiver, with baseline scheme is MU-MIMO (e.g., has the capability of spatial differentiation) Companies to provide analysis of complexity between baseline vs. advanced receivers |