3GPP TSG RAN WG1 Meeting #95	R1-1812682

# Spokane, USA, November 11th – 16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on rate matching in URLLC due to the ambiguity between PDCCH AL8 and AL16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current version of 38.214, the ambiguity between AL8 and AL16 PDCCH candidates and the corresponding solution is restricted to the configuration of a non-interleaved CORESET with 1 symbol duration. But this does not cover all cases, the ambiguity happens in all CORESET configurations when AL8 and AL16 have the same starting CCE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the restriction of the specific CORESET configuration for the ambiguity between AL 8 and AL16 PDCCH candidates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ambiguity between AL8 and AL16 PDCCH candidates will happen in other CORESET configurations, and UE will then do PDSCH rate matching around the wrong resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.4.1	PDSCH resource mapping with RB symbol level granularity

< Unchanged parts are omitted >

If a UE monitors PDCCH candidates of aggregation levels 8 and 16 with the same starting CCE index and if a detected PDCCH scheduling the PDSCH has aggregation level 8, and if a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to the aggregation level 16 PDCCH candidate and associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is equal to allContiguousRBs, the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the aggregation level 16 PDCCH candidate.

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is equal to allContiguousRBs, the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH.