3GPP TSG-RAN WG1 Meeting #95	R1- 1812925

# Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.9.2.1

Source:	Apple Inc.

Title:	Power Saving Techniques based on UE Adaptation

Document for:	Discussion, Decision

# Introduction

To reduce the power consumption at UE side, UE can take advantage of the information available inside handset such as file size, traffic arrival rate, application layer traffic’s throughput/delay requirements, implementation specific power consumption characteristics of RF and BB for different frequency, bandwidth, number of carriers, scheduling time line etc. Some of this information could be directly used at UE side for reducing power, but in most cases this information should be transferred to network to help network configure/schedule UE for power saving. This is because the power consumption of UE highly depends on UE configuration, and the configuration is done by the network.

In this paper, we discuss UE-assisted techniques for UE power saving.

# UE-assisted Techniques for Power Saving

## UE-preferred Bandwidth Part Configuration

### Motivation

NR supports higher data rate with wider bandwidth (up to 400MHz for single carrier). This requires much higher rate of data processing in both RF side and baseband side; supporting higher sampling rate, smaller slot lengths, higher encoder/decoder throughput, faster clocks, higher voltage (with higher leakage), larger memory, and LTE-NR dual connectivity require significantly higher power consumption. Especially, the support of mmWave requires additional power consumption for beam forming operation and beam management.

The BWP and cross-slot scheduling were introduced to overcome the increased power consumption. Network can use BWPs to adapt bandwidth to meet data transmission/reception demand. Network can configure narrow BWP for monitoring PDCCH only or wider BWP for receiving large-sized data. Channel bandwidth adaptation using BWP is an effective power saving technique.

However, whether UE can expect power saving or not is up to network configuration / scheduling decision. If network does not configure multiple BWPs with different sizes or if network does not change BWPs considering traffic demand and UE power consumption, then, even with multiple different BWPs configured, there could be no power saving at UE side (e.g., the case when BWP is used just for load balancing purpose). Similar example in LTE is DRX configuration. If only very simple configuration is used due to network complexity, then it is hard to expect intended power saving gain at UE side. UE having better knowledge on channel status and traffic demand/pattern/usage may help network to make better decisions for both BWP configuration and scheduling in both CONNECTED mode and IDLE mode as shown in Figure 1.

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for BWP configuration.

Figure 1 Current BWP framework (left) and improved BWP framework 
with UE preferred BWP configuration (right)

### Proposals

If UE can indicate its preferred BWP configuration, then UE can potentially achieve higher power saving. It is clear that UE has more information about device owner’s preference on applications, types/amount/duration of traffics to be received/transmitted, etc. This kind of information could be used in conjunction with internal power consumption statistics to determine the right size of BWP. Another reason is that different UE could have different RF capabilities and different implementations. This means optimal BWP for a certain traffic could be different for different UEs.

We see that there are strong motivation for network that it configures larger BWP (and larger number of component carriers (CC)). It is because that, with larger BWPs and larger number of CCs, network can keep its scheduling flexibility high to improve network capacity. Thus, in many cases, we see that component carriers are active without data transmission. In such case, UE should be able to request to use a smaller BWP or turn off unused/idling component carriers.

Proposal 1

- NR supports that UE can send UE-preferred BWP configuration to network.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices, and BWP inactivity timer.

### Performance Evaluation

#### Assumptions

A traffic model with deterministic inter-arrival time 200ms is used. File sizes of 0.1, 0.5, 2, and 4 MB are assumed. Each slot contains 2 PDCCH symbols, 10 PDSCH symbols, and 1 PUCCH symbol. FR1 power model from [7] is used. DL BWP and UL BWP are assumed to be the same.

#### Performance Analysis

Case 1 : Continuous PDCCH Monitoring without Sleep

Figure 2 shows our analysis results when UE continuously monitors PDCCH without any sleep. We make following observations.

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption. A larger bandwidth consumes more RF power, which increases power consumption. A smaller bandwidth leads to longer DL PDSCH reception, which also increases power consumption. For example, Figure 2 (left) shows that 20MHz BWP is optimal with 2MB packet size.

- The optimal BWP size changes based on traffic load. The optimal BWP size decreases with light traffic and increases with heavy traffic. The reason is that PDCCH-only dominates when traffic is sporadic. Smaller BWPs reduce average DL power. When traffic load increases, larger BWPs reduces PDSCH reception time and minimize average DL power. For example, 10MHz BWP is optimal with 0.1MB packet size while 20MHz or 40MHz is optimal with 4MB packet size.

- The optimal BWP is also dependent on UL power level. With higher UL power levels, the optimal BWP may increase. Because it reduces not only PDSCH reception time but also ACK transmissions in UL. For example, with 4MB packet size, 20MHz and 40MHz BWP are optimal at 0dBm and 23dBm UL tx power, respectively.

Figure 2 UE power consumption with tx power 0dBm (left) and 23dBm (right). Continuous PDCCH monitoring is assumed.

Observation 2

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption.

- The optimal BWP changes based on traffic load.

- The optimal BWP is also dependent on UL power level.

Case 2 : Grant arrival with Sleep Operation

When sleep operation between traffic is considered, the optimal BWP size for a given traffic size could be changed. Figure 3 show that the optimal BWP size is 100MHz for most packet sizes except for 0.1MB. It turns out that a larger BWP allows UE to complete DL PDSCH reception as soon as possible and then switch to sleep mode to save power. This is true for most cases.

Figure 3 UE power consumption with tx power 0dBm (left) and 23dBm (right). Sleep operation assumed between file transfer.

Figure 4 shows power saving gain of optimal BWP with sleep operation w.r.t smallest BWP of 10MHz.

Figure 4 UE power saving ratio with tx power 0dBm (left) and 23dBm (right). Sleep operation assumed between file transfer.

Observation 3

- A larger BWP allows UE to complete DL PDSCH reception quickly and then to switch to sleep mode to save power.

- The optimal BWP size depends on the existence of sleep operation.

- With sleep operation, power saving gain of optimal BWP could be as large as 70%.

## UE-preferred Timeline Adaptation for Power Saving

### Motivation

NR has very flexible HARQ scheduling timing. Unlike LTE with fixed HARQ timing, the transmission timing for NR’s PDSCH and corresponding ACK can be dynamically determined and indicated by DCI. The increased flexibility makes network scheduler much easier to serve multiple UEs with different processing capability, different traffic demand, different latency requirement, etc.

We see that self-contained slot scheduling could potentially help UE power saving. For example, note that a UE which receives PDCCH in slot N, PDSCH in slot N+4, and sends ACK in slot N+8 can enter micro sleep state between slot N and N+4 and between N+4 and N+8. If network can schedule this UE with PDCCH, PDSCH, and ACK all in the same slot N (self-contained slot scheduling) or at least very close in time, then the UE can quickly finish data reception and enter deeper sleep mode for longer time, which save more power as shown in Figure 5. Since there is power cost for ramp up and ramp down, it is better to reduce those power cost and make modem sleeps longer. This can be achieved by UE’s indication of preferred K0,K1,K2 value signaling to network.

Figure 5 Reduced power consumption of self-contained slot scheduling compared slot scheduling with K0>0 and K1>0 illustrating the reduced power consumption (dark green) and lower power consumption during sleep

Proposal 2

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

### Performance Evaluation

To evaluate the performance gain, we consider a case where UE monitors PDCCH every D slots and UE receives a grant every D slots. This is static model where traffic arrives regularly. The scheduling of PDSCH and ACK is up to network decision taking into account other UEs resource allocation. In real scenario, the period D could also understood as random grant inter-arrival time. But, in this analysis, we assumed the fixed value for simplicity. In this analysis, we evaluate the average power consumption during D slots for various K0 and K1 values.

#### Case 1

In this case, we consider the case with PDCCH(2)/PDSCH(12)/ACK(14), where number inside parenthesis is the number of symbols.

Assumptions

We make following assumptions for the evaluation purpose.

- UE monitors PDCCH once every D=30 slots.

- Network schedules one PDSCH every D slots.

- The number of OFDM symbols for PDCCH/PDSCH/PUSCH is 2/12/14 respectively.

- If K1>0, UE receives PDSCH only for 12 symbols without PDCCH decoding in the slot where PDSCH is transmitted.

- For PDSCH-only slot, we introduce a new power state MS(2)PDSCH(12) where the number denote the length of OFDM symbols for Micro Sleep and PDSCH respectively. The power consumption number was determined based on the agreed numbers and breakdown method as described in [6].

  - MS(2)PDSCH(12) with power = 274.8

- UE can enter sleep state for the slots where UE does not monitor PDCCH, receive PDSCH, or transmit ACK, where the sleep state is chosen such that UE consume lowest power consumption during the sleep.

Performance Analysis

We evaluate the average power consumption for duration D for different combination of K0 and K1 values. Figure 6 shows the typical timeline for PDCCH/PDSCH/ACK transmission and sleep operation between active slots where transmission/reception happens. For D slots, one PDCCH, one PDSCH and one ACK is received. If there is any time duration for potential sleep for K0>0, K1>0, then UE enters one of the three sleep states which gives the lowest power consumption.

Figure 6 A typical time line for PDCCH/PDSCH/ACK transmission (K0=3, K1=3)

Figure 7 shows the average power consumption for four different cases (K0,K1)=(0,1), (1,1),(3,3), and (6,6). The case (0,1) has 31.5% power saving gain compared to (6.6). The general trend is that the short values for K0/K1, the higher power saving gain. This is due to the fact that UE can enter deeper sleep mode when sleep time is longer. Considering that there is significant inter-grant arrival duration between grants as shown in [5], it is expected that controlling processing time line of tx/rx activity close in time gives a significant potential for UE power saving.

Figure 7 Average power consumption for PDCCH#=2, PDSCH#=12, PUCCH#=14

#### Case 2

In this case, we consider PDCCH(2)/PDSCH(9)/PUCCH(2).

Assumptions

To evaluate this scenario we introduced following additional states. We have used the same methodology and the make following additional assumptions.

- The number of OFDM symbols for PDCCH/PDSCH/PUCCH is 2/9/2 respectively. This replaces previous assumption 3)

- If K1>0, UE receives PDSCH only for 9 symbols without PDCCH decoding in the slot where PDSCH is transmitted. This replace previous assumption 5).

- We introduce following new states and corresponding power consumption numbers for evaluation. The state definitions and power number calculations are given in [6].

  - PDCCH(2)PDSCH(9)MS(1)PUCCH(2) with power = 284.8

  - MS(12)PUCCH(2) with power = 74.3

  - MS(2)PDSCH(9)MS(1)PUCCH0dBm(2) with power = 246.7

  - PDCCH(2)PDSCH(9)MS(3) with power = 242.3

  - MS(2)PDSCH(9)MS(3) with power = 217.3

Performance Analysis

Figure 8 shows the typical timeline for PDCCH(2)/PDSCH(9)/PUCCH(2) transmission and sleep states between active slots where transmission/reception happens.

Figure 8 The time line for PDCCH/PDSCH/PUCCH transmission (K0=3, K1=3) with PUCCH#=2

Figure 9 shows that the self-contained slot approach (K0=K1=0) could give up to 48% of power saving gain under the assumption of PDCCH#=2, PDSCH#=9, PUCCH#=2.

Figure 9 Average power consumption for PDCCH#=2, PDSCH#=9, PUCCH#=2

Observation 4

- Controlling K0, K1 values can give the power saving gain of 31% ~ 48%.

## UE-preferred Time Domain Resource Allocation Table Configuration

### Motivation

In NR, 3GPP 38.331 defines very flexible PDCCH/PDSCH time-domain resource allocation (TDRA) pattern. In particular, RRC can configure one table per BWP, which includes a list of possible (up-to 16) TDRA patterns. Each entry of the TDRA table includes three fields, namely, K0, PDSCH mapping type, and StartSymbolAndLength. K0 defines the distance between PDCCH and PDSCH in slot (i.e., K0=0 indicates same-slot scheduling, whereas K0>0 indicates cross-slot scheduling); mapping type defines the scheduling to be either slot-based or mini-slot based; and startSymbolAndLength defines the location and span of PDSCH allocation within the slot. At PHY layer, PDCCH DCI decoding provides an entry index of the table, indicating to UE which entry of the TDRA table should be used in current slot. In theory, gNB can change TDRA from slot to slot by DCI. UE does not know the pattern used for current slot before PDCCH decoding finishes.

Based on our study, we found that many different TDRA patterns, including both same-slot scheduling and cross-slot scheduling patterns, can provide significant power savings for UE, provided that the pattern (i.e., the entry of the table) is known by UE before DCI decoding. In the performance evaluation sections, we provide a few examples of such patterns, and use agreed power model as well as newly derived power model to project UE power saving.

### Performance Evaluation

#### Assumptions

We make following assumptions for following evaluation.

- PDCCH occupies the first 2 symbols of the slot.

- PDCCH reception and decoding takes a total of 5 symbol-time.

- RRC configures the TDRA table with multiple entries, which may cover a wide range of TDRA patterns including both same-slot and cross-slot scheduling patterns. Examples of such table can be 38.214 Table 5.1.2.1.1.-2, Table 5.1.2.1.1.-4, and Table 5.1.2.1.1.-5.

- In order to project power consumption of the new power model, we derive the following breakdown power number based on existing agreed power model. The details are explained in [6].

| Use Case | Power Number |
| --- | --- |
| DL RF power consumption per symbol | 13.2 |
| PDCCH baseband processing power | 5 |
| PDSCH baseband processing power per symbol | 110/12 |
| Micro-sleep power per symbol | 45/14 |

#### Case Study

##### Existing Power Model Scenarios

Case-1: baseline LTE-like PDCCH/PDSCH allocation. For the baseline case, there is no chance for UE power saving, no matter UE knows TDRA in advance or not. The power consumption for UE with and without grant, as agreed in the last meeting, is projected to be 300 and 100, respectively.

Case-2: cross-slot scheduling with LTE-like PDCCH/PDSCH allocation.

Figure 10 (a) plots the LTE-like cross-slot scheduling pattern, which was also discussed in the last meeting. With cross-slot scheduling, if TDRA is known to UE before DCI decoding, UE can shut-off RF after 2 symbols in case there is no grant from previous slot. The power consumption for UE, as agreed in last meeting, can be reduced by 30% from 100 to 70 if there is no grant.

Figure 10 (b) shows the power consumption comparison as a function of grant arrival probability. We can see the gain of such TDRA diminishes when the grant arrival probability becomes higher.

(a) TDRA pattern                          (b) Power consumption

Figure 10 Case-2: cross-slot scheduling with LTE-like time-domain resource allocation.

##### New Power Saving Model Scenarios

Case-3: same-slot scheduling with scattered PDCCH/PDSCH allocation. Figure 11 (a) plots a new power model, which is same-slot scheduling with scattered PDCCH/PDSCH. If TDRA is known to UE before DCI decoding, UE can blindly shut-off RF after 2 symbols. Depending on whether grant is present or not, UE can turn back RF on to receive PDSCH. In such case, no matter grant is present or not, UE can achieve power saving. We can show that the UE achieves a constant of 30 power saving across the board which is plotted in Figure 12 as well.

(a) case-3                                           (b) case-4

Figure 11 same-slot scheduling power saving cases.

Case-4: same-slot scheduling with overlapped PDCCH/PDSCH allocation. Figure 11 (b) plots same-slot scheduling with overlapped PDCCH/PDSCH allocation. Similar to Case 3, if UE knowns TDRA before DCI decoding, UE can achieve constant power saving no matter there is grant or not.

(a) case-3                                    (b) case-4

Figure 12 power consumption comparison b/w knowing TDRA vs. unknown TDRA before DCI decoding

Finally, Figure 13 plots the power saving comparison between different cases. Case 3 and Case 4 provide significant power saving across the board.

Figure 13 Power saving comparison between different cases.

### Proposals

In order to realize the power saving illustrated above, UE has to know the TDRA patterns before DCI decoding. Based on current standard, this is only possible if RRC configures the TDRA table with a single entry. Each BWP supports a single TDRA pattern. To change TDRA, gNB/UE has to switch BWP. Obviously, there are many drawbacks of using this approach to achieve power saving:

- This almost removes the all the flexibilities of TDRA defined in the standard.

- There could have many different resource allocation patterns (for different purposes), but we may not have enough BWP (<= 4) to support them.

- The concept of BWP can be utilized for many other important purposes. This approach hijacks the BWP just for different TDRA.

- Multiple BWP is an optional feature which may not be supported by infra or UE vendors at least in the initial deployment phase. However, UE does want to get power saving benefit even without multiple BWP support.

To summarize, from case study and above discussion, we have the following observations:

Observation 5

- The key to achieve power saving is not really same-slot scheduling vs. cross-slot scheduling. Rather, power saving is tied to specific TDRA configuration, and UE has to know such TDRA before DCI decoding.

Observation 6

- Multiple BWP may not be the efficient way to achieve power saving from different TDRA patterns.

To solve this problem, we make following proposals.

Proposal 3

- NR supports a MAC CE which can activate/deactivate a subset of TDRA entries for a particular BWP. A scheduling DCI can indicate one of activated entries only.

Proposal 4

- NR supports that UE report its preferred TDRA entries back to gNB among the entries in the RRC-configured TDRA table.

Proposal 5

- NR supports that UE sends a TDRA entry change request.

## Fast SCell Activation/Deactivation

### Motivation

It has been noted that LTE CA consumes significant amount of power. But, the real problem is not that it consumes high power but that a lot portion of power for activating secondary carrier is being wasted instead of being used for data transmission. In typical CA scenarios, the following problems have been observed.

- Activation delay : SCells are typically activated only after network DL queue builds up beyond a certain threshold, which delays quick data transfer due to the delay for activation.

- Deactivation delay : Network does not quickly deactivate after data transfer finishes although there is no active data transfer happening. Even though there is MACCE-based dynamic activation/deactivation mechanism, it is hardly used in the field and deactivation mostly rely on SCell deactivation timer which is typically configured in the order of hundreds of ms (typically on the next on-duration interspersed by a long CDRX cycle)

- SCells causing low overall achievable throughput: Sometimes it is observed that SCells with low achievable throughput do not always contribute to the overall achievable throughput by a huge margin. In such situations, the advantages of additional carriers for higher achievable throughput is limited by the power penalty of CA activation itself.

In such situations, a UE based input to the network will always be advantageous in terms of CA activation/de-activation. The network can make better decisions based on these UE provided inputs.

### Proposals

Fast activation / deactivation could help UE in power savings. To reduce the required delay for activation and deactivation, the first approach we can think is DCI-based signaling. Since DCI is processed in the physical layer, it is faster than current MAC-CE based scheme.

DCI-based implicit Activation Mechanism

More specifically, we can consider DCI-based implicit activation mechanism using cross-carrier and cross-slot scheduling. This approach requires CA configured with cross carrier scheduling support. To activate a carrier (which is currently deactivated), network can simply send a scheduling DCI with Carrier Indicator Field (CIF) field set to the scheduled component carrier. In this case, the scheduling DCI should have K0 value larger than the time required for component carrier activation.

Proposal 6

- NR supports the scheduling DCI-based carrier activation mechanism.

UE-initiated Fast SCell Activation / Deactivation

Another aspect of improvement is UE-initiated activation/deactivation approach. This is quite useful for reducing the delay for activation/deactivation since UE can use information available in application layer such as file size to download or upload.

- SCell activation in file downloading : When a UE wants to download a file, UE can know that it will start downloading it soon from application layer and sends an explicit request to network to turn on additional component carriers in advance. This is really effective in reducing delay since network usually does not have access to this information primarily due to the data transfers being performed within TLS sessions. Network usually turns on additional component carrier only after downlink queue starts to build up beyond a certain threshold, which is a bit slow. From our tests, we observe that activation delay can be reduced from 35ms to 12ms.

- SCell deactivation in file downloading : Current LTE/NR supports sCellDeactivationTimer which is used to deactivate SCells. The timer is reset every time a DL/UL grant is received. It expires when there is no DL/UL grant is received for sCellDeactivationTimer duration. The configurable timer values ranges from 20ms to 1280ms and usually end up with values which are of the range of a long DRX cycle for best effort flows. If this parameter is not configured, then a value of infinity is used and SCell can be deactivated by MACCE only. In the field logs, we indeed see wide ranges of values for this timer from 20ms to infinity. Due to such large de-activation values, even after file downloading is completed, UE has to wait until the timer expires in order for SCells to be de-activated. This problem is exaggerated especially when the downloaded file is smaller in size. For small file downloads, the actual active transfer time of data is much smaller compared to the PDCCH monitoring time during which the SCell is active yielding to highly inefficient RRC connections on the UE that consume significant power. In this scenario, if the UE has a mechanism to indicate to the network a request for SCell deactivation, the UE can then avoid the power penalty of PDCCH monitoring with CA. From our tests, we observe that deactivation delay is reduced on an average from 130ms to 20ms.

- SCell activation in file uploading : When UE starts sending data in UL from RRC IDLE (which is typical case when uploading large file), it first has to connect to network to get to RRC-CONNECTED mode. In this case, MSG3 could include BSR. It is kind of earliest time that a UE in out of sync can tell network that it has a lot of data to transmit to network. So, in this case, there is not much benefit of UE-initiated SCell activation. However, when UE starts transmitting during RRC-CONNECTED mode from PCell only, UE can proactively send SCell activation request before BSR transmission occasion comes. In this case, there is some reduced delay, which is on average the half of BSR reporting period. So, in this case, the reduced delay depends on BSR reporting period.

- SCell deactivation in file uploading : This case is very similar to SCell deactivation in file downloading. Since UE knows whether file uploading has been finished or not, the UE can immediately send SCell deactivation request to network to avoid wasting energy for SCell cells. From our tests, we observe that deactivation delay is reduced from 192ms to 16ms.

Observation 7

- UE can significantly reduce SCell activation/deactivation delay using UE-initiated SCell activation/deactivation mechanism based on UE’s knowledge in UE’s application layer (e.g., file size to download/upload). Since such information is available only in the UE, UE-initiated approach is better than network-initiated approach.

Proposal 7

- NR supports UE-initiated SCell activation/deactivation request mechanism.

### Performance Evaluation

In this section, we show the experiment results in real networks under given test scenarios.

#### Power Efficiency of UE-initiated Fast SCell Activation/Deactivation

The first scenario we have consider is file transfer. Since, in this scenario, the activation / deactivation of SCell kicks in only in the beginning and end of file transfer respectively, the file size has impact on overall power consumption. We have following setup for this experiment.

- Consider two file sizes of 1MBs (most typical download size) and 25MBs (average size).

- Total three carriers (1 PCell and 2 SCells) are activated for downloading a file.

- The power efficiency is measured for the following two schemes.

  - S1) Network-initiated SCell activation/deactivation mechanism : SCells are activated/deactivated based on indication from network

  - S2) UE-initiated Fast SCell activation/deactivation mechanism : UE sends a message to network to request activation/deactivation of SCells.

- Power saving gain is used as performance measure, which is defined as follows.

  - Power saving gain = (the total energy consumed by S1 - the total energy consumed by S2 ) / the total energy consumed by S1

- The throughput status of each carrier is denoted as either G(Good), M(Medium), B(Bad) depending on the achievable throughput (instead of channel condition or SNR). This means that if a carrier supports many UEs simultaneously, then it’s throughput status could be B(Bad) even with a very high SNR.

| Condition | Throughput (Mbps) |
| --- | --- |
| Good (G) | 25 |
| Medium (M) | 15 |
| Bad (B) | 5 |

Figure 14 shows the power efficiency of UE-initiated Fast SCell activation/deactivation mechanism. For small file size (1MB), we see a significant power saving gain up to 43%. The gain is essentially coming in from cutting down the PDCCH monitoring time in a multi-carrier scenario. For 25MB file size, the gain is limited to 3% since it takes much longer time to download due to the PDCCH monitoring duration being insignificant. The gain gradually reduces as the total aggregated throughput decreases. This is because the reduced aggregated throughput increases the total file transfer time.

Figure 14 Power saving gain of UE-initiated fast SCell activation/deactivation mechanism compared to network-initiated mechanism. G/M/B denotes Good/Medium/Bad throughput for each component carrier.

Observation 8

- UE-initiated SCell activation/deactivation scheme always provide power saving gain compared to default network-initiated CA activation/deactivation mechanism.

- The power saving gain for most typical size file (1MB) is around 43%.

#### Power Efficiency of UE-initiated Approach vs Single Component Carrier (PCell only)

In the second scenario, we consider the same file transfer with following setup.

- Consider two file sizes of 1MBs (most typical download size) and 25MBs (average size).

- Following two schemes are considered.

  - S3) PCell only : Only PCell is used for file transfer.

  - S2) UE-initiated Fast SCell activation/deactivation mechanism

- For S2, total three carriers (1 PCell and 2 SCells) are activated for downloading a file.

- Same metric is used as in previous section.

In this scenario, it has been observed that, especially for cases with Bad component carriers activated, the power saving gain is negative as shown in Figure 15. Note that in those cases with negative gains (GMB, GBB, MBB), the additional component carriers (2nd and 3rd) have Medium or Bad throughput (except the BBB case). The throughput contribution of carriers with Bad/Medium throughput is low compared to amount of additional energy consumed. The power cost for activating additional carrier (×2.5 for 3CA) does justify the marginal throughput improvement from UE power saving perspective. Thus, the power efficiency is negative. In such case, it is better to use Good PCell only.

In our field test, we often see cases like GMB, GBB, or MBB. We guess such configuration happens since some networks do not consider the achievable throughput of additional component carrier. The UE-initiated activation/deactivation request is very effective measure to handle such case. If there is no UE-initiated activation/deactivation mechanism, then UE’s power consumption would increase significantly without throughput increase, which is the situation that UE should avoid.

In the last case (BBB), primary carrier is also in Bad status. Thus, by activating additional carriers, throughput is tripled anyway. This is exactly the same situation as GGG, thus we see high power saving gain in the case.

Figure 15 UE-initiated fast SCell activation/deactivation power saving gain w.r.t PCell only case. G/M/B denotes Good/Medium/Bad throughput.

Observation 9

- In case network activates additional carriers which supports low throughput, UE-initiated SCell deactivation mechanism could be effectively used to request the deactivation of the carriers with low power efficiency (bit/Joules).

## MIMO Layer Restriction

### Motivation

NR supports up to two codewords with variable number of MIMO layers. First four layers are mapped to the first codeword and remaining 4 layers are mapped to the second codeword. Once the number of codeword supported is RRC-configured, then the actual number of MIMO layers for PDSCH transmission is dynamically indicated in DCI.

The number of layers indicated in DCI is up to network implementation, but typically it is determined based on the rank indication reported from UE. However, note that the rank indication from UE does not guarantee that network schedules PDSCH only the layers less or equal to the reported rank. It is possible that network schedules number of layers larger than reported rank, although it is less likely. Due to this reason, UE is supposed to be ready for receiving up to maximum number of layers always which is RRC-configured; 4 for a single codeword and 8 for dual codewords.

At UE side, buffering up to maximum number of layers consumes significant amount power/energy. RFIC includes a dedicated receiver chain for each layer and some blocks in BB such as ADC is also layer specific. Thus, power consumed by those blocks linearly increases as the number of max layer increases.

The problem is that UE has to make maximum number of RF chains ready even when reported rank is lower than the maximum. In addition to that, the RRC-configured maximum number of layers is not adaptable to dynamic traffic arrival, which makes UE waste power in buffering unnecessary signals for unused layers.

For UE power saving, we propose to restrict the maximum number of MIMO layers. Since the number of layers is directly related to the amount of data rate to be supported, the maximum number should be controlled based on traffic arrival. When traffic arrival rate is high, and rank is high, then large maximum number could be configured. When traffic arrival rate is low, the small maximum number of layers (equal or lower than rank) could be configured.

Proposal 8

- NR supports limiting the maximum number of MIMO layers. The possible maximum number ranges from 1 to 4 for single codeword and 1 to 8 for dual codeword.

### Performance evaluation

In this section, we do simple back-of-the-envelope type calculation based on power model agreed.

#### Assumptions

We make following assumptions.

- Assume K0=0, K1=0

- In each slot, grant arrival occurs i.i.d fashion with probability p.

- We assume channel rank is limited to 2.

- We consider following two cases.

  - C1) Configured max number of MIMO layer is 4.

  - C2) Configured max number of MIMO layer is 2.

- Let PPDCCH-only,4Rx be the power for PDCCH-only state with 4 Rx.

- Let PPDCCH-only,2Rx be the power for PDCCH-only state with 2 Rx.

  - The power scaling factor for 2Rx is 0.7 according to [7].

  - Thus, we have PPDCCH-only,2Rx = 0.7PPDCCH-only,4Rx

- Let PPDCCH+PDSCH,4Rx be the power for PDCCH decoding, PDSCH decoding with 4 Rx.

- Let PPDCCH+PDSCH,2Rx be the power for PDCCH decoding, PDSCH decoding with 2 Rx.

  - The power scaling factor for 2Rx is 0.7 according to [7].

  - Thus, we have PPDCCH+PDSCH,2Rx = 0.7PPDCCH+PDSCH,4Rx

#### Analysis

Case 1 : Continuous PDCCH monitoring with no sleep

In the Case 1, we make an additional assumption that UE continuously monitors PDCCH every slot.

For slots without grant arrival, UE performs PDCCH decoding only with configured number of antennas in both cases C1 and C2. Thus there is 30% of power saving for 2Rx.

For slots with grant arrival, UE performs PDCCH and PDSCH decoding with configured number of antennas in both cases C1 and C2. Thus there is 30% of power saving for 2Rx.

Considering both cases, we have following average power.

- Average Power of C1 = PPDCCH+PDSCH,4Rx × p + PPDCCH-only,4Rx × (1-p)

- Average Power of C2 = PPDCCH+PDSCH,2Rx × p + PPDCCH-only,2Rx × (1-p) = 0.7 × Average Power of C1

Observation 10

- This there is 30% of power saving gain when UE is configured with max MIMO layer of 2 compared to the case with max MIMO layer of 4.

Case 2 : PDCCH monitoring with sleep between grant arrival

In the Case 2, we assume that there exist a Genie sleep indication which makes UE to enter sleep state between two consecutive grants. Using the statistical model in our companion paper [6][8], we can get following results. For evaluation of 4Rx, we used P0. For evaluation of 4Rx, we used 0.7P0.

Figure 16 shows the average power consumption of UE configured with max 4Rx and that of UE configured with max 2Rx. Unlike the Case 1, when sleep operation is introduced, the power saving gain is reduced because there is no more power saving between 4Rx and 2Rx during sleep operation.

Figure 16 Average Power consumption for 4Rx and 2Rx with Genie sleep indication between grants (left). Power saving gain of 2Rx compared to 4Rx is shown as a function of grant arrival probability p.(right)

Observation 11

- The sleep operation could reduce the power saving gain of MIMO layer restriction.

## UE-assisted DRX configuration

DRX mechanisms is one of the key component mechanism for UE power saving, but it is less utilized in LTE. For example, it has been observed that many LTE networks use only limited set of DRX parameters and they are hardly changed. Long inactivity timer value (~200ms) is one of such examples. Another important observation we made is that in many cases the same DRX configuration is applied to all the UEs implying that network does not usually take into account different characteristics of UE’s traffic. The only exception we have observed is VoLTE case. When VoLTE call is going on, network changes DRX parameters to support it in more power efficient manner. Considering that NR DRX design is essentially is not different from that of LTE, we expect that NR network would have similar configurations as LTE network.

However, since the scope of NR use case / scenarios are much wider than that of LTE, we expect much more variation in traffics arrival / demand in NR. For power efficient operation at UE, it is critical to quickly adapt to such variations. The DRX parameters are one of key parameters which enables such adaptation. But, as pointed out above, it is quite static and not fully utilized in most of cases. Thus, we propose to utilize it further for adaptation and to support better, we think UE can provide assistance information to network based on the knowledge inside UE (such as traffic arrival statistics, user behavior statistics, power consumption statistics).

Proposal 9

- NR supports that UE’s indication of its preferred DRX parameters.

In above discussion, DRX cover both Connected mode DRX (CDRX) and Idle mode DRX. But CDRX is much more relevant in the context of adaptation to dynamically varying traffic.

Considering the most common use case of BWP is power saving, we see the connection between BWP and DRX. Given that both features can be configured for quick adaptation to traffic arrival while trying to reduce power consumption, we can consider the joint operation of the two features. For example, NR could support the per BWP DRX configuration. For example, a narrow bandwidth part BWP1 and wide bandwidth part BWP2 could be configured with different values for short/long DRX cycle timer, on duration timer, inactivity timer, etc. The values for BWP1 could be configured as different from than that for BWP2 to further reduce power consumption when there is not much traffic coming in BWP1. This kind of association between BWP and DRX parameters could provide increased flexibility enabling better adaptation.

Proposal 10

- NR study the association between a BWP and DRX operation.

# Conclusions

In this contribution, we have identified a few important UE-assisted power saving mechanisms in the area of bandwidth part, UE-preferred Timeline Adaptation for Power Saving, cross-slot scheduling, UE-initiated SCell activation/ deactivation, MIMO layer restriction and BWP-DRX parameter association. We made following observations and proposals based on the discussion.

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for both BWP configuration.

Proposal 1

- NR supports that UE can send UE-preferred BWP configuration to network.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices, and BWP inactivity timer.

Observation 2

- For a given traffic load, there exists an optimal bandwidth that minimizes power consumption.

- The optimal BWP changes based on traffic load.

- The optimal BWP is also dependent on UL power level.

Observation 3

- A larger BWP allows UE to complete DL PDSCH reception quickly and then to switch to sleep mode to save power.

- The optimal BWP size depends on the existence of sleep operation.

- With sleep operation, power saving gain of optimal BWP could be as large as 70%.

Proposal 2

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

Observation 4

- Controlling K0, K1 values can give the power saving gain of 31% ~ 48%.

Observation 5

- The key to achieve power saving is not really same-slot scheduling vs. cross-slot scheduling. Rather, power saving is tied to specific TDRA configuration, and UE has to know such TDRA before DCI decoding.

Observation 6

- Multiple BWP may not be the efficient way to achieve power saving from different TDRA patterns.

Proposal 3

- NR supports a MAC CE which can activate/deactivate a subset of TDRA entries for a particular BWP. A scheduling DCI can indicate one of activated entries only.

Proposal 4

- NR supports that UE report its preferred TDRA entries back to gNB among the entries in the RRC-configured TDRA table.

Proposal 5

- NR supports that UE sends a TDRA entry change request.

Proposal 6

- NR supports the scheduling DCI-based carrier activation mechanism.

Observation 7

- UE can significantly reduce SCell activation/deactivation delay using UE-initiated SCell activation/deactivation mechanism based on UE’s knowledge in UE’s application layer (e.g., file size to download/upload). Since such information is available only in the UE, UE-initiated approach is better than network-initiated approach.

Proposal 7

- NR supports UE-initiated SCell activation/deactivation request mechanism.

Observation 8

- UE-initiated SCell activation/deactivation scheme always provide power saving gain compared to default network-initiated CA activation/deactivation mechanism.

- The power saving gain for most typical size file (1MB) is around 43%.

Observation 9

- In case network activates additional carriers which supports low throughput, UE-initiated SCell deactivation mechanism could be effectively used to request the deactivation of the carriers with low power efficiency (bit/Joules).

Proposal 8

- NR supports limiting the maximum number of MIMO layers. The possible maximum number ranges from 1 to 4 for single codeword and 1 to 8 for dual codeword.

Observation 10

- This there is 30% of power saving gain when UE is configured with max MIMO layer of 2 compared to the case with max MIMO layer of 4.

Observation 11

- The sleep operation could reduce the power saving gain of MIMO layer restriction.

Proposal 9

- NR supports that UE’s indication of its preferred DRX parameters.

Proposal 10

- NR study the association between a BWP and DRX operation.

# References

[1] R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

[2] R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

[3] RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

[4] R1- 1714092 Control Information for UE Power Saving, Apple

[5] R1-1811127 Network-indication based Approaches for UE power saving, Apple, Oct 2018

[6] R1-1812924 Evaluation methodology for UE power saving SI, Apple, Nov, 2018.

[7] R1-xxxxxx Chairman’s Note RAN1 94b, Cheng-du, Oct, 2018

[8] R1-1812926 Power saving techniques based on explicit indication, Apple, Nov, 2018